Annex 3

to the Procurement Conditions

TECHNICAL SPECIFICATION OF A DIGITAL DOSIMETER

2024 ___________________ No.

- Digital dosimeter (hereinafter abbreviated as a – Dosimeter) shall measure individual dose equivalent Hp(10).

- The detectors of the Dosimeter shall measure gamma and register neutron radiation.

- All the detectors of the Dosimeter shall be integrated into the Dosimeter.

- The Dosimeter shall directly indicate dose rate and accumulated dose on a display.

- The Dosimeter shall have a function of a switching the display backlight on and off.

- The Dosimeter shall be compatible to set one threshold value for dose rate and one threshold value for dose above which an audible and (or) visual alarm would be activated.

- The Dosimeter shall provide an audible and (or) light alarm if the dose rate or dose thresholds are exceeded in order to avoid ineligible exposure.

- The Dosimeter shall have a possibility to switch off the audible and (or) light alarm indicating that the dose rate or dose thresholds are exceeded.

- The Dosimeter shall have a retaining device(s) in order to hang the Dosimeter on the neck or slip it to the clothing.

- The dimensions of the Dosimeter shall not exceed 15 cm in length, 8 cm in width and 3 cm in depth, excluding any retaining device.

- The weight of the Dosimeter shall not exceed 350 g, excluding any retaining device and batteries.

- The data shall be expressed in SI system units Syverts (Sv) and Syverts per hour (Sv/h) and their orders of magnitude (nano, mikro, mili etc.).

- The IP rating of the Dosimeter for ingress by dust and liquid according to EN 60529 or equivalent standard shall be at least IP65.

- The Dosimeter shall be suitable for use in the temperature range at least from -10°C to +40°C.

- The |Dosimeter shall operate on a standard types of batteries (AA, AAA, CR type or equivalent) or shall have a rechargeable battery or equivalent power supply.

- The Dosimeter shall signal about the battery discharge and there shall be a possibility to switch the signal off, or shall display the battery charge level.

- The Dosimeter shall perform measurements at least 100 hours without changing the batteries or with one charge of the batteries.

- The Dosimeter shall store the accumulated gamma, neutron or gamma and neutron dose measurement data regardless of whether power supply/batteries are installed.

- The Dosimeter shall have an internal data logger where not less than 500 data points of gamma and neutron dose shall be stored.

- If the Dosimeter is that of the direct reading, with additional or separate reader, for each 100 Dosimeters one reader shall be provided.

- If the Dosimeter is that of the direct reading, and connectable to the computer for setting operating parameters, reading data, for each 50 Dosimeters there shall be one portable computer provided with all software installed and a function to transfer data to a computer.

- The range of gamma photons energies measured by the Dosimeter shall be not less than from 80 keV to 1,5 MeV.

- The range of dose rate for gamma radiation measured by the Dosimeter shall be not less than from 0,5 µSv/h to 1 Sv/h.

- The range of dose for gamma radiation measured by the Dosimeter shall be not less than from 100 µSv to 9,9 Sv.

- The Dosimeter shall perform automatic start-up self-tests and indicate errors found.

- The Dosimeter shall be hardened against the electromagnetic radiation according the requirements of EN 61000-6-3 ir EN 61000-6-2 (or equivalent) standards.

- The color of the external surfaces of the Dosimeter and all the accessories (except display and connector sockets) shall be painted with either black (RAL 9005 or equivalent), or dark grey (RAL 7013, 7016, 7021, 7022, 7026 or equivalent), or dark green (RAL 6007 „Rifle green“ or equivalent), or dark blue (RAL 5008 or equivalent) mat paint.

- The Dosimeter shall be new and unused.

- The Dosimeter shall have a warranty period of at least 24 months. The warranty period is calculated from the date of signing the deed of acceptance of the buyer and the seller. The seller shall commit to eliminate the defects during the warranty period within a maximum period of 30 calendar days or to replace the defective components with functional ones. After warranty repairs, the Dosimeter shall be metrologically inspected by an accredited company. During the warranty period, the supplier shall carry out the repair at his own expense, including transportation costs.

- The manufacturer or supplier shall organize at least 8 hours of training concerning the use of the Dosimeter in Lithuania no later than within three weeks from the delivery of the goods, as well as to issue training certificates to the participants of the training (up to 20 participants).

- The Dosimeter and individual parts (spare parts) of the Dosimeter, including software, shall be manufactured in a European Union, European Free Trade Association, Organization for Security and Co-operation in Europe or NATO country and meet the European Union's requirements for safety, health and the environment. The Dosimeter shall have the Certificate of Origin

- The Dosimeter shall have the CE marking (a copy manufacturer’s issued certificate or declaration, or equivalent document shall be provided together with the documents for the tender).

- The Dosimeter shall not be a unitary (experimental) model.

- A certificate of metrological verification shall be provided together with the product.

__________________________