Appendix

to the Annex 8 „Proposal form“

(LOT 2)

TABLE OF CONFORMITY OF THE PROPOSED PROTECTIVE IMPERMEABLE DECONTAMINATION CLOTHING TO THE TECHNICAL SPECIFICATION

|  | Requirements | Compliance with requirements (to be filled in by the Supplier, indicating the specific meaning (if requested) or whether the offered product complies or does not comply with a specific mandatory requirement) | Document confirming compliance (to be filled in by the Supplier, indicating the names of the documents and/or links to their points proving the compliance of the offered goods with the technical requirements)* |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| 1. | Protective impermeable decontamination clothing (hereinafter – decontamination clothing) is a set of multiple-use decontamination clothing designed to protect the body against chemical warfare agents, biological agents and radioactive particles (hereinafter – hazardous substances), when conducting decontamination tasks with high-pressure decontamination systems. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 2. | The decontamination clothing shall be manufactured (only one of the 3 options should be offered): |  |  |
| 2.1. | either as a one-piece overall whose trousers, a jacket with an integral hood and protective boots are integrated and combined; | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 2.2. | or as a partially one-piece overall whose trousers and a jacket with an integral hood are integrated and combined, and the protective boots are added separately; | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 2.3. | or as a costume consisting of 3 separate parts: a blouse with an integral hood, trousers, protective boots. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 3. | The decontamination clothing shall be made of material, which shall be suitable for multiple use and shall be resistant to spraying of high-pressure liquids. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 4. | It shall be possible to decontaminate the decontamination clothing with liquid decontaminants. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 5. | The decontamination clothing shall be manufactured in at least 3 different sizes from 170 cm to 190 cm. | To specify exact sizes of the decontamination clothing from 170 cm to 190 cm: __ __ __ | Link to the source of information |
| 6. | Seams of the decontamination clothing (if any) shall be glued or covered by other means to protect against external impacts. | YES/NO | Link to the source of information |
| 7. | The construction of the decontamination clothing shall be such as to prevent the penetration of hazardous substances through its zippers, seams, flaps and closures. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 8. | The decontamination clothing shall bear labels on the inside, indicating at least the following information: product size, date of manufacture, maintenance instructions (standard and understandable symbols may be used). | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 9. | The hood of the decontamination clothing shall: |  |  |
| 9.1. | be sealed elastically on the frame of the gas-mask and so that no hazardous substances can penetrate through the connection. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 9.2. | be suitable for use with different models of gas masks (Avon Protection FM-50, 3M Scott Safety M-95); | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 9.3. | have an elastic grommet, which shall fit on the frame of the gas mask without gaps and wrinkles and would adhere to the rubber of the gas mask; | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 9.4. | have a hook and loop (velcro) fastener (for adjustment) in the chin area, which shall allow the hood to be fixed on the frame of the gas mask together with the rubber grommet. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 10. | The design of the cuffs of the decontamination clothing jacket shall ensure tightness and protection to prevent penetration of hazardous substances between the protective gloves and the cuffs of the jacket. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 11. | The design of trousers and a jacket of the decontamination clothing (if presented as separate parts) shall ensure tightness and protection to prevent penetration of hazardous substances between the trousers and a jacket. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 12. | The design of trousers and boots of the decontamination clothing (if presented as separate parts) shall ensure tightness and protection to prevent penetration of hazardous substances between the trousers and the boots. If additional multiple-use protective socks are required to ensure tightness, they shall be included in the set. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 13. | The material of the decontamination clothing shall be tested with HD chemical warfare agent droplets and provide protection against the penetration of droplets of HD for at least 24 hours. | To specify duration of the provided protection against the penetration of droplets of HD chemical warfare agent droplets of the material of the decontamination clothing __ h. | Protocols issued by acredited testing laboratory not earlier than 3 years before the date of the submition of the proposal confirming duration of the provided protection against the penetration of droplets of HD chemical warfare agent droplets of the material of the decontamination clothing |
| 14. | The tensile strength of the decontamination clothing material (except protective boots) shall not be less than 700 N according to ISO 1421 or equivalent standard. | To specify exact tensile strength of the decontamination clothing material according to ISO 1421 or equivalent standard ___ N | Protocols issued by acredited testing laboratory not earlier than 3 years before the date of the submition of the proposal confirming exact tensile strength of the decontamination clothing material according to ISO 1421 or equivalent standard |
| 15. | The boots of the decontamination clothing shall be: |  |  |
| 15.1. | black (RAL 6007 or equivalent), brown grey (RAL 7013 or equivalent), or bottle green (RAL 6007 or equivalent) colour (outer side); | To specify colour of the boots of the decontamination clothing (outer side) RAL ____ | Link to the source of information |
| 15.2. | tested with HD chemical warfare agents drops and ensure at least 8-hour protection from HD material penetration; | To specify exact duration of the provided protection of the boots of the decontamination clothing against the penetration of HD chemical warfare agents drops __ h. | Protocols issued by acredited testing laboratory not earlier than 3 years before the date of the submition of the proposal confirming duration of the provided protection of the boots of the decontamination clothing against the penetration of HD chemical warfare agents drops |
| 15.3. | resistant to flexing, i. e. it shall be no cracking before 125000 flex cycles in accordance with EN ISO 20344 or equivalent standard; | To specify exact quantity of flex cycles of the boots of the decontamination clothing in accordance with EN ISO 20344 or equivalent standard ______ flex cycles | Protocols issued by acredited testing laboratory not earlier than 3 years before the date of the submition of the proposal confirming exact quantity of flex cycles of the boots of the decontamination clothing in accordance with EN ISO 20344 or equivalent standard |
| 15.4. | produced in at least 5 different sizes with an interval from 270 mm to 310 mm in accordance with the length of the foot (insole); | To specify exact sizes of the boots of the decontamination clothing in accordance with the length of the foot (insole) from 270 mm to 310 mm: __ __ __ __ __ | Link to the source of information |
| 15.5. | with reinforced (thicker) soles. The sole shall be slip-resistant, rugged and have a large pattern. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 16. | The decontamination clothing and the protective boots shall be suitable for use at temperatures from 0℃ till +40℃. | To specify exact diapason of temperature the decontamination clothing and the protective boots are suitable for use: from -0°C to +__°C. | Link to the source of information |
| 17. | The weight of the maximum size of the decontamination clothing without boots shall not exceed 8 kg. | To specify exact weight of the maximum size of the decontamination clothing without boots ___ kg. | Link to the source of information |
| 18. | The outer side of the decontamination clothing shall be of brown grey (RAL 7013 or equivalent), or bottle green (RAL 6007 or equivalent), or black (RAL 9005 or equivalent) colour. | To specify colour of the outer side of the decontamination clothing RAL ____ | Link to the source of information |
| 19. | A multi-use means of transport (storage, safekeeping) shall be provided together with the decontamination clothing, made of a durable material and suitable for outdoor use (e.g. a case, plastic or metal box, bag or equivalent means of transport that cannot be made of paper or cardboard). | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 20. | A multi-use means of transport shall be of brown grey (RAL 7013 or equivalent), or bottle green (RAL 6007 or equivalent), or black (RAL 9005 or equivalent) colour. | To specify colour of a multi-use means of transport RAL ____ | Link to the source of information |
| 21. | The decontamination clothing shall be accompanied by the manufacturer’s recommendations (in Lithuanian and English) on daily and periodic maintenance, decontamination methods and decontaminants to be used after contamination with hazardous substances. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 22. | The decontamination clothing shall have a shelf life period of at least 10 years, under which the protective characteristics specified in this Technical Specification shall be ensured and the protective properties shall be maintained intact. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 23. | The decontamination clothing shall have a warranty period of at least 2 years. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 24. | During the 10-year period, the decontamination clothing shall retain the protective characteristics specified in this Technical Specification without additional conditions and without additional maintenance or service. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 25. | The warranty period shall be calculated from the date of signature of the acceptance and delivery act between the Buyer and the Seller. The Seller shall undertake to eliminate the deficiencies that have arisen during the warranty period within a period not exceeding 30 working days. During the warranty period, the supplier shall carry out repairs at its own expense, including transportation costs. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 26. | The manufacturer or supplier shall organise training in Lithuania on how to use the decontamination clothing no later than four weeks after delivery of the goods, as well as to issue training certificates to the participants of the training (up to 20 participants). | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |

*Notes:

1) Important: The Supplier, when indicating the specific meaning of the goods or their conformity to it, must indicate to the contracting authority which document submitted by the Supplier contains the data confirming/proving the requirement (indicate the specific folder/file in which the document is submitted, the name of the document and the page);

2) each point of the technical requirements that requests a specific value must be based on the manufacturer's technical documentation;

3) each point of the technical requirements requiring manufacturer's approval, if the offer is submitted by the manufacturer of the goods himself, this is confirmed in column 3 and no supporting documents need to be submitted, and the corresponding column in column 4 is not filled in. If the offer is not submitted by the manufacturer of the goods, the supplier must:

a) fill in the corresponding column in column 4;

b) submit a declaration signed by the manufacturer together with the offer confirming compliance with the specific requirement;

4) "or equivalent" is written next to all specified specific materials and/or specific product names. A supplier offering a product with equivalent properties must, at the time of submitting the offer, prove by reliable means that the offered product is equivalent and fully complies with the requirements set out in the technical specification.