Appendix

to the Annex 8 „Proposal form“

(LOT 3)

TABLE OF CONFORMITY OF THE PROPOSED DIGITAL DOSIMETER TO THE TECHNICAL SPECIFICATION

|  | Requirements | Compliance with requirements (to be filled in by the Supplier, indicating the specific meaning (if requested) or whether the offered product complies or does not comply with a specific mandatory requirement) | Document confirming compliance (to be filled in by the Supplier, indicating the names of the documents and/or links to their points proving the compliance of the offered goods with the technical requirements)* |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| 1. | Digital dosimeter (hereinafter abbreviated as a – Dosimeter) shall measure individual dose equivalent Hp(10). | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 2. | The detectors of the Dosimeter shall measure gamma and register neutron radiation. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 3. | All the detectors of the Dosimeter shall be integrated into the Dosimeter. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 4. | The Dosimeter shall directly indicate dose rate and accumulated dose on a display. | YES/NO | Link to the source of information |
| 5. | The Dosimeter shall have a function of a switching the display backlight on and off. | YES/NO | Link to the source of information |
| 6. | The Dosimeter shall be compatible to set one threshold value for dose rate and one threshold value for dose above which an audible and (or) visual alarm would be activated. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 7. | The Dosimeter shall provide an audible and (or) light alarm if the dose rate or dose thresholds are exceeded in order to avoid ineligible exposure. | YES/NO | Link to the source of information |
| 8. | The Dosimeter shall have a possibility to switch off the audible and (or) light alarm indicating that the dose rate or dose thresholds are exceeded. | YES/NO | Link to the source of information |
| 9. | The Dosimeter shall have a retaining device(s) in order to hang the Dosimeter on the neck or slip it to the clothing. | YES/NO | Link to the source of information |
| 10. | The dimensions of the Dosimeter shall not exceed 15 cm in length, 8 cm in width and 3 cm in depth, excluding any retaining device. | To specyfy exact dimensions of the Dosimeter, excluding any retaining device: length __ cm; width _ cm; depth _ cm. | Link to the source of information |
| 11. | The weight of the Dosimeter shall not exceed 350 g, excluding any retaining device and batteries. | To specify exact weight of the Dosimeter, excluding any retaining device and batteries ___ g. | Link to the source of information |
| 12. | The data shall be expressed in SI system units Syverts (Sv) and Syverts per hour (Sv/h) and their orders of magnitude (nano, mikro, mili etc.). | YES/NO | Link to the source of information |
| 13. | The IP rating of the Dosimeter for ingress by dust and liquid according to EN 60529 or equivalent standard shall be at least IP65. | To specify IP rating of the Dosimeter for ingress by dust and liquid according to EN 60529 or equivalent standard IP__ | Link to the source of information |
| 14. | The Dosimeter shall be suitable for use in the temperature range at least from -10°C to +40°C. | To specify exact diapozone of temperature the Dosimeter is suitable for use: from -__°C to +__°C. | Link to the source of information |
| 15. | The ¦Dosimeter shall operate on a standard types of batteries (AA, AAA, CR type or equivalent) or shall have a rechargeable battery or equivalent power supply. | YES/NO | Link to the source of information |
| 16. | The Dosimeter shall signal about the battery discharge and there shall be a possibility to switch the signal off, or shall display the battery charge level. | YES/NO | Link to the source of information |
| 17. | The Dosimeter shall perform measurements at least 100 hours without changing the batteries or with one charge of the batteries. | To specify exact duration of usage without changing the batteries __ h. | Link to the source of information |
| 18. | The Dosimeter shall store the accumulated gamma, neutron or gamma and neutron dose measurement data regardless of whether power supply/batteries are installed. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 19. | The Dosimeter shall have an internal data logger where not less than 500 data points of gamma and neutron dose shall be stored. | To specify exact internal data points of an internal data logger of gama and neutron dose storage __ data points | Link to the source of information |
| 20. | If the Dosimeter is that of the direct reading, with additional or separate reader, for each 100 Dosimeters one reader shall be provided. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 21. | If the Dosimeter is that of the direct reading, and connectable to the computer for setting operating parameters, reading data, for each 50 Dosimeters there shall be one portable computer provided with all software installed and a function to transfer data to a computer. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 22. | The range of gamma photons energies measured by the Dosimeter shall be not less than from 80 keV to 1,5 MeV. | To specify the range of dose rate for gamma photons energies: from __ keV to __ MeV. | Link to the source of information |
| 23. | The range of dose rate for gamma radiation measured by the Dosimeter shall be not less than from 0,5 µSv/h to 1 Sv/h. | To specify the range of dose rate for gama radiation: from __ µSv/h to __ Sv/h. | Link to the source of information |
| 24. | The range of dose for gamma radiation measured by the Dosimeter shall be not less than from 100 µSv to 9,9 Sv. | To specify the range of dose for gamma radiation: from __ µSv to __ Sv. | Link to the source of information |
| 25. | The Dosimeter shall perform automatic start-up self-tests and indicate errors found. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 26. | The Dosimeter shall be hardened against the electromagnetic radiation according the requirements of EN 61000-6-3 ir EN 61000-6-2 (or equivalent) standards. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 27. | The color of the external surfaces of the Dosimeter and all the accessories (except display and connector sockets) shall be painted with either black (RAL 9005 or equivalent), or dark grey (RAL 7013, 7016, 7021, 7022, 7026 or equivalent), or dark green (RAL 6007 „Rifle green“ or equivalent), or dark blue (RAL 5008 or equivalent) mat paint. | To specify the color of the external surfaces of the Dosimeter and all the accessories RAL ____ | Link to the source of information |
| 28. | The Dosimeter shall be new and unused. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 29. | The Dosimeter shall have a warranty period of at least 24 months. The warranty period is calculated from the date of signing the deed of acceptance of the buyer and the seller. The seller shall commit to eliminate the defects during the warranty period within a maximum period of 30 calendar days or to replace the defective components with functional ones. After warranty repairs, the Dosimeter shall be metrologically inspected by an accredited company. During the warranty period, the supplier shall carry out the repair at his own expense, including transportation costs. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 30. | The manufacturer or supplier shall organize at least 8 hours of training concerning the use of the Dosimeter in Lithuania no later than within three weeks from the delivery of the goods, as well as to issue training certificates to the participants of the training (up to 20 participants). | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 31. | The Dosimeter and individual parts (spare parts) of the Dosimeter, including software, shall be manufactured in a European Union, European Free Trade Association, Organization for Security and Co-operation in Europe or NATO country and meet the European Union's requirements for safety, health and the environment. The Dosimeter shall have the Certificate of Origin | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 32. | The Dosimeter shall have the CE marking (a copy manufacturer’s issued certificate or declaration, or equivalent document shall be provided together with the documents for the tender). | YES/NO | Copy of CE Certificate |
| 33. | The Dosimeter shall not be a unitary (experimental) model. | Manufacturer‘s confirmation DO CONFIRM/DO NOT CONFIRM | Not to be completed when the offer is submitted by the manufacturer of the goods. If the offer is submitted by a supplier who is not the manufacturer of the goods, he must submit a declaration from the manufacturer of the goods regarding compliance with the requirements of this point. |
| 34. | A certificate of metrological verification shall be provided together with the product. | YES/NO |  |

*Notes:

1) Important: The Supplier, when indicating the specific meaning of the goods or their conformity to it, must indicate to the contracting authority which document submitted by the Supplier contains the data confirming/proving the requirement (indicate the specific folder/file in which the document is submitted, the name of the document and the page);

2) each point of the technical requirements that requests a specific value must be based on the manufacturer's technical documentation;

3) each point of the technical requirements requiring manufacturer's approval, if the offer is submitted by the manufacturer of the goods himself, this is confirmed in column 3 and no supporting documents need to be submitted, and the corresponding column in column 4 is not filled in. If the offer is not submitted by the manufacturer of the goods, the supplier must:

a) fill in the corresponding column in column 4;

b) submit a declaration signed by the manufacturer together with the offer confirming compliance with the specific requirement;

4) "or equivalent" is written next to all specified specific materials and/or specific product names. A supplier offering a product with equivalent properties must, at the time of submitting the offer, prove by reliable means that the offered product is equivalent and fully complies with the requirements set out in the technical specification.