Technical specification

Combat Management System for LNS SĖLIS P15

TABLE OF CONTENTS

1.	General requirements	3

2.	CMS main components and their general requirements	5

3.	Electro-optical Fire Control System requirements	7

4.	Remotely controlled Search and Rescue light requirements	9

5.	X-band navigational radar sensor requirements	10

6.	Inertial Navigational System requirements	11

7.	Wind and Weather System requirements	11

8.	CMS display control and settings requirements	12

9.	CMS integration with main gun requirements	14

10.	CMS general integration requirements	15

11.	Technical support and documentation requirements	15

# General requirements

  - All works and equipment installation stated in this specification to be carried out on board Lithuanian Navy ship “Sėlis“, hull number P15 (Flyvefisken class patrol ship).

  - The Contracting Authority does not allow suppliers (legal entities) / subcontractors (legal entities) which are from the Russian Federation, the Republic of Belarus, territories of Ukraine (annexed Crimea and other territories not controlled by the Government of Ukraine), the Transnistrian territory not controlled by the Government of the Republic of Moldova, territories of Abkhazia and South Ossetia not controlled by the Sakartvel Government, and the People's Republic of China (excluding the separate customs territory of Taiwan, including Penghu, Kinmen, and Matsu).

  - The vendor must ensure that the main parts of the offered Goods and/or assemblies will not be manufactured in the states or territories listed in paragraph 1.2.

  - If the third parties are involved in the project as subcontractors, same requirements apply to them as in paragraph 1.2. All issues concerning quality of the work done and with it related possible impact on CMS operation is at the vendor’s responsibility.

  - Vendor must commit to not provide any information for third parties without customer agreement about system specific settings and technical drawings.

  - The development phase (Development Phase) of the Tactical Tasks Management System (TUVS), including both software and hardware, must be completed.

  - The TUVS must be qualified for safe use for its intended purpose.

  - The TUVS and its components must be newly manufactured (not used or refurbished).

  - The TUVS and its components must comply with MIL-S-901D standard for Grade B Class II equipment, MIL-STD-167-1 standard for Type I equipment (MIL-STD-167-1 Mechanical Vibrations of Shipboard Equipment (Type I – Environmental, and Type II – Internally Excited)).

  - The TUVS Electromagnetic Compatibility must comply with the requirements of the MIL-STD-461F standard for surface ships.

  - TUVS equipment and components installed outside the ship must reliably operate under heavy rain or fog, withstand wind speeds up to 30 m/s, and operate within M1-M3 environmental temperature ranges from –20 ºC to +45 ºC.

  - All TUVS equipment and components installed outside the ship must be certified to at least IP56 class according to IEC standard 60529.

  - Equipment installed inside the ship must operate within a temperature range from +5 ºC to +40 ºC.

  - This equipment must be certified to at least IP33 class according to IEC standard 60529.

  - TUVS equipment and components installed both inside and outside the ship must operate flawlessly within a temperature range from –25 ºC to +45 ºC.

  - The storage temperature range of TUVS indoor and outdoor equipment must be from –34 ºC to +48 ºC according to the manufacturer’s recommended storage conditions for TUVS equipment and components.

  - The Manufacturer must provide the Buyer with the storage requirements of TUVS.

  - The Manufacturer must ensure continuous operation of TUVS and its related equipment and components when connected to the ship’s power supply system: 115 V, 440 V, 60 Hz (AC) and 24 V (DC).

  - The TUVS must be equipped with an independent Uninterruptible Power Supply system (hereinafter – UPS), which must ensure continuous operation of the system at full capacity (except the operation of the main gun) for not less than 5 minutes in case of ship power supply failure.

  - The UPS must provide the additional time required for the system, after the period indicated in paragraph 1.19, to complete a fully automatic safe shutdown in case of ship power supply failure.

  - The Manufacturer is responsible for providing a technical solution based on the possible options for UPS installation locations on the ship:

    - The UPS system must be installed in ship compartment No. 1D-17.

    - The UPS system must be installed in the consoles located on the ship’s bridge.

    - Another UPS installation location is possible, but it must be agreed with the Customer in advance.

  - The Manufacturer is responsible for the installation, connection, and functional testing of the UPS equipment.

  - TUVS equipment and components (except the TUVS Multi-functional Console), intended for installation inside the ship, must be installed in ship compartment No. 1D-17.

  - The TUVS Multi-functional Console must be installed on the ship’s bridge.

  - All TUVS connection cables and wires must be properly marked and routed through special cable routes.

  - After completing the installation works, the Manufacturer must ensure that the integrity and watertightness of the ship’s citadel is not compromised.

  - All TUVS equipment and components must not cause negative interference to the operation of other ship-installed systems, mechanisms, radio, or navigational equipment.

  - TUVS equipment and components installed outside the ship must be painted grey color RAL 7001 “Silver Grey” or RAL 7042 “Traffic Grey A”.

  - Mission Critical TUVS equipment and components installed outside the ship must have a de-icing function or another technical solution ensuring safe and full operational capability under M1-M3 environmental conditions.

  - The Lithuanian version of this technical specification shall prevail over the English one if any discrepancies or interpretation issues arise between them.

# CMS main components and their general requirements

  - The Manufacturer must provide the proposed TUVS Work Breakdown Structure, indicating the TUVS elements as well as the subsystems and components within the TUVS.

  - The CMS must consist at least of:

    - One Multi-functional Console (MFC);

    - Electro-optical Fire Control System (EOFCS);

    - Remotely controlled Search and Rescue light (SAR light);

    - X-band navigational radar sensor (XRAD);

    - Inertial Navigation system (INS);

    - Wind and Weather System (WX SYS);

    - Fire control and mission management processing cabinet (FCMMC)

    - Operator Control Panel (OPC)

  - The CMS must have one control and operation (MMI – man machine interface) working station which has to be arranged on the ship’s bridge.

  - Control and operation working station must include new shock resistant MFC with main display (it could be more than one display if vendor meets other requirements), rail mounted shock resistant chair and operation and control panel.

  - The MFC has to meet following requirements:

    - MFC must be mounted on the left side of existing 3D radar console. Fixing MFC to the bridge floor is at the vendor cost.

    - MFC must be not more than 135 cm and not less than 65 cm in width, not more than 135 cm in depth and not higher than 172cm (Console must be ergonomic and suitable for long working hours of operator).

    - MFC must be fixed on the ship’s bridge lower floor and mounted on the shock and vibration absorbing plate or dampers.

    - MFC must have rack for the main display (display must meet requirements for monitor display stated in this specification).

    - Rack for main display (displays) must be arranged so that operator would feel comfortable during the usage of MFC.

    - MFC must include rack for track ball, keyboard, joystick and active keyboard (all these items must have at least level 3 liquid ingress protection in accordance with IEC standard 60529, integrated in the console and must be easy accessible for operator from his sitting place).

    - MFC color must be agreed with the customer.

  - The main display must meet below stated requirements:

    - Resolution of monitor must be not less than 1920x1080;

    - Brightness must be not less than 350 cd/m² and sunlight readable;

    - Viewing angle (V/H) must be not less than 170º/160º;

    - Backlight life must be not less than 45000 hours;

    - Contrast ratio must be not less than 900:1;

    - Display size must be not less than 22’ and specially designed for heavy duty use on board of the ship;

    - All displays (display) must be rack mounted;

    - Front screen of the main display (displays) must have at least level 3 liquid ingress protection in accordance with IEC standard 60529;

    - Display must have at least following display options easy accessible for operator for adjustment::

      - Brightness;

      - Contrast;

      - Dimming function;

      - Resolution.

  - Operator control panel must have at least following accessories:

    - A track ball and/or joystick must be provided, allowing the operator to select various options on the screen or to obtain information about contacts displayed on the screen.

    - The component specified in paragraph 2.7.1 must be ergonomic, easy to use, and fixed to the panel.

    - The joystick and/or track ball must be within the operator’s arm’s reach from the designated working position in a seated posture.

    - The OCP keyboard must be a full-size ANSI or ISO standard keyboard with numeric keys (Full-Size) and a “QWERTY” layout, intended for additional data input on the MFC screen.

    - The keyboard must have an integrated key backlighting system with brightness adjustment functionality.

    - The OCP must be equipped with Active Quick-Access Keys (these keys must be arranged as an interactive application on the touchscreen or on a separate panel):

        - Hook track;

        - Deselect track;

        - At least 6 keys for custom settings (the operator may assign an additional pre-programmable quick function to each key).

    - The Active Keys must have a light dimming option.

    - All Operator Control Panel (OCP) elements must have at least level 3 liquid ingress protection in accordance with IEC standard 60529.

  - Rail mounted shock resistant sitting chair has to meet following requirements:

    - Chair base have to be mounted on existing rail system (Vejvad Hansen track system) on the Flyvefisken class ship’s bridge (rail width 36,2 mm) with ability to be fixed at certain positions.

    - Chair must meet or exceed the current P15’s bridge chairs functionality (Current chair rail model Type R 205; current chair model KT106-V).

    - Chair soft part must to be covered by leather or other material suitable for intensive use on board of the ship.

    - Chair external material and other visible items must be in black color (other colors can apply but have to be agreed with the customer).

    - Chair must have shock absorbers (dampers) to reduce explosion impact for operator.

# Electro-optical Fire Control System requirements

  - The EOFCS must be designed to provide the operator with the view of the target and surrounding area during both daytime and nighttime, while the ship is performing tasks under M1–M3 environmental conditions.

  - The EOFCS must be capable of operating in conditions of poor visibility and determining the distance to the target/object.

  - The EOFCS must ensure:

    - Assistance during visual search and rescue operations;

    - Search, tracking, and identification of surface and air targets/objects;

    - Accurate synchronization of the target tracking system with the ship’s main gun control system.

  - The EOFCS must be installed on the top of the bridge, on the starboard side of the ship.

  - EOFCS must have the at least the following components:

    - TV color camera.

      - Requirements for EOFCS TV camera:

        - Transfer of target and surrounding view to the operator with continuous optic zooming of at least 20x.

        - Resolution at least 1920x1080.

        - View transferred by the camera must be in color with a possibility to switch to a black/white regime.

        - Range of an overview angle at least from 2° to 50° (with zooming).

        - Sensor gain must have automatic and manual modes.

        - Sensitivity at least 0.05 lux.

    - IR camera.

      - Requirements for EOFCS IR camera:

        - Reliability of the IR camera must have at least 8000 working hours mean time between failures.

        - Camera sensitivity at least 15 mK but not more than 30 mK.

        - Resolution at least 640x480 pixels.

        - Range of an overview angle at least from 2° to 30° (with continues zooming).

        - STANAG 4347 Detection of 2.3x2.3m target at least 16 km, recognition at least 7 km, identification at least 4 km

    - Eye safe Laser range finder.

      - Requirements for EOFCS Laser range finder:

        - Laser range finder must be able to measure far distances not less than 20000 meters.

        - Laser range finder must be able to measure close distances from 100 meters.

        - Metering precision must be not worse than +/- 5 meters.

        - Operator must be able to use single shot for range finding of tracked target.

  - General requirements for EOFCS:

    - Outdoor installed EOFCS equipment weight must be not more than 200 kg.

    - Type of EOFCS director platform to be 2-axis – elevation over azimuth, with the ability to turn around continuously 360° in a horizontal plane and at least -20° to 80° in a vertical plane.

    - Slewing rate at least 120°/sec.

    - Stabilization accuracy at least 0.05 mRad.

# Remotely controlled Search and Rescue light requirements

  - Remotely controlled SAR light must be installed in the place of the current P15 search light, on the top of the bridge, centerline of the ship.

  - Disassembly works of current search light is on vendor’s cost.

  - Basic requirements for the SAR light body and housing:

    - Dimensions (WxDxH): less than 1200x800x1000 mm;

    - Total turret weight with internal components cannot exceed 100 kg;

    - Unlimited horizontal and vertical movement;

    - The SAR light design must ensure flawless functionality under M1–M3 environmental conditions in accordance with STANAG 4370 AECTP-230 (Edition 1), including information sheets 2311/1, 2311/2, and 2311/3;

    - Splash resistant against seawater;

  - Light payloads:

    - White search light with the parameters:

      - Multi LED or HMI type;

      - Average life not less than 1000 hours;

      - If HMI type bulb used – at least 3 spare light bulbs included in the set;

      - Range parameter: at least 1 Lux at 3000 meters;

    - Separate LED light for Morse code signaling;

      - Average life time: at least 25000 hours;

      - At least one spare replacement set to be included;

      - Morse code signaling can be activated manually through dedicated button or automatically with text input via dedicated panel or MFC.

  - Remotely controlled SAR light can be controlled from dedicated panel installed in the front of the bridge or through MFC.

# X-band navigational radar sensor requirements

  - Radar sensor must be suitable to be used as primary navigation aid in narrow channels and shallow waters, ports and in open seas.

    - Vendor is responsible to replace current Furuno X-band radar, monitor and control panel on board P15.

    - Any required bridge panel adjustments for the installation of new radar sensor is on Vendor’s cost.

  - Radar sensor must have Automatic radar plotting aid (ARPA) and compliant with the provisions of the standard IEC 62388.

  - Mean time between failures not less than 50000 hours.

  - Transmitter type: Solid state.

  - Tolerable relative wind speed for antenna unit during operation: not less than 80 knots.

  - Radar sensor antenna length must be between 6-7.5ft.

  - Horizontal beam width less than 1.2°.

  - Vertical beam width at least 20°.

  - Radar sensor must be able to send and display surface tracks on MFC.

  - Radar will be used on board of the patrol ship and must be able operate 24/7.

  - Radar system must be capable to track not less than 100 surface tracks (Tracking means that system has whole information about the track) simultaneously.

  - Radar system must be integrated with W-AIS and be able to display AIS tracks with different symbol to distinguish them from the ARPA tracks.

  - Radar instrumented range for surface tracks must be not less than 40 nm.

  - Radar minimum detection range less than 50 meters.

  - Radar system must have possibility to create at least one silent sector (not to transmit electromagnetic waves in selected direction). Width of the sector must be without any restrictions.

  - Radar system must have preinstalled world map containing coastlines.

  - Radar software system must have possibility to load and display IHO S-52, S-57 and S-63 format charts.

  - There must be solution to switch off electromagnetic radiation from radar antenna at operator’s sitting place immediately (up to 2 s).

  - Radar sensor system must have a spare port with dedicated cable for surface track data output into National RMP management system.

# Inertial Navigational System requirements

  - INS must be based on either Fiber Optic Gyroscope technology, Ring Laser Gyroscope technology or Hemispherical Resonator Gyroscope technology.

  - INS must be dedicated for the CMS and installed next to the current gyroscope of P15 in the navigation equipment cabinet. The leveling planes must be aligned with the current gyroscope of P15.

  - Vendor must integrate P15 current Gyro compass (Polaris FOG 50) as a secondary heading/pitch/roll data input as backup.

  - Performance of the INS must be:

    - Mean time between failures: >100000 hours;

    - Setting time (static): <15 minutes;

    - Setting time (dynamic): <30 minutes;

    - Heading Accuracy with no GPS aiding: <0.1° Sec (Lat) RMS

    - Roll/Pitch Accuracy: <0.01° RMS

    - Position Accuracy with GNSS signal available: <3 meters CEP

    - Position Accuracy with LOG aiding: ≤ 1 NM / 12h CEP

    - Position Accuracy full inertial: ≤ 2 NM / 12h CEP, with ability to manually input updated position data.

  - Resistance to vibration at least to the standard of IEC-60945 or equivalent.

# Wind and Weather System requirements

  - Wind and Weather System must be dedicated for the CMS, and all outside sensors mounted on the mast.

  - WX system must be able to provide these parameters:

    - Wind speed (true and relative);

    - Wind direction (true and relative);

    - Air temperature;

    - Humidity;

    - Barometric pressure;

  - Outputs from the WX system must be displayed on separate weather display and also on MFC.

  - Parameters from the WX system must automatically used by CMS for ballistic calculations, but CMS would also have the possibility for manual adjustment of these parameters.

# CMS display control and settings requirements

  - CMS must have option to initiate tracks automatically.

  - Air and surface tracks must be displayed differently.

  - CMS software must have possibility to change time zone (+/- 12h).

  - CMS software must have possibility to record and store at least 12 hours of video with all information showed on the screen in high video resolution format.

  - CMS software must have possibility to make screen shots and save them in at least one of the following formats: tiff, bmp and jpeg.

  - Operator must have possibility to store CMS video record in at least one of the following video formats: avi, avchd, mov, flv.

  - Operator must have possibility to extract all recorded videos or screenshots via compact disc or via USB port.

  - CMS software must have option to present layered geographical preloaded application (maps, charts, etc.) simultaneously with track data.

  - CMS must have possibility to list tracks according to their type.

  - CMS operator must have possibility to determine and manually assign at least the following track identification (for each track): neutral, friend, unknown, suspect, hostile (graphical application preferred as stated in latest version of NATO publication AAP-6(C) (or later edition) “NATO joint military symbology” or MIL STD 2525B (or later edition) “Military Standard for Common Warfighting Symbology”).

  - CMS video display interface(software) must have at least following working tools for track management:

    - Measuring tool (for distance and bearing measurements from own position or from any other reference point).

    - Ability to set areas (polygons, circles, lines).

    - Ability to set reference point.

    - Zoom in/out function.

    - Centre PPI on own, selected track or any geographical point position.

    - Relative/true vector setting mode.

    - North up, head up mode.

    - Record and print screen button.

    - Joystick/mouse interactive symbol (usually cross) which helps to select functions on the screen.

  - All measurement units have to be as follow:

    - Distance in meters (kilometers) and nautical miles (cables).

    - Altitude in meters and in feet.

    - Bearing in degrees and mils (1/6400).

    - Speed in meters/second and in knots.

    - Coordinates in WGS84 (accuracy up to seconds) and in MGRS (accuracy up to 1 meter).

    - Time in hours (European 0-24 h, accuracy up to seconds).

  - CMS must have possibility graphically display following information about the track on the screen side:

    - Distance.

    - Bearing.

    - Position.

    - Speed.

    - Altitude (for air track).

    - Course.

    - CPA (Closest point of approach).

    - TCPA (Time to closest point of approach).

    - Track history.

    - Track number (must have possibility to be renamed manually with at least 4 figures).

    - AIS/WAIS information.

    - Comment text (manual input by operator at least 10 symbols).

  - The operator must have possibility to see (and switch off) information about the track on the screen.

  - The operator must have possibility to create tracks manually and input all know parameters.

  - CMS system software must have possibility to filter air tracks at least in three groups:

    - Air tracks up to 3000 meters in height.

    - Air tracks from 3000 to 10000 meters in height.

    - Air tracks more than 10000 meters in height.

  - CMS system software must have possibility to filter tracks on track list by following criteria:

    - ID criteria (neutral, friend, unknown, suspect, hostile).

    - Air, surface, land.

  - CMS must be able to create SAR patterns in accordance with IMO IAMSAR Vol.3.

  - CMS must be able to automatically fuse tracks with similar parameters from different sensors. Operator could also disable such function and select tracks manually, which should be fused/split.

# CMS integration with main gun requirements

  - CMS must be integrated with the main gun of P15: OTO Melara 76/62 100 RpM, Serial Nr. 058.

  - CMS must have a Fire control module designed for remote control of the main gun and performing of automatic ballistic firing calculations.

  - CMS must be able to carry out such main gun engagement modes:

    - Sensor controlled mode. Position of the target is given to the ballistic computer from EOFCS, 3D radar or surface radar.

    - Direct target coordinates mode (including land engagements with altitude inputs). Position of the target is inserted by the operator. Position can be given either as absolute geographic coordinates (Lat-Long or MGRS) or as polar coordinates with respect to the own ship (range-azimuth).

    - Indirect target coordinates mode. Position of the target is inserted by the operator through a suitable window, but it is given as the position in the polar coordinates with respect to another geographical reference (not own ship). In this mode corrections of firing must be able to be added by the operator in accordance the procedures of NATO publication ATP-4 (Allied Naval Gunfire Support).

    - Emergency mode for short-range engagements when laser data from EOFCS is not available. In this mode, gun barrel to be directly aligned with the center of TV/IR camera elevation and heading, operator to adjust firing by adjusting TV/IR view angles.

  - First three gun engagement modes must have the ability for the operator to input offset Aim.

  - MFC must have a dedicated pedal or other easily distinguished device for quick and safe opening fire with the gun.

  - The operator must be able to remotely set the speed of firing.

  - The operator must be able to set what type of ammunition is loaded into the gun, including remotely setting to burn or keep the fuse operational when using ammunition with proximity fuses.

  - A dedicated panel on the bridge with physical key must be activated by Ship Commander or Weapons Officer in order for the gun to fire.

  - The Vendor undertakes to provide the Customer, together with the system, for permanent use

    - A removable radar system for measuring the projectile muzzle velocity of the main gun barrel;

    - Equipment for calibrating the main gun barrel with the CMS and the EOFCS.

# CMS general integration requirements

  - CMS must be able to receive IEC 61162-1 (also known as NMEA 0183), IEC 61162-3 (also known as NMEA 2000), IEC 61162-450 standard signal sentences and use data from existing class ship equipment (GPS, DGPS, Gyro compass, echo sounder, AIS, W-AIS, W-ECDIS). In case converter is needed it is at the vendor’s cost.

  - CMS must be integrated with current 3D radar “Columbus MkII” to display air and surface tracks and also raw radar video data on MFC. CMS must be able to track/engage on selected tracks.

  - CMS must have integrated built in test equipment which helps operator to identify possible system failures.

# Technical support and documentation requirements

  - The vendor must deliver all technical documentation and operator manuals regarding all CMS related equipment and accessories in two paper copies (of each equipment) and one digital copy in English language.

  - The vendor must deliver CMS interface control documents (ICD).

  - Licenses of all copy right products used in the system must be payed and delivered to the customer (if it is software it should be delivered in CD/DVD).

  - The vendor must deliver scheduled maintenance plan of CMS with specialist qualification, special tools (if any) required and description of maintenance procedure. Maintenance plan must include MTTF (mean time to failure) for all sensitive components and estimated time required to conduct repair work.

  - Operator and first level maintainer training (up to 5 trainees) course must be included in the contract and provided to the customer. All training must be done in Lithuania, Klaipėda. Classroom with visual aids for presentations will be arranged by Lithuanian NAVY.  On the job training for operators and first level maintainers must be included as the final stage of education.

  - CMS manufacturer must declare during the tender that he is able to support product with spare parts delivery at least for 15 years. Technical support must be possible up to 25 years (counting from CMS acceptance date).

  - Vendor must grant at least 30 months warranty for the whole CMS and with it related equipment after installation work acceptance date.

  - Vendor must provide customer with recommended spare parts for the CMS to be stored on board for the mission of 30 days.

  - Vendor must provide customer with recommended spare parts for the CMS to be stored in base for 24 months post warranty.

  - Vendor must perform factory acceptance test with customer representatives, harbor and sea acceptance trials after CMS installation and integration, including live firing tests at sea. All required gun calibration to CMS components x-y-z planes on the ship, barrel calibration to EOFCS and muzzle velocity calibration equipment and dedicated personnel must be provided by vendor. All ship related expenses (crew, supply for crew, fuel and ammunition) are at the customer’s cost and all other expenses (vendor’s personnel, their food supply, travel arrangements and etc.) are at the vendor’s cost.

  - All equipment transportation, installation and material expenses (related with CMS or other equipment mentioned in this specification) are at the vendor’s cost.

  - All CMS installation work must be conducted in Lithuanian Naval Flotilla.

  - Vendor must deliver and install whole CMS system during 24 month period from the date that the contract comes into effect (The system acceptance date (must be accepted by customer) counts as the date when installation is completed).

__________________