INSTALLATION AND ASSEMBLY OF RADIO MONITORING STATIONS

TECHNICAL SPECIFICATION

- The supplier must specify the exact manufacturer (manufacturer‘s name) and exact model of the proposed Item in the tender and technical specifications.

- The tender must be accompanied by the manufacturer‘s or its authorized representative‘s specifications, technical characteristics and/or references to the website, and/or other documents confirming that the proposed Item meets the requirements specified in the Technical Specification for this purchase.

| No. | Characteristic name | Characteristic value, parameters | Enter the specific conformity parameters proposed for the Item. Write “Complies“ or “Yes“ only when the requirement is not expressed in physical amounts. | Document title, page number and/or exact reference to the website for justification of the Item conformity |
| --- | --- | --- | --- | --- |
| I | II | III | IV | V |
| Item name: purchase, installation, and assembly of radio monitoring station equipment |  |  | (The Seller shall indicate here the manufacturer, name, and quantity (quantities) of the Item. If the Seller and/or the manufacturer of the Item uses third-party and/or manufacturers‘ devices, hardware and/or software (hereinafter - the equipment (together with devices)) to manufacture the Item, this equipment shall be specified as a parameter of the Item conformity. |  |
|  | General technical requirements for the purchase of equipment |  |  |  |
|  | Purpose of the Item | The radio spectrum monitoring system is designed for monitoring, control, and analysis. The system consists of different types of spectrum monitoring stations, devices, and software that ensure the reception, measurement, analysis, and recording of radio signals across the entire specified frequency range. Centralized management and data processing software allows remote control of stations, collection and analysis of measurement data, detection, identification, and localization of radio sources and interference. The system is used for effective monitoring of radio spectrum resource utilization and detection of possible violations. |  |  |
|  | Comments indicating conformity | By indicating conformity with “Conforming“ or “Yes“ (English: “Conforming“ or “Yes“), indicating the physical quantity and/or measured parameters as conformity with the requirement set out in this specification, it is permitted to add a comment in Lithuanian and/or English, including an indication of the third-party equipment used. If “Partially conforming“ or similar is indicated, it is necessary to add an explanation and documentation that would allow conformity to be assessed (e.g., the parameter is not used in the Baltic region or in EU countries, so it cannot be measured in Lithuania, etc.). etc.). |  |  |
|  | Calibration | For a broadband spectrum receiver: accredited calibration. During the warranty period, the supplier must periodically calibrate the receiver at an accredited laboratory every 2 years. All periodic calibrations of the receiver during the warranty period must be included in the supplier‘s tender price. For the direction finder: Accredited calibration. During the warranty period, the supplier must periodically calibrate the polarimeter at an accredited laboratory every 2 years. All periodic calibrations of the direction finder during the warranty period must be included in the supplier‘s tender price. The supplier shall remove the direction finder, replace it with a spare, and reinstall it after calibration (at no additional charge). Receiver: Accredited calibration. During the warranty period, the supplier must periodically calibrate the receiver at an accredited laboratory every 2 years. All periodic calibrations of the receiver during the warranty period must be included in the supplier‘s tender price. The supplier shall dismantle the receiver, replace it with a spare and reinstall it after calibration (at no additional cost). |  |  |
|  | Electromagnetic compatibility | The proposed Item must comply with all applicable European Union and Lithuanian legal requirements, be marked with the CE mark, and have an EU declaration of conformity issued by the manufacturer or its authorized representative in accordance with the applicable standards. |  |  |
|  | Composition of the main parts of the Item | The equipment set consists of the following devices: • receiver with a directional antenna with an operating frequency of not less than 20 MHz to 8 GHz, set of 3 pcs. • a broadband receiver with directional antenna(s) with an operating frequency of not less than 9 kHz to 8 GHz, set of 3 pcs; • radio monitoring data recording device – 1 unit; • receiver with omnidirectional antenna(s) with an operating frequency of not less than 9 kHz to 8 GHz, set of 3; • antenna switch – 3 pcs.; • antenna rotator – 3 pcs.; • remote radio monitoring receiver with antenna set – 8 units; • all necessary accessories (adapters for mounting on a tower, signal, control and power cables and their connectors, GNSS antennas, etc.) and auxiliary materials ensuring the operation of the system are supplied with the Item |  |  |
|  | Structural diagrams of stationary radio monitoring stations and remote radio monitoring stations | Provided in Annex 1 to these Technical Specifications |  |  |
|  | Language | Conformity with the requirements set out in these Technical Specifications must be provided in Lithuanian or English. |  |  |
|  | Technical documentation | Technical documentation for the Item must be provided, including technical and functional Item data, technical and other conditions for expansion and supplementation (Technical Data Sheet) Technical documentation for the use of the Item intended for the user must be provided, covering installation, connection diagrams, use, functionality, change management, conditions of use, measurement management and execution, scenario preparation, report preparation, use of integrated third-party solutions (e.g., working with maps), additions, extensions including descriptions of cloud services offered/provided by the manufacturer of the Item, terms of use, technical documentation for independent use of the service intended for the user, in general – user manual or manuals (in general User Manuals, Installation Manuals, and/or Operating Manuals and/or Service Manuals, etc.). The specified documentation must be provided in printed and/or digital form, and the specified documentation may be provided as an exact URL link on the Internet or by providing access to the manufacturer‘s online resource intended for users, specifying the data required for login. Technical documents must be available in non-discriminatory, universally accessible file formats (e.g., .pdf, .doc, .docx, .xls, .xlsx, .ppt, .pptx, .txt, etc.). The Purchaser shall have the right to request the original documents. |  |  |
|  | Technical documentation for the proposed Item when submitting a tender | When submitting a tender, it is necessary to attach technical descriptions, instructions, etc. of the proposed equipment (including third-party equipment) so that the contracting authority (hereinafter referred to as the Purchaser) can assess the Seller‘s declared conformity with the requirements. The documentation may also be submitted on a digital medium, and additional URL (Uniform Resource Locator) links to Internet resources may also be provided. |  |  |
|  | Detailed work plan | Within 10 days of the contract coming into force, the Supplier must prepare, submit, and agree with the Purchaser a detailed work plan covering the stages of installation, testing, and acceptance testing of the monitoring systems. This plan must show all project activities, their interdependencies and responsibilities in terms of time. The plan must clearly indicate the essential (critical) stages. The Supplier‘s plan must clearly indicate the start and end dates of each activity. |  |  |
|  | Testing | Before signing a Deed of Acceptance and Transfer, the Supplier must perform testing to assess whether the delivered, installed, and commissioned equipment meets the requirements set out in this specification. The schedule for performing the equipment acceptance testing procedures must be agreed in advance by mutual agreement. |  |  |
|  | Training | The Purchaser must be provided with technical training material and/or documentation (Training or Learning manual) and/or video material for training purposes and other necessary documentation required to train the user on how to use the purchased equipment by preparing measurement scenarios and/or scripts, performing measurements and processing measurement results (filtering, comparison, reports, visualization; preparation of measurement protocols, etc.). The supplier shall provide practical training (instruction) on the use of the equipment to at least 10 (ten) of the Purchaser‘s employees, and the duration of the practical training (instruction) shall be at least than 16 hours. The Purchaser will provide suitable premises for the training in Vilnius. The practical training must be completed within 3 (three) months from the date of installation of the equipment in Vilnius. After installing the equipment in Kaunas, the Supplier shall provide practical training (instruction) on the use of the equipment to at least 5 (five) employees of the Purchaser, and the duration of the practical training (instruction) shall be at least 8 hours. The Purchaser shall provide suitable premises for the training in Kaunas. The practical training must be completed within 3 (three) months from the date of installation of the equipment in Kaunas. After installing the equipment in Klaipėda, the Supplier shall provide practical training (instruction) on the use of the equipment to at least 5 (five) of the Purchaser‘s employees, with the practical training (instruction) lasting no less than 8 hours. The Purchaser shall provide suitable premises for the training in Klaipėda. Practical training must be completed within 3 (three) months from the date of installation of the equipment in Klaipėda. The Supplier shall provide practical training (instruction) on the use of remote radio monitoring station equipment to at least 10 (ten) of the Purchaser‘s employees, with the practical training (instruction) lasting no less than 16 hours. The Purchaser shall provide suitable premises for the training in Vilnius. The practical training must be completed within 3 (three) months from the date of installation of the remote radio monitoring station equipment in Vilnius. |  |  |
|  | Data protection and cyber security | The software must have access control, user authentication, and data protection mechanisms. Data transmission must be encrypted using a secure virtual private network (VPN) connection. |  |  |
|  | Delivery schedule and payment terms | The Purchaser‘s binding orders: - Delivery of 1 set of stationary radio monitoring station equipment (Vilnius) and 3 sets of remote radio monitoring station equipment (Stage I) – 5 months from the effective date of the contract - Installation and testing of equipment - 1 month from delivery of equipment - Payment for stage I – 30 calendar days from the signing of a Deed of Acceptance and Transfer - Delivery of 1 set of stationary radio monitoring station equipment (Kaunas) (Stage II) – 10 months from the date of entry into force of the contract - Installation and testing of equipment – 1 month from delivery of equipment - Payment for stage II – 30 calendar days from the signing of a Deed of Acceptance and Transfer. - Delivery of 1 set of stationary radio monitoring station equipment (Klaipėda) (Stage III) - 15 months from the date of entry into force of the contract - Installation and testing of equipment - 1 month from delivery of equipment - Payment for stage III – 30 calendar days from the signing of a Deed of Acceptance and Transfer. - Delivery of centralized control point equipment - 5 months from the date of entry into force of the contract - Installation of the centralised control point - 1 month from delivery of the equipment - Final testing of the centralised control point – 30 months from the date of entry into force of the contract - Payment for the centralized control point - 30 calendar days from the signing of a Deed of Acceptance and Transfer Non-binding orders from the Purchaser: - Delivery of 3 sets of remote radio monitoring station equipment - 22 months from the date of entry into force of the contract. - Installation and testing of 3 sets of remote radio monitoring station equipment - 1 month from delivery of the equipment. - Payment for 3 sets of remote radio monitoring station equipment - 30 calendar days from the signing of a Deed of Acceptance and Transfer. - Delivery of 2 sets of remote radio monitoring station equipment - 26 months from the effective date of the contract. - Installation and testing of 2 sets of remote radio monitoring station equipment - 1 month from delivery of the equipment. - Payment for 2 sets of remote radio monitoring station equipment - 30 calendar days from the signing of a Deed of Acceptance and Transfer. |  |  |
|  |  | Quantity of identical components to be purchased: 3 sets of stationary radio monitoring station equipment; Maximum quantity 8 sets, but not less than 3 sets, of remote radio monitoring station equipment; The Service does not undertake to purchase the entire maximum quantity of remote radio monitoring station equipment sets. |  |  |
|  | Contract performance guarantee | The supplier must provide a contract performance guarantee |  |  |
|  | Condition of the Item | The Item must be new, unused, and may not contain renewed or refurbished components. The equipment must not cause radio interference to other radio monitoring equipment operating nearby. The Goods must be fit for their intended purpose, without any hidden defects or disabled functions and/or licenses that would prevent the goods from being used for their intended purpose in accordance with the requirements set out in this technical specification, or that would reduce the usefulness of the Item. |  |  |
|  | Conformity of the Item with standards and legislation | The Item must comply with the standards in force in the Republic of Lithuania and other legal requirements for such goods (and their components). |  |  |
|  | Warranty requirements |  |  |  |
|  | Warranty period | The warranty period for software and hardware and their installation must be not less than 60 months. |  |  |
|  | Repair of technical equipment during the warranty period | - During the warranty period, equipment repairs must be carried out within 20 days of the submission of the request. If the repair takes longer than 20 days, a replacement device must be provided. - When replacing individual equipment parts or individual devices with others, new equipment parts and/or devices must be used, the technical parameters of which must not be worse than those that existed before the failure. |  |  |
|  | Provision of updates | - Updates to software, device firmware, and technical documentation, as well as the provision of the latest versions, must be provided free of charge throughout the warranty period. - During the warranty period, the correction of software errors and security vulnerabilities must also be provided free of charge. This also includes functional improvements to the software and device firmware. - Response time (the time from when the Contracting Authority informs the Supplier about the equipment malfunction to when the Supplier informs the Contracting Authority that it is actually starting to fix the malfunction) – 48 hours; - Fault repair time (the time from when the Supplier informs the Contracting Authority that it has actually started repair work to when the Supplier informs the Contracting Authority that it has repaired the fault) – 10 days. - If the Supplier, for objective reasons and/or reasons beyond the Supplier‘s control, is unable to remedy the disruption within the specified time, it must immediately inform the Contracting Authority in writing (by e-mail) of such circumstances and reasons. After the Contracting Authority has assessed the validity of the circumstances and reasons indicated, the Contracting Authority and the Supplier may, by mutual agreement, set a different time for the elimination of the malfunction than that specified. |  |  |
|  | Restoration of measurement and radio monitoring data, software settings, and functionality | - When assembling the Item, provision must be made for the free restoration of radio monitoring and measurement data (hereinafter referred to as data), software settings and functionality in the measurement and recording devices and in the software provided by the Supplier (hereinafter collectively referred to as the Equipment) with the previous functionality and licenses (e.g., by providing new software and licenses or restoring the software and activating the previous licenses) - If this is not possible, the Equipment must have a backup feature that allows periodic backup of all equipment software (including licenses and settings) and stored data, from which the full functionality of the Item prior to the failure can be restored; - These requirements also apply if it is necessary to replace memory media or individual parts of the Equipment and/or individual devices because they cannot be used in the repaired Equipment due to software and technical changes made during the repair. - restoration of data obtained during radio monitoring and measurements (hereinafter referred to as data), software settings and functionality in measurement and recording devices and in the software provided by the Supplier (hereinafter collectively referred to as the Equipment) with the previous functionality and licenses must work in the Equipment returned after repair, regardless of whether the Item (or Equipment) were transferred for repair with or without memory media. |  |  |
|  | Requirements for memory media maintenance and data protection during the warranty period | - During the warranty period, in the event of failure of technical equipment or other Supplier devices provided to meet the requirements of the technical specifications, magnetic disks, hard disks, or other memory media (hereinafter collectively referred to as memory media) must be replaced with new ones. - Faulty storage media must be destroyed at the Purchaser‘s premises (the Supplier may be present if they so wish). Destroyed storage media shall not be returned to the Seller; - If the equipment is transferred to the Supplier and/or the manufacturer of the Item for repair, and the storage media cannot be mechanically removed without damaging the equipment, the Supplier and/or the manufacturer of the Item must provide the Purchaser with data protection confidentiality guarantees that the data on the memory media will not be accessible to third parties and will not be used without the Purchaser‘s permission; - If, in the event of a malfunction, the equipment and/or individual devices are sent to the Supplier (or the manufacturer of the Item) for repair, the Purchaser shall deliver the Item without the memory media installed in them, if they can be mechanically removed without damaging the devices; - If the Supplier indicates that it is necessary to supply the Item with memory media in order to perform a high-quality repair, the Supplier must provide the Purchaser with all the means free of charge to enable the Purchaser to remove any data sensitive to the Purchaser from the equipment (e.g. logins to the Purchaser‘s network and/or data on the network or cloud resources used by the Purchaser, etc.), except for data that the Purchaser has decided not to remove itself; - The Supplier‘s confidentiality obligations and data protection apply to the data that has not been removed, as regulated by the laws in force in the European Union. |  |  |
|  | Supplier‘ help centre services | The manufacturer‘s help centre services must be provided (but not limited to those listed): - Management of licenses granted (other software may also be used for this purpose); - Consultations on features, functionality, software operation, measurements, and data processing - Analysis and elimination of operational errors; - Solutions to problems arising from malfunctioning (if the declared functions do not work); - Correction of deficiencies in technical documentation (e.g., incomplete configuration description in the submitted version); - If necessary, remote diagnostics and software updates must be provided via a secure communication channel |  |  |
|  | Support after the warranty period | Support for software (updates) and hardware spare parts after the warranty period must be provided for at least 60 months, if the technical capabilities of the equipment manufacturer allow this. If this is not possible, other specific technical solutions must be offered. After this period, support services must continue to be provided for at least the portion of the products purchased by the buyer, but the response times, consultations, error corrections, software updates, explanations, etc., and the scope of cloud services shall be determined by the Supplier. |  |  |
|  | Technical requirements for the direction finder (+receiver) |  |  |  |
|  | Frequency range | Not less than 20 MHz to 8 GHz |  |  |
|  | Minimum frequency tuning resolution | Not greater than 1 Hz |  |  |
|  | Frequency setting error | Not greater than 2 × 10–7 |  |  |
|  | Real-time intermediate frequency bandwidth | Not less than 40 MHz |  |  |
|  | Third-order intercept (TOI) | Typical value not less than: -1 dBm at frequencies up to 5.3 GHz, -6 dBm at frequencies above 5.3 GHz |  |  |
|  | Displayed Average Noise Level (DANL) | Typical value not greater than: -164 dBm at frequencies up to 5.3 GHz, -162 dBm at frequencies above 5.3 GHz |  |  |
|  | Noise figure | Typical value not greater than: 12 dB at frequencies up to 3.3 GHz, 16 dB at frequencies from 3.3 to 5.3 GHz, 18 dBm at frequencies above 5.3 GHz |  |  |
|  | Image rejection | Typical value not less than: 110 dB at frequencies up to 1 GHz, 80 dB at frequencies above 1 GHz |  |  |
|  | Radio signal direction finding sensitivity, vertical polarization (system direction finding antenna and receiver) | Signal electromagnetic field strength dBµV/m: in the frequency band from 50 MHz to 3 GHz not greater than 5, in the frequency band from 3 GHz to 4.5 GHz not greater than 10, in the frequency band from 4.5 GHz to 7 GHz not greater than 15 |  |  |
|  | Direction finding sensitivity, horizontal polarization (system consisting of a direction finding antenna and receiver) | Electromagnetic field strength of the signal dBµV/m: in the frequency band from 50 MHz to 3 GHz not greater than 5, in the frequency band from 3 GHz to 4.5 GHz not greater than 15, in the frequency band from 4.5 GHz to 6.5 GHz not greater than 25 |  |  |
|  | Sensitivity | When demodulating an FM signal (at SINAD = 20 dB, fmod = 1 kHz): in the frequency band from 20 MHz to 35 MHz, with a bandwidth of 9 kHz and a deviation of 2.4 kHz, not greater than 97 dBm; in the frequency band from 35 MHz to 6 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not greater than 107 dBm; in the frequency band from 6 GHz to 7.5 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not greater than 102 dBm; in the frequency band from 7.5 GHz to 8 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not greater than 92 dBm |  |  |
|  | Detect signals with 100% probability of interception at the specified receiver parameters: FFT real-time IF span of at least 40 MHz, resolution bandwidth (RBW = 2 MHz) | Not worse than 1 µs |  |  |
|  | Intermediate frequency band width shape factor | Signal bandwidth at 3 dB and 50 dB attenuation limit, coefficient not greater than 1:1.6 for filters with an operating frequency up to 40 MHz |  |  |
|  | Analog/digital (A/D) converter (ADC) resolution | Not less than 14 bits |  |  |
|  | Maximum signal level at input | Not less than +20 dBm |  |  |
|  | Input signal level attenuator | Not less than 30 dB with 1 dB switching steps (switching can be selected manually or automatically) |  |  |
|  | Signal level resolution | Not greater than 0.1 dB |  |  |
|  | Signal level measurement error | Not greater than ±3 dB (in the temperature range –10 °C to +55 °C) |  |  |
|  | Signal level measurement detectors | Not less than: peak, root mean square (RMS), average |  |  |
|  | Spectrum scanning speed, displaying the entire frequency range | Not less than: 60 GHz/s (at 1 MHz RBW), 55 GHz/s (at 100 kHz RBW), 45 GHz/s (at 25 kHz RBW) |  |  |
|  | Frequency scan with user-selectable start/end frequency and step width | Not less than 2000 frequencies/s |  |  |
|  | Number of radio frequencies (addresses) stored in memory | At least 1000 radio frequencies (addresses) |  |  |
|  | Memory scan speed of radio frequencies (addresses) stored in memory | Scanning of radio frequencies (addresses) stored in memory (not less than 1000 addresses/s) |  |  |
|  | Bandwidth of the IQ data output connection for wideband spectrum (up to 40 MHz bandwidth) | Not less than 10 GBit |  |  |
|  | Antenna connectors | At least 2 connectors, input impedance 50 Ω |  |  |
|  | GNSS antenna connection | Must have a connection for an external active or passive GNSS antenna that ensures the reception and processing of satellite navigation signals from at least the following navigation systems: GPS, GLONASS, BeiDou, Galileo, connected |  |  |
|  | Direction finding error in AOA mode (system: direction finding antenna and receiver) | Not greater than ±2º RMS in mode |  |  |
|  | Minimum signal duration for direction finding | Not longer than 2 ms |  |  |
|  | Signal duration (for a single burst signal) for source direction determination in AOA mode | Not longer than 1.5 ms |  |  |
|  | Squelch modes | At least: off, gate, normal |  |  |
|  | Methods of recording data to internal memory and/or external recording device | Signal spectrum, IQ data (IQ snapshot), demodulated signal audio |  |  |
|  | Direction finding device (receiver) start-up/reload time | Must be fully operational no later than 180 s after the start of reloading |  |  |
|  | General functional requirements for the direction finder (+receiver) |  |  |  |
|  | Signal direction/source location determination functionalities | Must support AOA (Angle of Arrival), TDOA (Time Difference of Arrival) and hybrid (AOA together with TDOA) functionalities |  |  |
|  | Signal source location using TDOA and hybrid functionalities | TDOA functionality must be ensured by connecting the direction and location determination device (receiver) to a network of three remote radio monitoring receivers. |  |  |
|  | Types of demodulated signals | At least AM, FM, PM, pulse, I/Q, LSB, USB, CW |  |  |
|  | Be able to perform measurements in accordance with ITU recommendations | Must have the functionality to perform measurements in accordance with ITU recommendations, including: bandwidth measurements in accordance with ITU-R SM.443-4 (02/2007), radio frequency measurements according to ITU-R SM.377, modulation measurements in accordance with ITU-R SM.328, level and field strength measurements in accordance with ITU-R SM.378-7 |  |  |
|  | Direction finding functionality in the absence of a GNSS signal | Direction finding functionality must be ensured even in the absence of a GNSS signal. It must be possible to enter the coordinates of the equipment location |  |  |
|  | Software | Designed to connect and control the direction finding device (receiver) and remote radio monitoring receivers remotely, and to select measurement actions and settings |  |  |
|  | Direction finding modes | Direction finding functionality must be ensured in fixed frequency mode (FFM), frequency scan mode, and memory scan mode. |  |  |
|  | Broadband signal direction finding | Direction finding functionality must be ensured in fixed frequency mode (FFM). |  |  |
|  | DF band width | DF band width setting methods: manual, automatic (broadband signal direction determination depending on FFT selectivity) |  |  |
|  | General requirements for the direction finder (+receiver) |  |  |  |
|  | Operating temperature range | Not less than -30°C to +55°C |  |  |
|  | Remote control via LAN interface | RJ-45, 1 Gbit WAN |  |  |
|  | Power supply | 230 V 50 Hz AC network |  |  |
|  | Set of remote radio monitoring receiver control commands | The supplier, together with the direction-finding device (receiver), must provide a complete set of remote radio monitoring receiver control commands, including both remote and local control commands. The command set must include a description of the command syntax, parameters, and response structure. The set of control commands provided must ensure the integration of all equipment into the customer‘s control infrastructure. |  |  |
|  | Internal computer | Must have an integrated internal computer (PC) with at least 16 GB of DRAM RAM and an M.2 NVMe type hard drive for data storage. |  |  |
|  | Technical requirements for the spectrum monitoring receiver |  |  |  |
|  | Frequency range | Not less than 9 kHz to 8 GHz |  |  |
|  | Minimum frequency tuning resolution | Not greater than 1 Hz |  |  |
|  | Frequency determination error | Not greater than 2 × 10–7 |  |  |
|  | Real-time intermediate frequency bandwidth | Not less than 40 MHz |  |  |
|  | Third-order intercept (TOI) | Typical value not less than: -1 dBm at frequencies up to 5.3 GHz -6 dBm at frequencies above 5.3 GHz |  |  |
|  | Displayed Average Noise Level (DANL) | Typical value not greater than: -164 dBm at frequencies up to 5.3 GHz -162 dBm at frequencies above 5.3 GHz |  |  |
|  | Noise figure | Typical value not greater than: 12 dB at frequencies up to 3.3 GHz 16 dB at frequencies from 3.3 to 5.3 GHz 18 dBm at frequencies above 5.3 GHz |  |  |
|  | Image rejection | Typical value not less than: 110 dB at frequencies up to 1 GHz 80 dB at frequencies above 1 GHz |  |  |
|  | Sensitivity | When demodulating a FM signal (at SINAD = 20 dB, fmod = 1 kHz): in the frequency band from 20 MHz to 35 MHz, at a bandwidth of 9 kHz and a deviation of 2.4 kHz, not greater than 97 dBm; in the frequency band from 35 MHz to 6 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not greater than 107 dBm; in the frequency band from 6 GHz to 7.5 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not greater than 102 dBm; in the frequency band from 7.5 GHz to 8 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not greater than 92 dBm |  |  |
|  | Detect signals with 100% probability of interception at the specified receiver parameters: FFT real-time IF panorama (span) with a width of at least 40 MHz, Resolvable bandwidth (RBW = 2 MHz) | No worse than 1 µs |  |  |
|  | Types of demodulated signals | At least AM, FM, PM, pulse, I/Q, LSB, USB, CW |  |  |
|  | Automatic frequency correction (AFC) | Not greater than ± 1/2 of the selected intermediate frequency bandwidth (in the range from 10 Hz to 40 MHz) |  |  |
|  | Shape factor of the intermediate frequency bandwidth | Signal bandwidth at 3 dB and 50 dB attenuation limit, coefficient not greater than 1:1.6 for filters with an operating frequency up to 40 MHz |  |  |
|  | Analog/digital (A/D) converter (ADC) resolution | Not less than 14 bits |  |  |
|  | Maximum signal level at input | Not less than +20 dBm |  |  |
|  | Input signal level attenuator | Not less than 30 dB with 1 dB switching steps (switching can be selected manually or automatically) |  |  |
|  | Signal level resolution | Not greater than 0.1 dB |  |  |
|  | Signal level measurement error | Not greater than ±3 dB (in the temperature range –10 °C to +55 °C) |  |  |
|  | Signal level measurement detectors | Not less than: peak, root mean square (RMS), average |  |  |
|  | Spectrum scanning speed, displaying the entire frequency range | Not less than: 60 GHz/s (at 1 MHz RBW) 55 GHz/s (at 100 kHz RBW) 45 GHz/s (at 25 kHz RBW) |  |  |
|  | Data transfer rate for recording | Data from radio frequency bands not less than 9 kHz to 8 GHz must be recorded at a rate of not less than 30 GHz/s (at 12.5 kHz RBW) |  |  |
|  | Frequency scan with user-selectable start/end frequency and step width | Not less than 2000 frequencies/s |  |  |
|  | Number of radio frequencies (addresses) that can be stored in memory | At least 1000 radio frequencies (addresses) |  |  |
|  | Memory scan speed of radio frequencies (addresses) stored in memory | Scanning of radio frequencies (addresses) stored in memory (not less than 1000 addresses/s) |  |  |
|  | Bandwidth of the IQ data output connection for wideband spectrum (up to 40 MHz bandwidth) | Not less than 10 GBit |  |  |
|  | Antenna connectors | At least 2 connectors, input impedance 50 Ω |  |  |
|  | GNSS antenna connection | Must have a connection for an external active or passive GNSS antenna that ensures the reception and processing of satellite navigation signals from at least the following navigation systems: GPS, GLONASS, BeiDou, Galileo, connected |  |  |
|  | Record spectrum data for later analysis | Has this option |  |  |
|  | Ability to record radio spectrum (FFT) | Recording to internal memory and/or recording device |  |  |
|  | Ability to record signal spectrum (IQ data) | Recording to internal memory and/or recording device |  |  |
|  | Radio monitoring software for remote connection and control of the device and selection of measurement actions and settings | Has this option |  |  |
|  | Remote control | Has this option |  |  |
|  | Capable of performing measurements in accordance with ITU recommendations | Must have the functionality to perform measurements in accordance with ITU recommendations, including: bandwidth measurements in accordance with ITU-R SM.443-4 (02/2007), frequency measurements in accordance with ITU-R SM.377, modulation measurements in accordance with ITU-R SM.328, level and field strength measurements in accordance with ITU-R SM.378-7 |  |  |
|  | Storage of radio monitoring and measurement results. | A dedicated external or internal recording device must be used |  |  |
|  | Receiver start-up/reload time | Must be fully operational within 180 s of start-up |  |  |
|  | Internal computer | Must have an integrated internal computer (PC) with at least 16 GB of DRAM RAM and an M.2 NVMe type hard drive for data storage |  |  |
|  | General requirements for spectrum monitoring receivers |  |  |  |
|  | Operating temperature range | Not less than -30°C to +55°C |  |  |
|  | Remote control via LAN interface | RJ-45, 1 Gbit WAN |  |  |
|  | Power supply | 230 V 50 Hz AC mains |  |  |
|  | Technical requirements for broadband receiver |  |  |  |
|  | Frequency range | Not less than 9 kHz to 8 GHz |  |  |
|  | Sensitivity | In low noise mode: when demodulating an FM signal (at SINAD = 20 dB, fmod = 1 kHz): in the frequency band from 20 MHz to 6 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not greater than -108 dBm; when demodulating an AM signal (at SINAD = 12 dB, fmod = 1 kHz): in the frequency band from 20 MHz to 6 GHz,with a bandwidth of 6 kHz, not greater than 108 dBm; |  |  |
|  | Noise figure | Not greater than: 15 dB at frequencies up to 20 MHz, 13 dB at frequencies from 20 MHz to 650 MHz, 15 dB at frequencies from 650 MHz ≤ f < 3.6 GHz, 16 dB at frequencies 3.6 GHz ≤ f < 6 GHz, 12 dB at frequencies above 6 GHz |  |  |
|  | Displayed Average Noise Level (DANL, normalized to 1 Hz) value. | Not greater than: -159 dBm at frequencies up to 20 MHz, -164 dBm at frequencies from 20 MHz to 3.6 GHz, -162 dBm at frequencies above 3.6 GHz |  |  |
|  | Third-order intercept (TOI) in low distortion mode | Typical value not less than: 30 dBm at frequencies from 1 MHz to 20 MHz 20 dBm at frequencies from 20 MHz to 650 MHz, 23 dBm at frequencies from 650 MHz to 6 GHz, 9 dBm at frequencies from 6 GHz to 8 GHz |  |  |
|  | Analog/digital (A/D) converter (ADC) resolution. | Up to 125 MHz bandwidth: not less than 16 bits, Above 125 MHz bandwidth: not less than 12 bits |  |  |
|  | FFT real-time IF panorama. | Not less than 500 MHz bandwidth |  |  |
|  | Minimum signal duration | For a bandwidth of 500 MHz with 10 MHz resolution, minimum detectable signal duration not greater than 150 ns |  |  |
|  | IF bandwidth selection | Not less than: 10/30/50/100/150/300/600 Hz, 1/1.5/2.1/2.4/2.7/3.1/4/4.8/6/8.333/9/12/15/25/30/50/75/120/150/250/300/500/800 kHz, 1/1.25/1.5/2/5/8/10/12.5/15/20/40/80/125 MHz |  |  |
|  | Maximum input level at maximum signal attenuation | Not less than +15 dBm at frequencies from 9 kHz to 1.5 GHz, Not less than +10 dBm from 1.5 GHz to 8 GHz |  |  |
|  | Signal attenuation in steps of no more than 5 dB | Not less than 25 dBm at frequencies from 9 kHz to 20 MHz, Not less than 40 dBm at frequencies from 20 MHz to 1.5 GHz, Not less than 30 dBm at frequencies from 1.5 GHz to 8 GHz |  |  |
|  | Automatic attenuation of signals outside the monitoring range (interference rejection) | Not less than 90 dB at frequencies from 9 kHz to 8 GHz |  |  |
|  | Panoramic scanning speed of the frequency band | Not less than 110 GHz/s with 25 kHz resolution |  |  |
|  | Must have an optical data output with a speed of at least | 100 Gb/s (to ensure 500 MHz bandwidth I/Q data recording) |  |  |
|  | Number of radio signals measured in real time | At least 2 |  |  |
|  | Automatic detection and classification of unwanted emissions | Has such functionality |  |  |
|  | Storage of records | A dedicated external or internal recording device (recording method) must be used to store radio monitoring and measurement results |  |  |
|  | Level and field strength measurements. Conversion of received signal level to field strength, measurement of strong fields | Conformity with ITU recommendation ITU-R SM.378-7 |  |  |
|  | Radio signal parameter measurements | Functionality to perform measurements of AM, FM, PM, and IQ signal modulation parameters, including modulation depth, frequency deviation, and phase deviation, signal level change monitoring |  |  |
|  | Bandwidth measurements based on x dB and ß % methods | Conformity with ITU recommendation ITU-R SM.443-4 (02/2007) |  |  |
|  | Channel power measurement | Has such functionality |  |  |
|  | Device management software | Designed to connect and control the device remotely and select measurement actions and settings |  |  |
|  | Detect signals with 100% probability of detection at the specified receiver parameters: FFT real-time IF span of at least 40 MHz width, Resolvable bandwidth (RBW = 2 MHz) | No worse than 1 µs |  |  |
|  | Waterfall display | Spectrogram with integrated history buffer and up to 100 μs time resolution |  |  |
|  | Real-time intermediate frequency bandwidth. | Not less than 40 MHz |  |  |
|  | Signal level measurement detectors. | Not less than: peak, root mean square (RMS), average |  |  |
|  | Verification equipment/function installed in the device with the ability to save and export report data | The broadband receiver must have verification equipment installed that is responsible for the smooth operation of the device and its testing during both start-up and operation. |  |  |
|  | Graphical user interfaces installed on the user‘s computer are provided | Spectrum display together with temporary spectrum display (waterfall); Three fully configurable spectrum display modes (trace) with freely selectable RBW; Spectrum history mode – spectrum reproduction from the device memory must be ensured according to user needs; Spectrum and audio recording playback using a graphical user interface |  |  |
|  | General requirements for a broadband spectrum receiver |  |  |  |
|  | Power supply | 230 V 50 Hz AC mains. |  |  |
|  | Technical requirements for radio monitoring data recording equipment |  |  |  |
|  | RJ45 data transmission interface for device control | At least 1 unit, with a speed of at least 1 Gb/s |  |  |
|  | RJ45 data transmission interface for data transmission from/to the recording device and/or for connecting device users | At least 2 units, with a speed of not less than 10 Gb/s |  |  |
|  | SFP+ data transfer interface for connecting external devices | At least 4 units, with a speed of at least 10 Gb/s |  |  |
|  | QSFP28 data transmission interfaces for connecting broadband receivers | At least 2 units, with a speed of at least 100 Gb/s |  |  |
|  | QSFP28 data transmission interfaces for data transmission from/to the recording device and/or for connecting device users | At least 2 units, with a speed of not less than 100 Gb/s |  |  |
|  | Maximum radio frequency band that must be supported via a 10 Gb/s interface while the device is continuously recording/reading | Not less than 125 MHz |  |  |
|  | Maximum radio frequency band that must be supported via a 100 Gb/s interface for continuous recording by the device | Not less than 500 MHz |  |  |
|  | Number of supported server SSD modules | At least 2 |  |  |
|  | Total capacity of server SSD modules | Not less than 50 TB |  |  |
|  | I/Q data stream recording | I/Q data stream must be recorded in a radio frequency band not less than 500 MHz and for no less than 5 hours. |  |  |
|  | Number of parallel data streams supported by the recording device | At least 50 |  |  |
|  | Types of data recorded | - Must be capable of recording spectrum (FFT) in parallel 24/7 across the entire radio frequency band of radio monitoring receivers from at least 20 remote radio monitoring devices. - Must be capable of recording I/Q data in parallel from at least 10 remote radio monitoring devices; - Additionally, it must be capable of recording audio; - Both the radio spectrum (FFT) and I/Q data must be recorded simultaneously; - Recordings must also support an open data format so that they can be played back using third-party software |  |  |
|  | Connection of additional data storage/server | The recording device must be able to connect to a data storage device/server via an interface/interfaces with a speed of at least 100 Gb/s, which is used to transfer data to/from the recording device |  |  |
|  | Number of simultaneous users supported by the recording device | At least 2 |  |  |
|  | # Technical requirements for antennas, antenna rotation system, antenna switching. |  |  |  |
|  | Technical requirements for broadband directional antennas in the frequency range from 80 MHz to 8 GHz |  |  |  |
|  | Technical requirements for broadband directional antennas |  |  |  |
|  | Frequency range | Not less than 80 MHz to 1.5 GHz |  |  |
|  | Polarization | Linear |  |  |
|  | Type | Directional |  |  |
|  | Gain | At frequencies from 80 to 1.5 GHz, it must be not less than 5 dBi without an amplifier |  |  |
|  | Technical requirements for a broadband directional antenna with a reflector |  |  |  |
|  | Frequency range | Not less than 1.5 GHz to 8 GHz |  |  |
|  | Type | Directional |  |  |
|  | Polarization | Linear |  |  |
|  | Gain | At frequencies from 1.5 GHz to 8 GHz, not less than 12 dBi, with the amplifier turned off |  |  |
|  | Gain parameters | Gain at frequencies from 1.5 GHz to 8 GHz not less than 30 dB, Noise figure not greater than 3.5 dB |  |  |
|  | Technical requirements for direction finder‘s direction finding antenna |  |  |  |
|  | Polarization of the directional antenna for V/U/SHF direction finders | Vertical and horizontal |  |  |
|  | Operating frequency range of the direction-finding antenna for V/U/SHF direction finders | Vertical polarization not less than 20 MHz to 8 GHz, horizontal polarization not less than 20 MHz to 7 GHz |  |  |
|  | Amplifier activation/deactivation | Electronically. Control software |  |  |
|  | Direction finding error (system: direction finding antenna and receiver) | Not greater than ±2º RMS in mode |  |  |
|  | Deviation of the horizontal plane diagram of the horizontal polarization antenna of the direction finder from the non-directional antenna diagram | Not greater than 3 dB |  |  |
|  | Polarization change. | Electronically. Control software |  |  |
|  | Technical requirements for the direction finder‘s monitoring antenna |  |  |  |
|  | Operating frequency range of the direction finder radio monitoring antenna | Vertical polarization not less than 20 MHz to 8 GHz, horizontal polarization not less than 20 MHz to 8 GHz |  |  |
|  | Polarization of the operating frequencies of the direction finder radio monitoring antenna | Vertical and horizontal |  |  |
|  | Standing wave ratio in the frequency band specified in point 6.4.1 | Not greater than 2.0 |  |  |
|  | Technical requirements for omnidirectional monitoring antennas |  |  |  |
|  | Frequency range | Not less than 20 MHz to 8 GHz |  |  |
|  | Polarization | Linear vertical and horizontal |  |  |
|  | Vertical plane diagram unevenness | Not greater than 3 dB |  |  |
|  | Horizontal plane diagram unevenness | Not greater than 5 dB |  |  |
|  | Third-order intercept (TOI) of vertical polarization | Typical value not less than: 25 dBm at frequencies up to 850 MHz, 22 dBm at frequencies above 850 MHz |  |  |
|  | Third-order intercept (TOI) for horizontal polarization | Typical value not less than: 20 dBm at frequencies up to 850 MHz 23 dBm at frequencies above 850 MHz |  |  |
|  | Technical requirements for omnidirectional antennas for low-frequency monitoring |  |  |  |
|  | Frequency range | Not less than 9 kHz to 20 MHz |  |  |
|  | Polarization | Vertical |  |  |
|  | Third-order intercept (TOI) | Typical value not less than: 33 dBm at frequencies up to 20 MHz |  |  |
|  | Technical requirements for antenna switches. |  |  |  |
|  | Antenna switch | Must be capable of connecting any of the system‘s antennas to the measurement receiver. Designed for outdoor use and mounted on an antenna tower next to the antenna rotation device |  |  |
|  | Number of signal inputs for the antenna switch | At least 5 |  |  |
|  | Frequency range of the antenna switch | Not less than DC to 8 GHz |  |  |
|  | Antenna switch attenuation in the frequency range up to 8 GHz | Not greater than 1.7 dB |  |  |
|  | Antenna switching time | Not greater than 15 ms |  |  |
|  | Antenna switch control | Electronically, Control software |  |  |
|  | Technical requirements for antenna rotation device |  |  |  |
|  | Antenna rotator | Must be able to rotate antennas horizontally (azimuth) and change antenna polarization vertically (elevation/polarization). Must be hollow so that the antenna cable can be routed inside the adapter |  |  |
|  | Weight of the antenna rotator. | Not more than 200 kg |  |  |
|  | Antenna rotator control unit | Designed for outdoor use and mounted on the antenna tower next to the antenna rotation device |  |  |
|  | Technical requirements for the operation of the antenna rotation device in the horizontal plane |  |  |  |
|  | Range of directional variation in the horizontal plane (azimuth) | Not less than 0 to 400 degrees |  |  |
|  | Rotation speed in the horizontal plane | Not less than 2.2 degrees/sec (typical) |  |  |
|  | Direction repeatability (positioning accuracy) in the horizontal plane | Not worse than ±0.2 degrees (typical) |  |  |
|  | Type of rotation device | Rotation device type - moving, i.e. there is a vertical pipe in the centre of the device, around which the moving part of the rotation device rotates. There are flanges at both ends of the pipe, allowing it to be used as an adapter between two stationary sections of the antenna tower |  |  |
|  | Rotation device operation | The configuration must ensure reliable operation of the proposed system. |  |  |
|  | Protection when changing the antenna azimuth | There must be internal switches that cut off the motor power supply when the rotation device reaches either end of its azimuth range. |  |  |
|  | Technical requirements for the operation of the antenna rotation device in the vertical plane |  |  |  |
|  | Range of variation in the vertical plane (polarization) | Not less than -90 to +90 degrees (step not greater than 1 degree) |  |  |
|  | Rotation speed in the vertical plane | Not less than 9 degrees/sec (typical) |  |  |
|  | Direction repeatability (positioning accuracy) in the vertical plane | No worse than ±0.2 degrees (typical) |  |  |
|  | Protection when changing the antenna direction (polarization) in the vertical plane | There must be internal switches that cut off the motor power supply when the direction of rotation of the rotating device reaches either end of the range in the vertical plane |  |  |
|  | Attenuation of the signal radio frequency cable for connection to the antenna switch in the receiver room | Not more than 25 dB/100 m at 8 GHz |  |  |
|  | Signal radio frequency cable for connecting antennas to the antenna switch | Not more than 40 dB/100 m at 8 GHz |  |  |
|  | Software control equipment | Must be provided |  |  |
|  | Technical requirements for GNSS antennas. |  |  |  |
|  | GNSS antenna | Must ensure reception and processing of satellite navigation signals from at least the following navigation systems: GPS, GLONASS, BeiDou, Galileo |  |  |
|  | Antenna type | Active CRPA type antenna |  |  |
|  | Active GNSS antenna low-noise amplifier gain | Not less than 15 dB |  |  |
|  | General requirements for antennas, antenna rotation system, antenna switching. |  |  |  |
|  | Operating temperature range | Not less than -30°C to +55°C |  |  |
|  | Protection against strong static electric fields | Must be ensured |  |  |
|  | Lightning protection | The direction-finding antenna must have integrated lightning protection. Lightning protection elements must not adversely affect the directional accuracy of the signal measured by the direction-finding device |  |  |
|  | Technical requirements for remote radio monitoring receivers |  |  |  |
|  | Frequency range | Not less than 9 kHz to 8 GHz |  |  |
|  | Minimum frequency tuning resolution | Not greater than 1 Hz |  |  |
|  | Frequency setting error | Not greater than 2 × 10–7 |  |  |
|  | Real-time intermediate frequency bandwidth | Not less than 40 MHz |  |  |
|  | Record spectrum data for later analysis | Has such functionality |  |  |
|  | Third-order intercept (TOI) | Typical value not less than: -1 dBm at frequencies up to 5.3 GHz, -6 dBm at frequencies above 5.3 GHz |  |  |
|  | Displayed Average Noise Level (DANL) | Typical value not greater than: -164 dBm at frequencies up to 5.3 GHz, -162 dBm at frequencies above 5.3 GHz |  |  |
|  | Noise figure | Typical value not greater than: 12 dB at frequencies up to 3.3 GHz, 16 dB at frequencies from 3.3 to 5.3 GHz, 18 dBm at frequencies above 5.3 GHz |  |  |
|  | Image rejection | Typical value not less than: 110 dB at frequencies up to 1 GHz, 80 dB at frequencies above 1 GHz |  |  |
|  | Sensitivity | When demodulating an FM signal (at SINAD = 20 dB, fmod = 1 kHz): in the frequency band from 20 MHz to 35 MHz, at a bandwidth of 9 kHz and a deviation of 2.4 kHz, not greater than -97 dBm; in the frequency band from 35 MHz to 6 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not greater than -107 dBm; in the frequency band from 6 GHz to 7.5 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not greater than 102 dBm; in the frequency band from 7.5 GHz to 8 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not greater than 92 dBm |  |  |
|  | Detect signals with 100% probability of interception at the specified receiver parameters: FFT real-time IF panorama (span) not less than 40 MHz wide, resolution bandwidth (RBW = 2 MHz) | Not worse than 1 µs |  |  |
|  | Types of demodulated signals | At least AM, FM, PM, pulse, I/Q, LSB, USB, CW |  |  |
|  | Automatic frequency correction (AFC) | Not greater than ± 1/2 of the selected intermediate frequency bandwidth (in the range from 10 Hz to 40 MHz) |  |  |
|  | Intermediate frequency band width shape factor | Signal bandwidth at 3 dB and 50 dB attenuation limit, coefficient not greater than 1:1.6 for filters with an operating frequency up to 40 MHz |  |  |
|  | Resolution of the analog/digital (A/D) converter (ADC) | Not less than 14 bits |  |  |
|  | Maximum signal level at input | Not less than +20 dBm |  |  |
|  | Input signal level attenuator | Not less than 30 dB with 1 dB switching step (switching can be selected manually or automatically) |  |  |
|  | Signal level resolution | Not greater than 0.1 dB |  |  |
|  | Signal level measurement error | Not greater than ±3 dB (in the temperature range –10 °C to +55 °C) |  |  |
|  | Signal level measurement detectors | Not less than: peak, root mean square (RMS), average |  |  |
|  | Spectrum scanning speed, displaying the entire frequency range | Not less than: 60 GHz/s (at 1 MHz RBW), 55 GHz/s (at 100 kHz RBW), 45 GHz/s (at 25 kHz RBW) |  |  |
|  | Data transfer rate for recording | Data from radio frequency bands not less than 9 kHz to 8 GHz must be recorded at a rate of not less than 30 GHz/s (with 12.5 kHz RBW). |  |  |
|  | Frequency scan with user-selectable start/end frequency and step width | Not less than 2000 frequencies/s |  |  |
|  | Frequency scan with user-selectable start/end frequency and step width | Not less than 2000 frequencies/s |  |  |
|  | Number of radio frequencies (addresses) stored in memory | At least 1000 radio frequencies (addresses) |  |  |
|  | Memory scan speed of radio frequencies (addresses) stored in memory | Scanning of radio frequencies (addresses) stored in memory (not less than 1000 addresses/s) |  |  |
|  | Broadband (up to 40 MHz bandwidth) IQ data output connection bandwidth | Not less than 10 GBit |  |  |
|  | Antenna connectors | At least 2 connectors, input impedance 50 Ω. |  |  |
|  | GNSS antenna connector | There must be a connection for an external active or passive GNSS antenna that would ensure the reception and processing of satellite navigation signals from at least the following navigation systems: GPS, GLONASS, BeiDou, Galileo, connect |  |  |
|  | Record spectrum data for later analysis | Has this option |  |  |
|  | Ability to record radio spectrum (FFT) | Recording to internal memory and/or recording device |  |  |
|  | Ability to record signal spectrum (IQ data) | Recording to internal memory and/or recording device |  |  |
|  | Radio monitoring software for remote connection and control of the device and selection of measurement actions and settings | Has such functionality |  |  |
|  | Remote control | Has the following functionality |  |  |
|  | Ability to perform measurements in accordance with ITU recommendations | Must have functionality to perform measurements in accordance with ITU recommendations, including: bandwidth measurements in accordance with ITU-R SM.443-4 (02/2007), frequency measurements in accordance with ITU-R SM.377, modulation measurements in accordance with ITU-R SM.328, level and field strength measurements in accordance with ITU-R SM.378-7 |  |  |
|  | Storage of radio monitoring and measurement results | A dedicated external or internal recording device must be used |  |  |
|  | Possibility to use TDOA for source location | Has such functionality |  |  |
|  | RF spectrum recording with user-selectable start/end frequency and resolution | Has such functionality |  |  |
|  | Automatic detection and classification of unwanted emissions | Has such functionality |  |  |
|  | Autonomous operation for 12 hours in the event of a power failure | Has this option |  |  |
|  | Ability to store data without internet connection for at least 4 hours and then transfer it to a data storage server | Has this option |  |  |
|  | Ability to connect to a network of remote radio monitoring receivers | Has this option |  |  |
|  | Receiver start-up/reboot time | Must be fully ready for operation within 180 seconds of start-up |  |  |
|  | Internal computer | Must have an integrated internal computer (PC) with at least 16 GB of DRAM RAM and an M.2 NVMe type hard drive for data storage |  |  |
|  | General requirements for remote radio monitoring receivers |  |  |  |
|  | Operating temperature range | Not less than -30°C to +55°C |  |  |
|  | Remote control via LAN interface | RJ-45, 1 Gbit WAN |  |  |
|  | Power supply | 230 V 50 Hz AC mains |  |  |
|  | Functional requirements for software |  |  |  |
|  | Functional requirements for software designed to control a radio monitoring receiver with direction finding function |  |  |  |
|  | Control of a radio monitoring receiver with direction finding function (hereinafter referred to as the receiver) and antenna module (hereinafter referred to as the antenna) | Control of the receiver and antenna via LAN interface. Direct control via virtual panels |  |  |
|  | Display of electromagnetic field strength in dBµV/m | The manufacturer calibrates the monitoring antennas with the amplifier turned on and off and enters the data into the monitoring antenna factor tables. The manufacturer calibrates the radio cable(s) between the receiver and the antenna and enters the data into the cable attenuation tables. The electromagnetic field strength is calculated automatically using data from the corresponding tables. |  |  |
|  | Data import and export | Uploading data on licensed radio frequencies and channels from standard Windows data programs. Exporting monitoring and measurement data to standard Windows programs |  |  |
|  | Antenna polarization and amplifier control | The program window displays the currently used antenna and amplifier status (on/off). All settings can be changed with a click of the mouse |  |  |
|  | Selection of measurement tasks | In the general task window, the user controls the receiver and performs measurements. It contains several tabs. Each tab corresponds to a selected measurement method. Depending on the measurement method, the measurement results are displayed using different types of graphs, tables, or digital displays. Measurement results can be saved for further analysis |  |  |
|  | Measurement methods | At least the following: Direct, real-time. The user monitors the radio spectrum in real time, can change the radio frequency overview range limits and resolution, record the spectrum, demodulate the selected signal, and record the demodulated signal sound. Monitors the radio spectrum in real time and simultaneously displays the color radio spectrum over time (waterfall) or the change in the selected signal over time (zero span). Measures the electromagnetic field strength of the selected signal. Interactive, in real time. The user selects the radio frequency range, scan step, noise threshold, and table containing data on licensed radio frequencies or transmitters. During measurement, signals that exceed the noise threshold and are not included in the table of licensed radio frequencies or transmitters are automatically detected. The results are displayed on the screen in real time and recorded. Analyze the recorded data, select the required frequency(ies) from the list, and generate a report. The report must display at least the following parameters: frequency, strength, detection time, occupancy. Automatic, according to a schedule set by the user. Automatically determine whether the direct result is outside the limits defined by the user and respond as defined by the user. Perform at least the following tasks: Measure the electromagnetic field strength of transmitters from a predefined table, compare the results with the limits set by the user, and generate reports on non-conformities. Detection of signals that exceed the set noise threshold and are not included in the table of licensed radio frequencies or transmitters. After detection, if specified by the user, automatically determine the direction of the signal. The measurement results can be analyzed during or after the task has been completed. Real-time signal direction detection. The user monitors the radio spectrum in real time and can change the limits and resolution of the radio frequency overview range. The detected directions of the selected signal are displayed in real time in a table and on a map and are automatically recorded. |  |  |
|  | Functional requirements for software for controlling radio monitoring receivers |  |  |  |
|  | Modes supported by the software | At least the following: Direct, real-time. The user monitors the radio spectrum in real time, can change the limits and resolution of the radio frequency overview range, record the spectrum, demodulate the selected signal, and record the sound of the demodulated signal. Monitors the radio spectrum in real time and simultaneously displays the color radio spectrum over time (waterfall) or the change in the selected signal over time (zero span). Measures the electromagnetic field strength of the selected signal. Interactive, in real time. The user selects the radio frequency range, scan step, noise threshold, and table containing data on licensed radio frequencies or transmitters. During measurement, signals that exceed the noise threshold and are not included in the table of licensed radio frequencies or transmitters are automatically detected. The results are displayed on the screen in real time and recorded. Analyze the recorded data, select the required frequency(ies) from the list, and generate a report. The report must display at least the following parameters: frequency, strength, detection time, and occupancy. Automatic, according to a schedule created by the user. Automatically determine whether the direct result is outside the the user-defined values and respond as defined by the user. Perform at least the following tasks: Measure the electromagnetic field strength of transmitters from a predefined table, compare the results with user-defined limits, and generate reports on non-conformity. Detection of signals that exceed the specified noise threshold and are not included in the table of licensed radio frequencies or transmitters. After detection, if specified by the user, automatically determine the direction of the signal. Measurement results can be analyzed during or after the task is completed. Real-time location of the signal source. The user monitors the radio spectrum in real time and can change the limits and resolution of the radio frequency overview range. Real-time location of the selected signal and display in a table and on a map. |  |  |
|  | Spectrum recording from radio monitoring receivers | The software must ensure 24/7 parallel recording of the spectrum (FFT) across the entire radio frequency band of radio monitoring receivers from at least 20 remote radio monitoring devices |  |  |
|  | Conformity of measurements performed with the software with ITU recommendations | Measurements performed by the software must comply with the following ITU recommendations, including: - Bandwidth measurements in accordance with ITU-R SM.443-4 (02/2007); - Frequency measurements in accordance with ITU-R SM.377; - Modulation measurements in accordance with ITU-R SM.328; - Level and field strength measurements in accordance with ITU-R SM.378-7 |  |  |
|  | Control of antenna switches, rotors, and filter equipment | The software must be able to control antenna switches, rotors, and filter equipment depending on the measurements being performed. |  |  |
|  | Remote task assignment | The software must be able to assign tasks remotely to both individual remote radio monitoring receivers and user-defined groups of remote radio monitoring receivers |  |  |
|  | User hierarchy and identification | The software must support different user levels (user rights) and user identification. |  |  |
|  | Automatic antenna selection according to the monitored frequency band | If several antennas are required for the radio frequency band specified during radio frequency range data recording, the software must ensure automatic antenna switching depending on the operating radio frequency band of the antennas. |  |  |
|  | Continuous recording of the monitored radio frequency range after an interruption | All radio monitoring stations (both remote and stationary) must ensure 24/7 recording of data for the monitored radio frequency range, except in cases where the equipment of a specific monitoring station or stations is used to perform other tasks – in such cases, after the tasks have been completed, the equipment of the station or stations must automatically resume recording of data for the monitored radio frequency range.is used to perform other tasks – in such cases, after the tasks have been completed, the equipment of the station/stations must automatically resume recording radio frequency range data |  |  |
|  | Centralized management | The software must ensure: - Centralized management of the “Alarm“ function; - Task formation for mobile radio monitoring station teams; - Short-term access to stationary stations and remote radio monitoring receivers for mobile radio monitoring station teams, as necessary to perform the tasks received |  |  |
|  | Functional requirements for software designed to process data received during radio monitoring |  |  |  |
|  | Detection, identification, and classification of monitored/recorded radio signals | The software must be able to automatically recognize radio signals, detect them, identify them, and classify them according to its database |  |  |
|  | Adding user data to the software‘s radio signal database | The software‘s radio signal database must be able to be supplemented with user data |  |  |
|  | List of radio frequencies used with permits | The software must be able to import information from the RRT license database |  |  |
|  | Spectrum occupancy check | The software must be able to automatically measure how much of the radio frequency band of interest to the user is occupied. |  |  |
|  | Automatic detection of radio frequencies used without a license | The software must be able to automatically compare the list of radio frequencies imported from the RRT database with the list of radio frequencies used with permits, determine which radio frequencies from the radio frequency band of interest to the user are being used without a permit, and provide a list of these radio frequencies used without a permit. |  |  |
|  | Recording, playback, and processing of radio signals | The software must be able to record, reproduce, or process several signals simultaneously. |  |  |
|  | Signal demodulation and decoding | The software must be capable of demodulating and/or decoding signals both automatically according to an internal software list and according to user-defined parameters |  |  |
|  | Recording and playback of demodulated audio signals. | The software must be capable of recording and playing back demodulated audio signals |  |  |
|  | Identification of real-time monitored/recorded data | The software must be able to clearly distinguish between signals that are monitored in real time and those that are presented from recorded data |  |  |
|  | Number of digital down converter (DDC) channels | At least 128 |  |  |
|  | Presentation of measurement results | The software must process the measurement data, present statistics, and generate measurement result reports |  |  |
|  | Storage of records | All records are stored locally in a single database located at the central control point Records are also stored in an open format Records are stored for at least 3 months. |  |  |
|  | Technical equipment | To ensure the above-mentioned functions, the supplier may also use hardware modules such as broadband signal processors, etc. |  |  |
|  | Waterfall function | There must be a spectrum display function together with a waterfall function with an integrated history buffer for recorded data for a period of not less than 24 hours and a time resolution of up to 100 μs time resolution |  |  |
|  | General requirements for software |  |  |  |
|  | User interface operating system | Must be Windows 11 or newer (64-bit). |  |  |
|  | Network operation | Controlled from the control centre. User logins with identification must be local and remote. |  |  |
|  | I/Q data stream processing | I/Q data recovery and processing for 24/7 radio monitoring receivers, direction finder receivers, remote radio monitoring receivers, and broadband receivers. |  |  |
|  | Support for map layers from different providers | The software must have automatic access to several online map providers, including at least Microsoft‘s Bing Maps, Bing Satellite, Bing Hybrid, and OpenStreetMap |  |  |
|  | Number of software user licenses | At least 12 users at a time |  |  |
|  | Control centre and autonomous operation |  |  |  |
|  | Requirements for the control centre | The Item must be controlled from a control centre |  |  |
|  | Requirements for equipment installation |  |  |  |
|  | Equipment installation requirements for fixed spectrum monitoring stations |  |  |  |
|  | The supplier must ensure that all supplied equipment is fully installed, connected, configured, and ready for operation (on a “fully operational“ basis) |  |  |  |
|  | All work must be carried out in accordance with the manufacturer‘s recommendations and applicable occupational safety, electrical installation, and radio equipment installation standards |  |  |  |
|  | The supplier must have civil liability insurance for the performance of such work. |  |  |  |
|  | The supplier must coordinate the timing of the work with the customer. It is not mandatory to avoid disrupting the operation of the existing radio spectrum monitoring station |  |  |  |
|  | The supplier must ensure that the existing physical infrastructure (tower, building, grounding system, etc.) is not damaged during the work. |  |  |  |
|  | The supplier must dismantle the existing equipment that will be replaced with new equipment (antennas, unnecessary cables, brackets, etc.). |  |  |  |
|  | The dismantled equipment must not be damaged during dismantling and must be handed over to the customer. |  |  |  |
|  | A technical equipment installation project must be prepared and agreed with the customer for the installation of new equipment, which must assess the permissible load on the existing tower. |  |  |  |
|  | The technical equipment installation project must include the fastening elements for the equipment (antennas, cables, rotors, etc.) or their manufacture must be included in the scope of supply |  |  |  |
|  | The supplier is responsible for connecting the new equipment (antennas, receivers, analyzers, data storage, etc.) and connecting it to the power supply and data communication network. |  |  |  |
|  | The supplier must configure the equipment system in accordance with the parameters and requirements specified in the technical specifications. |  |  |  |
|  | The supplier must perform functional tests and submit test reports to the customer for approval. |  |  |  |
|  | The installation work will be completed by signing a Deed of equipment installation and configuration Acceptance and Transfer, which will confirm that everything has been installed correctly. |  |  |  |
|  | The installation work is covered by a 24-month warranty period |  |  |  |
|  | The supplier is responsible for any defects arising from poor-quality installation or system configuration. |  |  |  |
|  | After dismantling and installing new equipment, the site must be cleaned up and waste removed |  |  |  |
|  | Equipment installation requirements for remote spectrum monitoring units |  |  |  |
|  | The supplier must ensure that all supplied equipment is fully installed, connected, configured, and ready for operation (on a “fully operational“ basis) |  |  |  |
|  | All work must be carried out in accordance with the manufacturer‘s recommendations and applicable occupational safety, electrical installation, and radio equipment installation standards. |  |  |  |
|  | The supplier must have civil liability insurance for the performance of such work. |  |  |  |
|  | The supplier must coordinate the timing of the work with the customer |  |  |  |
|  | The supplier must ensure that the existing physical infrastructure (building, grounding system, etc.) is not damaged during the work |  |  |  |
|  | A technical equipment installation project must be prepared and agreed with the customer for the installation of new equipment |  |  |  |
|  | The technical equipment installation project must include the fastening elements for the equipment (antennas, cables, etc.) or their manufacture must be included in the scope of supply |  |  |  |
|  | The supplier is responsible for connecting the new equipment (antennas, receivers) to the power supply network and the data communication network |  |  |  |
|  | The supplier must configure the equipment system in accordance with the parameters and requirements specified in the technical specifications |  |  |  |
|  | The supplier must perform functional tests and submit test reports to the customer for approval |  |  |  |
|  | The installation work will be completed by signing a Deed of equipment installation and configuration Acceptance and Transfer, which will confirm that everything has been installed correctly |  |  |  |
|  | The works are covered by a 24-month warranty period |  |  |  |
|  | The supplier is responsible for any defects arising from poor installation or system configuration |  |  |  |
|  | After the work is completed, the work site must be cleaned up and waste removed |  |  |  |
|  | Before submitting a tender, the supplier will be given the opportunity to assess the objects on site. Visual material of the stationary spectrum monitoring stations is provided in Annex 2 to these Technical Specifications. |  |  |  |