TECHNICAL SPECIFICATION

Real-Time system kit

The offered product must meet the following requirements:

| No. | ## Technical characteristics | ## Requirements | ## Parameters of the equipment offered by the supplier and their values according to the requirements of columns 2 and 3 ## Must be filled out in detail |
| --- | --- | --- | --- |
| 1 | ## 2 | ## 3 | ## 4 |
|  | ## Real-Time system kit |  |  |
| 1. | Type | Real-Time system kit |  |
| 2. | General requirements | 1. Performance Real-Time Target Machine; 2. FPGA - Configurable Workflow 3. PROFINET; 4. Speedgoat Fundamentals – Half-Day Web Credit; 5. 5 Years Systems Software Maintenance and Support Service. |  |
| 3. | Complete set | 1. Performance Real-Time Target Machine: a) Performance real-time target machine (P3) 1 pcs. Complete, fully assembled, and tested Performance realtime target machine with access to I/O modules at the front side. Including 19" 4U aluminum chassis, integrated AC/DC power supply, low-noise fans, 8 GB Memory, 128 GB SSD, industrial-grade mainboard with 4-core 3.1 GHz Intel Core processor and onboard expansion slots to accommodate up to 7 I/O modules. Onboard communication: 1x Gigabit Ethernet interface for host-target communication, 1x Gigabit Ethernet interface supporting network protocols such as EtherCAT Master, UDP, TCP/IP, XCP Master, Modbus TCP and many more (the supported protocols might depend on the MATLAB release and separately purchased run-time licenses), 2x RS232 serial ports supporting baud rates up to 115.2 kbps, 4x Type-A and 1x Type-C USB ports supporting USB 3.1 Gen 2 (10 Gbps). Supported operating temperature range: 0 to 60°C. b) Performance-P3-CPU-4.0GHz-6Core 1 pcs. Replaces the 4-core 3.1 GHz Intel Core with a 6-core 4 GHz Intel Xeon processor, providing maximum performance and scalability to support your real-time application. 2. FPGA - Configurable Workflow: a) IO324-Performance 1 pcs. Simulink-programmable FPGA I/O module with an Artix 7 FPGA featuring 200k logic cells, 32 single-ended or 16 differential 16-bit analog inputs with a total of 4 AD converters supporting simultaneous acquisition of up to 4 AD channels, 8x 16-bit analog outputs and 32x 3.3 V/5 V (selectable) digital TTL I/O lines: up to 16 of the TTL lines can be reconfigured as 8 RS422/485 interfaces. In addition, this I/O module provides 64 LVTTL/32 LVDS 3.3 V rear I/O lines compatible with the IO3XX-2X signal conditioning rear plug-ins. Includes Speedgoat driver blockset for R2025a, I/O cable, and terminal board. Installed in Performance real-time target machine and fully tested. Rev.A2 b) IO3xx-21-Performance 1 pcs. This module converts the 3.3V rear LVCMOS I/O lines of the IO3xx into 56 x 3.3V/5V (software selectable) TTL I/O lines, and three additional (optional) I2C Master and/or Slave channel all accessible at the front of the enclosure. The module adds ESD and overvoltage protection, and provides 24mA current output support. Includes Speedgoat driver blockset for R2025a, I/O cable, and terminal board. Installed in Performance real-time target machine and fully tested. Rev.A1 c) IO324-Configuration Package 1 pcs. The I/O configuration package includes 8 configurations: 3 configurations one for RCP, HIL and Communication for the IO324 with 32 TTL lines and with the IO3xx-21. Part of the configuration can also be used when no IO3xx-21 rear plug-in is available 3 Configurations one for RCP, HIL and Communication for the IO324 with 16 TTL and 8 RS422/485 lines and with the IO3xx-22. Part of the configuration can also be used when no IO3xx-22 rear plug-in is available. 1 Configuration file for resolver emulation and measurement for the IO324 with 32 TTL lines and with the IO3xx-24. Part of the configuration can also be used when no IO3xx-24 rear plug-in is available. 1 Configuration file for 2x PWR-TPI6020 (Speedgoat Power Inverter) for the IO324 with 32 TTL lines. This configuration file can also be used for 1x PWR-TPI6020. 3. PROFINET: IO751-Performance 1 pcs. I/O module providing one PROFINET Controller port to communicate with other nodes. Includes Speedgoat driver blockset for R2025a. Installed in Performance real-time target machine and fully tested. 4. Speedgoat Fundamentals – Half-Day Web Credit: The Getting Started Jumpstart Web Session is an instructor-led course providing an overview of the capabilities of Simulink Real-Time and Speedgoat realtime target machines. This offer includes access to all tutorial material. For an in-depth instruction you may consider our full-day. Jumpstart Workshop offerings. Duration: Half day. Level: This workshop is tailored for beginners and intermediate users. Topics covered: Configuration of development PC and real-time target machine; Instrumentation; Optimization of real-time performance; Usage of selected I/O drivers. Prerequisites: Basic knowledge of MATLAB and Simulink. Format: Instructor-led via remote meeting Location: Online (live from Central European time zones). Maximum number of attendees: 5. 5. 5 Years Systems Software Maintenance and Support Service: Real-time target machines are typically used for many years for a range of projects leveraging the latest MathWorks software technology. The 5-Year Systems Software Maintenance and Support package provides added protection for your investment by extending the standard 1-Year Service to a total of five years. This package is available at a preferential rate if purchased at the same time as your system. The Systems Software Maintenance and Support Service provides access to the latest Speedgoat software compatible with future MathWorks software releases, and professional, technical phone, email and Web support. The Systems Software Maintenance and Support Service can be renewed after the initial period to ensure your investment is well protected over the years to come. |  |