E. TECHNICAL SPECIFICATION

TECHNICAL SPECIFICATION

FOR THE PROCUREMENT OF A IP-2 TYPE TRANSPORTation CONTAINER

Date ____________ No. _______________

Visaginas

# PROCUREMENT TYPE

Purchase of the Goods.

# OBJECTIVE

Purchase a standard 20-foot, IP-2 type certified transportation container (hereinafter – the Container), which will be used to transport radioactive waste by public road from the temporary storage facilities of the SE Ignalina Nuclear Power Plant (hereinafter – the Employer) to the near-surface repository for low- and intermediate-level short-lived radioactive waste.

The Container is classified as a product important to safety in accordance with the Nuclear Safety Requirements BSR 3.1.2-2017 “Pre-disposal Management of Radioactive Waste at Nuclear Facilities”.

# DESCRIPTION OF THE GOODS IR SCOPE OF SUPPLY

The Goods to be purchased comprise the following Product: 1 (one) container for the entire duration of the Contract.

Delivery of the Product shall be completed no later than 60 calendar days from the date of entry into force of the Contract.

The Supplier shall notify the Employer in writing of the Product’s delivery date no later than 3 calendar days prior to the delivery of the Product to the Employer’s warehouse. The Product shall only be delivered on working days, before 14:00. If the Supplier delivers the Product after 14:00, the SE INPP shall not be obliged to accept it on the day of delivery.

The Product shall comply with the following requirements:

Table 1

| No. | Technical requirements |
| --- | --- |
| 1. | Requirements for container weight and dimensions: |
| 1.1. | Loaded container weight – no more than 30,480 kg |
| 1.2. | External container dimensions shall be as follows: length – 6.058±0.01 m, height – 2.591±0.01 m, width – 2.438±0.01 m |
| 1.3. | The internal dimensions of the container must be as follows: length not less than 5.867 m, height not less than 2.350 m, width not less than 2.330 m |
| 1.4. | Internal container dimensions shall be as follows: length – no less than 5.867 m, height – no less than 2.350 m, width – no less than 2.330 m |
| 1.5. | Container dimensions and weight shall comply with ISO 668:2020 or an equivalent standard |
| 1.6. | The container shall comply with IP-2 safety standards established by the International Atomic Energy Agency (IAEA) in IAEA SSR-6, as well as the IAEA guidelines on the transport safety infrastructure for nuclear or radioactive materials, IAEA SSG-26 |
| 2. | Requirements for container design: |
| 2.1. | The container design and technical parameters shall comply with requirements of ISO 1496-1:2013 or an equivalent standard |
| 2.2. | The design shall include a removable solid steel lid (Open Hard Top) suitable for top loading |
| 2.3. | Doors shall be installed at the rear of the container and allow for attachment of a seal |
| 2.4. | A steel floor base and rings/rails for securing the load shall be installed |
| 2.5. | Forklift pockets shall be installed |
| 2.6. | Corner fittings shall comply with requirements of ISO 1161:2016 or an equivalent standard |
| 2.7 | A solid, leak-proof body shall be provided to prevent material spillage or evaporation |
| 2.8. | Sealing elements shall be resistant to environmental factors and transportation vibrations |
| 2.9. | All weld seams shall be continuous (without gaps). Step welding is prohibited |

The container shall have a factory marking. The marking shall include the following data:

  - Manufacturer’s name;

  - Model name (if applicable);

  - Container serial or factory number;

  - Maximum container gross weight;

  - Empty container weight.

# WARRANTY OBLIGATIONS

The Product shall be covered by a warranty of no less than 2 (two) years.

During the warranty period, the Supplier shall repair or replace the Product or parts thereof that does not meet the parameters specified in the operating documentation.

Container or parts thereof that have become radioactively contaminated shall not be returned.

The Product to be purchased, as well as the Product to be replaced during the warranty period, and parts thereof shall be new and unused.

# REQUIREMENTS FOR THE PROCUREMENT OF THE PRODUCT IMPORTANT TO SAFETY

The Supplier shall follow the listed below documents (actual versions):

  - Description of the Procedure for the Assessment of Suppliers and Sub-Suppliers Providing Products Important to Safety and the Control of Their Activities at the SE INPP, DVSta-1708-4 (https://altra.lt/apie-mus/administracine-informacija/planavimo-dokumentai/vidiniai-teises-aktai/);

  - Nuclear Safety Requirements BSR-1.4.1-2016 “Management System”, approved by Order No. 22.3-56 of the Head of VATESI on 21 June 2010 (revised by Order No. 22.3-13 of the Head of VATESI on 29 January 2016, Register of Legal Acts (TAR), No. 1891 of 1 February 2016);

  - Nuclear Safety Requirements BSR-3.1.2-2017 “Pre-disposal Management of Radioactive Waste at Nuclear Facilities”, approved by Order No. 22.3-120 of the Head of VATESI on 31 December 2010 (revised by Order No. 22.3-132 of the Head of VATESI on 31 June 2017).

The Supplier shall ensure that authorised employees of the SE INPP and/or authorised employees of VATESI are given the opportunity to participate in product testing and acceptance, as well as to carry out independent checks (such as audits, inspections, etc.) of the activities of the Supplier (and its sub-suppliers at all levels). Non-conformities identified during these checks must be eliminated in a timely manner, but not later than the completion of the contract. The Supplier must provide authorised employees of the the SE INPP and/or authorised employees of VATESI with the opportunity to assure whether the requirements of the procurement documents, the contract, and the management system documents are being implemented, i.e., upon request, the Supplier must provide the related documents.

The Supplier shall provide certificates of conformity for the components and materials of the equipment (goods).

The Supplier shall ensure the traceability of the materials used in the manufacture of the product important to safety (the Container) and provide documentary evidence thereof.

# DOCUMENTS TO BE SUBMITTED

Together with the offer, the Supplier shall provide documents prepared by the manufacturer proving that the proposed Product meets all the requirements specified in this Technical Specification.

Together with the delivered Product, the Supplier shall provide the SE INPP with the following documents and their electronic copies (in PDF format) in Lithuanian or English:

  - Certificates of conformity with ISO 668:2020, ISO 1496:2013, and ISO 1161:2016, or equivalent standards;

  - Certificate of Conformity issued by a competent authority, confirming that the container complies with IP-2 safety standards established by the International Atomic Energy Agency (IAEA) in IAEA SSR-6, as well as the IAEA guidelines on the transport safety infrastructure for nuclear or radioactive materials, IAEA SSG-26;

  - Along with the Product, the Supplier shall provide container operating documentation in Lithuanian and/or English, including the following information:

  - Technical data;

  - Operating instructions;

  - Container drawing with specified dimensions;

  - Maintenance instructions.

# PACKAGING AND TRANSPORTATION

The Supplier shall pack the container in such a way that ensures its protection against damage during transportation.

# OTHER REQUIREMENTS

The goods, in accordance with the Order No. D1-508 of the Minister of the Environment of the Republic of Lithuania of 28 June 2011, approved by Order No. D1-508, Section 4.4.1. of the Procedure for the Application of Environmental Protection Criteria in Green Procurement, are purchased as an environmental and environmentally friendly product, which falls within the indicative list of environmental and environmentally friendly goods and services in accordance with Commission Implementing Regulation (EU) 2015/2174 of 24 November 2015 on the indicative list of environmental and environmentally friendly goods and services, the format for the transmission of data for European environmental economic accounts and the conditions, structure and periodicity of quality reports in accordance with Regulation (EU) No. 691/2011 of the European Parliament and of the Council on European environmental economic accounts. The purchased product falls within the categories – Lead containers for radioactive waste.