This is translation from the Lithuanian language.

In case of any discrepancies between

Lithuanian and English texts,

the Lithuanian version prevails.

Pollution incident response and Search And Rescue vessel

Technical specification

GENERAL

The technical specifications (hereinafter referred to as the Specifications) are intended to define the essential requirements for the implementation of the construction project for  a pollution incident response and search and rescue vessel (hereinafter referred to as the Vessel).

The Vessel shall be constructed and delivered to the customer, the Lithuanian Armed Forces, which is implementing the project together with the Environmental Projects Management Agency under the Ministry of Environment of the Republic of Lithuania, in accordance with the project prepared on the basis of this Specification.

Taking into account the functional purpose(s) of the Ship, it must comply with the ship class notations listed below, which must be verified and certified by the classification society(ies) of the EU and/or NATO countries. The vessel must comply with and have certificates for the specified or equivalent vessel class notations or higher:

| Lloyd's Register (or equivalent) | Meaning (in detail) |
| --- | --- |
| 100A1 | Hull class: "100" – built under the supervision of LR, "A" – commercial ship, "1" – seaworthy. |
| Fire Fighting Ship 1 + water spray | Firefighting ship according to FiFi-1 (2×1200 m³/h pumps, two monitors) with additional deck water spray protection. |
| Oil Recovery Duties | Ship adapted for oil recovery operations |
| 1D ice class | Ice class according to LR |
| LMC | Lloyd’s Machinery Certificate – machinery class complying with LR rules. |
| UMS | Unattended Machinery Space – engine room can operate without a permanent watch, controlled from a central station/bridge. |
| DP 1 | Dynamic Positioning 1 |
| NAV1 | Redundancy of top-level navigation equipment. |

The supplier, in accordance with or exceeding the requirements specified in the Specification, must provide the customer with:

- Using the services of a ship classification society and under its supervision, build and deliver the Vessel, having fully completed functional tests in accordance with the agreed and coordinated functional test programme, to the Klaipėda State Seaport (hereinafter referred to as the Klaipėda Port) a fully functional and fully equipped Vessel that complies with the Ship's class (100A1 according to the Republic of Lithuania or equivalent) and the requirements of classification societies.

- The supplier must provide a warranty from the official handover to the customer, immediately after delivery, until the first intermediate inspection, in accordance with the ship class requirements (two years after delivery), which must cover defective materials/components/systems, construction errors and workmanship defects. The warranty shall also apply to equipment supplied and installed by subcontractors. The supplier shall ensure that a response to the customer's claims during the warranty period is provided within 5 working days. The supplier must take action to resolve the claim no later than four weeks from the date of submission of the claim. The supplier must take immediate action to resolve complaints and eliminate defects related to the safe operation of the ship or other major defects or deficiencies that may affect the ship's compliance with the shipbuilding standards of classification societies.

- The supplier must provide all documentation related to the project (drawings, diagrams, user and detailed maintenance instructions containing detailed service information, tools, equipment, etc. in Lithuanian or English, in paper manuals and easily readable electronic media). The supplier must use SI (International System of Units) units of measurement.

- All costs related to the vessel and its operation until the final transfer of the vessel to the customer shall be covered by the Supplier, including, but not limited to:

4.1 vessel crew costs (including additional crew required for sea trials or vessel relocation);

4.2 the costs of supplying, travelling and accommodating the ship's crew appointed by Lithuania, if sea or other trials are carried out outside the territory of Lithuania;

4.3 fuel, lubricants and other consumables required for testing and delivery.

- All costs related to the training of the crew appointed by the customer shall be covered by the Supplier. This includes travel, accommodation, meals, per diems, insurance, training materials and other necessary measures.

- All costs related to the transport, delivery to the shipbuilding site, installation, testing and materials of the equipment intended for the ship shall be covered by the Supplier.

- The ship's crew must be fully trained to a level that ensures the proper and safe functioning of the Vessel. Crew training periods must take place before the final handover of the Vessel to the customer.

- Prior to the commencement of the ship crew training, the Supplier shall submit a visualised detailed digital 3D ship design.

- The supplier must submit a complete planned project implementation schedule for approval, including the periods for the performance of the main works, certification and planned crew training, within two months of the date of signing the contract.

GENERAL REQUIREMENTS

- The ship design on the basis of which the ship will be built must reflect the entire life cycle of the ship (from design to disposal), assuming that the nominal useful life will be 30 (thirty) years.

- The vessel must be built and delivered to the customer no later than 32 months from the date of signing the purchase contract.

- The vessel must be built in accordance with the shipbuilding standards of one classification society chosen by the Supplier and must fully comply with the requirements of these technical specifications.  The ship's architecture, hull, equipment and machinery, including pollution collection, neutralisation and search and rescue equipment, must comply with the standards and requirements applicable to the ship's class.

- The materials, technologies, equipment and working methods used in the shipbuilding process must comply with the applicable environmental and energy efficiency requirements of the European Union and the Republic of Lithuania, ensuring minimal impact on the environment.

- The Supplier must ensure that technical support for the ship (spare parts and equipment, as well as software) will be available on the market for a period of not less than 10 (ten) years.

- The Supplier shall be responsible for the quality of the equipment, tools, mechanisms and other integral functional elements of the vessel not mentioned herein.

- The vessel must be delivered together with the equipment and materials described in these specifications, fully installed and tested. The vessel must be delivered together with all the equipment necessary for the operation of the vessel and to meet the operational requirements specified in these specifications. At the time of delivery, all systems must be filled with the necessary oils and lubricants. The supplier shall supply the ship with fuel, lubricants and hydraulic oils necessary for testing the ship's systems and running trials. Fuel residues in storage and service tanks, hydraulic oils and lubricants shall be left on board at the time of delivery. A so-called "turnkey solution".

- The concept of the layout of sections/premises, installation, equipment layout and other features shall be agreed in writing before the final design solutions are presented, i.e. before the construction of the Vessel begins.

- The Supplier must ensure that no circumstances specified in Article 45(2)(1)of the Public Procurement Law of the Republic of Lithuania (hereinafter referred to as the PPL) arise during the conclusion and performance of the Contract. The Customer shall have the right at any time to request the Supplier to submit the supporting documents specified in Article 51(12) of the PPL to prove that the conditions specified in Article 45(2)(1)of the PPL do not apply.

- The Supplier must ensure that the main parts of the ship (hull, main and auxiliary engines, navigation, communication and control equipment, propulsion mechanisms, crane and all other equipment installed on the ship) and components are not manufactured in the countries or territories specified in Article 92(14) and (15) of the Public Procurement Law, as amended.

- The supplier must ensure that, prior to delivery of the ship, all ship information technology (IT) systems, data transmission networks, as well as the ship's control, automation, navigation and other operational technology (OT) systems are designed, installed and tested in accordance with the cyber security requirements applied by the IACS (International Association of Classification Societies): UR E26 and E27, and comply with the cyber security class or notation issued by it (Lloyd's Register Cyber Security, DNV Cyber Secure or equivalent).

- Cyber security solutions and procedures must also comply with the provisions of International Maritime Organisation (IMO) Resolution MSC.428(98) and Guidelines MSC-FAL.1/Circ.3, ensuring the integration of cyber risk management into the ship's safety management system, network segmentation, prevention of unauthorised access, secure software update mechanisms, incident management processes and other requirements set out in these documents.

- The supplier must submit a final cyber security compliance certificate or classification society approval prior to ship acceptance and final settlement.

- Thermal, fire and sound insulation shall be of the mineral wool type, of suitable thickness and density. External partitions in accommodation and service spaces shall be covered with thermal insulation (mineral wool or equivalent) on all steel (or equivalent) surfaces, including the frame, where required by class. Partitions in accommodation spaces, the bridge and the mess room shall be covered with marine-grade multi-layer panels (mineral wool filling or equivalent). The bulkheads, ceilings of all cabins, bridge and mess room shall be constructed of sound-absorbing materials to minimise noise in accordance with the noise levels required in each area. Ventilation ducts (including air intakes) in the ship's living and service areas must be properly insulated to prevent condensation and reduce noise transmission.

I FUNCTIONAL PURPOSE OF THE SHIP, BASIC REQUIREMENTS

The ship will be owned by the Lithuanian Armed Forces and will be used for:

| Primary purpose | To respond to pollution incidents in the Lithuanian Republic's area of responsibility at sea or to provide assistance to other countries in accordance with international agreements. Pollution incidents are eliminated by: determining the extent and preliminary amount of spilled oil and/or its products on the water surface, collecting spilled oil and/or its products, storing it on board, spraying oil and/or its products with neutralising, dispersing and sinking agents. Search and rescue operations are carried out in the Lithuanian area of responsibility at sea or in providing assistance to other countries in accordance with international agreements. Search and rescue operations are carried out by: determining the location of victims, rescuing victims from the water, transferring victims to the ship, providing medical assistance on board the ship, evacuating victims from the ship with the help of aircraft and/or other ships, transporting victims to port by ship. |
| --- | --- |
| Secondary purpose | Firefighting in the Port of Klaipėda, in the Republic of Lithuania's area of responsibility at sea, or providing assistance to other countries in accordance with international agreements. Towing ships in distress. |

Key vessel data:

| Vessel operating time | At least 30 years from the date of delivery of the vessel to the customer. |
| --- | --- |
| Overall length (LOA) | No more than 80 metres |
| Vessel width (B) | Not less than 10 metres Not more than 15 metres |
| Draught (maximum) | 5.0 |
| Speed | 18 knots at SS4 (Sea State, hereinafter referred to as SS with the corresponding number) |
| Autonomy | Not less than 2,500 nautical miles at a speed of 10 knots, with 14 days' full provisions for the entire crew |
| Sailing area | Baltic Sea region |
| Cargo (liquid, packaged, other/weight, volume, location) | Fuel – the amount of fuel stored in the ship's tanks must ensure a period of operation at sea of not less than 14 days at nominal mode, reaching a residual fuel supply of not less than 30%. Fresh water – the amount of fresh water stored in the ship's tanks must ensure a period of operation at sea of not less than 7 days, achieving a residual fresh water reserve of not less than 30%. Without additional fresh water supplies, the ship must be able to operate at sea for at least 10 days using an integrated seawater desalination and filtration system. At least 100 litres of water per day for each crew member, providing for additional fresh water filtration systems for decontamination of bacteria before consumption. Provisions – a quantity ensuring the ship's autonomy for at least 14 days of operation at sea, achieving a residual dry provision stock of not less than 30%. Provisions must be stored in installed freezers, refrigerators and provision stores. Dispersants – intended for breaking down petroleum products. Stored in stationary/mobile containers on the ship. At least 400 litres of this substance must be safely stored on the ship. Firefighting foam agent – used for the ship's stationary fire extinguishing systems, used to extinguish fires on the ship itself and to extinguish fires on other ships. Stored in stationary/mobile containers on the ship. The amount of agent must be sufficient for at least 30 minutes of extinguishing another ship using all stationary fire-fighting equipment on the ship at full capacity. Collected pollutants – at least 100 tonnes. Stored in specially designated heated ship tanks. Other – stored in mobile containers on board the ship. There must be a designated place for storing mobile containers for other liquids not mentioned above (not classified as hazardous substances) that are necessary to ensure the safe operation of the ship for up to 14 days. |
| Additional cargo (liquid, packaged, other/weight, volume, location) | Additional containers (at least six 20-foot containers, each with a capacity of at least 10 tonnes (excluding the weight of the container itself)), other equipment. On the open deck. The container holders on the deck must be suitable for both 20-foot and 10-foot containers (three positions). There must be an electrical connection at the container storage locations. If the container securing positions on the ship are located in places where it is not possible to load them into the designated positions with the help of an external crane and additional pushing/moving equipment is required, this equipment must be included in the general equipment package of the ship. |
| People on board | Crew – 24 people. Additional temporary crew – 12 people (these are additional specialists on board for temporary accommodation according to operational needs). Survivors – up to 50. |
| Premises | Work – work premises must be ergonomic and fully equipped. More detailed information is provided in other sections of the Specification. Living quarters – the premises must be ergonomic and fully equipped. More detailed information is provided in other sections of the Specification. Non-residential – the premises must be ergonomic and fully equipped. More detailed information is provided in other sections of the Specification. |

Vessel operating environment:

| - Meteorological conditions |  |
| --- | --- |
| Maximum wind speed | 25 m/s |
| Precipitation | 50 mm/h |
| Air temperature max | 40º C |
| Minimum air temperature | - 25º C (for outdoor installation) |
| Air humidity | 80-95 |
| Visibility | Particular attention should be paid to night-time operations |
| Sun intensity (UV spectrum) | 8 maximum |
| Water flora and fauna | Algae during the warm season. |
| - Water surface |  |
| Maximum wave height | Full ship operation up to SS5, transit up to SS6 and ship survivability and controllability up to SS7 |
| Waves washing over the ship's deck(s) | Rare occurrence |
| Ice | Ice class at least 1D according to Lloyd's Register (or equivalent). |
| Water quality | Normal, except when pollution incident response operations are being carried out. |
| - Underwater environment |  |
| Water salinity | 6–8 ‰ |
| Water density | 1025 kg/m³ |
| Water temperature | From 0º to 25º C |
| - Geotechnical environment |  |
| Features of the Port of Klaipėda | The ship is moored at its designated location in the Port of Klaipėda, at berths 34/35 or at other berths in the Port of Klaipėda. |
| - Other environmental factors (man-made) |  |
| Mooring speed (max) | 4 knots. |
| Method of launching the ship | Slipway, slip, dock, semi-submersible barge, etc. |
| Noise and vibration (if exceeding permissible limits in residential and working environments) | It is expected that noise and vibration will not exceed permissible limits. |

Basic operating provisions for the vessel:

| Subordination of the ship | The ship will belong to the Lithuanian Armed Forces Naval Forces |
| --- | --- |
| Applicable restrictions | Cargo – not applicable to cargo related to the ship's functions; Loading – not applicable to loading methods related to the ship's design and functions; Loads – not applicable if this does not significantly alter the ship's stability, buoyancy, manoeuvrability or structural characteristics. |
| Specific requirements for the ship according to its purpose | Taking into account the purpose of the ship, the safety of the ship, its crew, passengers and victims must be ensured by selecting/adapting the necessary technical solutions, equipment and measures for the ship. |
| Operations with aircraft | The main ship operations are planned to be carried out with helicopters, such as receiving aircraft on the helicopter deck, receiving/transferring cargo, passengers and casualties. More detailed information is provided in other sections of the Specification. |
| Operations with the Ship's boats | The main ship operations with boats are expected to involve pollution collection operations, transporting casualties to/from the ship, transporting passengers, etc. The boats must be selected and equipped in such a way that the ship can operate safely and fully for its intended purpose. More detailed information is provided in other sections of the Specification. |
| Anchoring/mooring | Two anchors must be installed in the anchor lockers at the front of the vessel. The vessel must have all the necessary anchoring equipment to ensure that it can be safely moored at the anchorage. |
| Towing (not applicable in emergency situations) | The vessel is designed to moor independently (without tugs), navigate through ports, straits and canals. The ship must have the necessary solutions to ensure that it can be towed and tow other ships with a displacement of at least 5,000 tonnes. Towing force ( Bollard pull) or emergency towing hook certification – not less than 20T. |
| Citadel | The ship's crew must be protected when extinguishing fires on other ships and collecting spilled oil and oil products. Operating conditions are created by using the ship's ventilation (its configuration) and the Universal Ship Washing System (UVAS) accordingly. For operations in hazardous and potentially toxic environments, at least one separate living citadel must be provided on board. Overpressure in the citadel spaces must be monitored. Pressure indicators for each zone must be installed in the operations centre. If the pressure in any of the zones falls below the specified parameters, an audible and visual alarm must be activated. Air pressure monitoring sensors shall be located in the citadel zones in accordance with the heating, ventilation and air conditioning (HVAC) system plan. The outer boundary of any citadel must be sealed to ensure a toxin-free environment within the citadel. The citadel is a closed area with a controlled safe environment and: - maintained at positive pressure to prevent contamination from the outside environment, - a constant supply of fresh filtered air through filter stations. Windows in citadel rooms must be designed to withstand a shock wave at the expected positive pressure. Entrances to the citadel should ensure that the citadel remains airtight under all conditions. Entrances to the citadel must have air locks to ensure that positive pressure is maintained inside. The air lock must consist of two doors located at a distance from each other to allow access and transport of stretchers with victims and medical personnel. |
| Gas detection system | One stationary gas detection system is installed. Not less than eight (8) lower pressure (LEL) sensors shall be located to protect the living quarters, engine room and deck. Signals are displayed via the alarm and monitoring system (AMS). When gas is detected, the alarm is activated via the AMS. There is one (1) common gas detector for all sampling points. Gas samples are taken from each sampling point in sequence according to the sampling sequence and transported to the gas detector via a sampling tube. The sampling time is adjustable. In addition, two (2) portable hand-held gas detectors for O2, H2S, 0-100% lower explosive limit (LEL) hydrocarbon gas detection are provided. The layout of the equipment on the vessel must be coordinated with the customer. |
| Ship hull maintenance | Ship hull maintenance is planned to be carried out during periodic docking (optimum time 2 times in 5 years) and by inspecting the underwater part (with the help of divers). |
| Buoyancy and stability | The stability and buoyancy of the ship must not be compromised when performing the main functions of the ship. |
| Ship propulsion complex, power plant, additional equipment and mechanisms. | Basic provisions: The ship's engineering systems must be such that they can fully ensure the functionality of the ship. They shall be controlled remotely and using a local control panel, but a control system shall be installed that allows the equipment to be controlled in real time from a single location, with the control location indicated on the relevant control panels. Critical ship equipment (propulsion complex, rudders, winches, cranes, spiles, navigation equipment, fire detection and extinguishing system equipment, de-icing system equipment, emergency power supply equipment ) must be capable of being controlled in an emergency, i.e. at their location without external control and/or monitoring systems. Critical equipment must be duplicated, i.e. solutions must be in place to compensate for non-functional critical equipment. All ship systems and the locations where they are located must be integrated into the ship's monitoring and control system. Propulsion complex: The propulsion complex of the Ship must be such that it fully complies with the functional purpose of the Ship. The Ship uses marine diesel fuel (Marine Gas Oil) that complies with ISO 8217 or equivalent requirements. Control (manoeuvring system): The ship's control (manoeuvring system) must be such that it fully complies with the conditions of the ship's functional purpose. Additional equipment and mechanisms: The ship's additional equipment and mechanisms must be such that they fully comply with the conditions of the ship's functional purpose. Electrical power plant: The ship's power plant shall be such that it fully complies with the conditions of the ship's functional purpose. Reactive power compensation devices shall be installed. High-voltage system: Voltage 440V Low voltage system: 220V/115V/24V Control systems: Control systems must be designed to ensure a continuous supply of the necessary electrical power. |
| Fire safety | Essential fire safety principles must be maintained, taking into account the ship's functions and operating conditions. The ship must be equipped with effective fire detection, warning and extinguishing systems and a full set of fire-fighting equipment. The ship must have at least six (two for immediate use on board, four for assisting another ship) complete sets of firefighting equipment in accordance with SOLAS II-2/10.10 requirements, and provision must be made for the proper storage of these items on board: - Protective clothing set - Non-flammable, heat-reflective and water-resistant outer clothing (jacket + trousers). - Compliance with EN 469 or equivalent standard. - Helmet - With face shield, back of head protection, suitable for use with SCBA. - Compliance with EN 443 or equivalent standard. - Fire-resistant gloves - Resistant to heat, flame and mechanical damage. - Compliance with EN 659 or equivalent standard. - Fire-resistant boots - With metal toe caps and toe protection. - Compliance with EN 15090 or equivalent standard. - Self-contained breathing apparatus (SCBA) - Min. 30 min. operating time, pressure gauge, air reserve alarm, replaceable cylinder. - Compliance with EN 137 or equivalent standard or SOLAS requirements. - Personal rescue torch - Fireproof, EX/ATEX certified, with attachment to helmet or clothing. - Communication devices (personal) - Can be integrated into helmet headsets or handheld radio stations (VHF/UHF), EX certified, with hands-free capability. |
| Evacuation, rescue | Effective and safe means of evacuation and rescue must be installed on board. |
| Communications | Communication and information systems equipment must be designed and installed on board: - GMDSS equipment adapted to GMDSS A1, A2, A3 areas. - Internal ship communication system (intercom). - The ship must be equipped with very high frequency radio communication equipment capable of communicating with aircraft (airband). - In addition to the GMDSS requirements, the ship must be equipped with at least 3 stationary very high frequency radio stations. - The ship must be equipped with at least 4 waterproof handheld very high frequency stations. - The ship must be equipped with at least 12 portable radio stations for communication between crew members. - The ship must be equipped with a direction finder for distress signals integrated into ECDIS. Other communication systems/devices: - The ship must have TCP/IP data reception and transmission capability in all areas of operation, ensuring both GSM (4G/5G) and Low Earth Orbit (LEO) data transmission links. - The ship must have an internal network (LAN) ensuring access in the required premises; - The ship must have a CCTV system with continuous surveillance cameras with recording capability. The layout of all communication equipment on board must be agreed with the Customer. |
| Navigation | Environmental factors: The ship must have a system/equipment and measures in place to ensure the safe navigation of the ship in hazardous and potentially toxic environments. The ship must be equipped with systems/equipment and measures that allow it to perform its functions safely and effectively at any time of the day, even in adverse weather conditions. The ship must have systems/equipment and measures in place to ensure a suitable microclimate on board, regulated according to the ambient temperature. Navigation safety. The ship must be equipped with systems/equipment that ensure safe navigation in the event of failure of the ship's main navigation systems. The planned (nominal) speed of the ship is 14 knots. The tactical diameter of the vessel must not exceed 4.5 times the length of the vessel. The dynamic positioning system must operate autonomously, keeping the ship at a fixed point. Bridge workstations: Planned workstations on the bridge not related to navigation: Receiving of helicopter (also including Analogue indicators for Roll, Pitch and Heading), pollution incident response operations, Search and rescue, Assistance to other vessels, Coordination of operations. Additional workstations required/necessary/planned on the bridge: Command of the fight for the ship's survivability, Coordination of evacuation. Required observation/monitoring positions on the bridge related to pollution collection operations, supply reception/transfer operations, anchoring/mooring operations, other operations not listed that require a specific observation/monitoring angle and position on the bridge. Ability to control the vessel from the "vessel mooring control position" (bridge wings) during stock receipt/transfer operations. Ability to use ship manoeuvring equipment (e.g. rudders, horizontal active rudders/side thrusters, propulsion) from the "ship mooring control position" (bridge wings). Ship course control requirements: The ship must be equipped with all necessary sensors to ensure safe navigation and prevent accidents. It must be possible to communicate from the bridge with: All designated main, alternative and emergency workstations on the vessel (including operator workstations) and make announcements utilising public announcement system. The bridge must be equipped with an Integrated Navigation Bridge System (INBS). Alternative ship control stations: Requirements for an alternative ship control station: The designated method for monitoring the RAS location, mooring/berthing location and other locations from the alternative ship control station; Ability to control the ship's manoeuvring equipment; Ability to control the dynamic positioning system; Ability to activate distress signal. Automatic ship course control must be installed. A means of collecting data from the integrated navigation bridge system must be provided. Monitoring of the marine environment is ensured by radar and visually. Ship identification data is sent and data from other ships is received using the Automatic Identification System (AIS). Requirements for the installation of an emergency ship control station: When the bridge and alternative ship control stations are not functioning, ship control must be ensured from the ship control centre (engineering centre), with separate control of the propulsion complex (from the ship control centre and/or the engine room) and separate control of the rudder from the ship control centre and/or the wheelhouse. It must be ensured that from the Emergency Ship Control Station it is possible to: – direct and coordinate damage control / the fight for the ship’s survivability; – communicate with all designated primary, alternative, and emergency workstations on board the vessel (including operator workstations) and make announcements via the ship’s general public announcement system. Requirements for ship safety systems: Ship safety systems must provide information on: Water level in ship compartments; Fire hazards on board the ship; Failures of critical ship equipment (propulsion, steering, firefighting, navigation, safety and lighting equipment systems). Main ship control and command stations: Ship bridge (including alternative control stations); Ship control centre (engine room). The ship must be equipped with navigation lights that comply with: - the 1972 International Regulations for Preventing Collisions at Sea (COLREG-72), - SOLAS Convention requirements, - Classification society rules. Requirements: Navigation lights must be controlled from the central control panel on the bridge. An indication and alarm for lamp or circuit failure must be provided. Navigation lights must be connected to an emergency power supply circuit to ensure uninterrupted operation in the event of a failure of the main power supply. The brightness of navigation lights must comply with COLREG-72 requirements. The vessel must have a set of lights and signals that allows it to display all signals specified in COLREG in accordance with its operational purpose, including the signals "Vessel out of control" and "Vessel with restricted manoeuvrability" and all signal flags in accordance with INTERCO (International Code of Signals). In addition, for signalling purposes during daylight hours, all necessary daytime signals must be displayed in accordance with COLREG-72 requirements, together with navigation lights. |
| Cargo lifting crane | The ship must be equipped with a folding telescopic boom type cargo lifting crane (crane used in port). Lifting capacity of at least 2 tonnes with the load at a distance of at least 3 metres from the ship's side. The lifting rope must be of sufficient length to allow the lifting hook to reach the water surface in any position of the crane boom ( ). The lifting hook must be suitable for lifting cargoes while the ship is operating in sea conditions. It must be possible to control it locally and remotely. The crane base must be reinforced to withstand static and dynamic loads. The crane must have an integrated hydraulic system, and all crane parts must be adapted and resistant to seawater. The crane's hydraulic system must be heated to function properly at low temperatures (down to -25°C). |
| Integrated ship management and control system | The ship must be equipped with an integrated management and control system that combines the main functional systems of the ship into a single information and control space. The system must ensure the monitoring and control of the ship's systems: monitoring of the main and emergency electrical networks, load management, generator performance and automatic load shedding. The system must integrate navigation data: information from radars, GPS, AIS, gyroscopes, logs, echo sounders and other sensors required by the specification. The system must ensure the monitoring of auxiliary systems: pumps, ventilation, ballast systems, fuel systems, fire detection and other subsystems specified in the Specification. Alarms, warnings and faults must be recorded centrally, with event sequencing, prioritisation and diagnostics. The operations centre must be equipped with a central control console that allows monitoring and control of the main systems. The system must have a data recording function that stores at least 30 days of system operation history. |
| Integrated bridge navigation system | The ship's bridge must be designed as an integrated navigation and ship control environment. The navigation interface must combine: at least two independent radars, two ECDIS terminals, AIS, autopilot, gyroscopes, GPS, logs, wind and depth sensors. The bridge must have centralised control, allowing the selection of sensor sources, mode changes, autopilot and course control. All critical navigation elements must be duplicated (in accordance with NAV1 or an equivalent standard). The bridge must have an integrated navigation and technical alarm system. Solutions for night mode screen and workstation lighting must be provided. |
| Ship scheduled maintenance system | A computerised scheduled maintenance system must be installed on the ship. The system must ensure a register of all ship equipment and components with technical documentation. Maintenance schedules must be created and administered based on manufacturer recommendations and critical system priorities. The system must automatically generate reminders of scheduled maintenance based on calendar intervals or operating hours. The fault log must record the nature of the fault, the actions taken, the persons responsible and the dates. The system must generate monthly and quarterly maintenance reports for the customer. |

Ship design and operational characteristics:

| II SHIP CONSTRUCTION |  |  |
| --- | --- | --- |
| No. | Requirement | Comments |
|  | The ship shall be designed and constructed in such a way as to ensure throughout its service life: |  |
|  | The integrity and watertightness of the hull; |  |
|  | Withstand all design loads; |  |
|  | Safe performance of duties by the ship's crew; |  |
|  | The safety of persons on board in emergency situations until the ship is abandoned; |  |
|  | The ability to assess the condition of the ship's structures, carry out repairs, and check/service/replace ship equipment with minimal intervention, both when the ship is in the water and when it is in dry dock (out of the water). |  |
|  | The ship must be designed and built in such a way that all materials are environmentally friendly, i.e. recyclable, but this must not affect the safety and operational capabilities of the ship. The vessel must be able to be disposed of and the materials recycled by a company engaged in the collection and recycling of secondary raw materials. |  |
|  | The materials used in shipbuilding must be such that they meet the design requirements of the ship and the operating environment of the ship: |  |
|  | The following materials may be used in shipbuilding: steel, aluminium, reinforced composite polymer fibre (its fabric). | A combination of these materials may be used for separate sections, compartments and rooms of the ship. |
|  | Materials or combinations of materials that are unsuitable for the operations intended for the ship's purpose, pose a threat to people and prevent safe evacuation from the ship may not be used for ship construction (sections, compartments, rooms). |  |
|  | The use of asbestos or materials containing asbestos, as well as other materials harmful to health, in shipbuilding is prohibited. |  |
|  | During the construction of the ship, a painting scheme must be drawn up for the coating of all structures with protective materials (paints, varnishes, etc.), detailing the selection of materials, surface preparation, surface coating, technological processes and their effective functional life. The painting scheme shall be agreed with the Customer before it is drawn up. | The selection of protective coatings for surfaces may be, and in certain cases must be, optimally utilised in conjunction with other protective measures, such as cathodic protection, etc. |
|  | The ship (on the mast) must have free space and the possibility of installing electrical wiring for the installation of additional equipment provided by the Customer, with an area of at least 1 m² , but not limited to this area (the preliminary weight of the equipment is estimated at 100 kg). |  |
| III BOAT FLOATABILITY, STABILITY, MANOEUVRABILITY |  |  |
| No. | Requirement | Comments |
|  | The characteristics of the vessel's buoyancy, stability, layout of main compartments, and distance from the structural waterline (WL) must ensure: |  |
|  | Appropriate design reserve buoyancy, taking into account possible emergency scenarios in the ship's operating environment; |  |
|  | Appropriate stability of the ship, protecting it from capsizing, taking into account possible emergency scenarios in the ship's operating environment; |  |
|  | Safe performance of tasks by the ship's crew; |  |
|  | The safety of persons on board in emergency situations and when abandoning the ship; |  |
|  | The characteristics of the ship must be such that the possible negative consequences arising from: The ship capsizing, caused by dynamic loads due to bad weather conditions and/or static loads caused by improper distribution of cargo on the ship and/or errors in the management of the ship; Loss of hull integrity (collision, grounding, etc.), negative impact and damage to the crew and equipment, are assessed in advance and reduced to an acceptable level. |  |
|  | The integrity of the ship is ensured by: |  |
|  | Watertight bulkheads and doors designed to protect undamaged ship compartments from water ingress; |  |
|  | The main deck of the ship, located above WL, which, when moving along the ship's diametrical plane, must ensure access to all ship compartments/sections using quick-closing/opening mechanisms for the hatches, and which must be equipped with means of communication. |  |
|  | The bulkheads of the engine room and other engineering spaces/compartments of the ship shall be watertight. |  |
|  | The fore and aft sections of the ship, from the keel and outer deck, shall be such that the remaining sections of the ship are protected in the event of a collision. |  |
|  | All technological hatches, portholes and other openings in the ship shall be watertight. |  |
|  | Damage to and/or loss of equipment attached to the hull of the vessel shall not affect the integrity of the vessel. |  |
|  | The ship's bilge pipes shall be protected against water ingress into the ship in the event of damage (breakage). |  |
|  | Ship openings: |  |
|  | Ship openings that must remain open must be protected against direct water ingress when the ship is operating under the intended conditions (worst-case scenario); |  |
|  | The number of openings below the main deck of the ship must be as small as possible, but sufficient to ensure the functionality of the ship; |  |
|  | Openings in watertight compartments shall be fitted with approved closing mechanisms; |  |
|  | Other openings not mentioned in subparagraph 9.1. shall have approved closures, the construction (coamings) of which on open decks shall be of sufficient height and strength to ensure the safe movement of crew members and protection against water ingress into compartments, withstand damage and ensure safe closure of the closure; |  |
|  | Air, ventilation or similar openings shall be provided only at/above the main deck of the ship; |  |
|  | Openings for moving parts passing through the main deck of the ship must have approved type seals; |  |
|  | Discharge openings located below the waterline must be of an approved type (must have a sufficient number of easily accessible and controllable opening closing mechanisms (valves). |  |
|  | Watertight (transverse) bulkhead doors: |  |
|  | Ensure the safe working conditions of the ship's personnel; |  |
|  | Must be easy to use and quick to close under all anticipated ship operating conditions; |  |
|  | When closed, they must ensure tightness as a single bulkhead structure; |  |
|  | Must ensure access to the compartment from any adjacent ship's space/compartment; |  |
|  | Must be as far away as possible from the most vulnerable areas of the ship; |  |
|  | They must be equipped with a "closed/open" indicator system linked to the ship's monitoring and control system; |  |
|  | When the doors are controlled electrically/hydraulically/pneumatically or by other means, they shall: be operated from both sides of the bulkhead using a local control panel and/or the ship's monitoring and control system (for closing only), regardless of the ship's trim and/or heel the door shall close within an acceptable time (not exceeding 160 seconds from the receipt of the signal). The shutters shall have two independent power sources to ensure power for three operating cycles in the event of a power failure. The use of the shutters shall be indicated by an audible and visual alarm. | There must be an emergency manual control mechanism for the damper. When it is used, other control methods must be inactive. |
|  | Technological cavities in the hull decks and bulkheads (for electrical cables, ventilation shafts, pipes, etc.): |  |
|  | Must ensure the integrity and watertightness (sealing) of the deck and bulkhead; |  |
|  | They must be easy to use and quick to close under all anticipated operating conditions of the ship. The air intake and exhaust vents of the ship's ventilation system must be remotely controlled. |  |
|  | Must not be located in the ship's forepeak and aftpeak compartments; | Except for justified design exceptions |
|  | Must be made of materials that are resistant to heat in the event of fire. |  |
|  | They must be approved. |  |
|  | Drainage system: |  |
|  | Designed to remove water accumulating in the lower compartments of the ship during normal operation. Its minimum capacity must be sufficient to remove the amount of water that accumulates. An emergency drainage system must also be provided for use in the event of fire/accident. |  |
|  | The ship shall be equipped with a water level monitoring system linked to the ship's monitoring and control system. |  |
| SHIP FLOATABILITY |  |  |
|  | The buoyancy of the ship shall fully meet the operating conditions of the ship and shall reduce the negative impact of flooding on the ship to an acceptable level during submersion. |  |
|  | A ship operating under the specified conditions must have sufficient freeboard to remain afloat in extreme conditions, thus ensuring that the people on board can be rescued. |  |
|  | The ship's draught markings must be on the hull of the ship, and the actual trim and heel of the ship must be visible according to the aforementioned markings. |  |
|  | The ship must be equipped with a liquid level measurement system in the tanks, which is used as the primary means of data acquisition. The secondary means of data acquisition is measurement using rulers or other means. |  |
| SHIP STABILITY |  |  |
|  | The ship must be designed and constructed so that: |  |
|  | It has sufficient righting moment to prevent the ship from capsizing; |  |
|  | It has sufficient characteristics to reduce the negative effects of transverse and longitudinal roll and pitch on the crew on board. |  |
| SAFETY OF PERSONS ON BOARD |  |  |
|  | The ship shall be designed and constructed in such a way that the persons on board can safely and effectively perform their duties, taking into account the specified operating conditions of the ship, with due consideration of complex hydro-meteorological conditions and providing for certain limitations on the functions of the crew when these conditions prevail, but without affecting the functionality of the ship. |  |
|  | Open decks of the ship must be equipped with measures (e.g. railings, guardrails, handrails) to protect people from falling (into the water or onto another deck). |  |
|  | All movement routes on the ship (on the outer decks and inside the ship) leading to functional and/or emergency areas must have measures (e.g. handrails) in place to ensure the safe movement of people in difficult hydrometeorological conditions. |  |
|  | All functional areas (workplaces) on board the ship must have measures (e.g. handrails, safety belts) to ensure the performance of normal functions in difficult hydrometeorological conditions. The ship's design must provide for safe access for the crew from both sides and a place for storing/securing the ship's gangway. The ship's equipment must include a gangway and internal removable steps (if required by the ship's design). |  |
|  | In the event of an accident, the vessel must have safe areas where people can safely gather and leave the vessel using the vessel's evacuation equipment. |  |
| VESSEL FLOATABILITY, STABILITY, MANOEUVRABILITY NOMENCLATURE |  |  |
|  | Data describing the ship's buoyancy, stability, manoeuvrability, evacuation and other relevant information must be systematised, approved and presented in the formats specified in Appendix No. 1. |  |
| IV SHIP PROPULSION, POWER, SYSTEMS, EQUIPMENT AND MECHANISMS (ENGINEERING SYSTEMS) |  |  |
| No. | Requirement | Comments |
|  | The ship's engineering systems must be: functional when the ship is operating under the intended conditions, pose no danger to persons on board, have optimal physical parameters, not reduce the ship's buoyancy and stability characteristics, be capable of starting to function without external assistance, in the event of a dead ship starting situation (applies to systems ensuring the safety of the ship), pose no fire hazard, ensure the safety of people on board until they reach evacuation points or until the threat is controlled, be easy to maintain/repair in accordance with the maintenance plan. |  |
|  | The ship's engineering systems must be constructed in such a way and from such materials that they are optimally suited to their purpose of operating continuously under the intended operating conditions of the ship, taking into account environmental and maintenance/repair requirements. |  |
|  | The risks associated with the ship's engineering systems must be properly assessed and reduced to an acceptable level: for people on board the ship, equipment integrated into the system and/or located near it, the ship, third parties and the environment. |  |
|  | Where technologically possible, three operating modes must be ensured in these engineering systems: normal, partial and emergency. |  |
|  | Engineering systems must be such that they remain functional and safe even if one of the system components fails or malfunctions. This is achieved by selecting particularly reliable system components and duplicating their functions, using reliable methods of isolating non-functional components. | User documentation is provided, describing operating modes (normal, partial, emergency), fault detection and elimination instructions, and maintenance/repair intervals and procedures. |
| PROPULSION |  |  |
|  | The propulsion complex must enable controlled navigation/manoeuvring of the vessel. |  |
|  | The functions of the propulsion complex must be duplicated. Measures must be provided for and implemented to enable the complex to continue operating in the event of failure of one or more (auxiliary) components of the complex. |  |
|  | The propulsion complex must be such that it does not pose a threat to the crew, does not adversely affect the functionality of the ship and/or other systems, and does not pose a danger when removed in the event of a failure. |  |
|  | Appropriate measures must be in place to enable the propulsion complex to become functional without external assistance, provided that no other systems are operational (dead ship starting). |  |
|  | The fuel supply system must be such that consumers are fully supplied with fuel without using a continuous supply from the ship's fuel tanks, but have a sufficient reserve of fuel of appropriate quality (e.g. service tanks). |  |
|  | Means of communication must be provided between the normal ship control location (bridge) and the emergency ship control location and any locations on the ship from which the ship's speed is/can be controlled. |  |
| SHIP CONTROL (MANOEUVRING) |  |  |
|  | The ship's steering systems shall be such as to meet the requirements specified in "Ship's manoeuvrability". |  |
|  | Ship control systems must be duplicated. Means must be provided and installed to enable the systems to continue to function in the event of failure of one or more (auxiliary) components of the complex without loss of ship controllability. |  |
|  | Measures must be provided to allow the failed ship control equipment to be mechanically locked in the required position. |  |
|  | Ship control must be ensured in several locations, depending on the ship's design. Each of these locations must have ship control system control and management panels with redundant functions in case of power supply failures. |  |
|  | Means of communication must be provided between the normal ship control location (bridge) and the emergency ship control location and any locations on the ship from which the ship's speed (propeller rotation direction/propeller pitch angle) is/can be controlled. |  |
|  | The ship's control systems shall be such as to ensure sufficient redundancy of power for their continuous operation and control, characterised by diversity. System failures shall not endanger the ship, other systems or persons on board. | User documentation shall be provided, describing the operating modes (normal, partial, emergency), instructions for fault detection and elimination, and maintenance/repair intervals and procedures. |
|  | Ship control systems must have adequate electrical protection to ensure the functionality of their control systems. |  |
| PIPING AND PRESSURE SYSTEMS |  |  |
|  | The ship's piping and pressure systems must be such that they do not pose a danger to the ship, the people on board or the adjacent systems. |  |
|  | The ship's piping and pressure systems must be such that the ingress of contaminants into them is limited by appropriate control measures, and means of removing contaminants must be provided. |  |
|  | The ship's piping and pressure systems and their components must fully comply with the ship's integrity requirements as defined in "Ship's buoyancy, stability and manoeuvrability" and must also be protected against the generation and accumulation of electrostatic charges. |  |
|  | The ship's piping and pressure systems must be equipped with pressure control measures – pressure relief valves that do not pose a threat to the ship, the people on board or nearby ships. |  |
|  | The ship must be equipped with means and equipment for filling breathing air cylinders. |  |
|  | The ship shall be equipped with a domestic sewage disposal and treatment system of sufficient capacity (from showers, toilets, and other sanitary facilities) with the necessary tanks, fittings, and equipment. | Must comply with the environmental protection requirements for the navigation area. |
|  | The ship shall be equipped with a system of sufficient capacity for the removal and treatment of oil-contaminated water (bilge water from the ship's holds) with the necessary tanks, fittings, and equipment. | Turi atitikti laivo aplinkos apsaugos keliamus reikalavimus plaukiojimo rajonui. |
| SHIP STABILISATION SYSTEM(S) |  |  |
|  | If necessary, the vessel shall be equipped with a stabilisation system(s) that reduces the vessel's sway to a level to fulfil listed operational and classification requirements. |  |
|  | The ship's stabilisation system(s) shall fully comply with the ship's integrity requirements as defined in "Ship's buoyancy, stability and manoeuvrability". |  |
|  | The stability requirements of the ship shall not be based on the ship's stabilisation system(s). |  |
|  | The ship's stabilisation system(s) shall be fully automatic. |  |
|  | The ship's stabilisation system(s) shall operate in conjunction with the ship's control and command systems and the ship's warning systems. |  |
|  | Measures shall be provided to allow the ship's stabilisation system(s) to be mechanically locked in the required position in the event of failure. |  |
|  | Failure of the ship's stabilisation system(s) and its components shall not adversely affect or endanger the ship's functions. |  |
| GENERATION AND DISTRIBUTION OF ELECTRICAL ENERGY ON BOARD |  |  |
|  | The ship shall be capable of generating and distributing sufficient electrical power to ensure the full operation of all ship and crew functions without the use of an emergency source of electrical power. |  |
|  | Ship systems that require an uninterrupted supply of electrical power shall be provided with the necessary electrical power in emergency situations. |  |
|  | Ship systems that require an uninterrupted supply of electricity must be supplied by electricity accumulators of sufficient capacity, operating for an optimal period of time (not less than the time required to restore the electricity supply from the emergency system and not less than 60 minutes) in the event of a power failure. | Appropriate spaces on the ship, optimally located, must be provided for electrical energy storage devices, ensuring suitable conditions for their operation and proper maintenance/servicing. |
|  | The most optimal combination of generators capable of operating synchronously must be selected for the ship's power generation, switching on automatically when the demand for electrical power increases, thus protecting the power distribution system from overload. The power generation and distribution system must be duplicated. |  |
|  | Appropriate primary and local protection systems must be installed without affecting the functionality of the ship. |  |
|  | It must be possible to supply the vessel with electricity from shore infrastructure. | The Ship shall be equipped with electrical cables of not less than 50 m in length to ensure the full electrical power required for the Ship in port mode. |
|  | Appropriate measures must be in place to ensure that critical ship systems can become functional without external assistance, provided that no system is operational (dead ship starting) for at least 10 minutes. |  |
|  | Provision must be made (reserved capacity) for connecting additional electrical consumers on board. |  |
|  | Appropriate electrical power conversion equipment must be installed on board. |  |
|  | The ship must have emergency generator(s) to ensure the functionality of the necessary ship systems. |  |
|  | The ship's electrical network (power parameters, grounding circuit) must be such that it safely reaches consumers and does not pose a threat to the integrity of the ship, its systems, the people on board or the equipment. |  |
|  | The ship's electrical distribution boards, their sizes, capacities and number must meet the functional requirements of the ship. |  |
|  | The main electrical network of the ship and the emergency electrical network of the ship must be independent of each other. |  |
|  | The use of the ship's electrical networks (main and emergency) must be properly configured and allow for the disconnection and isolation of electrical consumers. |  |
|  | The part of the ship's electrical network intended for portable electrical equipment must have appropriate protective measures. |  |
|  | Ship compartments containing electrical distribution boards must be equipped with fixed means of communication. |  |
|  | The portable emergency power supply system must not pose any additional risks to people, systems, equipment or the ship. |  |
|  | The ship's lighting system must be functional under normal and emergency conditions. |  |
|  | The ship's lighting system is divided into: primary – stationary, used under normal conditions and ensuring the required level of lighting on all decks, compartments/rooms of the ship; emergency – stationary, used when the primary system is not working, the level of lighting may be reduced; also designed to illuminate escape routes and assembly points; operational – stationary, operating on the basis of the primary and secondary systems, with adjustable intensity and different tones (spectrum); portable – non-stationary, portable, used where additional lighting is required. | Lighting fixtures on board must be such that they do not pose a danger to people, systems or equipment by emitting heat or being leaky, depending on the room in which they are installed or used (portable lighting fixtures). |
|  | The lighting system shall be such that if one light source in the circuit fails, the remaining ones shall continue to function (parallel power supply to the system components). |  |
|  | The intensity of the lighting must be adjustable separately in each deck/section/room and the primary lighting system must be used. |  |
|  | Navigation lights are described in the Navigation section. |  |
|  | All electrical equipment on board must be adequately protected against damage from improper use and injury to persons on board. |  |
|  | Uninsulated metal (electrically conductive) parts (housings) of equipment/mechanisms that do not carry electrical current must be earthed. |  |
|  | Control measures must be installed in electrical distribution boards to detect electrical leakage. |  |
|  | Ship equipment/mechanisms must be sufficiently sealed to prevent the ingress of dust, liquids and gases. |  |
|  | The ship must have appropriate measures in place, properly arranged, to protect against the risks posed by electrical overcurrent, mechanical and/or electrical overloads. |  |
|  | All electrical equipment/mechanisms and electrical cables must be adequately protected against mechanical damage. |  |
|  | Electrical cables must be non-flammable. |  |
|  | Main electrical distribution boards, connections, etc. must be protected against unauthorised access (locks or similar must be used). |  |
|  | The ship must have adequate protection against lightning strikes. |  |
|  | The ship must have alternative cooling methods for ship systems/equipment in case of failure of the main ship cooling system (ventilation). |  |
|  | Appropriate measures must be in place to reduce radiation emitted by systems/equipment/mechanisms/electrical cables. |  |
| CONTROL AND MANAGEMENT OF SHIP ENGINEERING SYSTEMS (CONTROL SYSTEM) |  |  |
|  | The control system covers the control of ship systems, including automatic systems, and control from designated locations on board the ship. The control system must not affect the technical parameters of the controlled systems. | It must be possible to switch off automatic or remotely controlled systems. |
|  | Control and actual management of systems must be possible from a single control location, clearly indicated on all control consoles. The transfer of control must be smooth and must not affect the operation of the systems, with notification of this via audiovisual means installed on all control consoles. | Provision shall be made for the possibility of forcibly taking over control from the console in actual control in the event of its failure, with the failure of the console being indicated by audiovisual means. Measures shall be taken to ensure that, in the event of a console failure, all control parameters remain unchanged and do not affect the functioning of the ship's systems. |
|  | Operator actions and feedback must be clearly and comprehensively displayed on all consoles. The status of automatic systems must also be displayed. |  |
|  | The control and management system must indicate if the operating systems are not functioning in accordance with the specified functional parameters. | Functional parameters may be set by the user (using a superuser account accessed with a password). The location on the ship must be indicated and the cause of the indication must be visible until it is cleared. |
|  | The console operator must have the means to safely disconnect the power sources to the operating systems in order to avoid any potential danger if they continue to operate. | Once the systems have been disconnected in this way, they must be activated manually before they can be restarted. |
|  | The control system must be duplicated (there must be independent information management centres/computers and data servers). |  |
|  | Data storage must be ensured and the system must have data output access (e.g. printer). |  |
| SHIP TANKS |  |  |
|  | Ship tanks must be such that the liquid cargoes they contain are safe. |  |
|  | Each ship tank must have accurate liquid measurement systems (remote and manual). |  |
|  | Tanks must have ventilation systems to protect against overpressure and/or vacuum formation. | Ventilation systems must be such that, if they lose their integrity, sea water and/or rainwater cannot enter the tanks. Ventilation systems must be such that liquid vapours formed in the tanks/drums do not ignite and/or explode. |
| SHIP VENTILATION (MICROCLIMATE) SYSTEM |  |  |
|  | The ship's ventilation system must ensure a proper functional and controllable microclimate for the ship's systems and the people on board. |  |
|  | The ship's ventilation system must ensure adequate ventilation in potentially hazardous areas of the ship. | Its location shall not pose a risk to other areas of the ship. |
|  | The ship's ventilation system shall be such that, in the event of failure, uninterrupted cooling/ventilation/air supply to the necessary systems (compartments/rooms in which they are located) is ensured. |  |
|  | The ship's ventilation system shall comply with the ship's integrity, fire protection and other functional requirements not mentioned herein. |  |
| SHIP DECK EQUIPMENT |  |  |
|  | The ship shall be equipped with the necessary equipment to safely receive/transfer liquid cargo, packaged cargo and people from the ship to the ship. | Communication equipment must be installed between the main operating positions. Measures must be provided for and installed to enable the immediate termination of operations and the disconnection of transfer equipment. |
|  | The ship must be equipped with the necessary equipment to keep the ship safely at a fixed point at sea using an anchor and at the quay using a bow/stern line. |  |
|  | Appropriate means shall be provided to lower the ship's anchor by gravity without using the ship's equipment. Secure the anchor in the required position without using energy sources, disconnect the anchor and its chain in the event of deck equipment failure and/or the anchor becoming stuck on the seabed. Release the entire ship's chain and take it back on board. |  |
|  | The ship must be equipped with the necessary equipment to enable the ship to be towed safely and to tow another ship. |  |
|  | The vessel must be equipped with the necessary lifting equipment to ensure that the vessel's functions can be performed without endangering the people on board, the lifting equipment itself or the vessel. |  |
|  | The lifting equipment must be equipped with the necessary safety measures and be safe to operate in all modes. Information on the safe working load and maximum test load must be clearly visible on or near the equipment. The equipment operator's workplace must be set up so that they can see the load, and if they cannot see it, the workplace must be equipped with the necessary means of communication between the operator's workplace and the load handling area. |  |
|  | Lifting equipment must be controlled from only one operator's workplace at a time. |  |
|  | In the event of a power failure to the lifting equipment, the load must remain in a fixed position and there must be means of safely completing the loading operation. |  |
|  | Mooring equipment: |  |
|  | Mooring cleats shall be installed in accordance with the general arrangement drawing. At least two bollards for diagonal mooring lines shall be installed in the middle of the ship. |  |
|  | Panama-type cleats shall be installed in accordance with the general arrangement drawing. |  |
|  | Rope fastening cleats shall be installed in appropriate locations on the gunwales and railings. |  |
|  | Mooring and towing ropes: two floating towing ropes made of triple-braided polypropylene (or equivalent material) with 2 m long protected loops at both ends must be provided. The length of the ropes shall be at least 100 m; the diameter shall be in accordance with the tonnage of the vessel. Three mooring ropes must be installed at both the stern and bow of the vessel. Mooring ropes shall be attached to rope reels. |  |
|  | Towing and mooring rope reels shall be arranged in accordance with the general drawings. All rope reels must be equipped with mechanical locks. |  |
| V FIRE SAFETY |  |  |
|  | The ship and its equipment shall be designed, constructed, maintained and operated in such a way as to prevent, detect, control and extinguish fires as far as possible, while maintaining essential safety functions during a fire. |  |
|  | The ship shall be constructed in such a way as to maintain the structural integrity of the ship and to prevent partial or total deterioration or loss of the structural integrity of the ship due to the effects of temperature. |  |
|  | The following requirements shall be complied with in the construction of the ship: |  |
|  | The hull, superstructure, transverse bulkheads, decks, deck superstructures and pillars shall be constructed of non-combustible materials or materials of appropriate composition that are resistant to fire. |  |
|  | The structural integrity of the ship, when exposed to fire for at least 15 minutes, shall not: be deformed in such a way as to restrict evacuation and emergency exits from the ship's compartments/rooms or access to them for the purpose of extinguishing the fire or removing its effects; lose the integrity of the ship due to the effects of temperature on transverse bulkheads, pillars, etc. in the compartment/space affected by fire or in adjacent compartments/spaces; damage or adversely affect transverse bulkheads, pillars, 'A' and 'B' class divisions or fire-resistant divisions of the ship constructed of materials other than steel; deformed in such a way as to adversely affect the use of rescue equipment and means; affected by deck valves/gates/fittings (provided that the fire continues for an indefinite period of time). |  |
|  | Measures (structural, technical, etc.) shall be taken on board to: prevent the spillage of flammable liquids, prevent the accumulation of flammable vapours, limit the ignition of flammable materials, limit potential sources of fire, ensure the proper storage of flammable and explosive materials on board the ship. Fuel with a flash point lower than 60ºC (to be agreed separately with the Customer, e.g. boat fuel) must not be used on board, ensuring an appropriate microclimate in the premises. |  |
|  | Measures (structural, technical, etc.) must be in place on board to ensure fire control and management: Ship ventilation control, distribution and storage of flammable and explosive materials on board, resistance of flammable and explosive material distribution systems to high temperatures. The spread of fire must be controlled in the room where it started, using active and/or passive measures (thermal insulation, cooling of partitions/decks/hatches/technological spaces (gland boxes)). |  |
|  | Materials and means used inside the ship, such as paints, deck coverings, combustible insulation materials, cables, non-metallic pipes, soft furniture materials, mattresses, etc., must be such that they do not emit smoke or other toxic substances. Measures (structural, technical, etc.) must be taken on board to ensure the containment and control of smoke spread in key areas of the ship: ventilation system, control areas (which must be fully protected), engine room, assembly and evacuation areas, etc. The ship must be equipped with appropriate redundant smoke removal measures (configuration of the ship's ventilation system, portable fans, etc.). |  |
|  | The ship must have a fire detection and alarm system that: informs the control and command stations on board (bridge/wheelhouse, control and command centre, etc.) in a timely manner, using appropriate fire detection (optimally selected according to the nature of the compartment/room) and alarm means The fire alarm must be audible throughout the ship. The system must be capable of being activated manually using activation mechanisms optimally located throughout the ship. |  |
|  | The ship must be equipped with the most optimal fire extinguishing systems and measures, taking into account the nature of the compartments/rooms. Stationary fire extinguishing systems (including automatic ones) must be located in those compartments/rooms where the risk of fire is greatest and the damage to the ship is likely to be greatest. Fire-fighting equipment must be located and ready for immediate use throughout the ship. The extinguishing agent shall be such as to be most effective and cause the least damage to the equipment in the compartment/room, but its effect on people, the environment, etc. shall be taken into account. The functions of fixed fire-extinguishing systems shall be duplicated by other fire-extinguishing means. | Fire-fighting systems and measures must be maintenance-free. The activation of fixed fire-fighting systems using gas as the agent must be possible only from another compartment/room. |
| VI EVACUATION, ABANDONING THE SHIP, RESCUE |  |  |
|  | The ship shall be designed and constructed so that the measures installed on board ensure: the safe evacuation of persons on board from emergency compartments/rooms to a safe place on board where they are safe or from where the abandonment of the ship can be carried out, safe evacuation from the ship (abandonment of the ship), rescue of casualties from the water. |  |
|  | The rescue equipment on board the ship must be resistant to: corrosion, decay, sunlight (including UV rays), mould, petroleum products, and adverse changes in temperature and humidity. |  |
|  | The storage locations of the ship's life-saving appliances must be: clearly marked, freely accessible, protected from water, smoke, fire, and hazardous vapours, not impair the condition of the life-saving appliances, and not restrict the use of other appliances. The storage locations of appliances stored on the outer deck must ensure that they float to the surface of the water if the ship sinks. |  |
|  | The life-saving appliances on board must be sufficient for all persons on board The ship's crew, additional temporary crew, casualties (total number – not less than 86) | Must comply with SOLAS requirements |
|  | Emergency signal characteristics: the signal must be distinctive, easily recognisable, audible to all persons on board, continuously operational, unaffected by fire, vibration, flooding, etc., and activated from the main ship control and evacuation areas. |  |
|  | Proper communication must be ensured. | Detailed description in the COMMUNICATIONS section |
|  | All evacuation equipment must be provided with the necessary independent power source for its activation/use. | Refer to the section ENGINEERING SYSTEMS |
|  | All escape routes, emergency exits and assembly and evacuation points on board the ship must be adequately lit to enable people to be rescued/evacuated safely. Escape routes and emergency exits and their routes, assembly and evacuation areas must be marked with clearly visible and understandable signs indicating their identification and direction. | Assembly point – a predetermined safe place on the ship where people gather in emergency situations. There is more than one safe place on the ship. |
|  | All evacuation routes and emergency exits on the ship must ensure the fastest and safest way to get from any part of the ship to the designated assembly/safe (ship abandonment) point on the ship. Alternative evacuation/emergency routes to a safe location on the Ship must also be provided and indicated. Evacuation routes must have measures in place to limit the spread of smoke, fire, water and hazardous vapours on the Ship ( , doors, hatches, curtains, screens, etc.). All evacuation routes and emergency exits on the ship, including the ship's systems located therein, their parts and other components of the ship, shall be designed (in accordance with SOLAS, IMO and 100A1 or equivalent standards), taking into account that they do not cause congestion, considering the transport of victims on stretchers and the clothing and equipment worn (e.g. firefighter clothing and breathing apparatus or life jackets). |  |
|  | Emergency self-contained breathing apparatus (SCBA) On board, these are intended to provide limited protection for the respiratory tract and eyes in hazardous conditions until people reach a safe place on board. They must be easy to use and not require the assistance of other people. Their use must not hinder or otherwise restrict the ability of people to evacuate safely from an unsafe environment. The number of AIKAs on board must meet the needs of the ship, taking into account potentially hazardous areas, the maximum number of people on board, evacuation routes and emergency exits. They must be optimally positioned and clearly visible. |  |
|  | Stretchers on board are intended to immobilise and transport/evacuate an incapacitated casualty. Stretchers and their optimal number on board must be selected taking into account the number of people on board, the compartments/areas of the ship and the escape routes. |  |
|  | The launching/lifting and embarkation/disembarkation equipment on board must ensure the safe and effective lowering of the ship's survival craft from their stowage positions and the safe embarkation of persons into them from the ship/into the ship under normal and emergency conditions. The equipment must be easy to use and not require maintenance, and the possibility of its functionality being impaired (appropriate protective measures in place, etc.) in the event of an accident on board the ship or in difficult hydro-meteorological conditions must be minimised. The equipment operating area must not be higher than the height of the lowering devices, i.e. the operator must have a clear view and control of the operation. Boarding/disembarking means and locations on the ship must be provided for after due consideration of the characteristics of the ship's hull and must ensure dry access to and exit from the survival craft. |  |
| 136. | The ship's lifeboat(s) must: ensure an acceptable level of safety for the people on board, accommodate the intended number of people, protect them from the dangers posed by the damaged ship and adverse weather and sea conditions, have the necessary propulsion and steering/navigation equipment, and meet the following requirements: 1. The ship must be equipped with two boat launching devices (cranes), one on the port side and one on the starboard side, capable of safely launching/recovering a lifeboat (at maximum possible boat weight) with crew (8 persons) and equipment (200 kg) into/out of the water in conditions not less than SS5. 2. The ship's lifeboat lowering/lifting devices must be capable of safely bringing the boat alongside the ship so that the crew can safely board/disembark from the deck. The boarding areas must also ensure the safe transport of a lying injured person from/to the boat. 3. The ship's lifeboat launching arrangements shall comply with SOLAS requirements and shall have emergency launching (with persons on board) in the event of loss of electrical power. 4. The ship must have at least two lifeboats, which must be identical. 5. The autonomy of a lifeboat carrying 8 persons must be at least 3 hours when travelling at a speed of 25 knots. 6. Both lifeboats must be capable of operating without restrictions 24/7 at any time of day, at an air temperature of -30°C /+35°C and a water temperature of -4°C / +25°C. 7. The ship's lifeboat must be able to carry at least 8 people at a time, including the driver. 8. The draught of the boats must not exceed 0.6 m. 9. The speed of the lifeboat must be no less than 36 knots at SS2 (when the boat's crew consists of 3 people). 10 The lifeboats must be able to operate without restrictions 24/7 at SS5. 11 The colour of the boat must be agreed with the customer. 12. The ship's lifeboat must comply with Fast Rescue Craft SOLAS requirements with a fully rigid hull without inflatable structural elements. 13. The ship's lifeboat must be capable of operating safely in ice conditions (when there are broken ice floes in the water). 14. The ship's lifeboat must have an internal engine(s) that uses diesel fuel (the same fuel as the ship). 15. Both lifeboats must have at least three operating positions: - fixed position for safe transit and operation of the ship in extremely difficult weather conditions (must withstand SS7) - the position of the boats for operations has been prepared, the boats have been brought to the main deck for the embarkation/disembarkation of the crew (must ensure the safe work of the crew and a secure fixed position in SS5); - Position for picking up/lowering the boat at water level (crew safety must be ensured in SS5). 16. Both boats must be equipped with a quick release hook. 17. Each boat must be equipped with waterproof VHF stations for communication between the ship and the boat crew at a distance of no less than 12 m from the ship. 18. The communication equipment installed in the boat must be installed in such a way that, when wearing specialised equipment and a helmet and in the presence of boat engine noise, reliable boat-ship communication is ensured. The boat operator must be able to communicate safely while transiting the boat(hands-free equipment or similar system). 19. The boat must be equipped with lights that comply with COLREG requirements for this type of vessel when operating 24/7 at sea. 20. The boat operator's position must be equipped in such a way that he is protected from headwinds (the position should be covered from the front by impact-resistant glass or similar equipment). 21. The ship's boats must be delivered with specialised equipment for the boat crews (16 wetsuits, 16 buoyancy aids, 16 helmets for the boat crew (with special fastenings for boat operations at sea)). All equipment must comply with SOLAS requirements, and sizes and colours must be agreed with the customer. 22. Each boat must be equipped with a waterproof GPS device and a navigation system (console with a sea chart showing the position of the boat). 23. The boat must ensure that the crew of 7 people has seating for longer transits. 24. The ship's boat must have a tow rope that allows it to tow other boats (no smaller than a boat of the same size). 25. The ship's boat must be designed so that it can be safely moored alongside a stationary ship and the rescue team can be disembarked. 26. The boat must be equipped to transport 1 injured person on a stretcher and at least 4 crew members at the same time. 27. Each lifeboat must be equipped with a SART (Search and Rescue Transponder) device. 28. The boat's helmsman must have a safety key that will shut down the boat's engine if the helmsman falls overboard. 29. Each boat must be equipped with an automatic capsize reversal mechanism. 30. The boat launching and recovery technology must ensure safe lifting/lowering of boats in SS5 at any time of the day. 31. The ship's lifeboat must be equipped with water jet propulsion systems that protect persons in the water from possible injury. 32 Fuel refuelling points must be installed near the lifeboats. 33 Lifeboats and their launching appliances must be designed so that the crew can use them safely 24/7 during daylight and darkness, and appropriate lighting must be provided in areas necessary for the safe launching/recovery of boats, embarkation/disembarkation of crew members. 34. Next to the lifeboat embarkation/disembarkation area (no further than 10 m from the embarkation/disembarkation area(may be another location, but this must be agreed with the customer)), there must be a place in the interior of the ship for storing and drying the special clothing (wetsuits, life jackets, helmets) of the boat crews. The size of the space must be such that 8 people can put on/take off specialised equipment without hindrance. |  |
|  | Life jackets on board must ensure that people wearing them float on the surface of the water with their faces facing upwards and protected from splashes and waves. Life jackets must be of various sizes to suit the range of people on board and must be suitable for wearing with the wetsuits or other protective equipment on board. The number of life jackets on board must meet the requirements for the maximum number of people on board. Life jackets must be easily detectable in the water under various conditions. | Additional work life jackets must be provided for crew members, taking into account their functions on board. |
|  | The wetsuits on board must protect the people wearing them from hypothermia. Wetsuits must be of various sizes to suit the range of people on board. They must be easy to use and not require the assistance of other persons, and their use must not hinder or otherwise restrict the ability of persons to evacuate the ship safely during rescue or other operations. | Additional work wetsuits must be provided for crew members, taking into account their functions on board the ship. |
|  | The rescue equipment and means on board the ship must ensure the rescue (return to the ship) of persons in the water, on the ship and in other rescue boats. They must operate effectively and quickly when a person falls overboard, minimise the risk to rescuers, and ensure the mass evacuation of people from another ship. | Including: a lowerable rescue net (fastening in the structure provided on both sides) in accordance with IMO SOLAS requirements; pilot boarding ladders in accordance with IMO SOLAS requirements. |
|  | The ship must be equipped with a helicopter landing pad to accommodate a helicopter up to SS4 in accordance with STANAG 1194 and STANAG 1474 requirements. Installation on the upper deck and stability requirements in accordance with the requirements of the classification society. The helicopter landing pad must be equipped with at least: - safety netting around the entire perimeter; - markings, lightening and signaling in accordance with STANAG 3619 (compass direction, marking lines, H mark, lighting for night operations and use of Night Vision Goggles); - fire protection equipment; - Grounding and mounting connections for helicopters; - an access way for the evacuation of casualties on stretchers. Characteristics of the helicopter to be received: - Minimum landing weight – not less than 10.5 t - Main rotor diameter: not less than 16.5 m. - Helicopter length: not less than 20 m. |  |
| VII SHIP COMMUNICATION SYSTEMS |  |  |
| No. | Requirement | Comments |
|  | GMDSS equipment |  |
|  | The ship's GMDSS communications equipment must comply with the requirements of SOLAS 4, as set out in Article A1, A2 and A3. |  |
|  | The ship must be equipped with a very high frequency (VHF) radio station that ensures: - transmission and reception of distress signals (Distress DSC) on a frequency of 156.525 MHz (channel 70); - voice communication on pre-programmed channels (16, 13, 6); - reduced power transmission function (1W function). |  |
|  | A receiver capable of receiving international maritime safety messages and warnings (NAVTEX); |  |
|  | Satellite emergency position-indicating radio beacon (satellite EPIRB), which must be: -Capable of transmitting a distress alert via a polar orbit satellite service operating in the 406 MHz frequency band; -Capable of being activated manually and automatically; |  |
|  | The ship must be equipped with a medium and high frequency (MF/HF) radio communication system that allows: -Send distress DSC messages using the 2187.5 kHz frequency; - be capable of maintaining continuous monitoring of distress signals on the 2187.5 kHz frequency; - send and receive distress (DSC) and other safety-related information in the frequency range from 1605 to 27500 kHz inclusive. |  |
|  | The ship must have a set of Inmarsat-C satellite system equipment that: - can receive/transmit distress messages via Inmarsat satellites; |  |
|  | All GMDSS equipment must be equipped with primary, emergency and backup power sources; |  |
|  | All GMDSS equipment must be permanently available (i.e. have an alternative). |  |
|  | Internal ship communication system (Intercom). |  |
|  | The ship's internal communication system must be accessible from all escape, evacuation and rescue stations, command and control stations and other locations specified by the customer. |  |
|  | The system must ensure two-way voice communication between all areas of the ship. |  |
|  | The system must be capable of broadcasting messages throughout the ship and must have the ability to control the flow of information being transmitted (e.g. only to external decks, only to internal spaces). |  |
|  | The system must have the capability to broadcast distress signals. |  |
|  | The system must have a backup internal communication system that is independent of the main system. |  |
|  | The layout and architecture of the system equipment must be agreed with the customer. |  |
|  | The ship must be equipped with a stationary very high frequency radio station capable of communicating with aircraft (airband). |  |
| - 2.5. | The radio station must operate in the very high frequency range from 118 to 174 MHz inclusive. |  |
|  | The radio station must be capable of receiving and transmitting bass communication in AM modulation. |  |
|  | The ship must be equipped with at least 3 stationary very high frequency radio stations. |  |
|  | Radio stations must operate in the frequency range from 156 MHz to 162 MHz inclusive; |  |
|  | Radio stations must have a power of at least 20 W, with a 1 W power limitation function; |  |
|  | The ship must be equipped with at least 4 waterproof handheld very high frequency stations. |  |
|  | Handheld radio stations must operate in the frequency range from 156 MHz to 162 MHz inclusive and have a DSC function that ensures the transmission of distress signals from rafts; |  |
|  | Handheld radio stations must be resistant to environmental factors – they must have a water resistance rating of at least IP67 (or equivalent). |  |
|  | Handheld radio stations must have stationary chargers; |  |
|  | The ship must be equipped with at least 16 portable radio stations for communication between crew members. |  |
|  | Radio stations must be programmable and operate in a very high frequency range; |  |
|  | The ship must have at least 3 charging stations for the stations, with a capacity to charge at least 4 stations at a time; |  |
|  | The stations must be suitable for indoor and outdoor use and have a water resistance rating of at least IP67 or equivalent. |  |
|  | The ship must be equipped with a direction finder for distress signals. |  |
|  | Designed to detect distress signals in the very high, ultra-high and medium frequency ranges; |  |
|  | The direction finder must have a digital display of the detected signal with an accuracy of no more than 1 degree; |  |
|  | The direction finder must be capable of integration with the ship's navigation radar. |  |
|  | The ship must have TCP/IP data reception and transmission capabilities in all areas of operation, ensuring both GSM (4G/5G) and Low Earth Orbit (LEO) data transmission connections. |  |
|  | The ship must have a low Earth orbit satellite system (Ka and Ku frequency range) for data transmission, ensuring a speed of not less than 100 Mbps. |  |
|  | The low Earth orbit satellite system must have an external antenna that is resistant to environmental factors, with a rating of at least IP66. |  |
|  | The ship is equipped with a 4G/5G router with an external 4G/5G antenna; |  |
|  | The ship must have an internal network (LAN) ensuring access in the required premises; |  |
|  | The ship must have a data transmission network that ensures network availability in all living quarters, conference rooms, bridges and control points with double sockets (RJ45); |  |
|  | The ship must have a local internet network with cables of at least category 6 (Cat6); |  |
|  | The network must have an SD-WAN network device that allows automatic and manual switching of the Internet access source (GSM/Satcom). |  |
|  | The ship must be equipped with a continuously recording video surveillance system. |  |
|  | The system must have an uninterruptible power supply that allows recording for at least 60 minutes. |  |
|  | Video recordings must be archived and stored for at least 30 days. |  |
|  | The video cameras must be powered via a network (PAO), have night vision capabilities and a resolution of at least 5 MP. |  |
|  | The system must have at least 8 cameras installed in locations specified by the customer. |  |
|  | All cameras must be resistant to environmental influences and have a rating of at least IP67; |  |
|  | The system must be capable of monitoring, changing and viewing at least 6 cameras simultaneously from the ship's wheelhouse, the ship's control centre (engineering centre), the watch room and the ship's captain's cabin. |  |
| VIII NAVIGATION |  |  |
| 190. | The ECDIS system must include all necessary electronic chart licences for both the region designated for sea trials and the entire Baltic Sea for a period of at least one year from the date of delivery of the vessel to the end user. |  |
|  | The ship's navigation systems must: ensure high reliability and reduce the risk of malfunction under all anticipated operating conditions, prevent accidents and emergency situations; maintain essential safety functions without interruption after at least one operational error and/or system/equipment failure. |  |
|  | The ship's bridge (wheelhouse) must be such that it fully ensures safe navigation and other functions performed from it. |  |
|  | Microclimate conditions must be ensured on the bridge using the ship's system and/or a separate system designed for the bridge. The layout of lighting, windows (windows must be protected from direct and indirect sunlight), equipment and tools must be ergonomic, and the consoles on which the equipment is mounted must be of uniform functional design. Equipment on board that can be controlled from other locations on the ship must have the same layout and functionality. The noise level on the bridge must not interfere with the performance of normal functions (in accordance with the requirements of the IMO and the selected classification society). The bridge must be equipped with an operator activity/safety system that records the operator's ability to perform functions. |  |
|  | The ship's bridge must be ergonomic, allowing free movement around it and access to the designated operator stations. The bridge must be equipped with rescue equipment and devices ready for use. |  |
|  | Workstations on the bridge must ensure that operators can work efficiently, continuously and effectively process information and make decisions in order to maintain proper observation and safe navigation. |  |
|  | Workstations on the bridge shall provide operators with convenient and continuous access to essential information, which shall be presented in a clear and unambiguous manner, using standard symbols and coding in control and display systems, allowing for the rapid, continuous and efficient processing of information and to make decisions on safe navigation and additional wheelhouse functions. Workstations shall indicate the operating status of automated functions and integrated components, systems and/or subsystems. |  |
|  | An alternative control position(s) shall be provided on board to enable operators not in the wheelhouse to process information and make decisions for proper monitoring and safe navigation in a timely, continuous and effective manner. |  |
|  | The alternative control position shall enable operators to safely control the ship independently of the ship's personnel in the wheelhouse. Alternative control stations and their layout shall facilitate the tasks of operators in fully assessing the situation and navigating the ship safely in all conditions, while ensuring the same functions as those performed in the wheelhouse. The same conditions for performing functions and safety as on the bridge or close to them must be ensured. |  |
|  | The ship shall be equipped with an emergency control station (or stations) where operators, unable to perform their direct functions on the bridge and/or in the alternative control station, would be able to perform essential but limited navigation functions until the ship reaches a safe place. The same conditions for the performance of functions and safety as on the bridge, in the alternative control station or similar locations shall be ensured. |  |
|  | The ship shall be equipped with sufficient sensors and systems to continuously and accurately determine, display and record the current time, position, orientation and movement relative to the earth and speed, and changes in the measured parameters, to ensure safe and independent navigation. |  |
|  | The ship shall be equipped with appropriate sensors and processing equipment to measure, analyse, evaluate, display and record its physical environment for safe navigation. |  |
|  | The sensors and systems on board the vessel must meet the following minimum conditions: ensure continuous monitoring of position, speed and time; generate an accurate count of the estimated time of travel without interruption; determine and enter course and turn information; always correct the course and bearings to True; determine and enter speed and distance travelled in water; determine and enter water depth; determine and update the ship's position; determine weather and sea conditions; measure, display, record and analyse the vessel's behaviour in real time under prevailing wind and sea conditions. |  |
|  | The ship shall be equipped with appropriate means for controlling propulsion and manoeuvring, navigation and other systems from the wheelhouse and emergency control station for safe navigation, collision avoidance and other operations. |  |
|  | The ship's captain or watch officer on the bridge can use the control devices to: control the speed and course of the ship, control the means of indicating the ship's navigational intentions or restrictions (navigational lights, signs, sound signals, etc.), see the factors limiting the ship's capabilities; the vessel's control conditions and the status of the vessel's operating equipment, control the vessel's lighting conditions, communicate with other vessels and aircraft at all hours of the day, to report an accident/disaster affecting the Ship by all means, to communicate with other control stations, workplaces and people on board the Ship in an emergency situation. |  |
|  | The ship's control and navigation equipment must be redundant, and critical equipment must also have emergency controls to ensure safe and uninterrupted manoeuvring of the ship in various conditions (propulsion, steering, firefighting, navigation, safety and lighting equipment systems). | In accordance with SOLAS and NAV1 requirements |
|  | In order to ensure the most optimal performance of the ship's control functions, the ship's bridge must be equipped with a single automated control system – an integrated ship bridge navigation system. |  |
|  | The ship must be equipped with equipment and systems for receiving, transmitting, recording and analysing data in formats acceptable for safe navigation. These measures must: record all navigation-related data for a minimum period of 12 hours, using sensor history, internal and external voice recordings (decision-making), and reproduce the ship's movement parameters. |  |
|  | The ship shall be equipped with systems and/or equipment to report its position and movement on a regular basis. |  |
|  | The ship shall be capable of automatically transmitting and receiving specified navigation and safety-related information to suitably equipped ships, aircraft and shore stations. |  |
|  | The ship shall be equipped with sufficient means of communication, at least two separate and independent means, to enable the transmission and reception of distress alerts, SAR coordination, maritime safety information, general radio communications and ship-to-ship communications. |  |
|  | The ship shall be equipped with sensors, systems and equipment that allow for proper monitoring in all circumstances and conditions and for early and accurate detection of collision risks. The ship must be equipped with systems and equipment that signal the nature and general movement of the ship and allow other ships to identify the risk of collision early and accurately. The ship's equipment and systems must allow the ship to manoeuvre in such a way as to avoid collision. |  |
|  | The ship must be capable of being clearly visible at all times of the day and in all weather conditions, using all the means and systems available on board, and its movement, activities, restrictions and other relevant information, enabling seafarers on other vessels to understand and predict the vessel's activities, thus creating conditions for safe navigation for all. The ship's systems must ensure automatic warning of the ship in the event of conditions dangerous to navigation, and must ensure rapid reverse movement of the ship and the necessary manoeuvrability of the ship. |  |
|  | The ship must have the necessary conditions and equipment in place to safely receive and disembark the pilot from any side of the ship. |  |
| IX DANGEROUS CARGOES ON BOARD (according to IMDG classes 2-6) |  |  |
|  | The ship shall be constructed in such a way that it can store and transport dangerous cargoes that are not categorised as part of the ship's systems, equipment or facilities. The ship shall have a designated area for storing such cargoes. When selecting the location, all safety aspects must be taken into account, i.e. the potential risk of dangerous cargo must be reduced to an acceptable/controllable level using the measures available on board the ship. |  |
| X SHIP PREMISES |  |  |
|  | The ship must be constructed in accordance with good shipbuilding practice and functional ergonomics, taking into account the primary and secondary purposes of the ship, ensuring adequate working, living/resting and leisure conditions for the ship's crew and other persons on board. Controlled microclimate conditions must be ensured in all compartments/rooms of the Ship. Noise, vibration and radiation levels must not exceed the levels specified in the requirements and must not have a negative long-term impact on the people and systems on board the Ship. All necessary measures to ensure the safety and health of people must be in place and arranged on the ship. | All ship compartments must comply with/meet the requirements of the ILO MLC (International Labour Organisation Maritime Labour Convention), IMO and the selected classification society. |
|  | Functional/service areas of the ship must be equipped in such a way that they are easy to maintain, have all the necessary durable equipment for performing functions and servicing the systems and equipment on board the ship, and are stored safely. Appropriate hygiene/sanitary conditions must be ensured. |  |
|  | Accommodation/recreation Ship's spaces (cabins) shall be arranged in such a way that they are easy to maintain and have all the necessary durable equipment for living and recreation conditions safely stored. All windows in accommodation, recreation, administrative and service spaces shall be fitted with drop-down curtains or roller blinds. Equipment and furniture on board, including fixed seating, should be made of plastic-coated marine plywood. Where applicable, strong, flame-retardant upholstery should be used. |  |
|  | The ship shall have 14 cabins (13 for permanent crew members and 1 for first aid). All cabins should have additional temporary sleeping places (e.g. fold-down beds) if the design allows. Basic cabin layout and number of people in each cabin: Captain 1+1 one additional place in it; EMS leader 1 +1 additional place in it; Crew - 11 cabins with 2 places each. First aid 1 +1 additional place in it. The cabin must be accessible with a stretcher. Additional temporary crew members may be accommodated in 1-2 cabins. The captain's and chief engineer's cabins must have a work table and a universal console providing passive information about what is happening on the ship: ECDIS and propulsion UMS (Unattended Machinery Space) system parameters. All cabins (except for the first aid cabin) must have a toilet (including a toilet bowl), a shower and a sink with all necessary accessories. All cabins must have adequate lighting on the ceiling. All beds, including extra beds, must have reading lights. All cabins must be equipped with sufficient electrical sockets (220V, at least 4 sockets in each cabin) and one radio and TV antenna connection each. All cabins must also be equipped with 2 RJ45 Ethernet sockets, which must be connected to one of the two local networks. |  |
| 218.1 | Additional requirements for living quarters: Minimum cabin areas in accordance with the ILO Maritime Labour Convention (MLC 2006). SOLAS Chapter II-2 (fire safety of accommodation). Fire insulation (A-60 class partitions between accommodation spaces and fire hazard areas). Each standard cabin must have at least: - Two separate beds. - Two lockable wardrobes. - One desk with desk lamp lighting. - One free-standing chair with armrests and fastenings for use in rough seas. - One bookshelf. - Two wall hooks for clothes. - One rubbish bin. - One whiteboard. - Curtains for portholes. - One set of bedding (including sheets, blankets, pillows and pillowcases, a set of towels) for each crew member for whom the ship is designed and certified to carry. Each standard cabin shall have a separate shower/WC room, which shall have at least: - One toilet with a fresh water flushing system; - One washbasin with hot/cold water mixer tap; - One shower with hot/cold water mixer, hand-held shower head, water drain, soap dish and curtain; - One cabinet with a mirror above the washbasin. All chairs on board must be of marine construction of the same type. All chairs must be secured for use in heavy sea conditions. |  |
|  | The ship's superstructure or hull (depending on the design) must have the following rooms: – mess room; – lounge; – shower room (including separate changing room); – changing rooms/sanitary facilities for work in contaminated environments, accessible from both the living quarters and the outside; – at least 2 shared toilets; – cleaning supplies storage rooms/storage rooms – at least 2; – a recreation room. |  |
|  | - The ship must have a separate room, 4 m² in size, with a cable passage (allowing additional cables to be passed through without compromising the ship's integrity) to the ship's mast (not less than 10 cm). The room must be windowless, with walls made of metal sheets at least 3 mm thick or similar materials that ensure equivalent resistance to break-ins. Three computerised workstations must be designed and installed in the room. Required equipment: height-adjustable tables – 3 pcs., height-adjustable chairs – 3 pcs., bookshelves – 2 pcs. Each computerised workstation must be equipped with 6 electrical power sockets and 4 RJ45 network sockets. - The doors of the premises must be adapted to an electronic access control system, with a direct-acting electromagnetic lock without a free passage function, a closer and appropriate locks and handles. In the event of a failure of the electromagnetic lock, it must be possible to open the door using the door lock. The doors shall be armoured (door leaf and frame tested in accordance with EN 1627, meeting the requirements of security class RC3 or higher, or the door leaf shall be made of profiled steel. The tubular profile is not thinner than 20 x 28 mm, the wall thickness is not thinner than 1.5 mm, and it is placed around the entire perimeter of the frame. The lock installation site is reinforced with a tubular profile box with a thickness of at least 20 x 28 mm around the entire perimeter of the lock on the door surface. If the door leaf width exceeds 800 mm, an additional strut is installed across the entire height of the door leaf. The gap between the door leaf and the frame is no more than 6 mm. Rock wool filling or similar material is no thinner than 28 mm. A steel sheet with a thickness of at least 1.5 mm is placed on the tubular profiles on the outer side of the door leaf from . The sheet must cover the gap between the frame and the door leaf by at least 12 mm. A steel sheet with a thickness of at least 1.5 mm is attached to the inner side of the door leaf frame. The frame is made of profiled steel with dimensions of at least 80 x 40 mm and a wall thickness of 3 mm. The frame is fastened at no less than six points with metal rods or bolts with a diameter of at least 14 mm. - The door is equipped with at least two locks, one of which has a cylindrical core tested in accordance with standard EN 1303, meeting the requirements of security class 4 or higher, and the other is a plate lock. and both locks shall be tested in accordance with standard 12209 and meet the requirements of at least level 4). The outer side of the door shall have a fixed "bubble" handle, and the inner side shall have a fold-down handle, both in stainless steel. - The electronic security system of the premises must ensure that an employee can enter the premises independently when his identity has been verified by at least two different means: biometric data and contactless card or token data (information transmitted by radio signal). Each employee's entry into and exit from the premises is recorded. The electronic access control system records any unauthorised opening of doors or unauthorised attempts to open doors and any doors left open. The control unit of the electronic security system must be installed in the same premises. The electronic security system must ensure emergency access to the premises in the event of a malfunction of the electronic system. - Entry into the room must be monitored by a video surveillance system that complies with the requirements of security class 3 of standard 62676. Video surveillance of the room must be carried out from a room agreed with the ship's owner. - The premises must be equipped with an intrusion detection system, and all system devices must be tested in accordance with standard 50131-1 and comply with the requirements of security class 3. |  |
|  | The ship must have service and storage rooms. The ship's superstructure or hull (depending on the design) must have the following rooms: – laundry room (including separate washing and drying rooms with an iron and ironing board); – fire-fighting equipment rooms; – diving equipment room; – galley; – deck storage rooms (accessible from outside); – emergency equipment storage room (accessible from outside); – dry storage rooms; – engine room; – additional equipment (navigation, etc.) room; – battery rooms; – ventilation room; – air conditioning rooms. |  |
|  | COMMON CREW ROOMS Common area Chairs and sofas must be upholstered and covered with fabric or leather. Table tops must be laminated. |  |
|  | Canteen and lounge (separate) Walls – PVC-coated steel cassette panels. Ceilings – painted steel ceiling cassettes. Floors – non-slip covering. The ship must have a lounge and a canteen. The lounge is intended for relaxation, while the canteen is for dining. The lounge must have soft sofas, a clock, a television (with a diagonal screen size of at least 100 cm) with a separate sound system that will be used to display the ship's crew instruction and training materials, two RJ45 Ethernet sockets and a stereo system. The canteen must have a sufficient number of cabinets and shelves for storing fixed dining tables and cutlery. The saloon and mess room shall be of such a size that they can accommodate at least 15 crew members at the same time. Canteen equipment: Table for 6 people 2 Chairs, components 12 Refrigerator 1 Cold water dispenser 1 Microwave oven 1 Cabinet with wall and floor sections 2 Salon accessories: Notice board 1 Television 1 Corner sofa 1 Bookshelf 1 |  |
|  | Sanitary facilities Washbasin Walls – all walls must be covered with PVC (or equivalent material) coated panels. Ceiling – fire-resistant ceiling panels. Floors – epoxy (or equivalent material) base and combined epoxy coating with underfloor heating. Equipment: 2 showers, mirror, soap dispenser, bench, wall partition (if necessary), washbasin and 2 soap and sponge holders. Changing/sanitary facilities (middle part of the main deck, accessible from outside) Walls – all walls must be covered with PVC (or equivalent material) coated panels. Ceiling – fire-resistant ceiling panels. Floors – epoxy (or equivalent material) base and combined epoxy coating with underfloor heating. Fixtures: steel (or equivalent material) cabinets, clothes hooks, washbasin. |  |
|  | Public toilets Cold surfaces must be insulated and all wall and ceiling surfaces must be covered with tiles. The floor structure must be made of an epoxy (or equivalent material) base and covered with an epoxy (or equivalent material) coating. Fixtures: bidet, washbasin, mirror with light, toilet paper holder, soap dispenser, coat hooks. |  |
|  | Office premises Walls – PVC (or equivalent material) coated steel cassette panels. Ceiling – fire-resistant ceiling panels. Flooring – textile carpet (or equivalent material). Equipment: Height-adjustable chair 2 Desktop computer (latest model) 1 Wall and floor cabinet 1 Filing cabinet 2 Clothes rack 1 Notice board 1 |  |
|  | Cleaning supplies cabinets Walls – PVC (or equivalent material) coated steel cassettes/panels (or equivalent). Ceiling – painted metal sheet cassettes. Floor – rubber mat with a knobbed surface. Cleaning supplies cabinets must have shelves for storing cleaning and other supplies. |  |
|  | Recreation room Walls – PVC (or equivalent material) coated steel cassettes. Ceiling – covered with sound insulation. Flooring – modular rubber carpet made of conductive, antistatic material. Training equipment: Stationary bicycle 1 Rowing equipment 1 |  |
|  | SERVICE AND STORAGE ROOMS Common area Cleaning equipment cabinets must be installed in the living quarters of the lower and main decks. The cabinets must be equipped with all necessary cleaning tools and materials. There must be at least 2 cleaning pumps on board. |  |
|  | Kitchen The ship must be equipped with all necessary tables, cabinets and shelves in accordance with the Supplier's drawings. Materials – stainless steel (polished). Walls – polished stainless steel. Ceilings – polished stainless steel. Floors – epoxy (or equivalent material) base and combined epoxy coating. One complete set of kitchen and service equipment corresponding to the Supplier's list of planned inventory must be provided. The vessel must be delivered with at least the following high-quality marine equipment and appliances: Stove with fan 1 Convection oven, min. 6 shelves 1 Refrigerator 1 Freezer 1 Waste trolley 1 Dishwasher 1 Waste shredder 1 Microwave oven 1 Sink with tap + rinse hose 2 Sinks with taps 1 Cleaning supplies cabinet 1 Integrated hot beverage station 1 All cold surfaces must be insulated and all surfaces must be coated. A vapour collector with a removable grease separator and an outlet must be installed at the top. The collector must be made of stainless steel. Grease separators must be suitable for washing in dishwashers. |  |
|  | Food storage rooms Refrigeration rooms (minimum temperature - 22°C); Refrigerator room (minimum temperature +4°C); Dry storage rooms (under the main deck). Walls – insulated with metal panels; uninsulated walls without formwork (only painted). Ceilings – fire-resistant ceiling panels. Floors – modular rubber carpet made of conductive, antistatic material. Equipment – shelves with storm railings. |  |
|  | Laundry rooms Walls – insulated with metal panels. Ceilings – insulated with fire-resistant panels. Flooring – modular rubber (or equivalent material) carpet made of conductive, antistatic material, with a water drainage trap. Equipment: Washing machine, min. 10 kg 2 Dryer, min. 10 kg 2 Sink with tap 1 |  |
|  | Drying rooms (connected to laundry rooms): Walls – insulated with corrosion- and fire-resistant metal (or equivalent material) panels. Ceiling – insulated with fire-resistant panels. Floors – modular rubber (or equivalent material) carpet made of conductive, antistatic material. Equipment: shelves, drying lines, etc., air hoses for drying diving and rescue suits. |  |
|  | Machine workshop Walls – insulated with corrosion- and fire-resistant metal (or equivalent material) panels. Ceiling – covered with sound insulation and glass fibre (or equivalent material) fabric. Floors – painted steel (or equivalent material) with non-fixable rubber mats. Equipment: Work table 1 Sink 1 Shelves and cabinets for spare parts and tools Repair tools 1 set |  |
|  | Other storage rooms and attics Must be equipped with shelves and cabinets for storing tools and equipment (deck storage rooms, linen storage rooms, kitchen storage rooms, etc.). Equipment rooms Additional equipment rooms for storing bridge/wheelhouse equipment must be installed on the bridge deck below the wheelhouse. Walls – insulated with corrosion- and fire-resistant metal (or equivalent material) panels. Ceilings – fire-resistant ceiling panels. Floors – modular surface rubber (or equivalent material) carpet made of conductive, antistatic material. Storage rooms in the basement Walls – insulated with corrosion- and fire-resistant metal (or equivalent material) panels. Ceiling – covered with fibreglass (or equivalent material) covering. Floors – painted steel with non-attachable rubber mats. Equipment: suitable shelves with storm rails and hooks. |  |
|  | Emergency equipment storage rooms Equipment: the shelves with storm rails and hooks provided must be suitable for: - a portable autonomous water pump with a discharge hose at least 20 m long and a suction hose at least 10 m long; - a portable 5 kWh diesel generator; - an axe; - a hammer; - cold cutting equipment (cutting discs); - a hydraulic metal cutting and expansion device; - lamps; - loading bag for air transport (capacity up to 100 kg); - gas equipment; - water spray injector. |  |
|  | The ship must have a separate medical room that would preferably be directly accessible from the main deck, through which the injured are brought from the boat/helicopter, and easily accessible from the living quarters, isolated from noise, odours and fire hazards. Requirements: At least one bed (with safety straps), a cupboard, a table, a chair, lighting, heating and ventilation must be provided. The room must have access to a toilet and washbasin with hot/cold water; the floor and walls must be easy to disinfect. A lockable medicine cabinet, refrigerator for medicines, first aid equipment (defibrillator, space for an oxygen apparatus, stretcher) must be provided. Communication with the bridge is ensured. Hygiene and waste management measures are carried out in accordance with SOLAS and MLC requirements (or equivalent). It must be possible to access the medical room with a stretcher carrying the injured person and accompanying medical personnel. The medical room must also be equipped with the following: 1. AED, 2. Electrocardiograph, 3. Electric suction device, 4. Chest compression system, 5. Oxygen supply system – oxygen cylinders with a regulator capable of supplying 12 litres of oxygen per minute through a mask/mask with a reservoir, or capable of supplying 2 litres per minute through nasal cannulas as needed when transporting the injured person, 6. Glucose meter, 7. Pulse oximeter, 8. Vital signs monitor, 9. Oxygen concentrator, 10. Portable DPV (artificial lung ventilation) device, 11. Intravenous fluid warming bag. 12. Soft stretchers – 2 units. 13. Rigid stretcher with restraint straps – 2 units. |  |
|  | The ship must have a telecommunications room of at least 4 square metres for the management and maintenance of the ship's internal network and surveillance system, with a switchboard cabinet of at least 19 inches for the equipment. |  |
| XI POLLUTION INCIDENT RESPONSE SYSTEMS |  |  |
|  | The ship must have the necessary system/equipment for detecting oil spills on the water surface (stationary oil detection radar), which must function effectively at any time of the day and in various hydrometeorological conditions, operating at maximum effective range. Oil detection radar Oil detection radar must be capable of detecting spilled oil products at any time of day. The oil detection radar must be connected to a standard X-band navigation radar. Oil detection radar must automatically detect spilled oil and create an area (polygon) to mark the spill. It must be possible to manually mark areas (polygons) of spilled oil products and update them if necessary. It must be possible to make screen copies from the screen and record raw radar images to an external storage medium for future evaluation. It must be possible to determine and calculate the area and volume of the oil spill. It must be possible to determine the location, speed and direction of movement of the oil spill. Detection of spilled oil at a distance of 1-4 nautical miles. The oil detection radar must be connected to an integrated bridge system and have its own screen with a diagonal of at least 19 inches or more. | Effectiveness is demonstrated by the performance of the equipment under relevant hydro-meteorological conditions. |
|  | The ship shall be equipped with a primary and secondary ultra-high efficiency pollution recovery system and a dispersant spraying system. These shall be capable of operating independently of each other. Automated oil recovery system integrated into the ship's hull The automated oil recovery system shall be installed on both sides of the ship and shall enable the recovery of oil products from both sides of the ship while the ship is moving. The capacity of the automated oil recovery system must be at least 120 m3/hour on each side of the ship (240 m3/hour in total) for oil products ranging from 300 to 1,000,000 cSt. The oil recovery system shall be designed to minimise the loss of recovered oil during oil recovery operations in order to ensure safe operation. The oil recovery system shall be capable of being fully deployed within 30 minutes. The oil recovery system shall be ready for operation, controlled and recovered by two operators without the use of any additional tools. The oil recovery system shall be prepared, operated and retrieved safely without any human intervention outside the ship. The oil recovery system shall be capable of operating at a ship speed of 0 to 3 knots and in waves up to 1.5 m high. The collection width of the oil recovery system from each side of the ship must be at least 10 m. The height of the floating booms used in the system must be at least 2 m. The oil recovery system shall be of a type that minimises the amount of water in the recovered oil, i.e. the oil/water ratio requirement shall be more than 95 % oil/less than 5 % water. Wave height shall not affect this ratio. The system must be designed for a draught ranging from the design draught to the minimum draught. The system requirements must be met at air temperatures down to -20°C and water temperatures down to 0°C. The materials used must withstand storage temperatures of at least -25°C. All control of the system must be provided on the working deck. If necessary, it must be possible to control the system remotely from the wheelhouse after it has been started. When the vessel is handed over, operating and maintenance manuals, a technical maintenance checklist and spare parts lists must be provided in paper and electronic format. A warranty for the equipment must be provided, valid for at least 24 months from the date of acceptance of sea trials. This warranty shall cover any defects resulting from faulty design, manufacture or use of substandard materials that become apparent during the warranty period for all equipment, fully assembled. The oil recovery system shall be equipped with a heating system for the recovery of heavy oil products. The parts of the oil recovery system equipment that are lowered onto the water surface must be made of highly resistant non-combustible material. The oil recovery system must be capable of adjusting the recovery width from each side of the vessel. The oil recovery system must have heated tanks for storing the recovered oil. The oil recovery system must be equipped with at least one pump on each side to pump the recovered oil into the recovery tanks on the ship. A certificate of performance must be provided for use with high-viscosity oil up to 1 million cSt. High-speed oil recovery system The high-speed oil recovery system must be operated using one ship and a boom. The system can be launched from a ship with a deck area of 5-6 metres from the reel. Three people are required to launch the high-speed system and collect oil at a speed of up to 5 knots in the water. The system must have an integrated oil/water separator and temporary storage tank and must have: - A closed bottom with pressure relief valves to store the collected oil - An open top for easy removal of debris, - To pump out the collected oil, a pump or skimmer can be lowered into a special pump zone to pump out the oil accumulated in the separator, - Waste network and wave motion damper, - Release valves for faster extraction. The integrated oil/water separator and temporary storage facility must have: - a total capacity of at least 65 m3, - the ability to collect and store at least 35 m3 of petroleum products, - the ability to form a layer of petroleum products at least 1 metre thick. The system must be of double-layer construction with a single outer protective frame and inner individual and replaceable hermetic floats. - An outer protective casing that is resistant to UV rays and abrasion, - Internal individual airtight chambers. - Individual air chambers can be replaced without any welding or gluing, - Two valves in each individual air chamber, one at each end, allow the system to be extracted and rolled up from both ends. - 7 x light packs for mounting marker lights for night work, - Reflection evenly distributed throughout the system, - Fabric, PU/PVC, yellow colour for better visibility. The system must operate with one boat with a screen or another boat (dinghy). - The screen must be unfolded and raised without the use of a crane or lifting device. The system can be deployed from the deck of a boat or from the shore. - When unfolding from the deck of a boat, no other boat or dinghy should be needed. It should be possible to deploy first with the separator or cleaning levers. - After deployment, it can be transferred to another vessel/boat. Deploy and store in a reel no larger than 1 x 7.5 m3, including: - power unit, storage for the barrier and all necessary accessories - maximum weight not exceeding 4200 kg The technology of the high-speed system must be tested: - In real oil spills. - In an open ocean test with oil on water, achieving a recovery efficiency of more than 75%. (Recovery efficiency = oil + water (75% oil + 25% water = 100% total recovered liquid)) Coverage area with a minimum front opening of 34 metres at a speed of 5 knots over 10 hours: 1.25 km2 (with 85% effective operating time) Free-floating oil skimmer (lowered by ship crane) The skimmer must be sturdy and made of marine and oil-resistant material. The skimmer and pump must have a capacity of at least 80 m3 /hour. The skimmer must have a single lifting point and be lowered by a ship's crane. The skimmer must be equipped with two engines so that the operator can manoeuvre the skimmer. The skimmer system must be controlled by a remote control panel. The skimmer must be adapted to work with a high-speed oil recovery system. The capacity of the pumps must be certified by a classification society. Dispersant spraying system The system must be designed to spray pure and diluted dispersants. The system must consist of: an electric pump, two spray wings and a set of hoses for pumping and spraying dispersants. The dispersant spraying system shall be equipped with an excess pressure valve and a flow meter. Main physical characteristics: - electric motor; - pump capacity of at least 9 m³/hour; - pressure not less than 4 bar; - the system must have at least 2 spray wings; - spray arm length must be at least 8 m. |  |
|  | Oil pollution detection, collection and neutralisation systems must be fully equipped and integrated into the ship's functionality. The ship must comply with the OILREC class. Pumps must comply with the ATEX directive, which ensures the safety of products used in explosive environments. |  |
|  | Collected oil pollutants are stored in a designated ship tank (tanks), from which they will be transferred using ship equipment to another ship and/or shore infrastructure in the Port of Klaipėda. |  |
| XII OTHER FUNCTIONAL EQUIPMENT AND MEANS OF THE SHIP |  |  |
|  | The ship must have the necessary systems, means and equipment for search and rescue, which are effective at any time of the day and in various hydrometeorological conditions. The following additional measures for detecting people on water shall be provided on board: Marine (impact-resistant) binoculars (7x50) with integrated magnetic compass – 4 units. |  |
| 244. | Requirements for remotely controlled search and rescue (SAR) lights: The remote-controlled SAR light shall be installed at the top of the bridge, on the ship's centre line. Basic requirements for the SAR light body and housing: The total weight of the tower with internal components must not exceed 150 kg; Unrestricted horizontal and vertical movement; IP66 protection rating or higher; Resistant to seawater; The SAR spotlight stand must be integrated with: White search light with parameters: Multi-LED or HMI type; Average service life of at least 1000 hours; If an HMI type lamp is used, the set must include at least 3 spare lamps; Range parameter: no less than 1 lux at a distance of 3000 metres; A separate UV light lamp is designed to detect oil stains; Effective UV light radiation distance of at least 1000 metres. TV camera, no less than full HD quality and capable of distinguishing UV light reflections; Optical zoom of at least 10x. Separate LED lamp for Morse code signals; Average service life: at least 25,000 hours; The remotely controlled SAR light must be controlled from a special panel installed at the front of the bridge, together with the necessary monitor to display the image transmitted by the daytime camera. |  |
| 245. | Electro-optical system (hereinafter referred to as EOS) requirements: The electro-optical system is designed to provide the operator with an image of the object and its surroundings during the day and at night, in poor visibility conditions. Its main functions include assistance in visual search and rescue operations. The system must be installed on top of the bridge, on one of the sides. The EOS system is controlled from a dedicated console on the bridge. The system must have built-in gyroscopic stabilisation. The EOS system must be capable of storing at least 24 hours of high-quality video recordings within the system or on a separate device, which can be transferred to other systems. The EOS must have at least the following components: Colour TV camera. IR camera. General EOS requirements: The weight of the EOS equipment installed outdoors must not exceed 100 kg. The EOS platform must be of the two-axis type – elevation and azimuth – with the ability to rotate continuously through 360° in the horizontal plane and at least from -20° to 80° in the vertical plane. The sliding speed must be at least 60°/sec. Requirements for the EOS TV camera: Transmission of target and surrounding images to the operator with continuous optical zoom of at least 20 times. Resolution not less than 1920x1080. The transmitted image must be in colour with the option to switch to black and white mode. Viewing angle range of at least 2° to 50° (with zoom). The sensor amplification must have automatic and manual modes. Requirements for EOS IR cameras: The reliability of the infrared camera between failures must be no less than 4000 operating hours. Resolution of at least 640x480. Viewing angle range not less than 2°-30° (with continuous zoom). |  |
|  | The ship must have the necessary systems, means and equipment for extinguishing fires that have broken out on other ships/shore infrastructure, in ports and at sea, which function effectively in various hydrometeorological conditions and operate at maximum effective range. | Not less than FIFI1 |
|  | The ship must have the necessary systems, means and equipment that function effectively in various hydrometeorological conditions, operating at maximum acceptable loads, designed to tow ships in distress. | Efficiency is characterised by the effectiveness of the equipment under the relevant hydrometeorological conditions, without obscuring/affecting the physical parameters of the equipment or the primary functionality of the ship. |

Appendix No. 1

| SHIP INTEGRITY (minimum information) |  |  |
| --- | --- | --- |
| No. | Description of information | Comments |
|  | All ship compartments and spaces, their dimensions, volumes, watertight bulkheads, doors, windows, portholes, ventilation ducts. |  |
|  | Ship hatch mechanisms (including ship openings), including remote control panels. |  |
|  | Drainage systems, their layout, capacity (including emergency water pumping systems). |  |
| SHIP FLOATABILITY AND STABILITY |  |  |
|  | General provisions for ship buoyancy and stability. |  |
|  | General measures to prevent the ship from capsizing. |  |
|  | Ship loading and operating restrictions. |  |
|  | Crossflooding compartments. | If present on the ship |
|  | Stability approval data. |  |
|  | Maximum ship trim and draught limits. |  |
|  | Effect of free surface effect on the ship. |  |
|  | The effect of weights on the ship's heel. |  |
|  | Ship loading/unloading procedure. |  |
|  | Measures ensuring the buoyancy and stability of the ship. |  |
|  | Trim control system. |  |
|  | Negative metacentric height (Loll). |  |
|  | Hull strength (resistance). |  |
|  | Stability assessment and calculation tool. |  |
|  | Conditions restricting the ship's departure. |  |
|  | Detailed data: on the empty ship's displacement, its components; Ship stability hydrostatic curves; Maximum displacement of the ship, geometric centre (centroid), maximum moment of inertia of the free surfaces of tanks and compartments, separate tables for measuring the volume of each tank, geometric centres and moments of inertia of free surfaces, tables for calculating stability according to the (with examples of calculations and results), examples of ship stability demonstrating compliance with stability requirements, stability calculator for ship survivability with buoyancy and stability maintenance measures (with examples), ship accident. | If these summary data are stored in a stability and buoyancy computer, a secondary computer and paper versions of the data shall be available. |
| SHIP MANOEUVRABILITY |  |  |
|  | The following ship characteristics are provided: circular, turning, including sharp turns, braking and acceleration, operation in shallow waters, the effect of wind on manoeuvrability, minimum effective ship speed, operation in difficult hydrometeorological conditions (Ship swaying, flooding, sailing sideways, tilting when turning, the effect of oscillations and ways to reduce them). |  |