2 priedas

DRONO – ORO TAIKINIO PALEIDIMO PASLAUGOS

TECHNINĖ specifikacija

- Pirkimo objekto paskirtis:

  - Drono – oro taikinio (toliau – DOT) paleidimo paslauga (toliau – Paslauga) skirta DOT naikinimui oro taikinių nustatymo, sekimo ir neutralizavimo veiksmų taikomuosiuose tyrimuose įvairiomis antidroninės kovos priemonėmis.

- Bendri reikalavimai:

  - Paslaugos, nurodytos 1.1. p., atlikimo tiksli vieta ir laikas bus nurodyti pirkimo dokumentuose;

  - Visi su Paslaugos vykdymu susiję darbai ir techniniai aptarnavimai turi būti vykdomi tik tiekėjo pajėgumais ir resursais šaudymo personalo nurodytoje vietovėje;

  - Tiekėjui vykdant DOT aptarnavimo ar remonto darbus ar esant kitiems nenumatytiems atvejams nurodytoje taikinių paleidimo vietoje, užsakovui turi būti paleidžiamas kitas DOT. Atsitikus nenumatytoms aplinkybėms, tokioms kaip DOT nepakilimas į nurodytą aukštį ir nuotolį, sprogimas ore, per trumpas taikinio buvimas ore ar pan., tiekėjas turi pašalinti gedimus per ne ilgesnį kaip 30 min. laikotarpį arba užsakovui pateikti naują DOT;

  - Tiekėjui padarius žalą aplinkai, žmonėms, materialiam turtui ir t. t., tiekėjas atsako pagal Lietuvos Respublikoje galiojančius teisės aktus ir įstatymus;

  - Tiekėjo siūlomas DOT turi būti išbandytas ir dalyvavęs tokio pobūdžio tyrimuose ar pratybose;

  - Tiekėjas ir visi tiekėjo nurodyti asmenys, dalyvaujantys tyrimuose, privalo vykdyti visus užsakovo nurodymus susijusius su tyrimų organizavimu ir sauga;

  - Tiekėjas paleidžia ir operuoja orlaivį pagal šaudymo vadovo numatytą skrydžio planą (aukštis, atstumas, geografinė lokacija, greitis) arba pagal šaudymo vadovo komandas realiu laiku;

  - DOT paleidimas tiksliai nurodytu laiku (su leistina 60 sekundžių paklaida);

  - DOT turi būti dalinai išardomas arba surenkamas ne ilgiau kaip per 30 min.;

  - Naujo DOT paruošimo/pertaisymo paleidimui laikas – ne ilgesnis kaip 5 min.;

  - DOT turi būti paruošiamas (pertaisomas) paleidimui naudojantis tik žmonių pagalba, be papildomų kėlimo įrenginių;

  - Skrydžio valdymui turi būti galimybė automatiniam režimui nustatant skrydžio planą.

- Techniniai reikalavimai:

  - DOT paleidimo paslauga:

    - Didžiausias horizontalus greitis – ne mažesnis nei 140 km/h ir ne didesnis kaip 400 km/h;

    - Horizontalus kruizinis greitis – ne mažesnis nei 140 km/h ir ne didesnis nei 400 km/h;

    - Išskleisto DOT sparnų mojis  – nuo 2 metrų iki 6 metrų;

    - DOT privalo turėti „Delta“ formos sparną (sparno formą, kuri turi trikampį kontūrą, plačią pagrindo dalį ir siaurą galą);

    - DOT turi patikimai veikti nuo –10ºC iki +40ºC oro temperatūrai ir esant 10 m/s vėjo greičiui;

    - Skrydžio laikas – ne mažiau kaip 30 min.;

    - Spalva turi būti ryški (oranžinė), kad išsiskirtų iš dangaus ir gamtovaizdžio fono, tam, kad būtų galima jį stebėti ir sekti vizualiai;

    - DOT turi turėti ne mažesnį kaip 0,1 m2 radarų sistemoms matomą atspindžio plotą arba papildomą įrenginį, kuris padidina jo atspindžio plotą iki keliamų reikalavimų.

    - Numatytu laiku (laikas derinamas tyrimų dienomis) paleisti DOT keičiant taikinio skrydžio kelią pagal šaudymo vadovo nustatymus ne mažiau 4 valandas su pertraukomis, siekiant nustatyti atspindžio plotą, saugų greitį, aukštį bei trajektoriją.

- Garantiniai reikalavimai:

Nėra.

- Papildoma informacija:

BVPŽ kodas: 75221000–1.

PRIDEDAMA:

2.1. priedas. Techninė specifikacija drono – oro taikinio (DOT) paleidimo paslauga anglų k. (2 lapai).

2.1. priedas (Techninė specifikacija

drono – oro taikinio (DOT)

paleidimo paslauga anglų k.)

TECHNICAL SPECIFICATION

DRONE - AIR TARGET (DAT) LAUNCH SERVICE

- Purpose of the procurement:

  - The drone – air target (hereinafter referred to as DAT) launch service (hereinafter referred to as the Service) is intended for the destruction of DATs during applied research exercises on the detection, tracking, and neutralization of air targets using various counter-drone systems.

- General requirements:

  - The exact place and time of the services specified in section 1.1. will be detailed in the procurement documents;

  - All work and technical services related to the performance of the service must be performed solely by the supplier's capabilities and resources in the area specified by the shooting personnel;

  - If the supplier is performing DAT maintenance or repair work, or in other unforeseen circumstances at the specified target launch site, another DAT must be launched for the customer. In the event of unforeseen circumstances, such as the DAT not reaching the specified height and distance, exploding in the air, remaining in the air for too short a time, etc., the supplier must eliminate the malfunction within 30 minutes or provide the customer with a new DAT;

  - If the supplier causes damage to the environment, people, material property, etc., the supplier shall be liable in accordance with the laws and regulations in force in the Republic of Lithuania;

  - The DAT offered by the supplier must be tested and have participated in exercises of this nature;

  - The supplier and all persons designated by the supplier participating in the exercises must comply with all instructions given by the customer regarding the organization and safety of the exercises;

  - The supplier launches and operates the DAT according to the flight plan specified by the live firing exercise director (altitude, distance, geographical location, speed) or according to the live firing exercise director's commands in real time;

  - DAT must be launched at the exact specified time (with a permissible margin of error of 60 seconds);

  - The DAT may be partially disassembled or assembled in no more than 30 minutes;

  - The time required to prepare/recycle a new DAT for launch is no more than 10 minutes;

  - DAT must be prepared (replaced) for launch using only human assistance, without additional lifting equipment;

  - Flight control must be capable of setting the flight path in automatic mode.

- Technical requirements:

  - Launch service for DAT destruction:

    - Horizontal speed is not less than 140 km/h and not more than 400 km/h;

    - Horizontal cruising speed is not less than 140 km/h and not more than 400 km/h;

    - Wingspan of unfolded DAT wings – from 2 meters to 6 meters;

    - DAT must have a delta-shaped wing (a wing shape that has a triangular outline, a wide base, and a narrow tip);

    - The DAT must operate reliably at air temperatures ranging from -10ºC to +40ºC and wind speeds of up to 10 m/s;

    - Flight time – at least 30 min.;

    - The color must be bright (orange) so that it stands out against the sky and the landscape background, allowing it to be observed and tracked visually;

    - DAT must have a radar cross section area of at least 0.1 m2 or an additional device that increases its reflection area to the required level.

    - Launch the DAT at the scheduled time (to be determined during exercise day), changing the target flight path in accordance with the live firing exercise director settings for at least 4 hours with breaks, in order to determine the radar cross section, safe speed, altitude, and trajectory.

- Warranty requirements:

No.

- Additional information:

BVPŽ code: 75221000–1.