TECHNICAL SPECIFICATION

FOR THE LEASE OF HYDROGEN-POWERED BUSES

1. GENERAL PROVISIONS

1.1. The object of the procurement is the lease, without a driver, of single-deck, low-floor hydrogen-powered buses (vehicle category M3CE) intended for passenger transport (hereinafter – the Vehicles, Buses). The procurement object shall not be divided into lots.

1.1.1. The Supplier shall offer no fewer than two (2) and no more than ten (10) Buses. The minimum quantity to be offered is 2 units.

1.1.2. The quantity to be procured is the maximum quantity, i.e. it is planned to lease up to 10 Buses.

1.1.3. It is noted that, upon conclusion of the Contract, the Contracting Authority undertakes to lease the entire number of Buses offered by the Supplier for the full duration of the Contract. The settlement and pricing model (e.g. fee per kilometre driven, engine hour, or other market-accepted models) shall be determined in the procurement documents, taking into account the information obtained during market consultations. Detailed settlement arrangements shall be defined in the Contract.

1.3. The leased Buses shall be operated within the territory of the City of Vilnius.

1.4. The successful tenderer shall deliver and hand over the Buses to the Contracting Authority within 120 calendar days from the effective date of the Contract. No extension of the delivery deadline shall be permitted. The delivery address is Verkių g. 52, Vilnius.

1.5. All Buses offered shall be of the same manufacturer and model and shall have identical specifications and configuration.

1.6. The leased Buses shall comply with safety and environmental protection requirements set out in EU Directives, United Nations Economic Commission for Europe (UNECE) Regulations, as well as other requirements specified in the tender conditions.

1.7. The leased Buses shall be fit for passenger transport, complete, undamaged, free of advertising stickers, and compliant with the technical requirements set out in the Technical Specification. Their technical condition shall ensure safe commencement of operation immediately upon delivery to the Contracting Authority, without the need for any repair works.

1.8. The Contracting Authority shall have the right, at its own expense, to repaint all Buses in red or to apply red wrapping film. Upon expiry of the lease period, the original colour shall not be restored, and the film shall not be removed.

1.9. The Supplier shall, at its own expense, perform all actions related to the registration of the Vehicles, including their registration in the national vehicle register, indicating the Contracting Authority as the user of the Vehicles.

1.10. The Supplier undertakes to ensure that the Buses delivered have valid mandatory periodic technical inspection certificates issued in the Republic of Lithuania, with a validity period of no less than five (5) months from the date of handover. If the date of the next technical inspection falls within less than five (5) months from the handover date, the Supplier shall carry out the inspection at its own expense and submit the relevant technical inspection certificates to the Contracting Authority.

1.11. Upon handover of the Buses, the Contracting Authority shall inspect their technical condition at its production base and assess whether the Vehicles are in proper working order and comply with the Requirements for Technical Motor Vehicles and Their Trailers approved by Order No. 2BE-260 of the Director of the Lithuanian Transport Safety Administration dated 20 October 2020. Once it is confirmed that no deficiencies exist or that they have been remedied within the time limits specified in the Contract, a Bus handover–acceptance certificate shall be signed.

1.12. Any deficiencies identified during the inspection of the Buses shall be remedied by the Supplier within ten (10) calendar days (or within a shorter period specified by the Supplier).

1.13. At the request of the Contracting Authority, during the examination and evaluation of tenders, Suppliers shall be required to provide an opportunity for the Contracting Authority’s representatives, at their own expense, to inspect the Buses offered for lease and to assess their technical and aesthetic condition as well as their compliance with the descriptions provided in the tenders.

1.14. During the term of the lease agreement, the operational maintenance of the Buses shall be carried out by the Contracting Authority.

1.14. The Contracting Authority undertakes to insure all leased Buses with mandatory motor third-party liability insurance for the entire period of operation.

1.15. During the term of the lease agreement, the successful tenderer shall, at its own expense, carry out repairs of the main assemblies of the leased Vehicles. The main assemblies shall include the following components: the hydrogen fuel cell system, the gearbox, and the drive axle. Warranty repairs shall be completed within no more than thirty (30) calendar days.

1.16. The Contracting Authority shall, at its own expense, install an e-ticketing system in the leased Buses. After removal of the e-ticketing system, technical openings shall remain in the passenger handrails and on the driver’s dashboard.

1.17. Upon handover of the Buses to the Contracting Authority, the Supplier shall ensure the integration of Vilnius city routes into the passenger information system.

2. MANDATORY TECHNICAL REQUIREMENTS FOR THE LEASED VEHICLES

2.1. The essential (mandatory) characteristics are set out in Annex 1 to this Technical Specification.

Annex 1 to the Technical Specification

TABLE OF COMPLIANCE OF THE GOODS WITH THE SPECIFIED REQUIREMENTS

| No. | Requirement | Description | Specific Compliance Parameters Offered by the Supplier (To be completed by the Supplier) | Document Title, Page Number and/or Reference to Website Supporting Compliance of the Goods (To be completed by the Supplier) |
| --- | --- | --- | --- | --- |
| 1. | Bus Type | 1.1. A two-axle, low-floor, single-deck hydrogen-powered bus (vehicle category M3CE). 1.2. Suitable for intensive urban operation, including outdoor parking, under winter (down to −20 °C) and summer (up to +40 °C) temperature conditions (the Buses are intended to be operated in the City of Vilnius). 1.3. Certified in accordance with the requirements of Regulation (EU) 2018/858 of the European Parliament and of the Council of 30 May 2018, as amended. |  |  |
| 2. | Date of First Registration | The date of first registration shall be no earlier than 2021. |  |  |
| 3. | Engine Operating Hours | The operating hours shall not exceed 6,700. |  |  |
| 4. | Overall Length | From 11,500 mm up to the maximum permitted under the legislation of the Republic of Lithuania applicable to this type of vehicle on the date of issue of the vehicle’s Certificate of Conformity; |  |  |
| 5. | Width | From 2,490 mm to 2,550 mm (measured in accordance with Clause 6.2 of ISO 612:1978 or an equivalent standard); |  |  |
| 6. | Height | Not exceeding 3,450 mm (measured in accordance with Clause 6.3 of ISO 612:1978). |  |  |
| 7. | Floor Height | Low-floor, with a boarding/alighting height from the road surface not exceeding 350 mm. |  |  |
| 8. | Total Passenger Capacity | 8.1. At least 25 seated passenger seats. 8.2. At least one (1) space for persons with disabilities (with special needs using a wheelchair) or for a baby stroller (equipped with securing belts, rails, or other wheelchair/stroller restraint systems), located in the low-floor area between the bus axles. 8.3. The total number of installed seated and standing passenger places shall be not less than 75; spaces designated for persons with disabilities shall not be included in this number. |  |  |
| 9. | Passenger Doors | Pneumatic or electric doors located on the right-hand side of the Bus. The clear opening width of the first and second doors shall be not less than 1,200 mm. Doors shall be equipped with anti-pinch protection (in the event an obstacle is detected during closing, the doors shall reopen) and emergency door release mechanisms. It shall be possible to operate each door individually and all doors simultaneously. When any door is open, the braking system shall be activated to ensure that the Bus cannot move with at least one door open. A ramp for wheelchair users or baby strollers shall be provided. |  |  |
| 10. | Number of Axles | Two (2) axles. |  |  |
| 11. | Fuel Type | 11.1. Fuel: hydrogen (H₂). 11.2. Fuel storage: interconnected composite high-pressure cylinders. The fuel capacity shall ensure a minimum driving range of at least 350 km. 11.3. Hydrogen system operating pressure: 350 bar (at +15 °C), depending on the system type. 11.4. Refuelling time: not more than 20 minutes (in accordance with SAE J2601 and SAE J2799 protocols). 11.5. Refuelling connector type: N1. 11.6. Safety valves, sensors and fuses shall be installed. |  |  |
| 12. | Hydrogen Fuel Cell System | 12.1. The hydrogen fuel cell shall be an electrochemical device that generates electrical energy using hydrogen and oxygen. 12.2. The hydrogen fuel cell shall be supplied from the hydrogen storage system (fuel tanks). 12.3. The rated power of the hydrogen fuel cell shall be not less than 60 kW. 12.4. The hydrogen fuel cell shall operate without additional pre-heating within an ambient temperature range from −20 °C to +40 °C. |  |  |
| 13. | Suspension | Pneumatic suspension with automatic ride height control, kneeling and lifting functions, and a right-side kneeling function. |  |  |
| 14. | Steering System | Power-assisted steering system, left-hand drive. The steering wheel position shall be adjustable together with the driver’s instrument panel or independently. |  |  |
| 15. | Braking System | All brakes shall be of disc type. The vehicle shall be equipped with an Electronic Braking System (EBS) or an equivalent system. |  |  |
| 16. | Wheels and Tyres | Tyres suitable for operation in urban public transport conditions. Tyres shall be suitable for all-season use (M+S) and marked with the 3PMSF symbol. Remaining tyre tread life shall be not less than 50%. |  |  |

| No. | Bus Manufacturer / Name, Model | Vehicle Identification Numbers (VIN) | Date of First Registration | Operating Hours as of the Proposal Submission Date | Bus Colour | Bus Value, EUR excluding VAT |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | / |  |  |  |  |  |
| 2 | / |  |  |  |  |  |
| 3 | / |  |  |  |  |  |
| 4 | / |  |  |  |  |  |
| 5 | / |  |  |  |  |  |
| ... | / |  |  |  |  |  |

OTHER IMPORTANT INFORMATION REGARDING THE PLANNED PROCUREMENT

1. REQUIREMENTS FOR SUBMITTING A TENDER

The Tender shall include the information and documents completed in the Tender Form in accordance with the requirements of the Procurement Documents and:

☒ Documents may be submitted in Lithuanian and/or English.

☒ A completed Annex 1 to the Technical Specification – “Table of Compliance of the Goods with the Specified Requirements” shall be submitted.

Along with this, the Manufacturer’s or its authorised representative’s specifications, technical characteristics, and/or references to websites, and/or other types of documents shall be provided to demonstrate that the proposed Goods comply with the requirements set out in this Procurement’s Technical Specification.

2. METHOD OF EVALUATING TENDERS

2.1. Criterion for evaluating the most economically advantageous tender:

☒ Price or the ratio of costs to quality

EVALUATION CRITERIA OF TENDERS

This section sets out the criteria for evaluating the most economically advantageous tender, including their parameters, comparative weights, and the formulas used to calculate the economic advantage of the tenders. The evaluation of the most economically advantageous tender will be carried out in accordance with the evaluation criteria and their relative weights. Tenders submitted by Suppliers that do not meet the technical requirements will not be evaluated and will be rejected as non-compliant with the requirements of the Procurement Documents.

Table 1. Evaluation Criteria of Tenders and Their Relative Weights

Price-to-Quality Ratio: Criterion; Description of the Criterion; Relative Weight (points). K; Tender price, excluding VAT; x = 80. R; Operating hours; z = 9. S; Bus colour; v = 1. M; Number of buses (units); w = 10. - The economic advantage (C) of a tender is calculated by summing the scores of the individual evaluation criteria of the Supplier’s offer C1)=K+R+S+M (1) 1) The score of each evaluation criterion for each Supplier (C) shall be rounded to two decimal places, in accordance with standard arithmetic rules. Only two decimal places are legally significant. - Tender Price (K) The score for the Tender Price, excluding VAT is calculated as follows (1): # K = (KMIN / KP) × x (1), where # KMIN – the lowest tender price, excluding VAT, among all participants; # KP – the tender price, excluding VAT, of the evaluated Supplier; # x – the relative weight of the criterion x =80; - Operating Hours (R) The score for the Operating Hours criterion is calculated as: (3): R = ((i=1n6700 - Rp6700 - 0)÷n)×z (3), where Rp – the operating hours of the i-th bus in the Supplier’s offer at the date of proposal submission; 6700 – maximum allowed operating hours; 0 – minimum possible operating hours at the date of proposal submission; n – total number of buses; # z – relative weight of the criterion z =9; # For the “Operating Hours” criterion, the average operating hours of all buses offered by the Supplier are calculated, regardless of the number of buses. # If any bus in the Supplier’s offer exceeds the maximum allowed operating hours specified in the Technical Specification, the entire tender will be rejected as non-compliant. - The score for the Bus Colour criterion is calculated as (4): # S =(r÷n) × v (4), where # r – number of buses in the Supplier’s offer painted red on the exterior;, n – total number of buses; # v – relative weight of the criterion v = 1; - The score for the Number of Buses criterion is calculated as (5): # N = ( n / nmax ) × y (6), where: # n – number of buses offered by the Supplier; # nmax – largest number of buses offered by any Supplier (not exceeding 10 units); # y – relative weight of the criterion, y = 10. Only tenders offering between 2 and 10 buses will be evaluated..

  - Applicable Environmental Requirements:

This procurement is subject to green procurement requirements, in accordance with the Order No. D1-508 of 28 June 2011 of the Minister of Environment of the Republic of Lithuania on the “Approval of the Procedure for the Application of Environmental Protection Criteria in Green Public Procurement” (hereinafter – the “Procedure”). By entering into the contract, the parties undertake to comply with the following environmental requirements:

    - In accordance with Section 4.4.4.1 of the Procedure, the Supplier shall reduce paper consumption and avoid unnecessary copying and printing of documents. All documents related to the performance of the contract shall be submitted to the Contracting Authority in electronic format only, unless otherwise provided in the contract or its annexes. In exceptional cases, contract-related documents may be submitted in paper format if required by law or if explicitly requested by the Contracting Authority. In such cases, only recycled paper meeting the minimum environmental criteria set out in Annex 2 of the Procedure shall be used.

    - This procurement is conducted as a green public procurement in accordance with Annex 2, Section 11 of the Procedure for the Application of Environmental Protection Criteria in Green Public Procurement, approved by Order No. D1-508 of 28 June 2011 of the Minister of Environment of the Republic of Lithuania, since the subject of the procurement is the rental of fully zero-emission vehicles, specifically hydrogen-powered vehicles of category M3.

SPECIAL REQUIREMENTS FOR CONTRACT PERFORMANCE

- CONTRACT DURATION

☒ 12 (twelve) months

Planned extension(s):

☒ Upon the expiry of the 12 (twelve) month Goods ordering period, the contract may be automatically extended twice, each time by 12 (twelve) months. If neither party notifies the other in writing at least 30 (thirty) calendar days before the end of the Goods ordering period of its intention to terminate the contract unilaterally, the contract shall be deemed extended for an additional 12 (twelve) months of the Goods ordering period. The total duration of the contract, taking into account the maximum Goods ordering periods, shall not exceed 36 (thirty-six) months from the date of entry into force of the contract.

SPECIAL REQUIREMENTS FOR SUPPLIER QUALIFICATION

☒ No qualification requirements are set.

https://www.e-tar.lt/portal/lt/legalAct/TAR.4B60A8C9678B/asr