INSTALLATION AND ASSEMBLY OF RADIO MONITORING STATIONS

TECHNICAL SPECIFICATION

- The Supplier must specify the exact manufacturer (manufacturer‘s name) and exact model of the proposed Item in the tender and technical specifications.

- The tender must be accompanied by the manufacturer‘s or its authorised representative‘s specifications, technical characteristics and/or references to the website, and/or other documents confirming that the proposed Item meets the requirements specified in the Technical Specification for this procurement.

| No. | Characteristic name | Characteristic value, parameters | Enter the specific conformity parameters proposed for the Item. Write “Complies“ or “Yes“ only when the requirement is not expressed in physical amounts. | Document title, page number and/or exact reference to the website for justification of the Item conformity |
| --- | --- | --- | --- | --- |
| I | II | III | IV | V |
| Item name: purchase, installation and assembly of radio monitoring station equipment |  |  | The Supplier shall indicate the manufacturer, name and quantity (quantities) of the Item here. If the Seller and/or the manufacturer of the Item uses third-party and/or manufacturers‘ devices, hardware and/or software (hereinafter - the equipment (together with devices)) to manufacture the Item, this equipment shall be specified as a parameter of conformity of the Item. |  |
|  | General technical requirements for the purchase of equipment |  |  |  |
|  | Purpose of the Item | The radio spectrum monitoring system is designed for monitoring, control and analysis. The system consists of different types of spectrum monitoring stations, devices and software that ensure the reception, measurement, analysis and recording of radio signals across the entire specified frequency range. Centralised management and data processing software allows remote control of stations, collection and analysis of measurement data, detection, identification and localisation of radio sources and interference. The system is used for effective monitoring of radio spectrum resource utilisation and detection of possible violations. |  |  |
|  | Comments indicating compliance | By indicating compliance with “Compliant“ or “Yes“ (Compliant or Yes), indicating the physical quantity and/or measured parameters as compliance with the requirement set out in this specification, it is permitted to add a comment in Lithuanian and/or English, including an indication of the third-party equipment used. If “Partially compliant“ or similar is indicated, it is necessary to add an explanation and documentation that would allow compliance to be assessed (e.g. the parameter is not used in the Baltic region or in EU countries, therefore it cannot be measured in Lithuania, etc.). etc.). |  |  |
|  | Calibration | For a broadband spectrum receiver: Calibration by the equipment manufacturer, followed by the submission of an official manufacturer‘s calibration certificate and documented calibration values. During the warranty period, the Supplier must calibrate the receiver periodically every 2 years. All periodic calibrations of the receiver during the warranty period must be included in the Supplier‘s tender price. For the direction finder: Calibration by the equipment manufacturer, followed by the issuance of an official manufacturer‘s calibration certificate and documented calibration values. During the warranty period, the Supplier must periodically calibrate the direction finder every 2 years. All periodic calibrations of the direction finder during the warranty period must be included in the Supplier‘s tender price. Receiver: Calibration by the equipment manufacturer, followed by the issuance of an official manufacturer‘s calibration certificate and documented calibration values. During the warranty period, the Supplier must periodically calibrate the receiver every 2 years. All periodic calibrations of the receiver during the warranty period must be included in the Supplier‘s tender price. The Supplier shall dismantle the receiver, replace it with a spare and reinstall it after calibration (at no additional cost). |  |  |
|  | Electromagnetic compatibility | The proposed Item must comply with all applicable European Union and Lithuanian legal requirements, be marked with the CE mark and have an EU declaration of conformity issued by the manufacturer or its authorised representative in accordance with the applicable standards. |  |  |
|  | Composition of the main parts of the Item | The equipment set consists of the following devices: |  |  |
|  |  | • receiver with a directional antenna with an operating frequency of not less than 20 MHz to 8 GHz, set of 3 units; | ENTER | ENTER |
|  |  | • broadband receiver with directional antenna(s) with an operating frequency of not less than 9 kHz to 8 GHz, set of 3 pcs. | ENTER | ENTER |
|  |  | • radio monitoring data recording device – 1 unit; | ENTER | ENTER |
|  |  | • receiver with omnidirectional antenna(s) with an operating frequency of not less than 9 kHz to 8 GHz, set – 3 units; | ENTER | ENTER |
|  |  | • antenna switch - 3 units; | ENTER | ENTER |
|  |  | • antenna rotator - 3 units; | ENTER | ENTER |
|  |  | • Remote radio monitoring receiver with antenna set – 8 units; | ENTER | ENTER |
|  |  | • All necessary accessories (adapters for mounting on the tower, signal, control and power cables and their connectors, GNSS antennas, etc.) and auxiliary materials ensuring the operation of the system are supplied with the Item. | ENTER | ENTER |
|  | Structural diagrams of stationary radio monitoring stations and remote radio monitoring stations | Provided in Annex 1 to these Technical Specifications. |  |  |
|  | Language | Compliance with the requirements set out in these Technical Specifications must be provided in Lithuanian or English. |  |  |
|  | Technical documentation | Technical documentation for the Item must be provided, including technical and functional data for the Item, technical and other conditions for expansion and supplementation (Technical Data Sheet). Technical documentation for the use of the Item intended for the user must be provided, covering installation, connection diagrams, use, functionality, change management, conditions of use, measurement management and execution, scenario preparation, report preparation, use of integrated third-party solutions (e.g. working with maps), additions, extensions including descriptions of cloud services offered/provided by the manufacturer of the Item, terms of use, technical documentation for independent use of the service intended for the user, in general – user manual or manuals (in general User Manuals, Installation Manuals, and/or Operating Manuals and/or Service Manuals, etc.). The specified documentation must be provided in printed and/or digital form, and the specified documentation may be provided as an exact URL link on the Internet or by providing access to the manufacturer‘s online resource intended for users, specifying the data required for login. Technical documents must be available in non-discriminatory, universally accessible data file formats (e.g. .pdf, .doc, .docx, .xls, .xlsx, .ppt, .pptx, .txt, etc.). The Purchaser has the right to request the original documents. | ENTER | ENTER |
|  | Technical documentation for the proposed Item when submitting a tender | When submitting a tender, it is necessary to attach technical descriptions, instructions, etc. of the proposed equipment (including third-party equipment) so that the contracting authority (hereinafter referred to as the Purchaser) can assess the Seller‘s declared compliance with the requirements. The documentation may also be submitted on a digital medium, and additional URL (Uniform Resource Locator) links to internet resources may also be provided. | ENTER | ENTER |
|  | Detailed work plan | Within 30 days of the date of entry into force of the contract, the Supplier must prepare, submit and agree with the Purchaser a detailed work plan covering the stages of installation, testing and acceptance testing of the monitoring systems. This plan must show all project activities, their interdependencies and responsibilities in terms of time. The plan must clearly indicate the essential (critical) stages. The Supplier‘s plan must clearly indicate the start and end dates of each activity. |  |  |
|  | Testing | Before signing a Deed of Acceptance and Transfer, the Supplier must perform testing to assess whether the delivered, installed and commissioned equipment meets the requirements set out in this specification. The schedule for the equipment acceptance testing procedure must be agreed in advance by mutual agreement. |  |  |
|  | Training | The Purchaser must be provided with technical training material and/or documentation (Training or Learning manual) and/or video material for training purposes and other necessary documentation required to train the user in the use of the purchased equipment by preparing measurement scenarios and/or scripts, performing measurements and processing measurement results (filtering, comparison, reports, display; preparation of measurement protocols, etc.). The Supplier shall provide practical training (instruction) on the use of the equipment to at least 10 (ten) of the Purchaser‘s employees, and the duration of the practical training (instruction) shall be no less than 16 hours. The Purchaser shall provide suitable premises for the training in Vilnius. The practical training must be completed within 3 (three) months from the date of installation of the equipment in Vilnius. After installing the equipment in Kaunas, the Supplier shall provide practical training (instruction) on the use of the equipment to at least 5 (five) of the Purchaser‘s employees, with the practical training (instruction) lasting no less than 8 hours. The Purchaser shall provide suitable premises for the training in Kaunas. The practical training must be completed within 3 (three) months from the date of installation of the equipment in Kaunas. After installing the equipment in Klaipeda, the Supplier shall provide practical training (instruction) on the use of the equipment to at least 5 (five) of the Purchaser‘s employees, with the practical training (instruction) lasting no less than 8 hours. The Purchaser shall provide suitable premises for the training in Klaipeda. The practical training must be completed within 3 (three) months from the date of installation of the equipment in Klaipeda. The Supplier shall provide practical training (instruction) on the use of remote radio monitoring station equipment to at least 10 (ten) of the Purchaser‘s employees, with the practical training (instruction) lasting no less than 16 hours. The Purchaser shall provide suitable premises for the training in Vilnius. The practical training must be completed within 3 (three) months from the date of installation of the remote radio monitoring station equipment in Vilnius. |  |  |
|  | Data protection and cyber security | The software must have access control, user authentication and data protection mechanisms. Data transmission must be encrypted using a secure virtual private network (VPN) connection. The Purchaser shall only provide the connection line, and the Supplier must supply, configure and install the network equipment. |  |  |
|  | Delivery schedule and payment terms | Commitments of the Purchaser: Stage I: - Delivery of 1 set of stationary radio monitoring station equipment (Vilnius) and 3 sets of remote radio monitoring station equipment – 9 months from the date of entry into force of the contract; - Installation and testing of equipment – 12 months from the date of entry into force of the contract. Stage II: - Delivery of 1 set of stationary radio monitoring station equipment (Kaunas) – 13 months from the date of entry into force of the contract; - Installation and testing of equipment – 16 months from the date of entry into force of the contract. Stage III: - Delivery of 1 set of stationary radio monitoring station equipment (Klaipeda) – 17 months from the date of entry into force of the contract - Installation and testing of equipment – 20 months from the date of entry into force of the contract. - Delivery of centralised control point equipment – 9 months from the date of entry into force of the contract - Installation of the centralised control point – 12 months from the date of entry into force of the contract; - Stage I: Connection of the stationary station to the control point and testing – 12 months from the date of entry into force of the contract; - Stage I: Connection of 3 sets of remote stations to the control point and testing – 14 months from the date of entry into force of the contract; - Stage II Connection of the stationary station to the control point and testing – 16 months from the date of entry into force of the contract; - Phase III: connection of the stationary station to the control point and testing – 20 months from the date of entry into force of the contract; - Final testing of the centralised control point – 30 months from the date of entry into force of the contract. Non-binding orders from the Purchaser: - Delivery of 5 sets of remote radio monitoring station equipment – 6 months from the date of order placement. - Installation and testing of 5 sets of remote radio monitoring station equipment – 3 months from the date of delivery of the equipment. |  |  |
|  | Quantity of identical components purchased | Stationary radio monitoring station equipment – 3 sets; remote radio monitoring station equipment – maximum quantity 8 sets, but not less than 3 sets; The Service does not undertake to purchase all sets of remote radio monitoring station equipment |  |  |
|  | Condition of the Item | The Item must be new, unused, and may not contain renewed or refurbished components. The equipment must not cause radio interference to other radio monitoring equipment operating nearby. The Item must be fit for its intended purpose, without any hidden defects or disabled functions and/or licences that would prevent the Item from being used for its intended purpose in accordance with the requirements set out in this technical specification, or that would reduce the usefulness of the Item. |  |  |
|  | Warranty requirements |  |  |  |
|  | Warranty period | The warranty period for software and hardware and their installation must be no less than 60 months. The warranty period for specific software and hardware and their installation shall commence on the date of Deed of Acceptance and Transfer for the completion of each installation stage. |  |  |
|  | Repair of technical equipment during the warranty period | - During the warranty period, equipment repairs must be carried out within 20 days of the submission of the request. If the repair takes longer than 20 days, a replacement device must be provided. - When replacing individual equipment parts or individual devices with others, new equipment parts and/or devices must be used, the technical parameters of which must not be worse than those before the failure. |  |  |
|  | Provision of updates | - Updates to software, device firmware and technical documentation, as well as the provision of the latest versions, must be provided free of charge throughout the warranty period. - The warranty period must also include free correction of software errors and security vulnerabilities. - All software versions must be updated to the latest version at the end of the final stage. - Response time (the time from when the Purchaser informs the Supplier about the equipment malfunction to when the Supplier informs the Purchaser that it is actually starting to fix the malfunction) – 48 hours; - Fault repair time (the time from when the Supplier informs the Purchaser that it has actually started repairing the fault to when the Supplier informs the Purchaser that it has repaired the fault) – 10 days. - If the Supplier is unable to remedy the fault within the specified time for objective reasons and/or reasons beyond the Supplier‘s control, it must immediately inform the Purchaser in writing (by e-mail) of such circumstances and reasons. After the Purchaser has assessed the validity of the circumstances and reasons indicated, the Purchaser and the Supplier may, by mutual agreement, set a different time for the elimination of the malfunction than that specified. |  |  |
|  | Restoration of measurement and radio monitoring data, software settings and functionality | - When assembling the Item, provision must be made for the free restoration of radio monitoring and measurement data (hereinafter referred to as data), software settings and functionality in measurement and recording devices and in the software provided by the Supplier (hereinafter collectively referred to as the Equipment) with the previous functionality and licences (e.g. by providing new software and licences or restoring the software and activating the previous licences) - If this is not possible, the Equipment must have a backup feature that allows periodic backup of all equipment software (including licences and settings) and stored data, from which the full functionality of the Item can be restored to its pre-failure state; - These requirements also apply if it is necessary to replace memory media or individual parts of the Equipment and/or individual devices because they cannot be used in the repaired Equipment due to software and technical changes made during the repair. - restoration of data obtained during radio monitoring and measurements (hereinafter referred to as data), software settings and functionality in measurement and recording devices and in the software provided by the Supplier (hereinafter collectively referred to as the Equipment) with the previous functionality and licences must work in the Equipment returned after repair, regardless of whether the Item (or Equipment) were sent for repair with or without memory media. |  |  |
|  | Requirements for memory media maintenance and data protection during the warranty period | - During the warranty period, in the event of a failure of technical equipment or other Supplier devices provided to meet the requirements of the technical specifications, magnetic disks, hard disks or other memory media (hereinafter collectively referred to as Memory Media) must be replaced with new ones; - Faulty Memory Media must be destroyed at the Purchaser‘s premises (the Supplier may be present if they wish). Destroyed Memory Media shall not be returned to the Supplier. - If the equipment is transferred to the Supplier and/or the manufacturer of the Item for repair, and the memory media cannot be mechanically removed without damaging the equipment, the Supplier and/or the manufacturer of the Item must provide the Purchaser with data protection confidentiality guarantees that the data stored on the memory media will not be accessible to third parties and will not be used without the Purchaser‘s permission; - If, in the event of a malfunction, the equipment and/or individual devices are sent to the Supplier (or the manufacturer of the Item) for repair, the Purchaser shall deliver the Item without the memory media installed in them, if they can be mechanically removed without damaging the devices; - If the Supplier indicates that it is necessary to submit the Item with memory media in order to perform a high-quality repair, the Supplier shall provide the Purchaser with all the means necessary to enable the Purchaser to remove any data sensitive to the Purchaser from the equipment (e.g. logins to the Purchaser‘s network and/or data on the network or cloud resources used by the Purchaser, etc.), except for data that the Purchaser has decided not to remove; - The Supplier‘s confidentiality obligations and data protection shall apply to the data that has not been removed, as regulated by the laws in force in the European Union. |  |  |
|  | Supplier help centre services | The following support centre services provided by the manufacturer of the Item must be provided (but not limited to those listed): - Management of granted licences (other software may also be used for this purpose); - Consultations on features, functionality, software operation, measurements and data processing; - Analysis and elimination of operational errors; - Solutions to problems arising from improper operation (if the declared functions do not work); - Correction of deficiencies in technical documentation (e.g. incomplete configuration description in the submitted version); - If necessary, remote diagnostics and software updates must be provided via a secure communication channel |  |  |
|  | Support after the warranty period | Support for software (updates) and hardware spare parts after the warranty period must be provided for at least 60 months, if the technical capabilities of the equipment manufacturer allow this. If this is not possible, other specific technical solutions must be offered. At the end of this period, support services must remain available for at least the portion of products purchased by the Purchaser, but the response times, consultations, error corrections, software updates, explanations, etc., and the scope of cloud services shall be determined by the Supplier. | ENTER | ENTER |
|  | Technical requirements for the direction finder (receiver) |  |  |  |
| - Frequency range | Frequency range | Not narrower than 20 MHz to 8 GHz. | ENTER | ENTER |
|  | Minimum frequency tuning resolution | Not greater than 1 Hz. | ENTER | ENTER |
|  | Frequency determination error | Not more than 2 × 10–7. | ENTER | ENTER |
|  | Real-time intermediate frequency bandwidth | Not less than 40 MHz | ENTER | ENTER |
|  | Third-order intercept (TOI) | Typical value not less than: -1 dBm at frequencies up to 5.3 GHz; -6 dBm at frequencies above 5.3 GHz. | ENTER | ENTER |
|  | Displayed Average Noise Level (DANL) | Typical value not exceeding: -162 dBm at frequencies up to 5.3 GHz; -157 dBm at frequencies above 5.3 GHz. | ENTER | ENTER |
|  | Noise figure | Typical value not exceeding: 12 dB at frequencies up to 3.3 GHz; 16 dB at frequencies from 3.3 to 5.3 GHz; 20 dB at frequencies above 5.3 GHz. | ENTER | ENTER |
|  | Image rejection | Typical value not less than: 110 dB at frequencies up to 1 GHz; 80 dB at frequencies above 1 GHz. | ENTER | ENTER |
|  | Radio signal direction finding sensitivity, vertical polarisation (system direction finding antenna and receiver) | Signal electromagnetic field strength dBµV/m: in the frequency band from 50 MHz to 3 GHz not exceeding 5; in the frequency band from 3 GHz to 4.5 GHz, not exceeding 10; in the frequency band from 4.5 GHz to 7 GHz, not exceeding 15. | ENTER | ENTER |
|  | Direction finding sensitivity, horizontal polarisation (system consisting of a direction-finding antenna and receiver) | Signal electromagnetic field strength dBµV/m: in the frequency band from 50 MHz to 3 GHz not exceeding 5; in the frequency band from 3 GHz to 4.5 GHz, not exceeding 15; in the frequency band from 4.5 GHz to 6.5 GHz, not exceeding 25. | ENTER | ENTER |
|  | Sensitivity | When demodulating an FM signal (at SINAD = 20 dB, fmod = 1 kHz): in the frequency band from 20 MHz to 35 MHz, with a bandwidth of 9 kHz and a deviation of 2.4 kHz, not exceeding -97 dBm; in the frequency band from 35 MHz to 6 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not exceeding -107 dBm; in the frequency band from 6 GHz to 7.5 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not exceeding -102 dBm; in the frequency band from 7.5 GHz to 8 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not exceeding -92 dBm. | ENTER | ENTER |
|  | Detect signals with 100% probability of interception at the specified receiver parameters: FFT real-time IF panorama (span) no less than 40 MHz wide, resolution bandwidth (RBW = 2 MHz) | No worse than 2 µs. | ENTER | ENTER |
|  | Intermediate frequency band width shape factor | Signal bandwidth at 3 dB and 50 dB attenuation limit, coefficient not greater than 1:1.6 for filters with an operating frequency up to 40 MHz | ENTER | ENTER |
|  | Analogue/digital (A/D) converter (ADC) resolution | Not less than 14 bits. | ENTER | ENTER |
|  | Maximum signal level at input | Not less than +20 dBm. | ENTER | ENTER |
|  | Input signal level attenuator | Not less than 30 dB with a switching step of no more than 2 dB (switching can be selected manually or automatically). | ENTER | ENTER |
|  | Signal level resolution | No more than 0.1 dB | ENTER | ENTER |
|  | Signal level measurement error | Not more than ±3 dB (in the temperature range from –10 °C to +55 °C) | ENTER | ENTER |
|  | Signal level measurement detectors | Not less than: peak, root mean square (RMS), average. | ENTER | ENTER |
|  | Spectrum scanning speed, displaying the entire frequency range | Not less than: 60 GHz/s (at 1 MHz RBW); 55 GHz/s (at 100 kHz RBW); 45 GHz/s (at 25 kHz RBW). | ENTER | ENTER |
|  | Frequency scan with user-selectable start/end frequency and step width | Not less than 2000 frequencies/s. | ENTER | ENTER |
|  | Number of radio frequencies (addresses) stored in memory | At least 1000 radio frequencies (addresses). | ENTER | ENTER |
|  | Memory scan speed of radio frequencies (addresses) stored in memory | Scanning of radio frequencies (addresses) stored in memory (not less than 1000 addresses/s). | ENTER | ENTER |
|  | Bandwidth of the IQ data output connection for wide spectrum (at least 40 MHz bandwidth) | Not less than 10 GBit. | ENTER | ENTER |
|  | Antenna connectors | At least 2 connectors, input impedance 50 Ω. | ENTER | ENTER |
|  | GNSS antenna connector | There must be a connection to an external active or passive GNSS antenna that ensures the reception and processing of satellite navigation signals from at least the following navigation systems: GPS, GLONASS, BeiDou, Galileo. | ENTER | ENTER |
|  | Direction determination error in AOA mode (system: direction determination antenna and receiver) | No greater than ±2º RMS in mode. | ENTER | ENTER |
|  | Minimum signal duration for direction finding | Not longer than 2 ms | ENTER | ENTER |
|  | Squelch modes | This functionality must be ensured. | ENTER | ENTER |
|  | Methods of recording data to internal memory and/or external recording device | Signal spectrum, IQ data (IQ snapshot), demodulated signal audio. | ENTER | ENTER |
|  | Direction-finding device (receiver) start-up/reload time | Must be fully operational no later than 180 seconds after the start of the reboot. | ENTER | ENTER |
|  | General functional requirements for the direction finder (+receiver) |  |  |  |
|  | Signal direction/source location determination functionalities | Must support AOA (Angle of Arrival), TDOA (Time Difference of Arrival) and hybrid (AOA together with TDOA) functionalities. | ENTER | ENTER |
|  | Signal source location using TDOA and hybrid functionalities | TDOA and hybrid location determination functions must be ensured by connecting the direction finders to a network of at least three remote radio monitoring receivers. | ENTER | ENTER |
|  | Types of demodulated signals | At least AM, FM, PM, pulse, I/Q, LSB, USB, CW. | ENTER | ENTER |
|  | Ability to perform measurements in accordance with ITU recommendations | Must have the functionality to perform measurements in accordance with ITU recommendations, including: bandwidth measurements in accordance with ITU-R SM.443-4 (02/2007); frequency measurements in accordance with ITU-R SM.377; modulation measurements in accordance with ITU-R SM.328; level and field strength measurements in accordance with ITU-R SM.378-7. | ENTER | ENTER |
|  | Direction finding functionality in the absence of a GNSS signal | Direction finding functionality must be ensured even in the absence of a GNSS signal. It must be possible to enter the coordinates of the equipment location. | ENTER | ENTER |
|  | Software | Used to connect and control the direction-finding device (receiver) and remote radio monitoring receivers remotely, and to select measurement actions and settings. | ENTER | ENTER |
|  | Direction finding modes | Direction finding functionality must be ensured in fixed frequency mode (FFM), frequency scan mode, and memory scan mode. | ENTER | ENTER |
|  | Wideband signal direction finding | Direction finding functionality must be ensured in fixed frequency mode (FFM). | ENTER | ENTER |
|  | DF band width | DF band width determination methods: manual; automatic (broadband signal direction determination depending on FFT selectivity). | ENTER | ENTER |
|  | General requirements for the direction finder (+receiver) |  |  |  |
|  | Operating temperature range | Not less than -30 °C to +55 °C. | ENTER | ENTER |
|  | Remote control via LAN interface | RJ-45, 1 Gbit WAN. | ENTER | ENTER |
|  | Power supply | 230 V 50 Hz AC mains. | ENTER | ENTER |
|  | Set of remote radio monitoring receiver control commands | The Supplier must provide a complete set of remote radio monitoring receiver control commands, including both remote and local control commands, together with the direction-finding device (receiver). The command set must include a description of the command syntax, parameters and response structure. The set of control commands provided must ensure the integration of all equipment into the Purchaser‘s control infrastructure. | ENTER | ENTER |
|  | Technical requirements for the spectrum monitoring receiver |  |  |  |
|  | Frequency range | Not less than 9 kHz to 8 GHz. | ENTER | ENTER |
|  | Minimum frequency tuning resolution | Not greater than 1 Hz. | ENTER | ENTER |
|  | Frequency determination error | Not greater than 2 × 10-7. | ENTER | ENTER |
|  | Real-time intermediate frequency bandwidth | Not less than 40 MHz | ENTER | ENTER |
|  | Third-order intercept (TOI) | Typical value not less than: -1 dBm at frequencies up to 5.3 GHz; -6 dBm at frequencies above 5.3 GHz. | ENTER | ENTER |
|  | Displayed Average Noise Level (DANL) | Typical value not exceeding: -162 dBm at frequencies up to 5.3 GHz; -157 dBm at frequencies above 5.3 GHz. | ENTER | ENTER |
|  | Noise figure | Typical value not exceeding: 12 dB at frequencies up to 3.3 GHz; 16 dB at frequencies from 3.3 to 5.3 GHz; 20 dB at frequencies above 5.3 GHz. | ENTER | ENTER |
|  | Image rejection | Typical value not less than: 110 dB at frequencies up to 1 GHz; 80 dB at frequencies above 1 GHz. | ENTER | ENTER |
|  | Sensitivity | When demodulating an FM signal (at SINAD = 20 dB, fmod = 1 kHz): in the frequency band from 20 MHz to 35 MHz, with a bandwidth of 9 kHz and a deviation of 2.4 kHz, not exceeding -97 dBm; in the frequency band from 35 MHz to 6 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not exceeding -107 dBm; in the frequency band from 6 GHz to 7.5 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not exceeding -102 dBm; in the frequency band from 7.5 GHz to 8 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not exceeding -92 dBm. | ENTER | ENTER |
|  | Detect signals with 100% probability of reception at the specified receiver parameters: FFT real-time IF panorama (span) no less than 40 MHz wide, resolution bandwidth (RBW = 2 MHz) | No worse than 2 µs. | ENTER | ENTER |
|  | Types of demodulated signals | At least AM, FM, PM, pulse, I/Q, LSB, USB, CW. | ENTER | ENTER |
|  | Automatic frequency control (AFC) | Must have this functionality. | ENTER | ENTER |
|  | Intermediate frequency band width shape factor | Signal bandwidth at 3 dB and 50 dB attenuation limits, coefficient not exceeding 1:1.6 for filters with an operating frequency of up to 40 MHz | ENTER | ENTER |
|  | Analogue/digital (A/D) converter (ADC) resolution | Not less than 14 bits. | ENTER | ENTER |
|  | Maximum signal level at input | Not less than +20 dBm. | ENTER | ENTER |
|  | Input signal level attenuator | Not less than 30 dB with a switching step of no more than 2 dB (switching can be selected manually or automatically). | ENTER | ENTER |
|  | Signal level resolution | No more than 0.1 dB | ENTER | ENTER |
|  | Signal level measurement error | Not more than ±3 dB (in the temperature range from –10 °C to +55 °C). | ENTER | ENTER |
|  | Signal level measurement detectors | Not less than: peak, root mean square (RMS), average. | ENTER | ENTER |
|  | Spectrum scanning speed, displaying the entire frequency range | Not less than: 60 GHz/s (at 1 MHz RBW); 55 GHz/s (at 100 kHz RBW); 45 GHz/s (at 25 kHz RBW). | ENTER | ENTER |
|  | Frequency scan with user-selectable start/end frequency and step width | Not less than 2000 frequencies/s. | ENTER | ENTER |
|  | Number of radio frequencies (addresses) stored in memory | At least 1000 radio frequencies (addresses). | ENTER | ENTER |
|  | Scanning speed of radio frequencies (addresses) stored in memory (memory scan) | Scanning of radio frequencies (addresses) stored in memory (not less than 1000 addresses/s). | ENTER | ENTER |
|  | Bandwidth of the IQ data output connection for wideband spectrum (at least 40 MHz bandwidth) | Not less than 10 GBit | ENTER | ENTER |
|  | Antenna connectors | At least 2 connectors, input impedance 50 Ω. | ENTER | ENTER |
|  | GNSS antenna connector | There must be a connector for an external active or passive GNSS antenna that ensures the reception and processing of satellite navigation signals from at least the following navigation systems: GPS, GLONASS, BeiDou, Galileo. | ENTER | ENTER |
|  | Record spectrum data for later analysis | Must have this capability. | ENTER | ENTER |
|  | Ability to record radio spectrum (FFT) | Recording to internal memory and/or recording device. | ENTER | ENTER |
|  | Ability to record signal spectrum (IQ data) | Recording to internal memory and/or recording device. | ENTER | ENTER |
|  | Radio monitoring software for remote connection and control of the device and selection of measurement actions and settings | This option is available. | ENTER | ENTER |
|  | Remote control | This option is available. | ENTER | ENTER |
|  | Ability to perform measurements in accordance with ITU recommendations | Must have the functionality to perform measurements in accordance with ITU recommendations, including: bandwidth measurements in accordance with ITU-R SM.443-4 (02/2007); frequency measurements in accordance with ITU-R SM.377; modulation measurements in accordance with ITU-R SM.328; level and field strength measurements in accordance with ITU-R SM.378-7. | ENTER | ENTER |
|  | Storage of radio monitoring and measurement results. | A dedicated external or internal recording device shall be used. | ENTER | ENTER |
|  | Receiver start-up/recovery time | Must be fully operational within 180 seconds of start-up. | ENTER | ENTER |
|  | General requirements for spectrum monitoring receivers |  |  |  |
|  | Operating temperature range | Not less than -30 °C to +55 °C. | ENTER | ENTER |
|  | Remote control via LAN interface | RJ-45, 1 Gbit WAN. | ENTER | ENTER |
|  | Power supply | 230 V 50 Hz AC mains | ENTER | ENTER |
|  | Technical requirements for broadband receivers |  |  |  |
|  | Frequency range | Not less than 9 kHz to 8 GHz. | ENTER | ENTER |
|  | Sensitivity | In low noise mode: when demodulating an FM signal (at SINAD = 20 dB, fmod = 1 kHz): in the frequency band from 20 MHz to 6 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not exceeding -108 dBm; when demodulating an AM signal (at SINAD = 12 dB, fmod = 1 kHz): in the frequency band from 20 MHz to 6 GHz, with a bandwidth of 6 kHz, not exceeding -108 dBm. | ENTER | ENTER |
|  | Noise figure | Not greater than: 15 dB at frequencies up to 20 MHz; 13 dB at frequencies from 20 MHz to 650 MHz; 15 dB at frequencies from 650 MHz ≤ f < 3.6 GHz; 16 dB at frequencies 3.6 GHz ≤ f < 6 GHz; 12 dB at frequencies above 6 GHz. | ENTER | ENTER |
|  | Displayed Average Noise Level (DANL, normalised to 1 Hz) value. | Not greater than: -159 dBm at frequencies up to 20 MHz; -162 dBm at frequencies from 20 MHz to 3.6 GHz; -157 dBm at frequencies above 3.6 GHz. | ENTER | ENTER |
|  | Third-order intercept ( TOI) in low distortion mode | Typical value not less than: 30 dBm at frequencies from 1 MHz to 20 MHz; 20 dBm at frequencies from 20 MHz to 650 MHz; 23 dBm at frequencies from 650 MHz to 6 GHz; 9 dBm at frequencies from 6 GHz to 8 GHz. | ENTER | ENTER |
|  | Resolution of the analogue-to-digital (A/D) converter (ADC). | Up to 125 MHz bandwidth: not less than 16 bits; Above 125 MHz bandwidth: not less than 12 bits. | ENTER | ENTER |
|  | FFT real-time IF panorama. | Not less than 500 MHz bandwidth. | ENTER | ENTER |
|  | Minimum signal duration | With a bandwidth of 500 MHz and a resolution of 10 MHz, the minimum detectable signal duration is no longer than 150 ns. | ENTER | ENTER |
|  | IF bandwidth selection | No less than the following options: 10/30/50/100/150/300/600 Hz; 1/1.5/2.1/2.4/2.7/3.1/4/4.8/6/8.333/9/12/15/25/30/50/75/120/150/250/300/500/800 kHz; 1/1.25/1.5/2/5/8/10/12.5/15/20/40/80/125 MHz | ENTER | ENTER |
|  | Maximum input level at maximum attenuation | Not less than +15 dBm at frequencies from 9 kHz to 1.5 GHz; Not less than +10 dBm from 1.5 GHz to 8 GHz. | ENTER | ENTER |
|  | Signal attenuation in steps of no more than 5 dB | Not less than 25 dB at frequencies from 9 kHz to 20 MHz; Not less than 40 dB at frequencies from 20 MHz to 1.5 GHz; Not less than 30 dB at frequencies from 1.5 GHz to 8 GHz. | ENTER | ENTER |
|  | Automatic suppression of signals outside the monitoring range (Interference rejection) | Not less than 80 dB at frequencies from 9 kHz to 8 GHz. | ENTER | ENTER |
|  | Panoramic scanning speed of the frequency band | Not less than 110 GHz/s with 25 kHz resolution. | ENTER | ENTER |
|  | Must have an optical data output with a speed of not less than | 100 Gb/s (to ensure recording of 500 MHz bandwidth I/Q data). | ENTER | ENTER |
|  | Automatic detection and classification of unwanted radiation | Must have this functionality. | ENTER | ENTER |
|  | Recording storage | A dedicated external or internal recording device (recording method) must be used to store radio monitoring and measurement results. | ENTER | ENTER |
|  | Level and field strength measurements. Conversion of received signal level to field strength, performing strong field measurements | Performed in accordance with ITU Recommendation ITU-R SM.378-7. | ENTER | ENTER |
|  | Radio signal parameter measurements | Must have the functionality to perform measurements of AM, FM, PM and IQ signal modulation parameters, including modulation depth, frequency deviation and phase deviation, signal level change monitoring. | ENTER | ENTER |
|  | Bandwidth measurements based on x dB and ß % methods | Compliance with ITU Recommendation ITU-R SM.443-4 (02/2007). | ENTER | ENTER |
|  | Channel power measurement | It has this functionality. | ENTER | ENTER |
|  | Device control software | Used to connect and control the device remotely and select measurement actions and settings. | ENTER | ENTER |
|  | Detect signals with 100% probability of acquisition at the specified receiver parameters: FFT real-time IF span (span) of at least 40 MHz width, Resolvable bandwidth (RBW = 2 MHz) | No worse than 2 µs. | ENTER | ENTER |
|  | Waterfall display | Spectrogram with integrated history buffer and up to 100 μs time resolution. | ENTER | ENTER |
|  | Signal level measurement detectors. | No less than: peak, root mean square (RMS), average. | ENTER | ENTER |
|  | The device has built-in verification equipment/functionality with the ability to save and export report data. | The broadband receiver must have verification equipment installed that is responsible for the smooth operation of the device and its testing during both start-up and operation. | ENTER | ENTER |
|  | Graphical user interfaces installed on the user‘s computer are provided | Spectrum display together with temporary spectrum display (waterfall); Three fully configurable spectrum display modes (trace) with freely selectable RBW; Spectrum history mode – spectrum reproduction from the device memory must be ensured according to user needs; Spectrum and audio recording playback using a graphical user interface. | ENTER | ENTER |
|  | General requirements for a broadband spectrum receiver |  |  |  |
|  | Power supply | 230 V 50 Hz alternating current network. | ENTER | ENTER |
|  | Technical requirements for radio monitoring data recording equipment |  |  |  |
|  | RJ45 data transmission interface for device control | At least 1 unit, with a speed of at least 1 Gb/s. | ENTER | ENTER |
|  | RJ45 data transmission interface for data transmission from/to the recording device and/or for connecting device users | At least 2 units, with a speed of not less than 10 Gb/s. | ENTER | ENTER |
|  | SFP+ data transfer interface for connecting external devices | At least 4 units, with a speed of not less than 10 Gb/s. | ENTER | ENTER |
|  | QSFP28 data transmission interfaces for connecting broadband receivers | At least 2 units, with a speed of no less than 100 Gb/s. | ENTER | ENTER |
|  | QSFP28 data transmission interfaces for data transmission from/to the recording device and/or for connecting device users | At least 2 units, with a speed of not less than 100 Gb/s. | ENTER | ENTER |
|  | Maximum radio frequency band to be supported over a 10 Gb/s interface while the device is continuously recording/reading | Not less than 125 MHz | ENTER | ENTER |
|  | Maximum radio frequency bandwidth that must be supported over a 100 Gb/s interface for the device to continuously record | Not less than 500 MHz | ENTER | ENTER |
|  | Number of supported server SSD modules | At least 2 units. | ENTER | ENTER |
|  | Total capacity of server SSD modules | At least 50 TB. | ENTER | ENTER |
|  | I/Q data stream recording | I/Q data stream shall be recorded in a radio frequency band not narrower than 500 MHz and for not less than 5 hours. | ENTER | ENTER |
|  | Number of parallel data streams supported by the recording device | At least 50. | ENTER | ENTER |
|  | Types of data recorded | - Must be capable of recording spectrum (FFT) in parallel 24/7 across the entire radio frequency band of radio monitoring receivers from at least 20 remote radio monitoring devices; - Must be capable of simultaneously recording I/Q data from at least 10 remote radio monitoring devices; - Additionally, it must be capable of recording audio; - Both the radio spectrum (FFT) and I/Q data must be recorded simultaneously; - Recordings must also support an open data format so that they can be played back using third-party software. | ENTER | ENTER |
|  | Connection of additional data storage/server | The recording device must be capable of connecting to a data storage device/server via an interface/interfaces with a speed of at least 100 Gb/s, which is used to transfer data to/from the recording device. | ENTER | ENTER |
|  | Number of simultaneous users of the recording device | Not less than 2. | ENTER | ENTER |
|  | # Technical requirements for antennas, antenna rotation system, antenna switching. |  |  |  |
|  | Technical requirements for broadband directional antennas in the frequency range from 80 MHz to 8 GHz |  |  |  |
|  | Technical requirements for broadband directional antennas |  |  |  |
|  | Frequency range | Not less than 80 MHz to 1.5 GHz. | ENTER | ENTER |
|  | Polarisation | Linear. | ENTER | ENTER |
|  | Type | Directional. | ENTER | ENTER |
|  | Gain | At frequencies from 80 to 1.5 GHz, it must be no less than 5 dBi without an amplifier. | ENTER | ENTER |
|  | Technical requirements for a broadband directional antenna with reflector |  |  |  |
|  | Frequency range | Not less than 1.5 GHz to 8 GHz. | ENTER | ENTER |
|  | Type | Directional. | ENTER | ENTER |
|  | Polarisation | Linear. | ENTER | ENTER |
|  | Gain | At frequencies from 1.5 GHz to 8 GHz, not less than 12 dBi, with the amplifier turned off. | ENTER | ENTER |
|  | Amplifier parameters | Gain at frequencies from 1.5 GHz to 8 GHz not less than 30 dB; Noise factor not exceeding 3.5 dB | ENTER | ENTER |
|  | Technical requirements for direction-finding antenna |  |  |  |
|  | Polarisation of the direction-finding antenna for V/U/SHF direction finders | Vertical and horizontal. | ENTER | ENTER |
|  | Polarisable direction finding antenna for V/U/SHF direction finders, operating frequency range | Vertical polarisation – not narrower than 20 MHz to 8 GHz; In horizontal polarisation – not narrower than 20 MHz to 7 GHz. | ENTER | ENTER |
|  | Amplifier on/off | Via control software – electronically. | ENTER | ENTER |
|  | Direction finding error (system: direction finding antenna and receiver) | No more than ±2º RMS in mode. | ENTER | ENTER |
|  | Deviation of the horizontal plane diagram of the horizontal polarisation antenna of the direction finder from the non-directional antenna diagram | Not greater than 3 dB | ENTER | ENTER |
|  | Polarisation change | Control software – electronically. | ENTER | ENTER |
|  | Technical requirements for the direction finder monitoring antenna |  |  |  |
|  | Operating frequency range of the direction finder radio monitoring antenna | Vertical polarisation – not narrower than 20 MHz to 8 GHz; In horizontal polarisation – not narrower than 20 MHz to 7 GHz. | ENTER | ENTER |
|  | Polarisation of the operating frequencies of the radio monitoring antenna of the direction finder | Vertical and horizontal. | ENTER | ENTER |
|  | Standing wave ratio, in the frequency band specified in point 7.3.1 | Not greater than 2.0. | ENTER | ENTER |
|  | Technical requirements for omnidirectional monitoring antennas |  |  |  |
|  | Frequency range | Not less than 20 MHz to 8 GHz. | ENTER | ENTER |
|  | Polarisation | Vertical and horizontal. | ENTER | ENTER |
|  | Vertical plane diagram unevenness | Not greater than 3 dB | ENTER | ENTER |
|  | Horizontal plane diagram unevenness | Not more than 5 dB | ENTER | ENTER |
|  | Third-order intercept (TOI) of vertical polarisation | Typical value not less than: 25 dBm at frequencies up to 850 MHz; 22 dBm at frequencies above 850 MHz | ENTER | ENTER |
|  | Third-order intercept (TOI) for horizontal polarisation | Typical value not less than: 20 dBm at frequencies up to 850 MHz; 23 dBm at frequencies above 850 MHz | ENTER | ENTER |
|  | Technical requirements for omnidirectional antennas for low-frequency monitoring |  |  |  |
|  | Frequency range | Not less than 9 kHz to 20 MHz | ENTER | ENTER |
|  | Polarisation | Vertical. | ENTER | ENTER |
|  | Third-order intercept (TOI) | Typical value not less than: 30 dBm at frequencies up to 20 MHz | ENTER | ENTER |
|  | Technical requirements for antenna switches. |  |  |  |
|  | Antenna switch | Must be capable of connecting any of the system‘s antennas to the measurement receiver. Designed for outdoor use and mounted on an antenna tower next to the antenna rotation device. | ENTER | ENTER |
|  | Number of signal inputs for the antenna switch | At least 4. | ENTER | ENTER |
|  | Frequency range of the antenna switch | Not less than DC to 8 GHz. | ENTER | ENTER |
|  | Antenna switch attenuation in the frequency range up to 8 GHz | Not greater than 1.7 dB | ENTER | ENTER |
|  | Antenna switching time | Not more than 15 ms | ENTER | ENTER |
|  | Antenna switch control | Control software – electronically. | ENTER | ENTER |
|  | Technical requirements for antenna rotation device |  |  |  |
|  | Antenna rotator | Must be able to rotate antennas horizontally (azimuth) and change antenna polarisation vertically (elevation/polarisation). Must be hollow so that cables can be routed inside the adapter. | ENTER | ENTER |
|  | Weight of the antenna rotator. | No more than 200 kg. | ENTER | ENTER |
|  | Antenna rotation device control unit | Designed for outdoor use and mounted on the antenna tower next to the antenna rotation device. | ENTER | ENTER |
|  | Antenna rotator maintenance and technical documentation | Includes a mechanical diagram of the rotor, technical maintenance instructions and specifications for lubricants. | ENTER | ENTER |
|  | Technical requirements for the operation of the antenna rotation device in a horizontal plane |  |  |  |
|  | Range of directional change in the horizontal plane (azimuth) | Not less than 0 to 400 degrees. | ENTER | ENTER |
|  | Rotation speed in the horizontal plane | Not less than 2.2 degrees/s (typical). | ENTER | ENTER |
|  | Directional repeatability (positioning accuracy) in the horizontal plane | Not worse than ±1 degree (typical). | ENTER | ENTER |
|  | Type of rotation device | Rotation device type – moving, i.e. there is a vertical pipe in the centre of the device, around which the moving part of the rotation device rotates. There are flanges at both ends of the pipe, allowing it to be used as an adapter between two non-moving sections of the antenna tower. | ENTER | ENTER |
|  | Operation of the rotating device | The configuration must ensure reliable operation of the proposed system. | ENTER | ENTER |
|  | Protection when changing the antenna azimuth | There must be internal switches that cut off the motor power supply when any edge of the azimuth range of the rotating device is reached. | ENTER | ENTER |
|  | Technical requirements for the operation of the antenna rotation device in the vertical plane |  |  |  |
|  | Range of variation in the vertical plane (polarisation) | Not less than -90 to +90 degrees (step not greater than 1 degree). | ENTER | ENTER |
|  | Rotation speed in the vertical plane | Not less than 9 degrees/s (typical). | ENTER | ENTER |
|  | Directional repeatability (positioning accuracy) in the vertical plane | Not worse than ±1 degree (typical). | ENTER | ENTER |
|  | Protection when changing the direction of the antenna (polarisation) in the vertical plane | There must be internal switches that cut off the motor power supply when the direction of rotation of the rotating device reaches either end of the range in the vertical plane. | ENTER | ENTER |
|  | Attenuation of the signal radio frequency cable for connection to the antenna switch in the receiver room | Not more than 25 dB/100 m at 8 GHz. | ENTER | ENTER |
|  | Signal radio frequency cable for connecting antennas to the antenna switch | Not more than 40 dB/100 m at 8 GHz. | ENTER | ENTER |
|  | Software control equipment | Must be provided | ENTER | ENTER |
|  | Technical requirements for GNSS antennas. |  |  |  |
|  | GNSS antenna | Must ensure the reception and processing of satellite navigation signals from at least the following navigation systems: GPS, GLONASS, BeiDou, Galileo. | ENTER | ENTER |
|  | Antenna type | Active CRPA type antenna. | ENTER | ENTER |
|  | Active GNSS antenna low-noise amplifier gain | Not less than 15 dB | ENTER | ENTER |
|  | General requirements for antennas, antenna rotation system, antenna switching. |  |  |  |
|  | Operating temperature range | Not less than -30 °C to +55 °C. | ENTER | ENTER |
|  | Protection against strong static electric fields | Must be ensured. | ENTER | ENTER |
|  | Lightning protection | The direction-finding antenna shall be equipped with adequate lightning protection as specified in the ITU Handbook on Spectrum Monitoring. Lightning protection elements shall not adversely affect the directional accuracy of the signal measured by the direction-finding equipment. | ENTER | ENTER |
|  | Technical requirements for remote radio monitoring receivers |  |  |  |
|  | Frequency range | Not less than 9 kHz to 8 GHz. | ENTER | ENTER |
|  | Minimum frequency tuning resolution | Not greater than 1 Hz. | ENTER | ENTER |
|  | Frequency determination error | Not greater than 2 × 10-7. | ENTER | ENTER |
|  | Real-time intermediate frequency bandwidth | Not less than 40 MHz | ENTER | ENTER |
|  | Record spectrum data for later analysis | Must have this functionality. | ENTER | ENTER |
|  | Third-order intercept (TOI) | Typical value not less than: -1 dBm at frequencies up to 5.3 GHz; -6 dBm at frequencies above 5.3 GHz. | ENTER | ENTER |
|  | Displayed Average Noise Level (DANL) | Typical value not exceeding: -162 dBm at frequencies up to 5.3 GHz; -157 dBm at frequencies above 5.3 GHz. | ENTER | ENTER |
|  | Noise figure | Typical value not exceeding: 12 dB at frequencies up to 3.3 GHz; 16 dB at frequencies from 3.3 to 5.3 GHz; 20 dB at frequencies above 5.3 GHz. | ENTER | ENTER |
|  | Image rejection | Typical value not less than: 110 dB at frequencies up to 1 GHz; 80 dB at frequencies above 1 GHz. | ENTER | ENTER |
|  | Sensitivity | When demodulating an FM signal (at SINAD = 20 dB, fmod = 1 kHz): in the frequency band from 20 MHz to 35 MHz, with a bandwidth of 9 kHz and a deviation of 2.4 kHz, not exceeding -97 dBm; in the frequency band from 35 MHz to 6 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not exceeding -107 dBm; in the frequency band from 6 GHz to 7.5 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not exceeding -102 dBm; in the frequency band from 7.5 GHz to 8 GHz, with a bandwidth of 15 kHz and a deviation of 5 kHz, not exceeding -92 dBm. | ENTER | ENTER |
|  | Detect signals with 100% probability of interception at the specified receiver parameters: FFT real-time IF span of at least 40 MHz, resolution bandwidth (RBW = 2 MHz) | Not worse than 2 µs. | ENTER | ENTER |
|  | Types of demodulated signals | Not less than AM, FM, PM, pulse, I/Q, LSB, USB, CW. | ENTER | ENTER |
|  | Automatic frequency control (AFC) | Must have this functionality. | ENTER | ENTER |
|  | Intermediate frequency band width shape factor | Signal bandwidth at 3 dB and 50 dB attenuation limits, coefficient not exceeding 1:1.6 for filters with an operating frequency of up to 40 MHz | ENTER | ENTER |
|  | Analogue/digital (A/D) converter (ADC) resolution | Not less than 14 bits. | ENTER | ENTER |
|  | Maximum signal level at input | Not less than +20 dBm. | ENTER | ENTER |
|  | Input signal level attenuator | Not less than 30 dB with a switching step of no more than 2 dB (switching can be selected manually or automatically). | ENTER | ENTER |
|  | Signal level resolution | Not greater than 0.1 dB | ENTER | ENTER |
|  | Signal level measurement error | Not more than ±3 dB (in the temperature range from –10 °C to +55 °C). | ENTER | ENTER |
|  | Signal level measurement detectors | Not less than: peak, root mean square (RMS), average. | ENTER | ENTER |
|  | Spectrum scanning speed, displaying the entire frequency range | Not less than: 60 GHz/s (at 1 MHz RBW); 55 GHz/s (at 100 kHz RBW); 45 GHz/s (at 25 kHz RBW). | ENTER | ENTER |
|  | Frequency scan with user-selectable start/end frequency and step width | Not less than 2000 frequencies/s. | ENTER | ENTER |
|  | Number of radio frequencies (addresses) stored in memory | At least 1000 radio frequencies (addresses). | ENTER | ENTER |
|  | Scanning speed of radio frequencies (addresses) stored in memory (memory scan) | Scanning of radio frequencies (addresses) stored in memory (not less than 1000 addresses/s). | ENTER | ENTER |
|  | Bandwidth of the IQ data output connection for wideband spectrum (at least 40 MHz bandwidth) | Not less than 10 GBit. | ENTER | ENTER |
|  | Antenna connectors | At least 2 connectors, input impedance 50 Ω. | ENTER | ENTER |
|  | GNSS antenna connector | There must be a connector for an external active or passive GNSS antenna that ensures the reception and processing of satellite navigation signals from at least the following navigation systems: GPS, GLONASS, BeiDou, Galileo. | ENTER | ENTER |
|  | Record spectrum data for later analysis | Must have this capability | ENTER | ENTER |
|  | Ability to record radio spectrum (FFT) | Recording to internal memory and/or recording device. | ENTER | ENTER |
|  | Ability to record signal spectrum (IQ data) | Recording to internal memory and/or recording device. | ENTER | ENTER |
|  | Radio monitoring software for remote connection and control of the device and selection of measurement actions and settings | Has this functionality. | ENTER | ENTER |
|  | Remote control | Has this functionality. | ENTER | ENTER |
|  | Ability to perform measurements in accordance with ITU recommendations | Must have functionality to perform measurements in accordance with ITU recommendations, including: bandwidth measurements in accordance with ITU-R SM.443-4 (02/2007); frequency measurements in accordance with ITU-R SM.377; modulation measurements in accordance with ITU-R SM.328; level and field strength measurements in accordance with ITU-R SM.378-7. | ENTER | ENTER |
|  | Storage of radio monitoring and measurement results | A dedicated external or internal recording device shall be used. | ENTER | ENTER |
|  | Ability to use TDOA for source location | It has this functionality. | ENTER | ENTER |
|  | RF spectrum recording with user-selectable start/end frequency and resolution ( ) | It has this functionality. | ENTER | ENTER |
|  | Automatic detection and classification of unwanted radiation | Has this functionality. | ENTER | ENTER |
|  | Autonomous operation in the event of a power failure for 12 hours. | This feature is available. | ENTER | ENTER |
|  | Ability to store data offline for at least 4 hours and then transfer it to a data storage server. | This option is available. | ENTER | ENTER |
|  | Ability to connect to a network of remote radio monitoring receivers | Has this capability. | ENTER | ENTER |
|  | Receiver start-up/reboot time | Must be fully operational within 180 seconds of start-up. | ENTER | ENTER |
|  | General requirements for remote radio monitoring receivers |  |  |  |
|  | Operating temperature range | Not less than -30 °C to +55 °C. | ENTER | ENTER |
|  | Remote control via LAN interface | RJ-45, 1 Gbit WAN. | ENTER | ENTER |
|  | Power supply | 230 V 50 Hz AC mains. | ENTER | ENTER |
|  | Functional requirements for software |  |  |  |
|  | Functional requirements for software designed to control a radio monitoring receiver with direction finding capability |  |  |  |
|  | Control of a radio monitoring receiver with direction finding function (hereinafter referred to as the receiver) and an antenna module (hereinafter referred to as the antenna) | Control of the receiver and antenna via a LAN interface. Direct control of the receiver via virtual access. | ENTER | ENTER |
|  | Electromagnetic field strength display in dBµV/m | The manufacturer performs calibration of the monitoring antennas with the amplifier turned on and off and enters the data into the monitoring antenna factor tables. The manufacturer calibrates the radio cable(s) between the receiver and the antenna and enters the data into the cable attenuation tables. The electromagnetic field strength is calculated automatically using data from the corresponding tables. | ENTER | ENTER |
|  | Data import, export | Uploading data on licensed radio frequencies and channels from standard Windows data programmes. Exporting monitoring and measurement data to standard Windows programmes. | ENTER | ENTER |
|  | Antenna polarisation and amplifier control | The program window displays the currently used antenna and amplifier status (on/off). All settings can be changed with a click of the mouse. | ENTER | ENTER |
|  | Selection of measurement tasks | In the general task window, the user controls the receiver and performs measurements. It contains several tabs. Each tab corresponds to a selected measurement method. Depending on the measurement method, the measurement results are displayed using different types of graphs, tables or digital displays. The measurement results can be saved for further analysis. | ENTER | ENTER |
|  | Measurement methods | At least the following: Direct, real-time. The user monitors the radio spectrum in real time, can change the limits and resolution of the radio frequency overview range, record the spectrum, demodulate the selected signal, and record the sound of the demodulated signal. Monitors the radio spectrum in real time and, in parallel, the colour radio spectrum over time (waterfall) or the change in the selected signal over time (zero span). Measures the electromagnetic field strength of the selected signal. Interactive, real-time. The user selects the radio frequency range, scan step, noise threshold, and table containing data on licensed radio frequencies or transmitters. During measurement, signals that exceed the noise threshold and are not included in the table of licensed radio frequencies or transmitters are automatically detected. The results are displayed on the screen in real time and recorded. Analyse the recorded data, select the required frequency(ies) from the list and generate a report. The report shall display at least the following parameters: frequency, strength, detection time, occupancy. Automatic, according to a schedule set by the user. Automatically determine whether the direct result is outside the limits defined by the user and respond as defined by the user. Perform at least the following tasks: Measure the electromagnetic field strength of transmitters from a table, compare the results with user-defined limits, and generate reports on non-compliance. Identify signals that exceed the specified noise threshold and are not included in the table of licensed radio frequencies or transmitters. After detection, if specified by the user, automatically determine the direction of the signal. The measurement results can be analysed during or after the task has been completed. Real-time signal direction detection. The user monitors the radio spectrum in real time and can change the limits and resolution of the radio frequency overview range. The detected directions of the selected signal are displayed in real time in a table and on a map and are automatically recorded. | ENTER | ENTER |
|  | Functional requirements for software for controlling radio monitoring receivers |  |  |  |
|  | Modes supported by the software | At least the following: Direct, real-time. The user monitors the radio spectrum in real time, can change the limits and resolution of the radio frequency overview range, record the spectrum, demodulate the selected signal, and record the sound of the demodulated signal in real time ( ). Monitors the radio spectrum in real time and simultaneously displays the colour radio spectrum over time (waterfall) or the change in the selected signal over time (zero span). Measures the electromagnetic field strength of the selected signal. Interactive, in real time. The user selects the radio frequency range, scan step, noise threshold, and table containing data on licensed radio frequencies or transmitters. During measurement, signals that exceed the noise threshold and are not included in the table of licensed radio frequencies or transmitters are automatically detected. The results are displayed on the screen in real time and recorded. Analyse the recorded data, select the required frequency(ies) from the list and generate a report. The report shall display at least the following parameters: frequency, strength, detection time, occupancy. Automatic, according to a schedule created by the user. Automatically determine whether the direct result is outside the user-defined values and respond as defined by the user the user. Perform at least the following tasks: Measure the electromagnetic field strength of transmitters from a table, compare the results with the limits set by the user, and generate reports on non-compliance. Identification of signals that exceed the specified noise threshold and are not included in the table of licensed radio frequencies or transmitters. After detection, if specified by the user, determine the direction of the signal. The measurement results can be analysed during or after the task has been completed. Real-time location of the signal source. The user monitors the radio spectrum in real time and can change the limits and resolution of the radio frequency overview range. Real-time location of the selected signal and display in a table and on a map. | ENTER | ENTER |
|  | Spectrum recording from radio monitoring receivers | The software must ensure 24/7 parallel recording of the spectrum (FFT) across the entire radio frequency band of radio monitoring receivers from at least 20 remote radio monitoring devices. | ENTER | ENTER |
|  | Compliance of software-based measurements with ITU recommendations | Software-based measurements shall comply with the following ITU Recommendations, including: - Bandwidth measurements in accordance with ITU-R SM.443-4 (02/2007); - Radio frequency measurements in accordance with ITU-R SM.377; - Modulation measurements in accordance with ITU-R SM.328; - Level and field strength measurements in accordance with ITU-R SM.378-7. | ENTER | ENTER |
|  | Control of antenna switches, rotors, filter equipment | The software must be capable of controlling antenna switches, rotors and filter equipment depending on the measurements being performed. | ENTER | ENTER |
|  | Remote task assignment | The software must be able to assign tasks remotely to both individual remote radio monitoring receivers and to a user-defined group of remote radio monitoring receivers. | ENTER | ENTER |
|  | User hierarchy and identification | The software must support different user levels (user rights) and user identification. | ENTER | ENTER |
|  | Automatic antenna selection according to the monitored frequency band | If several antennas are required for the radio frequency band specified during the recording of radio frequency range data, the software must ensure automatic antenna switching depending on the operating radio frequency band of the antennas. | ENTER | ENTER |
|  | Continuous recording of the monitored radio frequency range after an interruption | All radio monitoring stations (both remote and stationary) must ensure 24/7 recording of data in the monitored radio frequency range, except in cases where the equipment of a specific monitoring station/is used to perform other tasks, in which case the equipment of the station(s) must automatically resume recording data for the monitored radio frequency range after the tasks have been completed. | ENTER | ENTER |
|  | Centralised management | The software must ensure: - Centralised management of the “Alarm“ function (detection of discrepancies, decision-making and management from the control centre); - Task generation for mobile radio monitoring station teams; - Short-term access to stationary stations and remote radio monitoring receivers for mobile radio monitoring station teams, as needed to perform the tasks received. | ENTER | ENTER |
|  | Functional requirements for software for processing data obtained during radio monitoring |  |  |  |
|  | Detection, identification and classification of monitored/recorded radio signals | The software must be able to automatically recognise radio signals, detect them, identify them and classify them according to its database. | ENTER | ENTER |
|  | Adding user data to the software‘s radio signal database | The software‘s radio signal database must be able to be supplemented with user data. | ENTER | ENTER |
|  | List of radio frequencies used with licences | The software must be able to import information from the RRT licence database. | ENTER | ENTER |
|  | Spectrum occupancy check | The software must be able to automatically measure how much of the radio frequency band of interest to the user is occupied. | ENTER | ENTER |
|  | Automatic detection of unauthorised radio frequencies | The software must be able to automatically compare the list of licensed radio frequencies imported from the RRT database and determine which radio frequencies in the radio frequency band of interest to the user are being used without a licence, and provide a list of these unauthorised radio frequencies. | ENTER | ENTER |
|  | Recording, playback and processing of radio signals | The software must be capable of recording, reproducing or processing several signals simultaneously. | ENTER | ENTER |
|  | Signal demodulation | The software must be capable of demodulating signals both automatically according to an internal software list and according to user-defined parameters. | ENTER | ENTER |
|  | Recording and playback of demodulated audio signals | The software must be capable of recording and playing back demodulated audio signals. | ENTER | ENTER |
|  | Identification of real-time monitored/recorded data | The software must be able to clearly distinguish between signals that are monitored in real time and those that are presented from recorded data. | ENTER | ENTER |
|  | Number of digital down-converter ( -down converter – DDC) channels | No less than 128. | ENTER | ENTER |
|  | Presentation of measurement results | The software must process the measurement data, provide statistics and generate measurement result reports. | ENTER | ENTER |
|  | Storage of records | All records are stored locally in a single database located at the central control point. Records are also stored in an open format. Records are stored for at least 3 months. | ENTER | ENTER |
|  | Technical equipment | To ensure the above-mentioned functions, the Supplier may also use technical equipment modules, such as broadband signal processors, etc. | ENTER | ENTER |
|  | Waterfall function | There must be a spectrum display together with a waterfall function with an integrated history buffer for recorded data of at least 24 hours and a time resolution of up to 100 μs. | ENTER | ENTER |
|  | General requirements for software |  |  |  |
|  | User interface operating system | Must be Windows 11 or newer (64-bit). | ENTER | ENTER |
|  | Network | Managed from the control centre. User logins with identification must be local and remote. | ENTER | ENTER |
|  | I/Q data stream processing | I/Q data recovery and processing for directional receivers, spectrum monitoring receivers, broadband receivers and remote radio monitoring receivers. | ENTER | ENTER |
|  | Support for map layers from different providers | The software must have automatic access to several online map providers (at least one of the following): Bing Maps, Bing Hybrid or OpenStreetMap provided by Microsoft. | ENTER | ENTER |
|  | Number of software user licences | At least 12 users at the same time. | ENTER | ENTER |
|  | Control centre and autonomous operation |  |  |  |
|  | Control centre requirements | The Item must be controlled from the control centre. | ENTER | ENTER |
|  | Requirements for equipment installation |  |  |  |
|  | Equipment installation requirements for fixed spectrum monitoring stations. |  | ENTER | ENTER |
|  | The Supplier must ensure that all supplied equipment is fully installed, connected, configured and ready for operation (on a “turnkey“ basis). |  | ENTER | ENTER |
|  | All work must be carried out in accordance with the manufacturer‘s recommendations and applicable occupational safety, electrical installation and radio installation standards. |  | ENTER | ENTER |
|  | The Supplier must have civil liability insurance for the performance of such work. |  | ENTER | ENTER |
|  | The Supplier must coordinate the timing of the work with the Purchaser. It is not mandatory to avoid disrupting the operation of the radio spectrum monitoring station. |  | ENTER | ENTER |
|  | The Supplier must ensure that the existing physical infrastructure (tower, building, grounding system, etc.) is not damaged during the work. |  | ENTER | ENTER |
|  | The Supplier must dismantle the existing equipment that will be replaced with new equipment (antennas, unnecessary cables, brackets, etc.). |  | ENTER | ENTER |
|  | The dismantled equipment must not be damaged during dismantling and must be handed over to the Purchaser. |  | ENTER | ENTER |
|  | A technical equipment installation project must be prepared and agreed with the Purchaser for the installation of new equipment, which must assess the permissible loads on the existing tower. |  | ENTER | ENTER |
|  | The fasteners for the equipment (antennas, cables, rotors, etc.) specified in the technical equipment installation project or their manufacture must be included in the scope of supply. |  | ENTER | ENTER |
|  | The Supplier is responsible for connecting new equipment (antennas, receivers, analysers, data storage devices, etc.) and connecting it to the power supply and data communication network. |  | ENTER | ENTER |
|  | The Supplier must configure the equipment system in accordance with the parameters and requirements specified in the technical specifications. |  | ENTER | ENTER |
|  | The Supplier must perform functional tests and submit test reports to the Purchaser for approval. |  | ENTER | ENTER |
|  | The installation work shall be completed by signing an equipment installation and configuration Deed of Acceptance and Transfer, which shall confirm that everything has been installed correctly. |  | ENTER | ENTER |
|  | The installation work is covered by a 24-month warranty period. |  | ENTER | ENTER |
|  | The Supplier is responsible for all defects caused by poor installation or system configuration. |  | ENTER | ENTER |
|  | After dismantling and installing new equipment, the work site must be cleaned up and waste removed. |  | ENTER | ENTER |
|  | Equipment installation requirements for remote spectrum monitoring nodes. |  | ENTER | ENTER |
|  | The Supplier must ensure that all supplied equipment is fully installed, connected, configured and ready for operation (on a “turnkey“ basis). |  | ENTER | ENTER |
|  | All work must be carried out in accordance with the manufacturer‘s recommendations and applicable occupational safety, electrical installation and radio equipment installation standards. |  | ENTER | ENTER |
|  | The Supplier must have civil liability insurance for the performance of such work. |  | ENTER | ENTER |
|  | The Supplier must agree on the time of performance of the works with the Purchaser. |  | ENTER | ENTER |
|  | The Supplier must ensure that the existing physical infrastructure (building, grounding system, etc.) is not damaged during the work. |  | ENTER | ENTER |
|  | A technical equipment installation project must be prepared and agreed with the Purchaser for the installation of new equipment. |  | ENTER | ENTER |
|  | The technical equipment installation project must include the fastening elements for the equipment (antennas, cables, etc.) or their manufacture must be included in the scope of supply. |  | ENTER | ENTER |
|  | The Supplier is responsible for connecting new equipment (antennas, receivers), connecting it to the power supply network and the data communication network. |  | ENTER | ENTER |
|  | The Supplier must configure the equipment system in accordance with the parameters and requirements specified in the technical specifications. |  | ENTER | ENTER |
|  | The Supplier must perform functional tests and submit test reports to the Purchaser for approval. |  | ENTER | ENTER |
|  | The installation work shall be completed by signing an equipment installation and configuration Deed of Acceptance and Transfer, which shall confirm that everything has been installed correctly. |  | ENTER | ENTER |
|  | The work is covered by a 24-month warranty period. |  | ENTER | ENTER |
|  | The Supplier is responsible for all defects caused by poor installation or system configuration. |  | ENTER | ENTER |
|  | After the work is completed, the site must be cleaned up and waste removed. |  | ENTER | ENTER |
|  | Before submitting a tender, the Supplier will be given the opportunity to assess the objects on site. Visual material from stationary spectrum monitoring stations is provided in Annex 2 to these Technical Specifications. |  | ENTER | ENTER |
|  | Requirements for the Supplier |  |  |  |
|  | If the Supplier is not the manufacturer, it must be an official representative of the manufacturer of the equipment specified in the Technical Specifications, authorised to sell, install and maintain the proposed equipment. A document confirming that the Supplier is the manufacturer of the proposed equipment (Supplier‘s certificate) or an authorised representative of the manufacturer of the proposed equipment (documents confirming official representation) and/or has the possibility of providing warranty service, technical maintenance and repair services (a digital copy of the supporting contract with another company authorised to represent the manufacturer of the proposed equipment must be submitted). |  | ENTER | ENTER |