TECHNICAL SPECIFICATION

(PU-14652/26) [ITP25] Spreaders for Class N3 trucks

- TERMS AND ABBREVIATIONS

1.1. Purchaser – AB Kelių priežiūra.

1.2. Supplier – an economic entity – a natural person, private legal entity, public legal entity, other organizations and their divisions, or a group of such persons with whom the Purchaser enters into a Contract.

1.3. Contract – a contract concluded between the Supplier and the Purchaser regarding the Subject of the Procurement.

1.4. Name of the Subject of the Procurement – (hereinafter – the Goods/Item or the Equipment).

and

Services – Warranty technical service recommended by the manufacturer of salt spreaders and front universal car snow plows and offered by the Supplier.

- DESCRIPTION, SCOPE, AND REQUIREMENTS OF THE SUBJECT OF THE PROCUREMENT

  - Subject of the Procurement

2.1.1. First part of the Procurement - Salt spreaders and front universal snow plows for four-wheel drive vehicles Renault C (8x4(2+2)) (6 sets).

2.1.2. Second part of the Procurement - Salt spreader (pull-on) and front universal snow plow for four-wheel drive vehicles Renault C (8x4(2+2)) (1 set).

2.1.3. Third part of the Procurement – Salt spreader and front universal car snow plow, for two-axle drive cars (4x4) (1 set).

2.1.4. Fourth part of the Procurement – Salt spreaders with autonomous engine and hydraulic station for three-axle vehicles (6x4) (8 sets).

2.1.5. Fifth part of the Procurement – Salt spreader with autonomous engine and hydraulic station for two-axle vehicles (1 set).

2.1.6. Sixth part of the Procurement – Salt spreader with autonomous engine and hydraulic station for a two-axle vehicle, instead of a body (1 set).

2.1.7. Seventh part of the Procurement – Salt spreader for a three-axle vehicle (6x4) (1 set).

2.1.8. Eighth part of the Procurement – Salt spreaders for two-axle vehicles (3 sets).

2.1.9. Ninth part of the Procurement – Salt spreader for a three-axle vehicle (6x4) (new or used) (1 set).

  - Description of the Subject of the Procurement:

2.2.1. The Goods must comply with the technical requirements set out in the annexes to these Technical Specifications.

2.2.2. The Goods must be accompanied (no later than on the date of delivery of the Goods) by: operating and safety instructions in Lithuanian, electrical installation diagrams, hydraulic diagrams, spare parts catalogues, and an EC declaration of conformity.

2.2.3. The offered Goods shall be provided with a full warranty of not less than 24 months, calculated from the moment of handover–acceptance of the Goods, including mobile service at the Purchaser’s bases, with manufacturer confirmations submitted together with the tender. Technical maintenance and/or repair works shall be carried out no later than within 5 working days from the ordering of the Service by e-mail and a telephone call to the Supplier’s responsible person specified in the Contract, after the Parties have agreed in advance on the location of the Goods and the scope of the Services. During the warranty period, the Supplier shall provide free-of-charge repairs at the Purchaser’s bases in the event of warranty failures or defects, or, upon authorising the Purchaser (with a manufacturer-issued authorisation submitted together with the tender), allow technical maintenance and/or repair works to be performed by the Purchaser’s qualified employees, with the costs of technical maintenance and/or repair works compensated in accordance with the conditions set out in the Contract.

2.2.4. During the warranty period, all mandatory maintenance of snow removal equipment and salt spreading equipment must be detailed and submitted together with the offer, specifying the scope of each maintenance service, and the offer must specify the costs for the entire warranty period.

2.2.5. Upon delivery of the Goods, but no later than when the Purchaser begins to use the Goods or part of the Goods set, the Supplier must provide the following training, the cost of which must be included in the price of the offer:

-theoretical and practical training for operators to ensure the correct operation of the Goods.

-theoretical and practical training on the proper and correct installation and dismantling of the equipment and the preservation of dismantled equipment.

-technical training to ensure the correct maintenance of the Goods, explaining the nature of faults and fault symbols.

2.2.6. Before the end of the warranty period for the Goods, the Supplier shall, at its own expense, perform a detailed technical inspection of the Goods and, after the inspection, eliminate any technical non-conformities covered by the warranty.

2.2.7. For trucks operated by the Purchaser (parts 7, 8 and 9 of the Procurement) prior to the installation of the offered salt spreaders, the existing hydraulic systems in the trucks must be inspected and reconstructed, if necessary, to ensure the flawless and optimal operation of the offered salt spreaders. The scope and costs of the planned renovation of the hydraulic systems must be submitted by the supplier in the proposal forms.

2.3. Scope of the Procurement:

Part 1 of the Procurement

| Ser. No | Name of the item | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Salt spreaders and front universal snow plows for four-wheel drive vehicles Renault C (8x4 (2+2)) | set | 6 |

Part 2 of the Procurement

| Ser. No | Name of the item | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Salt spreader (pull-on) and front universal car snow plow for four-wheel drive vehicles Renault C (8x4 (2+2)) | set | 1 |

Part 3 of the Procurement

| Ser. No | Name of the item | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Salt spreader and front universal car snow plow for two-wheel drive vehicles (4x4) | set | 1 |

Part 4 of the Procurement

| Ser. No | Name of the item | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Salt spreaders with autonomous engine and hydraulic station for three-axle vehicles (6x4) | set | 8 |

Part 5 of the Procurement

| Ser. No | Name of the item | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Salt spreader with autonomous engine and hydraulic station for two-axle vehicle | set | 1 |

Part 6 of the Procurement

| Ser. No | Name of the item | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Salt spreader with autonomous engine and hydraulic station for two-axle vehicle, instead of body | set | 1 |

Part 7 of the Procurement

| Ser. No | Name of the item | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Salt spreader for three-axle vehicle (6x4) | set | 1 |

Part 8 of the Procurement

| Ser. No | Name of the item | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Salt spreader for two-axle vehicles | set | 3 |

Part 9 of the Procurement

| Ser. No | Name of the item | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Salt spreader for three-axle vehicle (6x4) (new or used) | set | 1 |

  - The Purchaser shall purchase the Goods with delivery. The Supplier shall undertake to deliver the Goods free of charge to the specified address:

| Part of the Procurement | Delivery addresses | Quantity in sets |
| --- | --- | --- |
| 1 | Kauno g. 72, Pagiriai village, Garliava eldership, 53282 Kaunas district | 6 |
| 2 | Kauno g. 72, Pagiriai village, Garliava eldership, 53282 Kaunas district | 1 |
| 3 | Kauno g. 72, Pagiriai village, Garliava eldership, 53282 Kaunas district | 1 |
| 4 | Kauno g. 72, Pagiriai village, Garliava eldership, 53282 Kaunas district | 2 |
|  | Gamyklų g. 12, 68108 Marijampolė | 1 |
|  | Kauno g. 1, 32134 Zarasai | 1 |
|  | Veiviržėnų g. 36, Pyktiškė village, 96307 Klaipėda district | 1 |
|  | Džiuginėnai village, Gadūnavas eldership, 87415 Telšiai district | 1 |
|  | Laižuvos g. 80, 89213 Mažeikiai | 1 |
|  | Viekšnių g. 14, 85372 Akmenė | 1 |
| 5 | Stoties 20, 39106 Pasvalys | 1 |
| 6 | Struikų g. 10, 75124 Šilalė | 1 |
| 7 | Turistų g. 11, 67102 Lazdijai | 1 |
| 8 | Miško g. 2a, Šilagalis village, 36220 Panevėžys district | 1 |
|  | Basanavičiaus g. 54, 41164 Biržai | 1 |
|  | Žeimių g. 18, 81488 Ginkūnai village, Šiauliai district | 1 |
| 9 | Panevėžio g. 7, 40134 Kupiškis | 1 |

2.5. The Supplier must be the official representative of the manufacturer of the Item offered (if it is not the manufacturer itself), having the right to perform technical maintenance and repair of the Item offered during the warranty period, or have concluded a contract for the provision of relevant services with another economic entity having such a right. The Supplier must submit a power of attorney from the manufacturer of the Item offered or another document confirming the Supplier’s right to perform technical maintenance and repair services during the warranty period, or provide evidence of a contract for the provision of relevant services concluded with another economic entity having such a right.

- PROCEDURE AND TERMS FOR THE FULFILLMENT OF CONTRACTUAL OBLIGATIONS

3.1. The Goods must be delivered within 210 calendar days from the date of entry into force of the Contract.

3.2. The delivery time of the Goods shall be assessed in terms of economic efficiency, taking into account the principles set out in the tender evaluation criteria. If the Supplier fails to comply with the offered delivery terms for the Goods, the Purchaser shall apply late payment interest until the Goods are delivered.

3.3. Individual sets of the Goods may be delivered before the delivery time specified in the tender. The Supplier is obliged to specify the expected delivery time for each complete set in the tender.

4. ENVIRONMENTAL REQUIREMENTS

The Purchaser seeks to ensure that its and the Supplier’s actions have as little impact on the environment as possible, therefore:

4.1.  During the public procurement and contract performance, communication between the Supplier and the Purchaser shall be carried out only by electronic means (CPP IS means, telephone, e-mail, etc.);

4.2.  All documentation related to the performance of the Contract shall be provided to the Purchaser and the Supplier by electronic means (e-mail, etc.);

4.3. The Contract shall be signed only by electronic means (electronic signature).

4.4. The Supplier shall undertake to reduce paper consumption, refrain from unnecessary copying and printing of documents, and if office supplies are used, they must be made from recycled raw materials or suitable for recycling.

4.5.  If the purchased Goods are to be delivered or transferred in secondary packaging, they must comply with the minimum environmental protection criteria set for packaging, unless this is contrary to hygiene standards. Packaging must be considered recyclable packaging in accordance with the provisions of the Republic of Lithuania Law on Environmental Pollution Tax.

4.6.  If the Goods purchased are delivered, the Supplier shall endeavour to ensure that the delivery of the Goods consumes fewer natural resources and thus complies with the environmental protection principle established by the Order of the Minister of the Environment of the Republic of Lithuania, i.e. to ensure that the optimal route is chosen when delivering the Goods to the delivery location specified by the Purchaser.

5. ANNEXES

Annex 1 – Technical parameters

__________

All references in the procurement document to a standard, technical certificate or general technical specifications mean that the contracting authority will accept evidence of equivalent measures from other participants. If the specification refers to a specific model or source of supply, a specific process characteristic of the goods or services supplied by a particular supplier, or a trademark, patent, types, specific origin or production, which would give certain entities or certain products an advantage or lead to their rejection, an equivalent object to that specified may be submitted. If an equivalent object is offered, the supplier undertakes to submit/justify/prove its equivalence to the object specified in the document.

Annex 1 to the Technical Specification

TECHNICAL PARAMETERS

The following must be submitted:

Technical documentation from the manufacturer of the Item/Equipment (catalogues, brochures) and/or declarations from the manufacturer of the Item/Equipment (if the manufacturer’s technical documentation does not fully reflect the compliance of the offered Item/Equipment with the technical specification requirements) and other documents specified in the technical specifications or other equivalent documents proving the compliance of the offered Item/Equipment with the technical requirements. In these documents, the Supplier must indicate (i.e., clearly mark – highlight in colour and/or indicate with arrows, and/or underline, etc.) the specific locations in the documents provided where the required technical characteristics are described and indicate which point of the technical requirements they comply with.

OR

links to a publicly accessible website where the Purchaser can verify the authenticity of the data provided, i.e., the compliance of the offered Item/Equipment with technical requirements. If the information provided on the specified website does not correspond to the data declared by the Supplier, the tender must be accompanied by a declaration from the manufacturer of the Item/Equipment or other equivalent documents confirming that the offered Item/Equipment comply with the technical specification requirements.

The Supplier’s declaration shall not be considered equivalent documentation, except in cases where the Supplier is the official representative of the manufacturer of the offered Item/Equipment.

In the event that the Supplier (who is not the manufacturer) submits technical documentation and/or declarations prepared by representatives of the manufacturer who have the relevant rights, the Supplier must also submit documents proving the relevant rights of the manufacturer’s representative together with the offer (authorization granted by the manufacturer of the Item/Equipment to prepare/approve technical documentation for the Item/Equipment).

If the technical documentation for the Item/Equipment is prepared by a Supplier who is not the manufacturer and who has not been granted the manufacturer’s authorization to prepare/approve technical documentation, in such a case the technical documentation prepared by the Supplier itself does not meet the criteria of objectivity, reliability, traceability of information, and other criteria, therefore, during the evaluation of the tender, the Purchaser will consider that the Supplier has only provided declarative information that is not substantiated by objective data and will reject such a tender.

## Documents (technical documentation of the manufacturer of the Item/Equipment (catalogues, brochures) and/or declarations by the manufacturer of the Item/Equipment or other equivalent documents proving that the offered Item/Equipment comply with the technical characteristics) may be submitted in Lithuanian and/or English. When evaluating the Suppliers’ tenders and at the Purchaser’s request, the Suppliers shall be required to submit the specified documents or parts thereof, translated into Lithuanian, and a confirmation of the translation.

## All specified specific materials, solutions and/or specific names or standards of the Item/Equipment are subject to the “or equivalent” clause. A Supplier offering the Item/Equipment with equivalent characteristics must additionally indicate this in the technical specification and/or tender and prove by reliable means that the offered Item/Equipment is equivalent and fully complies with the requirements set out in the technical specification.

PART 1 OF THE PROCUREMENT. SALT SPREADERS AND FRONT UNIVERSAL SNOW PLOWS FOR FOUR-WHEEL DRIVE VEHICLES (8X4(2+2)) (6 sets)

| Ser. No | Name of characteristics | Characteristics required by the Purchaser | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the Item offered. In the clauses where required, the Supplier must enter the name of the document submitted, proving the values ​​of the parameters, and/or a reference to the source confirming the parameters offered. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1. | Purpose. | The equipment (a set consisting of a combined salt spreader and snow plow) is designed for clearing loose and compacted snow and spreading anti-skid materials on road surfaces. The salt spreader is designed for installation and operation in place of the body on four-axle trucks. The snow plow is designed for installation on the front attachment plate. | Yes/No (delete as appropriate). |
| 1.2. | Year of production | The salt spreader and snow plow must be new and unused prior to the date of transfer. | Yes/No (delete as appropriate). Year and quarter of production – (enter) ___________. |
| 1.3. | The dominant colour of the salt spreader and of the painted metal surfaces and front snow plow | Orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific group of equipment. All spreaders and snow plows (6 sets) must be of the same colour. Galvanized metal surfaces, elements and assemblies made of stainless-steel, aluminium or other corrosion-resistant materials may be unpainted if this is not technically necessary and the welds are properly treated. | Yes/No (delete as appropriate). Proposed dominant colour – (enter parameter) _____________. |
| 1.4. | Warranty against corrosion perforation | All components included in the set, without exception, have a minimum warranty period of 72 months. | Yes/No (delete as appropriate). Proposed parameter - ____ months. |
| 1.5. | Right to perform warranty repairs and warranty technical maintenance | The Supplier must be the official representative of the manufacturer of the offered Goods (if the Supplier is not the manufacturer), having the right to perform technical maintenance and repair of the offered Goods during the warranty period, or must have concluded a corresponding service agreement with another entity authorized to perform such services. The tender must include the manufacturer’s authorization or another document confirming the Supplier’s right to provide technical maintenance and repair services during the warranty period, or evidence of a concluded service agreement with another entity authorized to provide such services, or the Supplier must provide a manufacturer-issued authorization granting the Purchaser the right to perform technical maintenance and repair services during the warranty period. | Yes/No (delete as appropriate). |
| 1.6. | Terms of technical service and warranty repairs | The Supplier must perform warranty repair works within no more than 5 working days after the Purchaser’s request is submitted in writing (by e-mail) and by a phone call to the Supplier’s responsible person indicated in the Contract, or may authorize the Purchaser during the warranty period (providing a manufacturer-issued authorization) to carry out warranty repair works by qualified Purchaser’s personnel, with the costs of the works and other related expenses reimbursed in accordance with the conditions specified in the Contract. | Yes/No (delete as appropriate). |
| 1.7. | Documentation | Operating and safety instructions in Lithuanian (user manuals for salt spreaders and snow plows) shall be provided no later than on the date of delivery of the Item. | Yes/No (delete as appropriate). |
| 1.8. | Training | At the time of delivery of the Item, but no later than when the Purchaser begins to use the Item or part of the set of the Item, the Supplier must provide the following training, the cost of which must be included in the offer price: -theoretical and practical training for operators to ensure the correct use of the Item. - technical training to ensure the correct maintenance of the Item, explaining the nature of faults and fault symbols. - technical training to ensure the correct and safe disassembly/assembly/replacement of the components of the set. | Yes/No (delete as appropriate). |
| 1.8.1 | Approving certificate | The Supplier (including partners – Suppliers of individual components of the set) shall prepare a certificate describing the scope of the training course for the Operator(s) who has/have completed theoretical and technical training and practical training. | Yes/No (delete as appropriate). |
| 1.9. | Manufacturer and installer of bundled equipment | Snow removal and salt spreading equipment shall be manufactured and installed on a truck by the manufacturer of the equipment or its official representative (in which case documents or copies thereof confirming the representation must be submitted with the tender). Alternatively, the Purchaser may carry out the installation, provided that the Supplier covers all costs incurred by the Purchaser for such installation. Responsibility for the flawless operation of the entire equipment complex and for remedying any defects discovered during operation shall rest with the manufacturer of the equipment or its official representative who carried out the installation, or with the Purchaser in the case where the Purchaser performed the installation. An additional point is awarded for the proposed salt spreading and snow removal equipment from the same manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2. SALT SPREADER |  |  |  |
| 2.1. | Purpose, adaptation Type, make | Designed for a four-axle truck with a wheel configuration of 8x4 (2+2) and a wheelbase of at least 3700 mm, mounted on an additional chassis frame. (The drawing of the vehicle on which the equipment will be installed is specified in Annex No. 2 to the Technical Specification.) Designed for spreading, with or without moistening, salt, sand, fine crushed stone and/or spraying salt solution on the road surface. Must comply with the requirements of standard LST EN 15597-1:2020. The spreader must be a standard product manufactured by the manufacturer in series, not specially modified or adapted to the requirements formulated by the contracting authority, except for specific marking and safety requirements. | Yes/No (delete as appropriate). Exact spreader model - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.2. | Quick connection and activation system for the hydraulic and electrical systems of the salt spreader. | The necessary electrical and hydraulic connections must be provided and installed by the manufacturer and marked accordingly. An additional control line and load sensor controller provided by the manufacturer for a load-sensitive truck system (Load Sensing or equivalent). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.3. | Centre of gravity of the mounted spreader | The centre of gravity of the spreader mounted/secured on the truck shall be positioned as close as possible to the truck’s third axle. | Yes/No (delete as appropriate). |
| 2.4. | Installation of a salt spreader on a vehicle. | The spreader must have a quick removal/installation system on the vehicle’s additional frame. The spreader must be removed and installed without external assistance, i.e. without the use of additional self-propelled lifting devices. Lifting devices – with hydraulic cylinders. The metal structure of the quick removal/installation (storage) system and the lifting capacity of the lifting devices must ensure safe, long-term, and stable storage of the spreader (including when loaded with spreading material and/or filled with salt solution). | Yes/No (delete as appropriate). Permissible lifting capacity of lifting device (of each of the four) - ______kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.5. | Control of spreader removal/installation on the truck system (storage legs) | Control of the hydraulic devices of the salt spreader storage legs is performed via the hydraulic system of the truck. The snow plow control panel or another additional control device located outside the operator's cab or a remote-control panel should be used for control. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.6. | Upper overall dimension of the spreader. | The height of the salt spreader, including the spreader bracket for mounting, shall not exceed 2500 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.7. | Weight of the spreader with all integral equipment. | 2 300 – 4 000 kg | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.8. | Spreader adjustment, calibration | No later than on the day of handover of the Goods to the Purchaser, testing, adjustment and calibration of the salt spreader installed on the vehicle shall be carried out, and a calibration/verification document approved by the Supplier or its authorised representative shall be provided, indicating the actual quantities of material spread/sprayed under various operating modes. All required works shall be performed at the Supplier’s expense, except for the quantities of salt, sand - salt mixture and brine solution required for testing and calibration, as well as the loading of materials into the spreader. In the event that the installation of the equipment is delegated to the Purchaser, a manufacturer-issued authorisation shall be provided to the Purchaser, granting the right to perform installation, testing, adjustment and calibration works. | Yes/No (delete as appropriate). |
| Materials hopper |  |  |  |
| 2.10. | Materials hopper | Coated with anti-corrosion coating and painted or made of stainless-steel. The metal surface painting process must be carried out in accordance with the requirements of ISO 8501-1. The coating layer thickness must be at least 180 µm. Colour - orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s specified base (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). Proposed material and colour - _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.11. | Hopper capacity | Possible load capacity of loose materials – no less than 8 m3. | Yes/No (delete as appropriate). Proposed parameter - ____ m3. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12. | Spreading materials feeding device. | A rubber belt conveyor, ensuring smooth feeding of materials to the spreading disc, regardless of the type and amount of the spreading material in the hopper. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12.1 | Material dosing device position control mechanism | The equipment shall include a manufacturer-supplied mechanism for controlling the amount of material dispensed, operated manually. An additional point is awarded for an additional installed mechanism that adjusts the height of the material dosing valve according to the material selected on the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.13. | Mechanism for even distribution of the spreading material. | A separate mechanism installed along the entire length of the hopper, with a separate hydraulic drive or other type of drive and drive control, which distributes the spreading material more evenly over the rubber belt conveyor. | Yes/No (delete as appropriate). Type and description of the proposed device - ____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.14. | Hopper cover | Covered with a metal grate (opening dimensions: no less than 40x40 mm, but no more than 70x70 mm, wire-rod diameter no less than 4 mm) and a moisture-proof, easily closable and openable side tarpaulin without additional means. The levers of the cover opening/closing mechanism must be accessible to the operator (the covering/uncovering process must be possible) while standing on the ground or on a ladder installed at the rear of the spreader. The grate structure is hot-dip galvanized. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.14.1 | Protection against falling down of the spreading material | The lower part of the materials hopper must be equipped with protection, or, at the Supplier’s expense, protection must be installed above the upper part of the truck frame to prevent the spreading material from falling directly onto the truck chassis and transmission components located between the longitudinal side profiles of the frame. It must be possible to wash away any residue of the spreading material that accumulates on the protective device with a water jet, without using additional tools (for removing the bottom (protection) of wrenches). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Salt solution tanks |  |  |  |
| 2.15. | Salt solution tanks | Made from corrosion-resistant material. Salt solution tanks are installed on the sides of the spreader hopper, at the bottom, and at the front of the spreader. The total volume of the salt solution tanks is at least 7,65 m³. Additional point is awarded for a larger total volume of salt solution tanks. | Yes/No (delete as appropriate). Proposed parameter - _____ m³. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.15.1 | Materials used in manufacturing | Tanks must be made of polypropylene or another equivalent polymer material that is resistant to corrosion, temperature fluctuations, and ultraviolet radiation. The resistance of the polymer materials used in salt solution tanks to ultraviolet radiation and other environmental conditions must comply with the DIN EN-17106 standard. | Yes/No (delete as appropriate). Proposed parameter – meets the requirement _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Control |  |  |  |
| 2.16. | Control of salt spreading and moistening equipment | Performed from the vehicle cab, using a digital control panel that is easily accessible to the driver from his workplace. | Yes/No (delete as appropriate). |
| 2.17. | Manual control | The possibility provided by the manufacturer to control the spreading process manually (in case of emergency). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18. | Control panel | The spreader control panel must ensure automatic spreading and/or solution spraying according to a selected, pre-programmed route. During the shift, the operator must be able to continuously view and control at least the following parameters on the control panel display: - Selected spreading material and spreading rate; - Spreading width and asymmetry; - Moistening of the spreading material with solution is on/off, selected moistening percentage (%); - Amount of spreading material; - Solution spraying is on/off, selected amount of sprayed material (ml/m² or g/m²) (depending on the manufacturer’s settings); - Amount of solution The control panel must: - collect data and ensure digital scanning and real-time (“On Line “) transmission of data to the dispatcher ‘s workstation; - inform the operator about the shortage of the spreading materials or salt solution with an additional audible signal; - have connections for data transmission in accordance with data encryption standard EN15430 or an equivalent standard. - have a menu in Lithuanian. The control panel menu must indicate the following names of spreading materials (may be without Lithuanian symbols, e.g., Misinys (mixture) 1:1): • Salt • Mixture 1:1 • Mixture 1:3 • Mixture 1:10 • Solution The Supplier must provide the Purchaser with the access codes required to adjust the control panel settings. The meanings of the control panel buttons must be provided in the user manual. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18.1 | Priority requirement – Interface of the spreading equipment with external road condition information systems and their stations | The spreading equipment (control panel, software, etc.) shall have a real-time interface/integration with the road condition information system and its stations located in the Lithuanian road infrastructure (“Klimator” or equivalent). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18.2 | Priority requirement – Functions of the interface with road condition information systems | The spreading equipment shall have the functionality to analyse or compare data received from the interface with the road condition information system (upcoming or recorded weather changes) and, according to the specified algorithm, automatically adjust the operational parameters of the spreader (dosage of anti-slip materials, spreading/spraying width, etc.) in order to timely and preventively ensure the preparation of the road surface for the upcoming/recorded weather changes. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.19. | Operation data transmission system | The Supplier must provide access to the control panel data transmission, at least for the following data (for the purpose of controlling the spreader operation): 1. Date, time, operating time. 2. Spreader operating parameters: spreading is on/off or spreading mode, spreading time, spreading width, name of the material being spread, dosage of the material being spread g/m², amount spread kg, solution dosage ml/m² or g/m² (depending on the manufacturer’s specifications), amount of solution dispensed in litres. 3. Position of snow plow: operating/transport position. Transport telemetry control services are provided to the Purchaser by UAB Xirgo Global. The tender must be accompanied by information that clearly and comprehensively shows the required data. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.20. | Generation of data transmission system reports | Reports must be generated freely by selecting the date and time. They must include: 1. The date of spreading, the start/end time of spreading. 2. The start/end location of spreading, total spreading time, spreading distance, and consumption of spreading material and salt solution (kg, litres). 3. Dosage of spreading material in g/m², dosage of solution in ml/m² or g/m² (depending on the manufacturer’s specifications). 4. Distance travelled by the vehicle and spreader, m. 5. Names of materials being spread. Detailed information about the reports generated and the required data included in them must be provided. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.21. | Even spreading and salt solution feeding. | The vehicle movement speed signal must be transmitted to the spreader control panel (linked). The even feed of salt and salt solution to the spreading disc, the even rotation of the disc and the spreading of materials, according to the settings on the control panel, must take place regardless of the vehicle movement speed, as well as when the vehicle is stationary. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.22. | Emergency unloading function. | Must be provided for by the manufacturer and activated via the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.23. | Spreading parameter programming and fault notification system. | Must be provided for and installed by the manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.24. | Instantaneous increase of the amount being spread. | A function that allows the amount of material being spread to be increased two or more times for a limited period of time. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Spreading (dispersal) mechanism |  |  |  |
| 2.25. | System for mixing the material being spread and solution. | Ensuring mixing of the spreading material and salt solution and feed of the mixture onto the spreading disc, or mixing of salt and solution directly on the spreading disc. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26. | Salt moistening and spreading mechanism | Variable length sleeve/trough and disc with hydraulic drive, liftable upwards and lockable in transport position. Air shock absorbers provided for and installed by the manufacturer, or equivalent solution to facilitate lifting of the mechanism into transport position. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27. | Spreading disc | Diameter not less than 600 mm. The height of the disc above the road surface is adjusted mechanically (manually), at interval of not less than 100 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.28. | Spreading disc hydraulic motor | Protected from direct exposure to salt. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29. | Salt spreading sensor | Must be contactless, electronic. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29.1 | Visual control | A rear-view camera with night vision, installed by the spreader manufacturer, is mounted at the rear of the spreader, as high as technically possible, for visual control of the spreading process and control of the vehicle’s reverse movement. The image transmitted by the camera must be visible at the operator’s workplace (in the cab of truck) on the monitor (in a separate section) located on the spreader control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.30. | Adjustable amount of material being spread per m² | Salt - within the range of 5-40 g/m², with a maximum “pace“ of 5 g/m². Sifted sand and crushed stone - within the range of 20-300 g/m², with a maximum “pace“ of 10 g/m². | Yes/No (delete as appropriate). Proposed parameter: Salt within the range of ____ g/m2. Spread gravel, sand, crushed stone within the range of ____ g/m2. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.31. | Adjustable spreading width of the material being spread. | From 3 m to at least 12 m, at intervals of no more than 1,0 m, with asymmetrical (increasing or decreasing the width of one side of the spread without changing the width of the other side) adjustment of the spreading material, set using the control panel. | Yes/No (delete as appropriate). Proposed parameter: Within ____ m, spreading width “pace” of __m. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.32. | Forced (pressure) salt solution feeding (to the spreading disc) and control of amount. | At interval of 5 %, starting from a minimum of 15% to 100% (FS15 – FS100), which automatically changes depending on the amount of salt being spread. It is assumed that the percentage of salt solution for salt moistening indicates the proportion of the solution in the spread rate (e.g., FS15 means that the spread rate consists of 15% solution + 85% dry matter). | Yes/No (delete as appropriate). Proposed parameter: At __ % interval from ___ % Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Solution spraying (dispersion) mechanism |  |  |  |
| 2.33. | Salt solution spraying (dispersal) mechanism | The solution spraying mechanism is installed in the rear part of the salt spreader. The mechanism's units and components are mounted on a bracket connected to the spreading mechanism or on a separate bracket(s). The mechanism is lifted up and locked in the transport position together with the spreading mechanism or separately. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.34. | Salt solution pump(s) | Pumps made of stainless steel AISI 316 or other equivalent material resistant to brine solution. The capacity of the hydraulic pumps shall be adjustable via the control panel: – Pump used for the brine solution spraying function, with a capacity in the range of 1 l/min to 290 l/min; – Pump used for the brine solution for the salt wetting function, with a capacity in the range of 1 l/min to 100 l/min. | Yes/No (delete as appropriate). Proposed parameter: material of pump(s) _________ capacity in the range from ___ litres/min to ___ litres/min Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.35. | Spraying widths | The control system must ensure: - different spray widths - (symmetrical) width from 3.0 to at least 12.0 m, and asymmetrical, at least 7.0 m to the left and right. - the ability to adjust the width of the sprayed material in meters (1.0 m offset from the driving axis). - at least three separate spray direction positions: right, left, and centre. - Uniform spraying regardless of the vehicle’s speed when traveling at speeds of up to 70 km/h. | Yes/No (delete as appropriate). Proposed parameters: Symmetrical width from ___ m to ___ m. Asymmetrical width ___ m Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.36. | Dosage | The amount of sprayed salt solution is adjustable: from 10 ml/m² or g/m² to at least 100 ml/m² or g/m² (depending on the manufacturer’s specifications). The dosing “pace” is no more than every 5 ml/m² or g/m². | Yes/No (delete as appropriate). Proposed parameters: from ___ ml/m² or g/m² to ___ ml/m² or g/m². Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Safety fence |  |  |  |
| 2.37. | Safety fence – lightweight construction, 1-1.15 m high, on both sides of the spreader. | Mounted on top of the salt spreader hopper and raised/folded for maintenance/repair before accessing the hopper. The raising/lowering of the Safety fence is performed by lowering/raising the ladder intended for access to the hopper or by a separate raising/lowering mechanism. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Signal marking |  |  |  |
| 2.38. | Dimensional marking, signal lighting | The working (spreading/dispersal) area must be illuminated by at least one light. At least one orange LED beacon installed at the rear of the spreader. Two flashing LED lights (round lights) with a diameter of at least 180 mm, installed at the upper corners of the rear part of the spreader. The rear part marked with warning light-reflecting diagonal red and white stripes. Sample pattern of warning marking: Protection of LED beacons and flashing LED lights against solid objects and liquids must be at least IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.39. | Mounting locations for road signs and road signs. | The rear part of the spreader (on the mounting/suspension unit/frame) shall be equipped with places for mounting warning/directional road signs or the necessary mounting structure, and no later than on the date of delivery of the Item, the following shall be installed: - a light-reflecting sign No 106 „Work“ not less than 1 (first) size: - Signal LED light pointer – a system of at least 13 lights complying with L8H, EN 12352 or an equivalent standard, fastened from the rear with screws or equivalent fasteners. The direction of the arrow indicating the mandatory side of the obstacle is changed from the operator's workplace using an electrical connection. A visual indicator informing about the activated direction of the arrow is mandatory at the operator's workplace. The feed voltage of the arrow lights is 24 V. The protection of the LED light system components against solid objects and liquids is not less than IP65. | Yes/No (delete as appropriate). |
| 3. FRONT SNOW PLOW |  |  |  |
| 3.1. | Purpose, adaptation Type, make | Designed to push snow both to the right and left. Enables snow removal while the truck is traveling at speeds of 60 km/h and above. The upper part of the blade is equipped with an additional C-shaped roof made of rigid plastic or equivalent rigid material along the entire length of the plow, which protects the vehicle's windshield and front cabin grille from snow. The snow plow is of the segmented type. Additional points are awarded to segmented snow plows where the segments can move (slide during snow removal) up/down independently of each other. The side (left-hand) part of the snow plow shall be folded or pushed to the left side (in this case, when the left-hand section is folded or pushed, its cleaning element blades shall be aligned in a straight line with the cleaning elements of the central section segments). | The proposed parameter is specified in the tender form. Exact model of front snow plow - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.2. | Body | Made of steel, with stiffening ribs (metal primed and coated with two coats of paint). | Yes/No (delete as appropriate). |
| 3.3. | Working surface of the plow | Made of steel (metal primed and coated with two coats of paint). The colour of painted metal surfaces is orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). Enter proposed colour - _________. |
| 3.4. | Control | Lowering, lifting, and turning left/right and folding are performed from the driver ‘s cab using a control panel located within easy reach of the driver. | Yes/No (delete as appropriate). |
| 3.4.1 | Control panel | The plow control panel is equipped with a connection for reading work control (cleaning process) data. The service provider installing the transport control system is given access and the opportunity to connect to the control panel by providing the necessary login codes. It must be possible to read the analogue signal for the plow position (snow being cleaned/not being cleaned), or the same data must be provided via the spreader control panel in accordance with EN 15430 or an equivalent standard. | Yes/No (delete as appropriate). |
| 3.5. | Installation | It is mounted (suspended) using a special device-suspension at the front of the vehicle, which corresponds to the dimensions and mounting locations of the snow plow mounting plate. The suspension is manufactured in accordance with the applicable standards for front vehicle mounting plates (DIN 76060 type “A“ or equivalent). An additional point is awarded for additional snow plow (hydraulic lines to be used for lifting) and truck connections and/or couplings with an external control device (on the outside of the truck: on the front bumper, next to the driver‘s cab steps), allowing the operator to control the lifting/lowering of the snow plow (by connecting the hydraulic hoses to the truck’s hydraulic outlets) without being in the cab. | Yes/No (delete as appropriate). |
| 3.6. | Cleaning elements (blades) | Combined cleaning elements with metal ceramics, fastened with screws. Cross-sectional layers of the operating part of the cleaning elements: special wear-resistant steel, rubber, metal ceramic elements in rubber, wear-resistant steel. The metal ceramic area must cover at least 40% of the contact surface area. In unused cleaning elements (supplied, mounted on snow plows), the lower plane of the metal ceramics must coincide with the plane passing through the lower (lowest) points of the wear-resistant steel. The thickness of the working part must be at least 36 mm. An additional point is awarded for an operating thickness of 50 mm. | The proposed parameter is specified in the tender form. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.6.1 | Priority requirement – Mounting brackets for the cleaning elements | The combined cleaning elements of the snow plow shall be attached to the body via cleaning element holders made of polyurethane or equivalent materials. The holders shall be manufacturer-supplied and installed along the entire width of the plow. The purpose of the holders is to reduce noise during snow removal and to mitigate the loads acting on the snow plow and the truck. | The proposed parameter is specified in the tender form. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.7. | Lowering/raising the plow to/from the road surface | Fully hydraulically operated, in all positions. | Yes/No (delete as appropriate). |
| 3.8. | “Floating“ plow position relative to the road, with assisted pressure function | Must be provided by the manufacturer and ensured by means of a hydraulic system. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.9. | Operation control | The plow control panel must have a connection for reading operation control (cleaning process) data. The service provider installing the transport control system must be given access and the opportunity to connect to the control panel by providing the necessary login codes. | Yes/No (delete as appropriate). |
| 3.10. | Plow length, measured along the cleaning elements. | The total plow length along the cleaning elements shall be at least 5000 mm. Additional points shall be awarded for a greater plow length. | Yes/No (delete as appropriate). Proposed parameter – ______(enter)_mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.11. | The width of the section being cleaned at a rotation of at least 30°. | From at least 4200 mm to 4400 mm or more An additional point is awarded for a larger width of the cleaned section. | Yes/No (delete as appropriate). Proposed parameter - ______(enter)_mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.12. | Plow rotation | Performed on both sides using hydraulic cylinders at an angle of no less than 30 o. | Yes/No (delete as appropriate). Proposed parameter - ______(enter)_ o angle. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.13. | Plow height | 1070 - 1700 mm (including cleaning elements and a rigid (polymer or equivalent) material roof). | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.14. | Plow marker lighting | Double-sided marker lighting, lights with LED bulbs. Regardless of the operating position of the plow (width of the section being cleaned), the marker lights must be installed so that they mark the outer corners of the plow. LED lamp protection against solid objects and liquids not less than IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.14.1 | Additional marker lighting | Additional marking of the working left and right edges of the snow plow with LED double-sided lights. LED light protection against solid objects and liquids is not less than IP65. The lights are installed in a protective stainless-steel cover that protects them from mechanical damage. | Yes/No (delete as appropriate). |
| 3.15. | Shock absorption and obstacle protection mechanism | Compensating for unexpected pressure increases in the hydraulic system due to an obstacle or impact. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.16. | Angle of attack | The inclination angle of the plow cutting edge shall be not less than 7° relative to the vertical plane. | Yes/No (delete as appropriate). Proposed parameter - ______(enter)_ o angle. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.17. | Weight of the plow and suspension equipment | No more than 1900 kg. An additional point is awarded for lower weight. | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.18. | Surface copying supports | The plow is equipped with support wheels of adjustable height, fitted with pneumatic or solid rubber or other polymer tires, with mudguards on the top and rear. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.19. | Plow storage legs | Must be supplied with the item. Storage legs with casters that allow the storage position of the plow to be changed and adjusted without the use of additional self-propelled lifting mechanisms. | Yes/No (delete as appropriate). |

* The Supplier must confirm compliance with the technical requirement by indicating: Yes/No (delete as appropriate) and, where required, enter the exact parameter/value of the proposed Item.

Where required, the Supplier must enter the name of the document proving the parameters and/or a reference to the source confirming the proposed parameters.

____________________                               _____________                                       _____________________

(position)	                            (signature)	       (forename, surname)

If the tender is signed by a person empowered by the Supplier, a document (power of attorney) granting the specified person the right to sign on behalf of the Supplier must be submitted together with the tender.

PART 2 OF THE PROCUREMENT. SALT SPREADER (PULL-ON) AND FRONT UNIVERSAL CAR SNOW PLOW FOR FOUR-WHEEL DRIVE VEHICLES (8X4(2+2)) (1 SET)

| Ser. No | Name of characteristics | Characteristics required by the Purchaser | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the Item offered. In the clauses where required, the Supplier must enter the name of the document submitted, proving the values ​​of the parameters, and/or a reference to the source confirming the parameters offered. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1. | Purpose | The equipment (a set consisting of a combined salt spreader and snow plow) is designed for clearing loose and compacted snow and spreading anti-skid materials on road surfaces. The salt spreader is designed for installation and operation in place of the body with a four-axle truck Renault C with a hydraulic crank lift-puller with a telescopic crank HIAB MULTILIFT ULTIMA 24S58. The snow plow is designed to be mounted on the front additional equipment suspension plate. | Yes/No (delete as appropriate). |
| 1.2. | Year of production | The salt spreader and snow plow must be new and unused prior to the date of transfer. | Yes/No (delete as appropriate). Year and quarter of production – (enter) ___________. |
| 1.3. | The dominant colour of the salt spreader and of the painted metal surfaces and of front snow plow | Orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s specified base (representative) colour for a specific group of equipment. Galvanized metal surfaces, elements and assemblies made of stainless-steel, aluminium or other corrosion-resistant materials may be left unpainted if this is not technically necessary and the welds are properly treated. | Yes/No (delete as appropriate). Proposed dominant colour – (enter parameter) _____________. |
| 1.4. | Warranty against corrosion perforation | All components included in the set, without exception, have a minimum warranty period of 72 months. | Yes/No (delete as appropriate). Proposed parameter - ____ months. |
| 1.5. | Right to perform warranty repairs and warranty technical maintenance | The Supplier must be the official representative of the manufacturer of the offered Goods (if the Supplier is not the manufacturer), having the right to perform technical maintenance and repair of the offered Goods during the warranty period, or must have concluded a corresponding service agreement with another entity authorized to perform such services. The tender must include the manufacturer’s authorization or another document confirming the Supplier’s right to provide technical maintenance and repair services during the warranty period, or evidence of a concluded service agreement with another entity authorized to provide such services, or the Supplier must provide a manufacturer-issued authorization granting the Purchaser the right to perform technical maintenance and repair services during the warranty period. | Yes/No (delete as appropriate). |
| 1.6. | Technical service and warranty repair terms | The Supplier must perform warranty repair works within no more than 5 working days after the Purchaser’s request is submitted in writing (by e-mail) and by a phone call to the Supplier’s responsible person indicated in the Contract, or may authorize the Purchaser during the warranty period (providing a manufacturer-issued authorization) to carry out warranty repair works by qualified Purchaser’s personnel, with the costs of the works and other related expenses reimbursed in accordance with the conditions specified in the Contract. | Yes/No (delete as appropriate). |
| 1.7. | Documentation | Operating and safety instructions in Lithuanian (user manuals for salt spreaders and snow plows) shall be provided no later than on the date of delivery of the item. | Yes/No (delete as appropriate). |
| 1.8. | Training | At the time of delivery of the Item, but no later than when the Purchaser begins to use the Item or part of the Item set, the Supplier must provide the following training, the cost of which must be included in the tender price: - theoretical and practical training for operators to ensure the correct use of the item. - technical training to ensure the correct maintenance of the item, explaining the nature of faults and fault symbols. - technical training to ensure the correct and safe disassembly /assembly/ replacement of the components of the set. | Yes/No (delete as appropriate). |
| 1.8.1 | Approving certificate | After completing theoretical and technical training and practical training, the Supplier (including partners – Suppliers of individual components of the set) shall issue a certificate describing the scope of the training course to the operator(s). | Yes/No (delete as appropriate). |
| 1.9. | Manufacturer, installer of complete equipment | Snow removal and salt spreading equipment shall be manufactured and installed on a truck by the manufacturer of the equipment or its official representative (in which case documents or copies thereof confirming the representation must be submitted with the tender). Alternatively, the Purchaser may carry out the installation, provided that the Supplier covers all costs incurred by the Purchaser for such installation. Responsibility for the flawless operation of the entire equipment complex and for remedying any defects discovered during operation shall rest with the manufacturer of the equipment or its official representative who carried out the installation, or with the Purchaser in the case where the Purchaser performed the installation. An additional point is awarded for the proposed salt spreading and snow removal equipment from the same manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2. SALT SPREADER |  |  |  |
| 2.1. | Purpose, adaptation Type, make | Designed for a four-axle truck Renault C with a wheel configuration of 8x4 (2+2) and a wheelbase of at least 3700 mm, mounted/pulled-on on the chassis using a hydraulic crank lift-puller with a telescopic boom. The frame of the salt spreader and the parameters of the pull loop must comply with standard DIN 30722. The salt spreader must comply with the requirements of standard LST EN 15597-1:2020. Designed for spreading, with or without moistening, salt, sand, fine crushed stone and/or spraying salt solution on the road surface. The spreader must be a standard product manufactured by the manufacturer in series, not specially modified or adapted to the requirements formulated by the contracting authority, except for the pair manufactured specifically for the spreader, specific marking and safety requirements. | Yes/No (delete as appropriate). Exact spreader model - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.2. | Quick connection and activation system for hydraulic and electrical systems of the salt spreader. | The necessary electrical and hydraulic connections must be provided and installed by the manufacturer and marked accordingly. The manufacturer must provide an additional control line and load sensor controller for a load-sensitive truck system (Load Sensing or equivalent). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.3. | Centre of gravity of the mounted spreader | The centre of gravity of the spreader mounted/secured on the truck shall be positioned as close as possible to the truck’s third axle. | Yes/No (delete as appropriate). |
| 2.4. | Installing of the salt spreader on a car | Pulling on/removing is performed using the truck’s hydraulic crank lift/puller HIAB MULTILIFT ULTIMA 24S58. The height of the spreader frame pull loop is 1570 mm. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.5. | Storage of the salt spreader | The manufacturer must provide support structures for the dismantled/removed salt spreader - rollers that allow the spreader to rest stably on the warehouse floor or asphalt or concrete surface and can be stored during the off-season. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.6. | Upper overall dimension of the spreader. | The height of the salt spreader, including the spreader bracket for mounting, shall not exceed 2 700 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.7. | Weight of spreader, complete with all mounting equipment, including bracket adapted for hydraulic crank lift-puller. | 2 500 – 4 000 kg | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.8. | Spreader adjustment, calibration | No later than on the day of handover of the Goods to the Purchaser, testing, adjustment and calibration of the salt spreader installed on the vehicle shall be carried out, and a calibration/verification document approved by the Supplier or its authorised representative shall be provided, indicating the actual quantities of material spread/sprayed under various operating modes. All required works shall be performed at the Supplier’s expense, except for the quantities of salt, sand–salt mixture and brine solution required for testing and calibration, as well as the loading of materials into the spreader. In the event that the installation of the equipment is delegated to the Purchaser, a manufacturer-issued authorisation shall be provided to the Purchaser, granting the right to perform installation, testing, adjustment and calibration works. | Yes/No (delete as appropriate). |
| Materials hopper |  |  |  |
| 2.10. | Materials hopper | Coated with anti-corrosion coating and painted or made of stainless-steel. The metal surface painting process must be carried out in accordance with the requirements of ISO 8501-1. The coating layer thickness must be at least 180 µm. Colour - orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s specified base (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). Proposed material and colour - _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.11. | Hopper capacity | Possible load capacity of loose materials – at least than 7 m3. | Yes/No (delete as appropriate). Proposed parameter - ____ m3. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12. | Spreading materials feeding device. | A rubber belt conveyor, ensuring smooth feeding of materials to the spreading disc, regardless of the type and amount of the spreading material in the hopper. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12.1 | Material dosing device position control mechanism | The equipment shall include a manufacturer-supplied mechanism for controlling the amount of material dispensed, operated manually. An additional point is awarded for an additional installed mechanism that adjusts the height of the material dosing valve according to the material selected on the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.13. | Mechanism for even distribution of the spreading material. | A separate mechanism installed along the entire length of the hopper, with a separate hydraulic drive or other type of drive and drive control, which distributes the spreading material more evenly over the rubber belt conveyor. | Yes/No (delete as appropriate). Type and description of the proposed device - ____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.14. | Hopper cover | Covered with a metal grate (opening dimensions: no less than 40x40 mm, but no more than 70x70 mm, wire-rod diameter no less than 4 mm) and a moisture-proof, easily closable and openable side tarpaulin without additional means. The levers of the cover opening/closing mechanism must be accessible to the operator (the covering/uncovering process must be possible) while standing on the ground or on a ladder installed at the rear of the spreader. The grate structure is hot-dip galvanized. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.14.1 | Protection against falling down of the spreading material | The lower part of the materials hopper must be equipped with protection to prevent the spreading material from falling directly onto the truck chassis, elements of hydraulic lift/puller and transmission located between the longitudinal side profiles of the frame. It must be possible to wash away any residue of the spreading material that accumulates on the protective device with a water jet, without using additional tools (for removing the bottom (protection) of wrenches). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Salt solution tanks |  |  |  |
| 2.15. | Salt solution tanks | Made from corrosion-resistant material. Salt solution tanks are installed on the sides of the spreader hopper, at the bottom, and at the front of the spreader. The total volume of the salt solution tanks is at least 5,9 m³. | Yes/No (delete as appropriate). Proposed parameter - _____ m³. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.15.1 | Materials used in manufacturer | Tanks must be made of polypropylene or another equivalent polymer material that is resistant to corrosion, temperature fluctuations, and ultraviolet radiation. The resistance of the polymer materials used in salt solution tanks to ultraviolet radiation and other environmental conditions must comply with the DIN EN-17106 standard. | Yes/No (delete as appropriate). Proposed parameter – meets the requirement _____ . Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Control |  |  |  |
| 2.16. | Control of salt spreading and moistening equipment | Performed from the vehicle cab, using a digital control panel that is easily accessible to the driver from his workplace. | Yes/No (delete as appropriate). |
| 2.17. | Manual control | The possibility provided by the manufacturer to control the spreading process manually (in case of emergency). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18. | Control panel | The spreader control panel must ensure automatic spreading and/or solution spraying according to a selected, pre-programmed route. During the shift, the operator must be able to continuously view and control at least the following parameters on the control panel display: - Selected spreading material and spreading rate; - Spreading width and asymmetry; - Moistening of the spreading material with solution is on/off, selected moistening percentage (%); - Amount of spreading material; - Solution spraying is on/off, selected amount of sprayed material (ml/m² or g/m²) (depending on the manufacturer’s settings); - Amount of solution. The control panel must: - collect data and ensure digital scanning and real-time (“On Line “) transmission of data to the dispatcher ‘s workstation; - inform the operator about the shortage of the spreading materials or salt solution with an additional audible signal; - have connections for data transmission in accordance with data encryption standard EN15430 or an equivalent standard. - have a menu in Lithuanian. The control panel menu must indicate the following names of spreading materials (may be without Lithuanian symbols, e.g., Misinys (Mixture) 1:1): • Salt • Mixture 1:1 • Mixture 1:3 • Mixture 1:10 • Solution The Supplier must provide the Purchaser with the access codes required to adjust the control panel settings. The meanings of the control panel buttons must be provided in the user manual. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18.1 | Priority requirement – Interface of the spreading equipment with external road condition information systems and their stations | The spreading equipment (control panel, software, etc.) shall have a real-time interface/integration with the road condition information system and its stations located in the Lithuanian road infrastructure (“Klimator” or equivalent). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18.2 | Priority requirement – Functions of the interface with road condition information systems | The spreading equipment shall have the functionality to analyse or compare data received from the interface with the road condition information system (upcoming or recorded weather changes) and, according to the specified algorithm, automatically adjust the operational parameters of the spreader (dosage of anti-slip materials, spreading/spraying width, etc.) in order to timely and preventively ensure the preparation of the road surface for the upcoming/recorded weather changes. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.19. | Operation data transmission system | The Supplier must provide access to the control panel data transmission, at least for the following data (for the purpose of controlling the spreader operation): 1. Date, time, operating time. 2. Spreader operating parameters: spreading is on/off or spreading mode, spreading time, spreading width, name of the material being spread, dosage of the material being spread g/m², amount spread kg, solution dosage ml/m² or g/m² (depending on the manufacturer’s specifications), amount of solution dispensed in litres. 3. Position of snow plow: operating/transport position. Transport telemetry control services are provided to the Purchaser by UAB Xirgo Global. The tender must be accompanied by information that clearly and comprehensively shows the required data. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.20. | Generation of data transmission system reports | Reports must be generated freely by selecting the date and time. They must include: The date of spreading, the start/end time of spreading. The start/end location of spreading, total spreading time, spreading distance, and consumption of spreading material and salt solution (kg, litres). Dosage of spreading material in g/m², dosage of solution in ml/m² or g/m² (depending on the manufacturer’s specifications). Distance travelled by the vehicle and spreader, m. Name of material being spread. Detailed information about the reports generated and the required data included in them must be provided. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.21. | Even spreading and salt solution feeding. | The vehicle movement speed signal must be transmitted to the spreader control panel (linked). The even feed of salt and salt solution to the spreading disc, the even rotation of the disc and the spreading of materials, according to the settings on the control panel, must take place regardless of the vehicle movement speed, as well as when the vehicle is stationary. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.22. | Emergency unloading function. | Must be provided for by the manufacturer and activated via the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.23. | Spreading parameter programming and fault notification system. | Must be provided for and installed by the manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.24. | Instantaneous increase of the amount being spread. | A function that allows the amount of material being spread to be increased two or more times for a limited period of time. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Spreading (dispersal) mechanism |  |  |  |
| 2.25. | System for mixing the material being spread and solution. | Ensuring mixing of the spreading material and salt solution and feed of the mixture onto the spreading disc, or mixing of salt and solution directly on the spreading disc. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26. | Salt moistening and spreading mechanism | Variable length sleeve/trough and disc with hydraulic drive, liftable upwards and lockable in transport position. Air shock absorbers provided for and installed by the manufacturer, or equivalent solution to facilitate lifting of the mechanism into transport position. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27. | Spreading disc | Diameter not less than 600 mm. The height of the disc above the road surface is adjusted mechanically (manually), at intervals of not less than 100 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.28. | Spreading disc hydraulic motor | Protected from direct exposure to salt. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29. | Salt spreading sensor | Must be contactless, electronic. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29.1 | Visual control | A rear-view camera with night vision, installed by the spreader manufacturer, is mounted at the rear of the spreader, as high as technically possible, for visual control of the spreading process and control of the vehicle’s reverse movement. The image transmitted by the camera must be visible at the operator’s workplace (in the cab of truck) on the monitor (in a separate section) located on the spreader control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.30. | Adjustable amount of material being spread per m² | Salt - within the range of 5-40 g/m², with a maximum “pace“ of 5 g/m². Sifted sand and crushed stone - within the range of 20-300 g/m², with a maximum “pace“ of 10 g/m². | Yes/No (delete as appropriate). Proposed parameter: Salt within the range of ____ g/m2. Spread gravel, sand, crushed stone within the range of ____ g/m2. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.31. | Adjustable spreading width of the material being spread. | From 3 m to at least 12 m, at intervals of no more than 1,0 m, with asymmetrical (increasing or decreasing the width of one side of the spread without changing the width of the other side) adjustment of the spreading material, set using the control panel. | Yes/No (delete as appropriate). Proposed parameter: Within ____ m, spreading width “pace” of __m. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.32. | Forced (pressure) salt solution feeding (to the spreading disc) and control of amount. | At interval of 5 %, starting from a minimum of 15% to 100% (FS15 – FS100), which automatically changes depending on the amount of salt being spread. It is assumed that the percentage of salt solution for salt moistening indicates the proportion of the solution in the spread rate (e.g., FS15 means that the spread rate consists of 15% solution + 85% dry matter). | Yes/No (delete as appropriate). Proposed parameter: At __ % interval from ___ % Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Solution spraying (dispersion) mechanism |  |  |  |
| 2.33. | Salt solution spraying (dispersal) mechanism | The solution spraying mechanism is installed in the rear part of the salt spreader. The mechanism's units and components are mounted on a bracket connected to the spreading mechanism or on a separate bracket(s). The mechanism is lifted up and locked in the transport position together with the spreading mechanism or separately. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.34. | Salt solution pump(s) | Pumps made of stainless steel AISI 316 or other equivalent material resistant to brine solution. The capacity of the hydraulic pumps shall be adjustable via the control panel: – Pump used for the brine solution spraying function, with a capacity in the range of 1 l/min to 290 l/min; – Pump used for the brine solution for the salt wetting function, with a capacity in the range of 1 l/min to 100 l/min. | Yes/No (delete as appropriate). Proposed parameter: material of pump(s) _________ capacity in the range from ___ litres/min to ___ litres/min Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.35. | Spraying widths | The control system must ensure: - different spray widths - (symmetrical) width from 3.0 to at least 12.0 m, and asymmetrical, at least 7.0 m to the left and right. - the ability to adjust the width of the sprayed material in meters (1.0 m offset from the driving axis). - at least three separate spray direction positions: right, left, and centre. - Uniform spraying regardless of the vehicle’s speed when traveling at speeds of up to 70 km/h. | Yes/No (delete as appropriate). Proposed parameters: Symmetrical width from ___ m to ___ m. Asymmetrical width ___ m Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.36. | Dosage | The amount of sprayed salt solution is adjustable: from 10 ml/m² or g/m² to at least 100 ml/m² or g/m² (depending on the manufacturer’s specifications). The dosing “pace” is no more than every 5 ml/m² or g/m². | Yes/No (delete as appropriate). Proposed parameters: from ___ ml/m² or g/m² to ___ ml/m² or g/m². Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Safety fence |  |  |  |
| 2.37. | Safety fence – lightweight construction, 1-1.15 m high, on the both sides of the spreader. | Mounted on top of the salt spreader hopper and raised/folded for maintenance/repair before accessing the hopper. The raising/lowering of the Safety fence is performed by lowering/raising the ladder intended for access to the hopper or by a separate raising/lowering mechanism. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Signal marking |  |  |  |
| 2.38. | Dimensional marking, signal lighting | The working (spreading/dispersal) area must be illuminated by at least one light. At least one orange LED beacon installed at the rear of the spreader. Two flashing LED lights (round lights) with a diameter of at least 180 mm, installed at the upper corners of the rear part of the spreader. The rear part marked with warning light-reflecting diagonal red and white stripes. Sample pattern of warning marking: Protection of LED beacons and flashing LED lights against solid objects and liquids must be at least IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.39. | Mounting locations for road signs and road signs. | The rear part of the spreader (on the mounting/suspension unit/frame) shall be equipped with places for mounting warning/directional road signs or the necessary mounting structure, and no later than on the date of delivery of the Item, the following shall be installed: - a light-reflecting sign No 106 „Work“ not less than 1 (first) size: - Signal LED light pointer – a system of at least 13 lights complying with L8H, EN 12352 or an equivalent standard, fastened from the rear with screws or equivalent fasteners. The direction of the arrow indicating the mandatory side of the obstacle is changed from the operator’s workplace using an electrical connection. A visual indicator informing about the activated direction of the arrow is mandatory at the operator’s workplace. The supply voltage of the arrow lights is 24 V. The protection of the LED light system components against solid objects and liquids is not less than IP65. | Yes/No (delete as appropriate). |
| 3. FRONT SNOW PLOW |  |  |  |
| 3.1. | Purpose, adaptation Type, make | Designed to push snow both to the right and left. Enables snow removal while the truck is traveling at speeds of 60 km/h and above. The upper part of the blade is equipped with an additional C-shaped roof made of rigid plastic or equivalent rigid material along the entire length of the plow, which protects the vehicle’s windshield and front cabin grille from snow. The plow is of the segmented type. Additional point is awarded to a segmented snow plow where the segments can move (slide during snow removal) up/down independently of each other. The side (left-hand) part of the snow plow shall be folded or pushed to the left side (in this case, when the left-hand section is folded or pushed, its cleaning element blades shall be aligned in a straight line with the cleaning elements of the central section segments) or the mounting structure of the snow plow allows it to be shifted to the left and/or to the right. | The proposed parameter is specified in the tender form. Exact model of front snow plow - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.2. | Body | Made of steel, with stiffening ribs (metal primed and coated with two coats of paint). | Yes/No (delete as appropriate). |
| 3.3. | Working surface of the plow | Made of steel (metal primed and coated with two coats of paint). The colour of painted metal surfaces is orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). Enter proposed colour - _________. |
| 3.4. | Control | Lowering, lifting, turning left/right, folding and deployment are performed from the driver ‘s cab using a control panel located within easy reach of the driver. | Yes/No (delete as appropriate). |
| 3.4.1 | Control panel | The plow control panel is equipped with a connection for reading work control (cleaning process) data. The service provider installing the transport control system is given access and the opportunity to connect to the control panel by providing the necessary login codes. It must be possible to read the analogue signal for the plow position (snow being cleaned/not being cleaned), or the same data must be provided via the spreader control panel in accordance with EN 15430 or an equivalent standard. | Yes/No (delete as appropriate). |
| 3.5. | Mounting | It is mounted (suspended) using a special device-suspension at the front of the vehicle, which corresponds to the dimensions and mounting locations of the snow plow mounting plate. The suspension is manufactured in accordance with the applicable standards for front vehicle mounting plates (DIN 76060 type “A“ or equivalent). An additional point is awarded for additional snow plow (hydraulic lines to be used for lifting) and truck connections and/or couplings with an external control device (on the outside of the truck: on the front bumper, next to the driver‘s cab steps), allowing the operator to control the lifting/lowering of the snow plow (by connecting the hydraulic hoses to the truck’s hydraulic outlets) without being in the cab. | Yes/No (delete as appropriate). |
| 3.6. | Cleaning elements (blades) | Combined cleaning elements with metal ceramics, fastened with screws. Cross-sectional layers of the operating part of the cleaning elements: special wear-resistant steel, rubber, metal ceramic elements in rubber, wear-resistant steel. The metal ceramic area must cover at least 40% of the contact surface area. In unused cleaning elements (supplied, mounted on snow plows), the lower plane of the metal ceramics must coincide with the plane passing through the lower (lowest) points of the wear-resistant steel. The thickness of the working part must be at least 36 mm. An additional point is awarded for an operating thickness of 50 mm. | The proposed parameter is specified in the tender form. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.6.1 | Priority requirement – Mounting brackets for the cleaning elements | The combined cleaning elements of the snow plow shall be attached to the body via cleaning element holders made of polyurethane or equivalent materials. The holders shall be manufacturer-supplied and installed along the entire width of the plow. The purpose of the holders is to reduce noise during snow removal and to mitigate the loads acting on the snow plow and the truck. | The proposed parameter is specified in the tender form. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.7. | Lowering/raising the plow to/from the road surface | Fully hydraulically operated, in all positions. | Yes/No (delete as appropriate). |
| 3.8. | “Floating“ plow position relative to the road, with assisted pressure function | Must be provided by the manufacturer and ensured by means of a hydraulic system. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.9. | Operation control | The plow control panel must have a connection for reading operation control (cleaning process) data. The service provider installing the transport control system must be given access and the opportunity to connect to the control panel by providing the necessary login codes. | Yes/No (delete as appropriate). |
| 3.10. | Plow length, measured along the cleaning elements. | The total plow length along the cleaning elements shall be at least 5000 mm. Additional points shall be awarded for a greater plow length. | Yes/No (delete as appropriate). Proposed parameter – ______(enter)_mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.11. | The width of the section being cleaned at a rotation of at least 30°. | From at least 4200 mm to 4400 mm or more An additional point is awarded for a larger width of the cleaned section. | Yes/No (delete as appropriate). Proposed parameter - ______(enter)_mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.12. | Plow rotation | Performed on both sides using hydraulic cylinders at an angle of no less than 30 o. | Yes/No (delete as appropriate). Proposed parameter - ______(enter)_ o angle. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.13. | Plow height | 1070 - 1600 mm (including cleaning elements and a rigid (polymer or equivalent) material roof). | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.14. | Plow marker lighting | Double-sided marker lighting, lights with LED bulbs. Regardless of the operating position of the plow (width of the section being cleaned), the marker lights must be installed so that they mark the outer corners of the plow. LED lamp protection against solid objects and liquids not less than IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.14.1 | Additional marker lighting | Additional marking of the working left and right edges of the snow plow with LED double-sided lights. LED light protection against solid objects and liquids is not less than IP65. The lights are installed in a protective stainless-steel cover that protects them from mechanical damage. | Yes/No (delete as appropriate). |
| 3.15. | Shock absorption and obstacle protection mechanism | Compensating for unexpected pressure increases in the hydraulic system due to an obstacle or impact. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.16. | Angle of attack | The inclination angle of the plow cutting edge shall be not less than 7° relative to the vertical plane. | Yes/No (delete as appropriate). Proposed parameter - ______(enter)_ o angle. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.17. | Weight of the plow and suspension equipment | No more than 1900 kg. An additional point is awarded for lower weight. | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.18. | Surface copying supports | The plow is equipped with support wheels of adjustable height, fitted with pneumatic or solid rubber or other polymer tires, with mudguards on the top and rear. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.19. | Plow storage legs | Must be supplied with the item. Storage legs with casters that allow the storage position of the plow to be changed and adjusted without the use of additional self-propelled lifting mechanisms. | Yes/No (delete as appropriate). |

* The Supplier must confirm compliance with the technical requirement by indicating: Yes/No (delete as appropriate) and, where required, enter the exact parameter/value of the proposed Item.

Where required, the Supplier must enter the name of the document proving the parameters and/or a reference to the source confirming the proposed parameters.

____________________                               _____________                                  _____________________

(position)	                             (signature)	(forename, surname)

If the tender is signed by a person empowered by the Supplier, a document (power of attorney) granting the specified person the right to sign on behalf of the Supplier must be submitted together with the tender.

PART 3 OF THE PROCUREMENT. SALT SPREADER AND FRONT UNIVERSAL CAR SNOW PLOW FOR TWO-WHEEL DRIVE VEHICLES (4X4) (1 set)

| Eil. Nr. | Name of characteristics | Characteristics required by the Purchaser | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the Item offered. In the clauses where required, the Supplier must enter the name of the document submitted, proving the values ​​of the parameters, and/or a reference to the source confirming the parameters offered. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1. | Purpose. | The equipment (a set consisting of a combined salt spreader and snow plow) is designed for clearing loose and compacted snow and spreading anti-skid materials on road surfaces. The salt spreader is designed for installation and operation in place of the body with a two-axle truck Renault K 4x4R. The snow plow is designed to be mounted on the front additional equipment suspension plate. | Yes/No (delete as appropriate). |
| 1.2. | Year of production | The salt spreader and snow plow must be new and unused prior to the date of transfer. | Yes/No (delete as appropriate). Year and quarter of production – (enter) ___________. |
| 1.3. | The dominant colour of the salt spreader and of the painted metal surfaces and front snow plow | Orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific group of equipment. Galvanized metal surfaces, elements and assemblies made of stainless-steel, aluminium or other corrosion-resistant materials may be unpainted if this is not technically necessary and the welds are properly treated. | Yes/No (delete as appropriate). Proposed dominant colour – (enter parameter) _____________. |
| 1.4. | Warranty against corrosion perforation | All components included in the set, without exception, have a minimum warranty period of 72 months. | Yes/No (delete as appropriate). Proposed parameter – ____ months. |
| 1.5. | Right to perform warranty repairs and warranty technical maintenance | The Supplier must be the official representative of the manufacturer of the offered Goods (if the Supplier is not the manufacturer), having the right to perform technical maintenance and repair of the offered Goods during the warranty period, or must have concluded a corresponding service agreement with another entity authorized to perform such services. The tender must include the manufacturer’s authorization or another document confirming the Supplier’s right to provide technical maintenance and repair services during the warranty period, or evidence of a concluded service agreement with another entity authorized to provide such services, or the Supplier must provide a manufacturer-issued authorization granting the Purchaser the right to perform technical maintenance and repair services during the warranty period. | Yes/No (delete as appropriate). |
| 1.6. | Terms of technical service and warranty repairs | The Supplier must perform warranty repair works within no more than 5 working days after the Purchaser’s request is submitted in writing (by e-mail) and by a phone call to the Supplier’s responsible person indicated in the Contract, or may authorize the Purchaser during the warranty period (providing a manufacturer-issued authorization) to carry out warranty repair works by qualified Purchaser’s personnel, with the costs of the works and other related expenses reimbursed in accordance with the conditions specified in the Contract. | Yes/No (delete as appropriate). |
| 1.7. | Documentation | Operating and safety instructions in Lithuanian (user manuals for salt spreader) shall be provided no later than on the date of delivery of the Item. | Yes/No (delete as appropriate). |
| 1.8. | Training | At the time of delivery of the Item, but no later than when the Purchaser begins to use the Item or part of the set of the Item, the Supplier must provide the following training, the cost of which must be included in the offer price: - -theoretical and practical training for operators to ensure the correct use of the Item. - technical training to ensure the correct maintenance of the Item, explaining the nature of faults and fault symbols. - technical training to ensure the correct and safe disassembly/assembly/replacement of the components of the set. | Yes/No (delete as appropriate). |
| 1.8.1 | Approving certificate | The Supplier (including partners – Suppliers of individual components of the set) shall prepare a certificate describing the scope of the training course for the Operator(s) who has/have completed theoretical and technical training and practical training. | Yes/No (delete as appropriate). |
| 1.9. | Manufacturer and installer of bundled equipment | Snow removal and salt spreading equipment shall be manufactured and installed on a truck by the manufacturer of the equipment or its official representative (in which case documents or copies thereof confirming the representation must be submitted with the tender). Alternatively, the Purchaser may carry out the installation, provided that the Supplier covers all costs incurred by the Purchaser for such installation. Responsibility for the flawless operation of the entire equipment complex and for remedying any defects discovered during operation shall rest with the manufacturer of the equipment or its official representative who carried out the installation, or with the Purchaser in the case where the Purchaser performed the installation. An additional point is awarded for the proposed salt spreading and snow removal equipment from the same manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2. | SALT SPREADER |  |  |
| 2.1. | Purpose, type, make | Designed for a dump truck (instead of a body), designed for spreading, with or without moistening, salt, sand, gravel, and fine crushed stone. Must comply with the requirements of standard LST EN 15597-1:2020. The spreader must be a standard product manufactured by the manufacturer in series, not specially modified or adapted to the requirements formulated by the contracting authority, except for specific marking and safety requirements. | Yes/No (delete as appropriate). Exact spreader model - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.2. | Quick connection and activation system for the hydraulic and electrical systems of the salt spreader. | Electrical and hydraulic connections provided and installed by the manufacturer. Additional control line and load sensor controller required for load-sensitive system (Load Sensing or equivalent). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.3. | Salt spreader dimensions | The spreader is installed in such a way as to maximize the length of the vehicle’s body and not exceed the maximum permissible width of the vehicle. | Yes/No (delete as appropriate). |
| 2.4. | Installing a salt spreader on a vehicle. | The spreader must have a quick removal/installation system on the vehicle's additional frame. The spreader must be removed and installed without external assistance, i.e. without the use of additional self-propelled lifting devices. Lifting devices – with hydraulic cylinders. The metal structure of the quick removal/installation (storage) system and the lifting capacity of the lifting devices must ensure safe, long-term, and stable storage of the spreader (including when loaded with spreading material and/or filled with salt solution). | Yes/No (delete as appropriate). Permissible lifting capacity of lifting device (of each of the four) - ______kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.4.1 | Control of spreader removal/installation on the truck system (storage legs) | Control of the hydraulic devices of the salt spreader storage legs is performed via the hydraulic system of the truck. The snow plow control panel or another additional control device located outside the operator’s cab or a remote-control panel should be used for control. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.5. | Upper overall dimension of the spreader. | The height of the salt spreader, including the spreader bracket for mounting, shall not exceed 2300 mm. | Yes/No (delete as appropriate). Proposed parameter – ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.6. | Weight of the spreader with all mounting equipment. | No more than 2800 kg | Yes/No (delete as appropriate). Proposed parameter – ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.7. | Spreader adjustment, calibration | No later than on the day of handover of the Goods to the Purchaser, testing, adjustment and calibration of the salt spreader installed on the vehicle shall be carried out, and a calibration/verification document approved by the Supplier or its authorised representative shall be provided, indicating the actual quantities of material spread/sprayed under various operating modes. All required works shall be performed at the Supplier’s expense, except for the quantities of salt, sand–salt mixture and brine solution required for testing and calibration, as well as the loading of materials into the spreader. In the event that the installation of the equipment is delegated to the Purchaser, a manufacturer-issued authorisation shall be provided to the Purchaser, granting the right to perform installation, testing, adjustment and calibration works. | Yes/No (delete as appropriate). |
| Materials hopper |  |  |  |
| 2.9. | Materials hopper | Coated with anti-corrosion coating and painted or made of stainless-steel. The painting process for metal surfaces must be carried out in accordance with the requirements of ISO 8501-1. The coating layer thickness must be at least 180 µm. Orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s specified default (representative) colour for a specific technical group. Galvanized metal surfaces, elements and assemblies made of stainless-steel, aluminium or other corrosion-resistant materials may be left unpainted if it is not technologically necessary and the welds are properly treated. An additional point is awarded for a spreading material hopper made of stainless-steel. | Yes/No (delete as appropriate). Proposed material and colour - _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.10. | Hopper capacity | Possible load capacity of loose materials – 6,0 – 7,0 m3 An additional point is awarded for a larger capacity of the salt spreader hopper. | Yes/No (delete as appropriate). Proposed parameter – ____ m3. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.11. | Feeder of spreading materials. | A rubber belt conveyor, ensuring smooth feeding of materials to the spreading disc, regardless of the type and amount of the spreading material in the hopper. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12. | Mechanism for even distribution of the spreading material. | A separate mechanism installed along the entire length of the hopper, with a separate hydraulic drive or other type of drive and drive control, which distributes the spreading material more evenly over the rubber belt conveyor. | Yes/No (delete as appropriate). Type and description of the proposed device - ____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.13. | Hopper cover | Covered with a metal grate (opening dimensions: no less than 40x40 mm, but no more than 70x70 mm, wire-rod diameter no less than 4 mm) and a moisture-proof, easily closable and openable side tarpaulin without additional means. The levers of the cover opening/closing mechanism must be accessible to the operator (the covering/uncovering process must be possible) while standing on the ground or on a ladder installed at the rear of the spreader. The grate structure is hot-dip galvanized. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.13.1 | Hopper bottom (protection) | The lower part of the materials hopper must be equipped with protection, or protection against direct contact of the scattered material with the truck chassis and transmission elements located between the longitudinal side profiles of the frame must be installed at the Supplier's expense above the upper part of the truck frame. It shall be possible to wash off any spilled material residues accumulated on the protective device with a water jet without using additional tools (for removing the bottom (protective) nut). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Salt solution tank |  |  |  |
| 2.14. | Salt solution tanks | Tanks must be made of polypropylene or another equivalent polymer material that is resistant to corrosion, temperature fluctuations, and ultraviolet radiation. The resistance of the polymer materials used in salt solution tanks to ultraviolet radiation and other environmental conditions must comply with the DIN EN-17106 standard. At least two parts (installed on the sides of the spreading materials hopper, in the lower part). Total volume of at least 2,6 m³. Depending on the volume of the spreader's spreader material hopper, the ratio of the salt solution tank volume must be maintained at 7:3. | Yes/No (delete as appropriate). Proposed parameter – ____ m³. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Control |  |  |  |
| 2.15. | Control of salt spreading and moistening equipment | Performed from the vehicle cab, using a digital control panel that is easily accessible to the driver from his workplace. | Yes/No (delete as appropriate). |
| 2.16. | Manual control | The possibility provided by the manufacturer to control the spreading process manually (in case of emergency). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.17. | Control panel | During the shift, the operator must be able to continuously view and control at least the following parameters on the control panel display: - Selected spreading material and spreading rate; - Spreading width and asymmetry; - Moistening of the spreading material with solution is on/off, selected moistening percentage (%); - Amount of spreading material; - Amount of solution; The control panel must: - collect data and ensure digital scanning and real-time (“Online “) transmission of data to the dispatcher ‘s workstation; - inform the operator about the shortage of the spreading materials or salt solution with an additional audible signal; - have connections for data transmission in accordance with data encryption standard EN15430 or an equivalent standard. - have a menu in Lithuanian. The control panel menu must indicate the following names of spreading materials (may be without Lithuanian symbols, e.g., Misinys (Mixture) 1:1): • Salt • Mixture 1:1 • Mixture 1:3 • Mixture 1:10 • Solution The Supplier must provide the Purchaser with the access codes required to adjust the control panel settings. The meanings of the control panel buttons must be provided in the user manual. An additional point is awarded for a spreader control panel equipped with an automatic spreading function according to a pre-programmed selected route. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18. | System for transferring data about operation | The Supplier must provide access to the control panel data transmission, at least for the following data (for the purpose of controlling the spreader operation): 1. Date, time, operating time. 2. Spreader operating parameters: spreading is on/off or spreading mode, spreading time, spreading width, name of the material being spread, dosage of the material being spread g/m², amount spread kg, solution dosage ml/m² or g/m² (depending on the manufacturer’s specifications), amount of solution dispensed in litres. 3. Position of snow plow: operating/transport position. Transport telemetry control services are provided to the Purchaser by UAB Xirgo Global. The tender must be accompanied by information that clearly and comprehensively shows the required data. | Yes/No (delete as appropriate). |
| 2.19. | Generation of data transmission system reports | Reports must be generated with freely selectable dates and times. They must include: The date of spreading, the start/end time of spreading. The start/end location of spreading, the total spreading time, the spreading distance, the consumption of the spreading material and salt solution (kg, litres). The dosage of the spreading material in g/m², the dosage of the solution in ml/m² or g/m² (depending on the manufacturer’s specifications). Distance travelled by the vehicle and spreader, m. Names of materials spread. The tender must include detailed information about the reports to be generated and the required data to be included in them. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.20. | Even spreading and salt solution feeding. | The vehicle speed signal must be transmitted to the spreader control panel (linked). The even supply of salt and salt solution to the spreading disc, the even rotation of the disc and the spreading of materials, according to the settings on the control panel, must take place regardless of the vehicle’s speed, as well as when the vehicle is stationary. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.21. | Emergency unloading function. | Must be provided by the manufacturer and activated via the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.22. | Spreading parameter programming and fault notification system. | Must be provided and installed by the manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.23. | Instantaneous increase of the amount being spread. | A function provided and installed by the manufacturer that allows the amount of material spread to be increased two or more times for a limited period of time. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Spreading (dispersal) mechanism |  |  |  |
| 2.24. | Spreading material and solution mixing system. | Ensuring mixing of the spreading material and salt solution and delivery of the mixture onto the spreading disc, or mixing of salt and solution directly on the spreading disc. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.25. | Salt moistening and spreading mechanism | Variable length sleeve/gutter and disc with hydraulic drive, lifted upwards and locked in the transport position. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26. | Spreading disc | Diameter not less than 600 mm. The height of the disc above the road surface is adjusted mechanically (manually) at interval not less than 100 mm. | Yes/No (delete as appropriate). Proposed parameter – ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27. | Spreading disc hydraulic motor | Protected from direct exposure to salt. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.28. | Salt spreading sensor | Must be provided and installed by the manufacturer - contactless, electronic. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.28.1 | Visual control | A rear-view camera with night vision, installed by the spreader manufacturer, is mounted at the rear of the spreader, as high as technically possible, for visual control of the spreading process and control of the vehicle’s reverse movement. The image transmitted by the camera must be visible at the operator’s workplace (in the cab of truck) on the monitor (in a separate section) located on the spreader control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29. | Adjustable amount of material being spread per m² | Salt - within the range of 5-40 g/m², with a maximum “pace“ of 5 g/m². Sifted sand and crushed stone - within the range of 20-300 g/m², with a maximum “pace“ of 10 g/m². | Yes/No (delete as appropriate). Proposed parameter: Salt within the range of ____ g/m2. Spread gravel, sand, crushed stone within the range of ____ g/m2. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.30. | Adjustable spreading width of the material being spread. | From 3 m to at least 12 m, at intervals of no more than 1,0 m, with asymmetrical (increasing or decreasing the width of one side of the spread without changing the width of the other side) adjustment of the spreading material, set using the control panel. | Yes/No (delete as appropriate). Proposed parameter: Within ____ m, spreading width “pace” of __m. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.31. | Forced (pressure) salt solution feeding (to the spreading disc) and control of amount. | At interval of 5 %, starting from a minimum of 15%, which automatically changes depending on the amount of salt being spread. It is assumed that the percentage of salt solution for salt moistening indicates the proportion of the solution in the spread rate (e.g., 15% means that the spread rate consists of 15% solution + 85% dry matter). | Yes/No (delete as appropriate). Proposed parameter: At __ % interval from ___ % Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Safety fence |  |  |  |
| 2.32. | Safety fence – lightweight construction, 1-1.15 m high, on the both sides of the spreader. | Mounted on top of the salt spreader hopper and raised/folded for maintenance/repair before accessing the hopper. The raising/lowering of the Safety fence is performed by lowering/raising the ladder intended for access to the hopper or by a separate raising/lowering mechanism. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Signal marking |  |  |  |
| 2.33. | Dimensional marking, signal lighting | The working (spreading/dispersal) area must be illuminated by at least one light. At least one orange LED beacon installed at the rear of the spreader. Two flashing LED lights (round lights) with a diameter of at least 180 mm, installed at the upper corners of the rear part of the spreader. The rear part marked with warning light-reflecting diagonal red and white stripes. Sample pattern of warning marking: Protection of LED beacons and flashing LED lights against solid objects and liquids must be at least IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.34. | Mounting locations for road signs and road signs. | The rear part of the spreader (on the mounting/suspension unit/frame) shall be equipped with places for mounting warning/directional road signs or the necessary mounting structure, and no later than on the date of delivery of the Item, the following shall be installed: - a light-reflecting sign No 106 “Work“ not less than 1 (first) size: - Signal LED light pointer – a system of at least 13 lights complying with L8H, EN 12352 or an equivalent standard, fastened from the rear with screws or equivalent fasteners. The direction of the arrow indicating the mandatory side of the obstacle is changed from the operator’s workplace using an electrical connection. A visual indicator informing about the activated direction of the arrow is mandatory at the operator's workplace. The feed voltage of the arrow lights is 24 V. The protection of the LED light system components against solid objects and liquids is not less than IP65. | Yes/No (delete as appropriate). |
| 3. FRONT SNOW PLOW |  |  |  |
| 3.1. | Purpose, adaptation Type, make | Designed to push snow to the right, or in some cases to the left. Enables snow removal while the vehicle is moving at speeds of up to 60 km/h. The upper part of the blade is equipped with an additional C-shaped roof made of rigid plastic or a similar rigid material along the entire length of the wiper, which protects the car windshield from snow being thrown onto it. The wiper is of a segmented type. The structure consists of at least three sections. An additional point is awarded for four or more sections. An additional point is awarded for a segmented snow wiper when the segments can move (slide during snow removal) up/down independently of each other. | The proposed parameter is specified in the tender form. Exact model of front snow plow - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.2 | Body | Made of steel, with rigid edges (metal primed and coated with two coats of paint). Colour: orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific group of equipment. Galvanized metal surfaces, elements and assemblies made of stainless-steel, aluminium or other corrosion-resistant materials may be left unpainted if this is not technically necessary and the welds are properly treated. | Yes/No (delete as appropriate). Enter proposed colour - _________. |
| 3.3. | Working surface of the plow | Made of steel (metal primed and coated with two coats of paint). The colour of painted metal surfaces is orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). |
| 3.4. | Control | Lowering, lifting, and turning left/right are performed from the driver ‘s cab using a control panel located within easy reach of the driver. | Yes/No (delete as appropriate). |
| 3.4.1 | Control panel | The plow control panel must be equipped with a connection for reading work control (cleaning process) data. The service provider installing the transport control system is given access and the opportunity to connect to the control panel by providing the necessary login codes. It must be possible to read the analogue signal for the plow position (snow being cleaned/not being cleaned), or the same data must be provided via the spreader control panel in accordance with EN 15430 or an equivalent standard. | Yes/No (delete as appropriate). |
| 3.5. | Mounting | It is mounted (suspended) using a special device-suspension at the front of the vehicle, which corresponds to the dimensions and mounting locations of the snow plow mounting plate. The suspension is manufactured in accordance with the applicable standards for front vehicle mounting plates (DIN 76060 type “A“ or equivalent). An additional point is awarded for additional snow plow (hydraulic lines to be used for lifting) and truck connections and/or couplings with an external control device (on the outside of the truck: on the front bumper, next to the driver‘s cab steps), allowing the operator to control the lifting/lowering of the snow plow (by connecting the hydraulic hoses to the truck’s hydraulic outlets) without being in the cab. | Yes/No (delete as appropriate). |
| 3.6. | Cleaning elements (blades) | Combined cleaning elements with metal ceramics, fastened with screws. Cross-sectional layers of the operating part of the cleaning elements: special wear-resistant steel, rubber, metal ceramic elements in rubber, wear-resistant steel. The metal ceramic area must cover at least 40% of the contact surface area. In unused cleaning elements (supplied, mounted on snow plows), the lower plane of the metal ceramics must coincide with the plane passing through the lower (lowest) points of the wear-resistant steel. The thickness of the working part must be at least 36 mm. An additional point is awarded for an operating thickness of 50 mm. | Proposed parameter _________ mm Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.7. | Lowering/raising the plow to/from the road surface | Fully hydraulically operated, in all positions. | Yes/No (delete as appropriate). |
| 3.8. | “Floating“ plow position relative to the road, with assisted pressure function | Must be provided by the manufacturer and ensured by means of a hydraulic system. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.9. | Operation control | The plow control panel must be equipped with a connection for reading work control (cleaning process) data. The service provider installing the transport control system is given access and the opportunity to connect to the control panel by providing the necessary login codes. | Yes/No (delete as appropriate). |
| 3.10. | The height of raising the lowest point of the plow from the road surface. | Not less than 300 mm. | Yes/No (delete as appropriate). Proposed parameter: - ______(enter)_mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.11. | Plow length at cleaning elements | No less than 3600 mm (during cleaning, with the plow turned to the maximum, the wheels on both sides of the car must fall within the cleaning area). | Yes/No (delete as appropriate). Proposed parameter - ______(enter)_mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.12. | Plow height | 1000-1500 mm (including cleaning elements and rigid material roof). | Yes/No (delete as appropriate). Proposed parameter - ______(enter)_mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.13. | Plow rotation | Fully hydraulically operated to both sides, with lockable intermediate positions. Possible rotation angle in both directions of at least 30 o. | Yes/No (delete as appropriate). Proposed parameter - ______ o (enter) angle. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.14. | The width of the section being cleaned, with a rotation of at least 30 o. | Not less than 3000 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.15. | Plow marker lighting and marking | Dimension lighting - double-sided, lamps with LED bulbs. The rear corners are marked with warning light-reflecting diagonal red and white stripes. Sample warning marking pattern: | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.15.1 | Additional marker lighting | Additional marking of the working left and right edges of the snow plow with LED double-sided lights. LED light protection against solid objects and liquids is not less than IP65. The lights are installed in a protective stainless-steel cover that protects them from mechanical damage. Example of double-sided lights: | Yes/No (delete as appropriate). |
| 3.16. | Shock absorption and obstacle protection mechanism | Compensating for unexpected pressure increases in the hydraulic system due to an obstacle or impact. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.17. | Angle of attack | The inclination angle of the plow cutting edge shall be not less than 7° relative to the vertical plane. | Yes/No (delete as appropriate). Proposed parameter - ______ o (enter)_ angle. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.18. | Weight of the plow and suspension equipment | 900 - 1 100 kg. | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.19. | Surface copying supports | The plow is equipped with support wheels of adjustable height, fitted with pneumatic or solid rubber or other polymer tires, with mudguards on the top and rear. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 3.20. | Plow storage legs | Must be supplied with the item. Storage legs with casters that allow the storage position of the plow to be changed and adjusted. | Yes/No (delete as appropriate). |

* The Supplier must confirm compliance with the technical requirement by indicating: Yes/No (delete as appropriate) and, where required, enter the exact parameter/value of the proposed Item.

Where required, the Supplier must enter the name of the document proving the parameters and/or a reference to the source confirming the proposed parameters.

____________________                               _____________                                       _____________________

(position)	                           (signature)	         (forename, surname)

If the tender is signed by a person empowered by the Supplier, a document (power of attorney) granting the specified person the right to sign on behalf of the Supplier must be submitted together with the tender.

PART 4 OF THE PROCUREMENT. SALT SPREADERS WITH AUTONOMOUS ENGINE AND HYDRAULIC STATION FOR TRIAXIAL VEHICLES (6x4) (8 sets)

The salt spreader will be operated with a truck:

| Vehicle location (KT, mastery) | Truck | Year | Registration plate No | Factory No | Inv. No | Spreader volume, m³ | Spreader operating method | Wheelbase, mm | Position of the centre of gravity of the superstructure | Spreader drive unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kauno KT, Kauno MT | MB Actros 3344 | 2005 | DDS506 | WDB9321631L033528 | 0301-00258 | 8-9 | To the body | 4 600 | at least 630 mm forward of the centre of the 2-3 axle tandem | Autonomous engine and hydraulic station |
| Kauno KT, Kauno MT | Renault Kerax 460.35 | 2012 | GHU295 | VF634DPA000012724 | 0403-00590 | 8-9 | To the body | 4 500 | at least 630 mm forward of the centre of the 2-3 axle tandem | Autonomous engine and hydraulic station |
| Marijampolės KT, Marijampolės MT | Iveco Trakker AD380T41 | 2013 | HGS256 | WJME3TSS40C277832 | 0403-00438 | 8-9 | To the body | 4 200 | at least 625 mm forward of the centre of the 2-3 axle tandem | Autonomous engine and hydraulic station |
| Rokiškio KT, Zarasų MT | Iveco Trakker AD260T36 | 2012 | GHU138 | WJME2NPS40C207763 | 0404-00425 | 8-9 | To the body | 5 600 | at least 630 mm forward of the centre of the 2-3 axle tandem | Autonomous engine and hydraulic station |
| Klaipėdos KT, Endriejavo MT | Renault Kerax 430.35 | 2013 | GZV346 | VF634DPA000014002 | 0403-00283 | 8-9 | To the body | 5 300 | at least 630 mm forward of the centre of the 2-3 axle tandem | Autonomous engine and hydraulic station |
| Telšių KT Telšių MT | MAN TGA 33.390 | 2010 | FEV426 | WMA26SZZXBP024517 | 0403-00238 | 8-9 | To the body | 4 800 | at least 630 mm forward of the centre of the 2-3 axle tandem | Autonomous engine and hydraulic station |
| Telšių KT Mažeikių MT | Iveco TrakerAD380T41K0 | 2011 | FNK174 | WJME3TSS40C240261 | 0403-00197 | 8-9 | To the body | 4 500 | at least 630 mm forward of the centre of the 2-3 axle tandem | Autonomous engine and hydraulic station |
| Telšių MT Akmenės MT | MAN TGA 33.390 | 2011 | FNE455 | WMA26SZZ6BP030492 | 0404-00436 | 8-9 | To the body | 4 800 | at least 630 mm forward of the centre of the 2-3 axle tandem | Autonomous engine and hydraulic station |

| Ser. No | Name of characteristics | Characteristics required by the Purchaser | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the Item offered. In the clauses where required, the Supplier must enter the name of the document submitted, proving the values ​​of the parameters, and/or a reference to the source confirming the parameters offered. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1. | Purpose | Equipment designed for spreading anti-skid materials on road surfaces. | Yes/No (delete as appropriate). |
| 1.2. | Year of production | The salt spreaders must be new and unused prior to the date of transfer. | Yes/No (delete as appropriate). Year and quarter of production – (enter) ___________. |
| 1.3. | The dominant colour of the salt spreader and of the painted metal surfaces | Orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific technical group. Galvanized metal surfaces and stainless-steel elements may be left unpainted. | Yes/No (delete as appropriate). Proposed dominant colour – (enter parameter) _____________. |
| 1.4. | Warranty against corrosion perforation | All components included in the set, without exception, have a minimum warranty period of 72 months. | Yes/No (delete as appropriate). Proposed parameter - ____ months. |
| 1.5. | Right to perform warranty repairs and warranty technical maintenance | The Supplier must be the official representative of the manufacturer of the offered Goods (if the Supplier is not the manufacturer), having the right to perform technical maintenance and repair of the offered Goods during the warranty period, or must have concluded a corresponding service agreement with another entity authorized to perform such services. The tender must include the manufacturer’s authorization or another document confirming the Supplier’s right to provide technical maintenance and repair services during the warranty period, or evidence of a concluded service agreement with another entity authorized to provide such services, or the Supplier must provide a manufacturer-issued authorization granting the Purchaser the right to perform technical maintenance and repair services during the warranty period. | Yes/No (delete as appropriate). |
| 1.6. | Terms of technical service and warranty repairs | The Supplier must perform warranty repair works within no more than 2 working days after the Purchaser’s request is submitted in writing (by e-mail) and by a phone call to the Supplier’s responsible person indicated in the Contract, or may authorize the Purchaser during the warranty period (providing a manufacturer-issued authorization) to carry out warranty repair works by qualified Purchaser’s personnel, with the costs of the works and other related expenses reimbursed in accordance with the conditions specified in the Contract. | Yes/No (delete as appropriate). |
| 1.7. | Documentation | Operating and safety instructions in Lithuanian (user manuals for salt spreader) shall be provided no later than on the date of delivery of the Item. | Yes/No (delete as appropriate). |
| 1.8. | Training | At the time of delivery of the Item, but no later than when the Purchaser begins to use the Item or part of the set of the Item, the Supplier must provide the following training, the cost of which must be included in the offer price: - theoretical and practical training for operators to ensure the correct use of the Item. - technical training to ensure the correct maintenance of the Item, explaining the nature of faults and fault symbols. - technical training to ensure the correct and safe disassembly/assembly/replacement of the components of the set. | Yes/No (delete as appropriate). |
| 1.8.1 | Approving certificate | The Supplier (including partners – Suppliers of individual components of the set) shall prepare a certificate describing the scope of the training course for the Operator(s) who has/have completed theoretical and technical training and practical training. | Yes/No (delete as appropriate). |
| 1.9. | Manufacturer and installer of bundled equipment | Equipment shall be manufactured and installed on a truck by the manufacturer of the equipment or its official representative (in which case documents or copies thereof confirming the representation must be submitted with the tender). Alternatively, the Purchaser may carry out the installation, provided that the Supplier covers all costs incurred by the Purchaser for such installation. Responsibility for the flawless operation of the entire equipment complex and for remedying any defects discovered during operation shall rest with the manufacturer of the equipment or its official representative who carried out the installation, or with the Purchaser in the case where the Purchaser performed the installation. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2. SALT SPREADER |  |  |  |
| 2.1. | Purpose, adaptation Type, make | Salt spreader designed to be installed (mounted) and operated in a body of a 3-axle truck. Designed for spreading, with or without moistening, salt, sand, gravel and fine crushed stone. Must comply with the requirements of standard LST EN 15597-1:2020. The spreader must be a standard product manufactured by the manufacturer in series, not specially modified or adapted to the requirements formulated by the contracting authority, except for specific marking and safety requirements. | Yes/No (delete as appropriate). Exact spreader model - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.1.1 | Salt spreader drive unit | Autonomous internal combustion engine provided and installed by the manufacturer, and an autonomous hydraulic station. All operations related to the start-up and functioning of the autonomous engine and salt spreader are fully controlled from the control panel installed in the operator's workplace (cab). | Yes/No (delete as appropriate). Proposed parameter - ____ (unit model) Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.1.2 | Parameters of an autonomous internal combustion engine | Power: at least 14.0 kW / 19.0 HP Fuel: diesel. Cooling type: liquid-cooled. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.2. | Quick connection and activation system for the hydraulic and electrical systems of the salt spreader. | The necessary electrical and hydraulic connections must be provided and installed by the manufacturer and marked accordingly. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.3. | Installation of the salt spreader on a vehicle. | The spreader must have a quick loading (mounting)/unloading system for loading/unloading onto/from a self-propelled truck body (RO-RO or equivalent): loading and unloading of the spreader is carried out without external assistance and without the use of additional self-propelled lifting equipment. The metal structure of the quick loading (installation)/unloading and storage system must ensure safe, long-term and stable storage of the spreader (including partially loaded spreading material and/or partially filled with salt solution). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.4. | Upper overall dimension of the spreader. | The top height of the spreader mounted on the body of a truck from the bottom of the body shall not exceed 2300 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. |
| 2.5. | Weight of the spreader with all integral equipment. | 2 300 – 3 000 kg | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.6. | Spreader testing, adjustment, calibration | No later than on the day of delivery of the Item to the Purchaser, the salt spreader installed in the body of the truck must be tested, adjusted, calibrated, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual amount of material spread in various operating modes. calibration of the salt spreader installed in the truck body must be performed, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual quantities of material spread in various operating modes. All necessary work shall be performed at the Supplier's expense, except for the amount of salt, sand-salt mixture, salt solution required for testing and calibration, and the loading of materials into the spreader. | Yes/No (delete as appropriate). |
| Materials hopper |  |  |  |
| 2.7. | Materials hopper and structural frame | The materials hopper is made of steel sheets no thinner than 3 mm, and profiled square and rectangular pipes with a wall thickness of at least 3 mm are used for the frame structure. The entire structure is coated with an anti-corrosion coating and painted or made of stainless-steel. The surface painting process must be carried out in accordance with the requirements of ISO 8501-1. The total thickness of the coating (anti-corrosion coating and paint layer) is not less than 180 µm. The colour shall be orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s specified default (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). Proposed material, its thickness, and colour - _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.8. | Hopper capacity | Possible load capacity of loose materials – 8 - 9 m3 An additional point is awarded for a larger capacity of the hopper. | Yes/No (delete as appropriate). Proposed parameter - ____ m3. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.9. | Feeder of spreading materials. | A rubber belt conveyor, ensuring smooth feeding of materials to the spreading disc, regardless of the type and amount of the spreading material in the hopper. An additional point is awarded for an additional installed mechanism that adjusts the height of the material dosing valve according to the material selected on the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.10. | Mechanism for even distribution of the spreading material. | A separate mechanism installed along the entire length of the hopper, with a separate hydraulic drive or other type of drive and drive control, which distributes the spreading material more evenly over the rubber belt conveyor. | Yes/No (delete as appropriate). Type and description of the proposed device - ____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.11. | Hopper cover | Covered with a metal grate (opening dimensions: no less than 40x40 mm, but no more than 70x70 mm, wire-rod diameter no less than 4 mm) and a moisture-proof, easily closable and openable side tarpaulin without additional means. The levers of the cover opening/closing mechanism must be accessible to the operator (the covering/uncovering process must be possible) while standing on the ground or on a ladder installed at the rear of the spreader. The grate structure is hot-dip galvanized. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Salt solution tank |  |  |  |
| 2.12. | Salt solution tanks | At least two parts (installed on the sides of the spreading materials hopper, in the lower part). Total volume of at least 3,5 m³. Depending on the volume of the spreader's spreader material hopper, the ratio of the salt solution tank volume must be maintained at 7:3. | Yes/No (delete as appropriate). Proposed parameter - _____ m³. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12.1 | Materials used in manufacturing | Tanks must be made of polypropylene or another equivalent polymer material that is resistant to corrosion, temperature fluctuations, and ultraviolet radiation. The resistance of the polymer materials used in salt solution tanks to ultraviolet radiation and other environmental conditions must comply with the DIN EN-17106 standard. | Yes/No (delete as appropriate). Proposed parameter – meets the requirement _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Control |  |  |  |
| 2.13. | Control of salt spreading and moistening equipment | Performed from the vehicle cab, using a digital control panel that is easily accessible to the driver from his workplace. | Yes/No (delete as appropriate). |
| 2.14. | Manual control | The possibility provided by the manufacturer to control the spreading process manually (in case of emergency). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.15. | Control panel | During the shift, the operator must be able to continuously view and control at least the following parameters on the control panel display: - Selected spreading material and spreading rate; - Spreading width and asymmetry; - Moistening of the spreading material with solution is on/off, selected moistening percentage (%); - Amount of spreading material; - Amount of solution The control panel must: - collect data and ensure digital scanning and real-time (“Online “) transmission of data to the dispatcher ‘s workstation; - inform the operator about the shortage of the spreading materials or salt solution with an additional audible signal; - have connections for data transmission in accordance with data encryption standard EN15430 or an equivalent standard. - have a menu in Lithuanian. The control panel menu must indicate the following names of spreading materials (may be without Lithuanian symbols, e.g., Misinys (Mixture) 1:1): • Salt • Mixture 1:1 • Mixture 1:3 • Mixture 1:10 • Solution The Supplier must provide the Purchaser with the access codes required to adjust the control panel settings. The meanings of the control panel buttons must be provided in the user manual. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.16. | System for transferring data about operation | The Supplier must provide access to the control panel data transmission, at least for the following data (for the purpose of controlling the spreader operation): 1. Date, time, operating time. 2. Spreader operating parameters: spreading is on/off or spreading mode, spreading time, spreading width, name of the material being spread, dosage of the material being spread g/m², amount spread kg, solution dosage ml/m² or g/m² (depending on the manufacturer’s specifications), amount of solution dispensed in litres. 3. Position of snow plow: operating/transport position. Transport telemetry control services are provided to the Purchaser by UAB Xirgo Global. The tender must be accompanied by information that clearly and comprehensively shows the required data. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.17. | Generation of data transmission system reports | Reports must be generated with freely selectable dates and times. They must include: The date of spreading, the start/end time of spreading. The start/end location of spreading, the total spreading time, the spreading distance, the consumption of the spreading material and salt solution (kg, litres). The dosage of the spreading material in g/m², the dosage of the solution in ml/m² or g/m² (depending on the manufacturer’s specifications). Distance travelled by the vehicle and spreader, m. Names of materials spread. The tender must include detailed information about the reports to be generated and the required data to be included in them. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18. | Even spreading and salt solution feeding. | The vehicle speed signal must be transmitted to the spreader control panel (linked). The even supply of salt and salt solution to the spreading disc, the even rotation of the disc and the spreading of materials, according to the settings on the control panel, must take place regardless of the vehicle's speed, as well as when the vehicle is stationary. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.19. | Emergency unloading function. | Must be provided by the manufacturer and activated via the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.20. | Spreading parameter programming and fault notification system. | Must be provided and installed by the manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.21. | Instantaneous increase of the amount being spread. | A function that allows the amount of material being spread to be increased two or more times for a limited period of time. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Spreading (dispersal) mechanism |  |  |  |
| 2.22. | System for mixing the material being spread and solution. | Ensuring mixing of the spreading material and salt solution and delivery of the mixture onto the spreading disc, or mixing of salt and solution directly on the spreading disc. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.23. | Salt moistening and spreading mechanism | Variable length sleeve/trough and disc with hydraulic drive, liftable upwards and lockable in transport position. The mechanism is lifted and locked in the transport position. Air shock absorbers provided for and installed by the manufacturer, or equivalent solution to facilitate lifting of the mechanism into transport position. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.24. | Spreading disc | Diameter not less than 600 mm. The height of the disc above the road surface is adjusted mechanically (manually) at interval not less than 100 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.25. | Spreading disc hydraulic motor | Protected from direct exposure to salt. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26. | Salt spreading sensor | Must be provided and installed by the manufacturer - contactless, electronic. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26.1 | Visual control | A rear-view camera with night vision, installed by the spreader manufacturer, is mounted at the rear of the spreader, as high as technically possible, for visual control of the spreading process and control of the reverse movement. The image transmitted by the camera must be visible at the operator’s workplace (in the cab of truck) on the monitor (in a separate section) located on the spreader control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27. | Adjustable amount of material being spread per m² | Salt - within the range of 5-40 g/m², with a maximum “pace“ of 5 g/m². Sifted sand and crushed stone - within the range of 20-300 g/m², with a maximum “pace“ of 10 g/m². | Yes/No (delete as appropriate). Proposed parameter: Salt within the range of ____ g/m2. Spread gravel, sand, crushed stone within the range of ____ g/m2. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.28. | Adjustable spreading width of the material being spread. | From 3 m to at least 12 m, at intervals of no more than 1,0 m, with asymmetrical (increasing or decreasing the width of one side of the spread without changing the width of the other side) adjustment of the spreading material, set using the control panel. | Yes/No (delete as appropriate). Proposed parameter: Within ____ m, spreading width “pace” of __m. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29. | Forced (pressure) salt solution feeding (to the spreading disc) and control of amount. | At interval of 5 %, starting from a minimum of 15%, which automatically changes depending on the amount of salt being spread. It is assumed that the percentage of salt solution for salt moistening indicates the proportion of the solution in the spread rate (e.g., 15% means that the spread rate consists of 15% solution + 85% dry matter). | Yes/No (delete as appropriate) Proposed parameter: At __ % interval from ___ % Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Safety fence |  |  |  |
| 2.30. | Safety fence – lightweight construction, 1-1.15 m high, on the both sides of the spreader. | Mounted on top of the salt spreader hopper and raised/folded for maintenance/repair before accessing the hopper. The raising/lowering of the Safety fence is performed by lowering/raising the ladder intended for access to the hopper or by a separate raising/lowering mechanism. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Signal marking |  |  |  |
| 2.31. | Dimensional marking, signal lighting | The working (spreading/dispersal) area must be illuminated by at least one light. At least one orange LED beacon installed at the rear of the spreader. Two flashing LED lights (round lights) with a diameter of at least 180 mm, installed at the upper corners of the rear part of the spreader. The rear part marked with warning light-reflecting diagonal red and white stripes. Sample pattern of warning marking: Protection of LED beacons and flashing LED lights against solid objects and liquids must be at least IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.32. | Mounting locations for road signs and road signs. | The rear part of the spreader (on the mounting/suspension unit/frame) shall be equipped with places for mounting warning/directional road signs or the necessary mounting structure, and no later than on the date of delivery of the Item, the following shall be installed: - a light-reflecting sign No 106 „Work“ not less than 1 (first) size: - Signal LED light pointer – a system of at least 13 lights complying with L8H, EN 12352 or an equivalent standard, fastened from the rear with screws or equivalent fasteners. The direction of the arrow indicating the mandatory side of the obstacle is changed from the operator's workplace using an electrical connection. A visual indicator informing about the activated direction of the arrow is mandatory at the operator's workplace. The feed voltage of the arrow lights is 24 V. The protection of the LED light system components against solid objects and liquids is not less than IP65. | Yes/No (delete as appropriate). |

* The Supplier must confirm compliance with the technical requirement by indicating: Yes/No (delete as appropriate) and, where required, enter the exact parameter/value of the proposed Item.

Where required, the Supplier must enter the name of the document proving the parameters and/or a reference to the source confirming the proposed parameters.

____________________                               _____________                                       _____________________

(position)	                     (signature)	(forename, surname)

If the tender is signed by a person empowered by the Supplier, a document (power of attorney) granting the specified person the right to sign on behalf of the Supplier must be submitted together with the tender.

PART 5 OF THE PROCUREMENT. SALT SPREADER WITH AUTONOMOUS ENGINE AND HYDRAULIC STATION FOR TWO-AXLE VEHICLES (1 set)

The salt spreader will be operated with a truck:

| Vehicle location (KT, mastery) | Truck | Year | Registration plate No | Factory No | Inv. No | Spreader volume, m³ | Spreader operating method | Wheelbase, mm | Position of the centre of gravity of the superstructure | Spreader drive unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pasvalio KT, Pasvalio MT | Iveco Trakker AD190T31 | 2007 | CGM447 | WJMB1VNS20C174540 | 0403-00160 | 5-6 | To the body | 3 800 | at least 520 mm forward of the centre of the rear axle | Autonomous engine and hydraulic station |

| Ser. No | Name of characteristics | Characteristics required by the Purchaser | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the Item offered. In the clauses where required, the Supplier must enter the name of the document submitted, proving the values ​​of the parameters, and/or a reference to the source confirming the parameters offered. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1. | Purpose | Equipment designed for spreading anti-skid materials on road surfaces. | Yes/No (delete as appropriate). |
| 1.2. | Year of production | The salt spreader must be new and unused prior to the date of transfer. | Yes/No (delete as appropriate). Year and quarter of production – (enter) ___________. |
| 1.3. | The dominant colour of the salt spreader and of the painted metal surfaces | Orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific technical group. Galvanized metal surfaces and stainless-steel elements may be left unpainted. | Yes/No (delete as appropriate). Proposed dominant colour – (enter parameter) _____________. |
| 1.4. | Warranty against corrosion perforation | All components included in the set, without exception, have a minimum warranty period of 72 months. | Yes/No (delete as appropriate). Proposed parameter- ____ months. |
| 1.5. | Right to perform warranty repairs and warranty technical maintenance | The Supplier must be the official representative of the manufacturer of the offered Goods (if the Supplier is not the manufacturer), having the right to perform technical maintenance and repair of the offered Goods during the warranty period, or must have concluded a corresponding service agreement with another entity authorized to perform such services. The tender must include the manufacturer’s authorization or another document confirming the Supplier’s right to provide technical maintenance and repair services during the warranty period, or evidence of a concluded service agreement with another entity authorized to provide such services, or the Supplier must provide a manufacturer-issued authorization granting the Purchaser the right to perform technical maintenance and repair services during the warranty period. | Yes/No (delete as appropriate). |
| 1.6. | Terms of technical service and warranty repairs | The Supplier must perform warranty repair works within no more than 2 working days after the Purchaser’s request is submitted in writing (by e-mail) and by a phone call to the Supplier’s responsible person indicated in the Contract, or may authorize the Purchaser during the warranty period (providing a manufacturer-issued authorization) to carry out warranty repair works by qualified Purchaser’s personnel, with the costs of the works and other related expenses reimbursed in accordance with the conditions specified in the Contract. | Yes/No (delete as appropriate). |
| 1.7. | Documentation | Operating and safety instructions in Lithuanian (user manuals for salt spreader) shall be provided no later than on the date of delivery of the Item. | Yes/No (delete as appropriate). |
| 1.8. | Training | At the time of delivery of the Item, but no later than when the Purchaser begins to use the Item or part of the set of the Item, the Supplier must provide the following training, the cost of which must be included in the offer price: -theoretical and practical training for operators to ensure the correct use of the Item. - technical training to ensure the correct maintenance of the Item, explaining the nature of faults and fault symbols. - technical training to ensure the correct and safe disassembly/assembly/replacement of the components of the set. | Yes/No (delete as appropriate). |
| 1.8.1 | Approving certificate | The Supplier (including partners – Suppliers of individual components of the set) shall prepare a certificate describing the scope of the training course for the Operator(s) who has/have completed theoretical and technical training and practical training. | Yes/No (delete as appropriate). |
| 1.9. | Manufacturer and installer of bundled equipment | Equipment shall be manufactured and installed on a truck by the manufacturer of the equipment or its official representative (in which case documents or copies thereof confirming the representation must be submitted with the tender). Alternatively, the Purchaser may carry out the installation, provided that the Supplier covers all costs incurred by the Purchaser for such installation. Responsibility for the flawless operation of the entire equipment complex and for remedying any defects discovered during operation shall rest with the manufacturer of the equipment or its official representative who carried out the installation, or with the Purchaser in the case where the Purchaser performed the installation. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2. SALT SPREADER |  |  |  |
| 2.1. | Purpose, adaptation Type, make | Salt spreader designed for installation (mounting) and operation in a body of a 2-axle truck. Designed for spreading, with or without moistening, salt, sand, fine crushed stone. Must comply with the requirements of standard LST EN 15597-1:2020. The spreader must be a standard product manufactured by the manufacturer in series, not specially modified or adapted to the requirements formulated by the contracting authority, except for specific marking and safety requirements. | Yes/No (delete as appropriate). Exact spreader model - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.2. | Salt spreader drive unit | Autonomous internal combustion engine provided and installed by the manufacturer, and an autonomous hydraulic station. All operations related to the start-up and functioning of the autonomous engine and salt spreader are fully controlled from the control panel installed in the operator's workplace (cab). | Yes/No (delete as appropriate). Proposed parameter - ____ (unit model) Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.2.1 | Parameters of an autonomous internal combustion engine | Power: no less than 14.0 kW / 19.0 hp Fuel: diesel. Cooling type: liquid-cooled. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.3. | Installation of the salt spreader on a vehicle. | The spreader must have a quick loading (mounting)/unloading system for loading/unloading onto/from a self-propelled truck body (RO-RO or equivalent): loading and unloading of the spreader is carried out without external assistance and without the use of additional self-propelled lifting equipment. The metal structure of the quick loading (installation)/unloading and storage system must ensure safe, long-term and stable storage of the spreader (including partially loaded spreading material and/or partially filled with salt solution). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.4. | Upper overall dimension of the spreader. | The top height of the spreader mounted into the body of a truck from the bottom of the body shall not exceed 2200 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. |
| 2.5. | Weight of the spreader with all integral equipment. | 2 200 – 3 000 kg | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.6. | Spreader testing, adjustment, calibration | No later than on the day of delivery of the Item to the Purchaser, the salt spreader installed in the body of the truck must be tested, adjusted, calibrated, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual amount of material spread in various operating modes. calibration of the salt spreader installed in the truck body must be performed, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual quantities of material spread in various operating modes. All necessary work shall be performed at the Supplier’s expense, except for the amount of salt, sand-salt mixture, salt solution required for testing and calibration, and the loading of materials into the spreader. | Yes/No (delete as appropriate). |
| Materials hopper |  |  |  |
| 2.7. | Materials hopper and structural frame | The materials hopper is made of steel sheets no thinner than 3 mm, and profiled square and rectangular pipes with a wall thickness of at least 3 mm are used for the frame structure. The entire structure is coated with an anti-corrosion coating and painted or made of stainless-steel. The surface painting process must be carried out in accordance with the requirements of ISO 8501-1. The total thickness of the coating (anti-corrosion coating and paint layer) is not less than 180 µm. The colour shall be orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s specified default (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). Proposed material, its thickness, and colour - _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.8. | Hopper capacity | Possible load capacity of loose materials – 5 - 6 m3 An additional point is awarded for a larger capacity of the hopper. | Yes/No (delete as appropriate). Proposed parameter - ____ m3. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.9. | Feeder of spreading materials. | A rubber belt conveyor, ensuring smooth feeding of materials to the spreading disc, regardless of the type and amount of the spreading material in the hopper. An additional point is awarded for an additional installed mechanism that adjusts the height of the material dosing valve according to the material selected on the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.10. | Mechanism for even distribution of the spreading material. | A separate mechanism installed along the entire length of the hopper, with a separate hydraulic drive or other type of drive and drive control, which distributes the spreading material more evenly over the rubber belt conveyor. | Yes/No (delete as appropriate). Type and description of the proposed device - ____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.11. | Hopper cover | Covered with a metal grate (opening dimensions: no less than 40x40 mm, but no more than 70x70 mm, wire-rod diameter no less than 4 mm) and a moisture-proof, easily closable and openable side tarpaulin without additional means. The levers of the cover opening/closing mechanism must be accessible to the operator (the covering/uncovering process must be possible) while standing on the ground or on a ladder installed at the rear of the spreader. The grate structure is hot-dip galvanized. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Salt solution tank |  |  |  |
| 2.12. | Salt solution tanks | At least two parts (installed on the sides of the spreading materials hopper, in the lower part). Total volume of at least 2,5 m³. Depending on the volume of the spreader's spreader material hopper, the ratio of the salt solution tank volume must be maintained at 7:3. | Yes/No (delete as appropriate). Proposed parameter - _____ m³. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12.1 | Materials used in manufacturing | Tanks must be made of polypropylene or another equivalent polymer material that is resistant to corrosion, temperature fluctuations, and ultraviolet radiation. The resistance of the polymer materials used in salt solution tanks to ultraviolet radiation and other environmental conditions must comply with the DIN EN-17106 standard. | Yes/No (delete as appropriate). Proposed parameter –meets the requirement _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Control |  |  |  |
| 2.13. | Control of salt spreading and moistening equipment | Performed from the vehicle cab, using a digital control panel that is easily accessible to the driver from his workplace. | Yes/No (delete as appropriate). |
| 2.14. | Manual control | The possibility provided by the manufacturer to control the spreading process manually (in case of emergency). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.15. | Control panel | During the shift, the operator must be able to continuously view and control at least the following parameters on the control panel display: - Selected spreading material and spreading rate; - Spreading width and asymmetry; - Moistening of the spreading material with solution is on/off, selected moistening percentage (%); - Amount of spreading material; - Amount of solution The control panel must: - collect data and ensure digital scanning and real-time (“Online “) transmission of data to the dispatcher ‘s workstation; - inform the operator about the shortage of the spreading materials or salt solution with an additional audible signal; - have connections for data transmission in accordance with data encryption standard EN15430 or an equivalent standard. - have a menu in Lithuanian. The control panel menu must indicate the following names of spreading materials (may be without Lithuanian symbols, e.g., Misinys (Mixture) 1:1): • Salt • Mixture 1:1 • Mixture 1:3 • Mixture 1:10 • Solution The Supplier must provide the Purchaser with the access codes required to adjust the control panel settings. The meanings of the control panel buttons must be provided in the user manual. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.16. | System for transferring data about operation | The Supplier must provide access to the control panel data transmission, at least for the following data (for the purpose of controlling the spreader operation): 1. Date, time, operating time. 2. Spreader operating parameters: spreading is on/off or spreading mode, spreading time, spreading width, name of the material being spread, dosage of the material being spread g/m², amount spread kg, solution dosage ml/m² or g/m² (depending on the manufacturer’s specifications), amount of solution dispensed in litres. 3. Position of snow plow: operating/transport position. Transport telemetry control services are provided to the Purchaser by UAB Xirgo Global. The tender must be accompanied by information that clearly and comprehensively shows the required data. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.17. | Generation of data transmission system reports | Reports must be generated with freely selectable dates and times. They must include: The date of spreading, the start/end time of spreading. The start/end location of spreading, the total spreading time, the spreading distance, the consumption of the spreading material and salt solution (kg, litres). The dosage of the spreading material in g/m², the dosage of the solution in ml/m² or g/m² (depending on the manufacturer’s specifications). Distance travelled by the vehicle and spreader, m. Names of materials spread. The tender must include detailed information about the reports to be generated and the required data to be included in them. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18. | Even spreading and salt solution feeding. | The vehicle speed signal must be transmitted to the spreader control panel (linked). The even supply of salt and salt solution to the spreading disc, the even rotation of the disc and the spreading of materials, according to the settings on the control panel, must take place regardless of the vehicle's speed, as well as when the vehicle is stationary. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.19. | Emergency unloading function. | Must be provided by the manufacturer and activated via the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.20. | Spreading parameter programming and fault notification system. | Must be provided and installed by the manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.21. | Instantaneous increase of the amount being spread. | A function that allows the amount of material being spread to be increased two or more times for a limited period of time. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Spreading (dispersal) mechanism |  |  |  |
| 2.22. | Spreading material and solution mixing system. | Ensuring mixing of the spreading material and salt solution and delivery of the mixture onto the spreading disc, or mixing of salt and solution directly on the spreading disc. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.23. | Salt moistening and spreading mechanism | Variable length sleeve/trough and disc with hydraulic drive, liftable upwards and lockable in transport position. The mechanism is lifted and locked in the transport position. Air shock absorbers provided for and installed by the manufacturer, or equivalent solution to facilitate lifting of the mechanism into transport position. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.24. | Spreading disc | Diameter not less than 600 mm. The height of the disc above the road surface is adjusted mechanically (manually) at interval not less than 100 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.25. | Spreading disc hydraulic motor | Protected from direct exposure to salt. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26. | Salt spreading sensor | Must be provided and installed by the manufacturer - contactless, electronic. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26.1 | Visual control | A rear-view camera with night vision, installed by the spreader manufacturer, is mounted at the rear of the spreader, as high as technically possible, for visual control of the spreading process and control of the vehicle’s reverse movement. The image transmitted by the camera must be visible at the operator’s workplace (in the cab of truck) on the monitor (in a separate section) located on the spreader control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27. | Adjustable amount of material being spread per m² | Salt - within the range of 5-40 g/m², with a maximum “pace“ of 5 g/m². Sifted sand and crushed stone - within the range of 20-300 g/m², with a maximum “pace“ of 10 g/m². | Yes/No (delete as appropriate). Proposed parameter: Salt within the range of ____ g/m2. Spread gravel, sand, crushed stone within the range of ____ g/m2. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.28. | Adjustable spreading width of the material being spread. | From 3 m to at least 12 m, at intervals of no more than 1,0 m, with asymmetrical (increasing or decreasing the width of one side of the spread without changing the width of the other side) adjustment of the spreading material, set using the control panel. | Yes/No (delete as appropriate). Proposed parameter: Within ____ m, spreading width “pace” of __m. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29. | Forced (pressure) salt solution feeding (to the spreading disc) and control of amount. | At interval of 5 %, starting from a minimum of 15%, which automatically changes depending on the amount of salt being spread. It is assumed that the percentage of salt solution for salt moistening indicates the proportion of the solution in the spread rate (e.g., 15% means that the spread rate consists of 15% solution + 85% dry matter). | Yes/No (delete as appropriate). Proposed parameter: At __ % interval from ___ % Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Safety fence |  |  |  |
| 2.30. | Safety fence – lightweight construction, 1-1.15 m high, on the both sides of the spreader. | Mounted on top of the salt spreader hopper and raised/folded for maintenance/repair before accessing the hopper. The raising/lowering of the Safety fence is performed by lowering/raising the ladder intended for access to the hopper or by a separate raising/lowering mechanism. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Signal marking |  |  |  |
| 2.31. | Dimensional marking, signal lighting | The working (spreading/dispersal) area must be illuminated by at least one light. At least one orange LED beacon installed at the rear of the spreader. Two flashing LED lights (round lights) with a diameter of at least 180 mm, installed at the upper corners of the rear part of the spreader. The rear part marked with warning light-reflecting diagonal red and white stripes. Sample pattern of warning marking: Protection of LED beacons and flashing LED lights against solid objects and liquids must be at least IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.32. | Mounting locations for road signs and road signs. | The rear part of the spreader (on the mounting/suspension unit/frame) shall be equipped with places for mounting warning/directional road signs or the necessary mounting structure, and no later than on the date of delivery of the Item, the following shall be installed: - a light-reflecting sign No 106 “Work“ not less than 1 (first) size: - Signal LED light pointer – a system of at least 13 lights complying with L8H, EN 12352 or an equivalent standard, fastened from the rear with screws or equivalent fasteners. The direction of the arrow indicating the mandatory side of the obstacle is changed from the operator’s workplace using an electrical connection. A visual indicator informing about the activated direction of the arrow is mandatory at the operator’s workplace. The feed voltage of the arrow lights is 24 V. The protection of the LED light system components against solid objects and liquids is not less than IP65. | Yes/No (delete as appropriate). |

* The Supplier must confirm compliance with the technical requirement by indicating: Yes/No (delete as appropriate) and, where required, enter the exact parameter/value of the proposed Item.

Where required, the Supplier must enter the name of the document proving the parameters and/or a reference to the source confirming the proposed parameters.

____________________                               _____________                                       _____________________

(position)	                         (signature)	(forename, surname)

If the tender is signed by a person empowered by the Supplier, a document (power of attorney) granting the specified person the right to sign on behalf of the Supplier must be submitted together with the tender.

PART 6 OF THE PROCUREMENT. SALT SPREADER WITH AUTONOMOUS ENGINE AND HYDRAULIC STATION FOR TWO-AXLE VEHICLE, INSTEAD OF BODY (1 set)

The salt spreader will be operated with a truck:

| Vehicle location (KT, mastery) |  |  | Truck | Year | Registration plate No | Factory No | Inv. No | Spreader volume, m³ | Spreader operating method | Wheelbase, mm | Spreader drive unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Klaipėdos KT, Šilutės MT |  |  | Iveco Trakker AD190T31W | 2006 | CGM451 | WJMB1VNS20C174541 | 0403-00329 | 6-7 | Instead of the body | 3 800 | Autonomous engine and hydraulic station |

| Ser. No | Name of characteristics | Characteristics required by the Purchaser | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the Item offered. In the clauses where required, the Supplier must enter the name of the document submitted, proving the values ​​of the parameters, and/or a reference to the source confirming the parameters offered. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1. | Purpose | Equipment designed for spreading anti-skid materials on road surfaces. | Yes/No (delete as appropriate). |
| 1.2. | Year of production | The salt spreader must be new and unused prior to the date of transfer. | Yes/No (delete as appropriate). Year and quarter of production – (enter) ___________. |
| 1.3. | The dominant colour of the salt spreader and of the painted metal surfaces | Orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific technical group. Galvanized metal surfaces and stainless-steel elements may be left unpainted. | Yes/No (delete as appropriate). Proposed dominant colour – (enter parameter) _____________. |
| 1.4. | Warranty against corrosion perforation | All components included in the set, without exception, have a minimum warranty period of 72 months. | Yes/No (delete as appropriate). Proposed parameter- ____ months. |
| 1.5. | Right to perform warranty repairs and warranty technical maintenance | The Supplier must be the official representative of the manufacturer of the offered Goods (if the Supplier is not the manufacturer), having the right to perform technical maintenance and repair of the offered Goods during the warranty period, or must have concluded a corresponding service agreement with another entity authorized to perform such services. The tender must include the manufacturer’s authorization or another document confirming the Supplier’s right to provide technical maintenance and repair services during the warranty period, or evidence of a concluded service agreement with another entity authorized to provide such services, or the Supplier must provide a manufacturer-issued authorization granting the Purchaser the right to perform technical maintenance and repair services during the warranty period. | Yes/No (delete as appropriate). |
| 1.6. | Terms of technical service and warranty repairs | The Supplier must perform warranty repair works within no more than 2 working days after the Purchaser’s request is submitted in writing (by e-mail) and by a phone call to the Supplier’s responsible person indicated in the Contract, or may authorize the Purchaser during the warranty period (providing a manufacturer-issued authorization) to carry out warranty repair works by qualified Purchaser’s personnel, with the costs of the works and other related expenses reimbursed in accordance with the conditions specified in the Contract. | Yes/No (delete as appropriate). |
| 1.7. | Documentation | Operating and safety instructions in Lithuanian (user manuals for salt spreader) shall be provided no later than on the date of delivery of the Item. | Yes/No (delete as appropriate). |
| 1.8. | Training | At the time of delivery of the Item, but no later than when the Purchaser begins to use the Item or part of the set of the Item, the Supplier must provide the following training, the cost of which must be included in the offer price: -theoretical and practical training for operators to ensure the correct use of the Item. - technical training to ensure the correct maintenance of the Item, explaining the nature of faults and fault symbols. - technical training to ensure the correct and safe disassembly/assembly/replacement of the components of the set. | Yes/No (delete as appropriate). |
| 1.8.1 | Approving certificate | The Supplier (including partners – Suppliers of individual components of the set) shall prepare a certificate describing the scope of the training course for the Operator(s) who has/have completed theoretical and technical training and practical training. | Yes/No (delete as appropriate). |
| 1.9. | Manufacturer and installer of bundled equipment | Equipment shall be manufactured and installed on a truck by the manufacturer of the equipment or its official representative (in which case documents or copies thereof confirming the representation must be submitted with the tender). Alternatively, the Purchaser may carry out the installation, provided that the Supplier covers all costs incurred by the Purchaser for such installation. Responsibility for the flawless operation of the entire equipment complex and for remedying any defects discovered during operation shall rest with the manufacturer of the equipment or its official representative who carried out the installation, or with the Purchaser in the case where the Purchaser performed the installation. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2. SALT SPREADER |  |  |  |
| 2.1. | Purpose, adaptation Type, make | Salt spreader designed for installation (mounting) and operation on a 2-axle truck (instead of the body). Designed for spreading, with or without moistening, salt, sand, fine crushed stone and/or spraying salt solution on the road surface. Must comply with the requirements of standard LST EN 15597-1:2020. The spreader must be a standard product manufactured by the manufacturer in series, not specially modified or adapted to the requirements formulated by the contracting authority, except for specific marking and safety requirements. | Yes/No (delete as appropriate). Exact spreader model - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.1.1 | Salt spreader drive unit | Autonomous internal combustion engine provided and installed by the manufacturer, and an autonomous hydraulic station. All operations related to the start-up and functioning of the autonomous engine and salt spreader are fully controlled from the control panel installed in the operator's workplace (cab). | Yes/No (delete as appropriate). Proposed parameter - ____ (unit model) Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.1.2 | Parameters of an autonomous internal combustion engine | Power: no less than 14.0 kW / 19.0 hp Fuel: diesel. Cooling type: liquid-cooled. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.2. | Quick connection and activation system for hydraulic and electrical systems of salt spreader. | The necessary electrical and hydraulic connections must be provided and installed by the manufacturer and marked accordingly. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.3. | Position of the spreader on the chassis (centre of gravity). | The centre of gravity of a spreader mounted on a truck (fixed in place of the body) must be in the superstructure mounting location recommended by the vehicle chassis manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.4. | Installation of a salt spreader on a vehicle. | The spreader must have a quick removal/installation system on the vehicle’s additional frame. The spreader must be removed and installed without external assistance, i.e. without the use of additional self-propelled lifting devices. Lifting devices – with hydraulic cylinders and mechanical locking pins. The metal structure of the quick removal/installation (storage) system and the lifting capacity of the lifting devices must ensure safe, long-term, and stable storage of the spreader (including when loaded with spreading material and/or filled with salt solution). | Yes/No (delete as appropriate). Permissible lifting capacity of lifting device (of each of the four) - ______kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.4.1 | Control of spreader removal/installation on truck system (storage legs) | The hydraulic devices of the salt spreader storage legs are controlled via the truck’s hydraulic system or from the spreader ‘s autonomous hydraulic station. The control panel of the snow plow or another additional control device located outside the operator’s cab or a remote-control panel should be used for control. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.5. | Upper overall dimension of the spreader. | The top height of the spreader mounted on the chassis of a truck from the chassis frame shall not exceed 2300 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. |
| 2.6. | Weight of the spreader with all integral equipment. | 2 300 – 3 000 kg | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.7. | Spreader testing, adjustment, calibration | No later than on the day of delivery of the Item to the Purchaser, the salt spreader installed in the body of the truck must be tested, adjusted, calibrated, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual amount of material spread in various operating modes. calibration of the salt spreader installed in the truck body must be performed, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual quantities of material spread in various operating modes. All necessary work shall be performed at the Supplier’s expense, except for the amount of salt, sand-salt mixture, salt solution required for testing and calibration, and the loading of materials into the spreader. | Yes/No (delete as appropriate). |
| Materials hopper |  |  |  |
| 2.8. | Materials hopper and structural frame | The materials hopper is made of steel sheets no thinner than 3 mm, and profiled square and rectangular pipes with a wall thickness of at least 3 mm are used for the frame structure. The entire structure is coated with an anti-corrosion coating and painted or made of stainless-steel. The surface painting process must be carried out in accordance with the requirements of ISO 8501-1. The total thickness of the coating (anti-corrosion coating and paint layer) is not less than 180 µm. The colour shall be orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s specified default (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). Proposed material, its thickness, and colour - _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.9. | Hopper capacity | Possible load capacity of loose materials – 6 - 7 m3 An additional point is awarded for a larger capacity of the hopper. | Yes/No (delete as appropriate). Proposed parameter - ____ m3. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.10. | Feeder of spreading materials. | A rubber belt conveyor, ensuring smooth feeding of materials to the spreading disc, regardless of the type and amount of the spreading material in the hopper. An additional point is awarded for an additional installed mechanism that adjusts the height of the material dosing valve according to the material selected on the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.11. | Mechanism for even distribution of the spreading material. | A separate mechanism installed along the entire length of the hopper, with a separate hydraulic drive or other type of drive and drive control, which distributes the spreading material more evenly over the rubber belt conveyor. | Yes/No (delete as appropriate). Type and description of the proposed device - ____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12. | Hopper cover | Covered with a metal grate (opening dimensions: no less than 40x40 mm, but no more than 70x70 mm, wire-rod diameter no less than 4 mm) and a moisture-proof, easily closable and openable side tarpaulin without additional means. The levers of the cover opening/closing mechanism must be accessible to the operator (the covering/uncovering process must be possible) while standing on the ground or on a ladder installed at the rear of the spreader. The grate structure is hot-dip galvanized. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Salt solution tank |  |  |  |
| 2.13. | Salt solution tanks | At least two parts (installed on the sides of the spreading materials hopper, in the lower part). Total volume of at least 2,6 m³. Depending on the volume of the spreader's spreader material hopper, the ratio of the salt solution tank volume must be maintained at 7:3. | Yes/No (delete as appropriate). Proposed parameter - _____ m³. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.13.1 | Materials used in manufacturing | Tanks must be made of polypropylene or another equivalent polymer material that is resistant to corrosion, temperature fluctuations, and ultraviolet radiation. The resistance of the polymer materials used in salt solution tanks to ultraviolet radiation and other environmental conditions must comply with the DIN EN-17106 standard. | Yes/No (delete as appropriate). Proposed parameter – meets the requirement _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Control |  |  |  |
| 2.14. | Control of salt spreading and moistening equipment | Performed from the vehicle cab, using a digital control panel that is easily accessible to the driver from his workplace. | Yes/No (delete as appropriate). |
| 2.15. | Manual control | The possibility provided by the manufacturer to control the spreading process manually (in case of emergency). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.16. | Control panel | During the shift, the operator must be able to continuously view and control at least the following parameters on the control panel display: - Selected spreading material and spreading rate; - Spreading width and asymmetry; - Moistening of the spreading material with solution is on/off, selected moistening percentage (%); - Amount of spreading material; - Amount of solution The control panel must: - collect data and ensure digital scanning and real-time (“Online “) transmission of data to the dispatcher ‘s workstation; - inform the operator about the shortage of the spreading materials or salt solution with an additional audible signal; - have connections for data transmission in accordance with data encryption standard EN15430 or an equivalent standard. - have a menu in Lithuanian. The control panel menu must indicate the following names of spreading materials (may be without Lithuanian symbols, e.g., Misinys (Mixture) 1:1): • Salt • Mixture 1:1 • Mixture 1:3 • Mixture 1:10 • Solution The Supplier must provide the Purchaser with the access codes required to adjust the control panel settings. The meanings of the control panel buttons must be provided in the user manual. An additional point is awarded for a spreader control panel with an automatic spreading function according to a selected pre-programmed route. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.17. | System for transferring data about operation | The Supplier must provide access to the control panel data transmission, at least for the following data (for the purpose of controlling the spreader operation): 1. Date, time, operating time. 2. Spreader operating parameters: spreading is on/off or spreading mode, spreading time, spreading width, name of the material being spread, dosage of the material being spread g/m², amount spread kg, solution dosage ml/m² or g/m² (depending on the manufacturer’s specifications), amount of solution dispensed in litres. 3. Position of snow plow: operating/transport position. Transport telemetry control services are provided to the Purchaser by UAB Xirgo Global. The tender must be accompanied by information that clearly and comprehensively shows the required data. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18. | Generation of data transmission system reports | Reports must be generated with freely selectable dates and times. They must include: The date of spreading, the start/end time of spreading. The start/end location of spreading, the total spreading time, the spreading distance, the consumption of the spreading material and salt solution (kg, litres). The dosage of the spreading material in g/m², the dosage of the solution in ml/m² or g/m² (depending on the manufacturer’s specifications). Distance travelled by the vehicle and spreader, m. Names of materials spread. The tender must include detailed information about the reports to be generated and the required data to be included in them. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.19. | Even spreading and salt solution feeding. | The vehicle speed signal must be transmitted to the spreader control panel (linked). The even supply of salt and salt solution to the spreading disc, the even rotation of the disc and the spreading of materials, according to the settings on the control panel, must take place regardless of the vehicle's speed, as well as when the vehicle is stationary. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.20. | Emergency unloading function. | Must be provided by the manufacturer and activated via the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.21. | Spreading parameter programming and fault notification system. | Must be provided and installed by the manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.22. | Instantaneous increase of the amount being spread. | A function that allows the amount of material being spread to be increased two or more times for a limited period of time. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Spreading (dispersal) mechanism |  |  |  |
| 2.23. | Spreading material and solution mixing system. | Ensuring mixing of the spreading material and salt solution and delivery of the mixture onto the spreading disc, or mixing of salt and solution directly on the spreading disc. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.24. | Salt moistening and spreading mechanism | Variable length sleeve/trough and disc with hydraulic drive, liftable upwards and lockable in transport position. The mechanism is lifted and locked in the transport position. Air shock absorbers provided for and installed by the manufacturer, or equivalent solution to facilitate lifting of the mechanism into transport position. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.25. | Spreading disc | Diameter not less than 600 mm. The height of the disc above the road surface is adjusted mechanically (manually) at intervals not less than 100 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26. | Spreading disc hydraulic motor | Protected from direct exposure to salt. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27. | Salt spreading sensor | Must be provided and installed by the manufacturer - contactless, electronic. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.28. | Visual control | A rear-view camera with night vision, installed by the spreader manufacturer, is mounted at the rear of the spreader, as high as technically possible, for visual control of the spreading process and control of the vehicle’s reverse movement. The image transmitted by the camera must be visible at the operator’s workplace (in the cab of truck) on the monitor (in a separate section) located on the spreader control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29. | Adjustable amount of material being spread per m² | Salt - within the range of 5-40 g/m², with a maximum “pace“ of 5 g/m². Sifted sand and crushed stone - within the range of 20-300 g/m², with a maximum “pace“ of 10 g/m². | Yes/No (delete as appropriate). Proposed parameter: Salt within the range of ____ g/m2. Spread gravel, sand, crushed stone within the range of ____ g/m2. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.30. | Adjustable spreading width of the material being spread. | From 3 m to at least 12 m, at intervals of no more than 1,0 m, with asymmetrical (increasing or decreasing the width of one side of the spread without changing the width of the other side) adjustment of the spreading material, set using the control panel. | Yes/No (delete as appropriate). Proposed parameter: Within ____ m, spreading width “pace” of __m. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.31. | Forced (pressure) salt solution feeding (to the spreading disc) and control of amount. | At interval of 5 %, starting from a minimum of 15%, which automatically changes depending on the amount of salt being spread. It is assumed that the percentage of salt solution for salt moistening indicates the proportion of the solution in the spread rate (e.g., 15% means that the spread rate consists of 15% solution + 85% dry matter). | Yes/No (delete as appropriate). Proposed parameter: At __ % interval from ___ % Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Safety fence |  |  |  |
| 2.32. | Safety fence – lightweight construction, 1-1.15 m high, on the both sides of the spreader. | Mounted on top of the salt spreader hopper and raised/folded for maintenance/repair before accessing the hopper. The raising/lowering of the Safety fence is performed by lowering/raising the ladder intended for access to the hopper or by a separate raising/lowering mechanism. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Signal marking |  |  |  |
| 2.33. | Dimensional marking, signal lighting | The working (spreading/dispersal) area must be illuminated by at least one light. At least one orange LED beacon installed at the rear of the spreader. Two flashing LED lights (round lights) with a diameter of at least 180 mm, installed at the upper corners of the rear part of the spreader. The rear part marked with warning light-reflecting diagonal red and white stripes. Sample pattern of warning marking: Protection of LED beacons and flashing LED lights against solid objects and liquids must be at least IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.34. | Mounting locations for road signs and road signs. | The rear part of the spreader (on the mounting/suspension unit/frame) shall be equipped with places for mounting warning/directional road signs or the necessary mounting structure, and no later than on the date of delivery of the Item, the following shall be installed: - a light-reflecting sign No 106 “Work“ not less than 1 (first) size: - Signal LED light pointer – a system of at least 13 lights complying with L8H, EN 12352 or an equivalent standard, fastened from the rear with screws or equivalent fasteners. The direction of the arrow indicating the mandatory side of the obstacle is changed from the operator's workplace using an electrical connection. A visual indicator informing about the activated direction of the arrow is mandatory at the operator's workplace. The feed voltage of the arrow lights is 24 V. The protection of the LED light system components against solid objects and liquids is not less than IP65. | Yes/No (delete as appropriate). |

* The Supplier must confirm compliance with the technical requirement by indicating: Yes/No (delete as appropriate) and, where required, enter the exact parameter/value of the proposed Item.

Where required, the Supplier must enter the name of the document proving the parameters and/or a reference to the source confirming the proposed parameters.

____________________                               _____________                                       _____________________

(position)	                       (signature)	(forename, surname)

If the tender is signed by a person empowered by the Supplier, a document (power of attorney) granting the specified person the right to sign on behalf of the Supplier must be submitted together with the tender.

PART 7 OF THE PROCUREMENT. SALT SPREADER FOR A TRIAXIAL VEHICLE (6X4) (1 set)

The salt spreader will be operated with a truck:

| Vehicle location (KT, mastery) | Truck | Year | Registration plate No | Factory No | Inv. No | Spreader volume, m³ | Spreader operating method | Wheelbase, mm | Spreader power unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Marijampolės KT, Lazdijų MT | MB Actros 3344 | 2005 | MBS846 | WDB9321631L036375 | 0404-00231 | 8-9 | To the body | 4 600 | Vehicle hydraulic system |

| Ser. No | Name of characteristics | Characteristics required by the Purchaser | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the Item offered. In the clauses where required, the Supplier must enter the name of the document submitted, proving the values ​​of the parameters, and/or a reference to the source confirming the parameters offered. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1. | Purpose | Equipment designed for spreading anti-skid materials on road surfaces. | Yes/No (delete as appropriate). |
| 1.2. | Year of production | The salt spreader must be new and unused prior to the date of transfer. | Yes/No (delete as appropriate). Year and quarter of production – (enter) ___________. |
| 1.3. | The dominant colour of the salt spreader and of the painted metal surfaces | Orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific group of equipment. Galvanized metal surfaces, elements of stainless-steel may be unpainted. | Yes/No (delete as appropriate). Proposed dominant colour – (enter parameter) _____________. |
| 1.4. | Warranty against corrosion perforation | All components included in the set, without exception, have a minimum warranty period of 72 months. | Yes/No (delete as appropriate). Proposed parameter - ____ months. |
| 1.5. | Right to perform warranty repairs and warranty technical maintenance | The Supplier must be the official representative of the manufacturer of the offered Goods (if the Supplier is not the manufacturer), having the right to perform technical maintenance and repair of the offered Goods during the warranty period, or must have concluded a corresponding service agreement with another entity authorized to perform such services. The tender must include the manufacturer’s authorization or another document confirming the Supplier’s right to provide technical maintenance and repair services during the warranty period, or evidence of a concluded service agreement with another entity authorized to provide such services, or the Supplier must provide a manufacturer-issued authorization granting the Purchaser the right to perform technical maintenance and repair services during the warranty period. | Yes/No (delete as appropriate). |
| 1.6. | Terms of technical service and warranty repairs | The Supplier must perform warranty repair works within no more than 2 working days after the Purchaser’s request is submitted in writing (by e-mail) and by a phone call to the Supplier’s responsible person indicated in the Contract, or may authorize the Purchaser during the warranty period (providing a manufacturer-issued authorization) to carry out warranty repair works by qualified Purchaser’s personnel, with the costs of the works and other related expenses reimbursed in accordance with the conditions specified in the Contract. | Yes/No (delete as appropriate). |
| 1.7. | Documentation | Operating and safety instructions in Lithuanian (user manuals for salt spreader) shall be provided no later than on the date of delivery of the Item. | Yes/No (delete as appropriate). |
| 1.8. | Training | At the time of delivery of the Item, but no later than when the Purchaser begins to use the Item or part of the set of the Item, the Supplier must provide the following training, the cost of which must be included in the offer price: - theoretical and practical training for operators to ensure the correct use of the Item. - technical training to ensure the correct maintenance of the Item, explaining the nature of faults and fault symbols. - technical training to ensure the correct and safe disassembly/assembly/replacement of the components of the set. | Yes/No (delete as appropriate). |
| 1.8.1 | Approving certificate | The Supplier (including partners – Suppliers of individual components of the set) shall prepare a certificate describing the scope of the training course for the Operator(s) who has/have completed theoretical and technical training and practical training. | Yes/No (delete as appropriate). |
| 1.9. | Manufacturer and installer of bundled equipment | Equipment shall be manufactured and installed on a truck by the manufacturer of the equipment or its official representative (in which case documents or copies thereof confirming the representation must be submitted with the tender). Alternatively, the Purchaser may carry out the installation, provided that the Supplier covers all costs incurred by the Purchaser for such installation. Responsibility for the flawless operation of the entire equipment complex and for remedying any defects discovered during operation shall rest with the manufacturer of the equipment or its official representative who carried out the installation, or with the Purchaser in the case where the Purchaser performed the installation. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2. SALT SPREADER |  |  |  |
| 2.1. | Purpose, adaptation Type, make | Salt spreader designed for installation (mounting) and operation in a body of a 3-axle truck. Designed for spreading, with or without moistening, salt, sand, fine crushed stone. Must comply with the requirements of standard LST EN 15597-1:2020. The spreader must be a standard product manufactured by the manufacturer in series, not specially modified or adapted to the requirements formulated by the contracting authority, except for specific marking and safety requirements. | Yes/No (delete as appropriate). Exact spreader model - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.2. | Quick connection and activation system for the hydraulic and electrical systems of the salt spreader. | The necessary electrical and hydraulic connections must be provided and installed by the manufacturer and marked accordingly. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.3. | Position of the spreader in the body (centre of gravity) | The centre of gravity of the spreader mounted into the truck body must be at least 600 mm forward of the centre of the 2-3 axle tandem. | Yes/No (delete as appropriate). |
| 2.4. | Installation of the salt spreader on a vehicle. | The spreader must have a quick loading (mounting)/unloading system for loading/unloading onto/from a self-propelled truck body (RO-RO or equivalent): loading and unloading of the spreader is carried out without external assistance and without the use of additional self-propelled lifting equipment. The metal structure of the quick loading (installation)/unloading and storage system must ensure safe, long-term and stable storage of the spreader (including partially loaded spreading material and/or partially filled with salt solution). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.5. | Upper overall dimension of the spreader. | The top height of the spreader mounted into the body of a truck from the bottom of the body shall not exceed 2300 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. |
| 2.6. | Weight of the spreader with all integral equipment. | 2 300 – 3 000 kg | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.7. | Spreader testing, adjustment, calibration | No later than on the day of delivery of the Item to the Purchaser, the salt spreader installed in the body of the truck must be tested, adjusted, calibrated, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual amount of material spread in various operating modes. calibration of the salt spreader installed in the truck body must be performed, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual quantities of material spread in various operating modes. All necessary work shall be performed at the Supplier's expense, except for the amount of salt, sand-salt mixture, salt solution required for testing and calibration, and the loading of materials into the spreader. | Yes/No (delete as appropriate). |
| Materials hopper |  |  |  |
| 2.8. | Materials hopper and structural frame | The materials hopper is made of steel sheets no thinner than 3 mm, and profiled square and rectangular pipes with a wall thickness of at least 3 mm are used for the frame structure. The entire structure is coated with an anti-corrosion coating and painted or made of stainless-steel. The surface painting process must be carried out in accordance with the requirements of ISO 8501-1. The total thickness of the coating (anti-corrosion coating and paint layer) is not less than 180 µm. The colour shall be orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s specified default (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). Proposed material, its thickness, and colour - _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.9. | Hopper capacity | Possible load capacity of loose materials – 8 - 9 m3 An additional point is awarded for a larger capacity of the hopper. | Yes/No (delete as appropriate). Proposed parameter - ____ m3. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.10. | Feeder of spreading materials. | A rubber belt conveyor, ensuring smooth feeding of materials to the spreading disc, regardless of the type and amount of the spreading material in the hopper. An additional point is awarded for an additional installed mechanism that adjusts the height of the material dosing valve according to the material selected on the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.11. | Mechanism for even distribution of the spreading material. | A separate mechanism installed along the entire length of the hopper, with a separate hydraulic drive or other type of drive and drive control, which distributes the spreading material more evenly over the rubber belt conveyor. | Yes/No (delete as appropriate). Type and description of the proposed device - ____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12. | Hopper cover | Covered with a metal grate (opening dimensions: no less than 40x40 mm, but no more than 70x70 mm, wire-rod diameter no less than 4 mm) and a moisture-proof, easily closable and openable side tarpaulin without additional means. The levers of the cover opening/closing mechanism must be accessible to the operator (the covering/uncovering process must be possible) while standing on the ground or on a ladder installed at the rear of the spreader. The grate structure is hot-dip galvanized. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Salt solution tank |  |  |  |
| 2.13. | Salt solution tanks | At least two parts (installed on the sides of the spreading materials hopper, in the lower part). Total volume of at least 3,5 m³. Depending on the volume of the spreader’s spreading material hopper, the ratio of the salt solution tank volume must be maintained at 7:3. | Yes/No (delete as appropriate). Proposed parameter - _____ m³. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.13.1 | Materials used in manufacturing | Tanks must be made of polypropylene or another equivalent polymer material that is resistant to corrosion, temperature fluctuations, and ultraviolet radiation. The resistance of the polymer materials used in salt solution tanks to ultraviolet radiation and other environmental conditions must comply with the DIN EN-17106 standard. | Yes/No (delete as appropriate). Proposed parameter – meets the requirement _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Control |  |  |  |
| 2.14. | Control of salt spreading and moistening equipment | Performed from the vehicle cab, using a digital control panel that is easily accessible to the driver from his workplace. | Yes/No (delete as appropriate). |
| 2.15. | Manual control | The possibility provided by the manufacturer to control the spreading process manually (in case of emergency). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.16. | Control panel | During the shift, the operator must be able to continuously view and control at least the following parameters on the control panel display: - Selected spreading material and spreading rate; - Spreading width and asymmetry; - Moistening of the spreading material with solution is on/off, selected moistening percentage (%); - Amount of spreading material; - Amount of solution The control panel must: - collect data and ensure digital scanning and real-time (“Online“) transmission of data to the dispatcher’s workstation; - inform the operator about the shortage of the spreading materials or salt solution with an additional audible signal; - have connections for data transmission in accordance with data encryption standard EN15430 or an equivalent standard. - have a menu in Lithuanian. The control panel menu must indicate the following names of spreading materials (may be without Lithuanian symbols, e.g., Misinys (Mixture) 1:1): • Salt • Mixture 1:1 • Mixture 1:3 • Mixture 1:10 • Solution The Supplier must provide the Purchaser with the access codes required to adjust the control panel settings. The meanings of the control panel buttons must be provided in the user manual. An additional point is awarded for a spreader control panel with an automatic spreading function according to a selected pre-programmed route. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.17. | System for transferring data about operation | The Supplier must provide access to the control panel data transmission, at least for the following data (for the purpose of controlling the spreader operation): 1. Date, time, operating time. 2. Spreader operating parameters: spreading is on/off or spreading mode, spreading time, spreading width, name of the material being spread, dosage of the material being spread g/m², amount spread kg, solution dosage ml/m² or g/m² (depending on the manufacturer’s specifications), amount of solution dispensed in litres. 3. Position of snow plow: operating/transport position. Transport telemetry control services are provided to the Purchaser by UAB Xirgo Global. The tender must be accompanied by information that clearly and comprehensively shows the required data. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18. | Generation of data transmission system reports | Reports must be generated with freely selectable dates and times. They must include: The date of spreading, the start/end time of spreading. The start/end location of spreading, the total spreading time, the spreading distance, the consumption of the spreading material and salt solution (kg, litres). The dosage of the spreading material in g/m², the dosage of the solution in ml/m² or g/m² (depending on the manufacturer’s specifications). Distance travelled by the vehicle and spreader, m. Names of materials spread. The tender must include detailed information about the reports to be generated and the required data to be included in them. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.19. | Even spreading and salt solution feeding. | The vehicle speed signal must be transmitted to the spreader control panel (linked). The even supply of salt and salt solution to the spreading disc, the even rotation of the disc and the spreading of materials, according to the settings on the control panel, must take place regardless of the vehicle's speed, as well as when the vehicle is stationary. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.20. | Emergency unloading function. | Must be provided by the manufacturer and activated via the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.21. | Spreading parameter programming and fault notification system. | Must be provided and installed by the manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.22. | Instantaneous increase of the amount being spread. | A function that allows the amount of material being spread to be increased two or more times for a limited period of time. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Spreading (dispersal) mechanism |  |  |  |
| 2.23. | Spreading material and solution mixing system. | Ensuring mixing of the spreading material and salt solution and delivery of the mixture onto the spreading disc, or mixing of salt and solution directly on the spreading disc. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.24. | Salt moistening and spreading mechanism | Variable length sleeve/trough and disc with hydraulic drive, liftable upwards and lockable in transport position. The mechanism is lifted and locked in the transport position. Air shock absorbers provided for and installed by the manufacturer, or equivalent solution to facilitate lifting of the mechanism into transport position. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.25. | Spreading disc | Diameter not less than 600 mm. The height of the disc above the road surface is adjusted mechanically (manually) at intervals not less than 100 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26. | Spreading disc hydraulic motor | Protected from direct exposure to salt. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27. | Salt spreading sensor | Must be provided and installed by the manufacturer - contactless, electronic. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27.1 | Visual control | A rear-view camera with night vision, installed by the spreader manufacturer, is mounted at the rear of the spreader, as high as technically possible, for visual control of the spreading process and control of the vehicle’s reverse movement. The image transmitted by the camera must be visible at the operator’s workplace (in the cab of truck) on the monitor (in a separate section) located on the spreader control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.28. | Adjustable amount of material being spread per m² | Salt - within the range of 5-40 g/m², with a maximum “pace“ of 5 g/m². Sifted sand and crushed stone - within the range of 20-300 g/m², with a maximum “pace“ of 10 g/m². | Yes/No (delete as appropriate). Proposed parameter: Salt within the range of ____ g/m2. Spread gravel, sand, crushed stone within the range of ____ g/m2. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29. | Adjustable spreading width of the material being spread. | From 3 m to at least 12 m, at intervals of no more than 0.5 m, with asymmetrical (increasing or decreasing the width of one side of the spread without changing the width of the other side) adjustment of the spreading material, set using the control panel. | Yes/No (delete as appropriate). Proposed parameter: Within ____ m, spreading width “pace” of __m. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.30. | Forced (pressure) salt solution feeding (to the spreading disc) and control of amount. | At interval of 5 %, starting from a minimum of 15%, which automatically changes depending on the amount of salt being spread. It is assumed that the percentage of salt solution for salt moistening indicates the proportion of the solution in the spread rate (e.g., 15% means that the spread rate consists of 15% solution + 85% dry matter). | Yes/No (delete as appropriate). Proposed parameter: At __ % interval from ___ % Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Safety fence |  |  |  |
| 2.31. | Safety fence – lightweight construction, 1-1.15 m high, on the both sides of the spreader. | Mounted on top of the salt spreader hopper and raised/folded for maintenance/repair before accessing the hopper. The raising/lowering of the Safety fence is performed by lowering/raising the ladder intended for access to the hopper or by a separate raising/lowering mechanism. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Signal marking |  |  |  |
| 2.32. | Dimensional marking, signal lighting | The working (spreading/dispersal) area must be illuminated by at least one light. At least one orange LED beacon installed at the rear of the spreader. Two flashing LED lights (round lights) with a diameter of at least 180 mm, installed at the upper corners of the rear part of the spreader. The rear part marked with warning light-reflecting diagonal red and white stripes. Sample pattern of warning marking: Protection of LED beacons and flashing LED lights against solid objects and liquids must be at least IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.33. | Mounting locations for road signs and road signs. | The rear part of the spreader (on the mounting/suspension unit/frame) shall be equipped with places for mounting warning/directional road signs or the necessary mounting structure, and no later than on the date of delivery of the Item, the following shall be installed: - a light-reflecting sign No 106 “Work“ not less than 1 (first) size: - Signal LED light pointer – a system of at least 13 lights complying with L8H, EN 12352 or an equivalent standard, fastened from the rear with screws or equivalent fasteners. The direction of the arrow indicating the mandatory side of the obstacle is changed from the operator's workplace using an electrical connection. A visual indicator informing about the activated direction of the arrow is mandatory at the operator's workplace. The feed voltage of the arrow lights is 24 V. The protection of the LED light system components against solid objects and liquids is not less than IP65. | Yes/No (delete as appropriate). |

* The Supplier must confirm compliance with the technical requirement by indicating: Yes/No (delete as appropriate) and, where required, enter the exact parameter/value of the proposed Item.

Where required, the Supplier must enter the name of the document proving the parameters and/or a reference to the source confirming the proposed parameters.

____________________                               _____________                                       _____________________

(position)	                           (signature)	         (forename, surname)

If the tender is signed by a person empowered by the Supplier, a document (power of attorney) granting the specified person the right to sign on behalf of the Supplier must be submitted together with the tender.

PART 8 OF THE PROCUREMENT. SALT SPREADERS FOR TWO-AXLE VEHICLES (3 sets)

The salt spreader will be operated with a truck:

| Vehicle location (KT, mastery) | Truck | Year | Registration plate No | Factory No | Inv. No | Spreader volume, m³ | Spreader operating method | Wheelbase, mm | Spreader power unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Panevėžio KT, Panevėžio MT | Renault Premium 310.19 | 2011 | FRB512 | VF629AHA000007449 | 0404-00072 | 6-7 | To the body | 3 900 | Vehicle hydraulic system |
| Pasvalio KT, Biržų MT | Renault Kerax 380.22 | 2013 | HAD768 | VF634BPA000001572 | 0404-00284 | 6-7 | To the body | 4 300 | Vehicle hydraulic system |
| Šiaulių KT, Šiaulių MT | Renault Premium 340.19 | 2013 | HAK943 | VF629AHA000009156 | 0404-00596 | 6-7 | To the body | 4 500 | Vehicle hydraulic system |

| Ser. No | Name of characteristics | Characteristics required by the Purchaser | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the Item offered. In the clauses where required, the Supplier must enter the name of the document submitted, proving the values ​​of the parameters, and/or a reference to the source confirming the parameters offered. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1. | Purpose | Equipment designed for spreading anti-skid materials on road surfaces. | Yes/No (delete as appropriate). |
| 1.2. | Year of production | The salt spreader must be new and unused prior to the date of transfer. | Yes/No (delete as appropriate). Year and quarter of production – (enter) ___________. |
| 1.3. | The dominant colour of the salt spreader and of the painted metal surfaces | Orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific technical group. Galvanized metal surfaces and stainless-steel elements may be left unpainted. | Yes/No (delete as appropriate). Proposed dominant colour – (enter parameter) _____________. |
| 1.4. | Warranty against corrosion perforation | All components included in the set, without exception, have a minimum warranty period of 72 months. | Yes/No (delete as appropriate). Proposed parameter - ____ months. |
| 1.5. | Right to perform warranty repairs and warranty technical maintenance | The Supplier must be the official representative of the manufacturer of the offered Goods (if the Supplier is not the manufacturer), having the right to perform technical maintenance and repair of the offered Goods during the warranty period, or must have concluded a corresponding service agreement with another entity authorized to perform such services. The tender must include the manufacturer’s authorization or another document confirming the Supplier’s right to provide technical maintenance and repair services during the warranty period, or evidence of a concluded service agreement with another entity authorized to provide such services, or the Supplier must provide a manufacturer-issued authorization granting the Purchaser the right to perform technical maintenance and repair services during the warranty period. | Yes/No (delete as appropriate). |
| 1.6. | Terms of technical service and warranty repairs | The Supplier must perform warranty repair works within no more than 2 working days after the Purchaser’s request is submitted in writing (by e-mail) and by a phone call to the Supplier’s responsible person indicated in the Contract, or may authorize the Purchaser during the warranty period (providing a manufacturer-issued authorization) to carry out warranty repair works by qualified Purchaser’s personnel, with the costs of the works and other related expenses reimbursed in accordance with the conditions specified in the Contract. | Yes/No (delete as appropriate). |
| 1.7. | Documentation | Operating and safety instructions in Lithuanian (user manuals for salt spreader) shall be provided no later than on the date of delivery of the Item. | Yes/No (delete as appropriate). |
| 1.8. | Training | At the time of delivery of the Item, but no later than when the Purchaser begins to use the Item or part of the set of the Item, the Supplier must provide the following training, the cost of which must be included in the offer price: -theoretical and practical training for operators to ensure the correct use of the Item. - technical training to ensure the correct maintenance of the Item, explaining the nature of faults and fault symbols. - technical training to ensure the correct and safe disassembly/assembly/replacement of the components of the set. | Yes/No (delete as appropriate). |
| 1.8.1 | Approving certificate | The Supplier (including partners – Suppliers of individual components of the set) shall prepare a certificate describing the scope of the training course for the Operator(s) who has/have completed theoretical and technical training and practical training. | Yes/No (delete as appropriate). |
| 1.9. | Manufacturer and installer of bundled equipment | Equipment must be manufactured and installed/mounted on a truck by the manufacturer of this equipment or its official representative (in which case, documents or copies of documents confirming representation must be submitted with the tender). Responsibility for the flawless operation of the entire equipment complex and the elimination of any defects discovered during operation lies with the manufacturer of this equipment or its official representative who installed the equipment complex. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2. SALT SPREADER |  |  |  |
| 2.1. | Purpose, adaptation Type, make | Salt spreader designed for installation (mounting) and operation in a body of a 2-axle truck. Designed for spreading, with or without moistening, salt, sand, fine crushed stone. Must comply with the requirements of standard LST EN 15597-1:2020. The spreader must be a standard product manufactured by the manufacturer in series, not specially modified or adapted to the requirements formulated by the contracting authority, except for specific marking and safety requirements. | Yes/No (delete as appropriate). Exact spreader model - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.2. | Quick connection and activation system for the hydraulic and electrical systems of the salt spreader. | The necessary electrical and hydraulic connections must be provided and installed by the manufacturer and marked accordingly. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.3. | Spreader position in the body (centre of gravity) | The centre of gravity of a spreader mounted and secured into the body of a truck and ready for operation must be at least 500 mm forward of the centre of the rear axle. | Yes/No (delete as appropriate). |
| 2.4. | Installation of the salt spreader on a vehicle. | The spreader must have a quick loading (mounting)/unloading system for loading/unloading onto/from a self-propelled truck body (RO-RO or equivalent): loading and unloading of the spreader is carried out without external assistance and without the use of additional self-propelled lifting equipment. The metal structure of the quick loading (installation)/unloading and storage system must ensure safe, long-term and stable storage of the spreader (including partially loaded spreading material and/or partially filled with salt solution). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.5. | Upper overall dimension of the spreader. | The top height of the spreader mounted into the body of a truck from the bottom of the body shall not exceed 2300 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. |
| 2.6. | Weight of the spreader with all integral equipment. | 2 000 – 2 800 kg | Yes/No (delete as appropriate). Proposed parameter: - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.7. | Spreader testing, adjustment, calibration | No later than on the day of delivery of the Item to the Purchaser, the salt spreader installed in the body of the truck must be tested, adjusted, calibrated, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual amount of material spread in various operating modes. calibration of the salt spreader installed in the truck body must be performed, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual quantities of material spread in various operating modes. All necessary work shall be performed at the Supplier's expense, except for the amount of salt, sand-salt mixture, salt solution required for testing and calibration, and the loading of materials into the spreader. | Yes/No (delete as appropriate). |
| Materials hopper |  |  |  |
| 2.8. | Materials hopper and structural frame | The materials hopper is made of steel sheets no thinner than 3 mm, and profiled square and rectangular pipes with a wall thickness of at least 3 mm are used for the frame structure. The entire structure is coated with an anti-corrosion coating and painted or made of stainless-steel. The surface painting process must be carried out in accordance with the requirements of ISO 8501-1. The total thickness of the coating (anti-corrosion coating and paint layer) is not less than 180 µm. The colour shall be orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s specified default (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). Proposed material, its thickness, and colour - _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.9. | Hopper capacity | Possible load capacity of loose materials – 6 -7 m3 An additional point is awarded for a larger capacity of the hopper. | Yes/No (delete as appropriate). Proposed parameter - ____ m3. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.10. | Feeder of spreading materials. | A rubber belt conveyor, ensuring smooth feeding of materials to the spreading disc, regardless of the type and amount of the spreading material in the hopper. An additional point is awarded for an additional installed mechanism that adjusts the height of the material dosing valve according to the material selected on the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.11. | Mechanism for even distribution of the spreading material. | A separate mechanism installed along the entire length of the hopper, with a separate hydraulic drive or other type of drive and drive control, which distributes the spreading material more evenly over the rubber belt conveyor. | Yes/No (delete as appropriate). Type and description of the proposed device - ____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12. | Hopper cover | Covered with a metal grate (opening dimensions: no less than 40x40 mm, but no more than 70x70 mm, wire-rod diameter no less than 4 mm) and a moisture-proof, easily closable and openable side tarpaulin without additional means. The levers of the cover opening/closing mechanism must be accessible to the operator (the covering/uncovering process must be possible) while standing on the ground or on a ladder installed at the rear of the spreader. The grate structure is hot-dip galvanized. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Salt solution tank |  |  |  |
| 2.13. | Salt solution tanks | At least two parts (installed on the sides of the spreading materials hopper, in the lower part). Total volume of at least 2,6 m³. Depending on the volume of the spreader's spreader material hopper, the ratio of the salt solution tank volume must be maintained at 7:3. | Yes/No (delete as appropriate). Proposed parameter - _____ m³. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.13.1 | Materials used in manufacturing | Tanks must be made of polypropylene or another equivalent polymer material that is resistant to corrosion, temperature fluctuations, and ultraviolet radiation. The resistance of the polymer materials used in salt solution tanks to ultraviolet radiation and other environmental conditions must comply with the DIN EN-17106 standard. | Yes/No (delete as appropriate). Proposed parameter – meets the requirement _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Control |  |  |  |
| 2.14. | Control of salt spreading and moistening equipment | Performed from the vehicle cab, using a digital control panel that is easily accessible to the driver from his workplace. | Yes/No (delete as appropriate). |
| 2.15. | Manual control | The possibility provided by the manufacturer to control the spreading process manually (in case of emergency). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.16. | Control panel | During the shift, the operator must be able to continuously view and control at least the following parameters on the control panel display: - Selected spreading material and spreading rate; - Spreading width and asymmetry; - Moistening of the spreading material with solution is on/off, selected moistening percentage (%); - Amount of spreading material; - Amount of solution The control panel must: - collect data and ensure digital scanning and real-time (“Online“) transmission of data to the dispatcher’s workstation; - inform the operator about the shortage of the spreading materials or salt solution with an additional audible signal; - have connections for data transmission in accordance with data encryption standard EN15430 or an equivalent standard. - have a menu in Lithuanian. The control panel menu must indicate the following names of spreading materials (may be without Lithuanian symbols, e.g., Misinys (Mixture) 1:1): • Salt • Mixture 1:1 • Mixture 1:3 • Mixture 1:10 • Solution The Supplier must provide the Purchaser with the access codes required to adjust the control panel settings. The meanings of the control panel buttons must be provided in the user manual. An additional score is awarded for a spreader control panel with an automatic spreading function according to a selected pre-programmed route. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.17. | System for transferring data about operation | The Supplier must provide access to the control panel data transmission, at least for the following data (for the purpose of controlling the spreader operation): 1. Date, time, operating time. 2. Spreader operating parameters: spreading is on/off or spreading mode, spreading time, spreading width, name of the material being spread, dosage of the material being spread g/m², amount spread kg, solution dosage ml/m² or g/m² (depending on the manufacturer’s specifications), amount of solution dispensed in litres. 3. Position of snow plow: operating/transport position. Transport telemetry control services are provided to the Purchaser by UAB Xirgo Global. The tender must be accompanied by information that clearly and comprehensively shows the required data. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18. | Generation of data transmission system reports | Reports must be generated with freely selectable dates and times. They must include: The date of spreading, the start/end time of spreading. The start/end location of spreading, the total spreading time, the spreading distance, the consumption of the spreading material and salt solution (kg, litres). The dosage of the spreading material in g/m², the dosage of the solution in ml/m² or g/m² (depending on the manufacturer’s specifications). Distance travelled by the vehicle and spreader, m. Names of materials spread. The tender must include detailed information about the reports to be generated and the required data to be included in them. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.19. | Even spreading and salt solution feeding. | The vehicle speed signal must be transmitted to the spreader control panel (linked). The even supply of salt and salt solution to the spreading disc, the even rotation of the disc and the spreading of materials, according to the settings on the control panel, must take place regardless of the vehicle's speed, as well as when the vehicle is stationary. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.20. | Emergency unloading function. | Must be provided by the manufacturer and activated via the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.21. | Spreading parameter programming and fault notification system. | Must be provided and installed by the manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.22. | Instantaneous increase of the amount being spread. | A function that allows the amount of material being spread to be increased two or more times for a limited period of time. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Spreading (dispersal) mechanism |  |  |  |
| 2.23. | Spreading material and solution mixing system. | Ensuring mixing of the spreading material and salt solution and delivery of the mixture onto the spreading disc, or mixing of salt and solution directly on the spreading disc. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.24. | Salt moistening and spreading mechanism | Variable length sleeve/trough and disc with hydraulic drive, liftable upwards and lockable in transport position. The mechanism is lifted and locked in the transport position. Air shock absorbers provided for and installed by the manufacturer, or equivalent solution to facilitate lifting of the mechanism into transport position. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.25. | Spreading disc | Diameter not less than 600 mm. The height of the disc above the road surface is adjusted mechanically (manually) at interval not less than 100 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26. | Spreading disc hydraulic motor | Protected from direct exposure to salt. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27. | Salt spreading sensor | Must be provided and installed by the manufacturer - contactless, electronic. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27.1 | Visual control | A rear-view camera with night vision, installed by the spreader manufacturer, is mounted at the rear of the spreader, as high as technically possible, for visual control of the spreading process and control of the vehicle’s reverse movement. The image transmitted by the camera must be visible at the operator’s workplace (in the cab of truck) on the monitor (in a separate section) located on the spreader control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.28. | Adjustable amount of material being spread per m² | Salt - within the range of 5-40 g/m², with a maximum “pace“ of 5 g/m². Sifted sand and crushed stone - within the range of 20-300 g/m², with a maximum “pace“ of 10 g/m². | Yes/No (delete as appropriate). Proposed parameter: Salt within the range of ____ g/m2. Spread gravel, sand, crushed stone within the range of ____ g/m2. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29. | Adjustable spreading width of the material being spread. | From 3 m to at least 12 m, at intervals of no more than 0.5 m, with asymmetrical (increasing or decreasing the width of one side of the spread without changing the width of the other side) adjustment of the spreading material, set using the control panel. | Yes/No (delete as appropriate). Proposed parameter: Within ____ m, spreading width “pace” of __m. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.30. | Forced (pressure) salt solution feeding (to the spreading disc) and control of amount. | At interval of 5 %, starting from a minimum of 15%, which automatically changes depending on the amount of salt being spread. It is assumed that the percentage of salt solution for salt moistening indicates the proportion of the solution in the spread rate (e.g., 15% means that the spread rate consists of 15% solution + 85% dry matter). | Yes/No (delete as appropriate). Proposed parameter: At __ % interval from ___ % Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Safety fence |  |  |  |
| 2.31. | Safety fence – lightweight construction, 1-1.15 m high, on the both sides of the spreader. | Mounted on top of the salt spreader hopper and raised/folded for maintenance/repair before accessing the hopper. The raising/lowering of the Safety fence is performed by lowering/raising the ladder intended for access to the hopper or by a separate raising/lowering mechanism. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Signal marking |  |  |  |
| 2.32. | Dimensional marking, signal lighting | The working (spreading/dispersal) area must be illuminated by at least one light. At least one orange LED beacon installed at the rear of the spreader. Two flashing LED lights (round lights) with a diameter of at least 180 mm, installed at the upper corners of the rear part of the spreader. The rear part marked with warning light-reflecting diagonal red and white stripes. Sample pattern of warning marking: Protection of LED beacons and flashing LED lights against solid objects and liquids must be at least IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.33. | Mounting locations for road signs and road signs. | The rear part of the spreader (on the mounting/suspension unit/frame) shall be equipped with places for mounting warning/directional road signs or the necessary mounting structure, and no later than on the date of delivery of the Item, the following shall be installed: - a light-reflecting sign No 106 “Work“ not less than 1 (first) size: - Signal LED light pointer – a system of at least 13 lights complying with L8H, EN 12352 or an equivalent standard, fastened from the rear with screws or equivalent fasteners. The direction of the arrow indicating the mandatory side of the obstacle is changed from the operator's workplace using an electrical connection. A visual indicator informing about the activated direction of the arrow is mandatory at the operator's workplace. The feed voltage of the arrow lights is 24 V. The protection of the LED light system components against solid objects and liquids is not less than IP65. | Yes/No (delete as appropriate). |

* The Supplier must confirm compliance with the technical requirement by indicating: Yes/No (delete as appropriate) and, where required, enter the exact parameter/value of the proposed Item.

Where required, the Supplier must enter the name of the document proving the parameters and/or a reference to the source confirming the proposed parameters.

____________________                               _____________                                       _____________________

(position)	                             (signature)	          (forename, surname)

If the tender is signed by a person empowered by the Supplier, a document (power of attorney) granting the specified person the right to sign on behalf of the Supplier must be submitted together with the tender.

PART 9 OF THE PROCUREMENT. SALT SPREADER FOR A THREE-AXLE VEHICLE (6X4) (new or used) (1 set)

The salt spreader will the operated with a truck:

| Vehicle location (KT, mastery) | Truck | Year | Registration plate No | Factory No | Inv. No | Spreader volume, m³ | Spreader operating method | Wheelbase, mm | Spreader power unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rokiškio KT, Kupiškio MT | MAN TGA 33.390 | 2006 | CEU157 | MAH26ZZ27M463843 | 0403-00538 | 7 | To the body | 3 700 | Vehicle hydraulic system |

| Ser. No | Name of characteristics | Characteristics required by the Purchaser | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the Item offered. In the clauses where required, the Supplier must enter the name of the document submitted, proving the values ​​of the parameters, and/or a reference to the source confirming the parameters offered. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1. | Purpose | Equipment designed for spreading anti-skid materials on road surfaces. | Yes/No (delete as appropriate). |
| 1.2. | Year of production | The salt spreader may be used, have been in operation until the date of transfer, but it must be manufactured no earlier than 2023. | Yes/No (delete as appropriate). Year and quarter of production – (enter) ___________. |
| 1.2.1 | Operating hours worked | In the event that a used salt spreader is offered, its operating hours (up to the time of submission of the tender) shall not exceed 650 operating hours. An additional point shall be awarded to a spreader with fewer operating hours. | Yes/No (delete as appropriate). Operating hours of the proposed spreader - (enter) ___________ h. |
| 1.3. | The dominant colour of the salt spreader and of the painted metal surfaces | Grey (Pantone 7544C) or the manufacturer’s default (representative) colour for a specific group of equipment. Galvanized metal surfaces and stainless-steel elements may be left unpainted. | Yes/No (delete as appropriate). Proposed dominant colour – (enter parameter) _____________. |
| 1.4. | Warranty against corrosion perforation | All components included in the set, without exception, have a minimum warranty period of 72 months. | Yes/No (delete as appropriate). Proposed parameter - ____ months. |
| 1.5. | Right to perform warranty repairs and warranty technical maintenance | The Supplier must be the official representative of the manufacturer of the offered Goods (if the Supplier is not the manufacturer), having the right to perform technical maintenance and repair of the offered Goods during the warranty period, or must have concluded a corresponding service agreement with another entity authorized to perform such services. The tender must include the manufacturer’s authorization or another document confirming the Supplier’s right to provide technical maintenance and repair services during the warranty period, or evidence of a concluded service agreement with another entity authorized to provide such services, or the Supplier must provide a manufacturer-issued authorization granting the Purchaser the right to perform technical maintenance and repair services during the warranty period. | Yes/No (delete as appropriate). |
| 1.6. | Terms of technical service and warranty repairs | The Supplier must perform warranty repair works within no more than 2 working days after the Purchaser’s request is submitted in writing (by e-mail) and by a phone call to the Supplier’s responsible person indicated in the Contract. | Yes/No (delete as appropriate). |
| 1.7. | Documentation | Operating and safety instructions in Lithuanian (user manuals for salt spreader) shall be provided no later than on the date of delivery of the Item. | Yes/No (delete as appropriate). |
| 1.8. | Training | At the time of delivery of the Item, but no later than when the Purchaser begins to use the Item or part of the set of the Item, the Supplier must provide the following training, the cost of which must be included in the offer price: - theoretical and practical training for operators to ensure the correct use of the Item. - technical training to ensure the correct maintenance of the Item, explaining the nature of faults and fault symbols. - technical training to ensure the correct and safe disassembly/assembly/replacement of the components of the set. | Yes/No (delete as appropriate). |
| 1.8.1 | Approving certificate | The Supplier (including partners – Suppliers of individual components of the set) shall prepare a certificate describing the scope of the training course for the Operator(s) who has/have completed theoretical and technical training and practical training. | Yes/No (delete as appropriate). |
| 1.9. | Manufacturer and installer of bundled equipment | Equipment must be manufactured and installed/mounted on a truck by the manufacturer of this equipment or its official representative (in which case, documents or copies of documents confirming representation must be submitted with the tender). Responsibility for the flawless operation of the entire equipment complex and the elimination of any defects discovered during operation lies with the manufacturer of this equipment or its official representative who installed the equipment complex. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2. SALT SPREADER |  |  |  |
| 2.1. | Purpose, adaptation Type, make | Salt spreader designed for installation (mounting) and operation in a body of a 3-axle truck. Designed for spreading, with or without moistening, salt, sand, fine crushed stone. Must comply with the requirements of standard LST EN 15597-1:2020. The spreader must be a standard product manufactured by the manufacturer in series, not specially modified or adapted to the requirements formulated by the contracting authority, except for specific marking and safety requirements. | Yes/No (delete as appropriate). Exact spreader model - _________. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.2. | Quick connection and activation system for the hydraulic and electrical systems of the salt spreader. | The necessary electrical and hydraulic connections must be provided and installed by the manufacturer and marked accordingly. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.3. | Spreader position in the body (centre of gravity) | The centre of gravity of the spreader mounted into the truck body must be at least 600 mm forward of the centre of the 2-3 axle tandem. | Yes/No (delete as appropriate). |
| 2.4. | Installation of the salt spreader on a vehicle. | The spreader must have a quick loading (mounting)/unloading system for loading/unloading onto/from a self-propelled truck body (RO-RO or equivalent): loading and unloading of the spreader is carried out without external assistance and without the use of additional self-propelled lifting equipment. The metal structure of the quick loading (installation)/unloading and storage system must ensure safe, long-term and stable storage of the spreader (including partially loaded spreading material and/or partially filled with salt solution). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.5. | Upper overall dimension of the spreader. | The top height of the spreader mounted into the body of a truck from the bottom of the body shall not exceed 2300 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. |
| 2.6. | Weight of the spreader with all integral equipment. | 2 100 – 2 600 kg | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.7. | Spreader testing, adjustment, calibration | No later than on the day of delivery of the Item to the Purchaser, the salt spreader installed in the body of the truck must be tested, adjusted, calibrated, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual amount of material spread in various operating modes. calibration of the salt spreader installed in the truck body must be performed, and a calibration/verification document certified by the Supplier or its authorised representative must be submitted, indicating the actual quantities of material spread in various operating modes. All necessary work shall be performed at the Supplier’s expense, except for the amount of salt, sand-salt mixture, salt solution required for testing and calibration, and the loading of materials into the spreader. | Yes/No (delete as appropriate). |
| Materials hopper |  |  |  |
| 2.8. | Materials hopper and structural frame | The materials hopper is made of steel sheets no thinner than 3 mm, and profiled square and rectangular pipes with a wall thickness of at least 3 mm are used for the frame structure. The surface painting process must be carried out in accordance with the requirements of ISO 8501-1. The total thickness of the coating (anti-corrosion coating and paint layer) is not less than 180 µm. The colour shall be orange (RAL 2004), grey (Pantone 7544C) or the manufacturer’s specified default (representative) colour for a specific group of equipment. | Yes/No (delete as appropriate). Proposed material, its thickness, and colour - _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.9. | Hopper capacity | Possible load capacity of loose materials – not less than 7 m3 | Yes/No (delete as appropriate). Proposed parameter- ____ m3. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.10. | Feeder of spreading materials. | A rubber belt conveyor, ensuring smooth feeding of materials to the spreading disc, regardless of the type and amount of the spreading material in the hopper. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.11. | Mechanism for even distribution of the spreading material. | A separate mechanism installed along the entire length of the hopper, with a separate hydraulic drive or other type of drive and drive control, which distributes the spreading material more evenly over the rubber belt conveyor. | Yes/No (delete as appropriate). Type and description of the proposed device - ____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.12. | Hopper cover | Covered with a metal grate (opening dimensions: no less than 40x40 mm, but no more than 70x70 mm, wire-rod diameter no less than 4 mm) and a moisture-proof, easily closable and openable side tarpaulin without additional means. The levers of the cover opening/closing mechanism must be accessible to the operator (the covering/uncovering process must be possible) while standing on the ground or on a ladder installed at the rear of the spreader. The grate structure is hot-dip galvanized. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Salt solution tank |  |  |  |
| 2.13. | Salt solution tanks | At least two parts (installed on the sides of the spreading materials hopper, in the lower part). Total volume of at least 3,0 m³. Depending on the volume of the spreader's spreader material hopper, the ratio of the salt solution tank volume must be maintained at 7:3. | Yes/No (delete as appropriate). Proposed parameter - _____ m³. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.13.1 | Materials used in manufacturing | Tanks must be made of polypropylene or another equivalent polymer material that is resistant to corrosion, temperature fluctuations, and ultraviolet radiation. The resistance of the polymer materials used in salt solution tanks to ultraviolet radiation and other environmental conditions must comply with the DIN EN-17106 standard. | Yes/No (delete as appropriate). Proposed parameter – meets the requirement _____. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Control |  |  |  |
| 2.14. | Control of salt spreading and moistening equipment | Performed from the vehicle cab, using a digital control panel that is easily accessible to the driver from his workplace. | Yes/No (delete as appropriate). |
| 2.15. | Manual control | The possibility provided by the manufacturer to control the spreading process manually (in case of emergency). | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.16. | Control panel | During the shift, the operator must be able to continuously view and control at least the following parameters on the control panel display: - Selected spreading material and spreading rate; - Spreading width and asymmetry; - Moistening of the spreading material with solution is on/off, selected moistening percentage (%); - Amount of spreading material; - Amount of solution The control panel must: - collect data and ensure digital scanning and real-time (“Online“) transmission of data to the dispatcher’s workstation; - inform the operator about the shortage of the spreading materials or salt solution with an additional audible signal; - have connections for data transmission in accordance with data encryption standard EN15430 or an equivalent standard. - have a menu in Lithuanian. The control panel menu must indicate the following names of spreading materials (may be without Lithuanian symbols, e.g., Misinys (Mixture) 1:1): • Salt • Mixture 1:1 • Mixture 1:3 • Mixture 1:10 • Solution The Supplier must provide the Purchaser with the access codes required to adjust the control panel settings. The meanings of the control panel buttons must be provided in the user manual. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.17. | System for transferring data about operation | The Supplier must provide access to the control panel data transmission, at least for the following data (for the purpose of controlling the spreader operation): 1. Date, time, operating time. 2. Spreader operating parameters: spreading is on/off or spreading mode, spreading time, spreading width, name of the material being spread, dosage of the material being spread g/m², amount spread kg, solution dosage ml/m² or g/m² (depending on the manufacturer’s specifications), amount of solution dispensed in litres. 3. Position of snow plow: operating/transport position. Transport telemetry control services are provided to the Purchaser by UAB Xirgo Global. The tender must be accompanied by information that clearly and comprehensively shows the required data. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.18. | Generation of data transmission system reports | Reports must be generated with freely selectable dates and times. They must include: The date of spreading, the start/end time of spreading. The start/end location of spreading, the total spreading time, the spreading distance, the consumption of the spreading material and salt solution (kg, litres). The dosage of the spreading material in g/m², the dosage of the solution in ml/m² or g/m² (depending on the manufacturer’s specifications). Distance travelled by the vehicle and spreader, m. Names of materials spread. The tender must include detailed information about the reports to be generated and the required data to be included in them. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.19. | Even spreading and salt solution feeding. | The vehicle speed signal must be transmitted to the spreader control panel (linked). The even supply of salt and salt solution to the spreading disc, the even rotation of the disc and the spreading of materials, according to the settings on the control panel, must take place regardless of the vehicle's speed, as well as when the vehicle is stationary. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.20. | Emergency unloading function. | Must be provided by the manufacturer and activated via the control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.21. | Spreading parameter programming and fault notification system. | Must be provided and installed by the manufacturer. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.22. | Instantaneous increase of the amount being spread. | A function that allows the amount of material being spread to be increased two or more times for a limited period of time. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Spreading (dispersal) mechanism |  |  |  |
| 2.23. | Spreading material and solution mixing system. | Ensuring mixing of the spreading material and salt solution and delivery of the mixture onto the spreading disc, or mixing of salt and solution directly on the spreading disc. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.24. | Salt moistening and spreading mechanism | Variable length sleeve/trough and disc with hydraulic drive, liftable upwards and lockable in transport position. The mechanism is lifted and locked in the transport position. Air shock absorbers provided for and installed by the manufacturer, or equivalent solution to facilitate lifting of the mechanism into transport position. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.25. | Spreading disc | Diameter not less than 660 mm. The height of the disc from the carriageway surface is adjusted mechanically (manually) at intervals not less than 100 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.26. | Spreading disc hydraulic motor | Protected from direct exposure to salt. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27. | Salt spreading sensor | Must be provided and installed by the manufacturer - contactless, electronic. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.27.1 | Visual control. | A rear-view camera with night vision, installed by the spreader manufacturer, is mounted at the rear of the spreader, as high as technically possible, for visual control of the spreading process and control of the vehicle’s reverse movement. The image transmitted by the camera must be visible at the operator’s workplace (in the cab of truck) on the monitor (in a separate section) located on the spreader control panel. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.28. | Adjustable amount of material being spread per m² | Salt - within the range of 5-40 g/m², with a maximum “pace“ of 5 g/m². Sifted sand and crushed stone - within the range of 20-300 g/m², with a maximum “pace“ of 10 g/m². | Yes/No (delete as appropriate). Proposed parameter: Salt within the range of ____ g/m2. Spread gravel, sand, crushed stone within the range of ____ g/m2. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.29. | Adjustable spreading width of the material being spread. | From 3 m to at least 12 m, at intervals of no more than 1.0 m, with asymmetrical (increasing or decreasing the width of one side of the spread without changing the spread width of the other side) adjustment of the spread material, set using the control panel. | Yes/No (delete as appropriate). Proposed parameter: Within ____ m, spreading width “pace” of __m. Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.30. | Forced (pressure) salt solution feeding (to the spreading disc) and control of amount. | At interval of 5 %, starting from a minimum of 15%, which automatically changes depending on the amount of salt being spread. It is assumed that the percentage of salt solution for salt moistening indicates the proportion of the solution in the spread rate (e.g., 15% means that the spread rate consists of 15% solution + 85% dry matter). | Yes/No (delete as appropriate). Proposed parameter: At __ % interval from ___ % Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Safety fence |  |  |  |
| 2.31. | Safety fence – lightweight construction, 1-1.15 m high, on the both sides of the spreader. | Mounted on top of the salt spreader hopper and raised/folded for maintenance/repair before accessing the hopper. The raising/lowering of the Safety fence is performed by lowering/raising the ladder intended for access to the hopper or by a separate raising/lowering mechanism. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| Signal marking |  |  |  |
| 2.32. | Dimensional marking, signal lighting | The working (spreading/dispersal) area must be illuminated by at least one light. At least one orange LED beacon installed at the rear of the spreader. Two flashing LED lights (round lights) with a diameter of at least 180 mm, installed at the upper corners of the rear part of the spreader. The rear part marked with warning light-reflecting diagonal red and white stripes. Sample pattern of warning marking: Protection of LED beacons and flashing LED lights against solid objects and liquids must be at least IP65. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference - ________. |
| 2.33. | Mounting locations for road signs and road signs. | The rear part of the spreader (on the mounting/suspension unit/frame) shall be equipped with places for mounting warning/directional road signs or the necessary mounting structure, and no later than on the date of delivery of the Item, the following shall be installed: - a light-reflecting sign No 106 “Work“ not less than 1 (first) size: - Signal LED light pointer – a system of at least 13 lights complying with L8H, EN 12352 or an equivalent standard, fastened from the rear with screws or equivalent fasteners. The direction of the arrow indicating the mandatory side of the obstacle is changed from the operator's workplace using an electrical connection. A visual indicator informing about the activated direction of the arrow is mandatory at the operator's workplace. The feed voltage of the arrow lights is 24 V. The protection of the LED light system components against solid objects and liquids is not less than IP65. | Yes/No (delete as appropriate). |

* The Supplier must confirm compliance with the technical requirement by indicating: Yes/No (delete as appropriate) and, where required, enter the exact parameter/value of the proposed Item.

Where required, the Supplier must enter the name of the document proving the parameters and/or a reference to the source confirming the proposed parameters.

____________________                               _____________                                     _____________________

(position)	                       (signature)	    (forename, surname)

If the tender is signed by a person empowered by the Supplier, a document (power of attorney) granting the specified person the right to sign on behalf of the Supplier must be submitted together with the tender.

Technical Specification, Annex No. 2

DIMENSIONAL SKETCH (Procurement part 1)