RENTAL OF HYDROGEN-POWERED BUSES for the transport of passengers in urban conditions without drivers

TECHNICAL SPECIFICATION

- GENERAL

  - The object of the purchase is the rental of single-deck low-floor hydrogen-powered buses (vehicle code M3CE) for the carriage of passengers (hereinafter referred to as vehicles, buses) without a driver. The object of the purchase is not divided into parts.

  - The supplier must offer at least 4 (four) and no more than 10 (ten) buses. Minimum quantity offered – 4 pcs.

    - It should be noted that the Contracting Entity, having concluded the contract, undertakes to rent the entire number of buses offered by the Supplier, however, the settlement under the contract will be carried out according to the number of kilometers actually travelled, applying the rate of 1 (one) kilometer specified in the Supplier's offer. This is specified in more detail in the Treaty.

  - The rented buses will be operated in Vilnius:

    - Preliminary theoretical mileage of one technically sound bus in one calendar month, which corresponds to the average mileage of similar buses operated by the Contracting Authority in one calendar month by 6 000 km.

    - The minimum and maximum rental fee for one technically sound bus per calendar month shall be established and calculated in accordance with the procedure established in the Agreement.

  - The winner of the tender  must deliver and hand over at least 4 (four) buses to the Contracting Entity no later than by 10 November 2026 from the date of entry into force of the Contract, and the remaining proposed buses – no longer than within 180 (one hundred and eighty) calendar days from the date of entry into force of the Treaty. There is no possibility to extend the deadline for bus delivery. The bus delivery address is Justiniškių str. 14, Vilnius.

  - All offered buses must be of the same manufacturer, model and have the same equipment.

  - Rented buses must comply with the safety and environmental protection requirements set out in EU directives, regulations of the United Nations Economic Commission for Europe (hereinafter referred to as UNECE) and other requirements specified in the conditions.

  - Rented buses must be suitable for transporting passengers, not disassembled, damaged, or covered with advertising stickers that meet the requirements set for them in the technical conditions. Their technical condition must ensure the safe start of their operation immediately after delivery to the Contracting Entity, without any repair work being carried out.

  - The contracting entity will have the right to paint all buses red or cover them with red film at its own expense. The original color will not be restored after the end of the rental period, the film will not be peeled off.

  - The supplier will be obliged to carry out all actions related to the registration of vehicles at its own expense, including their registration in the State Register, indicating the Contracting Entity as the user.

  - The Supplier undertakes to ensure that the transferred buses have valid vehicle inspection cards of the Republic of Lithuania, the validity period of which must be at least 5 (five) months from the date of transfer of the buses. If the date of the next roadworthiness test is closer than 5 (five) months from the date of transfer of the buses, the Supplier undertakes to carry it out at its own expense and to provide the Contracting Entity with the roadworthiness test cards confirming this.

  - During the transfer of buses, the Contracting Entity, in its production base, checks the technical condition of the buses and assesses whether the vehicle is in order and complies with the requirements of the Technical Motor Vehicles and Their Trailers approved by Order 2BE-260 of the Director of the Lithuanian Transport Safety Administration of 20 October 2020. After ascertaining that there are no defects or that they have been eliminated within the time specified in the contract, the act of acceptance and transfer of the bus is signed.

  - The supplier must eliminate the deficiencies found during the inspection of the buses within 10 calendar days (or within the deadline specified by the Contracting Authority).

  - If the contracting entity expresses its wish, during the examination and evaluation of the tenders, the suppliers will be obliged to provide an opportunity for its representatives to inspect the buses offered for rent at their own expense and assess their technical, aesthetic condition and their compliance with the descriptions provided in the tenders.

  - During the period of validity of the lease agreement, the Contracting Entity carries out the operational maintenance of the rented buses at its own expense, ensures their technical condition and performs minor repairs in accordance with the instructions of the bus manufacturer (replenishment of operating fluids as required, replacement of naturally wearing parts). Operational maintenance does not include the repair of main units or other essential units, as well as of assemblies or components for the repair or maintenance of which special qualifications, certificates or certificates are required in accordance with the legislation or the manufacturer's requirements. Damages to vehicles or parts thereof, which have occurred during a traffic accident due to the fault of the Contracting Entity, shall be eliminated at the expense of the Contracting Entity.

  - During the period of validity of the Lease Agreement, the Supplier shall be obliged to carry out mandatory maintenance of the rented vehicles (hereinafter referred to as the Mandatory Maintenance) and repair of the main units  at its own expense.

    - Mandatory maintenance - which includes the maintenance of vehicles in accordance with the maintenance intervals specified in the manufacturer's regulation.

  - The following units are considered to be the main units: hydrogen fuel cell system, complete hydrogen system, gearbox, drive axle, battery system, complete electric part of the bus, internal combustion or electric motors, steering system, body structure or parts thereof, passenger information systems.

  - There must be at least one service authorised by the manufacturer of the proposed buses in the territory of the Republic of Lithuania. When submitting a tender, the Supplier must indicate all the proposed service places (providing their exact addresses) located at a distance of not more than 50 (fifty) kilometers from the production premises of the Contracting Entity, at the address Justiniškių str. 14, Vilnius. The distance is determined using the www.google.com/maps calculator of the website, evaluating the average of the shortest routes along the streets to and from the production premises (address) of the Contracting Entity to the service location (address) specified by the Supplier and back.

  - The contracting entity will install an e-ticket system in the rented buses at its own expense. After the dismantling of the e-ticket system, technical holes will remain in the passenger railings of the bus and in the driver's dashboard. Closed cabs can be installed as needed, if they are not provided, technical holes will remain after dismantling.

  - When transferring buses to the Contracting Entity, the supplier will be obliged to ensure the integration of Vilnius city routes into the passenger information system.

  - The contracting entity undertakes to insure all rented buses with compulsory insurance against civil liability in respect of the use of vehicles throughout the entire period of operation.

  - The Supplier, who has delivered the first vehicles under the Contract, shall train at least 9 (nine) bus driver instructors  of the Contracting Entity in accordance with the Safe and Economical Driving Programme. The dates and schedules of the training are coordinated with the Contracting Entity. If necessary, the Supplier shall provide the training participants with methodological training material.

  - At least 6 (six) Contracting Entity technicians (higher-qualified repair specialists) must be trained:

- use the spare parts catalogue and select spare parts properly;

- read diagrams of electrical, hydraulic, pneumatic and hydrogen systems;

- requirements for safe operation and daily maintenance.

Until the end of the vehicle rental period, the Supplier undertakes to organize additional training for a group of at least 6 (six) technicians. The date of the additional training is agreed upon by agreement of the parties (e-mail).

  - All trainings are conducted in Lithuanian.

  - The training must be organised at the premises of the Contracting  Entity not later than within 10 (ten) working days from the day of signing the first act of transfer-acceptance of the bus.

  - The Supplier issues certificates or certificates to employees who have successfully completed the training, confirming the acquired qualification and the right to train other specialists in the same field.

  - The tender documents must be accompanied by photographs of each bus offered:

- from the front right corner (the front and side of the bus are visible);

- from the front left corner (the front and side of the bus are visible);

- from the rear right corner (the back and side of the bus are visible);

- from the rear left corner (the back and side of the bus are visible);

- Interior photos: driver's workplace, dashboard (clearly visible motorcycle hours), front to rear view and rear to front.

- MANDATORY TECHNICAL REQUIREMENTS FOR THE OBJECT OF THE PROCUREMENT

The technical requirements for the procurement objects are specified in Annex 1 to the Technical Specification.

- ACCESSORIES

- Table of compliance of the goods with the requirements that have been defined.

Annex 1

TABLE OF COMPLIANCE OF GOODS WITH ESTABLISHED REQUIREMENTS

| No. | Name | Description | Vendor-specific matching parameters (to be filled in by the supplier) | Document title, page number and/or link to the website To justify the conformity of the product (to be filled in by the supplier) |
| --- | --- | --- | --- | --- |
| 1. | Bus type | 1.1. Two-axle low-floor single-decker bus powered by hydrogen (vehicle code M3CE); 1.2. Suitable for intensive operation in the city, including outdoor parking, in the conditions of the winter (up to -25ºC) and summer (up to +40ºC) seasons (the buses are intended to be operated in the city of Vilnius); 1.3. Certified in accordance with the requirements of the Directive (EU) of the European Parliament and of the Council. |  |  |
| 2. | Date of first registration | Not earlier than 2021. |  |  |
| 3. | Motorcycle hours | Not more than 6000. |  |  |
| 4. | Overall length | 4.1. From 11 500 mm to the maximum permissible according to the legislation of the Republic of Lithuania in force on the date of issue of the vehicle conformity certificate for this type of vehicle. |  |  |
| 5. | Width | From 2,490 mm to 2,550 mm. (measured in accordance with the requirements of ISO 612-1978 (or equivalent standard) clause 6.2). |  |  |
| 6. | Height | Maximum 3,460 mm. (measured in accordance with the requirements of clause 6.3 of the ISO 612-1978 standard) |  |  |
| 7. | Floor height | Low floor, the height of entry/exit from the road surface is not more than 350 mm. |  |  |
| 8. | Total capacity | 8.1. at least 25 seats for passengers; 8.2. at least 1 place for persons with disabilities (with special needs with a stroller) or for a stroller (with fastening belts, rails or other stroller fastening equipment) in the low-floor area between the axles of the bus; 8.3. the total number of installed seating and standing places is not less than 75, this number does not include places intended for persons with disabilities; |  |  |
| 9. | Passenger boarding doors | Pneumatic or electric doors located on the right side of the bus. The width of the first and second door openings shall be at least 1 200 mm. With anti-pinching protection (if an obstacle appears between the doors during closing, the door must open) and emergency door openings. An opportunity to make the door individually and all together. When the door is opened, the brake system must be activated, ensuring that the bus cannot move with at least one door open. There is a ramp for a wheelchair or a child stroller. |  |  |
| 10. | Number of axles | 2-axis. |  |  |
| 11. | Fuel | 11.1. Fuel - hydrogen H₂; 11.2. Fuel tanks: composite pressure cylinders connected to each other. Fuel must be sufficient for a mileage of at least 350 km; 11.3. Hydrogen system operating pressure – 350 bar (at +15 °C), depending on the type of system; 11.4. Refuelling time: no more than 20 minutes (protocol SAE J2601 and SAEJ2799); 11.5. Type of refuelling tap - N1, which is easily accessible to the operator during refuelling without additional lifts. Connector or connectors according to SAE J2600; 11.6. There must be safety valves, sensors, fuses; |  |  |
| 12. | Hydrogen Fuel Cell | 12.1. Hydrogen fuel cell - an electrochemical device that produces electricity using hydrogen and oxygen; 12.2. The hydrogen fuel cell is powered by hydrogen storage tanks (fuel tanks); 12.3. The power of the hydrogen fuel cell shall be at least 60 kW; 12.4. The hydrogen fuel cell shall operate at ambient temperatures between -20 ° and +40 ° without additional heating. |  |  |
| 13. | Suspension | Pneumatic with automatic height support, forced take-off and landing and right-hand knee function. |  |  |
| 14. | Steering system | System with booster, steering wheel on the left side. The steering wheel position is adjusted together with the driver's instrument cluster or separately. |  |  |
| 15. | Brakes | All brakes are disc type. The vehicle must be equipped with an electronic braking force control system EBS or an equivalent system; |  |  |
| 16. | Wheels & Tires | The tyres are adapted for use in urban conditions for public transport. Tyres must be suitable for all seasons (M+S). All tyres have a tread depth of at least 7 mm. |  |  |
| 17. | Scoreboards and passenger information systems | All buses offered must be equipped with light boards and passenger information systems. Must be with the possibility of programming. |  |  |

| Sec. No. | Bus manufacturer / name, model | Bus identification numbers (VIN) | Bus first registration dates | On the day of submission of motorcycle hour offers, | Bus colors | Bus values, excluding VAT |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | / |  |  |  |  |  |
| 2 | / |  |  |  |  |  |
| 3 | / |  |  |  |  |  |
| 4 | / |  |  |  |  |  |
| 5 | / |  |  |  |  |  |
| ... | / |  |  |  |  |  |