# TECHNICAL SPECIFICATION

# minibuses for the implementation of the project “Comprehensive Support Initiative for Ukrainian Children”

I. Lot I of the object of procurement “Minibuses”

| No. | Feature | Technical specification requirement |
| --- | --- | --- |
|  | Object of procurement | Minibuses, fitted for passenger transport Vehicle must comply with UNECE M2 vehicle category. * the proposed vehicle is considered to meet the procurement object if it meets the following requirements |
|  | Place of delivery of goods | Kyiv, Ukraine. * Exact delivery address will be provided for the supplier in the execution of the contract. |
|  | Delivery term | Not more than 8 months from the date of order placement |
|  | Vehicle‘s equipment | The vehicle must be equipped to be used in territory of Ukraine without additional measures. The gauges and read-outs of the instrument cluster must be in the metric system, and the vehicle must be designed for right-hand drive (left-hand steering wheel). |
|  | Vehicle‘s manufacturing | The vehicle must be new and unused (made no earlier than 2024 and with mileage of not more than 3000 km). |
|  | Preliminary quantity | Minimum quantity: 7 unit Maximum quantity: 10 units |
|  | Number of passengers (with driver) | Not less than 10 and not more than 12 standard passenger seats and 1 driver seat. |
|  | Gearbox type | Automatic gearbox |
|  | Drive of the axles | Front wheel drive (FWD) or All/4 -wheel drive (AWD/4WD) |
|  | Engine | Diesel engine |
|  | Emission standards | Not lower than EURO V |
|  | Requirements for the driver and passenger compartments | Driver and passenger compartment must have following functionalities and features: - Passenger compartment must have at least 4 side windows, 2 per side of the vehicle. - Passenger compartment must be connected with driver compartment (without partition between compartments). - All seats in the driver and passenger compartments must be equipped with seat belts. - Seats of the passenger compartment must have have at least 3 seats fitted with Isofix (ISO 13216) attachments for the child safety seats, with not less than 2 such seats located in the second row of seats. Additional seats with Isofix attachments will be accepted but are not mandatory. - Passenger compartment must have at least full width door/doors in the rear and sliding doors on the right side of the vehicle. - Rear doors of the passenger compartment must have a window/s. - Driver and passenger compartments must be equipped with heating and air conditioning system (single system with ventilation channels reaching the passenger compartment or separate systems, controlled from driver position). - Floor of the driver and passenger compartments must be covered by non-slip material. - The ceiling, walls and internal door surfaces in the passenger compartment (except for the surfaces that are not designed to be covered by the manufacturer) must be covered with suitable cabin finish material (generally used for passenger transport). - The ceiling and walls of the passenger compartment must be fitted with an acoustic and thermal insulation layer (between the exterior walls and the interior cladding panels). - Interior of the vehicle must be equipped with grab handles at the access door for easier boarding and moving through the interior. - Passenger compartment must be fitted with separate interior lighting system, controlled from the passenger compartment. |
|  | Requirements for the features of the vehicle | Vehicle must have following functionalities and features: - Vehicle must be equipped with fire extinguisher weighing not less than 2 kg. - Vehicle must be equipped with first aid kit. - Vehicle must be equipped with multimedia system with navigation, supporting Apple CarPlay and Android Auto. - Vehicle must be fitted with a set of rear parking sensors and/or rear-view camera (can be part of the vehicle’s equipment/ multimedia system or separately permanently fitted by the supplier). - Vehicle must be equipped with at least 2 USB charging ports in the driver’s compartment (ports can be additionally fitted by the supplier but should be permanently installed). - Vehicle must have central locking system with remote activation. - Vehicle must be fitted with digital tachograph system. - Vehicle must be fitted with speed limiting device (SLD) limiting the top speed of the vehicle to 100 km/h. - Rear axle of the vehicle must be of single wheel type. |
|  | Bodywork color | - Vehicles should be painted in light nondescript color. - Corrosion protection must be applied to the bottom, chassis and bodywork (can be applied from the factory or additionally coated by the supplier). |
|  | Tires | The vehicle must be: - Fitted with a set of summer/all season tyres, matching the dimensions, recommended by the manufacturer. - Vehicle must be provided with additional set of wheels (4 units), consisting of rims fitted with suitable winter tyres, marked with 3PMSF (Three-Peak Mountain Snowflake) marking, matching the dimensions, recommended by the manufacturer. - Tyres, fitted to the vehicle and to the additional wheel set must be manufactured not earlier than 2024. - Vehicle must be fitted with spare wheel. - Vehicle must be provided with Tool kit for replacing the wheel, including lifting device. |
|  | Warranty | The Seller or its authorized representative shall ensure the warranty servicing of the vehicle at the service center specified by the Supplier or its representative in the territory of Ukraine, and if there are no service centers of the Supplier or its representative in the specified territory, the vehicle shall be transported and returned by the Supplier at its own expense from the location of the Beneficiary's deployment to the Seller's or its representative's premises for service and maintenance. The vehicle shall be covered by a warranty of at least 24 months or 100 000 km whichever comes first. |
|  | Documentation to be supplied by the Supplier together with the Goods | Printed vehicle maintenance and user manual with servicing instructions in English or Ukrainian must be provided with the vehicle. |

II. Lot II of the object of procurement “Minibuses adapted for persons with disabilities”

| No. | Feature | Technical specification requirement |
| --- | --- | --- |
|  | Object of procurement | Minibuses, fitted for passenger transport and equipped with disabled access equipment (wheelchair lift and dedicated space for the passenger in a wheelchair). Vehicle must comply with UNECE M1 vehicle category. * the proposed vehicle is considered to meet the procurement object if it meets the following requirements |
|  | Place of delivery of goods | Kyiv, Ukraine. * Exact delivery address will be provided for the supplier in the execution of the contract. |
|  | Delivery term | Not more than 8 months from the date of order placement |
|  | Vehicle‘s equipment | The vehicle must be equipped to be used in territory of Ukraine without additional measures. The gauges and read-outs of the instrument cluster must be in the metric system, and the vehicle must be designed for right-hand drive (left-hand steering wheel). |
|  | Vehicle‘s manufacturing | The vehicle must be new and unused (made no earlier than 2024 and with mileage of not more than 3000 km). |
|  | Preliminary quantity | Minimum quantity: 6 unit Maximum quantity: 9 units |
|  | Number of passengers (with driver) | Not less than 6 and not more than 7 standard passenger seats, 1 driver seat and 1 space for passenger in a wheelchair. |
|  | Gearbox type | Automatic gearbox |
|  | Drive of the axles | Front wheel drive (FWD) or All/4 -wheel drive (AWD/4WD) |
|  | Engine | Diesel engine |
|  | Emission standards | Not lower than EURO V |
|  | Requirements for the driver and passenger compartments | Driver and passenger compartment must have following functionalities and features: - Passenger compartment must have at least 4 side windows, 2 per side of the vehicle. - Passenger compartment must be connected with driver compartment (without partition between compartments). - All seats in the driver and passenger compartments must be equipped with seat belts. - Seats of the passenger compartment must have have at least 3 seats fitted with Isofix (ISO 13216) attachments for the child safety seats, with not less than 2 such seats located in the second row of seats. Additional seats with Isofix attachments will be accepted but are not mandatory. - Passenger compartment must have at least full width door/doors in the rear and sliding doors on the right side of the vehicle. - Rear doors of the passenger compartment must have a window/s. - Driver and passenger compartments must be equipped with heating and air conditioning system (single system with ventilation channels reaching the passenger compartment or separate systems, controlled from driver position). - Floor of the driver and passenger compartments must be covered by non-slip material. - The ceiling, walls and internal door surfaces in the passenger compartment (except for the surfaces that are not designed to be covered by the manufacturer) must be covered with suitable cabin finish material (generally used for passenger transport). - The ceiling and walls of the passenger compartment must be fitted with an acoustic and thermal insulation layer (between the exterior walls and the interior cladding panels). - Interior of the vehicle must be equipped with grab handles at the access door for easier boarding and moving through the interior. - Passenger compartment must be fitted with separate interior lighting system, controlled from the passenger compartment. |
|  | Requirements for the features of the vehicle | Vehicle must have following functionalities and features: - Vehicle must be equipped with fire extinguisher weighing not less than 2 kg. - Vehicle must be equipped with first aid kit. - Vehicle must be equipped with multimedia system with navigation, supporting Apple CarPlay and Android Auto. - Vehicle must be fitted with a set of rear parking sensors and/or rear-view camera (can be part of the vehicle’s equipment/ multimedia system or separately permanently fitted by the supplier). - Vehicle must be equipped with at least 2 USB charging ports in the driver’s compartment (ports can be additionally fitted by the supplier but should be permanently installed). - Vehicle must have central locking system with remote activation. |
|  | Requirement for the disabled access and safety systems | Vehicle must have following disabled person access and safety systems fitted: - Vehicle must be equipped with rear mounted electric or electromechanical wheelchair lift. - Wheelchair lift must have a lifting capacity of not less than 300 kg. - Wheelchair lift must reach the level of the road surface and provide easy access to the lift for wheelchair from that level. - Wheelchair lift when not in use must lift and fold into the interior of the vehicle and not protrude outside of the rear door. - Wheelchair lift must be controlled with wired or wireless remote control, allowing the control of the lift from outside of the vehicle. - Vehicle must be equipped with wheelchair restrain system – 4 wheelchair tiedowns with retractable tie down straps. - Vehicle must be fitted with 3-point safety belt, suitable for passenger traveling in the wheelchair. - Wheelchair tie-downs and safety belt system must be certified and comply with the ISO 10542 or equivalent passenger safety standard. |
|  | Bodywork color | - Vehicles should be painted in light nondescript color. - Corrosion protection must be applied to the bottom, chassis and bodywork (can be applied from the factory or additionally coated by the supplier). |
|  | Tires | The vehicle must be: - Fitted with a set of summer/all season tyres, matching the dimensions, recommended by the manufacturer. - Vehicle must be provided with additional set of wheels (4 units), consisting of rims fitted with suitable winter tyres, marked with 3PMSF (Three-Peak Mountain Snowflake) marking, matching the dimensions, recommended by the manufacturer. - Tyres, fitted to the vehicle and to the additional wheel set must be manufactured not earlier than 2024. - Vehicle must be fitted with spare wheel. - Vehicle must be provided with Tool kit for replacing the wheel, including lifting device. |
|  | Warranty | The Seller or its authorized representative shall ensure the warranty servicing of the vehicle at the service center specified by the Supplier or its representative in the territory of Ukraine, and if there are no service centers of the Supplier or its representative in the specified territory, the vehicle shall be transported and returned by the Supplier at its own expense from the location of the Beneficiary's deployment to the Seller's or its representative's premises for service and maintenance. The vehicle shall be covered by a warranty of at least 24 months or 100 000 km whichever comes first. |
|  | Documentation to be supplied by the Supplier together with the Goods | Printed vehicle maintenance and user manual with servicing instructions in English or Ukrainian must be provided with the vehicle. Printed instruction manual/s for the use of the wheelchair lift and wheelchair restrain system must be provided with the vehicle. |

NOTES:

Where the technical specification indicates a specific model or source of supply, a specific process specific to the goods or services supplied by a particular supplier, or a trademark, patent, types, specific origin or manufacture, it shall be understood that these characteristics are indicative only and that the suppliers are free to offer an equivalent option. The supplier must demonstrate the equivalence referred to in this paragraph.

In the event that the technical specification specifies values/parameters for a particular technical characteristic, suppliers may offer goods whose values/parameters are not inferior to those specified (goods with no inferior characteristics may be offered)

This is the minimum quantity that the Contracting Authority undertakes to purchase.

The Contracting Authority does not undertake to purchase the entire quantity. The Contracting Authority intends to purchase as many goods as can be acquired, based on the unit price (rate) offered by the supplier, without exceeding the maximum amount allocated for the procurement (EUR 500,000 including VAT), but in any case, no more than 10 units.

This is the minimum quantity that the Contracting Authority undertakes to purchase.

The Contracting Authority does not undertake to purchase the entire quantity. The Contracting Authority intends to purchase as many goods as can be acquired, based on the unit price (rate) offered by the supplier, without exceeding the maximum amount allocated for the procurement (EUR 497,000 including VAT), but in any case, no more than 9 units.