TECHNINĖ SPECIFIKACIJA

  - Duomenys – pirminiai ir apdoroti diagnostikos duomenys, susiję su konkrečiu paleidimu.

  - ILI – vidinė dujotiekio diagnostika.

  - Įranga – bet kokie į vamzdyną įleidžiami valymo, pralaidumo tikrinimo, diagnostikos, sekimo, stūmimo, įstrigusios įrangos pajudinimo ar išėmimo įtaisai ir jų dalys (įskaitant valymo stūmoklius, kalibravimo įtaisus, geometrinius įtaisus, diagnostikos įrenginius, sekimo siųstuvus, sandarinimo elementus ir kitas analogiškas priemones).

  - Neatitikimas – bet koks faktinių vamzdyno sąlygų ar parametrų neatitikimas Tiekėjo parinktai įrangai arba darbų programai.

  - Pirkėjas, Užsakovas – AB „Amber Grid“.

  - Sutartis – sutartis, sudaroma tarp Tiekėjo ir Pirkėjo dėl Pirkimo objekto.

  - Tiekėjas – ūkio subjektas – fizinis asmuo, privatus juridinis asmuo, viešasis juridinis asmuo, kitos organizacijos ir jų padaliniai ar tokių asmenų grupė, su kuriuo Pirkėjas, Užsakovas sudaro Sutartį.

  - Tiekėjo darbų programa – Tiekėjo parengtas rašytinis paleidimo ir vykdymo planas (slėgio, srauto režimai, leidžiami ribiniai dydžiai, veiksmų seka, sustabdymo sąlygos).

  - Tiesioginės išlaidos – išlaidos, kurios būtinos įrangos įstrigimo ar sugadinimo padariniams pašalinti (kasimo darbai, vamzdžio atvėrimas, pjovimas, suvirinimas, naujos atkarpos įvirinimas, bandymai, defektoskopija, rangovų mobilizacija, atstatymo darbai, dokumentavimas).

  - Vamzdynas (MD) – Pirkėjui priklausantis magistralinis dujotiekis, kuriame vykdomi valymo, pralaidumo tikrinimo ir vidinės diagnostikos darbai.

- PIRKIMO OBJEKTAS

AB „Amber Grid“ priklausančio magistralinio dujotiekio (toliau – MD) valymo ir diagnostikos paslaugos.

| REIKALAVIMAI PIRKIMO OBJEKTUI |  |
| --- | --- |
| - Bendri reikalavimai |  |
|  | Pirkėjas, dalyvaujant Tiekėjo specialistams ir jiems teikiant technines konsultacijas, atliks dujotiekio vamzdyno vidinės diagnostikos įrangos paleidimą, įstatant ją į paleidimo kamerą, kontroliuos jos judėjimo eigą vamzdynu ir išims ją iš priėmimo kameros. |
|  | Tiekėjas privalo atlikti visus būtinus parengiamuosius ir analitinius darbus, susijusius su įrangos paleidimu, kaip aprašyta API 1163 (naujausia versija) „In-Line Inspection Systems Qualification“ standarte. |
|  | Tiekėjas yra atsakingas už vidinės diagnostikos paslaugos kokybę, užtikrindamas atitiktį POF 300 „Sėkmingos vidinės diagnostikos užtikrinimas“ ir API 1163 standartų reikalavimams. |
|  | Tiekėjas turi registruoti ir išmatuoti 3 matmenų vamzdynų koordinates, naudojant inercinės navigacijos bloką. XYZ turi būti įdiegta į tikrinimo įrankį (Caliper arba MFL), kad būtų išvengta papildomų bandymų. Atskaitos taškų (markerių, vožtuvų ir pan.) DGPS (Diferencinė globalinė padėties nustatymo sistema) koordinates pateiks Pirkėjas. |
|  | Ataskaitose visų objektų, sujungimų ir anomalijų koordinatės turi būti pateiktos LKS94 (Lietuvos koordinačių sistemos) formatu. Pirkėjas taip pat pateiks koordinates LKS94 formatu. |
|  | Tiekėjas privalo turėti visą reikalingą įrangą, skirtą ILI įrenginio judėjimo sekimui vamzdyne, įskaitant padėties sekimo įrankius (angl. tracking tools, locators), automatinio įrangos judėjimo fiksavimo sistemas (AGM) ir siųstuvo–imtuvų komplektus, užtikrinančius galimybę nustatyti įrenginio buvimo vietą vamzdyne viso jo judėjimo metu, taip pat surasti užstrigusį įrenginį. |
| - Reikalavimai vamzdynų valymui ir parengties diagnostikos darbams įvertinimui |  |
|  | Vamzdynų valymo darbai turi būti vykdomi vadovaujantis POF 300 „Sėkmingos vidinės diagnostikos užtikrinimas“ dokumento rekomendacijomis. |
|  | Po kiekvieno valomojo stūmoklio praleidimo vamzdynu, Tiekėjas turi parengti ataskaitą, kurioje nurodomi kalibravimo plokštės pažeidimai (jei yra) ir pateikiamos išvados dėl vamzdyno praeinamumo bei vamzdyno išvalymo būklės po praleidimo. Galutinį sprendimą dėl to, ar vamzdynas yra paruoštas diagnostikai, turėtų priimti Tiekėjas. |
| - Reikalavimai vamzdyno geometrinių ypatybių nustatymui |  |
|  | Geometrinis patikrinimas atliekamas siekiant nustatyti vamzdyno įrengimo anomalijas, trečiųjų šalių padarytą žalą, vamzdžius su padidėjusiu ovalumu, įlinkimus, raukšles, deformacijas ir kitus pažeidimus, aprašytus gerosios praktikos ir rekomendacijų dokumente POF 100 („Pipeline Operators Forum Recommended Practice for In-Line Inspection“) (naujausia versija) ir standarte API 1163 (naujausia versija) „In-Line Inspection Systems Qualification“. Tiekėjas privalo nustatyti geometrinių defektų vietas įvardindamas koordinates. |
|  | Vamzdžiai, kurių ovalumas didesnis nei 2% turėtų būti registruojami ir matuojami. Šios ovalumo anomalijos turi būti vertinamos pagal POF 100 dokumentą. |
|  | Įlinkimai, raukšlės, įspaudimai ir kiti geometriniai pažeidimai ar deformacijos, viršijančios 1%, turi būti registruojami, išmatuojami ir įvertinami pagal POF 100 dokumentą. |
|  | Jeigu įlinkimas yra susijęs su metalo praradimo anomalija, jis turi būti registruojamas kaip sudėtinis įlinkimas ir pažymimas, pvz., DEML (angl. Dent with Metal Loss). |
| - Reikalavimai metalo praradimo požymiams aptikti |  |
|  | Vamzdžiai, kurių nominalus sienelės storis mažesnis, nei dokumentacijoje nurodytas dydis (neatitinka nustatytų kriterijų), taip pat vamzdžiai, pasižymintys lokaliu suplonėjimu, kurių gylis viršija ribinius gamykloje nustatytus leistinus nukrypimus, turi būti nustatyti. |
|  | Visi lokalūs suplonėjimai turi būti klasifikuojami ir dokumentuojami pagal POF 100 dokumentą, t. y. priskiriami vienai iš 7 rūšių: pitingas, taškinė korozija, bendroji korozija, išilginis ir skersinis griovelis, išilginis ar skersinis įtrūkimo defektas. Galima supaprastinti iki 3 ÷ 5 suplonėjimo tipų, tačiau pirmenybė teikiama maksimalaus detalumo klasifikacijos variantui (su specifikacijomis, skirtomis aptikti 90 % tikslumu). |
|  | Kiekvienos aptiktos ir išmatuotos suplonėjimo anomalijos parametras (pvz. ilgis, plotis, gylis, padėtis, pradžios taškas) turi būti nurodytas pagal POF 100 dokumentą. |
| - Reikalavimai įtrūkimams ir į įtrūkimus panašių darinių aptikimui |  |
|  | Turi būti nustatyti ir užregistruoti įtrūkimai arba į įtrūkimus panašios anomalijos vamzdžių korpuse arba išilginių, spiralinių ar skersinių suvirinimo siūlėse. |
|  | Įtrūkimai arba į įtrūkimus panašios anomalijos turi būti nustatomos pagal visas padėtis (kampus), net jei jų kampas tiksliai sutampa su išilgine vamzdžio ašimi (t. y. lygus 0 laipsnių). |
|  | Įtrūkimų kolonija nurodoma pagal giliausio atskiro įtrūkimo kolonijoje ilgį, plotį ir gylį pagal POF 100 dokumentą. |
| - Reikalavimai suvirinimo siūlių ypatybių aptikimui |  |
|  | Išilginių, spiralinių arba žiedinių suvirinimo siūlių suvirinimo anomalijos (defektai) turi būti identifikuoti ir užregistruoti. |
| - Reikalavimai gamybos ir montavimo metu paliktų ypatybių aptikimui |  |
|  | Gamybos metu atsiradusios anomalijos turi būti aprašytos pagal POF 100 dokumento reikalavimus (pvz., išsisluoksniavimai, atskilimai, antikūniai, paviršiaus iškilos ir pan.). |
| - Reikalavimai aptikimo tikimybei ir dydžio nustatymui |  |
|  | Aptikimo tikimybė (toliau POD, angl. Probability of Detection), t. y. tikimybė, kad tam tikro dydžio defektas bus aptiktas ILI įranga, turi atitikti 80/90/95 lygį (POD 80/90/95). |
|  | Anomalijų, esančių vamzdžio sienelėje arba šalia / ant išilginio suvirinimo siūlės, aptikimo ir dydžio nustatymo tikslumas turi būti toks: |

|  | Bendras metalo praradimas | Taškinė korozija | Išilginis griovelis | Žiedinis griovelis |
| --- | --- | --- | --- | --- |
| Gylio riba esant POD=90% | 0.10 t | 0.10 t | 0.10 t | 0.10 t |
| Gylio matavimo tikslumas 80% patikimumu | ±0.10 t | ±0.10 t | ±0.10 t | ±0.10 t |
| Pločio matavimo tikslumas 80% patikimumu | ±12 mm | ±9 mm | ±12 mm | ±12 mm |
| Ilgio matavimo tikslumas 80% patikimumu | ±12 mm | ±9 mm | ±10 mm | ±10 mm |
| Gylio matavimo tikslumas 90% patikimumu | ±0.13 t | ±0.13 t | ±0.13 t | ±0.13 t |
| Pločio matavimo tikslumas 90% patikimumu | ±15 mm | ±12 mm | ±16 mm | ±15 mm |
| Ilgio matavimo tikslumas 90% patikimumu | ±15 mm | ±12 mm | ±13 mm | ±13 mm |

|  | Anomalijų, esančių žiediniame suvirinimo siūle arba terminio poveikio zonoje (TPZ), aptikimo ir dydžio nustatymo tikslumas turi būti toks: |
| --- | --- |

|  | Bendras metalo praradimas | Taškinė korozija | Išilginis griovelis | Žiedinis griovelis |
| --- | --- | --- | --- | --- |
| Gylio riba esant POD=90% | 0.10 t | 0.15 t | 0.15 t | 0.15 t |
| Gylio matavimo tikslumas 80% patikimumu | ±0.15 t | ±0.15 t | ±0.15 t | ±0.15 t |
| Pločio matavimo tikslumas 80% patikimumu | ±22 mm | ±19 mm | ±22 mm | ±22 mm |
| Ilgio matavimo tikslumas 80% patikimumu | ±25 mm | ±20 mm | ±20 mm | ±20 mm |

|  | Įtrūkimai arba į įtrūkimus panašios anomalijos aptikimo ir dydžio nustatymo tikslumas turi būti: |
| --- | --- |

|  | Išilginis plyšys vamzdžio sienelėje/suvirinimo siūlėje | Išilginis įtrūkimas klasteris vamzdžio sienelėje | Žiedinis įtrūkimas vamzdžio sienelėje/ suvirinimo siūlėje | Spiralinis įtrūkimas vamzdžio sienelėje/ suvirinimo siūlėje |
| --- | --- | --- | --- | --- |
| Plyšio gylis esant POD=90% su ilgiu L=25 mm | 0.20 t | 0.20 t | 0.20 t | 0.20 t |
| Gylio matavimo tikslumas 80% patikimumu | ±0.20 t | ±0.20 t | ±0.20 t | ±0.20 t |
| Ilgio matavimo tikslumas 80% patikimumu | ±10 mm | ±30 mm | ±10 mm | ±10 mm |

|  | Sienelės storio matavimo paklaida turi būti ±1 mm arba ±0.10t priklausomai nuo to, kuri reikšmė yra didesnė, esant 80 % patikimumui. |
| --- | --- |
| - Sutarties vykdymo metu pateikiama dokumentacija |  |
|  | Tiekėjas privalo Pirkėjui pateikti preliminarią ataskaitą per 30 kalendorinių dienų nuo ILI diagnostikos atlikimo. Galutinė ataskaita turi būti pateikta per 20 kalendorinių dienų nuo to momento, kai Pirkėjas pateikė pastabas dėl preliminarios ataskaitos ir (jei taikoma) atliko vamzdyno patikrinimus atsikasant. |
|  | Galutinė ataskaita turi atitikti dokumentacijai, kuri priimta Europos vamzdynų transporto operatorių asociacijos (POF), vadovaujantis Sutarties sudarymo dieną galiojančia versija – POF „Specifikacijos ir reikalavimai vamzdynų vidinei diagnostikai“. |
|  | Visą dokumentaciją, įskaitant kvalifikacijos sertifikatus, techninę dokumentaciją ir ataskaitas, Tiekėjas turi pateikti anglų arba lietuvių kalba kartu su galutiniu pasiūlymu. |
|  | Kartu su ataskaitomis, Tiekėjas Pirkėjui pateikia apdorotus duomenis iš ILI įrenginių ir programinės įrangos su technine pagalba defektų vizualizacijai. |
| PASLAUGŲ ORGANIZAVIMAS |  |
| - Tiekėjo pareigos |  |
|  | Tiekėjas yra visiškai atsakingas už diagnostikai naudojamos įrangos parinkimą ir parengimą, atsižvelgiant į dujotiekio specifiką (pvz., konstrukcinius elementus, skirtingus vidinius diametrus, vamzdžių sienelių šiurkštumą, vidinius žiedus ir pan.). |
|  | Jei diagnostikos metu Tiekėjo įranga buvo sugadinta, Tiekėjas privalo ją suremontuoti arba, jei remontas nėra įmanomas, pristatyti naują įrangą. Nepaisant įprasto nusidėvėjimo, Pirkėjas atlygina už sugadintą įrangą tik tuo atveju, jei nustatoma, kad įranga sugadinta dėl Pirkėjo kaltės. Pareiga Pirkėjo kaltę įrodyti tenka Tiekėjui. |

# TECHNICAL SPECIFICATION

  - Data – raw and processed diagnostic data related to a specific run.

  - ILI – in-line inspection.

  - Equipment – any devices and parts thereof that are inserted into the pipeline for cleaning, flow testing, diagnostics, monitoring, pushing, moving or removing stuck equipment (including cleaning pistons, calibration devices, geometric devices, diagnostic devices, tracking transmitters, sealing elements, and other similar devices).

  - Non-conformity – any discrepancy between the actual conditions or parameters of the pipeline and the equipment or work program selected by the Supplier.

  - The Buyer, the Customer – AB “Amber Grid”.

  - Agreement – an agreement concluded between the Supplier and the Buyer regarding the Procurement object.

  - Supplier – an economic entity – a natural person, private legal entity, public legal entity, other organizations and their divisions, or a group of such persons with whom the Buyer or Customer concludes the Agreement.

  - Supplier work program – A written start-up and operation plan prepared by the Supplier (pressure, flow modes, permissible limit values, sequence of actions, shutdown conditions).

  - Direct costs – costs necessary to eliminate the consequences of equipment jamming or damage (excavation works, pipe opening, cutting, welding, welding of a new section, testing, defectoscopy, contractor mobilization, restoration works, documentation).

  - Pipeline (MGP) – A main gas pipeline owned by the Buyer, in which cleaning, flow testing, and internal diagnostics are performed.

# PROCUREMENT OBJECT

Cleaning and diagnostic services for the main gas pipeline (hereinafter - MGP) owned by AB “Amber Grid”.

| REQUIREMENTS FOR THE PROCUREMENT OBJECT |  |
| --- | --- |
| - General requirements |  |
| 2.1. | The Buyer, with the participation of the Supplier's specialists and their technical advice, will start up the internal diagnostic equipment for the gas pipeline, placing it in the start-up chamber, control its movement through the pipeline, and remove it from the receiving chamber. |
| 2.2. | The Supplier must perform all necessary preparatory and analytical work related to the commissioning of the equipment, as described in API 1163 (latest version) "In-Line Inspection Systems Qualification" standard. |
|  | The Supplier is responsible for the quality of the internal diagnostics service, ensuring compliance with the requirements of POF 300 "Ensuring Successful Internal Diagnostics" and API 1163 standards. |
|  | The Supplier must record and measure the coordinates of the 3-dimensional pipelines using an inertial navigation unit. XYZ must be installed in the inspection tool (Caliper or MFL) to avoid additional testing. Reference points (markers, valves, etc.) DGPS (Differential Global Positioning System) coordinates will be provided by the Buyer. |
|  | The coordinates of all objects, connections, and anomalies must be presented in LKS94 (Lithuanian coordinate system) format in the reports. The Buyer will also provide coordinates in LKS94 format. |
|  | The Supplier must have all the necessary equipment for tracking the movement of the ILI device in the pipeline, including tracking tools (locators), automatic equipment movement recording systems (AGM), and transmitter-receiver sets that ensure the ability to determine the location of the device in the pipeline throughout its movement, as well as to locate a stuck device. |
| - Requirements for pipeline cleaning and assessment of readiness for diagnostic work |  |
| 3.1. | Pipeline cleaning work must be carried out in accordance with the recommendations of POF 300 "Ensuring Successful Internal Diagnostics". |
| 3.2. | After each cleaning tool passes through the pipeline, the Supplier must prepare a report indicating any damage to the calibration plate (if any) and providing conclusions on the pipeline's passability and the condition of the pipeline after cleaning. The final decision on whether the pipeline is ready for diagnostics should be made by the Supplier. |
| - Requirements for determining the geometric characteristics of the pipeline |  |
| 4.1. | Geometric inspection is performed to identify pipeline installation anomalies, damage caused by third parties, pipes with increased ovality, bends, wrinkles, deformations, and other defects described in the POF 100 (Pipeline Operators Forum Recommended Practice for In-Line Inspection) (latest version) and standard API 1163 (latest version) In-Line Inspection Systems Qualification. The Supplier must identify the locations of geometric defects by specifying their coordinates. |
| 4.2. | Pipes with ovality greater than 2% should be recorded and measured. These ovality anomalies must be assessed in accordance with document POF 100. |
|  | Dents, creases, indentations, and other geometric defects or deformations exceeding 1% must be recorded, measured, and evaluated in accordance with POF 100. |
|  | If the dent is associated with metal loss, it must be recorded as a composite dent and marked, e.g., DEML (Dent with Metal Loss). |
| - Requirements for detecting signs of metal loss |  |
| 5.1. | Pipes with a nominal wall thickness less than that specified in the documentation (not meeting the specified criteria), as well as pipes with local thinning exceeding the permissible deviations specified by the manufacturer, must be identified. |
| 5.2. | All local thinning must be classified and documented in accordance with POF 100, i.e. assigned to one of seven types: pitting, spot corrosion, general corrosion, longitudinal and transverse grooves, longitudinal or transverse crack defects. It is possible to simplify this to 3 to 5 types of thinning, but preference is given to the most detailed classification option (with specifications designed to detect 90% of defects). |
|  | The parameters of each detected and measured thinning anomaly (e.g., length, width, depth, position, starting point) must be specified in accordance with document POF 100. |
| - Requirements for the detection of cracks and crack-like formations |  |
| 6.1. | Cracks or crack-like anomalies in the pipe body or longitudinal, spiral, or transverse welds must be detected and recorded. |
| 6.2. | Cracks or crack-like anomalies shall be detected in all positions (angles), even if their angle coincides exactly with the longitudinal axis of the pipe (i.e., equal to 0 degrees). |
|  | The crack colony is specified according to the length, width, and depth of the deepest individual crack in the colony in accordance with POF 100. |
| - Requirements for the detection of weld seam characteristics |  |
| 7.1. | Welding anomalies (defects) in longitudinal, spiral, or ring welds must be identified and recorded. |
| - Requirements for detecting features left during production and installation |  |
| 8.1. | Any anomalies that arise during production must be described in accordance with the requirements of document POF 100 (e.g., delamination, separation, inclusions, surface protrusions, etc.). |
| - Requirements for detection probability and size determination |  |
| 9.1. | The probability of detection (POD), i.e., the probability that a defect of a certain size will be detected by ILI equipment, must meet the 80/90/95 level (POD 80/90/95). |
| 9.2. | The accuracy of detecting and determining the size of anomalies in the pipe wall or near/on the longitudinal weld seam shall be as follows: |

|  | Total metal loss | Pitting corrosion | Longitudinal groove | Annular groove |
| --- | --- | --- | --- | --- |
| Depth limit at POD=90% | 0.10 t | 0.10 t | 0.10 t | 0.10 t |
| Depth measurement accuracy with 80% reliability | ±0.10 t | ±0.10 t | ±0.10 t | ±0.10 t |
| Width measurement accuracy with 80% reliability | ±12 mm | ±9 mm | ±12 mm | ±12 mm |
| Length measurement accuracy with 80% reliability | ±12 mm | ±9 mm | ±10 mm | ±10 mm |
| Depth measurement accuracy with 90% reliability | ±0.13 t | ±0.13 t | ±0.13 t | ±0.13 t |
| Width measurement accuracy with 90% reliability | ±15 mm | ±12 mm | ±16 mm | ±15 mm |
| Length measurement accuracy with 90% reliability | ±15 mm | ±12 mm | ±13 mm | ±13 mm |

|  | The accuracy of detecting and determining the size of anomalies in the circumferential weld or heat-affected zone (HAZ) shall be as follows: |
| --- | --- |

|  | Total metal loss | Pitting corrosion | Longitudinal groove | Annular groove |
| --- | --- | --- | --- | --- |
| Depth limit at POD=90% | 0.10 t | 0.15 t | 0.15 t | 0.15 t |
| Depth measurement accuracy with 80% reliability | ±0.15 t | ±0.15 t | ±0.15 t | ±0.15 t |
| Width measurement accuracy with 80% reliability | ±22 mm | ±19 mm | ±22 mm | ±22 mm |
| Length measurement accuracy with 80% reliability | ±25 mm | ±20 mm | ±20 mm | ±20 mm |

|  | The accuracy of detecting and measuring cracks or crack-like anomalies shall be: |
| --- | --- |

|  | Longitudinal crack in pipe wall/weld | Longitudinal crack cluster in pipe wall | Annular crack in pipe wall/weld seam | Spiral crack in pipe wall/weld seam |
| --- | --- | --- | --- | --- |
| Crack depth at POD=90% with length L=25 mm | 0.20 t | 0.20 t | 0.20 t | 0.20 t |
| Depth measurement accuracy with 80% reliability | ±0.20 t | ±0.20 t | ±0.20 t | ±0.20 t |
| Length measurement accuracy with 80% reliability | ±10 mm | ±30 mm | ±10 mm | ±10 mm |

|  | The wall thickness measurement tolerance shall be ±1 mm or ±0.10t, whichever is greater, at 80% confidence. |
| --- | --- |
| - Documentation to be provided during the performance of the Agreement |  |
| 10.1. | The Supplier shall submit a preliminary report to the Buyer within 30 calendar days of performing the ILI diagnostics. The final report shall be submitted within 20 calendar days of the Buyer submitting comments on the preliminary report and (if applicable) performing pipeline inspections by digging. |
| 10.2. | The final report must comply with the documentation adopted by the European Pipeline Transport Operators Association (POF) in accordance with the version valid on the date of conclusion of the Agreement – POF "Specifications and Requirements for Internal Pipeline Diagnostics". |
|  | All documentation, including qualification certificates, technical documentation, and reports, must be submitted by the Supplier in English or Lithuanian together with the final tender. |
|  | Together with the reports, the Supplier shall provide the Buyer with processed data from ILI equipment and software with technical assistance for defect visualization. |
| ORGANIZATION OF THE PROVISION OF SERVICES |  |
| - Obligations of the Supplier |  |
| 11.1. | The Supplier shall be fully responsible for the selection and preparation of the equipment used for diagnostics, taking into account the specific characteristics of the gas pipeline (e.g., structural elements, different internal diameters, pipe wall roughness, internal rings, etc.). |
| 11.2. | If the Supplier's equipment was damaged during diagnosis, the Supplier must repair it or, if repair is not possible, deliver new equipment. Notwithstanding normal wear and tear, the Buyer shall only compensate for damaged equipment if it is determined that the equipment was damaged due to the Buyer's fault. The burden of proving the Buyer's fault lies with the Supplier. |