Consulting services on development and deployment of

a new Required Navigation Performance (RNP) related Lithuanian airspace structure and efficiency of air traffic management services

TECHNICAL SPECIFICATION

# I. General information

Contracting Authority: public limited liability company Oro Navigacija (hereinafter referred to as the ON or Oro Navigacija), the sole provider of air navigation services, comprising aeronautical information services (AIS), air traffic services (ATS), airspace management services (ASM), air traffic flow management (ATFM), flight procedure design (FPD) also communications, navigation and surveillance (CNS) services at Vilnius flight information region (FIR). Also apron management services and single common information service provider of the U-space airspaces. Address: Balio Karvelio St. 25, LT-02184 Vilnius, Lithuania, VAT payer code LT100604610. Data collected and stored at the Register of Legal Entities, code of the company 210060460.

# II. Object of the contract

## Consulting services shall be designed to deliver clear, actionable outcomes and are structured to provide a logical progression: from the assessment of the current situation, through the development of future scenarios over the next 10 years, to the identification of implementation gaps and definition of a practical, phased roadmap. To ensure strategic viability and regulatory alignment, Suppliers recommendations shall be based on the conclusions of cost-benefit analysis, risk assessment, and compliance with European frameworks (SESAR, CP1, CP2, SES2+) shall be integrated into the core of the methodology. The result of consulting services shall encompass four work packages (WP):

WP1 - Understanding of the current operations of Oro Navigacija.

WP2 - Defining future goals/AIRSPACE AND ATM CONOPS of Oro Navigacija.

WP3 - GAP identification map.

## WP4 - Operational evolution plan.

Below is the detailed content of each WP.

1. WP1 - Understanding of the current operations of Oro Navigacija.

WP1 shall analyse the baseline of Oro Navigacija by carrying out a strategic review of current ATS provision and organisational structure in the wider European ATS context. It includes an assessment of air traffic management, including RNAV and RNP procedures that are in place and airspace structure, legal and regulatory obligations, and workforce composition.

The goal of this WP shall include but not be limited to building a comprehensive and objective understanding of the current state of air traffic management in Lithuanian airspace, the technical and institutional features thereof. Through documentation analysis, ATM/ANS sector observations, and stakeholder input, it shall map the organizational, technical, regulatory, and operational baseline upon which the rest of the assessment is built.

This part is intended to establish a comprehensive understanding of the key aspects of the implementation plan and to identify any potential issues and risks by conducting thorough internal and external factor assessment.

Review aims to analyse the following key areas in order to determine the most effective air traffic service provision methods in Lithuanian airspace and later define the applicable actions according to the implementation plan.

1.1. Internal factor assessment consists of four key areas:

1.1.1. Flight procedures and airspace structure

The Supplier shall review existing airspace structure and current air traffic operations and procedures in entire Vilnius FIR in cooperation with internal experts of Oro Navigacija to identify potential structural inefficiencies and bottlenecks. The Supplier shall collect and analyse all relevant valid documentation (might require translation from Lithuanian language, translation costs must be included in the supplier's offer), data sources, and previous assessments.

Desktop review of published instrument flight procedures for PBN navigation specifications (departure and arrival procedures at EYVI, EYKA and EYPA airports) in Lithuanian airspace shall be conducted and  optimisation opportunities shall be indentified, considering forecasted traffic levels, and the needs of airspace users.

A structured safety assessment of current agreement regarding hot air balloon operations within Vilnius Area of Responsibility (AoR) and military activities (e.g., firing ranges) at Vilnius and Kaunas TMA/CTR shall be conducted in accordance with  ICAO Annex 19, Regulation (EU) 2017/373, Lithuanian Airspace Organization Rules and the Oro Navigacija Safety Management Manual (SMM).. Analysis of different potential solutions shall be conducted based on current and forecasted traffic flows, safety criteria, and equitable access principles. Structured safety assessment of military aircraft activities within Vilnius and Kaunas TMA/CTR in accordance with GAT rules shall be conducted. Information on the previous assessments and stakeholder engagement will be provided by the ON.

1.1.2. Air Traffic Service (ATS) provision

Review of the organization of the airspace and all information on how the ATS is provided shall be conducted. The Supplier shall map how air traffic services are currently delivered and identify organizational models and interdependencies.

ATC personnel structure and shift organization assessment including staffing needs assessment and rostering analysis shall be conducted: the Supplier shall analyse the staffing model, positions and shift systems (rostering) for their adequacy in delivering safe and efficient 24/7 services. The Supplier shall assess the impact of current Lithuanian airspace design and identify potential optimisation opportunities.

1.1.3. CNS

The institutional Communications, Navigation and Surveillance (CNS) field framework shall be analyzed including an assessment of national and EU regulatory compliance (might need translation, translation costs must be included in the supplier's offer), a review of CNS functional arrangements  and the identification of optimisation opportunities for Oro Navigacija with a particular focus on Palanga airport.

Inventory of CNS infrastructure and technological systems: To catalogue and evaluate the operational status and obsolescence risk of CNS systems and supporting technologies, technical characteristics and status of the air navigation services and CNS infrastructure, systems, and equipment. Also potential needs to ensure current systems alignment with applicable regulation (ICAO Annex 10, 11, ICAO Doc 7030, (EU) 2017/373, Lithuanian Airspace Organization Rules and the Oro Navigacija Safety Management Manual (SMM)) .

1.1.4. Cross-border agreement

The Supplier shall review options of a potential cross-border airspace agreement between Lithuania and Poland, their pros and cons considering flight procedures, airspace restrictions, effectiveness from the perspectives of safety, cost, and efficiency. ON will provide information on previously conducted reviews and agreements and will engage with stakeholders if necessary.

1.2. External assessment:

1.2.1. Market trend and context analysis shall be conducted using structured methods such as PESTLE analysis, assess key external trends, forecasts, and strategic themes shaping ATM environment and CNS field over the upcoming decade.

1.2.2. The Supplier shall identify the areas that require improvement in case they affect air traffic coordination with neighbouring airspaces (of Latvia, Belarus, Russia, Poland and/or Sweden) as it will be essential to consult these countries in order to determine the feasibility of the change. The Supplier shall identify all relevant interfaces with neighbouring FIRs and analyse any existing gaps—such as procedural inconsistencies, traffic coordination issues, or diverging CNS capabilities. This shall be supported by structured interviews with neighbouring ANSPs (conducted by ON).

1.2.3. The Supplier shall define possible ways to implement changes identified in 1.2.2.

1.3. WP1 deliverables

At the conclusion of this WP, the Supplier shall deliver:

1.3.1. Comprehensive assessment of the current situation: a detailed document summarising all findings and insights from WP1, delivered in MS Word format with supporting executive summary.

1.3.2. Executive summary presentation: a summarised version of the key elements of the report, formatted as PowerPoint presentation for more convenient dissemination.

Both deliverables shall present feasible insights and clear recommendations to guide the subsequent phases of implementation plan development.

2. WP2 - Defining future goals/AIRSPACE AND ATM CONOPS of Oro Navigacija

This work package shall define the future direction of Oro Navigacija. The aim is to establish clear organisational, technical, and operational priorities aligned with i European ATM Master Plan and ICAO GANP as well as with the goals of GREEN CNS project.

It shall define future-oriented scenarios covering all operational, technical, and organizational domains, ensuring alignment with European ATM modernization trends and national airspace development goals.

Key findings indicated earlier in WP1 shall define the future strategy development process.

Key areas WP2 shall include vision and definition of strategic objectives (as defined by ON): define the future vision for Lithuanian ATS provision and align proposed improvements with strategic goals and to develop forward-looking, viable alternatives for each key domain listed below, including sub-tasks:

2.1. Flight procedures and airspace structure

The supplier shall:

2.1.1. Assess the impact of LSSIP (Local Single Sky ImPlementation) goals on the structure of air traffic management services. Will the implementation of A-CDM (airport collaborative decision making, AOP05 according to LSSIP) be planned by 2032, and if so, what impact will this process have on airspace and runway capacity at Vilnius and Kaunas airports? Will the increased capacity require a re-evaluation of previously planned airspace management optimization methods?

2.1.2. Assess the TMAs capacity and the forecasts: using the findings of EUROCONTROL capacity study also capacity demand and forecasts, make suggestions to optimise the airspace and air traffic services system structure accordingly to ensure maximum efficiency.

2.1.3. Identify optimisation opportunities for traffic flows and airspace structures considering forecasted traffic levels, flows, military, and general aviation airspace needs.

2.2. ATS provision

2.2.1. Kaunas and Palanga ATC: the Supplier shall evaluate capacity to determine at what occupancy Tower (TWR) and Approach (APP) positions should be combined or separated. ON will provide FTS/RTS for simulation.

2.2.2. Centralization of approach control: the Supplier shall evaluate under what conditions it would be appropriate to merge Vilnius, Kaunas, and Palanga APP positions and what impact it would have on flight safety. Assess the number of flights and movements at Vilnius ATCC at which it is expedient to separate the Ground (GND) and TWR positions.

The Supplier may be required to observe live Air Traffic Control (ATC) operations. Access to such observations shall be coordinated and scheduled in advance, subject to mutual agreement between both parties. If Fast-Time Simulations (FTS) or Real-Time Simulations (RTS) are necessary for scenario assessment, ON shall provide access to the relevant simulators. The scheduling of simulator use shall also be agreed upon in advance by both parties.

2.3. CNS

The Supplier shall:

2.3.1. Assess future CNS technological landscape: envision the architecture and components of a modern, resilient CNS system including cost-benefit analysis of different recommendations provided by the Supplier.

2.3.2. Review results of previous assessment regarding feasibility of transferring Klaipeda’s DVOR/DME to Palanga (provided by ON) and current potential options for a conventional navaid at Palanga Airport after planned NDB removal to ensure contingency in case of GPS jamming and spoofing. Assessment of different options based on current and forecasted traffic levels, safety criteria, and cost effectiveness, including cost-benefit analysis.

2.4. Cross-border agreement

The Supplier shall review potential options for a cross-border agreement with Poland and prepare a summary of potential solutions which ON will present to Polish ANSP for review.

2.5. Expected methods of WP2

2.5.1. A minimum of two strategic ambition workshops shall be held at Oro Navigacija premises, involving participation from executives and departmental heads (a preliminary estimate of 10 people). The purpose of these workshops is to explore organizational aspirations, identify capability gaps, and validate target outcomes. The workshop dates shall be mutually agreed upon by both parties no later than two months prior to the planned dates. All associated travel and accommodation expenses shall be included in the Supplier’s tender price.

2.5.2. Structured interviews (live or online) for definition of strategic objectives and key success factors: conducting targeted interviews with heads of the departments to gain insight into operations, workflows, and strategic priorities. Getting clarity on the expected outcomes and future service delivery goals. Schedule of the interviews shall be agreed by both parties at least two months in advance.

2.5.3. Live presentation of concept of operations (ConOps): articulating targeted service model of Oro Navigacija across all key domains including technology, airspace, procedures, personnel, and governance. ConOps shall act as a unifying vision and planning tool for subsequent phases.

2.6. WP2 deliverables

2.6.1. Operational strategy document: the strategic plan shall be developed in PowerPoint format. The strategy shall be vsupported by internal communication guidelines.

2.6.2. Concept of operations (ConOps): framework agreement at strategic level of what changes shall be addressed and how to accomplish it. Key elements include:

2.6.2.1. Flight procedures and Airspace Structure Optimisation: Conceptual design of airspace and flight procedures (detailed IFPs and aeronautical charts will be completed by ON). Recommendations to address safety concerns and to improve efficiency and accommodate future traffic demands.

2.6.2.2. ATS Optimisation: Summary of optimal operational scenarios including validation report – results from Fast-Time Simulation and Real-Time Simulation (using ON simulator) of the new airspace model.

2.6.2.3. CNS: A structured review of potential mitigations to the current threats (GNSS signal interference) and recommendations for resilient, cost-effective solutions of the future architecture of CNS network in Lithuania.

2.6.2.4. Cross-border agreement: Recommendation of optimal solution(s) regarding Lithuania-Poland cross border agreement from operational, safety, regulatory and financial perspective.

2.6.3. ConOps shall cover following aspects:

2.6.3.1. Strategic objectives: Align with ON’s strategic goals and GREEN CNS objectives.

2.6.3.2. Safety and Contingency Planning: Ensure robust flight safety measures and contingency provisions are integrated into the operational framework.

2.6.3.3. Alignment with international projects: Align with European ATM Master Plan, including Common Project One (CP1), Common Project Two (CP2), SES2+, ICAO GANP and the objectives of GREEN CNS project.2.6.3.4. Environmental: Ensure compliance with environmental regulations and minimize environmental impacts where operationally possible.

2.6.4. In addition to the full ConOps document, a high-level executive summary in the form of a PowerPoint presentation shall be delivered. This presentation will depict the target service model envisioned for Oro Navigacija, providing stakeholders with a clear and concise overview of the strategic direction and operational goals.

3. WP3 - GAP identification map

The aim of this WP is to convert Oro Navigacija's strategic vision into a tangible plan. The gaps between the current state and the future concept shall be identified and assessed in the Gap identification map. This shall include working out how serious each gap is (e.g. regulatory, operational, capability), which then helps to decide what to do first in WP4. This WP will link the findings of the baseline assessment to a strategic vision for the future of the Lithuanian air traffic services (ATS) system.

3.1. Key areas of WP 3 shall include:

3.1.1. Evaluation of the proposed changes based on flight safety, capacity and the potential to increase it, economic efficiency, and environmental impact criteria.

3.1.2. An assessment of what is needed to implement the proposed changes in terms of technical and software requirements, human resources, and work organization.

3.1.3. Legal and regulatory compliance: outlining of the regulatory transition required to enable implementation of the strategy defined in WP2.

3.1.4. Cross-border service implementation assessment: analysis of the feasibility, benefits, and constraints of establishing or participating in cross-border ATS arrangements.

3.1.5. Recommendations for ATS optimization: proposals for structural and operational strategies ensuring efficient and continuous (24/7) service provision.

3.2. WP3 deliverables

3.2.1. GAP identification map – a document clearly identifying the gaps that need to be addressed to implement the planned ConOps.

3.2.2. Organisational structure of operational/technical staff – a MS Word document presenting the recommended updates to the organisational structure to support ConOps implementation.

3.2.3. Change management plan – stakeholder management and communication plan, supplemented by a social plan for the personnel that will be affected by the new situation. The Change Management Plan shall be delivered as a written report in Microsoft Word format. An executive summary (maximum 2 pages) shall be included at the beginning of the report. The plan shall include any supporting materials (e.g., stakeholder maps, communication templates, training outlines) as annexes. The Supplier shall present the Change Management Plan to the Contracting Authority in a dedicated meeting (in person or online), and incorporate feedback as required.

3.2.4. The Supplier shall provide a tool which uses multiple criteria, previously used by the Supplier within this study in order to assess the outcomes of proposed changes or different operational scenarios. This could be generated in Excel spreadsheet (or equivalent) and shall have an ability to change input values (including but not limited to simulation outputs, traffic forecasts, ATCO availability, CNS equipment status, procedural constraints etc.) to then provide an indication on the biggest impacts across the planned future operations. This input capability must accommodate structured updates without requiring programming or custom development. The baseline functionalities of this tool must focus on enabling ON to assess, adjust, and monitor the operational evolution plan dynamically as new circumstances arise (e.g., traffic growth, regulatory change, infrastructure developments). The tool must be compatible with standard operating systems used by the Contracting Authority (including, but not limited to Windows 10/11). No proprietary or paid third-party software (other than Microsoft Office suite) shall be required for basic operation. The tool must provide clear outputs, including visualizations (e.g., charts, tables) and summary reports, to support decision-making. The tool must include user guidance or help documentation in English.

4. WP4: operational evolution plan

WP4 is the final phase which converts the business and organisational strategies into a cohesive operational evolution plan (OEP). This document must outline a practical, sequenced roadmap to achieving the future objectives of Oro Navigacija. The OEP in WP4 shall then introduce and structure the necessary objectives and actions across short, medium, and long-term horizons.

OEP shall include indicative timelines, resource and capability requirements, and implementation priorities across all areas addressed in the previous work packages.

4.1. WP4 deliverables

4.1.1. Operational evolution plan (OEP) – a report detailing the roadmap for implementing the future vision defined in WP2, with clear actions, timelines, responsibilities, and pre-requisites. This can be considered the main report, which also relies on the information contained in other documents.

4.1.2. Cost benefit analysis model and report (CBA) – a financial model with an accompanying report which appraises the implementation of different scenarios provided in WP2 assessing both the costs and the benefits. The assumptions and criteria for evaluating cost benefits must be proposed to and agreed with the customer. Requirements for CBA:

4.1.2.1. The Supplier must ensure that the CBA scope clearly distinguishes different scenarios (i.e. the data can be used as separate CBAs in the future). The Supplier must follow the general principles of cost-benefit analysis in the document 'Methodological Guidelines for the Conduct of Cost-Benefit Analysis of Investment Projects', commissioned by the European Commission. (This document can be accessed via this link: https://ec.europa.eu/regional_policy/en/information/publications/guides/2014/guide-to-cost-benefit-analysis-of-investment-projects-for-cohesion-policy-2014-2020)
4.1.2.2. CBA should cover all defined scenarios, to compare the costs and benefits created by each of them, to prepare a detailed summarizing CBA report comparing all defined scenarios, to coordinate it with ON and submit coordinated final version of the report, which shall reflect the reasoned conclusions obtained from the CBA.

4.1.2.3. In CBA, Supplier shall define and clearly identify the required investments, operating costs and benefits of each scenario (increase in revenue or saving in costs and/or time, improvement of performance indicators) and other related assumptions.

4.1.2.4. ON will provide the initial raw data needed for CBA but will not be involved in filling in any templates or making intermediate calculations at any CBA preparation stage.
4.1.2.5. CBA must include:

4.1.2.5.1. financial analysis;

4.1.2.5.2. financial flow analysis, must determine the essential financial analysis result indicators - net present value and internal rate of return (NPV and IRR);

4.1.2.5.3. risks identification;

4.1.2.5.4. sensitivity analysis.

4.1.2.6. Financial analysis for each scenario will have to cover:

4.1.2.6.1. investments (CAPEX) – including equipment and system costs (hardware and software acquisition), software licensing, integration costs and other costs related to the acquisition of long-term assets;

4.1.2.6.2. operating costs (OPEX) – staff costs (salary cost and other benefits as health insurance, etc.), training costs, maintenance and repair costs, administrations costs and other operating costs;

4.1.2.6.3. benefits - revenues and/or savings (increase in revenues, savings in internal resources, costs), improvement of performance indicators;

4.1.2.6.4. calculations of financial indicators – free cash flow (FCF), net present values (NPV), internal rate of return (IRR);

4.1.2.6.5. graphical representation of cash flows, identifying the payback, sensitivity, and other important analysis outcomes of each scenario.

4.1.2.7. The evaluation of scenarios assumptions, risks and the sensitivity analysis must include and assess the main aspects of implementation, operation, regulatory and competitive environment, market capacity needs, technology choice, financial and other aspects considered as important by the Supplier. A sensitivity analysis must be performed to determine how deviations from specific assumptions and risks could positively and/or negatively affect the results of the scenarios and the benefits and costs created by them.
4.1.2.8. In the final CBA report, the key results of all analyzed scenario must be summarized, indicating and describing all possible scenarios, how and why each alternative may be attractive from a cost-benefit analysis perspective, indicating the key risks and the sensitivity of the alternatives, determining whether the benefit of the alternative outweighs its implementation costs and whether the alternative is a more rational choice from an economic point of view than other.
4.1.2.9. The Supplier, after evaluating all the scenarios, must provide a summary table of the analyzed alternatives, which would contain the data of the criteria according to which the scenarios are compared;
4.1.2.10. The final report (in paper and electronic form) must be comprehensive, enabling further expert review, and must cover all the details of the service performed (including, but not limited to, the assumptions and reasoning chosen, the data used, their sources, the calculation algorithms and formulas applied, etc.). All calculations and related spreadsheets performed during the cost-benefit analysis shall be submitted to the Customer in ".xlsx" format.

4.1.3. Safety assessment of the change - assessment of safety risks at the level of the change that implies the airspace design, considering the whole scope of elements of ATM functional system.

III. Additional consultations

Any additional consulting hours (out of the scope) will be ordered separately (if needed). Topics for additional analysis (or expansion of scope) will be discussed at regular meetings including as well as number of hours required and timeline acceptable to both parties.

1. The following requirements apply:

1.1. Additional consulting services will be ordered and delivered only as required by ON during the contract period (including any extensions if applicable). ON will pay only for the additional services actually provided. These services will be purchased based solely on the Buyer’s needs and shall not exceed the maximum value (EUR, excluding VAT) specified in the contract.

1.2. ON will order consulting hours as required, specifying the scope, deliverables, and timeline for each assignment. Orders for consulting hours will be placed in writing (e.g., via email), with mutually agreed terms and scope for each assignment.

1.3. The Supplier shall confirm acceptance of each order and provide the requested services within the agreed timeframe.

## IV. Service provision conditions

1. The Supplier shall provide consulting services in accordance with the terms and timelines set out in the Contract and in full compliance with applicable international and national regulatory requirements, and industry best practices.

2. Consulting services shall be rendered with due care, diligence, and technical competence by appropriately qualified experienced personnel.

3. All consultation-related documentation and deliverables shall be submitted in English and provided in electronic format. Exceptions apply to operational documents of the Contracting Authority, which—if required by the Supplier for the implementation of consultations—shall be submitted in Lithuanian. The Supplier shall independently collect all data necessary for the execution of the tasks outlined in the Technical Specification, including from third parties as required. The Contracting Authority shall only intervene in cases where third parties explicitly refuse to provide the requested data in written form.

The Supplier shall be granted an opportunity to get acquainted with the activities of the Contracting Authority, documents, to visit air traffic control centres, identify the related subjects.

4. The Supplier shall ensure proper communication and availability for online, or in-person meetings, briefings, and workshops as required by the Contracting authority.

5. The Supplier shall report the progress of the project to the Contracting Authority. Regular status meetings shall be held online at least once a month and in person, upon the completion of each WP unless agreed otherwise, shall be organised during the entire Contract implementation period, attended by the Contracting Authority and the Supplier experts (whose experience was evaluated during the economic usefulness assessment, or experts who were replaced in accordance with the terms of the procurement contract), to exchange the necessary information and present the progress of the Project  During those meetings, Supplier shall submit draft versions of the deliverables for the relevant work package to the Contracting Authority for their assessment and approval.

6. The Consulting services shall be performed either on-site at locations designated by the Contracting authority or remotely, as agreed between the Parties. In cases where live meetings are held at the premises of the Supplier, travel and accommodation expenses are covered by the Supplier.

# V. Service provision term

1. The overall duration for the provision of consultancy services related to the assessment of air traffic service provision efficiency and optimisation opportunities shall not exceed 18 months from the date of Contract signature.

1.1. Schedule and maximum period of completion for each WP shall be agreed with the Contracting Authority within 10 working days after signing the Contract.

2. Extension of Contract Term

2.1. The  term of  completion for each WP or the final term of service provision may be extended only in the event of significant delays or obstacles that are beyond the Supplier’s reasonable control (e.g., force majeure, regulatory changes, delays caused by third parties, or other circumstances not attributable to the Supplier). Requests for extension must be submitted in writing by the Supplier to the Contracting Authority as soon as the relevant circumstances become known, and no later than 10 working days before the original service provision (interim or final) end date.

The request must include:

2.1.1. A detailed description of the reasons for the delay,

2.1.2. Evidence that the delay is due to circumstances beyond the Supplier’s control,

2.1.3. The proposed new completion date,

2.1.4. A revised implementation schedule, if applicable.

2.2. The Contracting Authority will review the request and, if justified, may grant an extension for a period not exceeding 2 months (throughout the entire Contract implementation period).

2.3. Any extension granted shall be formalized by a written amendment to the contract.

VI. Warranty liabilities

1. Warranty duration is 24 months from the day of signing the final note on transfer and acceptance.

2. The Supplier shall be liable to appoint a contact person, who shall be responsible for proper communication during the warranty period.

3. During the warranty period the Contracting Authority shall submit the documents prepared by the Supplier to the Board, national supervising institution, other relevant entities in order to initiate the implementation of the scenario.

In case the Contracting Authority receives essential remarks from the interested parties, it shall address the Supplier by e-mail with the following request:

3.1. To prepare a response for the entity that submitted the remarks. The Supplier’s contact person must ensure that the draft response is prepared and submitted to the Contracting Authority’s responsible person (the requester) within 5 working days from the date of receipt of the request to prepare the draft response or;

3.2. To specify feasibility study report, cost benefit analysis or concepts based on the remarks received from the third parties.

4. The Supplier shall revise the documents at their own expense and submit the revised document to the Contracting Authority not later than within 20 working days from the day the Contracting Authority addressed the Supplier.

5. In case significant changes are required, the Supplier may address the Contracting Authority with a request to establish another reasonable term. In such a case the Supplier shall submit a request to establish an extended term also providing a detailed list of actions to be implemented within the term, their implementation dates and responsible persons within a term of 10 working days from the day of reception of the letter of the Contracting Authority.

VII. Environment protection (green) criteria

1. In accordance with Paragraph 4 of the Order of the Minister of Environment of the Republic of Lithuania No. D1-508 of 28 June 2011 “Regarding the approval of the list of products, the public procurement and procurement of which are subject to environmental protection criteria, the description of the application procedure of environmental protection criteria and environmental protection criteria that procurement organizations and procuring entities must apply when purchasing goods, services or works”, the Procurement shall be considered green, since it meets the condition established under Item 4.4.3., i.e. only a service of intangible nature is procured, not related to the creation of tangible object, during which no significant negative impact upon the nature is expected, no source of pollution is created and no waste is generated.

VIII. Abbreviations

AIS – Aeronautical Information Service

ATS – Air Traffic Service

ASM – Airspace Management

ATFM – Air Traffic Flow Management

FPD – Flight Procedure Design

CNS – Communications, Navigation and Surveillance

FIR – Flight Information Region

SESAR – Single European Sky ATM Research

CP1 – Common Project 1

RNAV – Area Navigation

RNP – Required Navigation Performance

WP – Work Package

ATM – Air Traffic Management

ANS – Air Navigation Services

PBN – Performance Based Navigation

EYVI – Vilnius airport (ICAO code)

EYKA – Kaunas airport (ICAO code)

EYPA – Palanga airport (ICAO code)

AoR – Area of Responsibility

TMA – Terminal Control Area

CTR – Control Zone

GAT – General Air Transport

ATC – Air Traffic Control

PESTEL analysis – Political, Economic, Social, Technological, Legal and Environmental factor analysis

ICAO – International Civil Aviation Organization

GANP – Global Air Navigation Plan

GREEN CNS – Modernization of CNS in Air Traffic (project)

LSSIP – Local Single Sky ImPlementation

TWR – Tower

APP – Approach

GND – Ground

FTS – Fast Time Simulation

RTS – Real Time Simulation

DVOR – Doppler VHF Omidirectional Range (navaid)

VHF – Very High Frequency (30-300 MHz)

DME – Distance Measuring Equipment (navaid)

GNSS – Global Navigation Satellite System

NDB – Non-Directional Beacon (navaid)

ANSP – Air Navigation Service Provider

ConOps – Concept of Operations

OEP – Operational Evolution Plan

CBA – Cost Benefit Analysis

NPV – Net Present Value

IRR – Internal Rate of Return

CapEx – Capital Expenditure

OpEx – Operating Expenses

FCF – Free Cash Flow