Annex 1 to the technical specification

TABLE OF CONFORMITY OF GOODS WITH REQUIREMENTS

| No. | Name | Description | Compliance parameters specifically proposed by the supplier (to be completed by the supplier) | Document title, page number and/or link to web page Evidence of conformity of goods (to be filled in by the supplier) |
| --- | --- | --- | --- | --- |
| 1. | Bus type | 1.1. A two-axle low-floor single-deck bus fuelled with hydrogen (vehicle code M3 CE class I); 1.2. Suitable for intensive urban operation, including outdoor parking, in winter (up to -25 °C) and summer (up to + 40 °C) temperature conditions (buses to be operated in Vilnius city); 1.3. Certified in accordance with the requirements of Directive (EU) of the European Parliament and of the Council. |  |  |
| 2. | Date of first registration | No earlier than 2021 |  |  |
| 3. | Motovalanda | Maximum 6000 |  |  |
| 4. | Total length | 4.1. From 11 500 mm to the maximum permissible under the legislation of the Republic of Lithuania in force at the date of issue of the certificate of conformity of the vehicle for this type of vehicle. |  |  |
| 5. | Width | 2 490 mm to 2 550 mm. (measured in accordance with the requirements of paragraph 6.2 of ISO 612-1978 (or equivalent standard). |  |  |
| 6. | Elevation | Maximum 3 460 mm. (measured in accordance with the requirements of paragraph 6.3 of Standard ISO 612-1978) |  |  |
| 7. | Floor height | The low floor, the entry/exit height from the road surface shall not exceed 350 mm. |  |  |
| 8. | Total capacity | At least 25 passenger seats; At least 1 position for disabled persons (with special needs with wheelchair) or pushchair (with anchorage straps, rails or other wheelchair anchorages) in the low-floor zone between the bus axles; The total number of seats and standing places installed shall not be less than 75, excluding seats for persons with disabilities; |  |  |
| 9. | Passenger boarding doors | A pneumatic or electrical door on the right-hand side of the bus. The width of openings in first and second doors shall be not less than 1 200 mm. With protection against crushing (the door must open in the event of an obstacle occurring between the doors during closure) and emergency door openings. Possibility for the door to be operated individually and in combination. The opening of the door must be followed by a braking system ensuring that the bus cannot move when at least one door is opened. A ramp is installed for the wheelchair or the child’s wheelchair. |  |  |
| 10. | Number of axles | 2 axles. |  |  |
| 11. | Fuel | 11.1. ‘Fuel’ means hydrogen H2; 11.2. Fuel tanks: composite pressure cylinders connected together. The fuel shall be sufficient for a mileage of at least 350 km; 11.3. The operating pressure of the hydrogen system is 350 bar (at + 15 °C) depending on the type of system; 11.4. Refuelling time: maximum 20 minutes (Protocol SAE J2601 and SAEJ2799); 11.5. The type of fuelling tap is N1, which is easily accessible to the operator during refuelling without additional elevations. A connection or connections according to SAE J2600; 11.6. Safety valves, sensors, fuses must be present; |  |  |
| 12. | Hydrogen fuel cell | 12.1. ‘Hydrogen fuel cell’ means an electrochemical unit that generates electricity using hydrogen and oxygen; 12.2. The hydrogen fuel cell is powered from hydrogen storage (fuel tanks); 12.3. Hydrogen fuel cell power of 60 kW or more; 12.4. The hydrogen fuel cell shall operate at an ambient temperature between -20 °C and + 40 °C without further heating. 12.5. Hydrogen refuelling protocol: SAE J2601-2. Filling connector: TK16 |  |  |
| 13. | Suspension | Pneumatic with automatic elevation support, forced take-off and descent and right track. |  |  |
| 14. | Steering System | Power-assisted system, left-hand drive. The steering position shall be adjusted either in conjunction with the driver’s instrument panel or independently. |  |  |
| 15. | Brakes | All brakes of the disc type. The vehicle shall be equipped with an electronic braking force adjustment system EBS or an equivalent system; |  |  |
| 16. | Wheels and tyres | Tyres are designed for public transport in urban conditions. Tyres shall be suitable for all seasons (M+S). All tyres shall have a tread depth of not less than 7 mm. |  |  |
| 17. | Indicator panels and passenger information systems | All buses offered must be equipped with signboards and passenger information systems. Must be programmable. |  |  |

| Queue No. | Bus manufacturer/name, model | Bus identification numbers (VIN) | Dates of first registration of buses | Motovalanda on the date of submission of the initial tenders, | Colours of buses | Bus values, in EUR excluding VAT |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | / |  |  |  |  |  |
| 2 | / |  |  |  |  |  |
| 3 | / |  |  |  |  |  |
| 4 | / |  |  |  |  |  |