TECHNICAL SPECIFICATION

- TERMS AND ABBREVIATIONS

  - Purchaser – AB Amber Grid. More information about the Purchaser and its activities can be found www.ambergrid.lt.

  - Supplier – an entity (a natural person, a private legal entity, a public legal entity, other organisations and their divisions, or a group of such entities, including temporary associations of entities, that offer to supply goods for the Procurement object on the market).

  - Contract – The contract to be concluded by the Purchaser with the Supplier that will be declared the successful tenderer regarding the Procurement object.

  - Goods – spare parts for LEWA odorization units.

- PROCUREMENT OBJECT, QUANTITIES (VOLUMES), AND DETAILED DESCRIPTION OF THE PROCUREMENT OBJECT

2.1. The Procurement object – spare parts for the existing LEWA odorization units used by the Purchaser, their accessories, and related goods that meet the requirements listed in Chapter 7 of the technical specification (hereinafter - the Goods). The goods are intended for essential repairs to existing odorization units. If the Procurement documents specify particular models or sources, specific processes or trademarks, patents, types, specific origin or manufacture, etc., they may be replaced with equivalents. The spare parts purchased for the odorization units must be suitable for the use with the manufacturer’s LEWA DA7, DA8, and OD450 odorization units.

2.2. The total price of the Supplier’s tender will be for comparison purposes only and is intended solely to determine the most economically advantageous tender. The quantities of the Goods provided are preliminary (used only for the evaluation of tenders). The Goods will be purchased based on individual orders from the Purchaser. The Purchaser is not obligated to purchase the entire quantity of the Goods; they will be purchased as needed until the end of the Contract term or until the Contract price specified in the Contract is reached, whichever occurs first.

2.3. If the need arises, the Purchaser shall have the right to purchase other goods related to the Procurement object that are not specified herein (up to a maximum of 10% of the initial Contract value exclusive of VAT, which is included in the initial Contract value exclusive of VAT). Goods not listed in the goods list but related to the Procurement object will be paid for at prices not exceeding the valid prices of such goods indicated at the Supplier’s point of sale, in the catalogue, or on the website on the date of the order, or, if such prices are not published, at prices offered by the Supplier, that are competitive and in line with the market.

- PLACE OF PERFORMANCE OF CONTRACTUAL OBLIGATIONS

☒ Maksvytiškiai village, Verslo g. 11, Panevėžys district;

☒ Gudelių g. 49, Vilnius.

- PROCEDURES AND DEADLINES FOR THE FULFILLMENT OF CONTRACTUAL OBLIGATIONS

  - The Goods will be purchased only based on individual orders submitted by the Purchaser during the term of the Contract. The Supplier must deliver the Goods free of charge to the addresses specified in Chapter 3 (the specific address will be indicated in the order) during the Purchaser’s working hours (Monday through Thursday from 7:30 am to 4:30 pm, Friday from 7:30 am to 3:15 pm, lunch break 11:30 am – 12:15 pm).

  - The Purchaser shall submit orders to the Supplier via the e-mail address specified in the Contract.

  - The delivery deadline for the Goods is no later than 90 calendar days from the next working day following the date the Purchaser’s order is submitted to the specified e-mail address.

  - The Goods must be covered by a warranty of no less than 12 months from the date of transfer of the Goods (confirmation of invoice).

  - All Goods must be new, unused, unrepaired, and delivered in their original factory packaging. Components of the Goods must be manufactured by the same manufacturer.

  - If there are any defects or flaws in the Goods for which the Seller is responsible and which cannot be detected immediately upon receipt of the Goods, the Purchaser must notify the Seller of such defects or flaws via e-mail within 5 (five) working days from the date of delivery of the Goods. Upon receiving the Purchaser’s notification, the Seller must, at the Purchaser’s discretion: 1) replace the defective Goods with non-defective ones within 5 working days or a longer period agreed upon with the Purchaser and deliver them to the Purchaser; or 2) refund the amounts paid by the Purchaser for the defective Goods within the period specified by the Purchaser (if they have already been paid; if not paid, the Purchaser shall have the right to reduce the price of the order (deduct the value of the defective Goods)). In cases where the Purchaser chooses to have the Goods replaced, the Seller must, at its own expense and risk: 1) visit the Purchaser and collect the defective Goods; and 2) deliver quality Goods to the Purchaser. The collection of defective Goods and the delivery of non-defective Goods shall be carried out in accordance with the delivery time requirements set forth in Point 4.3 of the Technical Specification. Upon written agreement between the Seller and the Purchaser, a longer period for remedying defects may be agreed upon.

- DOCUMENTATION TO BE SUBMITTED DURING THE PERFORMANCE OF THE CONTRACT

  - The Supplier must provide the Purchaser, along with the Goods, with warranty documents, quality certificates issued by the manufacturer of the Goods, and specifications.

  - An invoice shall serve as a Deed of delivery and acceptance of the Goods

  - The Supplier shall issue an electronic invoice for the delivered Goods via the “SABIS” information system (https://sabis.nbfc.lt).

- GREEN PUBLIC PROCUREMENT (HEREINAFTER - GPP) REQUIREMENTS

  - The following green public procurement (hereinafter - GPP) requirements apply to the performance of the Contract:

    - The Contract shall be signed by electronic means;

    - In performing the Contract, the Supplier undertakes to comply with the following environmental requirements: to reduce paper consumption and to refrain from unnecessary copying and printing of documents;

    - All information and/or documents necessary for the performance of the Contract shall be provided and coordinated in electronic format via electronic means;

    - The Supplier shall undertake to carry out deliveries of the Goods in such a way as to minimize the impact of transport pollution: 1) to consolidate deliveries (if possible, to deliver the Goods for multiple orders in a single trip); 2) to plan routes to minimize empty runs; 3) deliver the Goods at the Supplier’s expense and risk to the addresses specified in Chapter 3 (the specific delivery address is indicated in the Purchaser’s order) at an environmentally friendly delivery time, on the Purchaser’s working days (during off-peak hours): Monday through Thursday from 9:00 am to 11:00 am and from 1:00 pm to 3:00 pm, and on Fridays from 9:00 am to 11:00 am and from 1:00 pm to 2:00 pm (the Purchaser's working hours are Monday through Thursday from 7:30 am to 4:30 pm, and Friday from 7:30 am to 3:15 pm; lunch break is from 11:30 am to 12:15 pm). On days preceding public holidays, working hours are one hour shorter. If the Goods are delivered using only a zero-emission road vehicle (electric vehicle), the Supplier has the right to deliver the Goods at any time during the Purchaser’s working hours, without adhering to the environmentally friendly delivery time slots specified in this clause. In such a case, the Supplier must provide evidence that the Goods were delivered using only a zero-emission vehicle (e.g., a copy of the vehicle registration certificate or another equivalent document). If no evidence is provided that the Goods were delivered using only a zero-emission road vehicle, it shall be deemed that the delivery was to be carried out in accordance with the environmentally friendly delivery time slots specified in this clause, and such delivery shall be deemed a breach of the Contract, for which the Supplier shall be subject to a fine of 50 EUR.

7. TECHNICAL SPECIFICATION OF THE GOODS

| Ser. No | Procurement object, requirements | Preliminary quantity, pcs. |
| --- | --- | --- |
| 1. | Odorization system hose (Type S). Used for filling odorant in LEWA DA7 and DA8 odorization units. Length: 80–120 cm. Material: stainless steel. Connection: 12 mm. Manufacturer: LEWA, catalogue No 013714.0150 (or equivalent). | 4 |
| 2. | Odorization system hose (Type D). Used for supplying nitrogen to LEWA DA7 and DA8 odorization units. Length: 80–120 cm. Material: stainless steel. Connection: 8 mm. Manufacturer: LEWA, catalogue No 013714.0143 (or equivalent). | 3 |
| 3. | Odorant barrel tap (Type S). Designed for use with GA50 and GA200 Wilhelm Schmidt odorant barrels. Body material: stainless steel; handle material: brass. Supplied with an adapter. Barrel connection thread ¼ NPT; thread of connection to the odorization system: 12S. Manufacturer: LEWA, catalogue No 103554.0587 (or equivalent). | 9 |
| 4. | Odorant barrel tap (Type D) or equivalent. Designed for use with GA50 and GA200 Wilhelm Schmidt odorant barrels. Body material: stainless steel; handle material: brass. Supplied with an adapter. Barrel connection thread ¼ NPT; thread of connection to the odorization system: 8D. Manufacturer: LEWA, catalogue No 103554.0588 (or equivalent). | 9 |
| 5. | Safety exhaust valve. Used for LEWA DA7, DA8, and OD450 odorization units. Material: stainless steel. Activation pressure: 0.5 bar. Bottom connection: 1/2" thread. Outlet chamber thread: ¾". Manufacturer: LEWA, catalogue No 059859.0070 (or equivalent). | 13 |
| 6. | Odorant meter ZHM01/3A7.EI+VTER-2L Measuring range: (0.005–0.25 l/min). Suitable for aggressive environments. Supplied with an electrical part and connection adapters for 8 mm diameter tube (or equivalent). | 1 |
| 7. | Flask for the KMM-1 odorant flow meter. Material: stainless steel. Manufacturer: LEWA, catalogue No 059047.0002 (or equivalent). | 11 |
| 8. | Adapter (Type S) Used to connect the odorant supply hose to the odorant barrel equipped with an odorization unit. Marking: GA-S-N1/4-NPT-12D. Material: stainless steel. Manufacturer: LEWA, catalogue No 090500.0380 (or equivalent). | 11 |
| 9. | Adapter (Type D) Used to connect the nitrogen supply hose of an odorant barrel to the odorizing unit. Marking: GA-D-N1/4-NPT-8D. Material: stainless steel. Manufacturer: LEWA, catalogue No 090500.0381 (or equivalent). | 11 |
| 10. | Tap membrane Marking DA7 22Dx2. Suitable for aggressive environments (ethyl mercaptan). Manufacturer: LEWA, catalogue No 041628.0273 (or equivalent)) | 26 |
| 11. | Hose connection adapter for the IC20 odorant container. Material: stainless steel. Manufacturer: TODO, catalogue No TODO-9200G-4407 (or equivalent) | 2 |
| 12. | MAH pump return valve. Material: stainless steel. Supplied with gaskets between components. Suitable for aggressive environments. Manufacturer: LEWA, catalogue No 055794.0002 (or equivalent). | 2 |
| 13. | MBH pump return valve. Material: stainless steel. Supplied with spacers between the elements. Suitable for aggressive environments. Manufacturer: LEWA, catalogue No 013233.0300 (or equivalent). | 2 |
| TOTAL (preliminary quantity, pcs.) |  | 104 |

An equivalent Procurement object is one the characteristics of which are not inferior (i.e., are the same or better) to the requirements set forth in the Procurement documents for the object being procured, and the proposed equivalent object can be used for its intended purpose without any restrictions (including but not limited to the following):

• without requiring additional modifications to interacting components;

• its use will not contribute to accelerated wear and tear, malfunctions, and/or loss of warranty for interacting components;

• the expected service life is not shorter;

• there is no lower level of technical sophistication.

When offering an equivalent Procurement object, it is mandatory to submit documents proving compliance with the requirements set for the Procurement object. Such documents may include a test report or certificate issued by a conformity assessment body established in the Republic of Lithuania, as well as certificates issued by equivalent conformity assessment bodies established in other countries. If the Supplier is unable to obtain the specified certificates or test reports due to circumstances beyond the Supplier’s control and proves, with objective, written evidence, that the proposed equivalent procurement object meets the requirements or criteria specified in the Technical Specification, the bid evaluation criteria, or the conditions for the performance of the procurement contract, the Purchaser shall accept other appropriate measures. However, a Supplier’s self-declaration without specific technical evidence shall not be considered an appropriate means. The Purchaser reserves the right to conduct a risk assessment if the documents submitted to demonstrate the equivalence of the proposed goods are insufficient.