ANNEX 1

TO THE PROCUREMENT CONDITIONS

(LOT  2)

# TECHNICAL SPECIFICATION FOR SPORTS JACKET

CHAPTER I

GENERAL PROVISIONS

- A sports jacket (hereinafter referred to as a jacket) is designed to be worn by soldiers during sports activities to protect them from wind and rain.

- Jackets must comply with the requirements set out in this technical specification.

- The fabric used for the production of jackets must comply with the minimum environmental protection criteria specified in Section IX “Textile Products” of Annex 2 to the Procedure for the Application of Environmental Protection Criteria in Green Public Procurement, approved by Order No. D1-508 of the Minister of Environment of the Republic of Lithuania on 28 June 2011.

- The sizes of jackets required for soldiers are presented in Table 1, compiled in accordance with the requirements of standard LST EN 13402 and the size system adopted by the Lithuanian Armed Forces.

- A precise table of sizes, with specified quantities, shall be provided to the Supplier upon signing the contract.

- If necessary, non-standard sizes of jackets may be requested, not exceeding 2% of the ordered quantity.

Table 1

# SIZES OF SPORTS JACKETS

| Height, cm | Chest circumference, cm | 88-92 | 96-100 | 104-108 | 112-116 | 120-124 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Waist circumference, cm | 76-80 | 84-88 | 92-96 | 100-104 | 108-112 |
| 164-170 |  |  |  |  |  |  |
| 176-182 |  |  |  |  |  |  |
| 188-194 |  |  |  |  |  |  |
| 200-206 |  |  |  |  |  |  |

- The design of products must be based on the data provided in LST ISO 8559 (ISO 8559) "Clothing design and anthropometric measurements. Body dimensions" (or an equivalent standard).

- The warranty period for the jackets is at least 12 (twelve) months under active use conditions (calculated from the date of delivery of the goods from the Buyer's warehouse) and 24 (twenty-four) months from the date of receipt of the goods at the Buyer's warehouse.

CHAPTER II

TECHNICAL REQUIREMENTS

- The jackets are men's style, unlined, straight cut, with a stand-up collar that transitions into a hood. The sleeves and front hem are straight. The back hem is curved and longer than the front. An overview of the sports jacket model is provided in the appendix to the technical specifications.

- The jacket fastens at the front with a fully detachable zipper (waterproof) along the entire length of the jacket and the height of the stand-up collar. On the inside, along the entire length of the zipper, there is a seam that covers the zipper teeth and covers the zipper head at the top (extending 2.0–3.0 cm to the outside).

- Jacket with back yoke, front-back sides (without side seams), with side inset pockets, fastened with zippers (waterproof). At the top of the sides (in the armpit area) – five small perforated vents for ventilation.

- Jacket with an inset pocket-bag on the back, fastened with a zipper (waterproof) and designed (if necessary) for storing/packing the jacket.

- Jacket with raglan sleeves. The sleeves are single-stitched, with three seams in the elbow area (two in the front and one in the back for better ergonomics). The bottom of the sleeves is gathered (folded into a pleat) with the help of a flat, elastic band (rubber).

- The hood of the jacket (together with the stove) consists of three parts: two identical side parts and a middle part. The middle part is slightly convex at the top (with a small peak). To hang the jacket, a hook made of the main material is sewn into the seam on the outside of the front.

- The width of the bottom of the jacket, the width of the hood, and the length around the face are adjustable with the help of a round, elastic band (rubber) and fasteners. The elastic band is threaded through tunnels-slits and pulled through a pair of metal eyelets on the inside of the jacket, on the back of the middle part of the hood, and on the front of the stand on both sides of the zipper.

- A woven emblem – the flag of the Republic of Lithuania – is sewn into the center of the back seam.

- On the left front side (chest area) – a shield-shaped stamp/drawing (width – 6.8 cm ± 0.2 cm, height 7.7 cm ± 0.2 cm). The overall image of the stamp is presented in the appendix. The exact size, location, and color of the stamp will be agreed upon with the winner of the competition after signing the contract.

- A 0.3–0.5 cm wide light-reflecting strip is sewn into the zipper seams (along the entire length). The color of the strip is moss green.

- On the upper back part of both sleeves, there is a light-reflecting striped pattern in moss green. The length of the reflective strip is 27.0–28.0 cm, and its width is 1.0 cm ± 0.1 cm.

- If necessary, during the performance of the contract, before approving the working model, the design and technological processing solutions of the products may be slightly adjusted, provided that this does not impair the properties and appearance of the product.

- The sports jacket must be made of a single-color, double-layer material (coated with 100 PU or equivalent coating on the inside of the product) with the technical characteristics specified in Table 2 (hereinafter referred to as the base material). The color of the main material is moss green, close to color code 19-0419TP according to the PANTONE TEXTILE color catalog.

Table 2

# TECHNICAL CHARACTERISTICS OF THE BASE MATERIALS

| No | Indicator name, dimension | Indicator value | Test method code |
| --- | --- | --- | --- |
| 1. | Fiber composition , % | Top layer : 90±3 PA, 10±3 EL; bottom layer: 100 PU (or equivalent) | specify |
| 2. | Surface density , g/m2 | 65 ± 10 | LST ISO 3801; LST EN 12127 or equivalent |
| 3. | Resistance to water vaporRet, m2Pa/W | ≤ 12 | LST EN ISO 11092 or equivalent |
| 4. | Elasticity , % warp direction weft direction | ≥ 5 ≥ 30 | LST EN ISO 20932-1 or equivalent |
| 5. | Resistance to crushing at a nominal pressure of 12 kPa, pumps | ≥ 100 000 | LST EN ISO 12947-2 or equivalent |
| 6. | Maximum force , N warp direction weft direction | ≥ 400 ≥ 150 | LST EN ISO 1421-1 or equivalent |
| 7. | Tear strength , N warp direction weft direction | ≥ 20 ≥ 15 | LST EN ISO 4674-1, Method A or equivalent |
| 8. | Siūlų skaičius centimetre warp direction weft direction | ≥ 90 ≥ 50 | LST EN ISO 7211-2 , or equivalent |
| 9. | Resistance to water penetration at a water pressure increase rate of 60 ± 3 cm/min, cm | ≥ 200 | LST EN ISO 811 or equivalent |
| 10. | Stain resistance, points |  |  |
| 10.1. | to washing at 40º | ≥ 4 | LST EN ISO 105-C06 or equivalent |
| 10.2. | to water | ≥ 4 | LST EN ISO 105-E01 or equivalent |
| 10.3. | to sweat | ≥ 4 | LST EN ISO 105-E04 or equivalent |
| 10.4. | to dry abrasion | ≥ 4 | LST EN ISO 105-X12,-X16 or equivalent |
| 10.5. | to wet abrasion | ≥ 3 | LST EN ISO 105-X12,-X16 or equivalent |
| 10.6. | to artificial light | ≥ 4 | LST EN ISO 105-B02 or equivalent |
| 11. | Change in dimensions after washing (Washing and drying procedures according to LST EN ISO 6330: washing procedure – 4G (40º, gentle wash), drying method – C),% | ± 3 | LST EN ISO 5077 or equivalent |
| 13. | Color difference , Δ ECMC | ≤ 2 | LST EN ISO 105-J03 or equivalent |
| 14. | Weaving | Rip stop, plain | specify |

Note:

Indicator 13, “Color difference,” is required during contract performance and determines the permissible color deviation from the agreed working sample.

- Product details are cut lengthwise, must be symmetrical, and paired details must be identical. No color variations are allowed between details of the same product or within details (the same details in different places).

- All seams of the product must be sealed and waterproof (they must be sealed from the inside with a special tape). The allowances for seams and the density of stitches must be selected so as to ensure the required strength and quality of the product's seams throughout the entire period of use. The ends of all seams must be neatly and securely fastened so that they do not come apart during use.

- GaThe seams, around the pocket openings, on both sides of the zipper, the sleeve seams, and the edge of the hood are trimmed with 0.1–0.2 cm wide piping. The bottom of the sleeves and the second hem of the hood are 3.0 cm ± 0.1 cm wide, and the bottom hem of the garment is 2.2 cm ± 0.1 cm wide.

- Side pockets made of mesh-type knitted fabric (color similar to the main fabric), rectangular in shape. The pockets are sewn into the front zipper (embossed) and bottom side seams. The upper part of the pocket bag (in contact with the front detail) is made of knitted mesh fabric, while the lower part of the bag is made of two parts: the main fabric and knitted mesh fabric.

- The jacket fastens at the front with a detachable, spiral, reversible, water-resistant (impermeable) zipper with a tooth width of 0.5+/- 0.05 cm. The zipper head is metal and magnetic. A facing is sewn to the inside of the left edge of the zipper, extending to the outside at the top and covering the zipper head. The facing is made of the main fabric.

- Side pockets of the jacket – fastened with spiral, reversible, water-resistant (impermeable) zippers with a tooth width of 0.3 ± 0.05 cm. The zipper heads are metal and magnetic. The head of the closed zipper is covered with an additional detail made of the main material at the end of the pocket opening.

- A rectangular pocket (bag with open sides) made of the main material is located on the back of the jacket, into which the jacket can be folded as needed. The bottom of the pocket bag is sewn into the bottom hem of the product. The pocket zipper is reversible, water-resistant (impermeable), with a tooth track width of 0.5 +/- 0.05 cm, with a reversible head so that the zipper can be conveniently closed after packing the product into the pocket bag. The head of the closed zipper is covered with an additional detail at the end of the pocket opening made of the main material.

- For ventilation of the jacket, five holes with a diameter of 2.0-3.0 mm are cut on each side and in the armpits. The edges of the holes must be treated so that they do not fray and remain functional throughout the entire service life of the product.

- The bottom of the sleeves is gathered with elastic tape 2.5 cm ± 0.2 cm wide.

- The width of the bottom of the jacket is adjustable with 1 fastener with 1 bead and an elastic 2.5 mm cord, which is pulled through two inserted metal eyelets with a diameter of 4 mm and runs through the bottom fold seam of the jacket on the left front side. The ends of the elastic cord are attached to the bottom of the product, inside the edges, with a 0.6–0.7 cm wide textile strip. The fastener is attached to the bottom of the jacket with a 0.6 cm wide textile strip.

- The length of the hood is adjustable with 2 fasteners with 2 beads and an elastic cord with a diameter of 2.5 mm, which passes through 4 metal eyelets with a diameter of 4 mm (two on each side of the hood) and is threaded through the hood tunnel. The width of the hood is also adjustable with a fastener and an elastic cord, for which a tunnel is formed in the back of the hood from the base material and 2 metal eyelets with a diameter of 4 mm are inserted. The elastic cord that adjusts the volume and width of the hood is attached to the edges of the hood with a 0.6–0.7 cm wide textile strip.

- All elastic bands and cords must be of high quality and retain their elasticity throughout the entire service life of the product.

- Fasteners and pins must be of high quality, with blunt edges, well and firmly fixed, not fall out, and perform their function properly throughout the entire service life of the product.

- Back details, in the center, a woven curved flag-type label with the image of the flag of the Republic of Lithuania is sewn in. The image is provided in the Annex.

- Light-reflecting strips, green moss color, are sewn into the seams of the jacket front zipper (on both sides of the zipper track). The width of the visible part of the reflective strips is 0.3–0.5 cm. The visible width of the strip must be uniform along its entire length and across the entire product. The edges of the strip must be treated so that they do not fray and the strip fulfills its purpose throughout the entire service life of the product.

- On the front of the jacket, on the left side (chest area), there shall be a yellow shield-shaped print applied by transfer or pigment printing, with a width of 6.8 cm ± 0.2 cm and a height of 7.7 cm ± 0.2 cm; for larger sizes, the print may be 15–20% larger than specified, as shown in the annex. On the back of both sleeves, a reflective print in moss green shall be applied, with an “anti-migration” finish to prevent the base material colour from showing through. The print must be flawless, of high quality, firmly adhered, free from cracking, and must not fade throughout the product’s service life. The exact size, position, and colour of the print must be agreed upon prior to submission of work samples for approval.

- The thickness of the sewing thread must be selected so as to ensure the quality of the seams and the required strength of the seam throughout the entire period of use.

- The color of the sewing threads and other accessories shall match the color of the main fabric of the jacket. The technological processing of the products and the quality of the seams must comply with the general quality requirements for this category/type of work, including those applied to product processing, seams, and materials. Seams must be free from broken threads, unstitched gaps, overlaps, skipped stitches, pulls, stretching, or transverse or longitudinal puckering. Seams must be straight and must not break when tugged. Edges of the product, after folding or hemming, must not show any fraying. Joined materials must be intact and free from punctures. Products must be free of textile defects, thoroughly cleaned, without stains or residual auxiliary marks, without loose thread ends, and properly finished (not wrinkled).

- Basic size of jackets: chest circumference – 96-100 cm; height – 176-182 cm. The measurements of the basic size product and permissible deviations in centimeters are given in Table 3.

Table 3

# SIZE CHART FOR BASE MODEL SPORTS JACKETS

| No. | Dimension name | Value, cm | Permissible deviation, ± cm |
| --- | --- | --- | --- |
| 1 | Back length (measured from the top to the bottom of the middle of the back) | 82,0 | 1,5 |
| 2 | Chest circumference, 1/2 (under the armpits) | 59,0 | 1,5 |
| 3 | Bottom width, 1/2 | 57,5 | 1,5 |
| 4 | Sleeve length (measured from the shoulder seam) | 86,0 | 1,5 |
| 5 | Sleeve width at the top (measured at the armpit), 1/2 | 27,5 | 0,5 |
| 6 | Sleeve width at the bottom (when stretched), 1/2 | 15,5 | 0,5 |
| 7 | Sleeve width at the bottom (unstretched), 1/2 | 12,5 | 0,5 |
| 8 | Width of the bottom yoke | 49,0 | 1,0 |
| 9 | Height of the stand at the centre of the front | 10,0 | 0,2 |
| 10 | Front seam length, 1/2 (measured from the center front to the center back) | 27,0 | 1,0 |

- The length of the front center from top to bottom (including the stand) is 79.0 cm ± 0.5 cm; the length of the front from the shoulder-collar point to the bottom is 72.5 cm ± 0.5 cm.

- The width of the bottom hem (fold) is 2.2 cm ± 0.2 cm. The eyelets are inserted on the left side, in the middle of the fold/hem, 22.0–23.0 cm from the center front; the distance between the eyelets is 3.0 ± 0.2 cm.

- Front-back width at the armpit – 18.0 cm ± 0.5 cm; at the bottom – 21.0 cm ± 0.5 cm.

- Sleeve width at the front neckline – 13.0 cm ± 0.5 cm. Sleeve length at the front elbow area – 5.5 cm ± 0.3 cm; length of the back elbow seam – 9.5 cm ± 0.3 cm. Sleeve bend – 3.0 cm ± 0.2 cm.

- The length of the hood’s central piece (measured at the centre) is 50.0 cm ± 0.5 cm; at the seam – 49.0 cm ± 0.5 cm; width at the collar – 10.5 cm ± 0.3 cm, at the drawstring/tunnel – 7.5 cm ± 0.3 cm, and at the top – 10.5 cm ± 0.3 cm. The height of the hood’s side pieces is 31.0 cm ± 0.5 cm; width at the drawstring/tunnel – 19.5 cm ± 0.5 cm. The drawstring/tunnel width is 3.0 cm ± 0.2 cm; the distance from the collar to the centre of the drawstring/tunnel is 16.0–17.0 cm. The width of the hood binding is 3.0 cm ± 0.2 cm. Eyelets are inserted in the middle of the hood’s central piece and the drawstring/tunnel, as well as on both sides of the zipper at the collar/stand, at a distance of 3.2 cm ± 0.2 cm from the front centre. The distance between eyelets is 3.0 cm ± 0.2 cm.

The length of the side pocket opening is 20.0 cm ± 0.5 cm. The distance from the front centre to the upper end of the opening is 13.5 cm ± 0.5 cm, and from the bottom – 28.0 cm ± 0.5 cm; the distance from the front centre to the lower end of the opening is 15.5 cm ± 0.5 cm. The length of the upper detail of the opening (to cover the zipper pull) is 1.8 cm ± 0.2 cm.

- The length of the back pocket opening is 22.5 cm ± 0.5 cm. The pocket is centred on the back, with a distance of 26.0 cm ± 0.5 cm from the bottom.

- The hanger loop (in the centre of the back collar seam) has a length of 6.5–7.5 cm and a width of 0.8 cm ± 0.1 cm.

- The main measurements of the jackets for all sizes and heights (measurements of products of all sizes/heights) shall be agreed with the winner of the tender upon signing the contract.

CHAPTER III

APPROVAL OF WORKING SAMPLES

- After concluding the contract, working samples are coordinated and approved in accordance with the Lithuanian Armed Forces' samples (working samples must correspond to the Lithuanian Armed Forces' samples in terms of their design and sewing technology).

- The following shall be submitted for approval of the working sample:

  - two identical products (base size);

  - Table of measurements for alignment;

  - product maintenance instructions for coordination (which must be attached to each product);

  - Technical description of the product (with samples and characteristics of the materials used in the manufacture of the product, proving their compliance with the requirements set out in the technical specifications).

- If necessary, the product's dimensions, sewing technology, etc. may be adjusted during the coordination process, provided that this does not impair the product's appearance and functional characteristics.

- Mass production may only commence after the working samples have been approved.

CHAPTER IV

PRODUCT LABELING AND PACKAGING

- Product labeling must comply with the requirements of the Rules for Labeling and Indicating Prices of Goods Sold in the Republic of Lithuania, approved by Order No. 170 of the Minister of Economy of the Republic of Lithuania on 15 May 2002 (in their current version or the version of any legal act amending them, if such has been adopted), as well as with the requirements set forth in this technical specification.

- A marking strip must be sewn onto the inside of each product (the location to be agreed upon during the approval of the working model), indicating:

- service provider name or brand name;

- manufacturer name or brand name (if different from the service provider);

- product name;

- fiber composition;

- size;

- contract date and number;

- batch and shipment index;

- index of the main fabric product batch;

- date of manufacture;

- care label symbols (in accordance with LST EN ISO 3758 (ISO 3758) or an equivalent standard);

- the label “Skalbti be optinių baliklių” (en. “Wash without optical brighteners”);

- the inscription “Pagaminta Lietuvos kariuomenei” (en. “Made for the Lithuanian Armed Forces”).

- The product labeling must indicate that the items can be washed in automatic washing machines (at a temperature of at least 40 °C) and tumble dried. The labeling tag may also include any additional information that the service provider considers necessary for every consumer to know.

- The labeling tags must be made of satin ribbon or an equivalent material (the edges of the ribbon must not be sharp). The information on the labeling tag must remain easily readable throughout the product’s entire period of use.

- Each product shall be labeled with a label indicating

- service provider name or brand name;

- manufacturer name or brand name (if different from the service provider);

- for imported goods, indicate the country of origin of the goods if it is not the same as the country where the manufacturer's registered office is located;

- product name;

- size;

- contract date and number;

- batch and shipment index;

- date of manufacture;

- NSN code.

- Labels must be securely attached, and the labeling information must be of sufficient size to be easily read and understood. The placement of the labels must be chosen so that they can be easily read when the product is packaged in a polyethylene bag.

- Each product must include care instructions in Lithuanian. The instructions may be attached directly to the product or provided via a QR code on the labeling tag (or on a separate tag attached to the product).

- Jackets shall be individually packed in polyethylene bags and placed into sturdy cardboard boxes (resistant to long-term storage and multiple transportations) according to sizes, in quantities agreed between the parties.

- Product packaging must comply with the minimum environmental protection criteria specified in Section II “Packaging” of Annex 2 to the Procedure for the Application of Environmental Protection Criteria in Green Public Procurement, approved by Order No. D1-508 of the Minister of Environment of the Republic of Lithuania on 28 June 2011.

- The weight of cardboard boxes containing products must not exceed 10 kg. Each cardboard box must be labelled with the following clearly visible information:

- service provider name or brand name;

- manufacturer name or brand name (if different from the service provider);

- for imported goods, indicate the country of origin of the goods if it is not the same as the country where the manufacturer's registered office is located;

- product name;

- size;

- quantity;

- contract date and number;

- batch and shipment index;

- date of manufacture;

- NSN code.

- Cardboard boxes containing products of different sizes must be marked with red sheets indicating all the information required for labeling the cardboard box.

- Cardboard boxes shall be delivered stacked on pallets (Euro pallets). Each pallet must have attached a white packaging sheet no smaller than A4 size, displaying the following clearly visible information:

- service provider or manufacturer name,

- product name,

- size,

- number of boxes,

- quantity of goods in one box,

- total quantity of goods on the pallet,

- other information (if necessary).

- Pallets (Euro pallets) on which cardboard boxes with products of different sizes are stacked must be marked with red sheets indicating all the details required on the pallet (Euro pallet) packing sheet.

CHAPTER V

ACCEPTANCE OF GOODS

- Products are accepted in batches and shipments. Each batch must be marked with the contractual mark and accompanied by a declaration of conformity in accordance with LST EN ISO/IEC 17050-1 (ISO/IEC 17050-1) Form A.2 or an equivalent standard template.

- The purchaser shall, in accordance with the contract terms, inspect the quality of products from the selected batch and may carry out laboratory tests. If the results do not meet the requirements specified in the technical specification or the approved work samples, the entire batch delivered at that time shall be rejected.

In case of differences in interpretation the Lithuanian text shall prevail

Technical specifications of sports jackets

ANNEX

SPORT JACKET MODEL

LABEL WITH THE IMAGE OF THE FLAG OF THE REPUBLIC OF LITHUANIA,

Woven,

Colors heraldic

THE EMBLEM OF THE LITHUANIAN ARMED FORCES