# Annex 2 "Technical Specification“

TECHNICAL SPECIFICATION

(PU-14859/26) [ITP26] Salt spreading and watering equipment for Category N3 vehicles

- TERMS AND ABBREVIATIONS

  - Buyer – AB Kelių priežiūra.

  - Supplier – an economic entity, including a natural person, private or public legal entity, other organizations and their divisions, or a group of such persons, with whom the Buyer concludes a Contract.

  - Contract – a contract concluded between the Buyer and the Supplier regarding the subject of the procurement.

  - Name of the purchase object – Salt spreaders (hereinafter – the Goods/Item or the Equipment)

- DESCRIPTION, SCOPE, AND REQUIREMENTS OF THE SUBJECT OF THE PROCUREMENT

  - The procurement is not divided into lots.

  - Description of the procurement object:

    - The Goods must comply with the technical requirements set out in the annexes to these Technical Specifications.

    - The following must be provided together with the Goods (no later than on the date of delivery of the Goods): operating and work safety instructions in Lithuanian or English, electrical installation diagrams, hydraulic diagrams, spare parts catalogs,  CE declaration of conformity, detailed installation instructions allowing the Buyer to perform the installation work independently, technical maintenance instructions in Lithuanian or English for the Buyer's qualified personnel to perform technical maintenance, warranty and repair work.

    - The proposed Equipment must be covered by a full warranty of not less than 12 months, calculated from the date of delivery and acceptance of the Goods. The offer must be accompanied by the manufacturer's confirmation of the warranty provided. The Goods are supplied without installation. Installation work on the Buyer's vehicles is carried out by the Buyer itself in accordance with the installation instructions and diagrams provided by the Supplier. Before starting the installation work, the Supplier or the equipment Manufacturer must train the Buyer's technical staff on the correct installation, disassembly, operation, and maintenance of the equipment. After training, the equipment manufacturer shall confirm in writing that the warranty obligations for the Equipment remain valid. The warranty is valid even if installation, periodic maintenance, including warranty and other work related to the Equipment maintenance, is performed by the Buyer's qualified personnel in accordance with the instructions provided by the Supplier.

    - During the warranty period, all anticipated mandatory technical maintenance work on the Goods and the maintenance kits required for such work (spare parts and materials required for specific maintenance) must be detailed and submitted together with the offer, specifying the scope of each service and the prices of service kits for the entire warranty period, taking into account the condition that the spreader will be operated for at least 2000 working (moto) hours per year. The buyer is not obliged to purchase all planned service kits if the relevant service is not yet required based on the actual operating time or mileage of the spreader. During the warranty period, the Supplier shall perform warranty repairs at the Buyer's premises at its own expense (together with the offer, the Supplier must submit the manufacturer's authorization for the proposed Goods or another document confirming the Supplier's right to perform repair services during the warranty period, or provide evidence of a contract for the provision of relevant services with another economic operator having such a right) or, together with the offer, submit a document confirming that the Buyer or other legal entities are entitled to perform warranty repairs during the warranty period, performing warranty repairs in accordance with the manufacturer's requirements. In cases where the right to perform warranty repairs is transferred to the Buyer, the Supplier undertakes to cover all costs related to the performance of warranty repairs without exception, including, but not limited to: labor, spare parts, materials, diagnostics, transportation, and other necessary expenses, by paying the invoice issued by the Buyer to the Supplier. All warranty period work performed by the Buyer shall not affect the validity of the warranty if the work is performed in accordance with the Supplier's instructions and the diagrams provided.  During the warranty period, the Supplier shall provide the necessary spare parts and materials for routine and seasonal technical maintenance, and the Buyer shall perform such maintenance using its own resources. If special equipment is required for warranty, diagnostic, technical maintenance, or other work with the Goods,  the Supplier shall provide the Buyer with one set of the manufacturer's diagnostic equipment and ensure free updates of all software components of the Goods and diagnostic equipment for a period of not less than 10 years, ensuring that the Buyer is able to perform these works independently in accordance with the instructions provided by the Supplier.

    - Upon delivery of the Goods, but no later than when the Buyer begins to use the Equipment or part of the Equipment set, The Supplier must provide the following training, the cost of which must be included in the offer price:

- theoretical, practical, and technical training for operators to ensure the correct and safe operation and maintenance of the Equipment, explaining the nature of faults and fault symbols.

- theoretical, practical, and technical training for the Buyer's technical personnel on the proper, correct, and safe installation, dismantling, and preservation of dismantled equipment, performance of technical maintenance and repair work, and other work related to the maintenance of the Goods, including training on the correct and safe maintenance of the Goods, explaining the nature of faults and fault symbols.

2.2.6. Before the end of the warranty period for the Goods, the Supplier shall, at its own expense, perform a detailed technical inspection of the Goods and eliminate any warranty defects found, or authorize the Buyer to perform such an inspection and eliminate any warranty defects found in accordance with the instructions provided by the Supplier, and the costs incurred shall be compensated in accordance with the procedure set out in the contract.

  - Scope of the Procurement:

| Ser. No | Equipment name | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Salt spreaders for Renault C four-wheel drive vehicles (8x4 (2+2)) | Unit | 44 |

  - The Buyer purchases the Goods  with delivery. The Supplier undertakes to deliver the Goods free of charge to the specified address:

| Delivery address | Quantity in sets |
| --- | --- |
| Kauno str. 72, Pagirių village, Garliava municipality, 53282 Kaunas district | 44 |

3. PROCEDURE AND TERMS FOR THE FULFILLMENT OF CONTRACTUAL OBLIGATIONS

  - Goods must be delivered no later than 150 calendar days  from the date of entry into force of the Contract.

  - The delivery time of the Goods is assessed in terms of economic efficiency, taking into account the principles set out in the tender evaluation criteria. If the Supplier fails to comply with the proposed delivery times for the Goods, the Buyer will apply late payment penalties until the Goods are delivered.

  - Individual sets of Goods may be delivered before the delivery date specified in the offer. The Supplier is obliged to specify the expected delivery date for each complete set in the offer.

4. ENVIRONMENTAL REQUIREMENTS

The Buyer seeks to ensure that its and the Supplier’s actions have as little impact on the environment as possible, therefore:

4.1.  During the public procurement and contract performance, communication between the Supplier and the Buyer shall be carried out only by electronic means (CPP IS means, telephone, e-mail, etc.);

4.2.  All documentation related to the performance of the Contract shall be provided to the Buyer and the Supplier by electronic means (e-mail, etc.);

4.3. The Contract shall be signed only by electronic means (electronic signature).

4.4. The Supplier shall undertake to reduce paper consumption, refrain from unnecessary copying and printing of documents, and if office supplies are used, they must be made from recycled raw materials or suitable for recycling.

4.5.  If the purchased Goods are to be delivered or transferred in secondary packaging, they must comply with the minimum environmental protection criteria set for packaging, unless this is contrary to hygiene standards. Packaging must be considered recyclable packaging in accordance with the provisions of the Republic of Lithuania Law on Environmental Pollution Tax.

4.6.  If the Goods purchased are delivered, the Supplier shall endeavour to ensure that the delivery of the Goods consumes fewer natural resources and thus complies with the environmental protection principle established by the Order of the Minister of the Environment of the Republic of Lithuania, i.e. to ensure that the optimal route is chosen when delivering the Goods to the delivery location specified by the Buyer.

5. ANNEXES

Annex 1 – Technical parameters

Annex 2 - Chassis visualization

__________

Note: All references in the procurement document to a standard, technical certificate, or general technical specifications imply that the Purchaser accepts equivalent evidence of conformity for the Goods offered by suppliers. The burden of proof regarding equivalence lies with the supplier.

Annex 1 – Technical parameters

TECHNICAL PARAMETERS

The following must be submitted:

Technical documentation from the manufacturer of the Item/Equipment (catalogues, brochures) and/or declarations from the manufacturer of the Item/Equipment (if the manufacturer’s technical documentation does not fully reflect the compliance of the offered Item/Equipment with the technical specification requirements) and other documents specified in the technical specifications or other equivalent documents proving the compliance of the offered Item/Equipment with the technical requirements. In these documents, the Supplier must indicate (i.e., clearly mark – highlight in colour and/or indicate with arrows, and/or underline, etc.) the specific locations in the documents provided where the required technical characteristics are described and indicate which point of the technical requirements they comply with.

OR

links to a publicly accessible website where the Buyer can verify the authenticity of the data provided, i.e., the compliance of the offered Item/Equipment with technical requirements. If the information provided on the specified website does not correspond to the data declared by the Supplier, the tender must be accompanied by a declaration from the manufacturer of the Item/Equipment or other equivalent documents confirming that the offered Item/Equipment comply with the technical specification requirements.

The Supplier’s declaration shall not be considered equivalent documentation, except in cases where the Supplier is the official representative of the manufacturer of the offered Item/Equipment.

In the event that the Supplier (who is not the manufacturer) submits technical documentation and/or declarations prepared by representatives of the manufacturer who have the relevant rights, the Supplier must also submit documents proving the relevant rights of the manufacturer’s representative together with the offer (authorization granted by the manufacturer of the Item/Equipment to prepare/approve technical documentation for the Item/Equipment).

If the technical documentation for the Item/Equipment is prepared by a Supplier who is not the manufacturer and who has not been granted the manufacturer’s authorization to prepare/approve technical documentation, in such a case the technical documentation prepared by the Supplier itself does not meet the criteria of objectivity, reliability, traceability of information, and other criteria, therefore, during the evaluation of the tender, the Buyer will consider that the Supplier has only provided declarative information that is not substantiated by objective data and will reject such a tender.

## Documents (technical documentation of the manufacturer of the Item/Equipment (catalogues, brochures) and/or declarations by the manufacturer of the Item/Equipment or other equivalent documents proving that the offered Item/Equipment comply with the technical characteristics) may be submitted in Lithuanian and/or English. When evaluating the Suppliers’ tenders and at the Buyer‘s request, the Suppliers shall be required to submit the specified documents or parts thereof, translated into Lithuanian, and a confirmation of the translation.

## All specified specific materials, solutions and/or specific names or standards of the Item/Equipment are subject to the “or equivalent” clause. A Supplier offering the Item/Equipment with equivalent characteristics must additionally indicate this in the technical specification and/or tender and prove by reliable means that the offered Item/Equipment is equivalent and fully complies with the requirements set out in the technical specification.

SALT SPREADERS FOR RENAULT C FOUR-WHEEL DRIVE VEHICLES (8X4(2+2)) (44 sets)

| Ser. No. | Characteristic name | Characteristics required by the Buyer | The Supplier must confirm compliance with the technical requirement by indicating: Yes/no and, where required, the exact value of the proposed Equipment, the name of the supporting document and/or a reference to the source confirming the proposed parameters. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1. | Purpose. | The equipment is designed for spreading anti-skid materials and spraying brine on the road surface. The salt spreader is adapted for installation and operation in place of a tipping body (40 units) and in place of a Push-Off horizontal push-off body (4 units), using a bracket whose mounting position on the chassis corresponds to the body mounting and fastening points, with four-axle Renault C trucks. The visualization of the chassis and the main dimensions are presented in the drawings in Appendix No. 2 to the Technical Specifications and are for informational purposes only. Deviations from the dimensions shown in the drawings are possible up to ±10%, depending on the actual dimensions of the Renault C truck chassis supplied. Upon signing the contract, the Buyer undertakes to cooperate with the Supplier and enable accurate measurements to be taken on specific chassis. These measurements are taken to ensure the accurate design and manufacture of the coupling and the proper integration of the equipment into the specific chassis. | Yes/No (delete as appropriate). |
| 1.2. | Year of manufacture | Salt spreaders must be new, unused prior to delivery, and manufactured no earlier than 2026. | Yes/No (delete as appropriate). Year of manufacture, quarter - _________(enter). |
| 1.3. | The dominant color of painted metal surfaces on salt spreaders | Orange (RAL 2011), gray (Pantone 7544C) or the manufacturer's default (representative) color for a specific group of equipment. Galvanized metal surfaces, elements and assemblies made of stainless steel, aluminum or other corrosion-resistant materials may be left unpainted if this is not technically necessary, , provided that all materials and surfaces used are adequately protected and adapted for use in a salty and humid environment, ensuring long-term corrosion resistance, and that welds are properly treated. | Yes/No (delete as appropriate). Suggested dominant color - (enter parameter) _____________. |
| 1.4. | Guarantee against perforation corrosion | For all components included in the set without exception - at least 72 months. | Yes/No (delete as appropriate). Proposed parameter - ____ months |
| 1.5. | Right to perform warranty repairs | The Supplier must be the official representative of the manufacturer of the goods offered (if it is not the manufacturer itself), having the right to repair of the goods offered during the warranty period, or have concluded a contract for the provision of relevant services with another economic entity having such a right. The proposal must be accompanied by the manufacturer's authorization or other document confirming the Supplier's right to perform repair services during the warranty period, or evidence of a contract for the provision of relevant services with another economic operator having such a right or the Supplier must submit a power of attorney issued by the manufacturer of the Goods granting the Buyer the right to perform repair services during the warranty period. | Yes/No (delete as appropriate). Title of the document submitted as evidence: _________ and page no.. |
| 1.6. | Warranty repair terms | The Supplier must perform warranty repairs within 5 business days after the Buyer submits a request in writing (by email) and by phone call to the Supplier‘s responsible person specified in the contract. or may authorize the Buyer during the warranty period (upon presentation of a power of attorney issued by the manufacturer of the Goods) to have warranty repairs performed by qualified employees of the Buyer and to compensate for the costs of the work and other related expenses in accordance with the terms and conditions set forth in the contract. | Yes/No (delete as appropriate). Title of the document submitted as evidence: _________ and page no.. |
| 1.7. | Documentation | Operating and safety instructions in Lithuanian or English shall be provided no later than on the date of delivery of the goods. | Yes/No (delete as appropriate). |
| 1.8. | Training | Upon delivery of the Goods, but no later than when the Buyer begins to use the Goods or part of the Goods set, the Supplier must provide the following training, the cost of which must be included in the offer price: - theoretical, practical, and technical training for operators to ensure the correct and safe operation and maintenance of the Equipment, explaining the nature of faults and fault symbols. - theoretical, practical, and technical training for the Buyer's technical personnel on the proper, correct, and safe installation, dismantling, and preservation of dismantled equipment, performance of technical maintenance and repair work, and other work related to the maintenance of the Goods, including training on the correct and safe maintenance of the Goods, explaining the nature of faults and fault symbols. | Yes/No (delete as appropriate). |
| 1.9. | Certificate of approval | The Supplier (including partners – Suppliers of individual components of the set) shall prepare a certificate describing the scope of the training course for operators and technical personnel who have completed theoretical and technical training and practical training. | Yes/No (delete as appropriate). |
| 2. SALT SPREADER |  |  |  |
| 2.1. | Purpose, application Type, brand | Suitable for a four-axle truck with a wheel configuration of 8x4 (2+2), wheelbase of at least 3700 mm, mounted on an additional chassis frame, instead of the body (An information drawing of the vehicle on which the equipment will be mounted is provided in Appendix No. 2 to the Technical Specifications). Designed for spreading salt, with or without irrigation, sand, fine gravel, and/or spraying brine on road surfaces. Must comply with the requirements of standards LST EN 15597-1:2020 and LST EN 17106-3-2:2021. The spreader must be a standard Equipment manufactured by the manufacturer in series, not specially modified or adapted to meet the requirements specified by the contracting authority, except for specific marking and safety requirements. The additional chassis frame (subframe) is an integral part of the product package. Together with the offer, the Supplier shall submit a drawing indicating the center of gravity of a fully loaded salt spreader and the distribution of loads on the axles with the maximum permissible load: first (steered) axle – 9 tons, second and third axles – 12.5 tons each, rear fourth, steered and liftable axle – 9 tons. The visualization of the chassis and the main dimensions are presented in the drawings in Appendix No. 2 to the Technical Specifications and are for informational purposes only. Deviations from the dimensions shown in the drawings are possible up to ±10%, depending on the actual dimensions of the Renault C truck chassis supplied. Upon signing the contract, the Buyer undertakes to cooperate with the Supplier and enable accurate measurements to be taken on specific chassis. These measurements are taken to ensure the accurate design and manufacture of the coupling and the proper integration of the equipment into the specific chassis. | Yes/No (delete as appropriate). Precise spreader model - _________. Title of the document submitted as evidence - _________ and page no.___________ or reference - ________. |
| 2.2. | Hydraulic system | A quick-connect and activation system for the hydraulic system of the salt spreader must be installed. Hydraulic connections provided, installed, and marked accordingly by the manufacturer. An additional control line and load sensor controller provided by the manufacturer for a load-sensitive truck system (Load Sensing or equivalent). The spreader must be driven by the truck's Load Sensing type hydraulic system, without additional hydraulic units or a separate autonomous engine. | Yes/No (delete as appropriate). Title of the document submitted as evidence - _________ and page no.___________ or reference - ________. |
| 2.3. | Electrical system | A quick connection and activation system for the salt spreader's electrical system must be installed. Electrical connections must be provided, installed, and marked accordingly by the manufacturer. All electrical equipment and control systems of the spreader must be adapted to operate from a 24 V DC truck electrical system. An emergency Equipment stop button is installed to stop all spreader operations. The button is red and has a diameter of at least 24 mm. | Yes/No (delete as appropriate). Title of the document submitted as evidence - _________ and page no.___________ or reference - ________. |
| 2.4. | Center of gravity of the mounted spreader | Together with the offer, the Supplier shall submit drawings indicating the center of gravity of the fully loaded and brine-filled salt spreader and the distribution of loads on the axles with the maximum permissible load: first (steered) axle - 9 tons, second and third axles - 12.5 tons each, rear fourth, steered and liftable axle - 9 tons. The visualization of the chassis and the main dimensions are presented in the drawings in Appendix No. 2 to the Technical Specifications and are for informational purposes only. Deviations from the dimensions shown in the drawings are possible up to ±10%, depending on the actual dimensions of the Renault C truck chassis supplied. Upon signing the contract, the Buyer undertakes to cooperate with the Supplier and enable accurate measurements to be taken on specific chassis. These measurements are taken to ensure the accurate design and manufacture of the coupling and the proper integration of the equipment into the specific chassis. | Yes/No (delete as appropriate). Title of the document submitted as evidence - _________ and page no.___________ or reference - ________. |
| 2.5. | Control of spreader removal/installation on truck systems (storage legs) | The lifting/lowering of the hydraulic, mechanical, or similar devices of the salt spreader storage legs is performed by a manual, hydraulic, or similar drive lifting device or by the hydraulic system of a truck. | Yes/No (delete as appropriate). Proposed parameter - ____ Title of the document submitted as evidence - _________ and page no.. ____________or reference -________. |
| 2.6. | Upper overall dimension of the spreader. | The height of the salt spreader shall not exceed 2,550 mm. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. Title of the document submitted as evidence - ____ and page no.. _______or reference -________. |
| 2.7. | Weight of spreader, including all integral equipment. | 2 500 – 4 000 kg | Yes/No (delete as appropriate). Proposed parameter - ____ kg. Title of the document submitted as evidence - ____ and page no.. ____or reference -________. |
| Material hopper |  |  |  |
| 2.8. | Material hopper | Coated with anti-corrosion coating and painted or made of stainless steel. The painting process for metal surfaces must be carried out in accordance with EN ISO 12944 C3M or similar standards. The coating thickness must be at least 180 µm. The color shall be orange (RAL 2011), gray (Pantone 7544C), or the manufacturer's specified base (representative) color for a specific group of equipment. Additional anti-corrosion coating and corrosion protection shall be provided for at least the following components: solution pump, hydraulic blocks, hydraulic pumps, hydraulic motors, quick-connect couplings, gas springs, frame, entire spreader structure, and other components susceptible to corrosion during operation. | Yes/No (delete as appropriate). Proposed material and color- _____. Title of the document submitted as evidence - _________ and page no.. ___________or reference -________. |
| 2.9. | Material hopper capacity | The maximum load capacity for bulk materials is at least 9 m³ | Yes/No (delete as appropriate). Proposed parameter - ___ m3. Title of the document submitted as evidence - ___ and page no.. ___or reference -________. |
| 2.10. | Spreading material feeding device. | Rubber belt conveyor - ensures smooth feeding of materials to the spreading disc, regardless of the type and quantity of material in the hopper. | Yes/No (delete as appropriate). Title of the document submitted as evidence - ____ and page no.. ____or reference -______. |
| 2.11. | Mechanism for regulating the amount of material to be spread | A manually operated mechanism for regulating the amount of spreadable material supplied, designed and installed by the manufacturer. Priority requirement T3. Additional economic efficiency points are awarded for: An electric or other drive device that automatically regulates the dosage of material according to the spreading material selected on the control panel. | Yes/No (delete as appropriate). Proposed parameter, description - ________________. Title of the document submitted as evidence:_ , p. No.. __and reference:__. |
| 2.12. | Covering the Material hopper | Covered with a metal grate (opening dimensions: not less than 50 × 50 mm and not more than 70 × 70 mm, wire/rod diameter not less than 4 mm) and a moisture-proof tarpaulin that can be easily closed and opened to the sides without the use of additional tools. The levers of the tarpaulin folding/unfolding mechanism must be accessible to the operator (the covering/uncovering process must be possible) while standing on the ground or using the folding steps installed at the rear of the spreader, ensuring that the grate structure is mechanically strong enough to safely withstand operational loads, including the presence of up to two operators on the grate at the same time with a total weight of up to 300 kg, as well as the simultaneous loading of bulk material of not less than 5 tonnes. The grate structure shall be protected against corrosion to ensure a corrosion resistance of not less than EN ISO 12944 C3M class or equivalent, suitable for operation in an environment exposed to salt and salt solutions. Priority requirement T23.Service steps and/or ladders integrated into the overall spreader design. Complies with EN 17106 standard. Ribbed step base for slip resistance. | Yes/No (delete as appropriate). Steps/lladders ________________ Title of the document submitted as evidence -___ and page no.. ____or reference -_____. |
| 2.13. | Protection against spilled material falling down | The lower part of the material Material hopper shall be equipped with protection or, at the Supplier‘s expense, protection shall be installed above the upper part of the truck frame to prevent the material being transported from falling directly onto the truck chassis and transmission components located between the longitudinal side profiles of the frame. It is possible to wash away the residues of the spread material accumulated on the protective device with a water jet without using additional tools (for removing the bottom (protective) nut). | Yes/No (delete as appropriate). Title of the document submitted as evidence - ____ and page no.. ___or reference -_____. |
| Brine tanks capacity |  |  |  |
| 2.14. | Brine tanks capacity | Brine tanks made from corrosion-resistant material. Brine tanks are installed on the sides of the spreader material hopper and at the front of the spreader. Brine tank fill indicator - Float or similar system. The total volume of brine tanks is not less than 7.4 m³. Priority requirement T11. Electronic indicator showing the fill level of the brine tanks. The indication must be at least every 10%. | Yes/No (delete as appropriate). Proposed parameter - _____ m³. Brine tank fill level gauge - _______ Title of the document submitted as evidence - _____ and page no.. ______or reference -________. |
| 2.15. | Materials used in production | Containers must be made of polypropylene or other equivalent polymeric material resistant to corrosion, temperature fluctuations, and ultraviolet radiation. The resistance of polymer materials used in brine tanks to ultraviolet rays and other environmental conditions must comply with the DIN EN-17106 standard. | Yes/No (delete as appropriate). Proposed parameter – complies with requirement _____ . Title of the document submitted as evidence - ____ and page no.. ____or reference -___. |
| Control |  |  |  |
| 2.16. | Control of salt spreading and irrigation equipment | Performed from the vehicle cab, using a digital control panel that is easily accessible to the driver from his workplace. | Yes/No (delete as appropriate). |
| 2.17. | Manual control | The manufacturer has provided the option of controlling the spreading process manually (in case of emergency). | Yes/No (delete as appropriate). Title of the document submitted as evidence - ___ and page no.. ____or reference -______. |
| 2.18. | Control panel | The spreader control panel must ensure the automatic spreading and/or spraying function according to the selected and pre-programmed route. The operator in the cab, in a convenient location, has a control panel adapted for safe installation, which must be able to continuously display and control at least the following parameters during the shift:- Selected spreading material and spreading rate; - Spread width and asymmetry; - Moistening of spread material with brine on/off, selected moistening percentage (%); - Brine spraying on/off, selected amount of sprayed material (ml/m² or g/m²) (depending on manufacturer settings); The control panel must: - collect data and ensure digital scanning and real-time transmission of data to the dispatcher's workplace; - inform the operator about a shortage of spreading materials or brine with an additional audible signal; - have connectors and an RS-232 data cable for data transmission in accordance with data encryption standard EN15430-1 or equivalent, and provide access to standard functions, including data collection and transmission. If a license is required for this function, it shall be unlimited in scope and valid for a period of at least 10 years. - have a menu in Lithuanian. The control panel menu must display the following names of the spreadable substances (may be without Lithuanian symbols, e.g., Misinys 1:1): • Salt • Mixture 1:1 • Mixture 1:3 • Mixture 1:10 • Brine The Supplier shall provide the Buyer with the necessary access codes to adjust the control panel settings. The meanings of the console buttons must be provided in the user manual. The control panel must be connected to the spreader via a detachable connection on the outside of the truck cab, allowing the spreader to be easily disconnected when not in use (e.g., during storage). The lengths of both cables must be selected according to the design of the specific truck, cabin layout, and equipment installation location, ensuring safe, neat, and reliable installation and operation of the console, with sufficient length reserve to allow for installation, maintenance, and operation without cable tensioning, extension, or the use of additional connectors. All connections and cable ends must have protective covers to ensure protection from environmental influences when not in use and be resistant to corrosion, salt solution, and other operational influences. | Yes/No (delete as appropriate). Title of the document submitted as evidence - _________ and page no.. ___________or reference -________. |
| 2.19. | Priority requirement T21 - Interface of spreading equipment with external road condition information systems and their stations | The spreading equipment (control panel, software, etc.) must have a real-time interface/integration with the road condition information system and KOS stations located in the Lithuanian road infrastructure (Klimator or equivalent). The spreading equipment must have the functionality to analyze or compare the data obtained from the existing interface with the road condition information system (approaching, recorded changes in weather conditions) and, accordingly, automatically adjust the spreading machine's operating parameters according to a predetermined algorithm (dosing of anti-skid materials, spreading/watering width, etc.) in order to ensure timely preventive preparation of the road surface for approaching/recorded changes in weather conditions. | Yes/No (delete as appropriate). Title of the document submitted as evidence - ____ and page no.. _____or reference -_____. |
| 2.20. | Data transfer system | The Supplier must provide access to the control panel data transmission, at least for the following data (for the purpose of controlling the spreader's operation): - Date, time, working hours. - Spreader operating parameters: spreading on/off or spreading mode, spreading time, spreading width, name of material being spread, dosage of material being spread g/m², amount spread kg, brine dosage ml/m² or g/m² (depending on manufacturer's specifications), amount of brine dispensed ltr - Distance traveled during spreading or spraying. Transport telemetry control services are provided to the Buyer by UAB Xirgo Global or equivalent. The proposal must be accompanied by information that clearly and comprehensively shows the required data. | Yes/No (delete as appropriate). Title of the document submitted as evidence - _________ and page no.. ___________or reference -________. |
| 2.21. | Generation of data transfer system reports | Reports must be generated with freely selectable dates and times. They must include: - The date of spreading, the start/end time of spreading. - Start/end point of spreading, total spreading time, spreading distance, consumption of spreading material and brine (kg, ltr). - Dosing of the material to be spread in g/m², dosing of the brine in ml/m² or g/m² (depending on the manufacturer's specifications). - Distance traveled by the vehicle and spreader, m. - Names of materials used. The proposal must be accompanied by detailed information on the reports to be produced and the data required data. | Yes/No (delete as appropriate). Title of the document submitted as evidence - _________ and page no.. ___________or reference -________. |
| 2.22. | Even spreading and feeding of brine. | The vehicle speed signal must be transmitted to the spreader control panel (linked). The even supply of salt and brine to the spreading disc, the even rotation of the disc and the spreading of materials, according to the settings on the control panel, must take place independently of the vehicle's speed, as well as when the vehicle is stationary. | Yes/No (delete as appropriate). Title of the document submitted as evidence -___ and page no.. ____or reference -______. |
| 2.23. | Unloading system | Must be provided by the manufacturer and activated via the control panel. | Yes/No (delete as appropriate). The title of the document submitted as evidence is__ and page no.__or reference -_. |
| 2.24. | Spreading parameter programming and fault notification system. | Must be provided and installed by the manufacturer. | Yes/No (delete as appropriate). Title of the document submitted as evidence - ____ and page no.. ______or reference -________. |
| 2.25. | Instantaneous increase in the amount of spreading. | A function that allows the amount of material spread to be increased two or more times for a limited period of time. | Yes/No (delete as appropriate). Title of the document submitted as evidence - ____ and page no.. _______or reference -________. |
| Spreading mechanism |  |  |  |
| 2.26. | Spreading materials and brine mixing system. | Ensures mixing of the spreading material and brine and delivery of the mixture onto the spreading plate, or mixing of salt and brine directly on the spreading plate. | Yes/No (delete as appropriate). Title of the document submitted as evidence - ____ and page no.. ______or reference -________. |
| 2.27. | Salt moistening and spreading mechanism | Variable length arm/gutter and plate with hydraulic drive, can be lifted up and locked in transport position. Air shock absorbers or an equivalent solution provided and installed by the manufacturer to facilitate lifting the mechanism into the transport position. | Yes/No (delete as appropriate). Title of the document submitted as evidence - ____ and page no.. _______or reference -________. |
| 2.28. | Spreading disc (plate) | Diameter not less than 500 mm. The height of the disc above the road surface is adjusted mechanically (manually) in increments of at least 100 mm. The spreading disc must ensure even spreading at speeds of at least 55 km/h. The spreading disc and the working area lighting system consist of 1 LED lamp. Priority requirement T19 The work area lighting system consists of at least 2 LED lights. | Yes/No (delete as appropriate). Proposed parameter - ____ mm. (Plate diameter) Proposed parameter - ____ km/h. Proposed parameter - ____ (number of LED lights). Title of the document submitted as evidence - _____ and page no.. ___________or reference -________. |
| 2.29. | Spreading disc hydraulic motor | Protected from direct exposure to salt. | Yes/No (delete as appropriate). Title of the document submitted as evidence - ____ and page no.. _______or reference -________. |
| 2.30. | Salt spreader sensor | It must be contactless and electronic. | Yes/No (delete as appropriate). Title of the document submitted as evidence - _________ and page no.. ___________or reference -________. |
| 2.31. | Adjustable amount of material spread per m² | Salt – within the range of 5-40 g/m², in increments of no more than 5 g/m². Spread sifted sand and gravel within the range of 20-300 g/m², with a maximum "step" of 10 g/m². | Yes/No (delete as appropriate). Proposed parameter: Salt _____ g/m2 limits. Sifted gravel, sand, crushed stone - within the range of ____ g/m2. Title of the document submitted as evidence - ____ and page no.. ______or reference -________. |
| 2.32. | Adjustable spreading width of the material to be spread. | From 3 m to at least 11 m, at intervals of no more than 1.0 m, with asymmetrical (increasing or decreasing the width of one side of the spread without changing the width of the other side) adjustment of the spread material, set using the control panel.. | Yes/No (delete as appropriate). Proposed parameter: Within a range of ____ m, the "step" of the spreading width is __ m. Title of the document submitted as evidence - ____ and page no.. _______or reference -________. |
| 2.33. | Forced (pressure) brine delivery (to the spreading disc (plate)) and quantity control. | In 5% increments, starting from a minimum of 15% up to 100% (FS 15 - FS 100). The amount of brine dispensed automatically changes depending on the amount of salt being replaced. It is assumed that the percentage of brine used for salt irrigation indicates the proportion of the brine in the application rate (e.g. FS15 means that the application rate consists of 15% brine + 85% dry material used for spreading, i.e., the salt used). | Yes/No (delete as appropriate). Proposed parameter: __ % interval from ___ % Title of the document submitted as evidence - ____ and page no.. ______or reference -_______. |
| Brine spraying (dispersion) mechanism |  |  |  |
| 2.34. | Brine spraying (dispersal) mechanism | The brine spraying mechanism is installed in the rear part of the salt spreader. The mechanism units and components are mounted on a bracket connected to the spreading mechanism or on a separate bracket(s). The mechanism is lifted up and locked in the transport position together with the spreading mechanism or separately. | Yes/No (delete as appropriate). The title of the document submitted as evidence is ________ and page no. _____or reference - _______. |
| 2.35. | Brine pumps | Pumps made of stainless steel AISI316 or other equivalent materials resistant to the effect of a brine. Pump capacity is controlled by a control panel: -pump capacity used for brine spraying function, in the range from 1 l/min to at least 290 l/min; -the capacity of the pump used for the brine for the salt humidification function is in the range from 1 l/min to at least 100 l/min. Priority requirement T26 For brine pumps that do not require additional flushing or special maintenance before the preservation period | Yes/No (delete as appropriate). Proposed parameter: pump material _________ Pump type ______ capacity in the range from_______ltr/min to_________ltr/min from_______ltr/min to_________ltr/min The title of the document submitted as evidence is ____ and page no. _______or reference - _______. |
| 2.36. | Spray widths | The management system must ensure: - different spray widths - (symmetrical) width from 3.0 to at least 11.0 m, and asymmetrical, at least 6.0 m to the left and right. - the ability to adjust each side separately, changing the width of the sprayed material in increments of no more than 1.0 m, independently of the other side; - even spraying regardless of the vehicle's speed when driving at speeds of at least 80 km/h. | Yes/No (delete as appropriate). Proposed parameters: Symmetrical width from ___ m to ___ m. Asymmetrical width ___ m The title of the document submitted as evidence is _________ and page no. __________or reference - ___________. |
| 2.37. | Dosage | The amount of sprayed brine is adjustable: from 10 ml/m² or g/m² to at least 100 ml/m² or g/m² (depending on the manufacturer's specifications). The dosage "step" should not exceed 5 ml/m² or g/m². | Yes/No (delete as appropriate). Proposed parameters: from ___ ml/m² or g/m² to ___ ml/m² or g/m². Title of the document submitted as evidence - ____ and page no. _____or reference - ______. |
| Protective fence |  |  |  |
| 2.38. | Protective fence – lightweight construction, 1–1.15 m high, on both sides of the spreader hopper. | Mounted on top of the salt spreader hopper and lifted, folded back for maintenance/repair before reaching the hopper. The fence is raised/lowered by lowering/raising a ladder designed for climbing onto the hopper or by a separate lifting/lowering mechanism.. | Yes/No (delete as appropriate). Title of the document submitted as evidence - ___ and page no.. _______or reference -________. |
| Signaling marking |  |  |  |
| 2.39. | Dimension marking, signal lighting | Lighting of the working area (spreading/distribution) with at least one LED lamp. At least one orange LED beacon, installed at the rear of the spreader. Two flashing LED lights (round flashers)with a diameter of at least 180 mm shall be installed at the upper corners of the rear of the spreader. The rear section is marked with warning markings consisting of diagonal red and white stripes that reflect light. Sample warning marking pattern: Protection of LED beacon and flashing LED lights against solid objects and liquids, not less than IP65. | Yes/No (delete as appropriate). Title of the document submitted as evidence - _________ and page no.. ___________or reference -________. |
| - ADDITIONAL PRIORITY REQUIREMENTS |  |  |  |
| 3.1. | Priority requirement T4 Protection of the hydraulic system during parking | Due to temperature changes and other operating conditions, the hydraulic system of the salt spreader is equipped with an integrated expansion tank and pressure relief device, ensuring safe pressure equalization during parking. | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.2. | Priority requirement T5 Mechanism for mixing the spread material | A separate mechanism installed along the entire length of the hopper, with a separate hydraulic drive or other type of drive and drive control, which distributes and mixes the spread material more evenly inside the hopper. | Yes/No (delete as appropriate). Type and description of the proposed equipment - ____. Title of the document submitted as evidence: _ , p. No.. __and reference:__. |
| 3.3. | Priority requirement T6 Control panel storage box | A storage box is provided for safe storage of the control panel when the equipment is not in use. | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.4. | Priority requirement T7 Mechanism for evenly distributing spreadable materials on a conveyor | Mechanism ensuring uniform distribution of the spreading material onto the conveyor along the entire length of the hopper, so that during the entire operation local material accumulation, uncontrolled or excessive pressure on the conveyor belt and other conveyor components, deformation, accelerated wear or jamming are avoided, controlled interaction between the spreading material and the conveyor mechanism is ensured, preventing the material from pressing on the conveyor belt during operation, and guaranteeing conveyor durability as well as stable and uninterrupted operation of the spreader throughout the entire operating cycle, regardless of the quantity and physical properties of the spreading material. | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.5. | Priority requirement T8 Splash protection | Mounted on the rear of the truck and on the spreading disc of the salt spreader to protect the vehicle and equipment during salt operation. The guards ensure reduced corrosion risk and longer service life for the vehicle and spreader. | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.6. | Priority requirement T9 Funnel vibrator | The installed funnel vibrator is designed to ensure that the scattered material does not accumulate and stick to the walls, thus ensuring a constant and even flow of material | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.7. | Priority requirement T10 Funnel cleaning function | The automatic funnel cleaning function is designed for preventive cleaning of the funnel during operation and at the end of the work cycle | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.8. | Priority requirement T12 Automatic brine tank filling stop | The equipment is supplied with an electronic filling equipment stop. The stop is activated when the brine tanks are filled to 100% capacity. | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.9. | Priority requirement T13 Integrated navigation system | The navigation system integrated into the control panel ensures that when performing automatic spreading along a pre-programmed route, the operator can see the route on the screen throughout the entire route and receive visual and audio instructions about the sections of the road where work needs to be performed. The navigation system maps must not be older than 2026, updates must be provided at least once every 12 months, and support and functional operation must be ensured for a period of not less than 10 years. | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.10. | Priority requirement T14 Road surface thermometer | Installed. The control panel displays the current road surface temperature. Temperature data is displayed and presented in reports, allowing for detailed analysis of temperature values by time and/or work operations. If a license is required for data analysis, it must be granted for a period of not less than 10 years or for the expected service life of the Equipment, whichever is shorter | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.11. | Priority requirement T15 Outdoor environment thermometer | Installed. The control panel displays the current outdoor temperature. Temperature data is displayed and presented in reports, allowing for detailed analysis of temperature values by time and/or work operations. If a license is required for data analysis, it must be granted for a period of not less than 10 years or for the expected service life of the Equipment, whichever is shorter | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.12. | Priority requirement T16 Snow plow indicator | Installed. The control panel displays the position of the front snow plow. The plow position data is displayed in the reports provided. If a license is required for data analysis, it must be granted for a period of at least 10 years or for the expected service life of the Equipment, whichever is shorter. | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.13. | Priority requirement T17 Additional rear lights | Additional LED lights installed on the spreader, duplicating the rear lights of the truck | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.14. | Priority requirement T18 Side marker lights | At least 2 additional orange LED lights are installed on each side of the spreader | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.15. | Priority requirement T20 Engine compartment lighting | Additional lighting installed under the engine cover | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.16. | Priority requirement T22 Simulation control panel and accompanying equipment for recording routes and uploading them to spreaders | All equipment comes with a single simulation control panel and other accessories that allow routes to be recorded and corrected without a salt spreader, using a separate vehicle (vehicle not included). The system must ensure that the recorded and corrected routes are remotely transferred to the selected salt spreader. If a license is required for the work, it must be unlimited in scope and valid for at least 10 years. | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.17. | Priority requirement T24 Crosswinds reduction system | The control panel reduces the influence of crosswinds at the touch of a button. The system automatically adjusts the spreading parameters (e.g., spreading width, direction, and/or dosage) to ensure more even spreading and minimal material loss when affected by crosswinds. In automatic spreading mode, the system automatically adjusts the set parameters until the end of the route or until the operator manually changes the settings. | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |
| 3.18. | Priority requirement T26 Hydraulic system pressure manometer | A hydraulic system manometer is installed in the engine compartment, in the hydraulic valve block, showing the current hydraulic pressure in the system. | Yes/No (delete as appropriate). Proposed parameter _____________ Title of the document submitted as evidence - _____________ and page no.. ___or reference -_____. |

Annex 2 - Chassis visualization