RENT OF A REMOTE IDENTIFICATION (REMOTE ID) AND DETECTION SYSTEM FOR KUN AIRPORT

TECHNICAL SPECIFICATION

CONTENTS

1.	DESCRIPTION OF THE OBJECT OF PURCHASE	2

2.	GENERAL SYSTEM REQUIREMENTS	3

3.	FUNCTIONAL REQUIREMENTS	3

4.	TECHNICAL REQUIREMENTS FOR SENSORS	4

5.	VISUALIZATION OF MAPS AND DATA	4

6.	FUNCTIONALITY OF HISTORICAL DATA AND REPORTS	4

7.	PROJECT PHASE, SUPPLIER'S RESPONSIBILITY AND DEADLINES	4

8.	REQUIREMENTS FOR SYSTEM TRAINING, INSTALLATION	5

9.	MAINTENANCE REQUIREMENTS	5

- The Client / Customer – Joint-stock company Lithuanian Airports.

- Service Provider/ Supplier – economic entity – a natural person, a private legal entity, a public legal entity, other organizations and their subdivisions or a group of such persons with whom the Customer enters a Contract.

- System - drone detection system consisting of at least two RF receivers (detectors), an antenna array, and software for monitoring the airspace and providing the user with the drone’s location.

- Goods / Services – Drone Detection Equipment, its components and licenses.

- UAV – Unmanned Aerial Vehicle

Vilnius – Vilnius branch of Public Limited Company Lithuanian Airports

- Contract – a contract concluded between the Service Provider/Supplier and the Customer regarding the object of the Purchase.

- REMOTE-ID - the unique identification number transmitted by the drone identifying the drone and/or its operator.

DESCRIPTION OF THE OBJECT OF THE PURCHASE

The joint-stock company Lithuanian Airports intends to rent a drone remote identification (REMOTE ID) and detection system for the identification, localization and real-time monitoring of drones and their operators for the supervision and control of the territory of Vilnius Airport. The system is designed exclusively for the reception and processing of REMOTE ID signals without the use of active radio frequency radiation.

- Objectives and scope of the project

- The technical specification sets out requirements for a drone remote identification (REMOTE ID) detection system for use in civil aviation infrastructure.

  - Detect drones using REMOTE ID technologies;

  - To determine the geographical location of the drone and its operator and display this information on the map within the supplier’s software;

  - Ensure passive airspace monitoring without active radio frequency interference or impact on radio communication;

  - Collect historical REMOTE ID data of detected drones for event investigation and reporting.

  - Before installing the System, the Supplier must carry out an assessment of the planned installation sites of the hardware and consult with the Customer on the optimal solution for the installation of the hardware.

  - The installation location(s) of the system, the arrangement of sensors, installation solutions and integration into the existing electrical and network infrastructure shall be determined and approved by mutual agreement between the Supplier and the Customer, taking into account technical, operational and security requirements.

  - Before starting any installation or configuration works, the Supplier must assess the topographical and building features of the territory, assess the possible restrictions on the propagation of radio signals, assess the technical capabilities of the existing infrastructure, and submit the proposed System installation solution to the Customer for coordination.

- Table 1. Table of purchased services and goods

| - No. | - Description of goods/services* | Maximum quantity | Unit of goods/services |
| --- | --- | --- | --- |
|  | - REMOTE ID detection system for drones with all the functionality described in these technical specifications (including licenses) | - 12 | - Month |
|  | - Installation of REMOTE ID detection system for drones and installation of hardware | - 1 | - Set |
| - 3. | - Maintenance and preventive servicing services to ensure the continuous operation of systems, including spare parts | - 12 | - Month |
|  | - Integration with Unmanned Traffic Management Application programming interface ON UTM (Lithuania) | - 1 | - Set |

- *The Customer does not undertake to order all the specified goods or services. If the Supplier indicates in the offer that part of the services or equipment is provided free of charge or at a significantly reduced price (e.g. The price will be transferred to the other quotation lines), the Supplier will have to provide these services at the price specified in the quotation, regardless of whether other goods or services from the table above are ordered.

GENERAL SYSTEM REQUIREMENTS

The system software must be web-based and accessible through standard web browsers (Google Chrome, Microsoft Edge, Mozilla Firefox or equivalent).

The supplier must ensure at least 20 (twenty) individual user licenses granting the right to connect to the system.

The system must support at least 20 (twenty) users connected simultaneously, ensuring stable operation and real-time data updates.

- 2.4. A user authentication and authorization system must be in place that allows different roles to be assigned (e.g. administrator, operator, observer).

- 2.5. The system must operate 24/7 mode and be adapted to the use of critical infrastructure.

- 2.6. The supplier shall configure the system to transmit all available object and drone position data, along with related information, as UTF-8 encoded JSON via RESTful requests over a secure HTTPS connection (TLS 1.2 or 1.3), and to integrate with the Unmanned Traffic Management (UTM) API in Lithuania. According to Annex No. 1 of the Technical specification “ON UTM system”.

FUNCTIONAL REQUIREMENTS

- The system shall provide the following information for each drone detected:

3.1. Manufacturer and model of the drone;

- 3.2. Geographical position (latitude and longitude) of the drone;

- 3.3. Geographical location of the drone operator (controller) if transmitted by REMOTE ID signal;

- 3.4. Unique identifier of the drone (serial number or UAS ID);

- 3.5. Operator ID, if applicable;

- 3.6. Flight altitude from the take-off site;

- 3.7. Altitude variation graph, in which flight segments exceeding an altitude of 120 m are visually distinguished;

- 3.8. Visual representation of the detected drone model, if available;

- 3.9. List of violated virtual airspace restrictions (geofence), if applicable, in case of violation of the intended space limitation to be notified by e-mail or SMS.

- 3.10. Identification of the sensor that performed the detection;

- 3.11. Historical REMOTE ID data about previous detections, if any.

TECHNICAL REQUIREMENTS FOR SENSORS

- 4.1. Sensors shall be passive and should not emit radio frequency emissions.

- 4.2. The operating frequency range is at least 2,4 to 2,5 GHz.

- 4.3. The minimum number of antennas is at least 2 (two).

- 4.4. Environmental resistance class – at least IP67.
4.5. Detection range: at least 4 500 m in an urban environment, up to 20 000 m under ideal conditions.

- 4.6. The system shall consist of at least two RF receivers (detectors) with an antenna array capable of determining the direction of incoming Remote ID signals.

VISUALIZATION OF MAPS AND DATA

  - The system shall provide an interactive geographic representation of data allowing the user to:

  - Switch between orthophoto and standard cartographic maps

  - Display Remote ID detections in real time

  - Enable or disable additional information layers

FUNCTIONALITY OF HISTORICAL DATA AND REPORTS

- 6.1. The system shall store historical REMOTE ID detection data.

- 6.2. The system shall allow viewing historical detection records by date and time period.

- 6.3. The system shall allow filtering detections by drone ID, operator ID, or location.

- 6.3. The system must create conditions to generate reports and notifications by e-mail or SMS to identified users about: 
6.3.1. violations of geofences;
6.3.2. cases of exceeding height;
6.3.3. repeated detections of the same drone.

STAGES OF ACQUISITION OF GOODS AND SERVICES AND THEIR DELIVERY/PROVISION TERMS

- 7.1 Design phase – carried out within 5 (five) working days from the date of signing the Agreement. During this stage, the technical solutions of the System and the installation plan are prepared and coordinated with the Customer.

- 7.2 Installation and system start-up phase – carried out within 5 (five) working days after the completion of the design phase, in accordance with the approved design solutions and the requirements of this Technical Specification.

- 7.3 Training phase - carried out within 2 (two) working days after the completion of the installation and start-up phase of the system or in parallel with the installation and start-up phase of the system, at least 6 representatives of the Customer are trained on the principle of "train – the trainer".

- 7.4. System operation phase – within 1 (one) working day from the training phase or in parallel with the training phase, at this stage the System shall be put into operation at full capacity as provided for in Technical specification.

- 7.5 Integration phase – within 1 (one) week after the System operation phase the supplier shall configure the system to transmit all object and drone position data and related information with the Lithuanian UTM API per Annex 1 of the Technical specification“ ON UTM system”.

- 7.6. Delivery/provision terms for Goods and Services specified in points No. 1 and 3 shall be calculated from the end of System operation phase.

- 7.7. Upon the end of the Contract, Goods will be handed over to the Supplier. The Supplier collects the Goods at his own cost.

- 7.8. The Supplier's personnel must have valid permanent permits for the provision of installation services described in the technical specifications during the term of the Contract. During the term of the Agreement, the Customer will not provide escort services to the Supplier's personnel in the controlled area if necessary. Information on the procedure and prices for the issuance of permits to work in the territory of the airport, as well as on the prices of fees required for obtaining permits, is available on the Client's website at https://www.ltou.lt/lt/galimybes-verslui/leidimai  and https://www.ltou.lt/lt/apie-lietuvos-orouostus/mokymai.  Airport fees for permitting and training shall be paid by the Supplier.

REQUIREMENTS FOR SYSTEM TRAINING, INSTALLATION

- 8.1. The Supplier must provide training to at least 6 (six) employees of the Customer, applying the train-the-trainer principle or an equivalent method by issuing a certificate/certificate that the trained person can train other persons.

- 8.2 The Supplier undertakes to install the System at the Customer's facility in accordance with this Technical specification, applicable legislation and technical solutions agreed with the Customer.

MAINTENANCE REQUIREMENTS

  - The Supplier will provide the following support services for the installed system:

  - Provide Help Desk services, which could be used by the Customer by phone or e-mail;

- 9.3.The supplier shall: prioritize requests for assistance based on a reasonable assessment of the level of seriousness of the problem identified, respond to all requests for assistance in accordance with the response actions and response times set out in the table below.

- Table 2. Fault Response Table

| - Error / Failure Severity Level | - Definition | - Service-level response and response time |
| --- | --- | --- |
| - Level 1 failure - Critical failure | - Critical failure: A system or equipment error or malfunction that prevents the use of the system | - Level 1 response: - Acknowledgment of receipt of the request for assistance within 12 hours at the latest. - Level 2 response: - Depending on the nature of the failure, the Supplier must connect to the system remotely so that the Customer can continue to use all system and equipment functions no later than 24 hours after the end of the Level 1 response period. - Level 3 response: - When remote connection is not possible due to the nature of the fault or it is not possible to solve the fault after connecting remotely, the Supplier must immediately send employees or identified faulty parts of the system to restore the functionality of the system. Restoration no later than within 38 hours after confirmation of receipt of the Customer's request for assistance. - If the Supplier provides the Customer with a temporary solution acceptable to ensure the restoration of the functionality of the system, the level of severity of the failure will be reduced to level 2 or less. |
| - Level 2 failure - Medium Criticality Failure | - System or equipment or system defect with temporary solution: - (a) a critical fault in the system or its equipment, or a failure for which there is a temporary alternative solution; on - b) a non-critical error of the system or its equipment, or a malfunction that affects the operations of the Customer's system or the functionality of its equipment, but the system can be used in operations. | - Level 1 response: - Acknowledgment of receipt of the request for assistance within 12 hours at the latest. - Level 2 response: - The supplier shall, no later than 24 hours after the expiry of the Level 2 response period: - (a) the urgent repair of the system or equipment or the implementation of an interim solution; and/or - b) to ensure corrective maintenance of the product in question, - which would allow the Customer to continue to use all the functions of the system in all essential respects. - Level 3 response: - The Supplier undertakes to ensure the complete elimination of the defect as soon as possible and no later than within 5 days after the confirmation of receipt of the Customer's request for assistance. |
| - Level 3 failure - Non-critical failure | - Minor error: - A single or minor system or equipment error or malfunction that: - a) does not significantly affect the functionality of the system or equipment; - b) does not allow you to use only certain non-essential functions. | - Level 1 response: - Confirmation of receipt of the request for assistance no later than within 1 working day. - Level 2 response: - The Supplier must ensure the complete elimination of the defect no later than within 10 days after the confirmation of receipt of the Customer's request for assistance. |

ENVIRONMENTAL REQUIREMENTS

  - All the proposed technical equipment must be sturdy, long-lasting, functional, and suitable for multiple cycles of use/work, and/or easily repairable and/or replaceable.

  - The Supplier must implement appropriate environmental measures to reduce the amount of waste generated during the provision of the Services and deliver these wastes for recycling. All used parts must be collected, sorted, and transferred to a waste management company.

- ATTACHED:

- Annex 1 – ON UTM system