| TECHNINĖ SPECIFIKACIJA (TS) | TECHNICAL SPECIFICATION (TS) |
| --- | --- |
| - SĄVOKOS IR SUTRUMPINIMAI | - DEFINITIONS AND ABBREVIATIONS |
| - Klientas – UAB Kauno kogeneracinė jėgainė. - Paslaugų teikėjas – ūkio subjektas – fizinis asmuo, privatusis juridinis asmuo, viešasis juridinis asmuo, kitos organizacijos ir jų padaliniai ar tokių asmenų grupė, su kuriuo Klientas sudaro Sutartį. - Sutartis - sutartis sudaroma tarp Kliento ir Paslaugų teikėjo dėl Pirkimo objekto. - Paslaugos – dūmų valymo įrenginių inžinerinio palaikymo paslaugos. - Užsakymas – Kliento Paslaugų teikėjui teikiamas užsakymas raštu dėl Paslaugų teikimo. | - Customer means UAB Kauno kogeneracinė jėgainė. - Service Provider means an economic entity – a private person, a private legal person, a public legal person, other organisations and their subdivisions or a group of such persons with whom the Customer concludes a Contract. - Contract – agreement between the Customer and the Service Provider regarding the Object of Procurement. - Services – engineering support services of the flue gas cleaning equipment. - Order – a written order from the Customer to the Service Provider regarding the provision of Services. |
| - PIRKIMO OBJEKTAS | - OBJECT OF PROCUREMENT |
| - Dūmų valymo įrenginių inžinerinio palaikymo paslaugos. | - Engineering support services of the flue gas cleaning equipment. |
| - PIRKIMO OBJEKTO APIMTYS | - SCOPES OF THE OBJECT PROCUREMENT |
| - Paslaugų kiekiai pateikiami žemiau esančioje lentelėje Nr. 1: Lentelė Nr. 1 Eil. Nr.; Paslaugų pavadinimas; Mato vnt.; Preliminarus kiekis* Sutarties galiojimo laikotarpiu. Planinės inžinieriaus paslaugos; valanda; 420. Planinės projektų vadovo paslaugos; valanda; 370. Neplaninės specialisto paslaugos; valanda; 320. * Preliminarus kiekis nurodytas tik pasiūlymų palyginimo ir vertinimo tikslais. Vadovaujantis Kainodaros taisyklių nustatymo metodika (aktuali redakcija, toliau - Metodika), įsigyjamų Paslaugų kiekis nustatomas pagal Sutartyje nurodytą maksimalią lėšų, skirtų Paslaugoms įsigyti, sumą. Klientas turi teisę koreguoti perkamų Paslaugų kiekius, neviršijant bendros Sutarties kainos. Klientas neįsipareigoja nupirkti viso nurodyto preliminaraus Paslaugų kiekio ar bet kokios jų dalies, taip pat Klientas neįsipareigoja išpirkti Paslaugų visai Sutarties kainai ar bet kokiai jos daliai. | - The quantities of Services are listed in table No 1 below: Table No. 1 Item No.; Name of the services; Unit of measure; Preliminary quantity* during the validity period of the Contract. Scheduled services of engineer; hour; 420. Scheduled services of project manager; hour; 370. Unscheduled specialist services; hour; 320. * A preliminary quantity has been indicated for the purposes of comparison and evaluation of tenders only. Pursuant to the Methodology for Establishing Pricing Rules (the relevant version, hereinafter – the Methodology), the quantity of the Services to be procured shall be determined based on the maximum amount of funds for the purchase of Services specified in the Contract. The Customer shall have the right to adjust the quantities of the Services to be procured, without exceeding the total Contract price. The Customer shall not be obliged to purchase the entire or any part of the indicated preliminary quantity of Services, nor shall the Customer be obliged to purchase the Services for the entire Contract Price or any part thereof. |
| - PASLAUGŲ TEIKIMO VIETA | - PLACE OF THE PROVISION OF SERVICES |
| - Paslaugos teikiamos: Jėgainės g. 6, Biruliškių k, Kauno r. 54469., Lietuva arba, esant poreikiui ir galimybei, nuotoliniu būdu. | - The Services shall be provided at Jėgainės str. 6, Biruliškės village, Kaunas region 54469, Lithuania, or online, if necessary and possible. |
| - REIKALAVIMAI PIRKIMO OBJEKTUI | - REQUIREMENTS FOR THE PROCUREMENT OBJECT |
| - Pirkimo objekto aprašymas - Klientas eksploatuoja atliekų deginimo jėgainę, kurioje įrengta dūmų valymo sistema. Ši sistema susideda iš šių dalių: Lab loop reaktorius ir reagentų dozavimas, rankoviniai filtrai (4 celės), lakiųjų pelenų pneumo transportas ir recirkuliacija į lab loop reaktorių, šlapia dūmų valymo dalis: kondenseris ir dūmų aušinimo kolona. - Sutarties vykdymo metu numatomas planinių ir neplaninių Paslaugų poreikis: - Planinės Paslaugos: - Konsultacijos, susijusios su dūmų valymo sistemos technologinio proceso klausimais, teikiamos anglų arba lietuvių kalbomis. - Paslaugų teikėjo mokymai, susiję su įrenginių priežiūra, vykdomi anglų arba lietuvių kalbomis. Temos: - Kondenseryje esančios įkrovos keitimo procedūra ir įkrovos medžiagos. - Dūmų valymo sistemos technologinių įrenginių aptarnavimas ir atsarginių dalių keitimas. - Aušinimo kolonos purkštukų aptarnavimas ir keitimas. - Susidėvėjusių rankovinių filtrų keitimas naujais. Procedūra ir aptarnavimo veiksmai. - Atlikus bet kokius pakeitimus dūmų valymo įrangos apsaugų sistemose ir / ar proceso valdymo sistemose, taip pat turi būti atliekami pakeitimai techninėje dokumentacijoje. - Techninė dokumentacija pildoma anglų kalba. - Planinės Paslaugos organizuojamos atsižvengiant į Paslaugų apimtis. - Programavimo Paslaugos gali būti teikiamos nuotoliniu būdu arba vietoje (žr. Techninės specifikacijos 4.1 p.). Programavimo Paslaugų užduotys derinamos su Klientu Sutarties galiojimo metu. - Jeigu po Paslaugų atlikimo atsiranda poreikis atlikti sistemos apsaugų bandymus, tuomet, dūmų valymo sistemos apsaugų bandymai atliekami, vadovaujantis norminių teisės aktų bei gamintojo reikalavimais. Esant notifikuotos įstaigos dalyvavimo poreikiui, Paslaugų teikėjas organizuoja šios įstaigos dalyvavimą. Bandymų programa turi būti suderinta 3 (trys) darbo dienos prieš bandymą. - Inžinieriaus konsultacijos gali būti teikiamos nuotoliniu būdu arba vietoje (žr. Techninės specifikacijos 4.1 p.). - Neplaninės Paslaugos, kurių poreikis gali iškilti Sutarties galiojimo metu: - Konsultacija dėl defektų arba skubus gedimų šalinimas. - Defektų bei gedimų atsiradimo priežasties nustatymas. - Eksploatacijos rekomendacijų teikimas, siekiant išvengti gedimų ar defektų pasikartojimo. - Paslaugų suteikimui reikalingas medžiagas pateiks Klientas (savo kaštais). | - Description of the Procurement Object - The Customer operates a waste incineration plant equipped with a flue gas cleaning system. This system consists of the following parts: Lab loop reactor and reagent dosing, bag filters (4 cells), residue ash pneumatic transport and recirculation to the lab loop reactor, flue gas wet cleaning part: condenser and quench. - During the execution of the Contract, the need for scheduled and unscheduled Services is expected: 5.1.2.1. Scheduled Services: 5.1.2.1.1. The consultations related with flue gas cleaning technology or process shall be provided in English or in Lithuanian. 5.1.2.1.2. The training of maintenance of equipment shall be provided by the Service Provider in English or Lithuanian. Topics: a) Procedure of condenser packing replacement. Presentation of packing material. b) Flue gas treatment equipment maintenance and spare parts replacement. c) Service and replacement of quench nozzles. d) Replacement of worn bag filters with new ones. Procedure and maintenance steps. 5.1.2.1.3. In case any modifications are made in the protection systems and/or process management systems of the flue gas cleaning, the changes must also be made in the technical documentation. 5.1.2.1.4. The technical documentation shall be filled in English. 5.1.2.1.5. The scheduled Services shall be organised in consideration to the Service scope. 5.1.2.1.6. The programming Services may be provided online or on site (see par. 4.1 of the Technical Specification). The tasks of the programming Services shall be coordinated with the Customer within the validity term of the Contract. 5.1.2.1.7. If there is a need to perform system protection tests after the Services have been performed, the protection tests of the flue gas cleaning system are carried out in accordance with the normative legal acts and the manufacturer's requirements. In case of the need for the participation of a notified institution, the Service Provider organizes the participation of this institution. The test program must be agreed 3 (three) working days before the test. 5.1.2.1.8. The engineer may give consultations online or on site (see par. 4.1 of the Technical Specification). 5.1.2.2. The unscheduled Services that may be needed during the Preliminary contract’s validity: 5.1.2.2.1. Consultation for defects or urgent troubleshooting. 5.1.2.2.2. Identification of the cause of defects or breakdowns. 5.1.2.2.3. Giving of operation recommendations in order to prevent recurrent defects and breakdowns. 5.1.3. Materials required for the provision of the Services will be provided by the Client (at own expenses). |
| - PASLAUGŲ VYKDYMO TVARKA IR TERMINAI | - PROCEDURE AND TIME LIMITS FOR THE PROVISION OF SERVICES |
| 6.1 Planinės Paslaugos (TS 5.1.2.1. p.) turės būti suteiktos ne vėliau kaip per 30 (trisdešimt) kalendorinių dienų nuo Užsakymo pateikimo Paslaugų teikėjui dienos. Neplaninės Paslaugos (TS 5.1.2.2. p.) turės būti suteiktos ne vėliau kaip per 5 (penkias) darbo dienas nuo Užsakymo pateikimo Paslaugų teikėjui dienos. 6.2 Paslaugų teikėjas turės suteikti Paslaugas Techninės specifikacijos 4 dalyje nurodytu adresu Kliento darbo laiku (I-IV 7.30–16.30 val. V 7.30–15.15 val.) arba, esant poreikiui ir galimybei, bus organizuojama nuotoliniu būdu, Klientui patogiu laiku. | - Scheduled Services (TS 5.1.2.1. p.) must be provided no later than within 30 (thirty) calendar days from the date of submission of the Order to the Service Provider. Unscheduled Services (TS 5.1.2.2. p.) must be provided no later than within 5 (five) working days from the date of submission of the Order to the Service Provider. - The Service Provider shall have to provide the Services at the address indicated in Section 4 herein during the Customer’s work hours (I-IV 7:30–16:30; V 7:30–15:15), or, if necessary and possible, they shall be organised online, at a time convenient for the Customer. |
| - KOKYBĖ IR TRŪKUMŲ PAŠALINIMAS | - QUALITY AND REMEDY OF DEFECTS |
| 7.1. Paslaugų ir (ar) Paslaugų rezultato trūkumais laikomi neatitikimai šio dokumento reikalavimams, netinkamas ar nekorektiškas dūmų valymo įrangos bei jos sistemų procesų veikimas, arba neveikimas. 7.2. Klientas turi teisę kreiptis į Paslaugų teikėją dėl Paslaugų ir (ar) Paslaugų rezultato trūkumų pašalinimo ne vėliau kaip per 10 kalendorinių (dešimt) dienų nuo perdavimo–priėmimo akto pasirašymo / trūkumų užfiksavimo dienos (trūkumą užfiksavus garantiniu laikotarpiu). 7.3. Kliento nustatytiems Paslaugų ir (ar) Paslaugų rezultato trūkumams šalinti nustatomas 20 (dvidešimt) darbo dienų terminas. | - The defects of the Materials, Services and/or their result shall mean non-compliance with the requirements of this document, inappropriate or incorrect operation of the flue gas cleaning equipment and its systems or their failure to operate. - The Customer shall have the right to ask the Service Provider to eliminate the defects of the Materials, Services and/or their result not later than within 10 (ten) calendar days after signing of the transfer and acceptance deed / recording of the defects. - The term of 20 (twenty) working days is established for correction of the defects of the Services and/or their results detected by the Customer (in case defects are detected during warranty period). |
| - KARTU SU TEIKIAMOMIS PASLAUGOMIS PATEIKIAMI DOKUMENTAI | - DOCUMENTS TO BE SUBMITTED TOGETHER WITH THE SERVICES TO BE PROVIDED |
| - Perdavimo-priėmimo aktas; - Duomenų laikmenos, atstatymo failai ir kt.; - Techninės dokumentacijos pakeitimai. | 8.1. Transfer and acceptance deed; 8.2. Data storage mediums, restoration files, etc.; 8.3. Changes to technical documentation. |