ANNEX 2

TO THE PROCUREMENT CONDITIONS

(LOT 1)

Coat of arms or brand name

(Supplier name)

(Legal form of the legal entity, registered office, contact information, name of the register in which data about the supplier is collected and stored, legal entity code, value added tax payer code, if the legal entity is a value added tax payer)

DEFENCE MATERIEL AGENCY UNDER THE MINISTRY OF NATIONAL DEFENCE

TENDER

FOR THE PURCHASE OF

JACKETS AND WINTER CLOTHING SETS

____________

(Date)

_____________

(Place of conclusion)

| Supplier name, company code/ Supplier group members, company codes |  |
| --- | --- |
| Supplier address / If a group of suppliers participates, all participants' addresses shall be listed. / |  |
| Name and surname of the responsible person, |  |
| Telephone number |  |
| Email address |  |
| Account number, bank name, and bank code(s) of the responsible partner of the supplier/group of suppliers: |  |
| Name, surname, and position of the supplier/group of suppliers signing the contract in case of winning: |  |
| In the event of a successful tender, the name, surname, telephone number, and email address of the person responsible for the performance of the contract on behalf of the supplier/group of economic operators: |  |
| A list of (1) the supplier / members of the supplier group, (2) economic operators whose capacities are relied upon, and (3) if the grounds for exclusion apply to all subcontractors – the subcontractors, members of the collegial supervisory body (Supervisory Board) and/or the collegial management body (Board) (if established), and/or the person authorised in accordance with Article 46(2)(2) of the LPP. |  |

Economic operators (including quasi-suppliers – natural persons who are to be employed in the event of a successful bid) whose capacities the supplier relies on to meet the qualification requirements:

| Name(s) | Code, address | Activities being transferred | Part of the activity being transferred from the total purchase agreement (EUR or ℅) |
| --- | --- | --- | --- |

The activities to be transferred to subcontractors (mandatory) and the names of these economic entities:

| Name(s) | Code, address | Activities being transferred | Part of the activity being transferred from the total purchase agreement (EUR or ℅) |
| --- | --- | --- | --- |

1. With this tender, we confirm that we agree to all the terms and conditions of purchase set out in:

1.1. procurement notice,

1.2. other procurement documents (their explanations, additions);

1.3. the main terms and conditions of the purchase and sale contract.

2. By submitting the tender using the CPP IS tools, I confirm that the digital copies of the documents and the data submitted by electronic means are authentic.

LOT 1– Universal jacket (five-layer field uniform)

We offer the following products:

| No. | Product name | Manufacturer name/country | Measurement unit | Quantity (in units) maximum * | Unit price, EUR (excl. VAT) | Total price of the tender, EUR (excl. VAT) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7(5x6) |
| 1. | Universal jacket (five-layer field uniform) |  | pcs. | 57 943 |  |  |
| Total price of the tender excl. VAT |  |  |  |  |  |  |
| VAT amount |  |  |  |  |  |  |
| Total price of the tender incl. VAT |  |  |  |  |  |  |

* The specified quantity of goods is the maximum quantity. The contracting authority does not undertake to purchase the entire specified maximum quantity of goods during the entire term of the contract.

Total price of the tender in EUR excl. VAT (amount in figures and words) EUR.......................

Total price of the tender in EUR incl. VAT (amount in figures and words) EUR.......................

Notes:

1.1. The price of the tender shall include all taxes payable by the supplier and all costs incurred by the supplier in connection with the performance of the procurement contract.

1.2. Prices must be rounded to two decimal places.

1.3. In cases where a foreign supplier participates in the procurement or where a supplier registered in the Republic of Lithuania is not required to pay VAT under the applicable legislation, it shall not fill in column 6 of the table and shall indicate the reasons for not paying VAT: ............................................ .

2. The goods offered fully comply with the requirements specified in the procurement documents and their main characteristics are as follows:

| No. | Required technical characteristics of goods | Specific values of the indicators of the goods offered by the supplier** | Reference to the document, page, paragraph |
| --- | --- | --- | --- |
| 1. | The fibre composition of the base fabric, %: top layer – 100 % PA; bottom layer – 100 % PTFE (or equivalent) | SPECIFY |  |
| 2. | The surface density of the base fabric shall be 105 g/m² (+20; -10) | SPECIFY |  |
| 3. | The breaking strength of the base fabric in the warp direction shall be ≥ 750 N | SPECIFY |  |
| 4. | The breaking strength of the base fabric in the weft direction shall be ≥ 500 N | SPECIFY |  |
| 5. | The tear strength of the base fabric in the warp direction shall be ≥ 40 N | SPECIFY |  |
| 6. | The tear strength of the base fabric in the weft direction shall be ≥ 40 N | SPECIFY |  |
| 7. | The abrasion resistance of the base fabric, at a nominal pressure of 12 kPa, shall be ≥ 100,000 cycles | SPECIFY |  |
| 8. | The water vapour resistance (Ret) of the base fabric shall be ≤ 5 m²·Pa/W | SPECIFY |  |
| 9. | The surface wetting resistance of the base fabric shall be ≥ 4 (class) | SPECIFY |  |
| 10. | The water penetration resistance of the base fabric, at a water pressure increase rate of 60 ± 3 cm/min, shall be ≥ 1000 cm when new | SPECIFY |  |
| 11. | The water penetration resistance of the base fabric, at a water pressure increase rate of 60 ± 3 cm/min, shall be ≥ 800 cm after 5 washes | SPECIFY |  |
| 12. | The air permeability of the base fabric, at 100 Pa pressure, shall be ≤ 5 mm/s | SPECIFY |  |
| 13. | The dimensional change of the base fabric after washing, in the warp direction, shall be no more than ± 4 % | SPECIFY |  |
| 14. | The dimensional change of the base fabric after washing, in the weft direction, shall be no more than ± 4 % | SPECIFY |  |
| 15. | The colourfastness of the base fabric to washing at 40 °C shall be ≥ 4 (rating) | SPECIFY |  |
| 16. | The colourfastness of the base fabric to water shall be ≥ 4 (rating) | SPECIFY |  |
| 17. | The colourfastness of the base fabric to perspiration shall be ≥ 4 (rating) | SPECIFY |  |
| 18. | The colourfastness of the base fabric to dry abrasion shall be ≥ 4 (rating) | SPECIFY |  |
| 19. | The colourfastness of the base fabric to wet abrasion shall be ≥ 3 (rating) | SPECIFY |  |
| 20. | The colourfastness of the base fabric to artificial light shall be ≥ 4 (rating) | SPECIFY |  |
| 21. | The dominant spectral reflectance of the base fabric in the near-infrared spectrum (750–1100 nm) shall be: gray-green (base) color: 60 – 70 green color: 40 – 50 hey color: 35 – 45 chaki color: 35 – 50 brown color: 20 – 30 | SPECIFY |  |
| 22. | The weave of the base fabric must be plain weave, with a ripstop base | SPECIFY |  |
| 23. | The number of threads per centimeter in the warp of the base fabric must be ≥ 40. | SPECIFY |  |
| 24. | The number of threads per centimeter in the base fabric must be ≥ 25. | SPECIFY |  |
| 25. | The fiber composition of the protective fabric must be 100 PA | SPECIFY |  |
| 26. | The surface density of the protective fabric shall be 140 g/m² (+20; -10) | SPECIFY |  |
| 27. | The maximum force of the protective fabric in the warp direction shall be ≥ 1300 N | SPECIFY |  |
| 28. | The maximum force of the protective fabric in the weft direction shall be ≥ 900 N | SPECIFY |  |
| 29. | The tear strength of the protective fabric in the warp direction shall be ≥ 90 N | SPECIFY |  |
| 30. | The tear strength of the protective fabric in the weft direction shall be ≥ 90 N | SPECIFY |  |
| 31. | The abrasion resistance of the protective fabric, at a nominal pressure of 12 kPa, shall be ≥ 200,000 cycles | SPECIFY |  |
| 32. | The surface wetting resistance of the protective fabric, when new, shall be ≥ 4 (class) | SPECIFY |  |
| 33. | The surface wetting resistance of the protective fabric, after 5 washes, shall be ≥ 3 (class) | SPECIFY |  |
| 34. | The water penetration resistance of the protective fabric, at a water pressure increase rate of 60 ± 3 cm/min, shall be ≥ 60 cm when new. | SPECIFY |  |
| 35. | The water penetration resistance of the protective fabric, at a water pressure increase rate of 60 ± 3 cm/min, cm after 5 washes must be ≥ 50. | SPECIFY |  |
| 36. | Change in dimensions of protective fabric after washing, % in the warp direction must not exceed ± 4 | SPECIFY |  |
| 37. | Change in dimensions of protective fabric after washing, % in the direction of the pile must not exceed ± 4 | SPECIFY |  |
| 38. | The color fastness of the protective fabric must be ≥ 4 when washed at 40º C | SPECIFY |  |
| 39. | The stain resistance of the protective fabric must be ≥ 4 on the water scale | SPECIFY |  |
| 40. | The resistance of the protective fabric to staining, measured in dry abrasion, must be ≥ 4 | SPECIFY |  |
| 41. | The resistance of the protective fabric to staining, measured in dry abrasion, must be ≥ 4 | SPECIFY |  |
| 42. | The resistance of the protective fabric to staining, measured in points for wet abrasion, must be ≥ 3 | SPECIFY |  |
| 43. | The color fastness of the protective fabric must be ≥ 4 on the artificial light scale | SPECIFY |  |
| 44. | The weave of the protective fabric must be plain weave | SPECIFY |  |
| 45. | The number of threads per centimeter in the warp of the protective fabric must be ≥ 35 | SPECIFY |  |
| 46. | The number of threads per centimeter in the protective fabric must be ≥ 25 | SPECIFY |  |
| 47. | The dominant spectral reflectance coefficient of the protective fabric in the near-infrared radiation spectrum range of 750-1100 nm, % shall be: gray-green (base) color: 60 – 70 green color: 40 – 50 straw color: 35 – 45 chaki color: 35 – 50 brown color: 20 – 30 | SPECIFY |  |
| 48. | The fiber composition of the lining must be 100% PA | SPECIFY |  |
| 49. | The surface density of the lining, g/m2, must be 45 ± 10 | SPECIFY |  |
| 50. | The maximum force of the lining, N, in the direction of the warp must be ≥ 600 | SPECIFY |  |
| 51. | The maximum force of the lining, N, in the direction of the fasteners must be ≥ 350 | SPECIFY |  |
| 52. | The tear strength of the lining, N in the direction of the warp, must be ≥ 20 | SPECIFY |  |
| 53. | The tear strength of the lining, N in the direction of the weft, must be ≥ 20 | SPECIFY |  |
| 54. | The resistance of the lining to abrasion at a nominal pressure of 12 kPa must be ≥ 100,000 revolutions | SPECIFY |  |
| 55. | The air permeability of the lining at a pressure of 100 Pa must be ≥ 80 mm/s | SPECIFY |  |
| 56. | The change in lining dimensions after washing, % in the warp direction, must not exceed ± 4% | SPECIFY |  |
| 57. | The change in lining dimensions after washing, % in the direction of the weft, must not exceed ± 4% | SPECIFY |  |
| 58. | The color fastness of the lining must be ≥ 4 when washed at 40º C | SPECIFY |  |
| 59. | The resistance of the lining to staining, measured in dry abrasion, must be ≥ 4 | SPECIFY |  |
| 60. | The resistance of the lining to staining, measured in points for wet abrasion, must be ≥ 3 | SPECIFY |  |
| 61. | The lining is woven, with a ripstop base | SPECIFY |  |
| 62. | The number of threads in lining per centimeter-three warp threads must be ≥ 55 | SPECIFY |  |
| 63. | The number of weft threads in lining per centimeter-three warp threads must be ≥ 25 | SPECIFY |  |
| 64. | The fiber composition of the insulation must be: polyester 55 ± 5, polyamide 25 ± 5 polyethylene 20 ± 5 | SPECIFY |  |
| 65. | The surface density of the insulation, g/m2, must be 110 ± 15 | SPECIFY |  |
| 66. | The insulation thickness, mm, must be 10 ± 2 | SPECIFY |  |
| 67. | The air permeability of the insulation (at a pressure difference of 200 Pa, 20 cm² opening) must be ≥ 1000 mm/s | SPECIFY |  |
| 68. | The thermal resistance of the insulation, m²K/W, must be ≥ 0.30 | SPECIFY |  |
| 69. | The change in insulation dimensions after washing, % in the longitudinal direction, must not exceed ± 4 | SPECIFY |  |
| 70. | The change in insulation dimensions after washing, % in the transverse direction, must not exceed ± 4% | SPECIFY |  |

** The supplier must record the specific technical indicator values that the proposed goods meet and that are indicated in the laboratory test reports submitted with the tender (as specified in clause 5.11.4 of the terms and conditions of purchase).

3. Description of the criteria for evaluating the most economically advantageous tender for the goods we offer:

| Criterion No. | Technical parameter indicators (Criterion T) | Values of the proposed criteria (entered by the supplier) |
| --- | --- | --- |
| T1 | Tensile strength of base fabric N |  |
| T2 | Resistance of the base fabric to tearing, twists |  |
| T3 | Resistance of base fabric to water penetration, cm |  |
| T4 | Thermal resistance of insulation material, m²K/W |  |

4. Documents to be submitted with the tender:

| No. | Document name | Number of pages | Confidential information (Yes/No) |
| --- | --- | --- | --- |

Note: If the supplier has not indicated which information is confidential, it shall be deemed that the tender does not contain any confidential information.

5. The tender shall remain valid until the deadline specified in the procurement documents.

| (Name of the position of the supplier or its authorized representative) |  | (Signature) |  | (Name and surname) |  |
| --- | --- | --- | --- | --- | --- |

In case of differences in interpretation the Lithuanian text shall prevail