Public Limited Liability Company „Oro Navigacija“

Technical Specification

Multi-Sensor Data Fusion (MSDF) tracker solution

For Preliminary Market Consultation

Contents

1.	Purpose of the Consultation	3

2.	Definitions & Abbreviations	3

3.	Scope of the System	4

4.	Data Input Requirement	4

5.	Data Fusion and Tracking	6

6.	Output and Data Distribution	6

7.	Interfaces and Integration	7

8.	Cybersecurity Requirements (NIS2 Compliance)	7

9.	Service-Based and Virtualized Architecture	7

10.	Operational and Architectural Considerations	8

11.	High-Level System Architecture	8

12.	Vendor Information Requested (for Market Consultation)	9

13.	CAPEX and OPEX estimation	9

# Purpose of the Consultation

The Surveillance Data Processing & Distribution is part of a ground based surveillance system that provides airspace situational awareness to air traffic controllers. The SDPD receives aircraft data from individual surveillance sensors, including ADS-B Ground Stations broadcasted from appropriately equipped aircraft, and serves fused surveillance updates to client systems, such as Controller Work Positions.

The purpose of this preliminary market consultation is to identify potential manufacturers and solution providers capable of delivering a Multi-Sensor Data Fusion (MSDF) tracker solution (hereinafter - MSDF tracker).

The system is intended to be used for the reception, processing, fusion of surveillance and detection data from both certified and non-certified sources, including data relevant to uncrewed traffic management systems and tracking data distribution to the external systems such as Air Traffic Management (ATM), Unmanned Aircraft System Traffic Management (UTM) and others.

Note: This document is non-binding and does not constitute a procurement specification. It is provided solely to assess market capabilities and available technical solutions.

# Definitions & Abbreviations

Definitions:

Certified Data – Surveillance data produced by sensors and systems certified according to applicable aviation regulations.

Non-Certified Data – Surveillance or detection data produced by sensors not certified for ATM use, typically used for U-space or auxiliary situational awareness.

UTM – digital ecosystem that enables the safe, efficient, and scalable management of drone operations in low-altitude airspace, especially where many drones operate simultaneously.

Abbreviations:

ADS-B – Automatic Dependent Surveillance – Broadcast

A-SMGCS – Advanced Surface Movement Guidance and Control System

ATM – Air Traffic Management

CAPEX – Capital Expenditure

CIS – Common Information Service (U-space)

C-UAS – Counter‑Uncrewed Aircraft Systems

EO – Electro‑Optical

ESASSP – EUROCONTROL Specification for ATM Surveillance System Performance

HTTPS – Hypertext Transfer Protocol Secure

IR – Infrared

MLAT – Multilateration

MQTT – Message Queuing Telemetry Transport

MSDF – Multi‑Sensor Data Fusion

NIS2 – EU Directive on Security of Network and Information Systems (2022/2555)

NTP – Network Time Protocol

OPEX – Operational Expenditure

PSR – Primary Surveillance Radar

PTP – Precision Time Protocol

RF – Radio Frequency

SDDS – Surveillance Data Distribution System

SDPD – Surveillance Data Processing & Distribution

SLA – Service Level Agreement

SSR Mode A/C/S – Secondary Surveillance Radar

STANAG 4586 – Standard NATO UAS Control Interface

TCP – Transmission Control Protocol

UAS – Uncrewed Aircraft System

UDP – User Datagram Protocol

U-Space – EU Framework for Drone Traffic Management

UTM – Unmanned Aircraft System Traffic Management

VM – Virtual Machine

WAM – Wide Area Multilateration

# Scope of the System

  - The MSDF tracker shall be designed to:

    - Receive and process data from multiple different certified and non-certified sensor sources.

    - Support continuous sensor data quality checking and configurable sensor data filtering (rejection from the processing) if sensor data doesn’t meet the quality requirements.

    - Support enabling and disabling selected sensor(s) data reception and processing.

    - Support enabling and disabling selected sensor(s) data filtering by vertical sectors/polygons.

    - Support enabling and disabling selected sensor(s) data filtering by horizontal sectors/polygons.

    - Support configurable fusion of selected data from certified sensor sources and distribute tracking data (tracks) to external systems.

    - Support configurable fusion of selected data from non-certified sensor sources and distribute tracking data (tracks) to external systems

  - The system shall be suitable for use in ATM and U-space environments, including mixed airspace scenarios.

  - The system shall be compliant with applicable ESASSP  and ED87D  mandatory requirements.

# Data Input Requirement

  - Certified Data Source

    - The system shall be capable of receiving and processing data from certified surveillance sources, including but not limited to:

      - Primary Surveillance Radar (PSR).

      - Secondary Surveillance Radar (SSR / Mode A/C/S).

      - Wide Area Multilateration (WAM).

      - Certified ADS-B data sources.

Note: Certified data sources are assumed to comply with applicable aviation standards and regulatory requirements.

  - Non-Certified Data Sources

    - The system shall generate a unified air picture with persistent tracks, associated confidence levels, and threat assessment. It shall provide real-time fusion, correlation, and tracking of cooperative and non-cooperative low-altitude airspace users, including but not limited to:

      - Uncrewed aircraft systems (UAS) of all classes.

      - Balloons and aerostats.

      - Small manned aircraft, ultralights, and gliders.

      - Other low-altitude or non-cooperative airspace users.

    - The system must ingest and fuse data from the following source categories into a single coherent air picture:

| Source Category | Description |
| --- | --- |
| RF Detection | Radio frequency monitoring and direction-finding sensors detecting drone C2 links, telemetry, video downlinks, and protocol signatures |
| Radar | Surveillance radar (short-range C-UAS, 3D air surveillance, passive radar) providing kinematic tracks on low, slow, and small targets |
| Electro-Optical / Infrared | EO and thermal imaging sensors for visual detection, AI-based classification, and target confirmation |
| Acoustic | Acoustic detection arrays exploiting UAS propulsion sound signatures for bearing estimation and classification |
| Cooperative Traffic | Remote ID (broadcast and network), ADS-B, or alternative (e.g. GSM-based) transponder traffic feeds,UTM/U-space feeds, flight plans, and UAS registration databases |
| External Intelligence | Geozones, no-fly zones, critical infrastructure protection zones |

    - The core requirement is a fusion engine that takes all the above sources and produces a single, coherent, real-time air picture. The following high-level requirements apply:

| Requirement | Description |
| --- | --- |
| Multi-Source Data Fusion | Fuse all source categories above into a single, persistent, de-duplicated air picture with unified track numbering |
| Track Correlation | Correlate detections across cooperative and non-cooperative sensors, resolving duplicates and maintaining track continuity |
| Confidence & Quality | Apply per-source confidence weighting based on sensor accuracy, range, and environmental conditions; present confidence levels on each track |
| Threat Assessment | Automatically classify and assess threat level using fused kinematic, classification, cooperative status, and geospatial context data |
| Sensor Cueing | Support cross-sensor cueing (e.g., radar or RF cueing an EO/IR sensor to confirm a detection) |
| Interface Standards | Support recognised integration standards (e.g., SAPIENT, ASTERIX, STANAG 4586) and standard transport protocols (TCP, UDP, MQTT, HTTPS) |
| Time Synchronisation | All data timestamped in UTC with sub-second precision, synchronised via NTP or PTP |
| Extensibility | Architecture must support addition of new sensor types without modification to the core fusion engine |
| Sensor Health Monitoring | Monitor connectivity and health status of all integrated sensors in real time |

  - Data Sources processing and filtering

    - The system shall support those functionalities:

      - Enable and disable reception and processing of sensor data.

      - Enable and disable sensor data filtering by vertical sectors/polygons.

      - Enable and disable sensor data filtering by horizontal sectors/polygons.

      - Disable the processing of sensor data if sensor data doesn’t meet the quality requirements.

      - Enable the processing of sensor data if sensor data meet the quality requirements.

# Data Fusion and Tracking

  - The MSDF tracker shall:

    - Perform multi-sensor correlation and tracking.

    - Support the coexistence of certified ATM data and non-certified lower-airspace (incl. Uncrewed and other cooperative or non-cooperative traffic) data.

    - Provide configurable fusion logic, allowing:

      - Full fusion of certified and non-certified sensor data.

      - Partial fusion based on selected sensor types or operational areas.

      - Complete separation of certified and non-certified tracks.

      - Addition of new sources without changing the core fusion engine functionality.

  - The fusion and separation mechanisms shall support operational, safety, and regulatory constraints, including segregation requirements between ATM-certified data and not certified lower-airspace traffic data when applicable.

# Output and Data Distribution

  - Output Format

    - The primary output format shall be ASTERIX Category 62.

    - The system shall be capable of generating:

      - Tracks derived exclusively from certified data sources.

      - Tracks derived exclusively from non-certified data sources.

      - Fused tracks combining certified and non-certified data, where enabled.

  - Data Stream Separation

    - The system shall provide mechanisms to:

      - Logically and/or physically separate data streams for:

        - Certified ATM data.

        - Non-certified lower airspace traffic data.

      - Independently distribute these streams to downstream systems.

  - Configurability and Switching

    - The system shall allow operators or administrators to:

      - Enable or disable fusion between certified and non-certified data sources.

      - Switch between:

        - Fully separated mode.

        - Fully fused mode.

        - Hybrid or partial fusion modes.

      - Apply configuration changes dynamically or with minimal system downtime.

# Interfaces and Integration

  - The system shall support Standardized interfaces for sensor data input and track data output.

  - Compatibility with common ATM and UTM related protocols and data formats is desirable.

  - The solution should support integration with:

    - Existing ATM surveillance data distribution  and tracking  systems.

    - UTM-type of systems (CIS, USSP, etc.), web-based UTM platforms.

# Cybersecurity Requirements (NIS2 Compliance)

  - The MSDF tracker solution shall comply with applicable European Union cybersecurity legislation, in particular Directive (EU) 2022/2555 (NIS2 Directive), and relevant national implementing legislation.

  - The proposed solution shall include cybersecurity measures appropriate for critical ATM and U-space operational environments and mixed-trust data sources.

# Service-Based and Virtualized Architecture

  - The MSDF tracker solution shall support modern ATM and U-space system environments and to allow flexible deployment and evolution of the solution over time.

  - The MSDF tracker solution should be designed as a service-based and virtualized system.

  - The solution should:

    - Be deployable as a software-based solution, independent of dedicated proprietary hardware.

    - Support virtualized environments, such as:

      - Virtual machines.

      - Container-based platforms.

      - Cloud or private cloud infrastructures.

    - Follow a service-based architecture, where system functions are implemented as independent or loosely coupled services.

    - Support flexible deployment, scaling, and lifecycle management of individual services

  - The architecture should enable:

    - Scalability based on traffic volume, number of sensors, and processing load.

    - Flexible integration with external ATM and U-space systems via service interfaces.

    - Efficient updates, maintenance, and future functional extensions.

# Operational and Architectural Considerations

  - The proposed solution should consider:

    - Scalability to support increasing numbers of sensors and tracked objects.

    - High availability and fault-tolerant architecture.

    - Cybersecurity measures appropriate for handling mixed-trust data sources.

    - Monitoring.

    - Logging.

    - Maintenance capabilities.

# High-Level System Architecture

This section presents a high-level architectural overview of the proposed Multi-Sensor Data Fusion (MSDF) tracker. The diagram illustrates the main functional building blocks of the system, including data ingestion from certified and non-certified sensor sources, sensor data filtering and control, tracking and fusion/separation logic, and distribution of tracking data to external systems. The architecture is intended to provide a conceptual view of the system structure and data flows and does not represent a detailed design or implementation specification.

Figure1. High-Level System Architecture

# Vendor Information Requested (for Market Consultation)

Vendors are invited to provide high-level, non-binding information on:

- Existing MSDF or tracking products and solutions.

- Experience with both ATM-certified and U-space-related data sources.

- Supported sensor types, interfaces, and data formats.

- ASTERIX Category 62 capabilities and customization options.

- System architecture and typical deployment models.

- Configurability related to fusion and separation of certified and non-certified data.

# CAPEX and OPEX estimation

  - General requirements

    - The Supplier shall provide an indicative or approximate price estimate for the proposed product or solution. The price should represent the expected total cost of acquisition, including all essential components, licenses, services, delivery, installation, and any recurring fees (if applicable).

    - The price estimate must include:

      - Breakdown of major cost elements (e.g., hardware, software, services, warranties, support.

      - One-time costs and recurring costs (if applicable).

      - Assumptions and conditions upon which the estimate is based.

Note: Supplier may provide price ranges where exact figures cannot be reasonably estimated. All information shared within this market consultation will remain confidential and used solely for planning and preparation of future procurement procedures

  - Warranty Cost Requirements

    - The Supplier shall include all cost elements related to the mandatory two (2) year warranty, including:

      - Corrective and preventive maintenance.

      - Patch management, security updates, and software fixes.

      - Hardware replacement and defect resolution.

      - Compliance with the specified Service Level Agreement (SLA).

    - Warranty-related costs must be presented separately from post-warranty OPEX.

  - Deployment Scenario Costing Requirements

    - The Supplier shall provide separate and scenario‑specific cost estimations for each of the following deployment models (if applicable):

      - On‑Premises Deployment

The Supplier shall provide:

        - Hardware specifications, sizing, and associated acquisition costs.

        - Data center resource requirements (e.g., power, cooling, rack units).

        - Network, security, and integration dependencies.

        - Estimated operational maintenance requirements.

      - Private Cloud Deployment

The Supplier shall provide:

        - Required virtual resources (CPU, RAM, storage, network).

        - Storage tiering and redundancy assumptions.

        - Backup and disaster recovery resource estimates.

        - Required middleware, virtualization, or orchestration platforms.

      - Customer Virtualized Infrastructure Deployment

The Supplier shall provide:

        - Supported virtualization platforms (e.g., VMware, Hyper‑V).

        - Required VM or container profiles (vCPU, RAM, storage).

        - OS and platform compatibility requirements.

        - Licensing model impacts for virtualized installations.

  - Support, Maintenance, and SLA Requirements

    - Support Services

      - The Supplier shall define SLA terms for both warranty and post-warranty support periods, including:

        - Response times.

        - Resolution times.

        - Service availability parameters.

        - Escalation procedures.

        - 24/7 or business-hour coverage levels.

    - System Updates and Upgrades

      - The Supplier shall estimate costs and activities related to:

        - Security updates and patches.

        - Functional updates and minor version releases.

        - Major version upgrades during the 10-year lifecycle.

        - Licensing impact of upgrades (if applicable).

__________________________________

EUROCONTROL Specification for ATM Surveillance System Performance (ESASSP) | EUROCONTROL

ED-87D | MASPS for A-SMGCS including new Airport safety Support Service Routing Service and Guidance Service - EUROCAE

Surveillance Data Distribution System (SDDS) | EUROCONTROL

Air traffic management surveillance tracker and server (ARTAS) | EUROCONTROL