Public Limited Liability Company „Oro Navigacija“

Technical Specification

for Search and Rescue Coordination and Decision Support System

Software development, DEPLOYMENT and integration

Contents

Subject	6

Reference Documents	6

List of Abbreviations	7

1.	Purpose and Scope	10

2.	System Overview	13

3.	General Requirements	13

4	Architecture	13

5.	Functional Requirements	14

5.1.	Alert Ingestion & Processing	14

5.2.	Mission Management (Optional)	15

5.3.	Outbound Messaging to Bodø NMCC	16

5.4.	Geospatial & Situation Awareness	16

5.5.	Beacons & Registry Data	16

5.6.	Reporting & Analysis	16

5.7.	User & Access Management	16

5.8.	Audit, Logging and Traceability	16

6.	Interfaces & Protocols	17

6.1.	Inbound/Outbound NMCC Feed (FTP/SFTP)	17

6.2.	Outbound to third party RCC	17

6.3.	Web Client API	17

6.4.	GIS	17

7.	Message and Schema Handling	17

7.1.	SIT 185 	17

7.2.	XML (optional)	17

7.3.	Error Handling	18

8.	Security Requirements	18

9.	Non-Functional Requirements	18

10.	Data flow and visualization	18

11.	Workflow Examples (Sequence)	19

11.1.	Alert to Mission	19

11.2.	Error flow	19

11.3.	Endpoint down flow	19

12.	Configuration and Administration	19

13.	Operations, Monitoring and Support	19

14.	Documentation and Training	20

15.	Technical Requirements	20

16.	Compliance	20

17.	Vulnerability assessment and penetration testing	21

18.	Support & Development	21

19.	Deliverables for market consultation	21

APPENDIX 1. DEFECT SEVERITY AND PRIORITY	23

APPENDIX 2. SOFTWARE DEVELOPMENT MILESTONES	24

APPENDIX 3. DATA FLOW AND VISUALIZATION DIAGRAM	25

Subject

Market consultation on the Search and Rescue Coordination and Decision Support System software development, deployment and integration at Vilnius Aeronautical Rescue Coordination Centre.

Note: This document is non-binding and does not constitute a procurement specification. It is provided solely to assess market capabilities and available technical solutions.

# Reference Documents

- ICAO Annex 12 _ Search And Rescue, 9th Edition, July 2024.

- IAMSAR Manual (International Aeronautical and Maritime Search and Rescue Manual), Volume I - Organization & Management (Doc 9731-1), 12th Edition, 2022.

- IAMSAR Manual (International Aeronautical and Maritime Search and Rescue Manual), Volume II - Mission Co-Ordination (Doc 9731-2), 9th Edition, 2022.

- COSPAS-SARSAT MISSION CONTROL CENTRES STANDARD INTERFACE DESCRIPTION Issue 8 – Revision 4, October 2024.

- COSPAS-SARSAT SYSTEM DOCUMENTS, https://www.cospas-sarsat.int/en/documents-pro/system-documents/system-documents.

# List of Abbreviations

| Abbreviation | Meaning |
| --- | --- |
| ADS-B | Automatic Dependent Surveillance-Broadcast |
| AIS | Automatic Identification System. It is a, automated, autonomous, and continuous tracking system used on ships and by vessel traffic services (VTS) to exchange information, such as vessel identification, position, course, and speed, with other ships and shore-based stations |
| API | Application Programming Interface |
| App | Application |
| ARCC | Aeronautical Rescue Coordination Centre |
| CIS | Center for Internet Security |
| CRUD | CRUD is an acronym for the four basic operations (Create, Read, Update, Delete) required to manage data in persistent storage (databases, file systems, etc.) |
| Cospas-Sarsat | International Satellite System for Search and Rescue |
| COTS | Commercial Off-The-Shelf |
| DR | Disaster Recovery |
| ELT | Emergency Locator Transmitter. A device typically installed on aircraft that automatically activates during a crash and transmits a distress signal |
| EPIRB | Emergency Position-Indicating Radio Beacon. Used primarily on ships; it sends a distress signal when activated, usually during maritime emergencies |
| FTP | File Transfer Protocol |
| GDPR | General Data Protection Regulation |
| GUI | Graphical User Interface |
| HEX | Hexadecimal |
| HSM | Hardware Security Module |
| IAMSAR | International Aeronautical and Maritime Search and Rescue Manual |
| ICAO | International Civil Aviation Organization |
| IDS | Intrusion Detection System |
| IPS | Intrusion Prevention System |
| JRCC | Joint Rescue Coordination Centre |
| LUT | Local Users Terminal |
| MCC | Mission Control Centre |
| NMCC | Norwegian Mission Control Centre |
| OPS | Operational |
| PLB | Personal Locator Beacon. A portable device carried by individuals (e.g., hikers, pilots) to send a distress signal in emergencies |
| PRE-OPS | Pre-operational |
| RBAC | Role-Based Access Control |
| RCC | Rescue Coordination Centre |
| RDBMS | Relational Database Management System |
| ROM | Rough Order of Magnitude: a preliminary, high-level estimate used early in the project lifecycle to determine feasibility and budget |
| RPO | Recovery Point Objective |
| RTO | Recovery Time Objective |
| SAML | Security Assertion Markup Language |
| SAR | Search and Rescue |
| SFTP | Secure File Transfer Protocol |
| SIEM | Security information and event management |
| SIT 185 | SIT 185 refers to a specific, standardized format for Cospas-Sarsat distress tracking messages used in aviation search and rescue |
| SPOC | Search and Rescue Points Of Contacts |
| SRU | Search and Rescue Unit |
| SSO | Single Sign-on |
| TLS | Transport Layer Security |
| UI | User Interface |
| UTC | Coordinated Universal Time |
| VLAN | Virtual Local Area Network |
| WAF | Web Application Firewall |
| WAL | WAL (Write-Ahead Log) archiving is the process of continuously copying PostgreSQL transaction logs to a secure, separate location, enabling point-in-time recovery (PITR) and database disaster recovery |
| Web SPA | Single Page Application |
| Web MPA | Multi Page Application |
| WMS | Web Map Service |
| WMTS | Web Map Tile Service |
| XML | Extensible Markup Language |

Introduction

Vilnius Aeronautical Rescue Coordination Centre (hereinafter – Vilnius ARCC) provides advanced planning, coordination and situational awareness for managing ELT, EPIRB, PLB related critical incidents within the Lithuanian Search & Rescue Region. Vilnius ARCC use the system with dedicated applications and Cospas-Sarsat interfaces for those operations.

The basic Cospas-Sarsat concept is composed of:

• distress radio beacons (ELTs for aviation use, EPIRBs for maritime use, and PLBs for personal use) which transmit signals during distress situations;

• instruments on board satellites in geostationary medium-earth orbit, and low altitude Earth orbits which detect the signals transmitted by distress radio beacons;

• ground receiving stations, referred to as Local Users Terminals (LUTs), which receive and process the satellite downlink signal to generate distress alerts; and

• Mission Control Centres (MCCs) receive alerts produced by LUTs and forward them to Rescue Coordination Centres (RCCs), Search and Rescue Points Of Contacts (SPOCs), or other MCCs.

MCCs perform the following functions:

• collect, store and sort the data from LUTs and other MCCs;

• provide data exchange within the Cospas-Sarsat System; and

• distribute alert and location data to associated RCCs or SPOCs.

This diagram illustrates the flow of distress signals in the Cospas-Sarsat system, including user segment (ELT, EPIRB, PLB), space segment (satellites), and ground segment (LUT, MCC, RCC, SAR).

# Purpose and Scope

Public limited liability company Oro navigacija is looking for the software development for Search and Rescue Coordination and Decision Support System (hereinafter – System) to be deployed and integrated at Vilnius ARCC. System shall provide distress alert’s management, SAR incident management, and situational awareness capabilities for managing ELT, EPIRB, PLB related critical incidents within the Lithuanian Search & Rescue Region.

Note: Technical specification contains general and high level requirements and prepared for the market consultation only. The final technical requirements for the actual deployment should be agreed at System design phase and it’s out of scope of this market consultation.

  - Purpose

    - System shall be designed to support Rescue Coordination Centre (RCC) officers in:

      - Receiving, validating, correlating, and prioritizing distress alerts from ELTs, EPIRBs, and PLBs from Bodø NMCC.

      - Sending outbound messages to Bodø NMCC.

      - Forwarding the received distress alerts to Maritime RCC (or other RCC).

      - Geospatial visualization of distress alerts, SAR Incidents:

        - Low-medium resolution multilayer (satellite, topo, etc.) geospatial visualization of distress alerts globally.

        - High resolution multilayer (orthophotographic, satellite, topo, etc.) geospatial visualization of distress alerts across the Lithuanian SRR. Orthophotographic map shall be included for the Lithuanian SRR.

        - Web SPA [Geospatial visualization] shall be used for geospatial visualization.

      - Distress alerts data sorting, filtering, forwarding and printing. Web MPA [SAR Incident Management] shall be used for SAR Incident Management.

      - Beacons and registry database data filing, editing, deleting, sorting, filtering, printing. Web SPA [Beacons and registry Management] shall be used for Beacons and registry management.

      - (Optional) Providing decision support, planning, and coordination tools (Mission Management GUI) for SAR missions.

    - System shall be designed to support System administrators in:

      - Audit, logging, reporting, and data retention.

      - Role-based access control (RBAC) and security hardening.

      - Maintaining the Beacons and registry database.

      - Maintaining2 the alert’s/incident’s database.

      - (Optional) Creating additional database.

  - In-scope

    - Deployment Diagram. Deployment Diagram for market consultation is provided in Figure 1.  Deployment Diagram shall be reviewed by Supplier and be amended if necessary. Final Deployment Diagram shall be agreed with Oro navigacija during the market consultation.

In-scope

Figure 1. Deployment Diagram

In-scope

System Architecture Diagram. Initial System Architecture Diagram for market consultation is provided in Figure 2. Initial System Architecture Diagram shall be reviewed by Supplier and be amended if necessary. Final System Architecture shall be agreed with Oro navigacija during the market consultation.

Figure 2. System Architecture Diagram

    - Technical specification for System technical infrastructure. Supplier shall provide the technical requirements for System technical infrastructure.

      - Oro Navigacija shall be responsible to provide System technical infrastructure for the System software installation.

    - Software installation. Supplier shall be responsible for the software installation into System technical infrastructure.

    - Integration. Supplier shall ensure System and Bodø NMCC FTP integration for SIT 185 and XML (optional) messages exchange [4].

      - Oro navigacija shall be responsible provide the secure VPN connection infrastructure for the FTP integration.

      - Oro navigacija shall be responsible lead the communication with Bodø NMCC for the FTP integration.

    - PRE-OPS System verification and testing. System testing shall be performed on PRE-OPS environment. The Bodø NMCC testing platform will be used for PRE-OPS testing.

    - Supplier shall be responsible for the PRE-OPS System testing program and testing procedures preparation.

    - PRE-OPS Site acceptance testing program and testing procedures for the System shall be delivered to Oro navigacija for the review and approval.

    - Supplier shall be responsible to perform PRE-OPS System testing on site. Oro Navigacija staff shall participate in PRE-OPS System testing on site.

    - Supplier shall be responsible to issue PRE-OPS Site Acceptance Test (SAT) report.

    - Pre-OPS SAT testing acceptance criteria shall comply with these requirements criteria:

      - There are no Priority 1, Priority 2 faults [ref. to Appendix 1].

      - 95% of tests steps are passed.

      - All comments raised in the Pre-OPS testing are closed or there is a plan to close them.

      - There is an agreed plan to fix detected Priority 3, Priority 4 faults and achieve 100% of test steps rate.

    - Pre-OPS testing acceptance note shall be signed by Supplier and Oro navigacija.

    - System validation on OPS environment. System validation shall be performed on OPS environment. The Bodø NMCC operational platform will be used for OPS validation and verification.

    - Supplier shall be responsible for the OPS System validation program and OPS testing procedures preparation.

    - OPS Site acceptance testing program and testing procedures for the System shall be delivered to Oro navigacija for the review and approval.

    - Supplier shall be responsible to perform OPS System testing on site. Oro Navigacija staff shall participate in OPS System testing.

    - Supplier shall be responsible to issue OPS Site Acceptance Test (SAT) report.

    - OPS SAT testing acceptance criteria shall comply with these requirements criteria:

      - There are no Priority 1, Priority 2 faults [ref. to Appendix 1]

      - 97% of tests steps are passed.

      - All comments raised in the OPS testing are closed or there is a plan to close them.

      - There is an agreed plan to fix detected Priority 3, Priority 4 faults and achieve 100% of test steps rate.

    - OPS testing acceptance note shall be signed by Supplier and Oro navigacija.

  - Out-of-scope

System expansion for the additional futures and functionalities are marked in this technical specification as optional items shall be available for the future System development.

# System Overview

The System comprises:

- Application Server: Core services, integrations, business logic, workflow engine, data processing.

- Database:  alert, beacon, asset, user, and audit data.

- Integration Gateway: FTP client for NMCC feed, message parsers (SIT 185 and XML (optional)), outbound message services.

- Web Client (RCC Officer App): Browser-based client for SAR situational awareness, SAR incidents management, Beacons and registry Management, geospatial presentation.

- GIS Services: Map tiles, layers, geocoding, coordinate conversions, overlays.

# General Requirements

  - The System shall:

    - Support COSPAS-SARSAT standards [5], data formats and protocols [4], and

      - be connected to Bodø NMCC, which is co-located with the Joint Rescue Coordination Centre (JRCC) Bodø in Northern Norway, for data exchange and processing.

    - Enable real-time exchange and processing of Search and Rescue (SAR) messages.

    - Have built-in test/self-test functionality.

    - Have status monitoring functionality (operational, degraded, not operational).

  - The software development process shall follow Agile methodology, ensuring that technical specifications are iteratively defined, refined, and validated throughout the project lifecycle.

# Architecture

  - Logical Architecture

- Presentation Layer: Web SPA/Web MPA, responsive UI, language - English.

- API Layer: REST (JSON) primary; optional SOAP for legacy.

- Service Layer:

  - Alert Ingestion Service (FTP polling, parsing, validation).

  - Correlation & De-duplication Service.

  - (Optional) Incident Management & Workflow Engine.

  - Notification Service (configurable).

  - (Optional) Mission Management & Workflow Engine.

  - (Optional) Reporting & Analytics Service.

  - Security & RBAC Service.

  - Audit & Logging Service.

- Integration Layer:

  - FTP Client to Bodø NMCC (inbound/outbound SIT 185 and XML (optional)).

  - Outbound Messaging to third parties (email).

  - (Optional) Read APIs for AIS/ADS-B (read-only overlays).

- Data Layer: RDBMS (e.g., PostgreSQL) with spatial extensions (PostGIS).

  - Deployment

- On-premises (Oro navigacija Data Centre, Balio karvelio str. 25, Vilnius. Lithuania).

- Environments: PRE-OPS, OPS.

- High Availability: Active–passive or active–active for server and DB (≥99.9% service availability target).

- Scalability: (Optional) Horizontal scaling of stateless services; DB vertical/horizontal scaling as needed.

  - Networking & Connectivity

- Inbound/Outbound: FTP(S)/SFTP connection to Bodø NMCC data endpoint (VPN; fixed IPs; firewall allow-list).

- Outbound to third parties: email.

- Internal: Segregated VLANs for front-end, app, DB; reverse proxy (TLS termination), WAF recommended.

# Functional Requirements

# Alert Ingestion & Processing

    - System shall poll Bodø NMCC via FTP(S)/SFTP for new files on a configurable schedule (default: 1–60s).

    - System shall parse SIT 185 and XML (optional) distress messages.

    - System shall validate message integrity (checksum where provided), schema (XSD), and required fields.

    - System shall extract beacon data (type: ELT/EPIRB/PLB, AIRCRAFT 24 BIT ADDRESS, HEX ID/15-hex, encoding/country, registration refs).

    - System shall extract positional data (lat/long, uncertainty, Doppler/MEO solution type, time).

    - System shall derive SRR membership to flag relevance (Lithuanian SRR, near-boundary incidents).

    - System shall auto-create an Alert entity (status: New, Validated, Closed) with full provenance.

    - System shall correlate duplicate alerts (same beacon, time window, spatial proximity) into a single Incident.

    - System shall prioritize alerts by severity (e.g., confirmed distress, test, self-test).

    - System shall notify RCC officers on reception of new alert. Audible and visual alarms shall be active until the acknowledgement.

    - RCC Officer shall have GUI to list all inbound and outbound messages and it’s contents. Data sorting and filtering shall be implemented.

    - System shall have SAR Incident database and Beacons and registry database data fusion for active and inactive alerts. Web SPA [Fusion] shall be used for fused data presentation.

      - RCC officer shall have an option to fuse active and inactive alert with corresponding beacon data.

      - The enable and disable alert and beacon data fusion function shall be deployed for both – active and inactive alerts.

    - System shall have active alert(s) and beacon data presentation on RCC Officer GUI.

      - Active alert shall be presented on SAR Incident Management GUI and Geospatial visualization GUI.

      - Active unacknowledged alert shall have visually identifiable markings.

      - RCC officer shall have an option to disable/enable the active alert presentation on Geospatial visualization GUI.

    - System shall have inactive alert(s) and beacon data presentation on RCC Officer GUI.

      - Inactive alert shall be presented on SAR Incident Management GUI.

      - RCC officer shall have an option to disable/enable the inactive alert presentation on Geospatial visualization GUI.

    - RCC officer shall have an option to select inactive and/or active alerts from SAR Incident database to be presented on SAR Incident Management GUI and Geospatial visualization GUI.

      - RCC officer shall have an option to fuse inactive alert with corresponding beacon data. Web SPA [Fusion] shall be used for the fused data presentation.

# Mission Management (Optional)

    - (Optional) Convert Alerts into Missions with unique IDs; link all associated alerts and evidence.

    - (Optional)Mission states: Planned → Activated → In-Progress → Suspended → Concluded → Archived.

    - (Optional) Assign and track SAR assets (helicopters, aircraft, vessels, ground teams) with availability and readiness.

    - (Optional) Define Search Areas (circles/polygons), Search Patterns (e.g., Expanding Square, Sector Search).

    - (Optional) Attach mission docs (PDF, images, notes); maintain version history.

    - (Optional) Checklist templates per incident type (aviation/maritime/land) per IAMSAR best practices.

    - (Optional) Time tracking (UTC and local), ops log, and decision rationale logging.

    - (Optional) Generate and distribute SITREPs, MAYDAY/relays summaries, and mission updates.

# Outbound Messaging to Bodø NMCC

    - The system shall allow RCC officer to send acknowledgments automatically and manually, status updates and test messages – manually to NMCC.

    - Ensure idempotency (duplicate-safe) and store delivery receipts/transaction IDs.

    - (Optional) Configurable routing rules for cross-border coordination (forward/cc neighbouring SRRs as permitted).

# Geospatial & Situation Awareness

    - Interactive map with base layers (orthophotographic, satellite, topo, etc.), admin boundaries, SAR zones, SRR boundaries.

    - Display alert positions and corresponding beacon information.

    - (Optional) Draw tools for search areas, tracks, and sectors; measurement tools (distance, bearing, ETA).

    - Coordinate formats: DD.DDDDD°, DD°MM’SS.S”, UTM (switchable).

    - Time display: UTC with local time toggle (Lithuania, GMT+2/ +3 DST).

    - (Optional) Weather information (METAR/TAF, marine weather) if available via APIs (read-only).

    - Map printing/export to PDF with legend and coordinates.

# Beacons & Registry Data

    - Store beacon HEX IDs, owner/registration details (if provided via message or registry lookup).

    - (Optional) Registry integration (read-only) if RCC has access (separate ICD).

    - Data minimization and masking for personal data in non-essential screens.

# Reporting & Analysis

    - (Optional) Dashboards (open alerts, missions by state, response times, outcomes).

    - (Optional) Periodic reports (monthly/quarterly/annual) exportable to PDF/Excel.

    - (Optional) KPI computation (e.g., Triage-to-Activation time, Time-to-On-Scene, Alert Correlation Accuracy).

# User & Access Management

    - RBAC roles: RCC Officer, Administrator.

    - SSO support (SAML 2.0 / OpenID Connect).

    - Fine-grained permissions (view/edit by mission, data sensitivity).

# Audit, Logging and Traceability

    - Immutable audit log for all CRUD operations, authentication events, configuration changes.

    - (Optional) Tamper-evident storage for legal defensibility.

    - (Optional) Retention policies (configurable; default ≥ 10 years for mission records unless national policy dictates otherwise).

# Interfaces & Protocols

# Inbound/Outbound NMCC Feed (FTP/SFTP)

- Transport: FTP over TLS (FTPS) preferred.

- Auth: Username/password or key-based; IP allow-list.

- File formats:

  - SIT 185 (txt/structured).

  - XML (optional) per NMCC ICD (XSD provided by NMCC).

- Mechanics:

  - Poll interval configurable; resumable downloads; checksum verification if provided.

  - Move-on-read with two-phase commit: temporary local staging → parse → persist → ack by moving file server-side (if permitted) or send control file (per ICD).

# Outbound to third party RCC

- Transport: standard email protocols.

# Web Client API

- REST/JSON endpoints with OpenAPI 3.x.

- JSON Web Tokens (JWT) with short-lived access tokens; refresh via SSO.

- Rate limiting (per user/app), request/response logging, correlation IDs.

# GIS

- WMS/WMTS tile layers via secure proxy.

- (Optional) Upload of GeoJSON/KML for mission planning layers.

# Message and Schema Handling

# SIT 185

- Parser shall extract: message id, time of detection, AIRCRAFT 24 BIT ADDRESS, beacon HEX ID, beacon type (ELT/EPIRB/PLB), position, uncertainty, solution method (Doppler/MEOSAR), country code, probable nature of distress if available, and routing metadata (origin MCC, destination RCC).

- Maintain raw payload and normalized fields; all timestamps stored in UTC.

# XML (optional)

- Enforce XSD validation; schema versioning (reject or route to quarantine if unsupported).

- Namespace-aware parsing; strict type coercion; default fallbacks logged.

# Error Handling

- Quarantine queue for malformed messages; operator tools to inspect and reprocess after correction.

- Detailed error codes and remediation hints; configurable alerts.

# Security Requirements

  - End-to-end TLS 1.2+ for all network traffic; mTLS for server-to-server with NMCC.

  - Secrets management (HSM or vault); credential rotation policies.

  - RBAC with least privilege; admin actions dual-control (4-eyes) for critical changes.

  - SIEM integration; log signing; time sync via NTP (authenticated).

  - Hardening (CIS benchmarks), WAF in front of APIs, IDS/IPS recommended.

  - Data encryption at rest (DB: AES-256; files: encrypted volumes).

  - GDPR: data minimization, access logging, purpose limitation, consent/notice where applicable, subject access request support.

  - Regular vulnerability scans and penetration tests (at least annually; after major releases).

# Non-Functional Requirements

- Availability: ≥ 99.9%.

- Redundancy: System shall be focused on data protection through regular and safe backups and manual recovery procedures.

- Performance:

  - Parse & persist inbound alert ≤ 3 seconds per message.

  - GUI: initial load ≤ 5 seconds under nominal load.

- Capacity:

  - Handle burst of 200 alerts/hour without degradation.

  - Retain 10 years of incident and mission (optional) data.

- Maintainability: System shall have modular design for updates and enhancements

- Localization: Full English UI; configurable formats (dates, numbers).

- Backup & DR:

  - Weekly full DB backup; 15-min WAL archiving (RPO ≤ 15 min).

  - DR environment RTO ≤ 4 hours.

# Data flow and visualization

  - Data flow and visualization diagram for market consultation is provided in APPENDIX 3. Data flow and visualization diagram shall be reviewed by Supplier and be amended if necessary. Final Data flow and visualization diagram shall be agreed with Oro navigacija during the market consultation.

# Workflow Examples (Sequence)

# Alert to Mission

    - FTP poll → new SIT 185 / XML (optional) file detected.

    - Download → validate → parse → create Alert.

    - Correlate → existing Incident? If yes, link; otherwise create new → audible/visual RCC Officer notification -> Acknowledgement request.

    - Acknowledge -> periodic notification until RCC Officer acknowledgement -> RCC Officer notified.

    - Incident progresses -> active -> inactive.

    - Incident concluded -> close Incident-> archive.

    - (Optional) Officer reviews → promotes to Mission → assigns assets.

    - (Optional) Mission progresses → periodic status updates.

    - (Optional) Mission concluded > close Mission-> → archive.

# Error flow

    - Invalid SIT 185 / XML (optional) → quarantine → RCC Officer notified.

    - Invalid SIT 185 / XML (optional) review -> read message -> archive message -> delete message.

# Endpoint down flow

    - NMCC endpoint down → retries → circuit breaker → RCC Officer notified → resume when available.

# Configuration and Administration

- Endpoints: NMCC FTP host, ports, credentials/keys, folders.

- Schedules: Poll intervals, retry policies.

- Mappings: Code lists (beacon types, severity), country codes, SRR polygons.

- Users/Roles: Provisioning, SSO federation configs.

# Operations, Monitoring and Support

- Runbooks: For FTP failures, parser failures, NMCC outages, DB failover, certificate rotation.

- Monitoring:

  - Service health (operational, degraded, not operational).

  - (Optional) Queue depths (inbound/outbound).

  - (Optional) Message lag and error rates.

- On-Call: (Optional) 24/7 incident response; P1 SLA (ack ≤ 15 min, restore ≤ 2 hours).

- Change Management: Versioning, stakeholder notifications, maintenance windows (optional).

# Documentation and Training

- System Admin Guide

- RCC Operator User Manual (mission workflows, SIT 185 / XML (optional) interpretation))

- Integration Documentation (ICDs, schemas, configuration)

- System Requirements Specification

- Security & Privacy Guide (GDPR, RBAC, audit procedures)

- Training: Classroom + sandbox with simulated alerts (ELT/EPIRB/PLB scenarios)

- Training documentation material

# Technical Requirements

  - System architecture shall be web-based client-server with secure access.

  - System shall be integrated with Cospas-Sarsat FTP communication, SIT 185 / XML (optional) message formats and protocols referring to COSPAS-SARSAT MISSION CONTROL CENTRES STANDARD INTERFACE DESCRIPTION [4].

  - System shall have one ARCC operator workstation.

  - System shall have role-based access control.

  - System shall have encrypted data transmission (details will be provided by Bodø NMCC).

  - System shall have real-time alert processing and map updates.

  - The Supplier shall ensure that System hardware and software has not announced the end of life or discontinued status.

  - The licences for System software shall be granted for the lifetime period of the System.

  - Supplier shall ensure that System supports future development options. The system shall be designed to accommodate future enhancements or changes. Any features labelled as "optional" in the technical specification documentation must be included in the planning, design, or architecture phase, ensuring they can be implemented later.

# Compliance

System shall comply with:

  - ICAO Annex 12 SAR [1] standard.

  - IAMSAR [2], [3] standards.

  - Cospas-Sarsat [5] data formats and transmission standards.

  - The Supplier including subcontractors shall comply with the requirements of Directive (EU) 2022/2555 of the European Parliament and of the Council of 14 December 2022 for measures to ensure a high common level of cybersecurity across the Union, amending Regulation (EU) No 910/2014 and Directive (EU) 2018/1972 and repealing Directive (EU) 2016/1148 (NIS 2 Directive) applicable to service providers and apply the appropriate measures referred to in Article 21(2) of the Directive.

# Vulnerability assessment and penetration testing

  - The Supplier shall ensure that the System does not contain any hidden features that weaken security, including malicious software, unauthorized access or functions, and other security breaches. Only after ensuring that the developed software does not contain malicious software code, unauthorized access capabilities, and/or other vulnerabilities, shall it be installed.

  - Because of cybersecurity regulation in National law, vulnerability assessment and vulnerability scanning should be done before the system will start to operate in the operational environment. This activity should be performed by Oro navigacija personnel or by third party organization, selected by the Supplier. Selected organization also needs to be approved by the Oro navigacija, as these vulnerability tests may disclose some sensitive information. Penetration testing companies should reside in EU or NATO countries.

# Support & Development

  - The Supplier shall provide not less than 2 (two) years warranty support.

  - The warranty period starts on the day when the Site Acceptance Test is signed.

  - The Supplier shall provide training courses for Vilnius ARCC personnel and technical personnel.

  - The Supplier shall provide post-warranty services for System bug fixes, upgrades.

  - The Supplier should provide System development services (on demand).

  - The Supplier should provide refresher courses for Vilnius ARCC personnel, technical personnel (on demand).

# Deliverables for market consultation

  - System documentation: System deployment diagram, System architecture diagram, Technical specification, Operational User manual, Technical User manual, System administration manual, etc.

  - System software development milestones. Please refer to the APPENDIX 2.

  - System software demonstration on-site or remotely, if software already available.

  - ROM. Please consider 2 years warranty services and indicate the prices for delivery of:

    - Software.

    - Software installation and integration.

    - Operational and technical personnel training.

    - PRE-OPS and OPS site acceptance testing.

    - Post warranty services. Please consider and up to 10 years post warranty services.

Note 1: facilities for hardware installation and network infrastructure will be provided by Oro navigacija.

Note 2: Hardware and network infrastructure will be provided by Oro navigacija.

Note 3: Remote access will be granted by Oro navigacija, if Supplier requests the software installation and integration remotely.

# APPENDIX 1. DEFECT SEVERITY AND PRIORITY

The Severity, classifying the impact, will be assigned following the criteria below:

Severity 1 – Critical

A defect that prevents core RCC operational functionality, causes system failure, incorrect distress alert handling, data corruption, or any condition that blocks mission-critical SAR operations. No workaround exists.

Severity 2 – Major

A defect that significantly impacts important functionality or system performance but does not stop RCC mission operations entirely. A workaround may exist, but it is not practical for operational use.

Severity 3 – Moderate

A defect that affects non-critical functionality, user interface behaviour, or secondary features. Operations can continue normally, and a reasonable workaround exists.

Severity 4 – Minor

A low-impact defect such as cosmetic issues, layout problems, minor inconsistencies, or text errors that do not affect operational use of the RCC application.

Defect Priority Categories (1–4)

Priority 1 – Immediate Fix Required

Defects that must be resolved before SAT can pass or before the application is used in any operational environment. Usually paired with Severity 1.

Priority 2 – High

Defects that should be fixed in the next available release or sprint, especially if they affect key workflows. Usually paired with Severity 2.

Priority 3 – Medium

Defects that should be corrected but do not affect acceptance. Usually planned into a future release. Usually paired with 3 items.

Priority 4 – Low

Defects that may be deferred, tracked, and resolved in routine maintenance. Usually paired with Severity 4.

# APPENDIX 2. SOFTWARE DEVELOPMENT MILESTONES

Software Development Milestones

# APPENDIX 3. DATA FLOW AND VISUALIZATION DIAGRAM

REGULATION (EU) 2016/679 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 27 April 2016 on the protection of natural persons with regard to the processing of personal data and on the free movement of such data, and repealing Directive 95/46/EC (General Data Protection Regulation)

database backups, backup verification, log monitoring, space check, integrity checks, cleanup tasks, security audit, patch management, performance tuning, disaster recovery drills