# „Technical Specifications“

TECHNICAL SPECIFICATIONS

(PU-14876/26) [ITP26] Snow plows for N3 class trucks

- TERMS AND ABBREVIATIONS

  - Buyer – AB Kelių priežiūra.

  - Supplier – an economic entity, including a natural person, private or public legal entity, other organizations and their divisions, or a group of such persons, with whom the Buyer concludes a Contract.

  - Contract – a contract concluded between the Buyer and the Supplier regarding the subject of the procurement.

  - Name of the purchase object –  Snow plows for N3 class trucks  (hereinafter – the Goods/Item or the Equipment)

- DESCRIPTION, SCOPE, AND REQUIREMENTS OF THE SUBJECT OF THE PROCUREMENT

  - The procurement is not divided into lots.

  - Description of the procurement object:

    - The Goods must comply with the technical requirements set out in the annexes to these Technical Specifications.

    - The following documentation, in Lithuanian or English, must be submitted together with the Goods (no later than the date of delivery of the Goods)::

- user manual;

- Work Safety Instructions;

- electrical system diagrams;

- Hydraulic system diagrams;

- spare parts catalog;

- EC Declaration of Conformity;

- detailed installation instructions (enabling the Buyer to perform the installation themselves);

- Maintenance manual intended for the Buyer’s qualified personnel to perform maintenance, warranty, and repair work.

    - The proposed Goods must be covered by a full warranty of at least 12 months, effective from the date of delivery and acceptance of the Goods. Longer warranty periods may apply to individual parts of the Goods, as specified in the Technical Specifications (e.g., corrosion protection - at least 72 months). The manufacturer’s warranty confirmations must be submitted along with the proposal. The Goods are purchased without installation. Installation work on the Buyer’s vehicles shall be performed by the Buyer itself in accordance with the installation instructions and diagrams provided by the Supplier. Before commencing installation work, the Supplier or the equipment Manufacturer must provide training to the Buyer’s technical staff covering the correct installation, removal, operation, and maintenance of the equipment. Following the training, the equipment manufacturer shall confirm in writing that the warranty obligations for the Goods remain valid. The training must take place at the following address: Kauno g. 72, Pagirių village, Garliava municipality, LT-53282, Kaunas district. The warranty remains valid even if installation, periodic technical maintenance, including warranty work and other tasks related to the maintenance of the Goods, are performed by the Buyer’s qualified personnel in accordance with the instructions provided by the Supplier.

    - During the warranty period, all scheduled mandatory maintenance work on the Goods and the maintenance kits required for such work (spare parts and materials needed for specific maintenance) must be detailed and submitted together with the proposal, specifying the scope of each service and the prices of the service kits for the entire warranty period, taking into account the condition that snow plows may be operated for up to 2000 operating hours per year. The Buyer is not obligated to purchase all planned maintenance packages if the relevant maintenance is not yet required based on the actual operating time or mileage of the snow plow. During the warranty period, the Supplier shall perform warranty repairs at the Buyer’s facilities at its own expense (along with the proposal, the Supplier must submit an authorization from the manufacturer of the proposed Goods or another document confirming the Supplier’s right to perform repair services during the warranty period or provide evidence of a contract for the provision of such services concluded with another economic operator holding such a right) or, together with the bid, submit a document issued by the Manufacturer confirming that the right to perform warranty repairs  during the warranty period is granted to the Buyer or other legal entities, with warranty repairs to be performed in accordance with the attached requirements specified by the manufacturer. In cases where the right to perform warranty repairs is transferred to the Buyer, the Supplier undertakes to cover all costs, without exception, related to the performance of warranty repairs, including but not limited to: labor costs, spare parts, materials, diagnostics, transportation, and other necessary expenses, by paying the invoice issued by the Buyer to the Supplier. Any work performed during the warranty period by the Buyer’s own personnel shall not affect the validity of the warranty, provided that such work is performed in accordance with the Supplier’s instructions and provided diagrams.  During the warranty period, the Supplier shall provide the necessary spare parts and materials for performing routine and seasonal technical maintenance, and the Buyer shall perform such maintenance using its own resources. If special equipment is required for warranty, diagnostic, technical maintenance, or other work on the Goods,  the Supplier must provide the Buyer with one set of the manufacturer’s designated diagnostic equipment and ensure free updates for all software components of the Goods and diagnostic equipment for a period of no less than 10 years, ensuring the Buyer’s ability to perform these tasks independently in accordance with the instructions provided by the Supplier. The Buyer is granted the right to use the software without additional license fees. The Buyer reserves the right, during the term of use of the Goods, to submit mandatory requirements, claims, or additional needs related to the diagnostic equipment, its functionality, compatibility, and adaptability, taking into account needs that have become apparent during operation and could not have been objectively determined at the time of acceptance of the Goods. The Supplier undertakes to ensure the fulfillment of such requirements, including the updating, supplementation, or expansion of the diagnostic equipment, at no additional cost to the Buyer.

    - Upon delivery of the Goods, but no later than when the Buyer begins to use the Goods or a part of the Goods set, the Seller must provide the following training, the cost of which must be included in the offer price:

- Theoretical, practical, and technical operational training for operators, covering the proper, correct, and safe installation and dismantling of equipment, the preservation of dismantled equipment, the operation and maintenance of the Goods, explanations of potential malfunctions, and other operational aspects.

- Theoretical, practical, and technical training for the Buyer’s technical staff, covering the proper, correct, and safe installation and dismantling of equipment, the preservation of dismantled equipment, the performance of maintenance and repair work, and other tasks related to the maintenance of the Goods, including training on the proper and safe maintenance of the Goods, the nature of possible malfunctions, and other operational aspects.

2.2.6. Before the warranty period for the Goods expires, the Supplier shall, at its own expense, perform a thorough inspection of the technical condition of the Goods and remedy any warranty defects found, or authorize the Buyer to perform such an inspection and remedy any warranty defects found in accordance with the instructions provided by the Supplier, and the costs incurred shall be reimbursed in accordance with the procedure set forth in the contract. The inspection shall be performed no later than 1 month prior to the expiration of the warranty..

  - Scope of the Procurement:

| Ser. No | Equipment name | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | Front snow plow (for AM and category 1 roads) | Unit | 26 |
| 2. | Front snow plow (for roads in other categories) | Unit | 18 |
| 3. | Side snow plow (for AM and category 1 roads) | Unit | 32 |
| 4. | Side snow plow (for roads in other categories) | Unit | 12 |

  - The Buyer purchases the Goods  with delivery. The Supplier undertakes to deliver the Goods free of charge to the specified address :

| Delivery address | Quantity |
| --- | --- |
| Kauno str. 72, Pagirių village, Garliava municipality, 53282 Kaunas district | 88 |

3. PROCEDURE AND TERMS FOR THE FULFILLMENT OF CONTRACTUAL OBLIGATIONS

  - Goods must be delivered no later than 150 calendar days  from the date of entry into force of the Contract.

  - The delivery time of the Goods is assessed in terms of economic efficiency, taking into account the principles set out in the tender evaluation criteria. If the Supplier fails to comply with the proposed delivery times for the Goods, the Buyer will apply late payment penalties until the Goods are delivered.

  - Individual sets of Goods may be delivered before the delivery date specified in the offer. The Supplier is obliged to specify the expected delivery date for each complete set in the offer.

4. ENVIRONMENTAL REQUIREMENTS

The Buyer seeks to ensure that its and the Supplier’s actions have as little impact on the environment as possible, therefore:

4.1.  During the public procurement and contract performance, communication between the Supplier and the Buyer shall be carried out only by electronic means (CPP IS means, telephone, e-mail, etc.);

4.2.  All documentation related to the performance of the Contract shall be provided to the Buyer and the Supplier by electronic means (e-mail, etc.);

4.3. The Contract shall be signed only by electronic means (electronic signature).

4.4. The Supplier shall undertake to reduce paper consumption, refrain from unnecessary copying and printing of documents, and if office supplies are used, they must be made from recycled raw materials or suitable for recycling.

4.5.  If the purchased Goods are to be delivered or transferred in secondary packaging, they must comply with the minimum environmental protection criteria set for packaging, unless this is contrary to hygiene standards. Packaging must be considered recyclable packaging in accordance with the provisions of the Republic of Lithuania Law on Environmental Pollution Tax.

4.6.  If the Goods purchased are delivered, the Supplier shall endeavour to ensure that the delivery of the Goods consumes fewer natural resources and thus complies with the environmental protection principle established by the Order of the Minister of the Environment of the Republic of Lithuania, i.e. to ensure that the optimal route is chosen when delivering the Goods to the delivery location specified by the Buyer.

5. ANNEXES

Annex 1 – Technical parameters

Annex 2 - Chassis visualization

__________

All references in the procurement document to a standard, technical certificate or general technical specifications mean that the contracting authority will accept evidence of equivalent measures from other participants. If the specification refers to a specific model or source of supply, a specific process characteristic of the goods or services supplied by a particular Supplier, or a trademark, patent, types, specific origin or production, which would give certain entities or certain products an advantage or lead to their rejection, an equivalent object to that specified may be submitted. If an equivalent object is offered, the Supplier undertakes to submit/justify/prove its equivalence to the object specified in the document.

Annex 1 – Technical parameters

TECHNICAL PARAMETERS

The following must be submitted:

Technical documentation from the manufacturer of the Item/Equipment (catalogues, brochures) and/or declarations from the manufacturer of the Item/Equipment (if the manufacturer’s technical documentation does not fully reflect the compliance of the offered Item/Equipment with the technical specification requirements) and other documents specified in the technical specifications or other equivalent documents proving the compliance of the offered Item/Equipment with the technical requirements. In these documents, the Supplier must indicate (i.e., clearly mark – highlight in colour and/or indicate with arrows, and/or underline, etc.) the specific locations in the documents provided where the required technical characteristics are described and indicate which point of the technical requirements they comply with.

OR

links to a publicly accessible website where the Buyer can verify the authenticity of the data provided, i.e., the compliance of the offered Item/Equipment with technical requirements. If the information provided on the specified website does not correspond to the data declared by the Supplier, the tender must be accompanied by a declaration from the manufacturer of the Item/Equipment or other equivalent documents confirming that the offered Item/Equipment comply with the technical specification requirements.

The Supplier’s declaration shall not be considered equivalent documentation, except in cases where the Supplier is the official representative of the manufacturer of the offered Item/Equipment.

In the event that the Supplier (who is not the manufacturer) submits technical documentation and/or declarations prepared by representatives of the manufacturer who have the relevant rights, the Supplier must also submit documents proving the relevant rights of the manufacturer’s representative together with the offer (authorization granted by the manufacturer of the Item/Equipment to prepare/approve technical documentation for the Item/Equipment).

If the technical documentation for the Item/Equipment is prepared by a Supplier who is not the manufacturer and who has not been granted the manufacturer’s authorization to prepare/approve technical documentation, in such a case the technical documentation prepared by the Supplier itself does not meet the criteria of objectivity, reliability, traceability of information, and other criteria, therefore, during the evaluation of the tender, the Buyer will consider that the Supplier has only provided declarative information that is not substantiated by objective data and will reject such a tender.

## Documents (technical documentation of the manufacturer of the Item/Equipment (catalogues, brochures) and/or declarations by the manufacturer of the Item/Equipment or other equivalent documents proving that the offered Item/Equipment comply with the technical characteristics) may be submitted in Lithuanian and/or English. When evaluating the Suppliers’ tenders and at the Buyer‘s request, the Suppliers shall be required to submit the specified documents or parts thereof, translated into Lithuanian, and a confirmation of the translation.

## All specified specific materials, solutions and/or specific names or standards of the Item/Equipment are subject to the “or equivalent” clause. A Supplier offering the Item/Equipment with equivalent characteristics must additionally indicate this in the technical specification and/or tender and prove by reliable means that the offered Item/Equipment is equivalent and fully complies with the requirements set out in the technical specification.

| Item No. | Characteristic Name | Features requested by the buyer | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the proposed Product, the name of the supporting document, and/or a reference to the source confirming the proposed parameters. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS FOR SNOW PLOWS (FOR ALL ITEMS IN THE PURCHASE) |  |  |  |
| 1.1 | Purpose. | Snow plows are designed for the effective removal of snow from roads and other hard surfaces, ensuring that snow is cleared from the roadway and the shoulder, and enabling the efficient removal of fresh, packed, or slushy snow without damaging the road surface. Snow plows must be designed to operate under conditions where clearing is performed while a class N3 vehicle is moving at a speed of at least 70 km/h, maintaining stable operation, high-quality clearing, reliable and safe operation, and ensuring traffic safety. | Yes/No (delete as appropriate). |
| 1.2 | Year of manufacture | Snow plows must be new, unused prior to the date of delivery, and manufactured no earlier than 2026. | Yes/No (delete as appropriate). Year of manufacture- (enter)___. |
| 1.3 | The painting process for snow plows | The painting process for metal surfaces must be carried out in accordance with the requirements of EN ISO 12944 C3M or an equivalent standard. The color of snow plows shall be orange (RAL 2011), gray (Pantone 7544C), or the manufacturer’s designated base (representative) color for the specific equipment group. Galvanized metal surfaces, as well as components and assemblies made of stainless steel, aluminum, or other corrosion-resistant materials, may be left unpainted unless technically necessary, provided that all materials and surfaces used are adequately protected and suitable for use in a salty and humid environment, ensuring long-term corrosion resistance, and that welded joints are properly treated. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 1.4 | Warranty against corrosion | For all components included in the set, without exception—at least 72 months. | Yes/No (delete as appropriate). Suggested parameter - ____ months |
| 1.5 | The right to perform warranty repairs | The supplier must be an official representative of the manufacturer of the goods being offered (if the supplier is not the manufacturer itself), authorized to repair the goods during the warranty period, or must have entered into a service agreement with another entity authorized to perform such repairs. The proposal must be accompanied by a letter of authorization from the manufacturer of the proposed goods or another document confirming the supplier’s right to perform repair services during the warranty period, or evidence of a contract for the provision of such services concluded with another economic operator possessing such rights or the Supplier must submit to the Buyer a power of attorney issued by the manufacturer of the Goods the right to perform repair services during the warranty period. | Yes/No (delete as appropriate). Title of the document submitted as evidence - _________ and page no. |
| 1.6 | Warranty repair terms | The Supplier must perform warranty repair work no later than 5 business days after the Buyer submits a request in writing (via email) and by telephone to the Seller’s designated representative specified in the contract, or the Supplier may authorize the Buyer, during the warranty period (upon presentation of an authorization issued by the manufacturer of the Goods), to have qualified employees of the Buyer perform warranty repairs, and to reimburse the costs of such work and other related expenses in accordance with the terms set forth in the contract. | Yes/No (delete as appropriate). Title of the document submitted as evidence - _________ and page no. |
| 1.7 | Spare parts during the warranty period | All necessary replacement parts during the warranty period shall be provided at the Supplier’s expense. This provision does not apply to parts or defects resulting from improper use of the Product, mechanical damage, or other causes related to the Buyer’s actions. | Yes/No (delete as appropriate). |
| 1.8 | Certificate of Confirmation | For operators and technical staff who have completed the theoretical and technical training and practical training, the Supplier (including partners—the sellers of individual components of the system) shall issue a certificate describing the scope of the training course. | Yes/No (delete as appropriate). |
| 1.9 | Security | All hydraulic hoses and fittings must be adequately protected against mechanical damage and environmental influences. Hydraulic hoses must be fully enclosed along their entire length in a protective plastic sleeve, spiral (corrugated) hose, or equivalent protection that is resistant to abrasion and impacts. All hydraulic and electrical connections (quick-connect couplings, plugs, etc.) must be supplied with tight-fitting protective caps (plugs) designed to protect the connections from dirt, moisture, corrosion, and mechanical damage during transport, storage, and operation when they are not connected. All hydraulic connections must have a designated and marked mounting or mounting location on the frame or structure so that they do not hang freely in the air and are not subject to mechanical stress or contact with other parts. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2. FRONT SNOW PLOW (FOR AM AND CATEGORY 1 ROADS) |  |  |  |
| 2.1 | Type of snow plow | Diagonal on both sides C-shaped front snow plow, with the left side lower while the right side is higher, and the upper part of the front snow plow, extending the entire length of the snow plow, is made of rigid plastic or an equivalent material and forms a C-shaped canopy that protects the vehicle’s windshield and front grille from snow and directs the flow of cleared snow toward the roadside. The central part of the snow plow is lower than the left and right sides of the snow plow. The design must ensure equally effective snow ejection to both sides, maintaining cleaning quality across different operating directions. The front snow plow must be equipped with a snow deflector or an equivalent solution that ensures controlled direction and height of the snow flow, thereby reducing snow throw onto road signs and other road structures and elements. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.2 | Purpose and Application | The front snow plow is designed to push snow to both the right and left sides. Designed for use with an N3-class truck on AM and category 1 roads. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.3 | Housing design and material | Made of steel or a similar material. The structure must have stiffening ribs or incorporate a similar technical solution that ensures at least the same structural rigidity and resistance to deformation. The working front snow plow plate (the part of the front snow plow that comes into contact with the snow) must be made of Hardox 450 or an equivalent wear-resistant material, with a thickness of no less than 3 mm. The thickness of the main metal frame shall be at least 5 mm (excluding stiffening ribs or equivalent structural features designed to increase rigidity) | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.4 | Installation of cleaning elements | The cleaning elements must be attached to the front snow plow via a mounting system in which the number of brackets is no less than the number of cleaning elements (i.e., at least one separate bracket must be provided for each cleaning element), ensuring that the holders for the cleaning elements have the ability to move (flex) independently of the snow plow‘s structure. The mounting solution must ensure the absorption of road surface irregularities, as well as the damping of impacts, vibrations, and noise during operation. The design must allow the cleaning element holders to bend upon impact with an obstacle of at least 5 cm and automatically return to their working position. Cleaning elements are fastened with bolts of strength class 10.9 or higher, with an additional metal strip at least 4 mm thick mounted behind the cleaning elements or an equivalent solution. This strip is designed to ensure a more even distribution of loads among the brackets of the connected cleaning elements, to ensure more uniform wear of the cleaning elements, to prevent lateral deformation of the brackets, and to increase the overall durability of the structure, especially when operating under difficult conditions. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.5 | Cleaning components | Cross-sectional layers of the working part of the cleaning elements: special wear-resistant steel, rubber, metal-ceramic elements embedded in the rubber, wear-resistant steel. The thickness of the working part is at least 36 mm. The distance between the mounting holes of the cleaning elements is 305 mm (with a tolerance of ±2 mm). The width of the cleaning elements is not less than 610 mm and not more than 915 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.6 | Shock Absorption and Obstacle Protection Mechanism | A shock absorption and obstacle protection system must be installed for the swivel cylinder to ensure pressure compensation and protection against sudden loads (shocks) during operation. The system may be implemented using a hydraulic shock-absorbing valve with a hydraulic accumulator or another equivalent technical solution (e.g., pressure-limiting valves in the cylinders) that ensures similar functionality and a comparable level of protection. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.7 | Mounting system and compatibility | DIN 76060-type mounting "A" mounting plate. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.8 | Management system | The front snow plow must be controlled by a hydraulic system designed to work with the existing (installed on the vehicle) hydraulic equipment and control panels, ensuring full compatibility with the existing hydraulic control system and control of all functions via the already installed control devices, without requiring additional control panels or significant system modifications. The number of required hydraulic connections (hydraulic pairs) shall not exceed 4. Hydraulic hoses must be equipped with quick-connect couplings compliant with ISO 7241-1, G 1/2" (external thread / male connection). The required hydraulic pressure is no more than 180 bar. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.9 | Angle of the snow plow | At least 35° in both directions. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.10 | Side shift of the snow plow | At least 320 mm from the vehicle’s longitudinal axis in both directions. The adjustment must be controlled from the driver’s cab using a hydraulic or similar system and must function throughout the entire operation without requiring additional stops or manual adjustment. Priority requirement T6 Points are awarded for greater displacement. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.11 | Angle of attack | Not less than 25° and not more than 30° (measured from the vertical plane V°) or not less than 60° and not more than 65° (measured from the horizontal plane H°) | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.12 | Length of the snow plow (Without signal and clearance markings) | At least 4600 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.13 | Height of the snow plow with cleaning elements (Excluding signal and clearance markings. The height is measured based on the side edges of the snow plow (left and right sides), while the height of the central section is not measured.) | Not less than 1400 mm and not more than 1700 mm | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.14 | Width of the cleaning path at an angle of ≥30° | At least 3200 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.15 | Width and length of the snow plow in the transport position | In the transport position, as defined by the manufacturer’s specified configuration, it must be ensured that: - the right edge of the snow plow does not protrude further than the side snow plow described in Section 4 of the technical specifications for this procurement; - The total width of the vehicle, including the front and side snow plow described in Section 4 of the technical specifications for this procurement, does not exceed 3600 mm; - The front snow plow extend forward from the mounting plate by no more than 3600 mm. The transport operation must comply with the applicable requirements of the Republic of Lithuania’s traffic regulations and other legal acts governing the dimensions of vehicles and their participation in public traffic. Other snow plow positions are considered working positions and are not evaluated under this section. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.16 | Weight of the snow plow (Without cleaning elements) | Not less than 1000 kg and not more than 1200 kg. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.17 | Weight reduction System | The equipment must be compatible and capable of operating with the vehicle’s existing hydraulic weight-reduction system, which allows the snowplow’s downforce to be adjusted from the driver’s cabin. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.18 | Surface copy function | The front snow plow must be equipped with a contour-following (floating) function that ensures the front snow plow automatically adapts to unevenness in the road surface and maintains constant, even contact with the surface being cleared throughout the entire operation, without requiring additional manual adjustment. The surface copying function may be implemented using a mechanical, hydraulic, or other equivalent technical solution. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.19 | Storage racks | Integrated or removable supports that allow the front snow plow to be stored in a stable position. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.20 | Support wheels | The frame of the front snow plow must be equipped with the manufacturer’s designated mounting points for support wheels (mounting locations, brackets, or similar solutions), allowing the Buyer to easily install the support wheels without any additional structural modifications. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.21 | Compatibility and total clearing width with a side snow plow | The front snow plow must be compatible for use in conjunction with the side snow plow, described in Section 4 of the Technical Specifications for this procurement, ensuring a total effective clearing width of at least 7.3 m on the roadway when the front snow plow is operating at an angle of at least 30°, and the side snow plow is operating at the maximum angle permitted by its technical capabilities. When both snow plows are in operation, their working widths must overlap so that, regardless of the clearing direction (whether snow is being pushed to the left or right), no uncleared section remains between the working zones of the snow plows, ensuring uniform and continuous clearing across the entire width of the surface. Along with the proposal, the Supplier shall submit visualizations showing: - the combined clearing efficiency of both snow plows, the overlap of their working areas, and the total width of the cleared surface when the front snow plow clears snow to the right; - the combined clearing efficiency of both snow plows, the overlap of their working areas, and the total width of the cleared surface when the front snow plow clears snow to the left. Visualizations must be submitted in PDF format, clearly indicating the working widths of the snow plows, the overlap zone, and the total clearing width. The outline of the vehicle must be visible in the visualization. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.22 | Lifting height of the front snow plow | The snow plow must be designed so that, when in the fully raised transport position, its lower edge rises by at least 250 mm, based on the snow plow‘s kinematics. This requirement applies only to the design of the snow plow itself (the lifting mechanism, hydraulic cylinder, and kinematic system) and is independent of the vehicle, the front mounting plate, or the mounting height. The lift height should be considered as the actual vertical displacement of the lower edge of the snow plow from the lowest working position to the transport position, rather than merely the stroke of the hydraulic cylinder or the movement of individual mechanism components. The snow plow‘s kinematic system must ensure that the entire width of the snow plow is raised by at least the specified amount (solutions in which only part of the snow plow or individual points meet the requirement are not permitted). | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.23 | Dimensional markings and reflectors | Install clearance posts, reflectors, and/or warning signs in accordance with the applicable road safety requirements of the Republic of Lithuania. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 2.24 | General lighting | Double-sided LED clearance lights that ensure the visibility and operation of the front snow plow in conditions of poor visibility. The LED lights must have a protection rating of at least IP65 against the ingress of solid objects and liquids. The entire electrical and lighting system must be designed to operate on a 24 V DC vehicle voltage. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3. FRONT SNOW PLOW (FOR ROADS IN OTHER CATEGORIES) |  |  |  |
| 3.1 | Type of snow plow | A diagonal C-shaped front snow plow, with the left side lower and the right is higher, and the upper part of the snow plow along its entire length is made of rigid plastic or an equivalent material and forms a C-shaped canopy that protects the vehicle’s windshield and front grille from snow and directs the flow of cleared snow toward the roadside. The design must ensure effective snow ejection to the right side. The front snow plow must be equipped with a snow deflector or an equivalent solution that ensures a controlled direction and height of the snow flow, thereby reducing snow accumulation on road signs and other road structures and elements. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.2 | Purpose and Application | Front snow plow designed to discharge snow to the right, but capable of clearing snow in both directions within the swivel angle specified in section 3.9. Designed for use on class N3 trucks on roads of category 2 and lower. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.3 | Housing design and material | Made of steel or a similar material. The structure must have stiffening ribs or incorporate a similar technical solution that ensures at least the same structural rigidity and resistance to deformation. The working front snow plow plate (the part of the front snow plow that comes into contact with the snow) must be made of Hardox 450 or an equivalent wear-resistant material, with a thickness of no less than 3 mm. The thickness of the main metal frame shall be at least 5 mm (excluding stiffening ribs or equivalent structural features designed to increase rigidity) | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.4 | Installation of cleaning elements | The cleaning elements must be attached to the front snow plow via a mounting system in which the number of brackets is no less than the number of cleaning elements (i.e., at least one separate bracket must be provided for each cleaning element), ensuring that the holders for the cleaning elements have the ability to move (flex) independently of the snow plow‘s structure. The mounting solution must ensure the absorption of road surface irregularities and the damping of impacts, vibrations, and noise during operation. The design must allow the cleaning element holders to bend upon impact with an obstacle of at least 5 cm and automatically return to their working position. Combined cleaning elements, fastened with bolts of strength class 10.9 or higher, with an additional metal strip at least 4 mm thick, mounted behind the cleaning elements or an equivalent solution. This strip is designed to ensure a more even distribution of loads among the brackets of the connected cleaning elements, to ensure more uniform wear of the cleaning elements, to prevent lateral deformation of the brackets, and to increase the overall durability of the structure, especially when operating under difficult conditions. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.5 | Cleaning components | Cross-sectional layers of the working part of the cleaning elements: special wear-resistant steel, rubber, metal-ceramic elements embedded in the rubber, wear-resistant steel. The thickness of the working part is at least 36 mm. The distance between the mounting holes of the cleaning elements is 305 mm (with a tolerance of ±2 mm). The width of the cleaning elements is not less than 610 mm and not more than 915 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.6 | Shock Absorption and Obstacle Protection Mechanism | A shock absorption and obstacle protection system must be installed for the swivel cylinder to ensure pressure compensation and protection against sudden loads (shocks) during operation. The system may be implemented using a hydraulic shock-absorbing valve with a hydraulic accumulator or another equivalent technical solution (e.g., pressure-limiting valves in the cylinders) that ensures similar functionality and a comparable level of protection. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.7 | Mounting system and compatibility | DIN 76060-type mounting "A" mounting plate. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.8 | Management system | The front snow plow must be controlled by a hydraulic system designed to work with the existing (installed on the vehicle) hydraulic equipment and control panels, ensuring full compatibility with the existing hydraulic control system and control of all functions via the already installed control devices, without requiring additional control panels or significant system modifications. The number of required hydraulic connections (hydraulic pairs) shall not exceed 4. Hydraulic hoses must be equipped with quick-connect couplings compliant with ISO 7241-1, G 1/2" (external thread / male connection). The required hydraulic pressure is no more than 180 bar. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.9 | Angle of the snow plow | At least 35° in both directions. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.10 | Side shift of the snow plow | At least 320 mm from the vehicle’s longitudinal axis in both directions. The adjustment must be controlled from the driver’s cab using a hydraulic or similar system and must function throughout the entire operation without requiring additional stops or manual adjustment. Priority requirement T6 Points are awarded for greater displacement. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.11 | Angle of attack | Not less than 15° and not more than 20° (measured from the vertical plane V°) or not less than 70° and not more than 75° (measured from the horizontal plane H°) | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.12 | Length of the snow plow (Without signal and clearance markings) | At least 4000 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.13 | Height of the snow plow with cleaning elements (Without signal and clearance markings) | Not less than 900 mm and not more than 1400 mm | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.14 | Width of the cleaning path at an angle of ≥30° | At least 3100 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.15 | Width and length of the snow plow in the transport position | In the transport position, as defined by the manufacturer’s specified configuration, it must be ensured that: - the right edge of the snow plow does not protrude further than the side snow plow described in Section 4 of the technical specifications for this procurement; - The total width of the vehicle, including the front and side snow plow described in Section 4 of the technical specifications for this procurement, does not exceed 3600 mm; - The front snow plow extend forward from the mounting plate by no more than 3600 mm. The transport operation must comply with the applicable requirements of the Republic of Lithuania’s traffic regulations and other legal acts governing the dimensions of vehicles and their participation in public traffic. Other snow plow positions are considered working positions and are not evaluated under this section. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.16 | Weight of the snow plow (Without cleaning elements) | Not less than 1000 kg and not more than 1200 kg. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.17 | Weight reduction System | The equipment must be compatible and capable of operating with the vehicle’s existing hydraulic weight-reduction system, which allows the snowplow’s downforce to be adjusted from the driver’s cabin. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.18 | Surface copy function | The front snow plow must be equipped with a contour-following (floating) function that ensures the front snow plow automatically adapts to unevenness in the road surface and maintains constant, even contact with the surface being cleared throughout the entire operation, without requiring additional manual adjustment. The surface copying function may be implemented using a mechanical, hydraulic, or other equivalent technical solution. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.19 | Storage racks | Integrated or removable supports that allow the front snow plow to be stored in a stable position. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.20 | Support wheels | The frame of the front snow plow must be equipped with the manufacturer’s designated mounting points for support wheels (mounting locations, brackets, or similar solutions), allowing the Buyer to easily install the support wheels without any additional structural modifications. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.21 | Compatibility and total clearing width with a side snow plow | The front snow plow must be compatible for use with the side snow plow, as described in Section 4 of the Technical Specifications for this procurement, ensuring a total effective clearing width of at least 7.0 m, when the front snow plow operates at an angle of at least 30°, and the side snow plow at the maximum angle according to its technical capabilities, and with the side snow plow described in Section 5 of the Technical Specifications for this procurement, ensuring a total effective clearing width of the roadway of no less than 6.3 m, when the front snow plow operates at an angle of no less than 30°, and the side snow plow at the maximum angle according to its technical capabilities. When both snow plows are in operation, their working widths must overlap so that no uncleared section remains between the working areas of the snow plows, ensuring uniform and continuous clearing across the entire width of the surface. Along with the proposal, the Supplier shall submit visualizations showing: - the combined clearing efficiency of both snow plows, the overlap of their working areas, and the total width of the cleared surface when snow is cleared to the right by the front snow plow in conjunction with the side snow plow, as described in Section 4 of this procurement document; - the combined clearing efficiency of both snow plows, the overlap of their working areas, and the total width of the cleared surface when snow is cleared to the right by the front snow plow in conjunction with the side snow plow, as described in Section 5 of this procurement document. Visualizations must be submitted in PDF format, clearly indicating the working widths of the snow plows, the overlap zone, and the total clearing width. The outline of the vehicle must be visible in the visualization. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.22 | Snow stop | The snow plow must be equipped with a snow flow brake that is controlled hydraulically, electrically, or by a similar method from the driver's cab and is mechanically free-floating (adapting to road surface irregularities), mounted on the right side of the snow plow. The snow flow brake must be at least 500 mm long and at least 200 mm high. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.23 | Lifting height of the front snow plow | The snow plow must be designed so that, when in the fully raised transport position, its lower edge rises by at least 250 mm, based on the snow plow‘s kinematics. This requirement applies only to the design of the snow plow itself (the lifting mechanism, hydraulic cylinder, and kinematic system) and is independent of the vehicle, the front mounting plate, or the mounting height. The lift height should be considered as the actual vertical displacement of the lower edge of the snow plow from the lowest working position to the transport position, rather than merely the stroke of the hydraulic cylinder or the movement of individual mechanism components. The snow plow‘s kinematic system must ensure that the entire width of the snow plow is raised by at least the specified amount (solutions in which only part of the snow plow or individual points meet the requirement are not permitted). | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.24 | Dimensional markings and reflectors | Install clearance posts, reflectors, and/or warning signs in accordance with the applicable road safety requirements of the Republic of Lithuania. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 3.25 | General lighting | Double-sided LED clearance lights that ensure the visibility and operation of the snow plow in conditions of poor visibility. The LED lights must have a protection rating of at least IP65 against the ingress of solid objects and liquids. The entire electrical and lighting system must be designed to operate on a 24 V DC vehicle voltage. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4. SIDE SNOW PLOW (FOR AM AND CATEGORY 1 ROADS) |  |  |  |
| 4.1 | Type of snow plow | A diagonal C-shaped side snow plow, with the left side lower and the right side (rear) higher. The design must ensure effective snow discharge to the right side. The side snow plow must be equipped with a snow deflector or an equivalent solution that ensures controlled direction and height of the snow flow, thereby reducing snow accumulation on road signs and other road structures and elements. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.2 | Purpose and Application | A side snow plow sesigned for use with an N3-class truck on AM and category 1 roads. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.3 | Housing design and material | Made of steel or a similar material. The structure must have stiffening ribs or incorporate a similar technical solution that ensures at least the same structural rigidity and resistance to deformation. The working side snow plow plate (the part of the side snow plow that comes into contact with the snow) must be made of Hardox 450 or an equivalent wear-resistant material, with a thickness of no less than 3 mm. The thickness of the main metal frame shall be at least 5 mm (excluding stiffening ribs or equivalent structural features designed to increase rigidity) | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.4 | Installation of cleaning elements | The cleaning elements must be attached to the side snow plow via a mounting system in which the number of brackets is no less than the number of cleaning elements (i.e., at least one separate bracket must be provided for each cleaning element), ensuring that the holders for the cleaning elements have the ability to move (flex) independently of the snow plow‘s structure. The mounting solution must ensure the absorption of road surface irregularities and the damping of impacts, vibrations, and noise during operation. The design must allow the cleaning element holders to bend upon impact with an obstacle of at least 5 cm and automatically return to their working position. Combined cleaning elements, fastened with bolts of strength class 10.9 or higher, with an additional metal strip at least 4 mm thick, mounted behind the cleaning elements or an equivalent solution. This strip is designed to ensure a more even distribution of loads among the brackets of the connected cleaning elements, to ensure more uniform wear of the cleaning elements, to prevent lateral deformation of the brackets, and to increase the overall durability of the structure, especially when operating under difficult conditions. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.5 | Cleaning components | Cross-sectional layers of the working part of the cleaning elements: special wear-resistant steel, rubber, metal-ceramic elements embedded in rubber, wear-resistant steel. The thickness of the working part is at least 36 mm. The distance between the mounting holes of the cleaning elements is 305 mm (with a tolerance of ±2 mm). The width of the cleaning elements is not less than 610 mm and not more than 915 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.6 | Shock Absorption and Obstacle Protection Mechanism | A shock absorption and obstacle protection system must be installed for the swivel cylinder to ensure pressure compensation and protection against sudden loads (shocks) during operation. The system may be implemented using a hydraulic shock-absorbing valve with a hydraulic accumulator or another equivalent technical solution (e.g., pressure-limiting valves in the cylinders) that ensures similar functionality and a comparable level of protection. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.7 | Mounting system and compatibility | The side snow plow must be supplied with a complete mounting system designed for the Renault K N3 four-wheel drive vehicle (the chassis drawing is provided in Appendix No. 2 to this TS), which: - ensures the distribution of loads across the vehicle's axles; - reduces the load on the frame and chassis; - ensures stable operation and vibration reduction; - allows for quick installation and/or removal of the side snow plow; - serves a protective function; The mounting system must be designed so that, in the event of a collision with an obstacle (in the event of an accident or other unforeseen circumstances), the loads are transferred to structural elements that have designated zones of deformation or failure (e.g., safety pins, joints, brackets, or hydraulic components), which: - protect the vehicle frame and main structural components from damage; - would deform or break first (the weaker element takes priority); - could be easily and quickly replaced under operating conditions without the need for complex repairs. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.8 | Management system | The side snow plow must be controlled by a hydraulic system designed to work with the existing (installed on the vehicle) hydraulic equipment and control panels, ensuring full compatibility with the existing hydraulic control system and control of all functions via the already installed control devices, without requiring additional control panels or significant system modifications. The number of required hydraulic connections (hydraulic pairs) shall not exceed 4. Hydraulic hoses must be equipped with quick-connect couplings compliant with ISO 7241-1, G 1/2" (external thread / male connection). The required hydraulic pressure is no more than 200 bar. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.9 | Angle of the snow plow | Horizontally no less than 25°. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.10 | Angle of attack | Not less than 25° and not more than 30° (measured from the vertical plane V°) or not less than 60° and not more than 65° (measured from the horizontal plane H°) | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.11 | Length of the snow (Without signal and clearance markings) | Not less than 4700 mm and not more than 5000 mm | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.12 | Height of the snow plow (Without signal and clearance markings) | Front (left) section not less than 600 mm and not more than 1300 mm; Rear (right) section not less than 1300 mm and not more than 1500 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.13 | Cleaning width at maximum swivel angle | At least 3800 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.14 | Width of the snow plow in the stowed position (distance from the side of the vehicle) | No more than 900 mm | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.15 | Weight of the snow plow (Without cleaning elements) | Not less than 1200 kg and not more than 1400 kg. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.16 | Weight reduction system | The equipment must be compatible and capable of operating with the vehicle’s existing hydraulic weight-reduction system, which allows the snowplow’s downforce to be adjusted from the driver’s cabin. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.17 | Surface copy function | The side snow plow must be equipped with a contour-following (floating) function that ensures the side snow plow automatically adapts to unevenness in the road surface and maintains constant, even contact with the surface being cleared throughout the entire operation, without requiring additional manual adjustment. The surface copying function may be implemented using a mechanical, hydraulic, or other equivalent technical solution. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.18 | Storage racks | Integrated or removable supports that allow the side snow plow to be stored in a stable position. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.19 | Support wheels, skids | The frame of the side snow plow must be equipped with the manufacturer’s designated mounting points for support wheels, skids, or other rolling/sliding elements specified by the manufacturer (mounting points, brackets, or similar solutions) that allow the Buyer to easily install them without additional structural modifications. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.20 | Compatibility and total clearing width with a front snow plow | The side snow plow must be compatible for use in conjunction with the front snow plow, as described in Section 2 of the Technical Specifications for this procurement, ensuring a total effective clearing width of at least 7.3 m when the front snow plow is operating at an angle of at least 30°, and the side snow plow at the maximum angle according to its technical capabilities and with the front snow plow, described in Section 3 of the Technical Specifications for this procurement, ensuring a total effective clearing width of the roadway of no less than 7.0 m, when the front snow plow operates at an angle of no less than 30°, and the side snow plow at the maximum angle according to its technical capabilities. When both snow plows are in operation, their working widths must overlap so that no uncleared section remains between the working areas of the snow plows, ensuring uniform and continuous clearing across the entire width of the surface. Along with the proposal, the Supplier shall submit visualizations showing: - the combined clearing efficiency of both snow plows, the overlap of their working areas, and the total width of the cleared surface when snow is cleared using the side snow plow in conjunction with the front snow plow, as described in Section 2 of this procurement document; - the combined clearing efficiency of both snow plows, the overlap of their working areas, and the total width of the cleared surface when snow is cleared using the side snow plow in conjunction with the front snow plow, as described in Section 3 of this procurement document. Visualizations must be submitted in PDF format, clearly indicating the working widths of the snow plow, the overlap zone, and the total clearing width. The outline of the vehicle must be visible in the visualization. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.21 | Lifting height of the side snow plow | The snow plow must be designed so that, when in the fully extended working position, its lower edge rises at least 250 mm, as determined by the snow plow‘s kinematics. This requirement applies only to the design of the snow plow itself (the lifting mechanism, hydraulic cylinder, and kinematic system) and is independent of the vehicle or the installation height. The lift height should be considered as the actual vertical displacement of the lower edge of the snow plow from its lowest working position, rather than merely the stroke of the hydraulic cylinder or the movement of individual mechanism components. The snow plow‘s kinematic system must ensure that the entire width of the snow plow is raised by at least the specified amount (solutions in which only part of the snow plow or individual points meet the requirement are not permitted). | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.22 | Dimensional markings and reflectors | Install clearance posts, reflectors, and/or warning signs in accordance with the applicable road safety requirements of the Republic of Lithuania. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 4.23 | General lighting | Double-sided LED clearance lights that ensure the visibility and operation of the snow plow in low-visibility conditions. At least 3 orange LED flashers directed toward the rear of the vehicle, which turn on automatically when the side snow plow is activated. The LED lights must have a protection rating of at least IP65 against the ingress of solid objects and liquids. The entire electrical and lighting system must be designed to operate on the vehicle’s 24 V direct current (DC) voltage. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5. SIDE SNOW PLOW (FOR ROADS IN OTHER CATEGORIES) |  |  |  |
| 5.1 | Type of snow plow | A diagonal C-shaped side snow plow, with the left side lower and the right side (rear) higher. The design must ensure effective snow discharge to the right side. The snow plow must be equipped with a snow deflector or an equivalent solution that ensures controlled direction and height of the snow flow, thereby reducing snow accumulation on road signs and other road structures and elements. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.2 | Purpose and Application | A side snow plow designed for use on class N3 trucks on roads of category 2 and lower. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.3 | Housing design and material | Made of steel or a similar material. The structure must have stiffening ribs or incorporate a similar technical solution that ensures at least the same structural rigidity and resistance to deformation. The working side snow plow plate (the part of the side snow plow that comes into contact with the snow) must be made of Hardox 450 or an equivalent wear-resistant material, with a thickness of no less than 3 mm. The thickness of the main metal frame shall be at least 5 mm (excluding stiffening ribs or equivalent structural features designed to increase rigidity) | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.4 | Installation of cleaning elements | The cleaning elements must be attached to the side snow plow via a mounting system in which the number of brackets is no less than the number of cleaning elements (i.e., at least one separate bracket must be provided for each cleaning element), ensuring that the holders for the cleaning elements have the ability to move (flex) independently of the snow plow‘s structure. The mounting solution must ensure the absorption of road surface irregularities, as well as the damping of impacts, vibrations, and noise during operation. The design must allow the cleaning element holders to bend upon impact with an obstacle of at least 5 cm and automatically return to their working position. Combined cleaning elements, fastened with bolts of strength class not lower than 10.9, with an additional metal strip at least 4 mm thick, mounted behind the cleaning elements or an equivalent solution. This strip is designed to ensure a more even distribution of loads among the brackets of the connected cleaning elements, to ensure more uniform wear of the cleaning elements, to prevent lateral deformation of the brackets, and to increase the overall durability of the structure, especially when operating under difficult conditions. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.5 | Cleaning components | Cross-sectional layers of the working part of the cleaning elements: special wear-resistant steel, rubber, metal-ceramic elements embedded in rubber, wear-resistant steel. The thickness of the working part is at least 36 mm. The distance between the mounting holes of the cleaning elements is 305 mm (with a tolerance of ±2 mm). The width of the cleaning elements is not less than 610 mm and not more than 915 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.6 | Shock Absorption and Obstacle Protection Mechanism | A shock absorption and obstacle protection system must be installed for the swivel cylinder to ensure pressure compensation and protection against sudden loads (shocks) during operation. The system may be implemented using a hydraulic shock-absorbing valve with a hydraulic accumulator or another equivalent technical solution (e.g., pressure-limiting valves in the cylinders) that ensures similar functionality and a comparable level of protection. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.7 | Mounting system and compatibility | The side snow plow must be supplied with a complete mounting system designed for the Renault K N3 four-wheel drive vehicle (a chassis drawing is provided in Appendix No. 2 to this TS), which: - ensures the distribution of loads across the vehicle's axles; - reduces the load on the frame and chassis; - ensures stable operation and vibration reduction; - allows for quick installation and/or removal of the side snow plow; - serves a protective function; The mounting system must be designed so that, in the event of a collision with an obstacle (in the event of an accident or other unforeseen circumstances), the loads are transferred to structural elements that have designated deformation or failure zones (e.g., safety pins, joints, brackets, or hydraulic components), which: - protect the vehicle frame and main structural components from damage; - would deform or break first (the weaker element takes priority); - could be easily and quickly replaced under operating conditions without the need for complex repairs. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.8 | Management system | The side snow plow must be controlled by a hydraulic system designed to work with the existing (installed on the vehicle) hydraulic equipment and control panels, ensuring full compatibility with the existing hydraulic control system and control of all functions via the already installed control devices, without requiring additional control panels or significant system modifications. The number of required hydraulic connections (hydraulic pairs) shall not exceed 4. Hydraulic hoses must be equipped with quick-connect fittings compliant with ISO 7241-1, G 1/2" (external thread / male connection). The required hydraulic pressure is no more than 200 bar. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.9 | Angle of the snow plow | Horizontally no less than 25°. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.10 | Angle of attack | Not less than 25° and not more than 30° (measured from the vertical plane V°) or not less than 60° and not more than 65° (measured from the horizontal plane H°) | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.11 | Length of the snow (Without signal and clearance markings) | Not less than 3900 mm and not more than 4200 mm | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.12 | Height of the snow plow (Without signal and clearance markings) | Front (left) section not less than 600 mm and not more than 1300 mm; Rear (right) section not less than 1300 mm and not more than 1500 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.13 | Cleaning width at maximum swivel angle | At least 2900 mm. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.14 | Width of the snow plow in the stowed position (distance from the side of the vehicle) | No more than 900 mm | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.15 | Weight of the snow plow (Without cleaning elements) | Not less than 1000 kg and not more than 1200 kg. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.16 | Weight reduction system | The equipment must be compatible and capable of operating with the vehicle’s existing hydraulic weight-reduction system, which allows the snowplow’s downforce to be adjusted from the driver’s cabin. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.17 | Surface copy function | The side snow plow must be equipped with a contour-following (floating) function that ensures the side snow plow automatically adapts to unevenness in the road surface and maintains constant, even contact with the surface being cleared throughout the entire operation, without requiring additional manual adjustment. The surface copying function may be implemented using a mechanical, hydraulic, or other equivalent technical solution. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.18 | Storage racks | Integrated or removable supports that allow the side snow plow to be stored in a stable position. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.19 | Support wheels, skids | The frame of the side snow plow must be equipped with the manufacturer’s designated mounting points for support wheels, skids, or other rolling/sliding elements specified by the manufacturer (mounting points, brackets, or similar solutions), allowing the Buyer to install them easily without additional structural modifications. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.20 | Compatibility and total clearing width with a front snow plow | The side snow plow must be compatible for use in conjunction with the front snow plow, described in Section 3 of the Technical Specifications for this procurement, ensuring a total effective clearing width of at least 6.3 m on the roadway when the front snow plow is operating at an angle of at least 30°, and the side snow plow at its maximum angle according to its technical capabilities. When both snow plows are in operation, their working widths must overlap so that, regardless of the clearing direction (whether snow is being pushed to the left or right), no uncleared section remains between the working zones of the snow plows, ensuring uniform and continuous clearing across the entire width of the surface. Along with the proposal, the Supplier shall submit visualizations showing: - the combined clearing efficiency of both snow plows, the overlap of their working areas, and the total width of the cleared surface when the front snow plow clears snow to the right; - the combined clearing efficiency of both snow plows, the overlap of their working areas, and the total width of the cleared surface when the front snow plow clears snow to the left. Visualizations must be submitted in PDF format, clearly indicating the working widths of the snow plows, the overlap zone, and the total clearing width. The outline of the vehicle must be visible in the visualization.. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.21 | Lifting height of the side snow plow | The snow plow must be designed so that, when in the fully extended working position, its lower edge rises at least 250 mm, as determined by the snow plow‘s kinematics. This requirement applies only to the design of the snow plow itself (the lifting mechanism, hydraulic cylinder, and kinematic system) and is independent of the vehicle or the installation height. The lift height should be considered as the actual vertical displacement of the lower edge of the snow plow from its lowest working position, rather than merely the stroke of the hydraulic cylinder or the movement of individual mechanism components. The snow plow‘s kinematic system must ensure that the entire width of the snow plow is raised by at least the specified amount (solutions in which only part of the snow plow or individual points meet the requirement are not permitted). | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.22 | Dimensional markings and reflectors | Install clearance posts, reflectors, and/or warning signs in accordance with the applicable road safety requirements of the Republic of Lithuania. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 5.23 | General lighting | Double-sided LED clearance lights that ensure the visibility and operation of the snow plow in conditions of poor visibility. At least 3 orange LED flashing lights directed toward the rear of the vehicle, which turn on automatically when the side snow plow is activated. The LED lights must have a protection rating of at least IP65 against the ingress of solid objects and liquids. The entire electrical and lighting system of the product must be designed to operate on the vehicle’s 24 V DC voltage. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| - ADDITIONAL PRIORITY REQUIREMENTS |  |  |  |
| 6.1. | Priority requirement T3 Mounting system and compatibility (Front snow plow for AM and category 1 roads and Front snow plow for roads in other categories) | The front snow plow features an interchangeable mounting system that allows it to be mounted on other types of front plates (e.g., DIN 76060 Type B or DIN EN 15432-1). | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 6.2. | Priority requirement T4 Preparation for installing an additional blade (All items) | The frame of the snow plow must be designed by the manufacturer to accommodate the installation of an additional (second) plow blade intended to improve clearing performance, including mounting points, brackets, and the ability to integrate with the hydraulic system. The design must allow for the installation of a hydraulically operated additional snow plow blade that operates independently or in conjunction with the main snow plow, without requiring additional structural modifications. Along with the proposal, the supplier must submit technical documentation for the additional (second) snow plow blade for which the equipment is designed, : the manufacturer’s technical specifications (in PDF format) and drawing(s) demonstrating compatibility with the proposed snow plow blade, as well as the intended installation and integration options. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |
| 6.3. | Priority requirement T5 Snow plow tilt angles (Side snow plow for AM and category 1 roads and Side snow plow for roads in other categories) | The side snow plow has an adjustable vertical tilt angle ranging from at least -7° to +7°. | Yes/No (delete as appropriate). Suggested parameter - ___________________________ Title of the document submitted as evidence - _________ and page no. |

Appendix No. 2 to the Technical Specifications