PUBLIC LIMITED LIABILITY COMPANY ORO NAVIGACIJA

# TECHNICAL SPECIFICATION FOR

# VILNIUS AERODROME AREA MULTILATERATION SURVEILLANCE SYSTEM
(V-MLAT) AND ITS MAINTENANCE SERVICES

CONTENTS

1. INTRODUCTION	4

1.1. Overview	4

1.2. Procurement object	4

1.3. Scope of procurement object	4

2. GENERAL REQUIREMENTS	5

2.1. Legal acts, specifications and standards	6

2.2. Logistics	6

3. Coverage and accuracy requirements	6

3.1. Accuracy in the horizontal plane	6

3.2. Periodicity of output	8

3.3. Probability of Detection	8

3.4. Types of Transponders	8

3.5. Interrogation	9

3.6. Replies reception	9

3.7. Information derivation by response type	10

3.8. Garbling, FRUIT and jamming	11

3.9. Data output management	11

3.10. Target capacity	12

4. Detection of unmanned aircraft	12

5. V-MLAT architecture	12

6. System structure	13

6.1. Software and hardware design	14

6.2. Central processing system (CPS)	14

6.3. Ground station (GS)	14

6.4. Control and monitoring system (CMS)	15

6.5. Energy supply infrastructure (ESI)	17

6.6. Environmental condition support equipment (ESCE)	18

6.7. Test and evaluation system (TES)	18

6.8. Data transmission infrastructure (DTI)	19

6.9. BITE	19

6.10. Time synchronization	20

7. System operation mode	20

7.1. Operational mode	20

7.2. Maintenance mode	20

7.3. Degraded operating mode	20

8. CYBER SECURITY REQUIREMENTS	21

8.1. Users authorization	21

9. ENVIRONMENT CONDITIONS	21

9.1. General information	21

9.2. Environmental resistance and testing methods	21

9.3. Ambient environment	21

9.4. Environmental tests	22

10. INSTALLATION CONDITIONS	22

10.1. General information	22

10.2. Scope	22

10.3. Installation requirements	23

11. RELIABILITY	24

11.1. Availability	24

11.2. Maintainability	25

12. V-MLAT OPERATION AND DURATION	26

12.1. Warranty and liabilities during the warranty period	26

12.2. V-MLAT maintenance services	27

12.3. V-MLAT hardware repair services	28

12.4. Spare parts	29

13. SAFETY	30

14. TRAINING	30

15. QUALITY ASSURANCE AND CONTROL	31

15.1. Acceptance tests, definitions and milestones	31

15.2. Project management	34

15.3. Documents	35

Annexes	37

# INTRODUCTION

## Overview

Public limited liability company Oro Navigacija (ANSP of the Republic of Lithuania, hereinafter referred to as the Customer) intends to improve air traffic control services at Vilnius Aerodrome and in the vicinity of Vilnius Aerodrome; therefore, a local multilateration surveillance system is being procured, the requirements for which are set forth in this technical specification.

## Procurement object

The procurement object shall consist of:

one local multilateration surveillance system installed at Vilnius Aerodrome (hereinafter collectively referred to as the System or V-MLAT – Vilnius Multilateration Surveillance System), consisting of hardware and software, as well as the installation of the System (hereinafter referred to as “installation”) and other services specified in the Technical Specifications Subitem [TBD]of the Technical Specification.

V-MLAT maintenance services, the provision of which shall be commenced after the V-MLAT warranty period, as defined under Subitem [TBD] of the Technical Specification.

V-MLAT hardware repair services, which are procured in accordance with the demand after the end of V-MLAT warranty period, as defined under Subitem [TBD] of the Technical Specification.

## Scope of procurement object

### V-MLAT

which shall be composed of (the price of the purchase shall include):

- Hardware – physical infrastructure necessary for the full, reliable, continuous and uninterrupted provision of the surveillance service, including but not limited to radio and electronic signal transmission subsystems, antennas and their control subsystems, central processing units, data transmission interfaces, other necessary components, housing containers and technical enclosures for equipment installation, which must contain all internal electronic and climate control systems and wiring necessary to ensure appropriate conditions for the normal operation and maintenance of the V-MLAT system;

- a set of spare parts, including all non-standard tools and/ or devices (see Subitem [TBD]), which are necessary for the maintenance and repair of the V-MLAT;

- Software and firmware, including its licenses not limited to that of the original manufacturer, but also third-party software supplied. It should be stressed that data communication interfaces (switches, routers, applications, etc.) or other components requiring a subscription license fee must be clearly identified in the proposal in order to indicate the extent of such costs.

- Software for simulating changes to the layout architecture of Ground stations, which allows for theoretical modelling of the impact of changes by structures and the environment within the aerodrome area on the System’s operation (see Subitem [TBD]).

Services:

- V-MLAT architecture and documentation preparation, updates taking into consideration the actual installation conditions/ opportunities;

- V-MLAT and component factory acceptance tests (FAT);

- dismantling some components of existing MLAT on site and installing new V-MLAT and components in the operational environment;

- V-MLAT and component on site acceptance tests (SAT);

- Customer employee training;

- V-MLAT pilot operation, transition into operational environment (see item [TBD]);

- A warranty indicated under the Tender of the Supplier, but not shorter than 36 months.

### V-MLAT maintenance services

V-MLAT maintenance services shall comprise:

- V-MLAT fault, operation disruption (malfunction) identification and elimination (when the Customer is unable to perform that), including cases where this relates solely to software malfunctions;

- Consulting on V-MLAT operation, usage, equipment maintenance, fault identification and elimination matters as well as on changes to the architectural layout of the System’s stations and their adaptation to changing operational locations;

- V-MLAT software (including software and firmware, operating systems, applications and software of third-parties) updates and licences, which are necessary for ensuring proper operation of the V-MLAT.

### V-MLAT hardware repair services

V-MLAT hardware repair services shall comprise undetermined number of repairs of any parts, components of the V-MLAT or acquisition of new parts, components (when they cannot be repaired or the repair is not cost-efficient) without exceeding the maximum amount allocated under the Contract for the acquisition of these services.

# GENERAL REQUIREMENTS

GEN_REQ-001	Before installing the System, the Supplier must inspect potential installation sites to verify and coordinate the locations of the Ground stations with the Customer. The installation location of the System’s central processing system is the Customer registered office at Balio Karvelio g. 25, Vilnius.

GEN_REQ-002	The V-MLAT shall be installed, operational and handed over to the Customer no later than 18 months after the entry into force of the Contract. In unforeseen circumstances, this deadline may be extended by up to 3 months in accordance with the procedures laid down in the Contract. Operational readiness of the V-MLAT means the installation of the V-MLAT in accordance with the terms and conditions set out in the Contract and the Technical Specification.

GEN_REQ-003	The Supplier shall be liable for all the aspects of works necessary for installation of the V-MLAT. That shall encompass hardware and software design, site preparation, telecommunication and radio interference analysis, on site planning and implementation of operations.

GEN_REQ-004	The Supplier shall be liable to prepare V-MLAT operation coverage analysis that includes the specified performance characteristics.

GEN_REQ-005	The Supplier shall be the manufacturer or authorised representative of the manufacturer of the offered V-MLAT.

GEN_REQ-006	The V-MLAT shall comply with the latest versions of the documents listed under Item 2.1. of the Technical Specification. In certain cases the requirements established under the Technical Specification might be stricter than those established in the respectful documents. In such cases the requirements established under the Technical Specification shall supersede those established under the respectful documents and shall automatically become minimum requirements.

GEN_REQ-007	The price of the V-MLAT shall include all the software (software licences), necessary to ensure the V-MLAT characteristics and functions indicated under the Technical Specification. The licences shall be perpetual  (granted for unlimited period of time). The number of V-MLAT users and the scope of the processed information cannot be restricted by licences.

GEN_REQ-008	The offered MLAT system must be successfully installed and launched no earlier than five (5) years ago before the date of submission of the initial proposal at least at one member state of the European Union or the European Free Trade Association or in the United Kingdom. Sufficient proof is a declaration of conformity in accordance with the detailed specification established by EASA (DS-GE.CER/DEC). A Certificate of Suitability for Use issued by the CAA or other respected authorities under previous regulations is also acceptable.

GEN_REQ-009	The offered MLAT system must be successfully used in at least one air traffic management service sector in at least one member state of the European Union or the European Free Trade Association or in the United Kingdom. A sufficient proof shall be certificate/ confirmation of the customer, that the offered V-MLAT is successfully used for the provision of air traffic management services.

## Legal acts, specifications and standards

GEN_REQ-010	The performance of the V-MLAT sensor system and its individual components shall confirm or exceed in every aspect, the requirements and recommendations set out in the appropriate sections of the documents referenced in this specification items 2.1.1 and 2.1.2.

GEN_REQ-011	The V-MLAT shall comply with ICAO Annex 10 to the Chicago Convention on aeronautical telecommunications current versions of documents as far as they are relevant to the provision of surveillance services requirements and shall be dedicated to air navigation surveillance service provision (SUR).

GEN_REQ-012	The Supplier must provide proof of radio compatibility for the proposed equipment as required by Lithuanian Law, in accordance with Directive (EU) 2014/30/EU of the European Parliament and of the Council of 26 February 2014 on the harmonisation of the laws of the Member States relating to electromagnetic compatibility .

GEN_REQ-013	The Supplier must comply with the Directive (EU) 2022/2555 of the European Parliament and of the Council of 14 December 2022 on measures for a high common level of cybersecurity across the Union, amending Regulation (EU) No 910/2014 and Directive (EU) 2018/1972, and repealing Directive (EU) 2016/1148 (NIS 2 Directive) (if applicable).

GEN_REQ-014	The Supplier must provide proof of compatibility for the proposed equipment as required by EASA Easy Access Rules for ATM /ANS – Equipment (Regulation (EU) 2023/1769 & Regulation (EU) 2023/1768).

### Specifications

GEN_SPC-001 	[...] TBD

### Standards

GEN_STD-001	[...] TBD

## Logistics

GEN_LOG-001 	At the request of the Supplier the Customer may allow the Supplier to store the hardware for installation of the V-MLAT at the premises of the Customer. Storage of the hardware shall be free of charge.

GEN_LOG-002 	The Customer shall not be liable for the hardware stored at the premises of the Customer, and the entire risk, for example, in case the premises are destroyed due to fire, vandalism or other events during the storage as a result causing damage or loss of the stored hardware, shall be undertaken by the Supplier.

# Coverage and accuracy requirements

COV_AER-001 	The V-MLAT system must cover the territory of Vilnius Aerodrome with the specified accuracy, including the apron, stands, and the maneuvering area, at an altitude of no less than 100 m above the surface.

COV_AER-002 	The V-MLAT system must cover the EYVI approach corridors within the Vilnius CTR at altitudes no lower than 4000 MSL/GND.

COV_AER-003 	The reference point for the V-MLAT system may be changed, but unless otherwise specified, this point is the Aerodrome Reference Point (54 38 13N, 025 17 16E).

## Accuracy in the horizontal plane

COV_AER-004 	The accuracy of horizontal position detection on the manoeuvring area (runway, taxiways, and centrelines in the apron) must not exceed 7,5 meters with 95% confidence and 12 meters with 99% confidence for any target.

COV_AER-005 	The accuracy of horizontal position detection in stands must not exceed an average accuracy of 20 meters, calculated over a 5-second measurement interval for any target.

COV_AER-006 	The accuracy of horizontal position detection for airborne targets at a distance of at least 7 nautical miles from the runway thresholds must not exceed 20 meters with 95% confidence for any targets within the corridor:

- glidepath, from 2° to 4° (±1° from a 3° glidepath; see Figure 1 and Figure 2, vertical profile, in green);

- ±20° from the runway centerline (see Figure 1 and Figure 2, horizontal profile, shown in yellow).

Figure 1. The southern section of the runway

Figure 2. The northern section of the runway

COV_AER-007 	The accuracy of horizontal position detection within the aerodrome area must not exceed 20 meters with 95 percent confidence for any target.

Note:	The precise coordinates of the Vilnius Aerodrome area, the apron, stands, the maneuvering area, and the approach corridors are publicly available in the Aeronautical Information Publication

-Vilnius aerodrome (EYVI AD 2.24 air navigation charts related to the aerodrome);

- Aerodrome layout (EYVI AD 2.24-01-1)

- Aerodrome movement and parking diagrams (EYVI AD 2.24-02-1; EYVI AD 2.24-03-1);

## Periodicity of output

COV_AER-008 	The data output interval must be configurable for any period between 0.5 s and 5.0 s, in increments of 0.5 seconds

COV_AER-009 	V-MLAT must be capable of providing target position readings in periodic mode, i.e., when the average of target positions over a regular time interval is required.

COV_AER-010 	It must be possible to adjust the output periodicity separately for each geographic zone and/or interrogator.

## Probability of Detection

### Probability of Position Detection (PD)

COV_PRB-001 	The position detection probability for Mode S transponders must be greater than 99.9% within a two-second (2 sec.) measurement interval in the maneuvering area.

COV_PRB-002 	The position determination probability for Mode S transponders must be greater than 99.9% over a five-second (5 sec.) measurement interval in stands.

### Probability of Identification (PID)

COV_PRB-003 	The system must provide the correct Mode S address (identity) of the aircraft, with a probability of identification of 99,9% or higher.

COV_PRB-004 	The system must provide the correct 24-bit ICAO code for the aircraft, with a determination probability of at least 99,9%

COV_PRB-005 	The system must generate a correct Mode A code with a detection probability of at least 98%

COV_PRB-006 	The system must return the correct code specified in other Mode S data fields, for which the probability of identification is greater than or equal to 96%.

### Probability of False Identification (PFD)

COV_PRB-007 	The probability of false target detection must be less than or equal to 10-4

### Probability of detecting a false code (PFCD)

COV_PRB-008 	The probability of incorrectly identifying an aircraft’s address must be less than or equal to 10-6 over a 5-second measurement interval per target.

COV_PRB-009 	The probability of a false positive for Mode A code detection must be less than or equal to 0,1%.

COV_PRB-010 	The probability of an incorrect pressure reading must be less than or equal to 0,1%.

### Updating of modified aircraft information

COV_PRB-011 	The system must report changes to the aircraft information specified below once every 3 update intervals, with a probability of at least 95%:

- ACID;

- Mode A code.

COV_PRB-012 	The system must report changes to the aircraft information specified below once per update interval, with a probability of at least 95%:

- Emergency codes;

- SPI.

## Types of Transponders

PRF_REQ-001 	V-MLAT system shall be capable of detecting, identifying and tracking targets equipped with:

- SSR Mode A, C and Mode S ELS and EHS (Mode S level 1…5) transponder compliant with ICAO Annex 10. Volume IV; ICAO Annex 10. Volume III;

1090 MHz Extended Squitter (ADS-B) avionics compliant with RTCA DO-260 and/or DO-260A + DO-260B;

1090 MHz Short (acquisition) Squitter (ACAS/TCAS) avionics;

transponders, generally that are not triggered by interrogation.

PRF_REQ-002	V-MLAT system shall be capable of interrogating in Mode A; Mode C; and Mode S ELS/EHS.

Note:	ELS means ICAO 24-bit Aircraft Address, Aircraft Identity (callsign used in flight), Transponder Capability, Altitude in 25ft intervals (depending on on-board equipment capability), Flight Status, and SI Code Capability.

EHS mean ELS and Selected Altitude, Roll Angle, Track Angle Rate (True Airspeed if Track Angle Rate is not available), True Track Angle, Ground Speed, Magnetic Heading, Indicated Airspeed, Vertical Rate.

## Interrogation

PRF_REQ-003	The interrogation power and repetition rate shall be kept to a minimum, consistent with the operational requirements, in order to minimize the effect of active V-MLAT operation on the 1030/1090 MHz radio frequency environment in compliance with ICAO Annex 10 requirements.

PRF_REQ-004	V-MLAT system shall not set “Mode S Interrogator Identifier (II) or Surveillance Identifier (SI) code lockout” on any target.

PRF_REQ-005	V-MLAT system shall not use Mode S all-call interrogations (UF11).

PRF_REQ-006	V-MLAT system shall not use Mode S inter-mode Mode A/C/S all-call interrogations (also known as Mode A/C/S with long P4).

PRF_REQ-007	V-MLAT system shall be able to interrogate Mode A/C targets using Mode A/C only all call (i.e. Inter-mode with short P4 pulse) and Mode A/C interrogation.

PRF_REQ-008	V-MLAT system shall not use the “Whisper and Shout” interrogations.

PRF_REQ-009	V-MLAT system shall be able to use some adaptive interrogation techniques in order to minimize RF interference.

PRF_REQ-010	V-MLAT system shall interrogate only in Mode A, C and Mode S.

## Replies reception

PRF_REQ-011	The System shall detect and process replies of Mode A (4096 and 64 code formats).

PRF_REQ-012	The System shall detect and process replies of Mode C (all 1278 ICAO defined codes + "0000" + 2 Non-ICAO codes used by encoder manufacturers: -1100 FT = code 0060; -1200 FT = code 0040).

PRF_REQ-013	The System shall detect and process replies of Mode S ELS. All transponders up to and including Mode S Level 5.

PRF_REQ-014	The System shall detect and process replies of Mode S EHS. All transponders up to and including Mode S Level 5.

PRF_REQ-015	The System shall detect and process squitters of 1090ES ADS-B (mainly DF17). All transponders/squitters up to and including Mode S Level 5.

PRF_REQ-016	The System shall detect and process squitters of 1090ES ACAS/TCAS (all TCAS levels).

PRF_REQ-017	The System shall detect and process squitters of Ground Vehicle Transponders (ADS-B; DF18; 1090MHz).

PRF_REQ-018	The System shall detect and process replies with SPI (any mode/ combination).

PRF_REQ-019	The System shall detect and process replies with civil emergency (any mode/combination).

PRF_REQ-020	The System shall be capable of simultaneously detecting and handling correctly no less than two aircraft with identical Aircraft Address separated by more than 10 NM.

PRF_REQ-021	This requirement shall cover not only the group codes and/or conspicuity codes (0000, 1000, 2000, 7000 etc.) but also all discrete codes.

PRF_REQ-022	The System shall be also capable of handling duplicate Emergency codes (multiple simultaneously) and distinguishing such targets from each other.

PRF_REQ-023	The System shall be also capable of handling non-standard or invalid Mode S/ ADS-B aircraft identification (containing leading spaces, spaces between symbols, and/ or punctuation symbols).

PRF_REQ-024	The System shall be capable to report the detection of said deviation, illegal or double use of codes.

## Information derivation by response type

PRF_REQ-025	V-MLAT system shall provide with the following information derived from Mode A, C transponder signals:

- Aircraft horizontal position;

Mode C information;

Mode A code;

Special codes (7700, 7600 and 7500);

SPI;

Geometric height – only for V-MLAT + trackers internal use.

PRF_REQ-026	V-MLAT system shall provide the following information derived from Mode S ELS:

- Aircraft horizontal position;

Barometric altitude;

Aircraft Identification – ACID (agency designator + figures or registration);

Special codes (7700, 7600 and 7500);

SPI;

ACAS active Resolution Advisory (RA; TCAS-RA) if the aircraft is equipped with TCAS II;

Mode S address

Geometric height - only for V-MLAT + Trackers internal use.

PRF_REQ-027	V-MLAT system shall provide with following additional EHS downlink parameters (for accordingly equipped aircrafts) from available BDS registers:

- 1.0;

1.7;

2.0;

3.0;

4.0;

5.0;

6.0;

and other data items may be made available to the transponder.

PRF_REQ-028	The V-MLAT system shall provide with the following information derived from 1090ES ADS-B signals:

- Aircraft identification;

- SPI;

- Emergency indicator;

- Barometric altitude;

- Vertical speed/ rate of change;

- Airborne position (latitude; longitude);

- Mode S address

- Geometric height;

- Quality and Accuracy indicators.

PRF_REQ-029	V-MLAT system shall provide the following information derived from ACAS/TCAS signals:

- Aircraft horizontal position (position update);

- Mode S address;

- Geometric height - only for V-MLAT + Trackers internal use.

PRF_REQ-030	V-MLAT system shall provide with the following information derived from other mode replies:

- Aircraft horizontal position (position update);

- Code of mode (if extracted);

- Special code(s) of emergency;

- Ident/ SPI;

- Geometric height - only for V-MLAT + Trackers internal use.

## Garbling, FRUIT and jamming

PRF_REQ-031	A mechanism shall be provided to detect when garbling (different kinds targets replies overlap, including due to multipath and reflections) and FRUIT (different kind of replies unsynchronized in time or non-surveillance pulse streams (like DME/TACAN signals and their harmonics on reply frequency) exists.

PRF_REQ-032	V-MLAT system shall contain built-in special logic to minimise possible data losses due to garbling and FRUIT.

PRF_REQ-033	V-MLAT system shall contain built-in special logic to minimise possible data losses due to jamming (and jamming-like situations to transponder and squitter faults).

PRF_REQ-034	Such logic shall include all important principles for control and minimization effects of jamming (for example, limitation by sector, limitation by range, limitation by reply signal level etc.).

## Data output management

### Data output modes

PRF_REQ-035	V-MLAT system shall be able to output data operating in one or more of the following output modes:

- Data driven mode;

- Periodic delayed mode;

- Periodic predicted mode.

### Processing delay for data output modes

PRF_REQ-036	In data-driven mode, the processing delay must be less than or equal to half a second (0,5 sec.), measured from the moment the target signal is received by the device until it is transmitted to the system that generates the target reports.

PRF_REQ-037	In periodic delayed mode, when the last received measured position within the output period is transmitted, the maximum processing delay shall be less than or equal to the duration of the output period plus less than or equal to half a second (0,5 sec).

PRF_REQ-038	In periodic predicted mode, when the predicted position at the time of output is transmitted, the maximum processing delay must be less than or equal to half a second (0,5 sec).

PRF_REQ-039	The delay of the communication infrastructure used within the V-MLAT system shall not exceed 100 ms.

### Target reporting requirements

PRF_REQ-040	V-MLAT system shall be able to provide target output as calculated plots without smoothing or extrapolation.

PRF_REQ-041	The coordinate system for the system's output data may be changeable, but unless otherwise specified, the WGS-84 coordinate system is used

PRF_REQ-042	V-MLAT system shall be able to provide target output as plots.

PRF_REQ-043	V-MLAT system shall be able to provide target output as tracks.

### Data output formats

PRF_REQ-044	V-MLAT system shall be capable of providing data output to data users in the following Eurocontrol ASTERIX Categories:

- Target reports (plot/ track) data/ MLT Messages - Asterix Cat 020;

- Service messages (overall system status, subsystem status, V-MLAT reference position)/ Multilateration System Status Messages – Asterix Cat 019;

- ADS-B messages – Asterix Cat 021;

- CNS/ATM Ground Station and Service Status reports – Asterix Cat 023;

- CNS/ATM Ground System Status reports – Asterix Cat 025.

Note:	The requirement is set for the entire system, regardless of whether this Asterix report is received directly from the ground station (CAT021) or calculated by a central processing unit (CAT019 – CAT025).

PRF_REQ-045	Each Asterix Category version shall be downgrade able to the older (including draft) versions/editions.

PRF_REQ-046	Each Asterix Category version shall be easily upgradeable to the newer (draft or final) versions/editions, and this upgradeability shall be foreseen at least until end of the warranty period.

PRF_REQ-047	Separate Asterix outputs shall be capable to output different versions/editions of the same Asterix Category simultaneously (and independently).

PRF_REQ-048	All optional fields shall be in use/on as default. Until SAT Customer will decide which optional fields to leave unused/off.

### Number of outputs

PRF_REQ-049	Each of the output channels shall be configurable to at least 2 simultaneous addresses of destination.

PRF_REQ-050	For each of these output channels at least the following parameters shall be definable by the V-MLAT system administrator:

- Addresses and ports (broadcast-, multicast- or unicast-addresses);

- Transmission protocol (UDP, TCP);

- Status of the output (either active, non-active or test);

- Costumer predefined areas to filter target reports;

- ASTERIX category and version;

- Output period.

## Target capacity

PRF_REQ-051	V-MLAT system shall be able to simultaneously acquire and maintain tracking of 300 targets within its required operational coverage volume.

PRF_REQ-052	An overload mechanism shall be provided to detect when the number of targets exceeds a configurable capacity threshold.

PRF_REQ-053	When the threshold is exceeded the V-MLAT system shall set an appropriate overflow indicator in the system status report (e.g. ASTERIX OVL bit in item I019/550).

# Detection of unmanned aircraft

UAV_REQ-001 	The system must be capable of detecting and reporting unmanned aircraft transmissions within its coverage area at least on the following frequencies:

- 433-435 MHz;

- 863-870 MHz;

- 2400-2483,5 MHz;

- 5725-5875 MHz.

UAV_REQ-002 	The system must be capable of determining the coordinates of the broadcast source and updating them at least every 2 seconds.

UAV_REQ-003 	The system must be capable of decoding unencrypted telemetry data from the broadcast source and may use it to determine the position of the unmanned aircraft.

UAV_REQ-004 	The accuracy of horizontal position detection in the manoeuvring area (runway, taxiways, and centrelines in the apron) must not exceed 20 metres with 95% confidence and 40 metres with 99% confidence for any target.

UAV_REQ-005 	The accuracy of horizontal position detection for airborne targets from the runway threshold up to a distance of 7 nautical miles, in a corridor ranging from 2° to 4° on the glide path and ±20° from the runway centerline, must not exceed 20 meters with 95% confidence for any target (see Figure 1 and Figure 2).

UAV_REQ-006 	The functionality for detecting unmanned aerial vehicles must not in any way impair or limit the System’s performance characteristics or quality requirements.

# V-MLAT architecture

SYS_ARC-001	To the extent that it is practical for the system's operation, efforts should be made to utilize the existing locations of Ground stations.

Note:	Customer shall provide the locations of the existing ground stations.

SYS_ARC-002	The system architecture must be designed to meet the requirements for reliability, maintenance, availability, and redundancy.

SYS_ARC-003	The system must be designed to minimize maintenance, with components arranged logically and easily accessible, and system downtime kept to a minimum.

SYS_ARC-004	For functional unit or individual components that serve for the complete V-MLAT, redundancy shall be achieved by duplicating so that there is a standby component able to take over from the main as and whenever necessary.

SYS_ARC-005	The architecture of the system's ground station layout must be easily adaptable, and the amount of equipment purchased must not be limited in a way that restricts future additions and integration.

SYS_ARC-006	Changes to the station layout architecture may be implemented by the Customer. To that end, the Supplier shall provide training of sufficient detail and grant the necessary level of access to the system configuration options.

SYS_ARC-007	For components that work together for a particular dedicated function, redundancy shall be achieved by providing sufficient components so that the function may continue to be provided after a certain number of the components have failed. When one or more components do not operate as intended or have inferior but tolerable performance, the mode will be considered as a degraded mode.

SYS_ARC-008	The V-MLAT hardware platform shall consist of workstations and server computers running the well-known and widely used operating systems and communicating through a high reliability network.

SYS_ARC-009	The Supplier shall state the calculated MTBF, MTBCF and MTTR of the proposed V-MLAT and components.

SYS_ARC-010	Every V-MLAT component, executing certain operational assignment function shall be duplicated so that the function could be executed with any one of the two components.

SYS_ARC-011	Components, which operate as a group in order to execute operational assignment function, must be sufficient, so that the function would be executed completely in case of failure of any component.

SYS_ARC-012	All hardware, software and firmware that is to be utilized shall be totally proven in an operational environment that has similar, if not more stringent requirements than those stated within this specification.

SYS_ARC-013	In particular, the design of the V-MLAT must be such that failure of components shall never cause a complete outage of the total V-MLAT. Only gradual degradation of performance is allowed.

SYS_ARC-014	Any data processor in the proposed configuration shall have sufficient spare capacity for subsequent extensions or modifications, i.e., spare memory capacity, spare I/O facilities and reserve in processing time (duty cycle).

SYS_ARC-015	This spare capacity shall be in the order of 40%.

SYS_ARC-016	The V-MLAT shall have a kind of control and monitoring system (CMS) position for technical personnel which allows operators to:

- View the functional status of the V-MLAT elements using colour marking;

- Control the V-MLAT;

- Collect and keep information about V-MLAT operation, generated alerts and warnings.

- View recorded reasonable period of V-MLAT data (not less than thirty (30) days) for the purpose of fault and malfunction investigation – Surveillance service provision data (Plots, Tracks and other), valid active configuration, actions and changes and any relevant operational information.

# System structure

SYS_STR-001	The V-MLAT system shall have appropriate means and necessary number of components to support its service provision and live monitoring for:

- Central processing system (CPS);

- Ground stations (GS);

- Control and monitoring System (CMS);

- Data transmission infrastructure (DTI);

Test and evaluation system (TES);

Energy supply infrastructure (ESI);

Environmental condition support equipment (ESCE).

Figure 3. Basic system structure and interfaces

## Software and hardware design

SYS_STR-002	The System software and hardware configuration shall be designed to be modular, fully scalable and configurable.

SYS_STR-003	The System hardware components, networks and operating system software shall use COTS technologies that have been proven in operational service.

SYS_STR-004	The software architecture shall operate in a manner that all sub-systems are autonomous, and that a failure in one sub-system shall not migrate to others via WAN/LAN.

SYS_STR-005	The System shall also be liable to cope with:

- Logical and user-friendly architecture;

- Use of latest technology and software solutions;

- Possibility for later upgrades and modifications;

- Follow ICAO, EUROCAE and EUROCONTROL standards and recommendations (even if not directly referred to or listed in this document).

SYS_STR-006	The System Supplier and any of the Sub-suppliers shall have access to all necessary tools for development and design.

## Central processing system (CPS)

SYS_STR-007	The V-MLAT system shall be designed in such way to minimize RF spectrum pollution, to reduce Interrogation to the minimum. The System supplier shall as part of its offer provide software tool which will monitor, analyse and log transponder load generated by any Surveillance system. System interrogation load shall be kept below 2% for any individual transponder.

## Ground station (GS)

SYS_STR-008	The ground station shall be housed in self-contained units, which can be installed at indoor or outdoor locations (in case of outdoor unit).

SYS_STR-009	The ground station self-contained units shall be installed at indoor or outdoor locations in an easily accessible convenient places whether it be the roof of a building or the site of an antenna tower.

SYS_STR-010	The layout of ground stations must be within the aerodrome boundaries. To improve the system's accuracy (which is particularly important outside the aerodrome boundaries), omnidirectional and directional antennas may be used.

Note: 	The ground station mounting box installation at height on the mast shall not considered a convenient location.

SYS_STR-011	Each ground station shall be equipped with at least one power socket (230V, 1 ph, N, PE; 50 Hz).

SYS_STR-012	The ground station shall have adequate protection against over-voltage, over- current, spikes, sags and other undesirable power supply forms as per national or international equivalent standard.

SYS_STR-013	The ground station shall have adequate protection against direct lightning strikes, induced surges, flashovers as per national or international equivalent standard.

SYS_STR-014	Component elements must only be installed in a lightning-protected ground station cabinet, unless those elements are equipped with additional adequate lightning protection.

SYS_STR-015	Each ground station (Tx/Rx site) shall be provided with a back-up power supply system such as a UPS system.

Note: 	The Supplier is responsible for installing the UPS system at the ground stations. The Customer is responsible for providing an uninterruptible power supply to the central processor.

SYS_STR-016	All ground station sites shall have guaranteed power supply using back-up power supply with UPS assistance for not less than four (4) hours after the main power outage.

SYS_STR-017	The Supplier shall, as part of the proposal, deliver the following information on the indoor and outdoor Remote Units:

- Dimension;

- Weight;

- Power consumption;

- Heat dissipation.

## Control and monitoring system (CMS)

SYS_STR-018	The Control and Monitoring System shall consist of at least these components:

- Remote Control and Monitoring System (RCMS);

- Local Control and Monitoring System (LCMS);

- Data Recording and Playback System (RECS).

SYS_STR-019	CMS shall be designed in such a way that when more than one RCMS is installed within the System all RCMS's can monitor the System.

SYS_STR-020	CMS shall be designed in such a way that when more than one RCMS is installed within the System that only one RCMS can control the System at any given time.

SYS_STR-021	CMS shall be designed in such a way that when more than one RCMS is installed within the System every RCMS show which RCMS is controlling the System.

SYS_STR-022	CMS shall be designed in such a way that when more than one RCMS is installed within the System after the required level authorization it is possible to take over the control function from any RCMS.

### Remote control and monitoring system (RCMS);

SYS_STR-023	The RCMS function including but not limited to shall provide the following sub-functions:

- System units status monitoring and control;

- System performance statistics and configuration;

- Target report quality evaluation;

- Display.

SYS_STR-024	User access to the system monitoring and control function shall be protected by password providing the following different authorization levels:

- Operator (lowest level; access to system units status monitoring functions);

- Maintenance engineer (medium level; access to system units status monitoring and control functions);

- System administrator (highest level; access to configuration functions and system units status monitoring and control functions and users management).

SYS_STR-025	RCMS shall provide security functions to prevent unauthorized access to the system (for example, password, encryption, access logging).

SYS_STR-026	RCMS shall provide at least the following real time system performance statistics:

- ground station synchronization drift and offset per site;

- ground network link usage per site;

- total number of targets, broken down by response type (Mode S, ADS-B, etc.) per system.

SYS_STR-027	RCMS shall include the collection, evaluation, graphical presentation and recording of at least the following status items and status reports:

- V-MLAT system units status and failure reports;

- Status of all processes performed within the different V-MLAT system units;

- V-MLAT system units users access and actions history.

SYS_STR-028	Each of the status items and status reports of the V-MLAT units shall be marked with at least the following identification data:

- Source identifier;

- Time stamp;

- Criticality.

SYS_STR-029	The RCMS configuration function shall provide the possibility to change at least the following system configuration items:

- Number of sensor units;

- Number of V-MLAT time synchronization units (if required);

- Mode of operation of V-MLAT system units / system and unit channels (Operational Mode / Maintenance Mode);

- Test transponders, if any.

SYS_STR-030	The RCMS shall provide the ability to remotely update software for a selected amount or all ground stations at once.

### Local control and monitoring system (LCMS)

SYS_STR-031	The software for the Local Control and Monitoring System shall be installed on the provided laptop.

SYS_STR-032	The LCMS shall be able to support all maintenance activities necessary to perform on each ground station and central processing system, these activities consisting of fault detection to Line Replaceable Unit (LRU) level, configuration and access to error logs.

SYS_STR-033	LCMS shall be able to support all configuration capabilities of the System.

SYS_STR-034	LCMS shall have a technical situation display to display in real time the available surveillance data and traffic situation.

SYS_STR-035	LCMS shall provide the ability to update software on the ground stations and central processing system locally.

### Data Recording and Playback System (RECS)

SYS_STR-036	The System shall record and replay at least the following data:

- Plot reports;

- Track reports;

- Target information after the several internal steps of data processing (including quality information, if available);

- Interrogation activity including which interrogator transmitted at what power to which target with which interrogation format and an interrogation success statistics correlating interrogations sent with successfully received replies.

SYS_STR-037	The record and replay functionality can be a separate subsystem, but shall be delivered with the WAM system acquisition.

SYS_STR-038	The data recording and playback function may be integrated into the RCMS system, but this integration must not in any way interfere with the core functions of the RCMS, and the operation of the RECS system must not be restricted by the functions of the RCMS.

SYS_STR-039	RECS must be capable of recording a period of no less than 60 calendar days and be able replay all output formats with a single tool into the same file so that the context is preserved.

SYS_STR-040	The System shall provide the functionality to export and import recorded data.

SYS_STR-041	The System shall provide the functionality to filter the data on target specific data items.

SYS_STR-042	The System shall provide the functionality to have a listing of all data in a readable format.

SYS_STR-043	The System shall provide the functionality to have a screen shot of the displayed data.

SYS_STR-044	The Supplier shall describe, as part of the proposal, the human machine interface as realized for the data recording and replay function.

SYS_STR-045	The System shall maintain a time stamped record of the operating status, states and modes of the equipment.

SYS_STR-046	The System shall be able to log any user interaction on configuration including:

- Access attempts;

- Control actions;

- Configuration changes.

SYS_STR-047	The system shall present operating status, states and modes of the equipment changes in a user - friendly graphical format.

SYS_STR-048	At least the following data items, describing the quality of the target reports, shall be calculated and included within the target reports:

- Number and identification of sensor units used for target position calculation;

- Horizontal position accuracy or dilution of precision;

- Vertical position accuracy or dilution of precision;

- Interrogation type, used interrogator, power, success rate of interrogations per target;

- Probability of detection per target.

Note: 	Accuracy must be given in the form of standard deviation and covariance or likely dilution of precision.

SYS_STR-049	The Supplier shall, as part of the proposal, describe which additional items describing the quality of the reported target data are calculated and included within target position reports and target track reports.

SYS_STR-050	The RECS System shall have a Technical Situation Display to display in real time the V-MLAT and ADS-B traffic situation based on ASTERIX CAT020 (V-MLAT), and CAT021 (ADS-B) target reports.

SYS_STR-051	The RECS shall be able to use all of the fields within the ASTERIX CAT019 and CAT023 or CAT025 and display them to the user.

SYS_STR-052	The RECS shall provide the functionality to display target reports under laid by area maps, system configuration maps and filtering maps.

SYS_STR-053	The RECS shall provide the functionality to display all data items of an individual target report by selecting the corresponding target on the display screen.

SYS_STR-054	The RECS shall provide the functionality to display target report quality evaluation data with all data items of an individual target report.

SYS_STR-055	The RECS shall provide the functionality to isolate and display only the data provided by a selectable output channel of the V-MLAT system.

SYS_STR-056	The RECS shall provide the functionality to define geographical maps to filter target reports (defining 3-dimensional volume).

SYS_STR-057	The RECS shall provide the functionality to define target filters depending on available target information.

SYS_STR-058	The display function shall provide a target search function.

SYS_STR-059	The display function shall provide panning and zooming techniques to allow any part of the coverage volume to be selected for display.

## Energy supply infrastructure (ESI)

SYS_STR-060	The equipment shall be designed for connection to AC main, and

SYS_STR-061	it shall be designed for and, without showing any malfunction, resist the following tolerances on the AC power supply:

- Mains voltage:	230V ± 10 %, 1 ph, N, PE (or 400V ± 10 %, 3 ph, N, PE);

- Frequency:	50 Hz ± 6 %;

- Frequency change:	≤ 0.25 Hz/s;

- Harmonic content:	≤ 6%

- Power factor (for equipment):	≥ 0.8

- Interruption of mains voltage:	≤10 ms (the equipment shall handle).

SYS_STR-062	The AC powered equipment shall function and maintain stated performance at interruption and simultaneous variation of mains voltage and frequency as above.

SYS_STR-063	Power supply status and measurements shall be displayed in CMS and shall be updated at the appropriate frequency.

SYS_STR-064	The safety conductor and the neutral in the distribution shall not be connected together (TN-S System).

SYS_STR-065	Without being permanently damaged the equipment shall be able to withstand voltage transients of max 2000 Volt with duration of max 1.0 ms.

SYS_STR-066	To protect the connected equipment from any damage due to lightning the incoming mains cables shall be equipped with surge diverters (non-linear resistance arresters).

SYS_STR-067	Equipment shall be connected to a one phase system,

SYS_STR-068	i and shall be provided by three terminals for connection of the Live (L), the Neutral (N) and Protective-Earth (PE) conductors.

SYS_STR-069	A cabinet ground conductor shall be connected to a cabinet ground terminal,

SYS_STR-070	which shall be connected to all accessible metal parts as front panels etc. (bonding), but isolated from all equipment circuitry.

SYS_STR-071	A signal ground conductor, common to all equipment circuitry but isolated from all cabinet grounds, shall be incorporated.

SYS_STR-072	The Supplier shall state the recommended maximum earth resistance.

SYS_STR-073	All fuses shall be easily accessible, and shall not be located inside the equipment units.

## Environmental condition support equipment (ESCE)

SYS_STR-074	The system's ground stations must be cooled exclusively using passive cooling elements, without the use of active rotating components.

SYS_STR-075	The system's ground stations must not have any active heating elements and must be designed so that the temperature generated by the technological processes is sufficient.

SYS_STR-076	The equipment shall not cause disturbing air movements.

SYS_STR-077	Ground stations must monitor internal temperature, power supply status, and door-opening conditions. Among other things, this information must be recorded and made available at the RCMS workstation.

## Test and evaluation system (TES)

SYS_STR-078	The Supplier shall provide the Test and Evaluation System (TES) for V-MLAT.

SYS_STR-079	TES purpose is V-MLAT System testing, validation and verification of any change of the operational system, training, replaying recorded data for analysis, etc.

SYS_STR-080	The TES can be implemented by introducing the ability to set the configuration under test to maintenance mode on one of the channels; however, a quick return (no longer than 3 minutes) to a properly functioning configuration must be ensured. Other testing solutions may be proposed.

SYS_STR-081	TES shall provide the same functions as the CPS.

SYS_STR-082	TES shall allow system testing and evaluation without influencing the operational system.

SYS_STR-083	TES shall be able to be installed remote and independent from (but parallel to) the CPS.

Note:	it is understood that TES will be connected to the same network environment as the CPS but, unless otherwise specified, will not perform active system management functions such as interrogation, calibration and etc. Note does not apply when running in backup mode.

SYS_STR-084	It shall be possible to select any subset of V-MLAT system units, including units in operational and in maintenance mode, to be used by the test system.

SYS_STR-085	TES shall be fully capable as a backup of the CPS from different geographical location.

### Modification of the stations deployment architecture to accommodate the changed environment

SYS_STR-086	The V-MLAT System is supplied with software for simulating changes to the layout of the System’s Ground stations deployment architecture, which takes into account reflections, secondary paths, coverage, and other parameters critical to designing high-quality performance.

Note:	The purpose of this requirement is to ensure uninterrupted system operation throughout the entire service life, even when structures within the system’s operational area are being modified, large-scale temporary construction work is being carried out, or other dynamic and static impacts typical of airport operations occur.

SYS_STR-087	The simulation system can be integrated into the V-MLAT system or offered as a standalone platform for this purpose.

SYS_STR-088	The modeling system must have capabilities that are no less comprehensive than those used by the Supplier in designing the V-MLAT system layout.

SYS_STR-089	Training on how to use the simulation system is included in the core training curriculum.

## Data transmission infrastructure (DTI)

SYS_STR-090	The connection between the ground stations and the Central Processing System is provided via a high-quality optical cable network owned by the Customer. If it becomes necessary to lay a new communication line to the site of a new ground station within the airfield, the Customer shall perform all design and construction work.

SYS_STR-091	The Supplier shall be responsible for the System’s data transmission interfaces (servers, switches, routers, firewalls, etc.). The Customer shall be responsible for the communication links between the System and the Data users of the surveillance data.

SYS_STR-092	The Supplier shall provide Customer with all related requirements and parameters for such communication channels.

SYS_STR-093	Communication between ground stations and central processing system shall be over IP network (WAN/LAN).

## BITE

SYS_STR-094	V-MLAT system shall include a Built in Test Equipment (BITE) such that there is a continuous monitoring of the operating status of the equipment.

SYS_STR-095	It shall be possible to operate BITE in both Operational and Maintenance modes.

SYS_STR-096	BITE coverage rate shall be at least 90 % (i.e. failures capable of degrading performance below minimum requirements shall be detected in 90 % of cases.

SYS_STR-097	BITE fault finding rate shall be at least 95 % (i.e. 95 % of all detected failures shall be isolated to within a three LRU group).

SYS_STR-098	V-MLAT system BITE shall be able to register a fault condition without the presence of an external control and monitoring system.

Note:	The ability to register a fault means that BITE information is stored within the V-MLAT system such that an RCMS may identify that a fault has occurred even if the occurrence was prior to the RCMS being connected.

SYS_STR-099	The BITE shall be designed for rapid assessment of test results and isolation of faults down to LRU or component level.

SYS_STR-100	BITE shall not cause System failures or false alarms.

SYS_STR-101	and shall be extensively self-tested to ensure the validity of the BITE output.

SYS_STR-102	There shall be a possibility to bypass the BITE function.

Note:	When BITE bypass activated, the possibility of checking the System with deviations from the limit values is maintained. The System does not perform automatic corrections, such as automatic switching of operating components, channel change, automatic disconnection of the service and other similar actions.

SYS_STR-103	Most of the key parameters controlled by BITE shall be able to be tested by independent measuring devices on-site.

SYS_STR-104	A local summary signal for pass/fail shall be indicated, for instance by a LED on the front edge of each major unit or PCB.

## Time synchronization

SYS_STR-105	The system must use at least two available GNSS signals in order to provide a common reference time.

Note:	Under GNSS signal in this document must also understood reliable and usable signals to assure any navigation and timing satellite systems like Galileo, GLONASS, EGNOS, WAAS etc.

SYS_STR-106	If the GNSS signal is lost and malfunctions occur, the System must continue to provide surveillance data for a minimum of 24 hours.

SYS_STR-107	The Supplier shall select the method of system time synchronization at its discretion, taking into account the system’s architectural layout.

# System operation mode

SYS_STR-108	V-MLAT system shall have two modes, namely:

- “Operational”: the system is available for operation;

“Maintenance”: components or the system itself is under maintenance and is not available for operation.

SYS_STR-109	The system must be fully operational within 3 minutes of startup or restart, following a power failure to the CPS and at all remote locations after a proper shutdown.

SYS_STR-110	The time required for the system to switch from the primary to the backup CPS must be less than 2 seconds, without losing any critical data.

## Operational mode

SYS_STR-111	Within the operational mode the V-MLAT system shall provide all system functions to meet the required system performance as detailed in this specification.

SYS_STR-112	It shall be possible to switch any unit of the V-MLAT system from the operational mode to the maintenance mode by local and by remote connection means.

SYS_STR-113	Within the operational mode V-MLAT ground station units set to maintenance shall not be used for target data processing in CPS.

## Maintenance mode

SYS_STR-114	Within the maintenance mode the V-MLAT system shall provide all system functions as available within the operational mode.

SYS_STR-115	The V-MLAT system shall transmit plot, track and status reports, only if required by the users.

SYS_STR-116	Within the maintenance mode the central processing unit in operation shall not be influenced by the central processing unit in operational mode.

## Degraded operating mode

SYS_STR-117	The System shall include a tool, which will continuously monitor status of different ground station units, and in case of predefined combinations of failures automatically announce unavailability of affected service

SYS_STR-118	In case of two receiver units are out of function (N-2 topology) or out of System parameters, WAM system shall continue to operate with the same performance as during normal operation.

SYS_STR-119	If more than two ground station units are inoperative or do not meet the System parameters (N-3 or more), the System must report a System failure only if the minimum required System accuracy is compromised.

SYS_STR-120	The V-MLAT system shall have the ability to provide continuous validation of data and timely alerts to the operator when the System cannot be safely used in operation as intended.

SYS_STR-121	The V-MLAT system shall include solutions (including, but not limited to, for example, comparing it with the information contained in the ADS-B signal) that will allow evaluation of performance and integrity monitoring for end-to-end testing of the System.

SYS_STR-122	When using reference transponders, it is mandatory to provide the ability to verify the query (pulse and code format, power, frequency, and geodetic position), as well as the parameters of the response (type, receiver sensitivity), so that any degradation in the accuracy of the transmitter or receiver subsystems and position measurement can be automatically determined.

SYS_STR-123	Reference transponders must ensure that the system integrity check is performed even when there are no valid Mode 3A/C, S, or ADS-B targets. This information must be continuously broadcast in the output formats.

# CYBER SECURITY REQUIREMENTS

## Users authorization

USE_AUT-001 	V-MLAT shall allow defining individual user roles for groups of user with different individual access rights.

USE_AUT-002 	V-MLAT shall be able to prevent concurrent access by multiple maintenance users to the same V-MLAT element.

USE_AUT-003 	V-MLAT shall prevent ground control or configuration access to any element by more than one authorized user at a time (subject of authorization level). The access mechanism shall be centrally managed.

USE_AUT-004 	V-MLAT shall notify the CMS when an authorized or unauthorized connection to an element is established.

# ENVIRONMENT CONDITIONS

## General information

ENV_CON-001	The equipment shall withstand variations and changes in the environment in which it will be operated.

ENV_CON-002	Operating conditions for the equipment shall be such that power will be on 24 hours per day.

ENV_CON-003	The equipment shall be intended for installation co-located with other electrical and electronic equipment. V-MLAT stations, VHF radio transmitters/receivers, and radio base stations (Digital Cellular Phones/Data, WMAN, WWAN, WRAN, WLAN, Mobile Broadcast etc.) are likely to be operating in proximity to the equipment.

ENV_CON-004	Serious attentions shall be paid to ensure the electromagnetic compatibility with the surrounding equipment.

ENV_CON-005	The Supplier shall describe how electromagnetic compatibility will be assured to allow for co-existence with the above mentioned telecommunication systems.

## Environmental resistance and testing methods

ENV_CON-006	The equipment and parts thereof shall, if not otherwise stated, have an environmental resistance as specified below without showing any damage, malfunction or loss of performance.

ENV_CON-007	V-MLAT equipment shall operate, under specified weather conditions including fog, hail, snow, icing wind and rainfall,

ENV_CON-008	and the operational performance defined in this document shall be still met during such weather conditions.

ENV_CON-009	The units and components that will be located outdoor shall be weatherproof.

ENV_CON-010	V-MLAT shall be lightning protected in order to continue operations during lightning storms without equipment failure.

## Ambient environment

ENV_CON-011	Meteorological statistics indicate the dimensioning outdoor temperature range shall be from -35°C to +40°C.

ENV_CON-012	The antennas shall withstand a wind velocity of at least 150 km/h.

ENV_CON-013	The antennas shall be capable to operate within the temperature range -30°C to +60°C and tolerate relative humidity up to 95%.

ENV_CON-014	Equipment installed at indoor locations shall minimum be capable of use at temperature controlled locations.

ENV_CON-015	The Supplier shall specify heat dissipation in terms of electrical power from all individual equipment. Racks, operator’s work stations, free-standing terminals, printers, etc. are specified or defined as individual equipment.

## Environmental tests

ENV_CON-016	The following requirements shall be verified by environmental testing:

- Temperature and humidity;

- Electric and electromagnetic compatibility;

- Radiation and fields from display;

- When using non-OEM components, these must have quality certificates for electric and electromagnetic radiation compatibility testing.

ENV_CON-017	In case the equipment already has been environmentally tested, the Supplier shall state this and also specify what environmental tests it has been subjected to.

# INSTALLATION CONDITIONS

## General information

INS_CON-001	The Supplier shall be responsible for installation of all equipment included in the delivery.

INS_CON-002	The Supplier shall select a special contact individual to be responsible for the installation and also to be available on the installation site throughout the installation period.

INS_CON-003	The contact shall regularly report to the representative(s) of the Customer in the form of, for instance, planned meetings.

INS_CON-004	The contact shall keep a diary, where daily events, detected deviations and any corrective action taken shall be noted.

INS_CON-005	In every respect the installation shall be accomplished in accordance with the regulations of Lithuanian and EU standards.

INS_CON-006	Electrical installations shall be performed by an installation engineer certified in accordance with Lithuanian and EU standards.

## Scope

INS_CON-007	The Supplier shall carry the total responsibility for all installation and commissioning activities.

INS_CON-008	The commitment shall include responsibility for the following activities (including dismantling of existing equipment):

Planning, including:

- interior planning;

- dismantling existing and new installation planning;

- time planning;

- resource planning.

Equipment delivery besides the actual V-MLAT equipment:

- delivery of required special needs equipment (storage boxes, interior studs, computer desks, etc.);

- delivery of required installation material (fasteners and attachments, connectors, cables, etc.).

Installation:

- dismantling of the equipment installed. The Customer shall be responsible for dismantled equipment removal from the site.

New V-MLAT transport, placement and fastening of any special interior equipment and fittings;

transport, placement, fastening, power connections and communications of all equipment included, also those from Sub-supplier;

supplementation of cables, trunking and conduit.

Commissioning:

- preparation of commissioning procedures;

- power-up of installed equipment;

- connection of installed equipment to data sources and data receivers;

- launch and adjustment of installed equipment;

- functional and performance checking of equipment and V-MLAT entities (as part of site acceptance test).

## Installation requirements

### Routing of cables

INS_CON-009	The fact that there are cables of different interference classes shall be taken into account during the installation. Three main classes can be specified:

- interference-emitting cables, mainly cables for electrical power but also coaxial cables for transmitter antennas;

- cables not emitting interference or not sensitive to interference, including certain telecommunication cables, ground cables, fiber optics devices and coaxial cables;

- interference-sensitive cables, including cables which do not carry interference signals but which are sensitive to interference, such as radio signal cables and certain control and monitoring cables.

INS_CON-010	The installation plans shall indicate routings and classifications.

INS_CON-011	Cables of different classes shall, when possible, be run separately and have different routings in separated trunking ducts and through different holes in walls and other inlets.

INS_CON-012	Where there are crossing cables of the same class, and above all those of different classes, provisions shall be made to ensure cable inter-distance as large as possible and a crossing as close to right angles as possible.

INS_CON-013	When cables have to be run close to each other, ground shielding plates well extending above the cable bundle shall separate cables of different classes.

INS_CON-014	Cables that run along a joint cable route and belong to the same interference class shall be collected in bundles.

INS_CON-015	Cables on cable shelves shall be bundled in such a way that the bundle is easy to open to accommodate supplementary cables.

### Fire seals

INS_CON-016	After performing the installation activities, the Supplier shall carry out sealing of cable ducts at floor vaults and wall through using validated and certified materials, craftsmen and methods.

### Marking

INS_CON-017	Marking shall be applied to these types of objects:

- cables for electrical power, telecommunication and ground systems;

- cabinets and racks;

- units in cabinets and racks;

- trunking.

INS_CON-018	All marking of the installations at different sites shall be uniform.

INS_CON-019	The Supplier shall propose the marking V-MLAT intended for use.

INS_CON-020	The marking shall include:

- labels on the equipment units, stating the designation;

- cable number and location marking of the cable;

- colour marking of the cable shelves or cable ducts that is used to separate trunking for power and telecommunication;

- marking of electrical cable conductors, etc.

INS_CON-021	All marking shall be made of durable material, with durable text and reliable fastening.

INS_CON-022	Marking shall be visibly placed and flexible.

INS_CON-023	Marking and documentation shall correspond.

### Physical protection

INS_CON-024	Ground station protection shall be guaranteed by a metal box compartment of the required resistance, protected from vandalism.

INS_CON-025	The core of the box lock shall be of European or Scandinavian type.

# RELIABILITY

The reliability is a probability that a V-MLAT or service under stated conditions for a specified amount of time throughout a specified geographic volume will perform its intended function without degradation, error or failure to the service. Important generic refinements are "availability" and "maintainability".

Availability is further refined in continuity and availability of the data items and of the complete surveillance V-MLAT. It is quantified as the proportion of time the V-MLAT is actually available to the time the V-MLAT is planned to be available. Periods of planned maintenance are discounted from availability figures.

The continuity is the probability that a V-MLAT will perform its required function without unscheduled interruption, assuming that the V-MLAT is available at the initiation of the intended operation.

Maintainability is defined as the ability of a V-MLAT to be retained in or restored to a specified condition when maintenance is performed by personnel having specified skill levels, using prescribed procedures and resources, at each prescribed level of maintenance and repair.

RAM_GEN-001	Reliability predictions for single channel MTBF and MTBCF (channel MTBF and V-MLAT MTBF - MTBCF) shall be provided for the following components:

- Linear replaceable units;

- Each major equipment group;

- Each single channel of the V-MLAT;

- The V-MLAT.

Note:	MTBF and MTTR hours shall be clearly shown in either the block diagram up to component function level or in a list showing the equipment breakdown to functional unit level, with identification of specific common failure mode (for example, switch over equipment).

## Availability

Depending on the circumstances, the provision of a surveillance data may or may not be affected, but technical measures should be employed to ensure graceful degradation of the V-MLAT from full to degraded performance. The availability figures given below exclude all scheduled maintenance, all scheduled non-operational hours and force majeure (for example, lightning strike, exceptional winds, fire) events.

RAM_GEN-002	V-MLAT availability, excluding agreed maintenance periods, shall not be less than 99.999%.

Availability = MTBCF/ (MTBCF + MTTR)

Note:	For a surveillance V-MLAT the MTBCF (critical failure) to be considered for when coherent and full surveillance data is no longer provided from all aircraft during at least 10 seconds at the output of the surveillance V-MLAT.

### Continuity

RAM_GEN-003	V-MLAT probability of critical failure shall be equal to or less than 2,5∙10-5 per hour of operation.

Continuity = 1/ MTBCF

Note:	Continuity is fault probability expressed per unit time. The continuity can be related to the Mean Time Between Critical Failures (MTBCF).

### Data item availability

RAM_GEN-004 	Excluding downtime, but including degraded mode of operation, the V-MLAT shall have the availability of 0.99999 (99.999%) for the following data:

- Aircraft horizontal position and history;

- Aircraft address;

- Aircraft vertical position;

- SPI.

Data items availability = (MTBF / (MTBF + MTTR))

## Maintainability

The level of V-MLAT maintainability is based on the premise of a highly reliable hardware design with a sufficient amount of redundancy at the V-MLAT level. The level of maintainability is determined by taking into account the probability that the personnel maintaining the V-MLAT will have the means to maintain the V-MLAT throughout its lifetime. The fulfilment of the requirements presented in this Chapter allows to judge whether the maintainability of the V-MLAT is likely.

RAM_MAI-001	Therefore necessary design features to keep maintenance requirements to a minimum shall be incorporated. Maintenance activities are classified into three items:

- V-MLAT management;

- Preventive maintenance;

- Corrective maintenance.

### Measurement, testing equipment and devices

RAM_MAI-002	All test and fault location facilities that are built in and form an integral part of the equipment shall be listed and described.

RAM_MAI-003	All standard and special instruments and test equipment which are needed for fault location, testing, maintenance and major overhaul, but are not an integral part of the equipment, shall be specified to its use and necessity and proposed by the Supplier.

RAM_MAI-004	The Supplier shall list the special test equipment and quote price for each item.

RAM_MAI-005	The Supplier shall list and price all general purpose instruments and tools which are available as off the shelf items and are considered necessary for the maintenance of the V-MLAT.

### V-MLAT management

RAM_MAI-006	Complete V-MLAT status information shall be protected against deletion, correction, modification and shall always be accessible to operators and maintenance personnel, remote and local (CMS).

RAM_MAI-007	The V-MLAT shall perform at least:

- Start-up of the V-MLAT;

- Restart of the V-MLAT;

- Switch-off the V-MLAT;

- Error handling, logging;

- Configuration/ Reconfiguration of the V-MLAT;

- Statistics.

Note: 	These requirements are related to the functionality of the V-MLAT management functions. The intention behind the listed functions is to make sure that these functions/ data are accessible to the technical maintenance personnel.

### Preventive maintenance

RAM_MAI-008	Preventive maintenance shall be carried out applying a method based on V-MLAT performance, i.e., preventive maintenance schedules shall be regularly reviewed and adjusted to reflect operational and technical requirements for optimum efficiency.

RAM_MAI-009	 The preventive maintenance schedule shall include:

- Parameter control regularity;

- Preventive part or component replacement regularity;

- Software version relevant wording preventive update, including also the software of third parties.

RAM_MAI-010	Parameter control regular testing must be executed using:

- Automatic V-MLAT software and firmware means, the operation period of which is provided in V-MLAT documentation (V-MLAT variables, parameters, registers, data fields and etc.);

- additional V-MLAT internal control devices (BITE, monitors and etc.);

- independent measurement devices.

### Corrective maintenance

This Corrective Maintenance activity includes fault determination, V-MLAT diagnostics, adjustments and replacement or repair of modules and units. This activity also includes software maintenance, software-based diagnostics, adjustments and recovery.

RAM_MAI-011	MTTR of V-MLAT shall be less than one (1) hour for any single repair action including the time necessary for fault localizing, repair, testing and resumption of normal operation (excluding travel time).

RAM_MAI-012	The maximum MTTR of the V-MLAT shall not exceed 8 (eight) hours for 95% of all repairs.

# V-MLAT OPERATION AND DURATION

WAR_GEN-001	The Supplier shall be liable to guarantee V-MLAT operation duration, which cannot be shorter than fifteen (15) years from the day of signing the final note of transfer and acceptance of the V-MLAT. During this period, the Supplier must ensure the ability to repair existing equipment and it’s components, order spare parts, supply software updates.

WAR_GEN-002	The Supplier must provide the Purchaser with updates to the equipment (software and hardware) without delay during the term of the Contract if it is aware of any cases where the same equipment and/or equipment built on the same base has been updated due to failure for other customers. The update must be accompanied by detailed information on the nature of the failure.

WAR_GEN-003	All V-MLAT equipment must be newly manufactured and cannot be refurbished or discontinued. When submitting a proposal, the Supplier shall ensure that the manufacturer of the equipment, including third-party equipment, has not declared the equipment End Of Life (EOL) or End Of Sales (EOS). The Supplier shall inform the Customer in advance when the Equipment is approaching the End of Life (EOL).

## Warranty and liabilities during the warranty period

WAR_GEN-004	V-MLAT and the provided installation services shall be granted warranty period established under the Tender of the Supplier, but not less than 36 months, which shall be calculated from the day of signing the final note on transfer and acceptance of the V-MLAT.

WAR_GEN-005	During the warranty period V-MLAT hardware and software faults shall be eliminated free of charge, broken V-MLAT components (including their transportation costs) shall be repaired or replaced free of charge, except for the cases, when V-MLAT faults can be eliminated and/ or broken V-MLAT components can be replaced (with new components provided by the Supplier) by the technical personnel of the Customer trained by the Supplier. During the envisaged trainings the Supplier shall be liable to train the employees of the Customer, so that they would be able to implement V-MLAT maintenance and repair. The concept “fault” comprises any improper operation and/ or disruption of the V-MLAT (or any part thereof). The faults, in accordance with their impact upon the V-MLAT operation, shall be classified into the following groups:

- Critical faults – when V-MLAT operation has failed in its entirety or in individual sectors and the failure results in the imposition of air traffic restrictions;

- Non-critical faults – when the V-MLAT operates in deteriorated mode, in case of other faults (that do not comply with the critical fault criteria).

WAR_GEN-006	The decision on the type of the disruption (critical, non-critical), shall be made by responsible persons appointed by the Customer.

WAR_GEN-007	Warranty maintenance shall encompass the following:

V-MLAT fault elimination and restoration of operation;

V-MLAT software, including software and firmware, operating systems and software of third parties, licences and updates;

Consulting on V-MLAT operation, usage, equipment maintenance, fault identification and elimination matters (on working days, by telephone, e-mail or other electronic means);

V-MLAT technical documentation updates.

Note: 	In order to enable the Supplier to analyse the faults, disruptions and claims regarding the warranty, the Supplier will be provided all the necessary information and material about malfunction or disruption.

WAR_GEN-008	V-MLAT warranty maintenance services shall be subject to the same requirements and terms as V-MLAT maintenance services (indicated under Item [TBD] of the Technical Specification), taking into consideration the level of criticality of the fault unless other fault repair term is agreed upon with the Customer. In case during the warranty period parts/ components are used for eliminating V-MLAT faults from the set of spare parts possessed by the Customer, the Supplier shall be liable to return these parts/ components to the Customer not later than within 1 month from the actual usage of the parts/ components or from the day of reception of the information about the usage of the parts/ components, at the expense of the Supplier (including the transportation costs). The broken part/ component shall be returned by the Customer to the Supplier, during the warranty period the shipment expenses of the broken part shall be covered by the Supplier.

## V-MLAT maintenance services

SUP_GEN-001	The provision of V-MLAT maintenance services shall be commenced after the end of V-MLAT warranty maintenance period. V-MLAT maintenance service duration shall be 120 months. A regular fee of an established amount will  be paid to the Supplier for V-MLAT maintenance services.

SUP_GEN-002	V-MLAT maintenance services shall comprise:

V-MLAT software malfunction (operation disruption) identification and elimination thereof (including software fault correction, when that is necessary for fault elimination), in accordance with the notifications submitted by the Customer;

V-MLAT software licence updating services. The service must enable the Customer to receive and install all V-MLAT software versions/ software updates produced by the V-MLAT manufacturer (including software firmware, operating systems and software of the third parties), necessary for ensuring proper operation of the V-MLAT. In case the Customer is unable to install the updates independently, the updates shall be installed by the Supplier;

Consultations on the operation and use of the System, equipment maintenance, the identification and resolution of faults and malfunctions, as well as on changes to the architectural layout of System stations and their adaptation to changing operational locations.

SUP_GEN-003	V-MLAT maintenance services shall be provided in accordance with Customer notifications about V-MLAT faults/ disruptions. The notifications shall be submitted by the Customer in written (on website, by e-mail or in any other written form acceptable to the parties). V-MLAT maintenance services shall be provided 8 hours per day, 5 days per week, during the work hours of the Customer.

SUP_GEN-004	Response time for notifications about V-MLAT faults/ operation disruptions (response time to notifications shall be perceived as the time period from sending the notification about the fault to the Supplier until start of solution):

Critical fault – 2 work hours;

Non-critical fault – 1 working day;

SUP_GEN-005	Restoration of V-MLAT operation (response time and time during which additional information is expected from the Customer, without which high-quality solution is reasonably not possible are not included):

- In case of critical fault – 1 working day, by remote access, or 72 hours, on V-MLAT installation site (when V-MLAT fault cannot be eliminated in remote manner), from sending the notification from the Supplier about fault notification reception to the Customer. In case the Supplier provides a temporary solution for V-MLAT operation/ restoration of operation, the Customer may recognise the fault as non-critical;

- In case of non-critical fault – 5 working days, by remote access, or within 10 working days on V-MLAT installation site (when V-MLAT fault cannot be eliminated in remote manner), from sending the Customer notification about fault notification reception to the Supplier.

Note: 	In case of complicated, non-standard situations or when detailed data analysis with the Customer is necessary, a different term for solution may be agreed upon.

SUP_GEN-006	The fault shall be considered eliminated (solved), when the Supplier provides the Customer with V-MLAT software corrections or indicates how to eliminate the cause of fault (disruption) and the Customer confirms in written manner that there is no V-MLAT fault (V-MLAT fault was eliminated). In case the Supplier is unable to eliminate the V-MLAT software faults in remote manner or with the support of Customer personnel, the Supplier shall be liable to eliminate such faults on V-MLAT installation site.

SUP_GEN-007	Consulting services shall be provided by Help Desk or responsible employee of the Supplier in accordance with inquiries submitted by the Customer by telephone or e-mail. Depending on the method, how the Customer submits the inquiry, the Supplier shall provide consulting services by telephone, e-mail or in any other form acceptable to the parties. In case the Supplier is unable to provide proper consultation at once, the Supplier shall be liable provide replies to unanswered inquiries not later than within 1 working day counting from the submission of the Customer inquiry. Under the agreement of the parties this term can be extended for reasonable period.

## V-MLAT hardware repair services

After the end of V-MLAT warranty period the V-MLAT hardware preventive maintenance and technical support works shall be carried out by the Customer at own expense and capacities, i.e. in accordance with the norms established by the Supplier the V-MLAT hardware preventive maintenance and repair shall be carried out by Customer employees trained by the Supplier.

SUP_GEN-008	In case V-MLAT hardware malfunctions are identified during the post-warranty period, the Customer shall procure the V-MLAT hardware (part/ component – hereinafter referred to as part or parts) repair services and/ or new parts in accordance with demand, submitting orders to the Supplier following the procedure established under this Chapter.

SUP_GEN-009	In case the Customer is unable to eliminate the V-MLAT hardware malfunctions at own capacities, such capacities on V-MLAT installation site shall be eliminated by the Supplier, as the Customer orders hardware repair on site services. The provided hardware repair on site services shall be reimbursed to the Supplier in accordance with actual time necessary for the representative(s) of the Supplier to repair the hardware, applying the rate established under the Tender of the Supplier, which must include travel and accommodation expenses of the representative(s) of the Supplier.

SUP_GEN-010	Information exchange and works are implemented in the following order:

- The Customer shall notify the Supplier in written by electronic means about the fault of the part, describing the fault in accordance with the form agreed upon with the Supplier;

- Within 1 (one) working day from the day of sending the notification of the Customer the Supplier shall notify the Customer about the preparation to accept the part and shall provide the Customer with repair order number, or, in case the part is not to be repaired (cannot be repaired), shall notify the customer about it;

- Having received the repair order number, the Customer shall send the broken part to the Supplier and shall provide the Supplier with written information about the shipment;

- Not later than within 5 (five) working days from the reception of the broken part the Supplier shall evaluate the fault and calculate (in accordance with the rates indicated under the Contract) the part repair price and indicate it to the Customer. At the request of the Customer, the Supplier shall be liable to provide repair price justification;

- The Customer shall evaluate the received information and notify the Supplier, whether the Customer is willing to repair the broken part or the broken part must be replaced by a new part at the rates indicated in the Tender of the Supplier (in case the price of the part is not indicated under the Tender – at the rates indicated in the sales place of the Supplier, catalogue or internet website or, in case they are not published, at the rates offered by the Supplier and acceptable to the Customer). In any case the repair price or the price of the new part (the price of which is not indicated under the Tender) must be agreed upon in advance and confirmed in written in the order or by e-mail. Part repair price cannot exceed the price of new equivalent part;

- The Supplier shall ensure the following to the Supplier not later than within:

  - 30 calendar days after the day of confirmation of the order of the Customer, i.e. written confirmation of the Customer to repair the broken part shall repair the broken part, test the operation thereof and deliver it the Customer. The Supplier shall provide the Customer with the note on transfer and acceptance for the provided repair services signed by the Supplier together with the part sent or by e-mail;

  - 15 calendar days after written confirmation of the order of the Customer, in case the Customer orders a new part, it shall be delivered to the Customer. The Supplier shall provide the Customer with the note on transfer and acceptance signed by the Supplier together with the part sent or by e-mail. The sending or return expenses of the part shall be covered by the Customer.

- The Customer shall be entitled at any time before the written confirmation of the Customer to repair the broken part, without compensating the Supplier for any incurred costs/ losses, to cancel the order by notifying the Supplier.

- The Supplier shall grant a warranty of 6 months to the repaired part. The warranty term shall be calculated from the day of reception of the repaired part, i.e. from signing the note on transfer and acceptance of the services. In case the repair of the part was executed in poor quality, the Supplier shall be liable to eliminate the identified defects within the warranty term (i.e. to repair the part or to provide a new analogous part) at own expense not later than within 30 days from the day of sending the written notification of the Customer about the defects to the Supplier. The expenses of sending and delivering the parts shall be covered by the Supplier.

- The Supplier shall grant a warranty of 24 months to new parts. The warranty term shall be calculated from the moment of reception of the new part, i.e. from signing the note on transfer and acceptance. During the warranty period identified defects of the parts must be corrected at the expense of the Supplier or the parts with defects must be replaced with new analogous parts not later than within 30 days from the day of sending the notification of the Customer about the defects to the Supplier. The expenses of sending and delivery of the parts shall be covered by the Supplier.

- The Customer shall procure hardware repair services or new parts (in case it is impossible to repair them or the repair is not cost-efficient) in accordance with actual demand, without exceeding the maximum amount allocated for the procurement of these services under the Contract.

## Spare parts

WAR_SPA-001	The Tender of the Supplier shall include spare components, spare parts, special tools and test equipment as specified below. Spare components are specified or defined as repairable items, which are kept in store to be exchanged for the same components in case of fault.

WAR_SPA-002	The Supplier shall supply a preliminary list of mandatory spare parts for the offered V-MLAT.

WAR_SPA-003	The list shall contain at least one device/ component (printed mounting board or Line Replaceable Unit) of each type in the quoted equipment.

WAR_SPA-004	Each component/ spare unit in the list shall be quoted and priced separately.

WAR_SPA-005	The list shall also contain the calculated figures of MTBF, MTBCF for each device separately.

WAR_SPA-006	The quantity of spare parts and spare units shall be balanced against the maintenance philosophy, cost and demand of spare parts to ensure uninterrupted V-MLAT operation.

WAR_SPA-007	The Supplier shall be liable to guarantee the supply of spare parts, units and components or proper replacement for the Customer during a period not shorter than the V-MLAT lifetime (15 years).

Note: 	Adequate replacement – when an original component cannot be provided, an alternative is offered. If an alternative is proposed, it shall be considered that its effect on the entire V-MLAT operation is evaluated by the manufacturer to be completely identical to the original component.

WAR_SPA-008	If delivery of a particular type of spare parts is becoming difficult or if the manufacturing of such is stopped, the Supplier shall notify Customer.

WAR_SPA-009	Such a notification shall be accompanied by a proposal for suitable replacement of such spare parts and by a proposal allowing Customer to procure enough to cover the remaining lifetime of the V-MLAT.

WAR_SPA-010	Spare parts shall be of the same quality as the originally installed parts, and equipment,

WAR_SPA-011	and shall be subject to the same part selection criteria, quality control, testing and burn-in as the original equipment.

WAR_SPA-012	The set of spare parts shall be delivered to the Customer no later than the date of installation and handover of the Palanga V-MLAT site, to Balio Karvelio st. 25, Vilnius adress.

### List of spare parts

WAR_SPA-013	For each of the spares (which cover all parts in the offered equipment) in the list, excluding piece parts, the following information shall be given:

- Item number (i.e., part, serial, commercial stock or drawing number, etc.);

- Descriptive title with indication of group e.g., analogue, digital etc.

- Category (repairable, non-repairable, multilayer, sub-contracted item etc.);

- Quantity of the equipment and to which equipment (functional unit) it belongs;

- Fault rate, MTBF, MTBCF;

- Unit cost, the purchase price of the item as spare;

- Safety requirements (if any);

- Size and weight;

- Manufacturer.

WAR_SPA-014	All parts (except for low cost material) which wear out life is less than the required life of the equipment shall to be identified and the following data supplied:

- Guaranteed life, operational hours;

- Guaranteed shelf life;

- Man hours to remove and replace.

# SAFETY

SAF_GEN-001	The Supplier shall have an established and documented processes for software development and provide evidence of using a standard such as ED 109A or ED-153.

SAF_GEN-002	A preliminary Safety Plan shall be included in the Tender explaining how the Supplier intends to manage the safety work.

SAF_GEN-003	The Supplier shall identify frequency of the hazard occurrence not less then following hazards:

- Loss of 2D position (or of all parameters);

- Display of erroneous 2D position (i.e. corruption, of position or of all parameters);

- Loss of barometric altitude;

- Display of erroneous barometric altitude (i.e. corruption);

- Loss of aircraft identity (call-sign);

- Display of erroneous aircraft identity (call-sign) (i.e. corruption).

SAF_GEN-004	The Supplier shall identify events causing hazards. Events shall include common cause problems.

SAF_GEN-005	The Supplier shall identify and implement mitigations to hazards or events.

SAF_GEN-006	The Supplier shall verify completeness of identified events and common causes causing hazards and mitigations.

SAF_GEN-007	The System software shall be SWAL3 compliant. The supplier will be required to demonstrate compliance with this requirement via FAT procedure.

SAF_GEN-008	Third-party software (COTS) shall have a proven track record of reliable operation, e.g. at least six months (4380 hours) of continuous operation without failure.

# TRAINING

This Chapter describes requirements for the training, allowing to ensure introduction, understanding, operation and technical maintenance of V-MLAT indicated under the Contract.

The objective of the training is to ensure that the engineers are thoroughly acquainted with V-MLAT, the impact thereof upon air traffic services and the differences between any Surveillance equipment.

TRA_QUA-001	The Supplier shall organise trainings for Customer employees. The following separate trainings shall be organised:

- Engineer, who shall be responsible for technical maintenance and repair of V-MLAT “Technical Training”. Number of training participants – not more than 10. Duration of trainings – not more than 10 days.

- Engineer, who is responsible for technical maintenance of air traffic system – the main surveillance data consumer maintainer. Number of training participants – not more than 4. Duration of trainings – not more than 3 days.

TRA_QUA-002	The Technical Trainings shall be organised at the territory of the Supplier before each FAT (in Vilnius and Palanga).

TRA_QUA-003	The Supplier shall be liable for the preparation of training programs, training material and organisation of the trainings. The Supplier shall provide the training material in printed form and/ or recorded in data media, the training material shall be retained by the training participants.

TRA_QUA-004	Travel and accommodation expenses of the Customer employees shall be covered by the Supplier.

TRA_QUA-005	The trainings shall be organised in accordance with training programs agreed in advance, prepared for target audience. The Supplier shall be liable to provide and coordinate with the Customer also to receive confirmation of the Customer regarding the training program and the contents thereof for every training course separately not later than within 1 month before the envisaged training. The training program shall indicate comprehensive training content, duration, training schedule, training location and provided educational means.

TRA_QUA-006	The trainings shall be organised and training documents shall be provided in Lithuanian or English languages.

TRA_QUA-007	Technical trainings shall comprise both theoretical and practical trainings. The Supplier shall undertake the liability to provide all necessary parts and equipment, which may be necessary during the practical trainings.

TRA_QUA-008	Technical trainings must include at least the following fields:

- V-MLAT structure;

- hardware;

- software;

- interaction and interfaces;

- configuration;

- technical maintenance, diagnostics (identification) of faults/ disruptions, possible faults/ disruptions and their elimination methodology and features;

- comprehensive information about V-MLAT operation and functions, maintenance, support and repair.

TRA_QUA-009	The main objective of the technical trainings is to train technical personnel to carry out V-MLAT maintenance, support and repair without external help. Technical personal trainings shall be of a certain level so that after the trainings the trained employees would be able to independently:

- train other employees;

- Carry out System operation maintenance, simulation of changes to the deployment architecture, configuration, support, technical maintenance, and repairs.

TRA_QUA-010	Upon successful completion of theoretical and practical training, a participant of technical trainings shall be issued technical maintenance specialist certification (certificate).

# QUALITY ASSURANCE AND CONTROL

## Acceptance tests, definitions and milestones

A description of the Customer test requirements is provided in this Chapter. It contains Customer test objectives, describes categories of tests to be conducted and sets out documentation requirements. The objective of the Customer test requirements is a full verification that the V-MLAT, when completed and installed on site, meets the contractual specifications, as laid down in the product specification.

QUA_ACT-001	The tests are subdivided into three categories: production tests, acceptance tests and pilot operation.

QUA_ACT-002	The Supplier shall perform production tests in order to verify compliance to specifications of items at lower levels.

QUA_ACT-003	Acceptance tests shall be performed on items at sub-system level and higher, in order to demonstrate that the item concerned meets the specified functional performance. The Customer is prepared to accept such tests being applied to units if the performance or capabilities of such units cannot be demonstrated properly under higher-level tests, or if testing of units will give a better verification of compliance.

QUA_ACT-004	Acceptance tests are of vital significance for the Customer. Every acceptance test shall be performed in the presence of Customer’s appointed representative(s), who will sign the acceptance documentation upon successful completion of the tests.

### Factory acceptance tests (FAT)

QUA_ACT-005	FAT shall be run to verify whether or not the V-MLAT, before delivery, satisfies the functional and performance requirements and technical specifications specified in the Contract.

QUA_ACT-006	FAT are tests carried out at the Supplier's premises and attended by representatives of the Customer. FAT tests shall be carried out in accordance with a test plan and at a time agreed in advance between the parties.

QUA_ACT-007	Upon successful completion of the FAT, the Customer and the Supplier shall sign a test report. The report must include all identified defects and failed tests. On the basis of this report, a FAT certificate shall be issued to the Customer and signed by the Supplier. If the FAT identifies minor deficiencies and comments, the Supplier shall draw up a plan for remedying the deficiencies, which shall be agreed with the Customer.

QUA_ACT-008	If the FAT identifies significant deficiencies, the FAT certificate shall not be signed. The Supplier shall agree with the Customer the date of the repeat FAT and the necessary documentation.  The Customer shall decide on the criticality of the deficiencies identified during the FAT.

QUA_ACT-009	If the FAT is unsuccessful due to the fault of the Supplier, the Supplier shall bear the costs of the repeat FAT at its own expense, including the travel and accommodation costs of the Customer's representatives.

QUA_ACT-010	Upon successful completion of the FAT (signing of the certificate) and prior to the provision of the training services by the FAT, the Customer and the Supplier shall sign a note on transfer and acceptance with the remark "End of Factory Acceptance Tests".

#### FAT documents

QUA_ACT-011	The Customer must approve the required documentation (test plan) as set out below.

QUA_ACT-012	The test plan shall be composed by the Supplier. Once approved by the Customer, these documents shall become obligatory for both parties.

QUA_ACT-013	The test plan must specify the general conditions and definitions, the test sequence which defines the sequence of test steps, the relevant test specifications and the test procedures. The test specification must include a detailed description of the various tests and include acceptance/rejection criteria, conditions and interfaces. In addition, test performance requirements such as test tools, test equipment and test software must be included.

QUA_ACT-014	It shall be ensured that test plan is available for revision and confirmation of the Customer not later than within two (2) months before the envisaged FAT start.

QUA_ACT-015	The Customer shall have two (2) weeks to evaluate and complete the documents submitted by the Supplier. The Supplier shall accept all comments from the Customer (provided that they comply with the requirements of the Technical Specification) and incorporate them into the test specification and other documents forming the test plan.

QUA_ACT-016	The final version of the documents constituting the FAT Test Plan must be agreed by both parties and submitted to the Customer at least three (3) weeks prior to the commencement of the FAT Test.

QUA_ACT-017	The test protocols must be adapted to capture the test records and follow the agreed test procedure.

QUA_ACT-018	The test report forms must be agreed before the FAT is launched.

QUA_ACT-019	The test report must specify the type of test, the test object and the result of the test, including any comments. The notes should indicate the agreed tolerances to the test procedure

#### FAT verification

QUA_ACT-020	The following set of documentation shall be transferred to the Customer as evidence of a successfully completed FAT:

- Test protocol with certification records, signed by the representatives of the Customer and Supplier after completion of the tests;

- A set of completed test records;

- FAT certificate.

- note on transfer and acceptance with the remark “End of Factory Acceptance Tests” (according to the form specified in the Annex to the Contract).

QUA_ACT-021	These FAT documents, properly signed by the representatives of the Supplier and the Customer shall be accepted as confirmation that the Customer agrees for the delivery of equipment to the site.

### Site acceptance tests (SAT)

QUA_ACT-022	SAT is specified or defined as a test at the premises of the Customer and equipment installation locations, which is conducted in accordance with the test plan agreed by the Parties in advance and on time agreed on in advance .

QUA_ACT-023	SAT must include flight tests and ground movement tests within the aerodrome area (hereinafter referred to as “flight tests”). Scheduled air traffic flights are also used for these tests.

QUA_ACT-024	The objective of the tests  (ICAO Doc 8071 Vol III) is to measure, verify and validate the V-MLAT’s overall performance including site location, V-MLAT implementation, optimisation of environmental influence and interoperability with simultaneously operated equipment and equipment intended to be operated, by collecting and analysing output data with respect to coverage, probability of detection and accuracy.

QUA_ACT-025	The Supplier shall coordinate the flight test program and may participate in these tests. Flight tests may also be conducted using chartered flights and vehicles. Flight tests are ordered by the Customer and they are executed at the expense of the Customer.

QUA_ACT-026	After successful completion of SAT the Customer and the Supplier shall sign a test protocol. The protocol shall include all the identified defects and failed tests. On the basis of this protocol the Customer shall be provided SAT certificate signed by the Supplier. If minor deficiencies are identified during SAT and comments are submitted, the Supplier prepares a deficiency elimination plan, which is coordinated with the Customer.

QUA_ACT-027	If significant deficiencies are identified during SAT, SAT test report shall not be signed. The Supplier must agree with the Customer on the date of the repeated SAT and the necessary documents. The Customer decides on the criticality of the deficiencies identified during SAT.

QUA_ACT-028	Upon successful completion of SAT (after signing the test protocol), the Customer and the Supplier shall sign a note on transfer and acceptance with the remark “End of V-MLAT tests on installation site”.

#### SAT documents

QUA_ACT-029	The set of necessary documents (test plan) shall be confirmed by the Customer as established below.

QUA_ACT-030	The test plan must be drawn up by the Supplier. Once approved by the Customer, these documents become binding to both Parties.

QUA_ACT-031	Test plan shall indicate general conditions and definitions, sequence of tests, which defines the test action sequence and establishes the specifications of the related tests and test procedures.  Test specification shall comprise comprehensive description of various tests, also include acceptance/rejection criteria, conditions and interfaces. Moreover, test execution requirements shall be included, such as test tools, test equipment and test software.

QUA_ACT-032	The test plan shall be submitted to the Customer for review and approval not later than two (2) months prior to the scheduled start of FAT.

QUA_ACT-033	The Customer shall be provided two weeks for evaluation and supplementation of the documents submitted by the Supplier. The Supplier shall be liable to accept all comments of the Customer (provided that they meet the requirements of the Technical Specification) and include them into test specification and other documents that are part of the test plan..

QUA_ACT-034	The final version of the documents constituting SAT test plan must be agreed upon by both Parties and submitted to the Customer at least three (3) weeks prior to the start of SAT tests.

QUA_ACT-035	Test records shall be adjusted for compiling test records in accordance with the test procedures coordinated with Customer.

QUA_ACT-036	Test record form shall be coordinated before commencing the execution of SAT.

QUA_ACT-037	The test report shall specify the type of test, the test object and the test result, including possible comments. The comments should indicate the agreed permissible deviations from the test procedure.

#### SAT verification

QUA_ACT-038	The following set of documents shall by provided to the Customer as evidence of successfully completed SAT:

- test protocol with certification records, signed by the representatives of the Customer and Supplier after the completion of the test;

- set of completed test records;

- SAT certificate;

- the note on transfer and acceptance with the remark “End of V-MLAT installation tests on site” (according to the form specified in the Annex to the Contract).

### Pilot operation

QUA_ACT-039	Pilot operation is a period from the performed V-MLAT SAT (after the Supplier and the Customer sign the note on transfer and acceptance with the remark “End of V-MLAT installation tests on site”) to V-MLAT preparation for surveillance services provision for operational environment – air traffic management system.

QUA_ACT-040	V-MLAT operation stability and continuity characteristics are tested during pilot operation period.

QUA_ACT-041	Testing plan shall be composed by the Supplier.

QUA_ACT-042	Pilot operation period lasts 14 calendar days and shall be planned so that it would be part of V-MLAT installation period.

QUA_ACT-043	The below indicated set of documents shall be provided to the Customer as evidence of successfully completed pilot operation:

- test protocol with records, which after the end of tests shall be signed by the representatives of the Customer and the Supplier;

- set of completed test records;

- successful pilot operation completion certificate;

- final note on transfer and acceptance with the remark “End of pilot operation” (according to the form specified in the Annex to the Contract).

## Project management

DOC_MAN-001	Not later than within 30 days since the Contract comes into force the Supplier shall be liable to prepare and coordinate with the Customer project management (V-MLAT Installation) plan, which shall specify the following:

- Project goals, priorities;

- Scope and results of the milestones;

- Work execution schedule;

- Risks and methods for managing them;

- Communication principles;

- Liabilities;

- Criteria for accepting interim and final results;

- Plan correction and change management procedure and their execution term;

- Problem solving procedure.

Note: 	The work execution schedule can be amended under the agreement of the parties, by amending the interim V-MLAT installation terms, nevertheless, the final V-MLAT installation and transfer to the Customer term cannot be changed (extended), except for term extension cases established under the Contract, provided that they are defined.

DOC_MAN-002	The Supplier shall be liable to prepare and provide the Customer with project execution course (V-MLAT installation) report at certain regularity (for example, every quarter, half a year) agreed upon by the parties and established under V-MLAT Installation Plan.

## Documents

DOC_MAN-003	Along with the Tender the Supplier shall be liable to submit:

- description of the offered V-MLAT, which would indicate ensuring the implementation of Technical Specification requirements (including the theoretical coverage to ensure the requested airspace coverage);

- V-MLAT technical documentation, which shall indicate:

envisaged V-MLAT hardware and software architecture and logical schemes (including the envisaged resources CPU, RAM, HDD/SSD capacity and etc.);

list of hardware and software comprising the V-MLAT, indicating the manufacturer, model/ product code of the devices (including measurement, testing and fault location determination components, which are installed and constitute an inseparable part of the V-MLAT);

list of special devices and testing equipment, necessary for fault location determination, testing, maintenance and repair (which is not part of the V-MLAT equipment), indicating the purpose thereof;

- List of spare parts/ set of components provided with the V-MLAT, indicating the information requested in accordance with Item [TBD] of the Technical Specification.

- Other relevant documents required by the Technical Specification:

a proof the letter of fulfilment of installation obligations [TBD];

a certificate issued by CAA or EASA or an equivalent [TBD]);

a proof that the offered V-MLAT is successfully used for the provision of air traffic management services [TBD];

Preliminary safety plan [TBD];

Declaration of conformity with the Directives of the European Parliament and the Council (NIS2, radio frequency compatibility, product quality, etc.) [TBD].

DOC_MAN-004	Not later than within one (1) month before the envisaged date of signing the final note on transfer and acceptance of the V-MLAT the Supplier shall be liable to provide the Customer with the following:

- The entire project documentation (digital media, which allow making document copies). Project documentation shall comprise the following documents:

- V-MLAT specification document;

- Updated comprehensive technical design of the installed V-MLAT;

- V-MLAT operation scenarios for launching separate parts and equipment of the V-MLAT, repeated launch, recovery of operation;

- Procedures for defining the respectful V-MLAT parameters (main/ routine operations/ acceptability);

- technical and operational guide of the testing platform;

- V-MLAT safety guarantee evaluation documents;

- Operational guide;

- Technical guide.

- Declaration(s) of compliance and/ or certificate(s) of compliance, confirming the compliance of the V-MLAT with the regulations, directives, standards and specifications indicated under [TBD] of the Technical Specification.

Note:	Operational guide shall be perceived as operational guide of the V-MLAT, functions, opportunities, usage and etc. thereof, it is intended for training employees.

DOC_MAN-005	Technical guide shall be perceived as technical description of the V-MLAT, which provides comprehensive information about certain functions, V-MLAT fault elimination and repair, user management, configuration, logics, links, communication and etc., it is intended for the technical personnel.

DOC_MAN-006	Documentation and guides shall be perceived not only as the documents themselves , taking into consideration their purpose, but they shall also include other files, such as: configuration files, video files, sound recording and etc., which are necessary in order to provide comprehensive information on certain topic.

DOC_MAN-007	During the warranty validity term, under the request of the Customer, the Supplier shall be liable to prepare additional documentation and/ or V-MLAT user guides, in case the previously provided and accepted documentation did not include certain V-MLAT technical or operational information.

DOC_MAN-008	Every document shall be marked with a document ID number and issue date. This applies to paper copy as well as data media.

DOC_MAN-009	All the submitted documents shall be prepared in Lithuanian or English.

| Annexes Explanations, definitions and abbreviations |  |  |
| --- | --- | --- |
| ABBREVIATIONS |  | EXPLANATION |
| °C |  | Degrees of Celsius (Centigrade) |
| 24-bit |  | ICAO 24-bit aircraft Address |
| 2D |  | 2-Dimensional |
| 3D |  | 3-Dimensional |
| A-SMGCS |  | Advanced Surface Movement Guidance and Control V-MLAT |
| A, C |  | Mode A + Mode C |
| ACAS |  | Airborne Collision Avoidance V-MLAT |
| ACC |  | Area control center |
| ACID |  | AirCraft ID |
| AD |  | Aerodromes |
| ADS |  | Automatic Dependent Surveillance |
| ADS-B |  | Automatic Dependent Surveillance - Broadcast |
| AFIS |  | Aerodrome Flight Information Service |
| AIC |  | Aeronautical Information Circular |
| AIM |  | Aeronautical Information Management |
| AIP |  | Aeronautical Information Publication |
| AIS |  | Aeronautical Information Services |
| AIXM |  | Aeronautical Information Exchange Model |
| An |  | Annex |
| ANS |  | Air Navigation Services |
| ANSP |  | Air Navigation Service Provider |
| APP |  | Approach |
| APT |  | Airport |
| ArcGIS |  | ArcGIS is a geographic information V-MLAT (GIS) by Esri |
| ARP |  | Aerodrome Reference Point |
| ARTAS |  | ATM Surveillance Tracker and Server |
| ASTERIX |  | All-purpose Structured Eurocontrol Surveillance Information Exchange (format) |
| AST |  | ASTERIX (ATM standard) |
| ATC |  | Air Traffic Control |
| ATCC |  | ATC Centre |
| ATCO |  | Air Traffic Control Officer |
| ATCRBS |  | ATC V-MLAT Beacon V-MLAT = SSR |
| ATM |  | Air Traffic Management |
| ATS |  | Air Traffic Services |
| ATSSA |  | ADS-B Traffic Surveillance Systems and Applications |
| BITE |  | Built-In Test Equipment |
| CAA |  | Civil Aviation Administration |
| CAT |  | Category |
| CE |  | Conformité Européenne / European Conformity |
| CE |  | Conducted Emissions (Test) |
| CEE |  | Commission internationale de réglementation en vue de l'approbation de l'Équipement Électrique / Certification of Electrotechnical Equipment |
| CFT |  | Call for Tender |
| CM |  | Certification Memorandum (EASA) |
| RCMS |  | Remote Control and Monitoring V-MLAT |
| RS |  | Ground Station |
| CNS |  | Communication, Navigation, Surveillance |
| COM |  | Communication |
| COTS |  | Commercial-Off-The-Shelf |
| CTA |  | Control Area |
| CTR |  | Control Zone |
| DAT |  | DAT-file, Data |
| DC |  | Direct Current |
| Doc |  | Document (ICAO etc.) |
| DoC |  | Declaration of Conformity |
| DOC |  | DOC-file, Document Format |
| DOC |  | Document |
| DTED |  | Digital Terrain Elevation Data |
| DTM |  | DTM-file, Digital Terrain Model |
| DV |  | Development Required |
| E |  | East (longitude) |
| EASA |  | European Aviation Safety Agency |
| EATCHIP |  | European Air Traffic Control Harmonization and Integration Program |
| EATM |  | European Air Traffic Management |
| EATMP |  | European Air Traffic Management Program(me) |
| EC |  | European Commission |
| ECAC |  | European Civil Aviation Conference |
| ECPI |  | European Consumer Price Index |
| Ed |  | Edition |
| ED |  | Eurocae Document |
| EEA |  | European Economic Area |
| EEC |  | European Economic Community |
| EGNOS |  | European Geostationary Navigation Overlay Service |
| EHS |  | Environmental, Occupational Health and Safety |
| EHS |  | Enhanced Mode S |
| ELEV |  | Elevation (MSL-GND) |
| ELS |  | Mode S Elementary Surveillance |
| EMC |  | Electro-Magnetic Compatibility |
| EMS |  | European Mode S Station |
| EN |  | English |
| ENR |  | En-Route |
| ER |  | Eurocae Report |
| ES |  | Extended Squitter |
| ESARR |  | EUROCONTROL Safety Regulatory Requirement |
| ESASSP |  | EUROCONTROL Specification for ATM Surveillance V-MLAT Performance |
| ET |  | Eurocontrol standards classification (ET1…ET4) |
| ETCAS |  | Enhanced TCAS |
| E-TCAS |  | Enhanced TCAS |
| EU |  | European Union |
| Eurocae |  | European Organisation for Civil Aviation Equipment |
| EUROCONTROL |  | European Organisation for the Safety of Air Navigation |
| EX |  | Existing Feature |
| F |  | Fixed |
| FA |  | Final Acceptance |
| FA-C |  | Final Acceptance Certificate |
| FAT |  | Factory Acceptance Test |
| FAT-C |  | Factory Acceptance Test Certificate |
| FC |  | Fully Compliant |
| FIR |  | Flight Information Region |
| FIS-B |  | Flight Information Services - Broadcast |
| FIZ |  | Flight Information Zone |
| FL |  | Flight Level (Altitude in hundreds of feet based on standard pressure 1013 hPa) |
| FM |  | Frequency Modulation (Radio Band) |
| FMECA |  | Fault Mode, Effects, and Criticality Analysis |
| FPD |  | Flat Panel Display |
| FT |  | Feet, foot |
| FTP |  | File Transfer Protocol |
| G |  | Wind turbine/ generator type |
| Galileo |  | Global Navigation Satellite V-MLAT currently being built by the European Union and European Space Agency |
| GDLP |  | Ground Data Link Processor |
| GHz |  | Gigahertz |
| GIS |  | Geographic Information V-MLAT |
| GND |  | Ground Level |
| GNSS |  | Global Navigation Satellite V-MLAT |
| GPS |  | Global Positioning V-MLAT |
| GS |  | Ground Station, LAM/ MLAT Ground Station |
| GUI |  | Guidance (Eurocontrol) |
| HD |  | European Standard / Harmonization Document |
| HR |  | Hours |
| Hz |  | Hertz |
| IAS |  | Indicated AirSpeed |
| IATA |  | International Air Transport Association |
| IC |  | Interrogator Code |
| ICAO |  | International Civil Aviation Organisation |
| ID |  | Identificator, ID number |
| IEC |  | International Electrotechnical Commission |
| II |  | Interrogator Identifier |
| IOP |  | Interoperability (regulation) |
| I/P |  | Identification of Position |
| IP |  | Internet Protocol |
| ISBN |  | International Standard Book Number |
| ISO |  | International Standards Organisation |
| kHz |  | Kilohertz |
| km² |  | Square kilometer |
| kV |  | Kilovolt |
| LAM |  | Local Area Multilateration |
| LAN |  | Local Area Network |
| LCMS |  | Local Control and Monitoring V-MLAT |
| LED |  | Light Emitting Diode |
| LGS |  | Latvijas Gaisa Satiksme - ANSP Latvia |
| LPST |  | Low Power SSR Mode-S Transponder |
| m |  | Meter |
| M |  | Meter |
| mm |  | Millimetre |
| m/s |  | Meters per second |
| ms |  | Milliseconds |
| MAC |  | Mach number |
| MHz |  | Megahertz |
| MIB |  | Management Information Base (in SNMP) |
| Mil |  | Military |
| MIL |  | Military (Standard) |
| MLAT |  | Multilateration |
| MM |  | Minutes |
| mm² |  | Square millimetre |
| MOPR |  | Minimum Operational Performance Requirements |
| MOPS |  | Minimum Operational Performance Standards |
| MPS |  | Minimum Performance Standard |
| MS |  | Microsoft |
| MSL |  | Mean Sea Level |
| MSSR |  | Monopulse Secondary Surveillance V-MLAT |
| MTBCF |  | Mean Time Between Critical Faults |
| MTBF |  | Mean Time Between Faults |
| MTTR |  | Mean Time To Repair |
| MRT |  | Mean Response Time in hours (the average time from notification of fault for a technician to be ready to commence repair action) |
| N |  | North (Latitude) |
| N-1 |  | One station off, |
| N-2 |  | Two stations off, |
| N-m |  | Degraded Mode, m stations off |
| NATO |  | North Atlantic Treaty Organisation |
| NAV |  | Navigation |
| NAVAID |  | Navigation Aid |
| NB |  | Nota Bene ("note well", "pay attention", "take notice", "note") |
| NC |  | Not Compliant |
| NET |  | Network |
| NIC |  | Navigation Integrity Category |
| NICBARO |  | Barometric Altitude Integrity Code |
| NM |  | Nautical Mile(s) |
| NW |  | Wind turbine/generator |
| No |  | Number |
| NRA |  | Non-V-MLAT Airspace |
| OB |  | Obstacle |
| OBST |  | Obstacle |
| OC |  | Over Compliant |
| P2 |  | Pulse 2 |
| P4 |  | Pulse 4 |
| PANS |  | Procedures for Air Navigation Services (ICAO) |
| PCB |  | Printed Circuit Board |
| PCD |  | Probability of Code Detection |
| PCS |  | Preliminary Case of Safety |
| PD |  | Probability of Position Detection |
| PDF |  | PDF-file, Portable Document Format |
| PFCD |  | Probability of False Code Detection |
| PFD |  | Probability of False Detection |
| PIC |  | Position Integrity Category |
| POEMS |  | Pre-Operational European Mode S |
| POINT_X |  | WGS-84 Latitude (E) DD.xxxxxxxx |
| POINT_Y |  | WGS-84 Longitude (N) DD.xxxxxxxx |
| pp. |  | Printed Pages |
| RA |  | Resolution Advisory |
| RAD |  | V-MLAT (Controlled Airspace) |
| RCMS |  | Remote Control and Monitoring V-MLAT |
| RCS |  | Risk Classification Scheme |
| RE |  | Radiated Emissions (Test) |
| ref |  | Reference |
| Ref |  | Reference |
| RJ45 |  | Registered Jack 45 |
| RMS |  | Root Mean Square |
| RMSD |  | Root Mean Square Deviation |
| RMSE |  | Root Mean Square Error |
| RMTR |  | Reference and Monitoring Transponder |
| ROM |  | Read-Only Memory |
| RS232 |  | Recommended Standard 232 |
| RTCA |  | Radio Technical Commission for Aeronautics |
| RWY |  | Runway |
| Rx |  | Receiver |
| RX |  | Receiver |
| S |  | South (Latitude) |
| s |  | Second(s) |
| SAF |  | Safety (Eurocontrol) |
| SAM |  | Safety Assessment Methodology |
| SASS-C |  | Surveillance Analysis Support V-MLAT for ATC-Centre |
| SAT |  | Site Acceptance Test |
| SAT-C |  | Site Acceptance Test Certificate |
| SCM |  | Software Configuration Management |
| SDA |  | V-MLAT Design Assurance |
| SDP |  | Surveillance Data Processing |
| SDPS |  | Surveillance Data Processing V-MLAT |
| SI |  | Surveillance Identifier |
| SI |  | Le Système International d'unités / International V-MLAT of Units |
| SIF |  | Selective Interrogation Format |
| SIL |  | Surveillance Integrity Level |
| SMR |  | Surface Movement V-MLAT |
| SMS |  | Safety Management V-MLAT |
| SNMP |  | Simple Network Management Protocol |
| SPC |  | Specification (Eurocontrol) |
| SPE |  | Specification (Eurocontrol) |
| SPI |  | Surveillance Performance and Interoperability |
| SPI |  | Special Position Indicator |
| SRC |  | Safety Regulation Commission |
| SRTM |  | Shuttle V-MLAT Topography Mission |
| SS |  | Short Squitter |
| SS |  | Seconds |
| SSA |  | V-MLAT Safety Assessment |
| SSAS |  | Software Safety Assurance V-MLAT |
| SSDD |  | V-MLAT/Segment Design Document |
| SSR |  | Secondary Surveillance V-MLAT |
| ST |  | Standard (Eurocontrol) |
| ST |  | Stress Test |
| ST-C |  | Stress Test Certificate |
| STD |  | Standard (Eurocontrol) |
| SUR |  | Surveillance |
| SURF |  | Surface |
| SurvSt |  | Surveillance Standard |
| T |  | Wind turbine/ generator |
| TAT |  | Turn-Around Time |
| TCAS |  | Traffic Collision Avoidance V-MLAT |
| TCAS-RA |  | TCAS Resolution Advisory |
| TCM |  | Technical Control and Monitoring |
| TDOA |  | Time Difference of Arrival |
| TG |  | Wind turbine/generator |
| THR |  | RWY Threshold |
| TIS |  | Traffic Information V-MLAT |
| TIS-B |  | Traffic Information Service - Broadcast |
| TLL |  | Tallinn Airport (IATA code) |
| TM |  | Technical Manual |
| TMA |  | Terminal Maneuvering Area, Terminal Area, Terminal Control Area |
| TOA |  | Time of Arrival |
| TR |  | Technical Report (Standard) |
| TS |  | Technical Specification (Standard) |
| TSG |  | Technical Study Group |
| TSOA |  | Time of Arrival |
| TV |  | TV mast/ TV-tower |
| TWR |  | Tower, Airport TWR |
| Tx |  | Transmitter |
| TX |  | Transmitter |
| UAT |  | Universal Access Transceiver |
| UDP |  | User Datagram Protocol |
| UF |  | Uplink Format |
| UHF |  | Ultra-High Frequency |
| UPS |  | Uninterruptable Power Supply |
| V |  | Volt(s) |
| V |  | Version |
| VAC |  | Volts of Alternating Current |
| VAT |  | Value Added Tax |
| VDC |  | Volts of Direct Current |
| VDL |  | VHF Data Link |
| VDU |  | Visual Display Unit |
| VFR |  | Visual Flight Rules |
| VHF |  | Very-High Frequency |
| Vol |  | Volume |
| W |  | West (Longitude) |
| V-MLAT |  | Wide Area Multilateration |
| WAN |  | Wide Area Network |
| WEA |  | Wind turbine/generator |
| WEEE |  | Waste Electrical and Electronic Equipment (Directive) |
| WG |  | Work Group (Eurocae) |
| WGS-84 |  | World Geodetic V-MLAT - 1984 |
| WGS84E |  | WGS-84 Latitude |
| WGS84N |  | WGS-84 Longitude |
| WLAN |  | Wireless Local Area Network |
| WT |  | Wind Turbine/ Generator |
| WWAN |  | Wireless Wide Area Network |
| X |  | X-pulse |
| x,y |  | X and Y co-ordinates |
| XLS |  | XLS-file, Excel spreadsheet |
| ZIP |  | ZIP-file, common archive/packing format |