TECHNICAL SPECIFICATION

INSPECTION SERVICE FOR THE MAIN GAS PIPELINES USING AERIAL SURVEYING

- DEFINITIONS AND ABBREVIATIONS

  - Buyer – AB “Amber Grid”, a company legally registered and operating in accordance with the laws of the Republic of Lithuania, company code 303090867, address: Laisvės pr. 10, Vilnius, Lithuania.   More information about the Buyer and its activities can be found at www.ambergrid.lt

  - Supplier – an economic entity – a natural person, private legal entity, public legal entity, other organizations and their divisions, or a group of such persons with whom the Buyer enters into an Agreement.

  - Agreement – an agreement entered into between the Supplier and the Buyer regarding territory monitoring services.

  - Protection zones for main gas pipelines and main gas pipeline facilities and structures (hereinafter - Protection zones) – a strip of land along the pipeline route, the boundaries of which extend 25 meters on either side of the pipeline axis, and a strip of land, the boundaries of which extend 25 meters around the perimeter of the area where these facilities or structures are located;

  - The class of territory of the main gas pipeline (hereinafter - Class of territory) – 200 meters on either side of the axis of the main gas pipeline or a 200-meter radius around the outermost point of the main gas pipeline within the territory.

  - MGP – main gas pipeline.

  - LiDAR – point cloud generation using laser scanning.

  - LAS 07 – Lithuanian National Height System.

  - Non-removable vegetation – Vegetation that does not meet the criteria for removal based on trunk diameter. This category also includes leaning or tilted trees (including tree branches) that extend over the area of vegetation to be removed.

- PROCUREMENT OBJECT

  - Identification of changes in activities (actions) carried out within the monitored area of the gas transmission system (also known as the main gas pipeline) (within the boundaries of Protection zones and Class of territories), preparation of spatial data and submission of reports based on aerial photographs of the gas transmission system (pipeline) (hereinafter - Service 1).

  - Generation of a point cloud using laser scanning (hereinafter - LiDAR) and the submission of the results of the analysis and reports of the spatial data sets created on that basis (vegetation assessment for removal and/or analysis and reports on the assessment of ground surface elevation) (hereinafter - Service 2).

  - Service 1 and Service 2 are collectively referred to as the Territory surveillance Services (hereinafter collectively - the Services).

  - The Procurement object shall not be divided into parts. Any Supplier submitting a tender shall offer the entire quantity/scope of the Procurement.

  - The Buyer intends to purchase the Services, the requirements for which are described in this Technical Specification, the draft Agreement and its annexes.

- SCOPE OF THE PROCUREMENT

  - During the provision of Territory surveillance Services, the gas transmission system within the territory of the Republic of Lithuania is patrolled and monitored along routes agreed upon in advance with the Customer, which are located:

- from the border of the Republic of Belarus to the border of the Republic of Latvia;

- from the border of the Republic of Belarus to the border of the Kaliningrad Oblast of the Russian Federation;

- from Vilnius to Visaginas;

- from Panevėžys to Klaipėda;

- from Marijampolė to Klaipėda;

- from Elektrėnai to the border of the Republic of Poland.

The total length of the entire transmission system is approximately 2,288 km (Fig. 1: Main gas pipeline network).  The total area of all Class of territories is approximately 668.44 km², of which the area of protection zones accounts for 94.92 km² (Technical Specification, Fig. 2: Boundaries of Protection Zones and Class of Territories).  Throughout the term of the Agreement, the total length of the main gas pipeline in the area of areal surveilling may vary by up to 10 percent.

  - The centerlines of the main gas pipeline networks, the boundaries of the main gas pipeline protection zones, and the boundaries of the Class of territories will be provided as a separate document in ESRI Personal Geodatabase or ESRI .shp format (or equivalent) after the Agreement is signed. Prior to each flight and data collection, the Supplier will be provided with the most recent axial lines of main gas pipelines, protection zones, and Class of territories in ESRI Personal Geodatabase or ESRI .shp format (or equivalent).

  - No later than 10 days after the conclusion of the Agreement, a preliminary schedule for the Territory surveillance services to be ordered for a 12-month period shall be agreed upon with the Supplier. Each specific Service order shall be submitted to the Supplier at least one month prior to the start of the provision of the specific Service. If necessary, the duration, scope, and frequency of Service orders may be modified during the term of the Agreement, subject to agreement with the Buyer.

  - Service 1 is ordered on a preliminary basis 2 (two) times a year for the entire transmission system:

- a survey in April–May;

- a survey in September–November.

Surveying should be scheduled to coincide with the vegetation periods.

  - Service 2 is ordered as needed:

- analyses for vegetation assessment and removal are ordered, tentatively, 1 (one) time per year for the entire transmission system;

- ground elevation assessment analyses are ordered as needed, tentatively 1 (one) time per year, for transmission system lengths of at least 50 km.

  - Data collection for Service 1 and Service 2 can be ordered either separately or as a combination. In such cases, payment shall be made based on the Services actually provided and calculated according to the rate specified in the winning Supplier’s Tender (see row No. 1 or row No. 2 of the Tender table).

  - Territory surveillance Services must be fully provided no later than 4 months from the date the Buyer submits each order via e-mail.

  - The Supplier is responsible for obtaining all flight permits or other mandatory documents required for the flight. In the event of circumstances that prevent the completion of the survey in certain sections of the gas pipeline:

    - The Supplier shall notify the Buyer in writing no later than 1 day after becoming aware of such circumstances, providing a description of the factors restricting the survey, the territory/branches (km and coordinates), and documents confirming the restriction;

    - The Supplier shall submit an alternative schedule for the previously postponed flight within 5 days;

    - The restricted segments must be flown at the earliest practicable time after the restrictions are lifted, but no later than 30 days after the end of the restrictions, provided that this fits within the final term of the Agreement;

    - If the restrictions continue and it is impossible to fly the entire route by the final term of the Agreement, the Buyer shall choose one of the following solutions: (i) extend the term of the Services for the restricted section; (ii) or reduce the scope by the restricted sections (only if the Buyer expressly requests this);

    - Payment for the part of the route actually flown is made on a pro-rata basis according to the number of kilometers flown (price per km), while payment for the remaining part is made only after it has actually been flown and the relevant data has been submitted. The Supplier shall not be entitled to claim payment for parts of the route that have not been flown;

    - Territory surveillance Services are deemed to have been provided when Services 1 and (or) Service 2 have been completed, unless the Customer opts in writing for a reduction in scope, and when the Supplier provides the Customer with the documents specified in clauses 4.11, 5.13, 5.14, 5.19, 6.10, 6.11, 6.13.8, 6.14.4, 6.14.5, and 6.14.6 of these Technical Specifications to the Buyer in accordance with the scope of the ordered Service. The Supplier will be paid for the scope of Services rendered, in accordance with the signed service transfer and acceptance act. The transfer and acceptance act may be prepared separately for each Service order or for several of them.

Fig. 1 Main gas pipeline

Fig. 2 Boundaries of the Protection zone and Class of territories

- GENERAL REQUIREMENTS FOR THE PROCUREMENT OBJECT

  - Territory surveillance Services provided by the Supplier consist of:

    - the acquisition of high-resolution aerial photographs, the creation of spatial datasets based on them covering the main gas pipelines (hereinafter - MGP) provided to the Supplier, their associated infrastructure protection zones, and creation of Class of territories, as well as data analysis and the submission of reports;

    - Generation of a point cloud using the laser scanning method (LiDAR) over the area of the specified width as outlined in the MGP submitted to the Supplier (recording);

    - Creation of a Digital Terrain Model (DTM) for the specified area;

    - Creation of a Digital Surface Model (DSM) for the specified area;

    - analysis of vegetation to be removed (including preparation of a report).

    - analysis of vegetation to be removed (including preparation of a report).

  - Before conducting an aerial survey, the Supplier must inform the Buyer's responsible personnel and provide the planned flight period, specify the flight route, and provide information about the pilots conducting the flight and the aircraft used for the flight.

  - If necessary, and subject to agreement with the Buyer, other sources of information may also be used. The Supplier, when proposing the use of an additional source of information, must specify the name of the source, the method of data collection, the date, the accuracy, the intended use, and the potential impact on the results of the analysis. The use of additional information sources shall not alter the minimum performance requirements for the Services set forth in this Technical Specification.

  - All data containing coordinates must be provided in the Lithuanian Coordinate System (LKS-94), and elevations must be provided in the Lithuanian Height System (LAS07).

  - The Supplier must use the same or equivalent data processing solutions (data processing algorithms) for the processing of photographs and datasets throughout the term of the Agreement to ensure the comparability of results. Data processing solutions may be changed only upon written agreement with the Buyer and upon providing justification that the change will not impair the accuracy, integrity, and comparability of the data with previously submitted data.

  - If any discrepancies are found in the results of the Supplier’s solution (data processing algorithm), the Supplier must immediately, but no later than within 5 days, revise the solution (data processing algorithm) and provide the data to the Buyer using the revised solution. Any non-compliance with the requirements for data format, accuracy, coverage, classification, report content, metadata, or submission deadline set forth in this technical specification shall be considered a discrepancy in the results. If the Buyer identifies a discrepancy, the Supplier must correct it free of charge and submit the corrected data no later than 5 working days from the date of receipt of the Buyer’s written notice, unless the Buyer sets a longer deadline, taking into account the nature of the discrepancy.

  - Analyses and reports may be submitted in agreed-upon parts for the entire area covered by the order for surveying.

  - The results of the analyses and reports must be organized and systematically presented.

  - The structure (content) of the analyses and reports must be agreed upon with the Buyer prior to their preparation. The requirements set forth in Tables 1–7 of these Technical Specifications are minimum requirements and may be adjusted provided that such adjustments do not alter the scope, quality, or supplier liability of the Services, or the rates specified in the Agreement, during the provision of the Services.

  - In performing the analyses provided for within the scope of the Territory surveillance Services, the Supplier will be provided with information on the status of activities recorded in previous reports, as agreed upon by the Supplier and the Buyer.

  - The data, ready for use (uncompressed), must be provided to the Buyer on portable external hard drives with a capacity of at least 4 TB, equipped with a USB 3.0 or USB-C interface (or equivalent) for connection to a computer, and/or by providing the data via an online storage service. The Service provider shall provide portable external hard drives and/or online storage.

  - Throughout the term of the Agreement, the Supplier shall provide all communications, documents, and reports in Lithuanian.

- Requirements FOR THE SERVICE 1

  - Aerial photography must be conducted only under favourable weather and lighting conditions that allow for the resolution, accuracy, and object identification quality of the orthophoto map specified in this technical specification to be achieved. Data shall not be considered acceptable if fog, smoke, clouds, cloud shadows, precipitation, snow cover, heavy shadows, or other obstacles prevent the reliable identification of the objects specified in this Technical Specification.

  - The image in digital color aerial photographs must be free of fog, smoke, clouds, their shadows, and other obstacles that obscure the aerial view. Isolated obstacles obscuring the aerial view may be permitted provided that these defects are eliminated when creating orthophotographic maps.

  - The resolution of the aerial photographs must be at least 60 megapixels.

  - The Supplier may provide higher-quality data provided that this does not affect the price specified in the Agreement and does not impair the comparability of the data. The submission of lower-quality data is permitted only in exceptional, objectively justified cases, upon receipt of the Buyer's prior written consent, and only if such data still allows for the achievement of the Service outcome specified in this Technical Specification.

  - The aerial photography equipment must be equipped with a compensation mechanism to eliminate vibration errors.

  - The spatial resolution (ground sample distance / GSD) of the orthophoto map must be no less than (no worse than) 10x10 cm throughout the entire flight area.

  - The overlap of aerial photographs must be no less than 75% (longitudinal) and 60% (transverse) to ensure high-quality overlap of oblique images and minimize blind spots around tall objects and terrain features.

  - All data must be provided in the national LKS-94 coordinate system. The absolute horizontal accuracy (RMSE_x,y) of the orthophoto map, verified using independent control points, must not exceed 10 cm.

  - Aerial photography work must be carried out within a corridor that is no less than 200 m wide on either side of the axis of the Main gas pipeline (MGP) (total corridor width – 400 m). Prior to flight planning, the Buyer shall provide the Supplier with vector data of the MGP pipeline axis in SHP format.

  - The orthophoto map must identify buildings and other objects with an area of 10 m² or larger, provided that their visibility is not obstructed by physical obstacles that the Supplier was unable to remove when creating the orthophoto mosaic. The Supplier must indicate in the report the locations where the identification of objects was limited by visibility obstacles (e.g., tree canopies).

  - In MGP protection zones, oblique images must be additionally captured to allow for a more comprehensive assessment of the height, shape, location, and actual use of objects (for example, the depth of excavation or building facades). Oblique images must be provided in JPG format, linked to the capture location (X, Y, Z coordinates in EXIF metadata), date, and the corresponding object in the dataset. The number of oblique images and the capture interval must be sufficient to reliably identify the changes specified in this technical specification.

  - Aerial photographs must be merged into a single orthophotomosaic covering the entire flight route (both in terms of length and width).  The orthophotomosaic must not contain any image gaps, duplicates, visible seam distortions, or other image distortions that would prevent the identification of the objects specified in this Technical Specification or the analysis of changes.

  - Since the measurement section is linear (corridor), the orthophotographic map shall be provided to the Buyer as a single mosaic in MrSID format and/or divided into logical, continuous route segments in GeoTIFF format, as previously agreed upon by the Supplier and the Buyer.

  - Based on the prepared orthophoto map, the Supplier must analyze changes in activities (actions) carried out in the vicinity of the transmission system and identify them by compiling datasets (in .shp and .xlsx formats):

- changes identified in protection zones (“MD_AZ_FIKS.shp” spatial data set is created in accordance with Table 1 of the technical specifications, and changes are classified according to Table 2 of the Technical Specification);

- changes identified in area class territories (“MD_VK_FIKS.shp” spatial dataset is created in accordance with Table 3 of the technical specifications, and changes are classified according to Table 4 of the Technical Specification). The “MD_VK_FIKS.shp” dataset contains changes identified within the Class of territories that do not fall within the boundaries of the Protection zones and do not overlap with the changes recorded in the “MD_AZ_FIKS.shp” dataset.

  - All changes specified in this technical specification must be recorded in the analyses (reports) by comparing them with the most recent orthophotographic maps prepared by the Supplier and submitted to the Buyer.

  - Each recorded change at a specific geographic location must be assigned a unique ID number. This number must remain the same in subsequent analyses of changes recorded at the same geographic location. Newly detected changes are assigned new, unique numbers, with the numbering continuing from the last one that preceded it.

  - If, during a flight, the Supplier observes hazardous activities taking place directly above the gas pipeline, in such a case, the Supplier must inform the Buyer of such activities no later than within 1 working day of becoming aware of them and provide separate information (without waiting for the submission of the full summary report to the Buyer) regarding the observed activity—including an aerial photograph, coordinates, and a description of the hazardous activities. Examples of hazardous activities above a gas pipeline include: construction, excavation work, storage, and the use of heavy machinery above the MGP.

  - Natural changes in vegetation due to seasonal variations and agricultural activities, such as harvesting, mowing, and plowing, do not need to be identified during the change analysis.

  - The .shp files submitted must be accompanied by fully completed metadata specifying the data collector, the equipment used, the GPS accuracy margin of error, the collection date, and the coordinate system.

Table 1. Changes recorded within the boundaries of protection zones (MD_AZ_FIKS.shp template).

| Area | Type | Definition / rules for completion |
| --- | --- | --- |
| ID | Double (Number) | Unique record identifier. |
| PAZEID_ID | Double (Number) | Classifier code ID (see Table 2 of the Technical Specification, “PAZEID_ID”). |
| PAZEID_TIP | Text | Type of violation (see Table 2 in the Technical Specification, “PAZEID_TIP”). |
| DATA_FIKS | Date | Record date (YYYY-MM-DD). |
| FOTO_REF | Text | The ID or link of the associated photo or video file. |
| PLOTAS_M2 | Double (Number) | Area of the polygon in m² (calculated automatically from the geometry; rounded to the nearest 0.1 m²). |
| PLOTAS_HA | Double (Number) | Area of the polygon in hectare (AREA_M2/10,000; rounded to 0.0001 ha). |
| ATST_MD | Double (Number) | Distance to MGP (m) from the edge of the polygon to the MGP axis; |
| STATUSAS* | Text | New / Approved / Resolved / Incorrect / Permitted activity. |
| DATA_PATVIRTINIMO** | Date | The date on which the violation was confirmed by an AG employee (remotely or via satellite imagery). |
| DATA_ISSPRENDIMO** | Date | The date on which the violation was resolved or rectified. |
| DATA_PERZIUROS** | Date | The date on which a review is scheduled or has been conducted (e.g., after 6 months), for cases where monitoring of the situation is required. |
| PASTABOS* | Text | Notes. |

*The Supplier should only fill in the “STATUS” field for recorded violations with the status “NEW”; the “NOTES” field should not be filled in, as it is intended for the Buyer.

**The Supplier does not fill in these fields.

Table 2. Classification of violation types in protected zones.

| PAZEID_ID | PAZEID_TIP | Definition |
| --- | --- | --- |
| PAZEID_AZ_01 | PASTATAS | Buildings and structures (residential, farm buildings, warehouses, etc.) |
| PAZEID_AZ_02 | SPORT_AISK | Sports, games, and recreation area |
| PAZEID_AZ_03 | KIEM_KONST | Outdoor structures, temporary structures (greenhouses, gazebos, sheds, outdoor toilets, etc.) |
| PAZEID_AZ_04 | SUST_VIETA | Stop area (for buses) |
| PAZEID_AZ_05 | AUTO_AISK | Parking lot |
| PAZEID_AZ_06 | KILN_DAIK | Portable items (campers, tents, etc.) |
| PAZEID_AZ_07 | MEDZ_BOKS | Hunting tower |
| PAZEID_AZ_08 | SAND_VIETA | Storage |
| PAZEID_AZ_09 | INZ_STAT | Engineering structures (ground-mounted structures, pylons, solar farms, wind turbines, communication towers, etc.) |
| PAZEID_AZ_10 | SIUKSL | Trash |
| PAZEID_AZ_11 | SAK_AKM | Branches, stones |
| PAZEID_AZ_12 | STAT_DARB | Construction work (including site preparation and vegetation removal) |
| PAZEID_AZ_13 | ISK_TVENK | Excavations, ponds, ditches |
| PAZEID_AZ_14 | KEL_ILGA | Road (paved structure along the MGP) |
| PAZEID_AZ_15 | PROV_VIET | Ruts |
| PAZEID_AZ_16 | TVORA | Fence |
| PAZEID_AZ_17 | ZELD_TERP | Plants |

Table 3. Changes recorded within the boundaries of local area classes (MD_VK_FIKS.shp template).

| Area | Type | Definition / rules for completion |
| --- | --- | --- |
| ID | Double (Number) | Unique record identifier. |
| PAZEID_ID | Double (Number) | Classifier code ID (see Table 4 of the Technical Specification, “PAZEID_ID”). |
| PAZEID_TIP | Text | Type of violation (see Table 4 in the Technical Specification, “PAZEID_TIP”). |
| DATA_FIKS | Date | Record date (YYYY-MM-DD). |
| FOTO_REF | Text | Linked photo/file ID/link. |
| PLOTAS_M2 | Double (Number) | Area of the polygon in m² (calculated automatically from the geometry; rounded to the nearest 0.1 m²). |
| PLOTAS_HA | Double (Number) | Area of the polygon in hectare (AREA_M2/10,000; rounded to 0.0001 ha). |
| ATST_MD | Double (Number) | Distance to MGP (m) from the edge of the polygon to the MGP axis; |
| STATUSAS* | Text | New / Approved / Resolved / Incorrect / Permitted activity. |
| DATA_PATVIRTINIMO** | Date | The date on which the violation was confirmed by an AG employee (remotely or via satellite imagery). |
| DATA_ISSPRENDIMO** | Date | The date on which the violation was resolved or rectified. |
| DATA_PERZIUROS** |  | The date on which a review is scheduled or has been conducted (e.g., after 6 months), for cases where monitoring of the situation is required. |
| PASTABOS* | Text | Notes. |

*The Supplier should only fill in the “STATUS” field for recorded violations with the status “NEW”; the “NOTES” field should not be filled in, as it is intended for the Buyer.

**The Supplier does not fill in these fields.

Table 4. Classifier of violation types in area classes.

| PAZEID_ID | PAZEID_TIP | Definition |
| --- | --- | --- |
| PAZEID_VK_01 | STATYBA | Any structures (including tent-style or modular structures that have a roof and interior spaces) |
| PAZEID_VK_02 | KASIMAS | Construction or installation work at various stages (foundations, preparatory construction work (significant changes to the ground surface)) |

- Requirements for the SERVICE 2

  - The following accuracy requirements apply to LiDAR data when providing Service 2: the vertical height error (RMSE_z) on hard surfaces (in open areas) must not exceed 5 cm, and under vegetation, it must not exceed 10 cm. The average density of laser reflections in the scanning corridor must be at least 20 points per square meter. This density is required to ensure reliable beam penetration through the tree canopy to the ground surface.

  - All LiDAR data must be georeferenced and provided in the national LKS-94 coordinate system and the LAS07 elevation system.

  - The coverage limits of the point cloud and analysis data shall be no less than 12 m (6 m on each side of the MGP pipeline axis and 1 m from the perimeter of the MGP facility). Prior to the commencement of the service, the Buyer shall provide the Supplier with vector data of the MGP pipeline axis in SHP format.

  - The creation and analysis of point cloud data is not required within areas enclosed by MGP objects. The Buyer shall provide the Supplier with the boundaries of these areas in a separate file in ESRI .shp format.

  - LIDAR data must be georeferenced and classified in accordance with the LAS specification (version 1.4 or higher) or an equivalent specification, with data provided in LAS or LAZ format.

  - LIDAR data must be classified into the following minimum classes in accordance with the ASPRS standard:

    - Class 0 and/or 1 – unclassified sites (Created, never classified; Unclassified);

    - Class 2 – Ground points;

    - Class 3 – Low Vegetation;

    - Class 4 – Medium Vegetation;

    - Class 5 – High Vegetation;

    - Class 6 – Building points;

    - Class 7 – Low Point (noise);

    - Class 12 – Overlap Points.

  - In performing the Agreement, the Supplier may, upon agreement with the Buyer, use additional classes for the classification of LIDAR data.

  - 100 percent of the LIDAR data must be classified; that is, there must be no points to which no class has been assigned. The classes specified in sub-clause 6.6 shall be assigned to the LIDAR data using automated or semi-automated methods.

  - LIDAR data must be saved in LAS format version 1.4 or higher and in the Point Data Record Format 6).   LAS format specifications are published on the website of the Society of Photogrammetry and Remote Sensing http://www.asprs.org. LAS files must contain the spatial X, Y, and Z coordinates of the LIDAR data, defined in the LKS-94 coordinate system and the LAS07 elevation system. Coordinates are given in meters, rounded to two decimal places.

  - The Digital Terrain Model (DTM) and Digital Surface Model (DSM) are provided in GeoTIFF format (or another equivalent format, subject to prior written agreement with the Buyer)

  - Analysis of land surface changes is performed using the point cloud generated during laser scanning (after removing all vegetation and other objects). The identified changes are presented as areal features within the coverage specified in Clause 6.3, as well as point features, i.e., the elevation of the ground surface above the MGP axis, spaced no more than 10 m apart. The data is provided in .shp format.

  - The analysis of vegetation to be removed is performed using the point cloud generated during laser scanning. The identified changes are presented as area and/or point objects within the coverage specified in Clause 6.3.

  - In assessing the vegetation in the vicinity of the transmission system (within the scope of Clause 6.3), the Supplier must:

    - highlight any detected vegetation changes in photographs and reports using a different color, classifying them according to the following criteria: i) permitted vegetation; ii) vegetation to be removed;

    - vegetation to be removed – trees and shrubs growing within the scope specified in Section 6.3, with a trunk diameter greater than 7 cm, measured 5 cm above ground level, and a height of more than 1 m. Vegetation that does not meet the criteria for vegetation to be removed—trunk diameter—is considered non-removable vegetation. This category (Non-removable vegetation) also includes leaning or tilted trees and tree branches that fall within the scope specified in Clause 6.3 or are capable of reaching it; these are assessed separately as vegetation posing a potential risk. The Supplier shall mark such objects in the dataset and indicate their location, approximate height, direction of lean, and proposed action in the report: to be removed, monitored, or not to be removed;

    - The dominant vegetation characteristic is determined based on the largest proportion of that characteristic within the assessed vegetation polygon. The report shall specify the criterion used to determine the dominant feature: area, number of objects, or another method applied by the Supplier and described in the report;

    - the photographs shall show the location of the MGP pipeline, vegetation areas, and the coverage specified in Clause 6.3;

    - determine the dominant tree height;

    - calculate the dominant trunk diameter;

    - identify the dominant tree species;

    - the results of the report must be submitted in the attached .xlsx and .shp files, which must be completed in accordance with the provided template (Table 5 of the Technical Specification).

Table 5. Vegetation assessment template.

| Area | Type | Definition / Completion |
| --- | --- | --- |
| ID | Double (Number) | Unique record identifier |
| X | Double (Number) | Enter the X-coordinate of the center of the designated vegetation area, in meters |
| Y | Double (Number) | Enter the Y-coordinate of the center of the designated vegetation area, in meters |
| AUGMENIJOS_PLOTAS | Double (Number) | Enter the area of the identified vegetation polygon, in hectares. Assess areas of ≥ 1 square meter. If there are mature individual trees, do not fill in this field—fill in the following fields instead. |
| DATA | Date | The date the photo was taken is displayed in the YYYY-MM-DD format |
| AUGMENIJOS_TIPAS | Text | Indicate the dominant vegetation type: 1. trees; 2. shrubs; 3. mixed (where both trees and shrubs are present) |
| AUGMENIJOS_RUSIS | Text | Indicate the types of dominant vegetation: 1. deciduous trees; 2. conifers; 3. xerophytes. |
| KAMIENO_STORIS | Double (Number) | The typical trunk diameter, in centimeters, is indicated |
| AUKSTIS | Double (Number) | The specified dominant height of trees and shrubs must be determined based on the height of the canopy |
| VEIKSMAI | Text | The following actions are specified: 1. vegetation to be removed; 2. vegetation not to be removed. |
| NUORODA | Text | A link to the map is provided |
| PASTABOS | Text | Notes |

  - In assessing the ground surface elevation in the vicinity of the transmission system (as defined in Clause 6.3), the Supplier must:

    - estimate the ground surface elevation based on data collected during the flight, after eliminating all vegetation and other objects;

    - compare the ground surface elevations obtained during the first flight with the MGP top elevation data provided by the Buyer (the Buyer is not obligated to provide the top elevations of all MGP pipelines);

    - comparison of ground elevation data collected during subsequent flights with historical data (including a comparison of MGP depths);

    - the results of the report must be submitted in the attached .xlsx and .shp files, which must be completed in accordance with the provided template (Table 6; Table 7 of the Technical Specification);

    - prepare a map of land surface changes showing areas of topographic changes (uplifts, subsidence), the extent of these changes, and changes in volume;

    - provide a longitudinal profile of the 2D MD pipeline. The profile must indicate the elevations of the ground surface (including any changes) and the top of the MGP, as well as any objects it crosses (water bodies, roads, railways, etc.) (in .pdf and .dwg formats). The profile is generated only in those locations where the Buyer has provided elevation data for the top of the MGP.

Table 6. Template for a report on the estimation of ground elevation.

| Area | Type | Definition / Completion |
| --- | --- | --- |
| ID | Double (Number) | Unique record identifier |
| X | Double (Number) | Enter the X-coordinate of the point, m. Previous aerial surveillance |
| Y | Double (Number) | Specify the Y-coordinate of the point, m. Previous aerial surveillance |
| Z | Double (Number) | Enter the Z-coordinate of the point, m. Previous aerial surveillance |
| DATA | Date | Enter the aerial surveillance date in the YYYY-MM-DD format. Previous aerial surveillance |
| X1 | Double (Number) | Enter the X-coordinate of the point, m. Subsequent aerial surveillance |
| Y1 | Double (Number) | Enter the Y-coordinate of the point, m. Subsequent aerial surveillance |
| Z1 | Double (Number) | Enter the Z-coordinate of the point, m. Subsequent aerial surveillance |
| DATA | Date | Enter the aerial surveillance date in the YYYY-MM-DD format. Subsequent aerial surveillance |
| TIPAS | Double (Number) | According to the LAS classification system |
| PASTABOS | Text | Notes |

Table 7. Template for a report on the estimation of ground surface elevations (above the axis of the MGP).

| Area | Type | Definition / Completion |
| --- | --- | --- |
| ID | Double (Number) | Unique record identifier |
| X | Double (Number) | Enter the X-coordinate of the point, m. |
| Y | Double (Number) | Enter the Y-coordinate of the point, m. |
| Z | Double (Number) | Enter the Z-coordinate of the point, m. |
| MD_GYLIS | Double (Number) | The calculated depth of the MGP is specified. This data is entered only if the Buyer has provided the elevation of the top of the MGP. |
| DATA | Date | Enter the aerial surveillance date in the YYYY-MM-DD format |
| TIPAS | Double (Number) | According to the LAS classification system |
| PASTABOS | Text | Notes |

- General requirements for data security and the provision of Services

  - Since the Supplier will be provided with information regarding facilities and assets critical to national security, which may pose a risk or threat to national security, the Supplier, its subcontractors, entities whose capabilities are relied upon, manufacturers, persons performing maintenance and support of hardware or software, or persons controlling them, must comply with the national security requirements set forth in the Law on the Protection of Objects of Importance to Ensuring National Security of the Republic of Lithuania and the Law on Public Procurement of the Republic of Lithuania (Article 92, paragraphs 14–15 of the Law on PP).

  - The provision of the Services must comply with the Law on Cyber Security of the Republic of Lithuania and the Description of Organizational and Technical Cyber Security Requirements Applicable to Cyber Security Entities, approved by Resolution No. 818 of the Government of the Republic of Lithuania on 13 August 2018 (current version with subsequent amendments) https://www.e-tar.lt/portal/lt/legalAct/aea15050a53411e8acb39f2e6db7935b/asr.

  - The green requirements applicable to the Procurement object are set forth in Clause 4.3. of Order No. D1-508 of the Minister of the Environment of the Republic of Lithuania dated 28 June 2011 “On the Approval of the Procedure for the Application of Environmental Protection Criteria in Green Procurement” (current version): During the performance of the Agreement, the Supplier must implement the EN ISO 14001 environmental management system standard or an equivalent standard. At the Buyer’s request, the Supplier must provide documentation demonstrating that the measures taken comply with this requirement.

  - Requirements for data centers:

    - The Supplier's online storage (or equivalent location) must be housed in secure data centers located within the EEA (European Economic Area) or in countries that comply with the Law on the Protection of Objects of Importance to Ensuring National Security of the Republic of Lithuania and the Law on Public Procurement of the Republic of Lithuania.  System data must not be transferred outside this area.

    - The data center must meet the "Tier 3" standard defined by the Uptime Institute or an equivalent level of data center reliability. Equivalence may be demonstrated by a certificate, audit report, or other document confirming that the data center meets at least the same level of reliability, availability, redundancy, and business continuity. The online storage and/or System must operate in cloud data centers of suppliers certified to ISO 27001 or an equivalent standard for information security.

  - The Supplier must ensure that only authorized and licensed hardware and software are used in the provision of the Services.

  - “Permitted” software:

    - must have valid licenses that correspond to the intended use and scope;

    - does not originate from countries that, under the applicable laws of the Republic of Lithuania, may pose a threat to national security, as defined in Article 37(9) and Article 47(9) of the Law on Public Procurement;

    - agreed upon with the Buyer. The Buyer may refuse to accept the software only for valid reasons, if it does not comply with licensing, information security, national security, or the technical requirements set forth in this technical specification.

  - Encryption requirements:

    - Information must be stored in an encrypted repository;

    - encryption keys must be used. The security of encryption keys must be ensured. The length of encryption keys must be at least 256 bits;

    - end-to-end encryption must be ensured, i.e., data must be encrypted during transmission between the Service and users’ computers, as well as (if applicable) in databases, servers, and backups;

    - sensitive and/or confidential information must be protected in such a way that it cannot be decrypted or accessed from outside, i.e., without proper authorization and access rights. Even if everything is encrypted, it must be ensured that internal access controls restrict access to information of varying sensitivity, and that encryption keys and mechanisms are not accessible to unauthorized users or third parties;

  - Requirements for logging:

    - audit information regarding data operations (logs) must be collected and retained for at least 6 months. Information regarding data operations, the users who performed the operations, and the dates and times of the operations must be retained;

    - Authorized users must be able to easily view the information contained in specific audit records (both on-screen and in reports);

    - administrators must not be able to delete or edit administrator log entries;

    - audit logs must not be edited or otherwise altered;

    - The following logging records must be collected (this list is not exhaustive; the exact list is subject to agreement):

      - User identifier;

      - Event times;

      - Information about the connecting computer, including the source IP address;

      - Records of successful and unsuccessful login attempts;

      - Use of administrator privileges;

      - Changes to access rights;

      - Configuration changes;

      - Changes to network parameters, time, and/or date;

      - Enabling/disabling of event logging;

      - Deleting, creating, and/or modifying events;

      - Resources to which access was granted;

      - System and security notifications;

      - Information about user actions

  - The Service provider's employees' work devices must have a valid antivirus system installed and running that complies with international security standards. Antivirus software must not originate from countries whose manufacturers or products, under applicable laws, may pose a threat to national security, as defined in the provisions of Article 37(9) and Article 47(9) of the Law on Public Procurement of the Republic of Lithuania;

  - All data collected about the facility during the flight must be processed exclusively within the European Union or in countries that comply with the Law on the Protection of Objects of Importance to Ensuring National Security of the Republic of Lithuania and the Law on Public Procurement of the Republic of Lithuania. All data collected must remain confidential. The data must be protected against unauthorized access or use.

  - The Supplier undertakes not to disclose the data it manages, not to transfer it to third parties, and to ensure its high-quality protection.

  - The Supplier undertakes to ensure that all archived information is protected from third parties during the term of the contract and, upon the Buyer’s written request after the contract expires, is transferred to the Buyer free of charge and securely in an agreed format.

  - The Buyer is the owner of all collected data. The Supplier undertakes, at the Buyer’s request (or in the event of termination of the Agreement) to export and transfer all data accumulated during the entire period of service provision (orthophotos, datasets, reports, etc.) in a format agreed upon with the Buyer (.xls, .csv, etc.).

  - The Supplier undertakes, no later than within 15 calendar days of receiving a written request from the Buyer, to irrevocably destroy all data provided by the Buyer, as well as data collected using the Supplier’s resources regarding the Buyer’s facilities. The Supplier must submit a signed official letter confirming the transfer and destruction of the data no later than 5 calendar days after the data has been destroyed.

  - The Supplier must immediately notify the Buyer of any possible cases of data misappropriation, leakage, or cybersecurity incidents within the Supplier’s Infrastructure and take all possible measures to manage the situation.

  - Cyber security incident management:

    - The Supplier must immediately notify the Buyer of any cyber security incident that may affect the Buyer’s data or information resources, and must provide early warning to the Buyer no later than within 24 hours.

    - No later than 72 hours after becoming aware of the incident, the Supplier shall submit a detailed report specifying:

      - The nature and scope of the incident;

      - An assessment of the impact;

      - The initial measures taken.

    - The Supplier shall submit interim and final reports at the Buyer’s request.

    - Communication channels: Reports shall be submitted to the Buyer's employees responsible for the performance of the Agreement or by e-mail to cyber@ambergrid.lt. The information must be authentic and, if requested, encrypted.

  - The Services must not pose a threat to national security, as specified in the Procurement documents.

  - The Buyer shall have the right, during the term of this Agreement, to inspect the organizational and technical information security measures implemented by the Supplier, to the extent that such measures relate to the performance of this Agreement and the processing of the Buyer’s data. The Buyer shall notify the Supplier of a scheduled inspection no later than 5 working days in advance. The inspection shall be conducted without disclosing the Supplier’s trade secrets, other clients’ data, or any other information unrelated to the performance of the Agreement.

SCOPE OF SERVICES AND PRELIMINARY VOLUMES FOR A 36-MONTH PERIOD

| No. | Object | Unit of measurement | Estimated quantity for 36-months* |
| --- | --- | --- | --- |
| 1. | Aerial photography services over the area monitored by the gas transmission system | km | 13,728 |
| 2. | Aerial photography and LiDAR point cloud generation services for the area monitored by the gas transmission system. | km | 6,864 |
| 3. | Service 1 – Identification of changes in ongoing activities (operations), preparation of spatial data, and submission of reports based on aerial photographs of the gas transmission system (pipeline network) (as specified in Clause 5 of the Technical Specification). | Report | 6 |
| 4. | Service 2 – analysis of vegetation assessment for removal, preparation of spatial data, and submission of reports based on the collected LiDAR data (as specified in Clauses 6.12–6.13.8 of Section 6 of the Technical Specification). | Report | 3 |
| 5. | Service 2 – analysis of ground elevation estimates, preparation of spatial data, and submission of reports based on the collected LiDAR data (as specified in Clauses 6.14–6.14.6 of Section 6 of the Technical Specification). | Report | 3 |

* The preliminary quantity of the Procurement object is specified. The Contracting entity does not undertake to purchase the entire specified preliminary quantity. The Buyer shall purchase the Services as needed throughout the term of the Agreement up to the maximum value of the Agreement.