Appendix 2

TECHNICAL SPECIFICATIONS FOR THE REMOTE OPERATING VEHICLE (ROV)

| No. | Technical characteristic | Requirements | The parameters of the equipment offered by the supplier and their values according to the requirements of columns 2 and 3. The exact value of the parameter must be specified. For the requirement “compulsory” must be indicated “Complies/Not complies” |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| 1 | Weight | < 35 kg |  |
| 2 | Tether | ≥ 300 m; strong enough for ROV to be deployed/recovered on it |  |
| 3 | Power | ROV is powered by batteries. ROV must have an option to be powered by the direct power as a future upgrade. |  |
| 4 | Batteries | Battery life of a single set is 4-6 hours (under normal ROV operation). Two sets of batteries must be included. Two sets of battery chargers must be included. |  |
| 5 | Maneuverability | ROV speed up to 3.5 knots, position holding against current up to 2 knots |  |
| 6 | Control modes | Up/down, left/right, lateral movements. Automatic station holding, heading holding, depth holding, altitude holding and route following |  |
| 7 | Sensors | Surface GPS, MRU, gyro, DVL, laser scaler |  |
| 8 | Doppler Velocity Log (DVL) | Bottom detection range from 0.05 to 125 m |  |
| 9 | Navigation | Inertial navigation from initial surface GPS position with DVL aiding (dead reckoning) |  |
| 10 | Lights | ≥ 7000 lumens |  |
| 11 | Camera | 4K camera, still shot capability, video logging into internal camera storage and on external storage, raw camera footage and navigation screen logging |  |
| 12 | Camera and instrument position | Camera, lights, laser scaler on tilted platform with at least 180° movements. When tilted directly down, camera is in the lowest point of ROV and have unobstructed view |  |
| 13 | Compatibility with other equipment | System is compatible and ready to be expanded with imaging sonar, hydrological multi-probe, cathodic protection probe. |  |
| 14 | Warranty | ≥ 2 years |  |