# ADS-B EXTENDED SQUITTER NON-TRANSPONDER DEVICE FOR GROUND VEHICLE SURVEILLANCE

TECHNICAL SPECIFICATION

- DEFINITIONS

- Definitions used in the technical specification:

Supplier – The participant whose bid for the procurement will be recognized as the winner in accordance with the procedure established in the procurement conditions and with whom the Contract will be concluded.

Purchaser – AB „Oro navigacija“.

Contract – A contract for the purchase and sale of goods, signed between the Suppler and the Purchaser.

Product – A non-transponder device, used for ground vehicle surveillance.

- THE OBJECT OF PURCHASE

- 10 units of devices designed to transmit position of vehicles (ADS-B extended "squitter") are being purchased, which are necessary to ensure safe ground vehicle traffic in the airport territory and the manoeuvring area, of which:

  - 4 units have a built-in battery;

  - 6 units do not have a built-in battery.

- DESCRIPTION OF GOODS AND SCOPE OF PURCHASE

- Minimum technical requirements for the Goods and the quantity of Goods purchased are specified in Point 2 of this technical specification.

- The goods must be new, unused.

- The Goods must be fit for their intended purpose, without any hidden defects in the Goods that would prevent the Goods from being used for their intended purpose or that would reduce the usefulness of the Goods.

- The Goods must comply with the standards in force in the Republic of Lithuania and other requirements for such Goods set out in legal acts.

Table 1. Minimum technical requirements for a vehicle ground position surveillance device

| 1. | General requirements: |
| --- | --- |
| 1.1 | Transferred information: ADS-B 1090MHz extended „squitter“ (Broadcast of automatic dependent surveillance monitoring information) |
| 1.2 | The devices are adapted for use at the airport territory and its manoeuvring area. |
| 1.3 | The unit should have an external, fixed by magnet or an internal antenna |
| 1.4 | The Purchaser shall have a possibility to program new and change the current ICAO codes and callsigns using a computer program. Programming should not require specific IT knowledge. |
| 1.5 | The manufacturer must not originate from territories deemed unreliable under the laws of the Republic of Lithuania: (1) the Russian Federation; (2) the Republic of Belarus; (3) the People’s Republic of China (except for the Taiwan Province); (4) Crimea annexed by the Russian Federation; (5) the territory of Transnistria not controlled by the Government of the Republic of Moldova; (6) the territories of Abkhazia and South Ossetia not controlled by the Government of Georgia. The offered equipment must not pose a threat to the national security and state interests of the Republic of Lithuania. |
| 2. | DF18 transmitter „Squitter“ requirements: |
| 2.1 | The message content must be transmitted in DF18 format. |
| 2.2 | Squitter repetition frequency shall be ≥1Hz, should have a possibility to configure the frequency. |
| 2.3 | ICAO address field. |
| 2.4 | Vehicle identification subfield. |
| 2.5 | Vehicle category. |
| 2.6 | Coordinates, position of a vehicle to be reported. |
| 2.7 | Ground speed of the vehicle to be reported. |
| 2.8 | Turn on (activation) function |
| 3. | Performance compliance: |
| 3.1 | DF18 registries squitter pulse duration according to the requirements of ICAO Annex 10, volume 4. |
| 3.2 | DF18 registries squitter pulse rise and fall time according to requirements of ICAO Annex 10, volume 4. |
| 3.3 | Frequency spectrum of the unit will be according to the requirements of ICAO Annex 10, volume 4. |
| 4. | Transmitter: |
| 4.1 | Adapted for operation from 9 V to 32 V voltage, shall have a standard connector for the automobile auxiliary power outlet and included extension peace |
| 4.2 | Carrier frequency: 1090 MHz ± 0,5 MHz |
| 4.3 | Peak power: ≥ 10 W. |
| 5. | Physical environmental specification: |
| 5.1 | Operating temperature within a temperature range of at least: from -40°C to +60°C |
| 5.2 | Shall operate in environment with air humidity up to 95% |

- PROCEDURE FOR DELIVERY OF GOODS

- The goods must be delivered no later than 30 days from the date of entry into force of the Agreement.

- The goods must be delivered to the following address: AB "Oro navigacija", Balio Karvelio g. 25, LT-02184 Vilnius. The goods must be delivered only on working days (Monday–Thursday 07:30–16:15, Friday 07:30–15:00).

- The goods must be supplied in packaging that protects them from mechanical damage and the effects of atmospheric precipitation during transportation.

- MARKING AND LABELLING

- The Goods must have factory marking (labels or inscriptions), which must indicate the following information:

  - manufacturer of the goods (model, serial or factory number);

  - date of manufacture/size/ dimensions/ net/ gross weight;

  - protection level/ resistance level/ safety category;

  - electrical characteristics (rated supply voltage, maximum current consumption or maximum power consumption);

  - CE mark.

- All parts of the offered Goods must be provided by the same manufacturer and marked with its trademark or this must be mentioned in the technical documentation.

- WARRANTY OBLIGATIONS

- The goods must be provided with a warranty period of at least 3 years (36 months), calculated from the date of signing the Goods Acceptance Certificate.

- During the warranty period, defects in the Goods must be repaired free of charge or the defective Goods replaced with new ones (including the costs of transporting the Goods or their parts). The term "defect" includes any improper operation and/or malfunction of the Goods (or any part thereof, including software).

- If updates to the Goods and/or accompanying software occur during the warranty period, the Supplier undertakes to provide these updates free of charge.

- During the warranty period, the functioning of the defective Goods must be fully restored or the defective Goods must be replaced with an equivalent new product, no later than 30 business days from the date of sending the notification to the Supplier.

- If the Supplier fails to timely eliminate defects in the Goods or replace defective Goods with new ones during the warranty period, the Purchaser has the right to eliminate defects that prevent the Goods from being used for their intended purpose by itself or by using third parties, and the Supplier undertakes to reimburse the Buyer for the costs of eliminating defects incurred by the Buyer.

- The Supplier must ensure that during the warranty period the repair of the Goods will be performed by the Manufacturer of the Goods or specialists certified by the Manufacturer, and provide documents confirming this (for example, a contract with a certified service center of the equipment manufacturer, etc.).

- DOCUMENTS

- The Supplier must submit, together with the offer, documents proving the compliance of the technical characteristics of the proposed Goods with the requirements of the technical specification specified in Table 1 of this technical specification.

- Together with the Goods, the Supplier must provide the Buyer with instructions for using the Goods and software (if any) in Lithuanian or English;

  - A product packaging sheet with a list of components, dimensions (overall) and weight of the product packaging;

  - A list of spare and normal wear parts for the offered Goods with order or manufacturer numbers.

VIII. ENVIRONMENTAL REQUIREMENTS

- In accordance with Sub-clause 4.4.4.4 of the Description of the Procedure for the Application of Environmental Protection Criteria in Green Public Procurement, approved by Order No. D1-508 of the Minister of Environment of the Republic of Lithuania of 28 June 2011, the following environmental criterion is established: the product is durable, long-lasting and functional; it or its components are suitable for repeated use and/or are easily repairable and/or replaceable. By submitting a tender, the Supplier declares compliance with this environmental (green) criterion.