## TECHNICAL SPECIFICATION

PART I. DESCRIPTION OF THE PURCHASE OBJECT

## DEFINITIONS

Customer means UAB “LTG Link”.

Service Provider means an economic operator – a natural person, a private legal entity, a public legal entity, other organizations and their divisions, or a group of such persons with whom the Customer enters into a Contract.

Services means routine ER-3 and major KR repairs of the bogies of EJ575 electric trains

Contract means a contract concluded between the Service Provider and the Client regarding the subject matter of the Purchase.

Parts / Spare Parts / Goods means parts used during the routine ER-3 and major KR repairs of the EJ575 electric train bogies.

## SUBJECT OF THE PURCHASE

  - Routine ER-3 and major KR repairs of the traction bogies of EJ575 electric trains (hereinafter referred to as the “Subject of the Procurement”).

  - The procurement is not divided into parts of the procurement object.

- REQUIREMENTS FOR THE SUBJECT OF THE PURCHASE

| No. | Compliance with the characteristics, parameters, or functions of the goods, device, or equipment | Compliance with the required parameter or function |
| --- | --- | --- |

| Routine ER-3 repair of EJ575 train bogies |  |  |
| --- | --- | --- |
|  | Technical requirements for services | The repair must comply with the technical requirements specified in items 3.10–3.16 of the table. |
|  | Scope of services | Specified in Appendix 1 of the Proposal Form at. |
|  | Description of services | Repair of components of the traction bogies of electric train EJ575 (axles, axle boxes, traction gearboxes and electric motors, brake cylinders and discs, etc.) shall be performed in accordance with the relevant maintenance and repair instructions of the manufacturers of the EJ575 electric trains and their components; |
|  | Requirements for fasteners, seals, and rubber components | When performing the Services, all fasteners, seals, rubber parts, compressed air hose fittings, protective corrugated sleeves for electrical cables and their connections, and silent blocks shall be replaced with new ones; |
|  | Requirements for balancing repaired wheels | After repair and replacement of components, the wheels must undergo dynamic balancing; the maximum permissible wheel imbalance shall not exceed 75 gm; |
|  | Testing of repaired equipment | The repaired equipment is tested in accordance with the technical requirements specified in items 3.10–3.16 of the table, and test reports are submitted; |
|  | Inspection Marking and Stamping | The bogies, axles, and other equipment of electric train EJ575 must be stamped/marked with inspection and repair stamps and markings in accordance with the requirements of the manufacturer and regulatory documentation. Stamps and markings must be clearly visible and legible, allowing for the identification of the company that performed the service and the determination of the repair date; |
|  | Painting | The painted surfaces of the bogies and their equipment on electric train EJ575 must be painted in the specified colour (base colour RAL 7011); |
|  | Preparation for transport and storage | The bogies (and their components) of electric trains EJ575 must be prepared for transport and storage in accordance with the relevant requirements. Their inlet surfaces, piping, and flanged connections must be protected from external influences (covered with technical plugs, caps, etc.); |
| Requirements set forth in legislation, standards, and the Customer’s internal regulations for the subject of the purchase |  |  |
|  | Industrial design and compliance with industrial standards. | Operating Manual for EJ575 Electric Trains |
|  |  | Maintenance and Repair Manual for the Traction Bogies of Motor Car 211 of the EJ575 Electric Train Lo 2008-14 |
|  |  | Maintenance manual No. 22-189 for the Ø920 wheels with brake discs installed on motor car 211 of the EJ575 electric train; |
|  |  | Drawing No. 455.8.117.000.40 of the Ø920 wheel mounted on motor car 211 of electric train EJ575 |
|  |  | Maintenance (service) manual EdP0450m for electric motor MLU 4245 K/6 |
|  |  | Maintenance manual ZB 1278 for DAKO braking devices on EJ575 electric trains |
|  |  | Instruction manual for the shaping, repair, and maintenance of axles for 1520 mm gauge traction rolling stock T/108 |
|  | Green criteria applicable to the procurement object | The procurement is carried out in accordance with Paragraph 4.4.4.3. of the Order No. D1‑508 of the Minister of Environment of the Republic of Lithuania of 28 June 2011 “On the Approval of the Description of the Procedure for the Application of Environmental Protection Criteria in Green Public Procurement” (as amended by Order No. D1‑401 of 13 December 2022). The Contracting Authority independently establishes environmental protection criteria related to the subject-matter of the procurement by applying at least one of the environmental principles set out therein at one, several, or all stages of the product life cycle. The Service Provider shall ensure that, during the performance of the Contract, the bogies are collected and delivered outside peak hours, i.e., the collection/delivery time shall fall within 9:30–12:00 or 12:45–14:00 (Monday–Friday), with the aim of reducing traffic congestion, fuel consumption, and the associated greenhouse gas emissions. |
| Overhaul of EJ575 electric train traction bogies |  |  |
| - 3.18. | Technical requirements for services | The repair must comply with the technical requirements specified in Table No. 3.28–3.34. |
| - 3.19. | Scope of Services | Specified in Appendix 1 of the Proposal Form. |
|  | Description of the service work | Repair of the auxiliary equipment of the EJ575 electric train’s traction bogies (axles, axle boxes, traction motors and electric motors, brake cylinders and discs, etc.) shall be performed in accordance with the relevant maintenance and repair instructions of the manufacturers of the EJ575 electric trains and their components |
|  | Requirements for axle boxes | When performing repairs on bogie axles KR, all their bearings, axle wheels, and brake discs are replaced with new ones |
|  | Requirements for fasteners, seals, and rubber components | When performing the Services, all fasteners, seals, rubber parts, compressed air hoses, electrical cable protective sleeves and their connections, and silent blocks are replaced with new ones; |
|  | Requirements for balancing repaired wheels | After repair and replacement of components, the wheels must undergo dynamic balancing; the maximum permissible wheel imbalance is no more than 75 gm; |
|  | Testing of repaired equipment | The repaired equipment is tested in accordance with the technical requirements specified in items 3.28–3.34 of the table, and test reports are submitted |
|  | Inspection Marking and Stamping | The bogies, axles, and other equipment of electric train EJ575 must be stamped/marked with inspection and repair stamps and markings in accordance with the requirements of the manufacturer and regulatory documentation. Stamps and markings must be clearly visible and legible, allowing for the identification of the company that performed the service and the determination of the repair date |
|  | Painting | The painted surfaces of the bogies and their equipment on electric train EJ575 must be painted in the specified colour (base colour RAL 7011); |
|  | Preparation for transport and storage | The bogies (and their components) of the EJ575 electric trains must be prepared for transport and storage in accordance with the relevant requirements. Their inlet surfaces, piping, and flanged connections must be protected from external influences (covered with technical plugs, caps, etc.); |
| Requirements set forth in legislation, standards, and the Customer’s internal regulations for the subject of the purchase |  |  |
|  | Industrial design and compliance with industrial standards. | Operating Manual for EJ575 Electric Trains |
|  |  | Maintenance and Repair Manual for the Traction Bogies of Motor Car 211 of the EJ575 Electric Train Lo 2008-14 |
|  |  | Maintenance manual No. 22-189 for the Ø920 wheels with brake discs installed on motor car 211 of the EJ575 electric train; |
|  |  | Drawing No. 455.8.117.000.40 of the Ø920 wheel mounted on motor car 211 of electric train EJ575 |
|  |  | Maintenance (service) manual EdP0450m for electric motor MLU 4245 K/6 |
|  |  | Maintenance manual ZB 1278 for DAKO braking devices on EJ575 electric trains |
|  |  | Instruction manual for the shaping, repair, and maintenance of axles for 1520 mm gauge traction rolling stock T/108 |

- DESCRIPTION OF THE PURCHASE OBJECT

  - New original or equivalent parts must be used to perform the services, and their characteristics (technical parameters, service life, and quality) must comply with the technical requirements specified in items 3.10–3.16 and 3.28–3.34. All rubber parts must have been manufactured no earlier than 12 months ago;

  - When inspecting the axles of the bogies without replacing components, a comprehensive (factory) inspection of the axle box bearings shall be performed;

  - During the performance of the Services, no modernization of the objects being repaired may be carried out without the Customer’s approval;

  - If equivalent parts and assemblies are used to perform the services, the Service Provider must coordinate this with the Customer and submit supporting documents, as well as written confirmation from the manufacturer of the EJ575 electric trains or the equipment that the proposed equivalent goods meet the technical manufacturing requirements and are suitable for their repair;

  - The Service Provider must agree with the Customer on the price of any additional work or parts identified during the provision of the Services by submitting a cost estimate/proposal for such additional work or parts (Special Terms and Conditions of the Contract, Clause 5.4).

  - The Customer shall decide on the feasibility of performing additional repair services based on the conclusions of the equipment condition assessment provided by the Service Provider.

  - The Service Provider shall organize and carry out, at its own expense and using its own resources, the transportation of the EJ575 electric train bogies to and from the repair facility (including packaging, loading, securing, etc.) to the Customer’s location (address: Pramonės st. 78, Vilnius) as well as their preparation.

  - Repair services for the traction bogies of the EJ575 electric train may be performed by their manufacturer or by companies certified (accredited, licensed, or authorized) by the manufacturer for the repair of such equipment, or by companies certified (certified, licensed) for such work and the repair of similar equipment.

  - The Service Provider must provide a 24-month warranty period for the Services provided, calculated from the date of commencement of operation of the repaired bogies; however, for individual
components of the repaired bogies:

- the strength of the axle element connections—not less than 24 months until re-pressing, replacement of axle elements, or a displacement test;

- axles – not less than 24 months;

- for axle end pieces – until replacement of the axle elements;

- axles for new wheels, based on material quality, for the entire service life until wear;

- for new springs, axle boxes, and the housing of the traction reducer on bogies—up to 24 months.

- DOCUMENTS TO BE PROVIDED DURING THE PERFORMANCE OF THE CONTRACT

| No. | Title | Requirements for content and form | Submission Date |
| --- | --- | --- | --- |
| 5.1. | Service Acceptance and Transfer Certificate | Provided in paper or electronic form, in Lithuanian or English. | Upon completion of each Service |
| 5.2. | Protocols issued by the Service Provider for the Services performed (certificates, declarations, or technical passports with notes on the repair work performed) | Submitted in paper or electronic form. Documents must be submitted in Lithuanian or English. If the original document is in another language, the original document and a translation of this document into Lithuanian or English must be submitted. | Upon completion of each Service |
| 5.3. | Reports on measurements and tests performed (certificates, records) | Submitted in paper or electronic form. Documents must be submitted in Lithuanian or English. If the original document is in another language, the original document and a translation of this document into Lithuanian or English must be submitted. | Upon completion of each Service |

PART II. PERFORMANCE OF OBLIGATIONS

- PLACE(S) OF PERFORMANCE OF OBLIGATIONS

☒ At the Service Provider’s registered office.

☒ Pramonės St. 78, Vilnius

- PROCEDURE AND TERMS FOR PERFORMANCE OF OBLIGATIONS

  - Orders:

    - Order fulfillment deadlines

The services must be provided within the timeframe specified in the Service Provider’s proposal, “Appendix 1 to the Proposal Form,” calculated from the date of the Service Provider’s confirmation (via email) of receipt of the Client’s order. The deadline proposed by the Service provider may not exceed: KR – 90 calendar days; ER-3 – 75 calendar days.

    - Order Submission Procedure

☒ By email

    - Order fulfillment procedure

2.1.3.1. The Service Provider must confirm (via email) receipt of the order and the start of its fulfillment no later than within 5 (five) calendar days from the date the Customer’s order is sent (via email) to the Service Provider.

2.1.3.2. The Service Provider shall load and unload the bogies, transport them for repair, and deliver the repaired bogies to the address specified in Section II, Item 1 of the Technical Specifications during the Customer’s business hours (Mon–Thu 8:00 a.m.–5:00 p.m., Fri 8:00 a.m.–3:45 p.m.).

2.1.3.3. The Service Provider shall not be entitled, during the performance of the Contract, to use parts and/or provide services that do not comply with the requirements of the Procurement Documents and/or whose provision is restricted due to international sanctions (as defined in the Law on International Sanctions of the Republic of Lithuania) and (or) due to a threat to national security, as defined in the Procurement Documents and the Law of the Republic of Lithuania on Procurement by Contracting Entities in the Fields of Water Management, Energy, Transport, and Postal Services.

2.1.3.4. The Service Provider, having performed a defect inspection and determined the scope of parts to be replaced, must inform the Customer in writing (via email) of the parts used during the provision of the Services (quantity, name, drawing / catalog number), their country of origin, and manufacturer (its name, legal entity code, country of registration). This applies to goods not listed in Appendix 1 of the Proposal Form. The provision of this information is included in the order fulfillment deadline.

2.1.3.5. The Service Provider shall provide services requiring spare parts only upon receipt of the Customer’s written confirmation of the suitability of the spare parts to be used during the planned repair.

    - Minimum order size/quantity (volume)

The minimum quantity of bogies to be ordered for repair is 2 units.

  - Procedure for signing the acceptance-transfer certificate

The service acceptance and transfer certificate shall be signed within 5 (five) business days of the unloading and transfer of the repaired equipment to the Customer.

  - Procedure and deadlines for rectifying defects

If, at the time of the handover of the Services or during the warranty period, it becomes apparent that the Services provided by the Service Provider are of poor quality, the Service Provider must remedy the defects no later than within 15 (fifteen) calendar days from the date of receipt of the notification (including notification by email) regarding the defects in the Services.