TECHNICAL SPECIFICATION

- SUBJECT AND SCOPE OF PROCUREMENT:

  - The subject of the procurement is an RGB laser video wall, including its installation works and maintenance.

  - The quantities of goods / scope of services are specified in the table below:

Table No. 1

| No. | Name of Goods / Services | Unit of Measure | Quantity |
| --- | --- | --- | --- |
| Stage I |  |  |  |
| 1. | Dismantling an existing video wall | Set | 1 |
| 2. | Installation of a new RGB laser video wall | Set | 1 |
| 3. | Integration and commissioning | Set | 1 |
| Stage II |  |  |  |
| 4. | Maintenance Services | Month | 60 |
| 5. | Parts and service option for unplanned non-warranty failures* | % | Up to 30% of the value of maintenance services |

* The Contracting Authority must be given the opportunity to purchase the Goods/Services specified in Row No. 5, but the Contracting Authority does not undertake to purchase all or part of this option. The Contracting Authority may apply 30% of the value of the maintenance services contract for the purchase of these Goods/Services.

- ADDRESS AND TERMS OF DELIVERY OF GOODS / PROCEDURE AND TERMS OF SUPPLY OF SERVICES:

  - Address for the delivery of goods and provision of services: Karlo Gustavo Emilio Manerheimo St. 8, Vilnius, Lithuania.

  - Terms of provision of services for the Goods and Services specified in Table No. 1:

    - Stage I Item No. 1, 2, 3 must be implemented no later than within 34 weeks from the date of entry into force of the Contract.

    - Stage II Item No. 4 maintenance services for 60 months from the date of system integration and commissioning. 
The Contracting Authority may, but does not undertake, purchase any part of them during the entire period of validity of the contract.

  - The delivery of goods must be organized in such a way that the equipment is delivered and installed only immediately before the installation work. Delivery of the Goods (equipment) earlier than before the moment of stopping the work of the System Control Centre, repair works and replacement is not possible, as the Contracting Authority does not have the opportunity to store the equipment.

  - Works can be organized and executed 24 (twenty-four) hours a day and 7 (seven) days a week, after coordinating their execution with the Contracting Authority.

- SCOPE OF SERVICE AND PRICING MODEL (information will be transferred to the terms of the contract)

  - The procurement consists of 5 separate parts of the service (Table No. 1), which are subject to mixed pricing:

- For goods or services listed in Table 1 from 1 to 4, the principle of a fixed price must apply.

- Parts and Services Option for Unplanned Non-Warranty Failures – Pricing Applicable Reimbursement of Contract Execution Costs Non-Binding Purchase Option Pricing Model, setting the maximum possible amount that the Contracting Authority does not undertake to use.

  - The payment procedure is specified in the Contract and is supplemented by the following clauses:

    - Payment must be made for the properly and timely delivery of the Goods and/or Services provided in accordance with each row of Table No. 1 separately, in accordance with the prices provided in the Supplier's offer.

    - Payment for 1 object of purchase (dismantling of the existing video wall) is made after the completion of the works and after the Contracting Authority confirms their proper execution.

    - Payment for Purchase Object 2 (video wall equipment) is made after delivery of the equipment to the Contracting Authority's object and its acceptance.

    - Payment for procurement object 3 (integration and commissioning) is made after the system has been installed, tested (SAT) and signed the final acceptance-transfer act.

    - Payment for maintenance services is carried out periodically, no more than once a quarter, according to the services actually provided.

    - Payment for the parts and services option for unplanned non-warranty faults is carried out according to the services actually provided and the parts used, up to the maximum amount set out in the contract.

    - Payment must be made under the conditions specified in the contract (within 30 calendar days from the date of submission of the invoice and after the Contracting Authority confirms the acceptance of the services or goods).

- Description of the current situation

4.1. Currently, the System Control Centre operates a video wall of the manufacturer "Leyard", consisting of 12×3 rear projection cubes (a total of 36 pieces, 67", 4:3 format), the total dimensions of which are about 16.3 m wide and 3.1 m high, and the total image area is about 51 m². The video wall has a curved shape (provided in Annexes 3, 4 and 5 of the technical specification).

The video wall has been used for more than 10 years for continuous 24/7 monitoring of the SCADA system, the status of the power grid and other parameters of the transmission network, but due to the long period of operation and the spare parts that are no longer supplied, the decision was made to replace it with a new one.

- SYSTEM INSTALLATION REQUIREMENTS

  - Dismantling an existing video wall

    - The Supplier must dismantle all existing video wall systems, including but not limited to: video wall cubes, mounting structures, controls, power and signal cables, power supplies and other system-related components.

    - During the dismantling and installation works, the System Control Centre will not operate. Installation of temporary or partial imaging solutions is not required.

    - The Supplier must remove all dismantled equipment and ensure its disposal in accordance with the applicable requirements for the management of electronic waste.

    - During the performance of contractual obligations, the Supplier must submit to the responsible person specified by the Contracting Authority documents confirming the transfer of the dismantled equipment for disposal to a licensed waste manager.

  - Installation of a new RGB laser video wall

    - The Supplier must install a new RGB laser rear projection (RPC - Rear-Projection Cube) system.

    - The projected video wall is 40 cubes (10 × 4 configuration), the total image area is about 54 m².

    - The video wall must have a curved shape, maintaining the existing architectural logic of the room. The current radius of curvature, the structure of the frame and the dimensions of the room are given in Annexes 3, 4 and 5 to the technical specification.

    - The Supplier must install a new video wall mounting structure. Reuse of the existing structure is not foreseen.

    - The video wall solution must provide rear access necessary for the operation and maintenance of RPC technology (if required by system design).

    - The Supplier must carry out all additional construction and finishing works necessary for the installation of the video wall, including drywall and frame cutting works necessary to increase the height of the video wall. After the works have been completed, the places of connection of the video wall with the walls of the premises must be completely completed, without visible cracks, rough edges or structural irregularities, and the condition of the decoration of the premises must not be deteriorated compared to the condition before the start of the works.

    - There are no restrictions on the load on the structure. The requirements for the depth of the structure and the current situation are set out in Annexes 3, 4 and 5 to the technical specification.

    - After the installation work, the premises must be cleaned: vacuumed, washed so that there are no traces of construction work.

  - Integration and commissioning

    - The Supplier must display the content of the DVS (dispatch control system) application, which accounts for 80% +- 5% of the total video wall area.

    - The Supplier must ensure the integration of the video wall with the applications operating in the Microsoft Windows environment for the display of additional operational information. (Specific applications will be specified during the project.)

    - The video wall management system must have a fully duplicated control architecture that ensures automatic switching to the backup controller and uninterrupted operation of the system in the event of a failure without user intervention.

    - The Supplier must ensure that at least 10% of the quantity of critical system components to be replaced (e.g. cubes, controllers or other critical elements for the system) are stored at the Contracting Authority's premises in order to ensure their prompt replacement in the event of a breakdown without the need for additional order or delivery. In case of justified necessity and in agreement with the Contracting Authority, part of the components may be stored in the Supplier's warehouse, but this cannot affect the established response and recovery periods.

    - The video wall must have an automatic solution for calibrating colors and brightness.

    - The Supplier is required to perform FAT (Factory Acceptance Test) and SAT (Site Acceptance Test) tests according to a pre-agreed test plan approved by the Contracting Authority.

      - The Supplier must submit the FAT and SAT test plan for approval with the Contracting Authority no later than 10 business days before the start of the planned tests.

      - The Contracting Authority must provide comments on the test plan no later than 5 business days after receipt of it.

      - FAT and SAT tests must verify compliance with all requirements set out in the technical specification.

      - After the FAT and SAT tests, the Supplier must provide test reports that record the test results and identify discrepancies.

      - The Contracting Authority must provide comments on the test results no later than 5 working days after receipt of the report.

      - If discrepancies are identified, the Supplier must eliminate them within the deadline agreed with the Contracting Authority, but not longer than within 10 working days, and re-perform the appropriate tests.

      - The system is considered properly installed only after successful completion of the SAT tests and the signing of the acceptance-transfer act.

    - The dismantling of the video wall, the delivery, installation and commissioning of the equipment must be planned in such a way that the downtime period of the System Control Centre (from the moment of suspension of activity to the full start-up of the system (M3-M8 according to Chapter 10)) is as short as possible and does not exceed 70 calendar days.

    - The Supplier must agree with the Contracting Authority on a detailed sequence and schedule of works, ensuring a minimum period of interruption of activity.

  - Training

    - The Supplier must provide user and administrator documentation and train at least 25 (twenty-five) employees of the Contracting Authority. At least 5 employees must be trained live (during contact training at the Contracting Authority's premises), and at least 20 employees must be trained remotely. Training must include system management, administration, incident management, and basic system maintenance actions.

  - Safety requirements

    - As the object is included in the List of Specific Equipment and Assets Important for Ensuring National Security (Government Resolution No. 558 of 6 June 2018), all employees of the Supplier and employees of the Supplier’s and subcontractors’ employees, who will have the right to enter the Contracting Authority's object without escort or to connect to communication and information systems when delivering goods and/or performing works, must be inspected in accordance with the procedure established by legal acts.

This requirement applies for the entire duration of the contract, including video wall installation, dismantling, integration, commissioning, maintenance, warranty and post-warranty service works. The Supplier must anticipate the need for employee screening and ensure that all employees assigned to carry out the work have valid permits to enter the facility.

    - The inspection of employees is carried out in accordance with Paragraph 8 of Article 17 of the Law of the Republic of Lithuania on the Protection of Objects Important for Ensuring National Security and Resolution No. 447 of the Government of the Republic of Lithuania of 9 May 2018.

    - The Supplier must provide the data necessary for verification on all employees no later than within 10 working days from the date of signing the contract.

    - The Supplier must assess that the process of checking employees may take time, therefore the planning of works must be carried out taking into account the possible duration of the inspection. Without carrying out an inspection, access to the object is not possible for employees.

    - All risk factors related to the submission of employees for inspection and obtaining permits must be borne by the Supplier and must not be considered as a basis for extending the terms of delivery, installation and/or performance of works of the Goods.

- OPERATIONAL REQUIREMENTS

  - Maintenance services

    - The Supplier must provide video wall maintenance services for a period of 60 months.

    - Maintenance services must include fault response, diagnostics, and software updates.

    - Maintenance services must be provided in 24/7 mode.

    - Support must be provided on site.

    - The following maximum time limits apply:

- critical failure – a malfunction of the entire video wall or 2/3 of its part, due to which it is impossible to display the DVS application or other operational information: reaction ≤ 1 hour; arrival on site ≤ 8 hours; restoration of functionality ≤ 24 hours;

- non-critical failure: reaction ≤ 8 hours; fault elimination ≤ 5 working days.

    - Maintenance services are subject to a Service-Level Agreement (SLA), and in the event of SLA violations, contractual penalties apply.

    - The Supplier must ensure the availability of spare parts for the supplied system for at least 10 (ten) years.

  - Parts and service option for unplanned non-warranty failures

    - The use of parts equivalent to the manufacturer's components is permitted, provided that this does not interrupt or limit the manufacturer's warranty for the entire video wall system, which must be confirmed in writing by the manufacturer.

    - The same maximum terms apply to the elimination of critical non-warranty faults as for maintenance services.

- Technical requirements

The technical requirements set out in this Chapter apply to procurement objects 2 and 3 specified in Table No. 1.

  - General technical requirements for a video wall

    - The video wall solution must be based on RGB laser rear projection (RPC) technology for use in critical infrastructure control centers.

    - The video wall must be adapted for continuous 24/7 work.

    - The image rendering technology used must not have a risk of burn-in or image retention.

    - The video wall solution must be designed for long-term use (at least 10 years), with the support of key components provided by the manufacturer.

    - The video wall must be suitable for use in rooms without natural light, in the presence of constant artificial light.

    - The video wall solution must ensure a low noise level not exceeding 30 dB(A) measured at a distance of 3 meters in front of the video wall.

    - The video wall solution must provide for the duplication of critical components to ensure the uninterrupted operation of the system.

    - The video wall solution must be compatible with applications running in the Microsoft Windows environment.

  - Requirements for video wall cubes (RGB laser RPC technology)

    - Video wall cubes must comply with the general requirements set out in Chapter 7.1.

    - The diagonal of one video wall cube is about 70 (±3%) inches.

    - The aspect ratio of the video wall cubes is 16:9.

    - The natural (native) resolution of one cube is at least Full HD (1920 × 1080 pixels).

    - The brightness of the video wall in normal operating mode is at least 500 cd/m².

    - The contrast ratio of the video wall is at least 1800:1.

    - The uniformity of the brightness of the video wall is at least 95%.

    - Separate RGB laser light sources must be used in the video wall cubes.

    - A laser light source has a lifespan of at least 100,000 hours to at least 50% of its original brightness in 24/7 operation.

    - Video wall cubes must support automatic color and brightness calibration, ensuring uniform support for image parameters across the entire area of the video wall.

    - The design of the video wall cubes must provide rear access.

    - The video wall cubes must be adapted to the curved video wall configuration as specified in Annexes 3, 4 and 5 to the technical specification.

    - Video wall cubes must be suitable for operation in the temperature range of 0 °C – +40 °C and up to 90% relative humidity (without condensation).

  - Video Wall Management System Requirements

    - Functional requirements:

      - The video wall management system must provide centralized management of the entire video wall, including the distribution of video sources, window layout, and real-time change of video content.

      - The video wall management system must ensure the ability to process and display the total natural (native) resolution of the entire video wall (at least 19,200 × 4,320 px), ensuring a smooth presentation of content over the entire wall area.

    - Requirements for management:

      - The video wall management system must ensure that at least 4 controllers' workstations are able to submit content to their assigned video wall areas.

      - For this purpose, an area of the video wall must be provided for at least 20 % (±5 %) of the total area of the video wall.

      - The system must ensure that this area can be dynamically distributed between one, several or all dispatcher workstations, depending on operational needs.

      - An equivalent rendering resolution of at least 1920 × 1080 pixels (Full HD) must be provided for each dispatcher workstation displayed simultaneously on the video wall.

      - The system must enable the dispatcher's workstation to use the video wall area assigned to it as an additional image display space (extended desktop) or equivalent functionality that ensures the ability to independently submit content to the video wall area assigned to the controller.

      - The control of the size, position and arrangement of the windows of the controllers' workstations displayed in the video wall must be carried out centrally from the video wall control workstation.

      - The system must ensure the ability to change the size, position and distribution of windows between the dispatchers' workstations in real time, without interrupting the operation of other sources displayed on the video wall.

      - A video wall management system must enable operators and administrators to control the layout, size and position of video sources on the video wall in real time, to create, store, edit and activate preset video wall operating modes (presets).

      - The video wall management system must support at least two levels of user rights: operator/administrator.

      - The operator's rights must enable the use of the system functions assigned to it, but not the ability to change the system configuration, user rights, or other administrative settings.

      - Administrator privileges must allow you to configure video wall layouts, user rights, preset video wall operating modes (presets), and other system settings.

    - Requirements for video sources:

      - The video wall management system must support at least 16 independent video sources (inputs), including workstation computers, server systems and applications, external data sources, presentation and service workstations.

      - The system must ensure the integration of video sources using live video signals (DisplayPort and/or HDMI), IP-based solutions (web applications, data streams, video streaming) and remote workplace solutions (KVM or equivalent).

      - The video wall management system must ensure simultaneous management of video sources and their representation and the ability to flexibly change their arrangement in the video wall in real time.

      - The integration of video sources must be adapted for uninterrupted 24/7 operation, ensuring stable rendering of different types of content regardless of the way they are integrated.

      - The final types of video sources, their quantities and ways of integration must be detailed and agreed with the Contracting Authority during the implementation of the project, taking into account the actual operational needs.

    - Application support requirements:

      - The video wall management system must be compatible with Microsoft Windows operating systems and must ensure the display of video content from applications running in the Windows environment.

The proposed video wall system must support the deployment of virtual machines (VMs) in the video wall infrastructure; In this case, the virtual machines would be equipped with Windows Enterprise OS used by the Contracting Authority or if the Video Wall System does not support the installation of VMs and their display in the video wall, software must be provided with the help of which the image would be transmitted over IP from the workstations located in the Contracting Authority's virtual infrastructure. In this case, the video wall software must emulate a high-resolution virtual video card (or equivalent) to transmit a high-resolution image from the Contracting Authority to the image of the workstations in the virtual infrastructure via IP. The workstations in the Contracting Authority's virtual infrastructure do not have a GPU. (Graphics Processing Unit) and there are no plans to install them, the solution must work without GPU acceleration.

    - Requirements for monitoring the status of the system:

      - The video wall management system must ensure automatic monitoring of the system status and the generation of fault notifications, including the status of video wall controllers, the operation of displays, the status of signal sources, network connection failures, software or server errors, as well as the condition of temperature and power supplies.

    - Software and license requirements:

      - The video wall management system must be integrated as part of an integrated system, and all software components and licenses necessary to ensure basic functionality must be included in the offer and supply, without the need for additional licenses during acquisition or operating fees.

  - System Architecture Requirements

    - The proposed solution must be based on a well-defined system architecture, including a video wall system, a control system, the integration of video sources and the interaction between the main components. During the implementation of the contract, the Supplier must provide a detailed scheme (diagram) of the system architecture and coordinate it with the Contracting Authority before starting the system installation works.

    - The proposed architecture must be adapted for uninterrupted 24/7 operation and ensure the stability and reliability of the system.

    - The architecture must be designed to minimize the risk of a single point of failure (SPOF) and to ensure automatic failover of critical components.

    - The Supplier must clearly indicate the controllers used, the main components of the system and their interactions with each other.

    - The Supplier must describe the chain of transmission and processing of video signals, including the types of inputs used (DisplayPort, HDMI, IP, etc.).

    - If the solution uses an IP-based architecture, the vendor must describe the network requirements, the protocols used, and the principles for ensuring the stability and security of the solution.

    - The architecture must enable the integration of applications and virtual machines (VMs) running in the Microsoft Windows environment, ensuring stable display of video content in the video wall.

  - Cybersecurity requirements

    - The proposed solution must comply with the principles of the European Union's Cyber Resilience Act (CRA).

    - The system must ensure:

- user authentication;

- User rights management (RBAC – Role-Based Access Control);

- Audit logging.

    - The Supplier must ensure:

- provision of software updates (patches);

- management of security vulnerabilities;

- incident management process.

    - The system must be suitable for use in an isolated network environment.

    - The solution must not depend on external cloud services to ensure basic functionality.

    - The Supplier must comply with the requirements set out in Annex No. 2 ("Minimum Information Security Requirements for the Provision of Services").

  - Manufacturer and Supplier requirements

    - The Supplier is responsible for the operation, integration, compatibility and compliance of the entire proposed system with the requirements of the technical specification.

    - The Supplier must ensure that the manufacturer of the proposed system:

- provides technical support;

- ensures software updates;

- ensure the availability of spare parts for at least 10 (ten) years from the date of signing the final act of acceptance-transfer act;

- has a well-defined fault and incident management process.

    - The Supplier must provide documents confirming the manufacturer's obligations.

    - All communication with the Manufacturer is carried out through the Supplier, who is the only contact person for the Contracting Authority.

- Proof of compliance

  - The Supplier must provide a completed technical correlation table with the tender, which must clearly indicate compliance for each requirement of the technical specification (Annex No. 1.)

  - Each requirement must be based on specific data and references to the manufacturer's official technical documentation (e.g. technical specifications, datasheets, instructions provided by the manufacturer or other documents).

  - The Supplier must specify a specific parameter for the specific product offered and the intervals specified in the Technical Specification cannot be repeated. Answers, "yes", "corresponding" or similar general endorsements are considered insufficient and may be considered non-compliant if no supporting documents are provided.

  - The Contracting Authority reserves the right to request additional explanations, documents or evidence during the evaluation confirming the compliance of the proposed solution with the requirements of the technical specification.

  - If an equivalent solution is proposed, the Supplier must provide a detailed justification demonstrating that the proposed performance is not inferior to those specified in the technical specification.

Equivalence may apply to individual system components or technological solutions (e.g. ways of integrating video sources, software functionality or control system), but not to the underlying imaging technology.

The video wall technology must be RGB laser rear projection (RPC).

  - Lack of technical documentation or other evidence may be considered as non-compliance of the tender with the requirements of the technical specification.

- Warranty conditions

  - The Supplier must provide a warranty of at least 84 months/7 years for the entire installed video wall system from the date of signature of the final acceptance-transfer act.

  - The warranty must cover all system components, equipment, software, and work performed.

  - During the warranty period, all detected faults must be eliminated at the expense of the Supplier.

  - After-sales service must be provided in accordance with the SLA requirements set forth in Chapter 6.

  - The warranty must not be limited to the use of individual system components or third-party software if they are part of the proposed solution.

  - Parts replaced or repaired during the warranty period must be no worse than the original and ensure the operation of the system in accordance with the requirements of the technical specification.

  - After warranty repairs or component replacements, the system must be fully restored and operate without additional functionality restrictions.

- Deadlines

  - The Supplier must carry out the work in accordance with the work schedule below, in which M0 is considered to be the date of signing the Contract, and achieve each of the guidelines specified therein within the specified deadlines.

  - If the work schedule provides for the execution of works in the period from 20 December to 2 January, such works must be coordinated with the Contracting Authority separately.

  - Work schedule

| Milestone | Result | Deadline |
| --- | --- | --- |
| M1 | The project kickoff meeting held, a detailed work schedule submitted and agreed with the Contracting Authority, the work progress and methodology approved | M0 + 2 weeks |
| M2 | Ready-to-Deliver Video Wall Equipment | M0 + 24 weeks |
| M3 | Dismantling of the existing video wall has been carried out | M0 + 24 weeks |
| M4 | Video wall equipment delivered to the Contracting Authority's object | M0 + 25 weeks |
| M5 | New RGB laser video wall installed | M0 + Week 31 |
| M6 | Alignment of the video wall and installation of the control system | M0 + 32 weeks |
| M7 | System Testing Performed (SAT) | M0 + 33 weeks |
| M8 | The system was started (go-live) and the act of acceptance-transfer act of all works was signed | M0 + 34 weeks |

- ENVIRONMENTAL REQUIREMENTS

  - The proposed video wall equipment must comply with the applicable requirements of the European Union and the Republic of Lithuania environmental protection legislation applicable to electrical and electronic equipment.

  - The proposed equipment must comply with the requirements for the restriction of hazardous substances in accordance with Directive 2011/65/EU on the restriction of hazardous substances (RoHS).

  - The proposed equipment must comply with the ecodesign requirements for electronic equipment, if such requirements apply to the proposed equipment.

  - Video wall equipment must be marked with the CE mark, confirming compliance with the applicable requirements of the European Union.

  - The Supplier must ensure that at the end of the service life all equipment is properly managed in accordance with the requirements of the Waste Electrical and Electronic Equipment (WEEE) Directive.

  - The Supplier, when carrying out dismantling works, must ensure that the dismantled equipment is handed over to a licensed waste manager and managed in accordance with the applicable requirements of environmental legislation.

  - The Supplier must provide documents or declarations confirming the conformity of the proposed equipment with the environmental requirements set out in this chapter.

- AnnexES

Annex 1 – COMPARATIVE TABLE OF CONFORMITY OF GOODS WITH THE REQUIREMENTS OF THE TECHNICAL SPECIFICATION

Annex 2 – Minimum Information Security Requirements for the Provision of Services

Annex 3 – Scheme of the premises

Annex 4 – Photograph of the existing video wall (view from the back)

Annex 5 – Photograph of the existing video wall (front view)

Annex 1 - COMPARATIVE TABLE OF COMPLIANCE OF GOODS WITH THE REQUIREMENTS OF THE TECHNICAL SPECIFICATION

| Sec. No. | Technical parameters, function, fulfillment or characteristic of the goods | Quantity (measure unit), required value, fulfillment or property of the parameter (measure unit) or function | Specifies the parameter of the proposed Item | Offer documents confirming the eligibility of the proposed goods or a reference to publicly available documents |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Document title and/or No./reference | Page No. |
| 1 | Video Wall Technology | RGB Laser Rear Projection (RPC) |  |  |  |
| 2 | Diagonal of one cube of a video wall | 70 (±3%) inches |  |  |  |
| 3 | Image format | 16:9 |  |  |  |
| 4 | Natural (native) resolution | At least 1920 × 1080 |  |  |  |
| 5 | Entire video walls in native resolution | At least 19 200 × 4 320 |  |  |  |
| 6 | Brightness | At least 500 cd/m² |  |  |  |
| 7 | Contrast ratio | At least 1800:1 |  |  |  |
| 8 | Brightness uniformity | At least 95 % |  |  |  |
| 9 | Laser Light Source Life | At least 100,000 hours up to at least 50% of the initial brightness |  |  |  |
| 10 | Access to maintenance | Rear access |  |  |  |
| 11 | Adaptation to curved configuration | Mandatory |  |  |  |
| 12 | Operating temperature | Not less than 0 °C – +40 °C |  |  |  |
| 13 | Relative humidity | Up to 90%, non-condensing |  |  |  |
| 14 | Centralized video wall management | Mandatory |  |  |  |
| 15 | Display of DVS content | Mandatory |  |  |  |
| 16 | Rendering other video sources from workstations | Mandatory |  |  |  |
| 17 | Number of simultaneously supported video sources (inputs) | At least 16 |  |  |  |
| 18 | User permission levels | At least 2 (operator, administrator) |  |  |  |
| 19 | Ways to integrate video sources (HDMI/DisplayPort/IP/KVM) | Mandatory |  |  |  |
| 20 | Preset video wall operating modes (presets) | Mandatory |  |  |  |
| 21 | Automatic switching in case of failure | Mandatory |  |  |  |
| 22 | System health monitoring and notifications | Mandatory |  |  |  |
| 23 | Integration with applications running in the Windows environment | Mandatory |  |  |  |
| 24 | Support for virtual machines (VMs) | Mandatory |  |  |  |
| 25 | Performing FAT tests | Mandatory |  |  |  |
| 26 | Performing SAT tests | Mandatory |  |  |  |
| 27 | Operating mode | 24/7 |  |  |  |
| 28 | Noise level | Not more than 30 dB(A) |  |  |  |
| 29 | Duplication of critical components | Mandatory |  |  |  |
| 30 | Long-term use | At least 10 years |  |  |  |
| 31 | User Authentication and Rights Management (RBAC) | Mandatory |  |  |  |
| 32 | Inventory storage for critical components | At least 10% of the critical components of the system must be stored at the Contracting Authority's premises |  |  |  |
| 33 | Support mode | 24/7 |  |  |  |
| 34 | Response and Troubleshooting Times (SLAs) | Critical failure: reaction ≤ 1 hour; Service restoration ≤ 24 hours Non-critical failure: reaction ≤ 8 hours; Disposal ≤ 5 working days |  |  |  |
| 35 | Operating in an isolated network environment | Mandatory |  |  |  |
| 36 | Dependence on external (cloud) services | Cannot be used to ensure basic system functionality |  |  |  |
| 37 | Submission of FAT and SAT test plans | Mandatory |  |  |  |
| 38 | Warranty period | At least 84 months (7 years) |  |  |  |
| 39 | Video Wall Control for Dispatcher Workstations | At least 4 workstations; at least 20 % (±5 %) of the video wall area; a resolution of at least 1920 × 1080 for each workstation; dynamic video distribution must be ensured |  |  |  |