# TECHNICAL SPECIFICATION

# Off-road capable ambulances

| No. | Feature | Technical specification requirement |
| --- | --- | --- |
|  | Object of procurement | Off-Road capable ambulances equipped with medical equipment, suitable for transporting medical personnel, patients and medical equipment. * the proposed vehicle is considered to meet the procurement object if it meets the following requirements |
|  | Place of delivery of goods | Yerevan, Republic of Armenia. Exact delivery address will be provided for the supplier in the execution of the contract. |
|  | Delivery term | No later than 8 months after the Contract enters into force |
|  | Vehicle‘s equipment | The vehicle must be equipped to be used in territory of the Republic of Armenia without additional measures. The gauges and read-outs of the instrument cluster must be in the metric system, and the vehicle must be designed for right-hand drive (left-hand steering wheel). |
|  | Vehicle‘s manufacturing | The vehicle must be new and unused (made no earlier than 2023 and with mileage of not more than 5000 km). |
|  | Vehicle‘s type | Off-road capable Sport utility vehicle (SUV) or Pickup-type vehicle manufactured as or modified/converted into an ambulance. |
|  | Quantity | 2 units |
|  | Number of passengers (with driver) | Not less than 3 (one in a cabin and 2 seats in patient compartment), additionally 1 patient on the stretcher and 1 driver seat. |
|  | Gearbox type | Manual or automatic gearbox with additional Low-range gear, selectable by the driver. |
|  | Engine | Diesel engine, equipped with forced induction system (Turbocharger) with emission standard not lower than EURO V |
|  | Drive system | 4-wheel drive system, selectable (4WD) or constant (AWD). |
|  | Requirements for the features of the vehicle | Vehicle must have following functionalities and features: - Not less than 1 blue priority lighting system (rotating or flashing), visible from all sides of the vehicle. - Audible siren, controlled from inside the vehicle. - Equipped with fire extinguisher weighing at least 2 kg. - Must be fitted with heating and air conditioning system for the driver compartment and medical compartment/superstructure (single system or separate systems for each compartment). |
|  | Requirements for the medical compartment/ ambulance superstructure | Medical compartment/ambulance superstructure must have following features: - The size of the superstructure must be able to carry one patient on a stretcher in a dedicated space, with additional 2 seats for seated patients/medical personnel and accommodate all medical equipment. - The width of the superstructure must not exceed the width of the vehicle. - The rear of the compartment/ superstructure must be fitted with 2 side-opening doors allowing for loading of wheeled stretcher. - If medical compartment/superstructure is fitted with windows, they must not be translucent (must be tinted or obscured in other ways). - 12V Electrical wiring for medical devices - at least 2 12V connectors must be installed. - Interior lighting with the ability to switch lights in patient area must be installed. - Shelves and compartments must be installed for the secure storage and transportation of medical equipment. Shelves and storage compartments must be designed and built with retention features to hold content secure during driving. |
|  | Medical equipment installed and supplied with the vehicle | Each vehicle must be fitted with following medical equipment: - Foldable mobile stretcher with wheeled base, mounted on stretcher base installed in the vehicle. - Pick-up (scoop) stretcher for carrying of the patient, with immobilization straps. - Rigid spinal board for patient lifting, with handles. - Oxygen supply system with no less than 10-liter oxygen cylinder and oxygen supply system for patient with oxygen control valve, regulator and humidifier and connection point for patient supply. |
|  | Bodywork color | Vehicles must have following paint scheme: - Base color of the vehicle and superstructure must be white. - Vehicle must have markings for the ambulance vehicle, corresponding to the requirements of Republic of Armenia. Exact marking scheme will be provided for the supplier in the execution of the contract. |
|  | Tires | The vehicle must be fitted with a set of tyres suitable for a wide range of on-road and off-road surfaces and meeting the following requirements: - Must comply with M+S (Mud+Snow) tyre category and be marked accordingly. - Must be suitable for road and off-road use (have All-Terrain or Mud-Terrain, or Hybrid-Terrain, or other comparable manufacturers marking). - Tyres must be non-studded. - The vehicle must be supplied with a spare wheel of the same size and with the same tyre as fitted to the vehicle and with all the necessary tools to replace the wheel including lifting device. |
|  | Warranty | The Seller or its authorized representative must ensure the warranty servicing of the vehicle at the service center specified by the Supplier or its representative in the territory of the Republic of Armenia, and if there are no service centers of the Supplier or its representative in the specified territory, the vehicle must be transported and returned by the Supplier at its own expense from the location of the Beneficiary's deployment to the Seller's or its representative's premises for service and maintenance. The vehicle must be covered by a warranty of at least 24 months or 100 000 km whichever comes first. Medical equipment must be covered by a warranty of at least 12 months. |
|  | Documentation | Printed vehicle maintenance and user manual with servicing instructions in English or Armenian must be provided with the vehicle. Maintenance and user manuals with servicing instructions in English or Armenian for the medical equipment, printed (one manual for each item of medical equipment, that requires manual to be used properly) should be provided with the vehicle. |

Notes:

The supplier will be required to submit with the tender documents (manufacturer's technical descriptions and/or other documents) confirming that the characteristics of the goods offered by the supplier comply with the requirements.

Where the technical specification indicates a specific model or source of supply, a specific process specific to the goods or services supplied by a particular supplier, or a trademark, patent, types, specific origin or manufacture, it shall be understood that these characteristics are indicative only and that the suppliers are free to offer an equivalent option. The supplier must demonstrate the equivalence referred to in this paragraph.

In the event that the technical specification specifies values/parameters for a particular technical characteristic, suppliers may offer goods whose values/parameters are not inferior to those specified (goods with no inferior characteristics may be offered).