Contact person:

Milda Levickienė

Specialist of CBRN protection section

Defence Materiel Agency

Ministry of National Defence of the Republic of Lithuania

E-mail: milda.levickiene@kam.lt

TECHNICAL SPECIFICATION OF A FILTERING BOX FOR A GAS MASK FOR NUCLEAR, BIOLOGICAL AND CHEMICAL PROTECTION

2026		No.

Filtering box

- A filtering box shall ensure protection from military chemical and biological agents, radioactive particles, and shall have at least A2B2E2K2P3 type and class in accordance with EN 14387, EN 143 or equivalent standard. A test report from a notified body (laboratory) shall be submitted for the tender with the gas capacity test results, which shall confirm the filter A2B2E2K2P3 type and class.

- A threaded screw shall be a NATO standard RD 40 x 3.63 mm and in compliance with NATO STANAG 4155, EN 148-1 or equivalent standard.

- At an inhalation air flow rate of 30 l/min, the inhalation resistance must not exceed 150 Pa.

- A filtering box shall be painted in dark (black RAL 9005 or dark grey RAL 7013, 7016, 7021, 7022, 7026, or ‘Rifle green’ RAL 6007 or equivalent) non-glossy paint.

- The expiry date of a filtering box shall be indicated on the package. The note shall be clear and indelible.

- A filtering box shall contain information on agents from which it protects (name, code and colour coding).

- A filtering box shall be accompanied by a description of the list of materials from which the filter box protects.

- The description shall explain the marking (icons, ideograms).

- A filtering box shall be packed in a vacuum package, which is easy to open without any additional tools and while wearing protective gloves.

- Openings in a filtering box shall be securely covered with lids, which are easy to remove while wearing protective gloves.

- The filter box shall meet the safety, health and environmental protection requirements of the European Union and shall have CE marking. A copy of the issued CE certificate or manufacturer's declaration or an equivalent document shall be submitted with the tender documents.

- The warranty term of the filter box for storage in vacuum packaging shall be at least 10 years from the date of signing the act of delivery and transfer of the goods.