Annex 2 „Technical specification“ to Procurement

- Table No. 1 „Technical requirements for Aviation NVG”

| Parameter | Description | Supplier's Proposal (The Supplier shall indicate, where applicable, the specific value, characteristic or a concise description demonstrating compliance with the technical requirement. General statements such as "Yes", "Compliant", "Meets the requirement", "+", "<... is not less than ...>", "<... will not exceed ...>", or similar shall not be accepted.) |
| --- | --- | --- |
| 1. General description | Helicopter pilot NVG (night vision goggles) shall be especially designed for airborne use inside NVIS compatible cockpit environment and shall correspond to the appropriate airborne standards in use. |  |
| 2. Compatibility with aircraft systems | NVG shall be compatible with aircraft cockpit lightning and instrumentation utilizing Class B filter. |  |
| 3. Optical parameters | 1. NVG shall be equipped with F/1 objective lens or better. 2. NVG shall ensure at least 47° FOV at 30mm eye relief. 3. At least 1.3 cy/mrad (minimum) goggle resolution shall be ensured. 4. Collimation ranges shall be ensured (at least): a) <1° convergence/divergence ; b) <0.5° dip-vergence. 5. Individual eyepiece adjustment shall cover range (at least): +2 to -4 diopters. |  |
| 4. Image intensifier tubes | 1. Intensifier tubes shall contain FOM (Figure of Merit) (at least): 2300 or better. On equipment delivery Supplier shall provide open declaration / any technical document which proves claimed FOM. 2. Intensifier tubes shall be equipped with WHITE phosphorus (P45 type). 3. Intensifier tubes shall cover spectral range between visible light and to 1.0 µm (infrared light), to ensure operation capability of tactical searchlight and laser pointers operating in far near infrared range (around 950 - 1000 nm). |  |
| 5. Electrical power supply | 1. NVG shall be powered utilizing standard AA or AAA battery packs. 2. NVG shall be powered with at least 2 (two) redundant battery supplies (hot switchover) to ensure continues and redundant operation if one of the battery packs fail. 3. NVG shall have low battery status / warning indicator. 4. Operation time from single battery pack shall be not less than 25 hours. 5. Electrical power system while in operation shall not create EMI (Electro-Magnetic interference) for aircraft intercommunication, radio and communication equipment. |  |
| 6. Ergonomics / adjustment | NVG shall be able to be adjusted within following directions and ranges: a) tilt – at least 10°; b) fore / aft – at least 30mm; c) vertical – at least 22 mm; d) Independent interpupillary adjustment for each eyepiece shall cover adjustable range (at least): 52 to 70mm. |  |
| 7. Weight | NVG shall correspond to following mechanical properties: a) Mass of the goggle (without bracket and battery pack) shall not be above 600 grams; b) Battery unit shall have possibility to be equipped with counter-balance weights; c) Attachment of battery unit shall be utilized using vector pads. |  |
| 8. Package contents | Each NVG kit shall contain following items: a) NVG assembly with helmet adapter; b) Power pack unit with attachment to the helmet; c) Helmet mount plate and helmet adaptors; d) Counter balance weights; e) Operators manual in English; f) Carry case; g) Lens protective caps, lens cleaning cloth; h) Mounting adapter (if required). |  |
| 9. Compatibility with pilot helmet | 1. NVG shall be equipped with four pins quick release ANVIS mount. 2. NVG shall be compatible with MSA LH-250 equipped with Sagem CN2H-AA mount. Existing Sagem mount can be replaced with new adapter, however back compatibility is required to ensure usage of CN2H-AA NVG. MSA adapter (P/N 077054 – “ANVIS night vision goggles screwed support” shall be provided to ensure mating with new NVG’s, unless original equipment manufacturer has specific solution which ensures firm mount while using current CN2H-AA NVG mount. Color of new MSA adapter P/N 077054 shall be - NATO green. |  |
| 10. Compatibility standards | 1. Equipment shall comply with following standard: RTCA DO-275 (“Minimum Operational Performance Standards for Integrated Night Vision Imaging System Equipment”) or ETSO-C164a ‘Night Vision Goggles (NVG); 2. Demonstration of compliance shall be ensured by presenting appropriate CoC - Certificates of Conformity and DDP - Declaration of Design and Performance issued by equipment manufacturer. |  |
| 11. User manuals, Servicing and technical documents | Supplier along with proposal shall provide corresponding technical documents / detailed description to ensure offer verification: - Detailed NVG technical description - Contents description; - NVG user manual; - NVG servicing manual; - Any appropriate technical documents deemed necessary for NVG / offer evaluation. |  |
| 11. Operational temperatures | At least within range of -30°C / + 50° C |  |
| 12. Warranty | At least 12 months equipment manufacturer warranty. |  |

2. Export control requirements:

- The proposed equipment and its supply process must comply with all applicable export control requirements of the European Union, the United States of America and other competent authorities, including the requirements of Regulation (EU) 2021/821, EAR (Export Administration Regulations) and ITAR (International Traffic in Arms Regulations), where applicable to the proposed equipment.

- The Supplier and/or the manufacturer must ensure that there are no legal obstacles, prohibitions or restrictions on the export, re-export, transfer, import and use of the proposed equipment in the Republic of Lithuania that would prevent proper performance of the procurement contract.

- Together with the tender, the Supplier shall submit a free-form declaration of the Supplier and/or the manufacturer confirming that the proposed equipment complies with the applicable export control requirements and that its supply to the Republic of Lithuania is not prohibited or restricted.

- Where export, re-export, transfer or import licenses are required for the proposed equipment, the Supplier shall ensure, at its own expense, that such licenses are obtained and remain valid.

3. Environmental Requirements

| 1. Environmental criteria applicable to the procurement object | Pursuant to subparagraph 4.4.4.4 of paragraph 4.4.4, Night Vision Goggles (NVG) are considered long-life products. In order to ensure the longest possible service life of the product and to reduce waste generation, worn or defective NVG parts, components and accessories shall be replaced with parts and components of no lower quality and technical performance, in accordance with the manufacturer's recommendations and technical documentation. Pursuant to subparagraph 4.4.4.5 of paragraph 4.4.4, upon the end of the service life of the Night Vision Goggles (NVG), the resulting waste electrical and electronic equipment shall be suitable for management in accordance with the applicable legal requirements. |
| --- | --- |
| 2. Documents proving compliance with the environmental criteria | A free-form declaration issued by the Supplier. |