Annex 6 Appendix 1

to the Procurement Conditions

COMPLIANCE TABLE TO TECHNICAL SPECIFICATION

OF THE OPTICAL SCOPE

1. Please note that Appendix 1 to Annex 6 of the procurement conditions constitutes an integral part of the offer.

2. The supplier must complete columns 3, 4 and 5 of the table.

3. By submitting a proposal for this procurement, the supplier confirms that the object to be acquired under the public procurement contract will comply with the specified requirements.

1. GENERAL PROVISIONS

1.1. Intended Purpose
The purpose of the optical sight (hereinafter – the sight) is to ensure target detection, observation, and accurate firing during daytime or in low-light conditions. The sight shall be intended for use with BARRETT 107A sniper rifles used by the Lithuanian Armed Forces (LAF), chambered for 12.7×99 mm ammunition.

1.2. Use and Position within the Organizational Structure
The sight shall be intended for use by units of the National Defence System of the Republic of Lithuania utilizing BARRETT 107A1 rifles.

2. FUNCTIONAL REQUIREMENTS FOR THE SIGHT

| No. | Requirements | Compliance with requirements (to be completed by the Supplier, indicating the specific value (if requested) or whether the offered product complies or does not comply with the specific mandatory or desirable requirement) |  | Compliance document (to be completed by the Supplier, indicating the titles of the documents and/or references to their clauses, which prove the compliance of the offered goods with the technical requirements) |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 |
| 2.1 | The color of the sight shall be green, navy grey, or another dark matte color. | YES/NO Color ___ |  | Reference to the source of information |
| 2.2 | The sight shall be new and unused. | Manufacturer's Confirmation YES/NO |  | Reference to the source of information |
| 2.3 | The sight shall have a NATO NSN code. | Manufacturer's Confirmation YES/NO |  | Reference to the source of information |
| 2.4 | The sight shall be compatible with the 12.7×99 mm caliber sniper rifles currently operated by the Lithuanian Armed Forces (LAF). | Manufacturer’s technical documentation confirming compliance with the requirement YES/NO |  | Reference to the source of information |
| 2.5 | The sight shall be resistant to shock and vibration loads generated during firing when used with 12.7×99 mm (.50 BMG) caliber weapons. The sight shall be tested in accordance with the MIL-STD-810 standard (Shock and Vibration test methods). After firing, the sight shall maintain its mechanical integrity, optical functionality, and zero retention. | Manufacturer’s technical documentation confirming compliance with the requirement Submit Shock and Vibration test reports for tests performed in accordance with the MIL-STD-810 standard (performed no earlier than 3 years ago). A test report performed by an accredited testing laboratory authorized to perform tests according to the MIL-STD-810 standard may be submitted, or a test report performed by the manufacturer's laboratory, approved by an independent organization supervising that laboratory. |  | Reference to the source of information |
| 2.6 | The design of the sight shall be sealed and resistant to dust, dirt, and moisture exposure. The sight shall maintain its operational performance after immersion in water at a depth of 1 meter for not less than 30 minutes and shall be protected against dust ingress into the optical mechanism. The sight shall comply with the requirements of the IP67 standard or an equivalent standard. | Manufacturer’s technical documentation confirming compliance with the requirement Submit a test report confirming that the sight complies with the requirements of IP67 according to IEC/EN 60529 or an equivalent standard. The tests shall have been performed no earlier than 3 years ago. A test report performed by an accredited testing laboratory authorized to perform tests may be submitted, or a test report performed by the manufacturer's laboratory, approved by an independent organization supervising that laboratory. |  | Reference to the source of information |
| 2.7 | The interior of the sight shall be filled with inert gas to protect the optics from internal fogging. |  | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement | Reference to the source of information |
| 2.8 | The light transmission of the sight’s optical lenses shall be not less than 90%. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.9 | The maximum adjustable magnification of the sight shall be not less than 25×, and the objective lens diameter shall be not less than 50 mm. | YES/NO Magnification: ___ times Objective lens diameter: ___ mm. |  | Reference to the source of information |
| 2.10 | The length of the sight shall not exceed 450 mm. | YES/NO Sight length: ___ mm. |  | Reference to the source of information |
| 2.11 | The weight of the sight (excluding the mounting monoblock) shall not exceed 1200 g. | YES/NO Sight weight: ___ g |  | Reference to the source of information |
| 2.12 | The sight shall be equipped with tactical-type turrets intended for rapid and frequent reticle position adjustment in field conditions. The sight shall include two turrets: - elevation adjustment turret (elevation turret); - windage adjustment turret (windage turret). The turrets shall allow elevation and windage adjustments to be performed manually without the use of additional tools. The turrets shall be lockable (locking turrets) or shall incorporate another protective solution against unintentional adjustment. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.13 | The minimum adjustment increment value of the elevation and windage turrets shall not exceed 0.1 MRAD (approximately 1 cm at a distance of 100 m). The turret tracking error throughout the entire adjustment range shall not exceed ±1%. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.14 | One full rotation of the turret shall provide not less than 100 adjustment increments (clicks). Precise adjustment control shall be ensured throughout the entire turret rotation range. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.15 | The rotation of the elevation turret shall be smooth and precisely adjustable. Throughout the full rotation range, not less than 275 distinct adjustment steps (clicks) shall be ensured. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.16 | The direction of rotation of the elevation turret for increasing adjustment values shall be counterclockwise (CCW). | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.17 | The turrets shall have a zero stop mechanism that prevents the elevation turret from being adjusted below the preset zero position and ensures quick and precise return to the zero position. |  | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement | Reference to the source of information |
| 2.18 | The sight shall have a reticle illumination system (red color) intended to improve reticle visibility during low-light conditions. The illumination system shall have two operating modes: a daytime mode and a mode adapted for use with a night vision attachment. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.19 | The sight shall have an automatic reticle illumination sleep mode function that activates when the sight is not in use and reactivates upon movement of the sight (Shake Awake / Motion Sensor / Motion Detection feature). In addition, an automatic shut-off function shall be provided when the system remains unused for more than 12 hours. Power supply: CR-type battery (e.g., CR2032 or CR2025). | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.20 | The sight shall be compatible with clip-on night vision or thermal imaging devices mounted in front of the optical sight. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.21 | The diopter adjustment range of the sight shall be not less than from –3 to +2 diopters. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.22 | The sight shall have a parallax adjustment function. The parallax adjustment range shall be not less than from 25 m or 50 m to infinity (∞). The parallax adjustment shall ensure that the target remains clearly visible and that the point of aim position does not change when the shooter’s eye position changes while looking through the sight. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.23 | Reticle: |  |  |  |
| 2.23.1 | The reticle shall be a tactical MRAD tree-type reticle (e.g., Christmas tree reticle, grid reticle, or hybrid reticle), such as SKMR4, SKMR4+, TREMOR3, TREMOR5, PR2-MIL, or equivalent. | YES/NO MRAD: ____; |  | Reference to the source of information |
| 2.23.2 | The reticle shall be located in the First Focal Plane (FFP), ensuring that the reticle size changes proportionally with changes in optical magnification. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.24 | The sight mounting bracket shall be of a one-piece monoblock scope mount design, Manufactured from a single solid body with integrated rings for mounting the optical sight. A two-piece rings design shall not be permitted. The mount shall be intended for installation on a MIL-STD-1913 (Picatinny) / STANAG 4694 NATO rail. The mount shall be attached to the rail using screws ensuring reliable and stable fixation of the optical sight during firing. The weight of the mount shall not exceed 300 g. The mount inclination angle shall not exceed 20 MOA ±1 MOA. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.25 | The monoblock design shall include a recoil lug that engages with the rail slot and prevents the mount from shifting due to weapon recoil. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.26 | The internal diameter of the monoblock rings shall match the diameter of the main tube of the proposed optical sight, ensuring uniform compression of the sight body. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.27 | The height of the optical axis of the sight above the rail shall be 38–39 mm. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.28 | The eye relief distance shall be not less than 90 mm and not greater than 100 mm, ensuring convenient and safe monitoring for different users. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.29 | The horizontal field of view of the sight shall be not less than 4 m at minimum optical magnification and not less than 0.8 m at maximum optical magnification (measured at a distance of 100 m). | YES/NO Reference to the source of information Specify the field of view width when measuring at a distance of 100 m: Minimum optical magnification ____ Maximum optical magnification ____ |  | Reference to the source of information |
| 2.30 | The exit pupil diameter of the sight shall be not less than 1.5 mm when using maximum optical magnification. | YES/NO Exit pupil diameter (at max magnification): ___ mm |  | Reference to the source of information |
| 2.31 | Sight scope must include: |  |  |  |
| 2.31.1 | A filter for protection against direct sunlight (sunshade / sunshield). |  | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement | Reference to the source of information |
| 2.32.2 | A filter for protection against objective lens glare and reflections (killflash or anti-reflection device). | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.32.2 | The sight shall be supplied with a protective optical sight cover (e.g., textile type) intended to protect the sight from moisture, dust, and mechanical damage during transportation or storage. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.33 | The sight shall be supplied with flip-up lens covers intended to protect the objective and ocular lenses. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.34 | The sight shall be intended for use under various meteorological conditions (rain, snow, dust, fog) and shall operate reliably within a working temperature range of not less than –40°C to +55°C. The sight shall be hermetically sealed, filled with inert gas, and resistant to internal fogging. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.35 | The transportation case shall be made of impact-resistant plastic and shall include an internal shock-absorbing insert for protection of the optical sight. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3 | MANDATORY SIGHT SYSTEM CONFIGURATION |  |  |  |
| 3.1 | Optical sight – 1 pc. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.2 | Optical sight mounting bracket – 1 pc. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.3 | Battery – 1 pc. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.4 | Sight optical protective covers – 1 set. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.5 | Sight filters – 1 set. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.6 | Sight protective cover – 1 pc. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.7 | Sight transportation case – 1 pc. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.8 | Cleaning and maintenance kit – 1 set. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.9 | User and maintenance documentation in English (Lithuanian preferred) – 1 set. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 4. | SUPPORT AND SUSTAINMENT |  |  |  |
| 4.1 | A warranty period of not less than 24 months shall be provided for the sight set, starting from the date of signing the acceptance-transfer certificate. During the warranty period, the seller shall undertake to eliminate identified defects no later than within 45 calendar days from the date the defect is registered. If the defect cannot be remedied within this period, the seller shall replace the defective sight with a new one or provide an equivalent replacement sight until the repair is completed. | YES/NO Warranty period ___ months Defect rectification period ___ days |  | Reference to the source of information |
| 4.2 | The supplier shall provide fault diagnostics and troubleshooting procedures and ensure the availability of maintenance and repair services during the warranty period and for not less than 5 years after its expiration. | YES/NO Manufacturer's Confirmation |  | Reference to the source of information |
| 4.3 | The supplier shall ensure technical and logistical support, including remote technical consultations (by telephone and electronic communication means). | YES/NO Manufacturer's Confirmation |  | Reference to the source of information |
| 4.4 | The supplier shall ensure the availability of spare parts and repair components for not less than 5 years after the expiration of the warranty period. | YES/NO Manufacturer's Confirmation |  | Reference to the source of information |
| 4.5 | The supplier shall organize user training for not fewer than 10 military personnel (train-the-trainer). Upon completion of the training, participants shall be able to operate the sight, perform preventive maintenance, and carry out user-level repair activities. | YES/NO Manufacturer's Confirmation |  | Reference to the source of information |

*Documents that the Supplier must submit together with the offer in accordance with Clause 9.10.3 of the Procurement Conditions. Filling in this section is mandatory.

Notes:

1. The Supplier, when specifying a particular item or confirming product compliance, is required to clearly indicate, to the procuring organization, the precise location within the Supplier's CPP IS where the claim confirmation and proving data is contained. This indication should include the specific folder or file, the document name, and the page number.

2. Each technical requirement item must be substantiated by technical documents and/or manufacturer-approved declarations (if not otherwise specified). These supporting documents are to be included with the offer submission.

3. The phrase "or equivalent" is appended to all specific materials and trade names. In the event a Supplier proposes a product with equivalent properties, it is imperative that they take necessary measures during the offer submission to validate that the proposed product is indeed equivalent and fully compliant with the requirements outlined in the technical specifications.