Annex 6 Appendix 1

to the Procurement Conditions

COMPLIANCE TABLE TO TECHNICAL SPECIFICATION

OF THE FIRE CONTROL SYSTEM (FOR A 40 MM AUTOMATIC GRENADE LAUNCHER)

1. Please note that Appendix 1 to Annex 6 of the procurement conditions constitutes an integral part of the offer.

2. The supplier must complete columns 3, 4 and 5 of the table.

3. By submitting a proposal for this procurement, the supplier confirms that the object to be acquired under the public procurement contract will comply with the specified requirements.

1. GENERAL PROVISIONS

1.1. The purpose of the Fire Control System (hereinafter – FCS) is to assist the Automatic Grenade Launcher (hereinafter – AGL) operator in accurately and rapidly calculating firing parameters based on the ballistic data of the target: distance to the target, target movement speed, direction, and weather conditions. The FCS shall ensure high AGL fire effectiveness when firing at maximum range or under limited visibility conditions. The FCS shall be intended for:

• Automatic grenade launchers Heckler & Koch Grenade Machine Gun E, 40 × 53 mm (hereinafter – HK GMG) used within the National Defence System of the Republic of Lithuania (hereinafter – NDS RL), regardless of time of day and weather conditions.

• Providing the operator with the capability to program programmable ammunition (English: airburst ammunition, hereinafter – PA) manufactured by NAMMO or Rheinmetall.

• Interoperability with other heavy weapon systems used by the Lithuanian Armed Forces: heavy machine guns 12.7 × 108 mm, M2HB QCB, M2A1, as well as the CG M4 recoilless anti-tank weapon.

1.2. Intended Use and Place within the Organizational Structure. The FCS is intended for HK GMG 40 mm automatic grenade launchers used by personnel of the National Defence System of the Republic of Lithuania.

2. FUNCTIONAL REQUIREMENTS FOR THE FCS

| No. | Requirements | Compliance with requirements (to be completed by the Supplier, indicating the specific value (if requested) or whether the offered product complies or does not comply with the specific mandatory or desirable requirement) |  | Compliance document (to be completed by the Supplier, indicating the titles of the documents and/or references to their clauses, which prove the compliance of the offered goods with the technical requirements) |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 |
| 2.1 | The color of the sight shall be green, navy grey, or another dark matte color. | YES/NO Color ___ |  | Reference to the source of information |
| 2.2 | The FCS shall be new and unused, manufactured no earlier than 12 months prior to the date of signing of the acceptance-transfer certificate between the Buyer and the Seller. If the FCS stores and transmits data, the cloud infrastructure used shall be located only within NATO and/or EU member states. | Manufacturer's Confirmation YES/NO |  | Reference to the source of information |
| 2.3 | The sight shall have a NATO NSN code. | Manufacturer's Confirmation YES/NO |  | Reference to the source of information |
| 2.4 | The FCS shall ensure the performance of the following functions: |  |  |  |
| 2.4.1 | Firing the HK GMG at visible and non-visible targets (indirect fire from covered positions). | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.4.2 | Rapid determination, calculation, and presentation of aiming data to the operator, taking into account: distance to the target, target movement speed and direction, and the type of ammunition used, when firing in direct fire mode. The system shall ensure rapid target acquisition, engagement, and a first-round hit probability of not less than 0.8, depending on the operator’s qualification. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.4.3 | Capability to employ the HK GMG against unmanned aerial vehicles (hereinafter – UAVs). |  | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement | Reference to the source of information |
| 2.5 | The design of the FCS shall allow the operator to determine the distance to stationary or moving targets. The speed of the moving target shall be no less than 30 km/h at distances up to 1000 m. The direction of movement shall be parallel to the front or diagonal relative to the HK GMG position. | YES/NO Speed range of a moving target (at a distance up to 1000 m): from ___ to ___ km/h |  | Reference to the source of information |
| 2.6 | The ballistic computer of the FCS shall calculate the data required for firing and provide the operator with the aiming point, taking into account the type of ammunition, weather conditions, and the distance to the target. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.7 | Laser Rangefinder: |  |  |  |
| 2.7.1 | Shall comply with Laser Class 1, Eye Safe requirements. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.7.2 | Measurement range shall be not less than 2000 m. | YES/NO Range: ____ m. |  | Reference to the source of information |
| 2.7.3 | Measurement error shall not exceed ±2 m. | YES/NO Measurement error: ±______ m |  | Reference to the source of information |
| 2.8 | The FCS shall allow the use of Rheinmetall and NAMMO programmable ammunition (PA), as well as non-programmable ammunition. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.9 | The FCS shall ensure the following programmable ammunition (PA) operating modes: |  |  |  |
| 2.9.1 | Non-programmable mode – impact detonation. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.9.2 | All rounds detonate at the same distance. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.9.3 | “Fire away from self” – the first round detonates at the set distance, subsequent rounds at greater distances. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.9.4 | “Fire toward self” – the first round detonates at the set distance, subsequent rounds at shorter distances. |  | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement | Reference to the source of information |
| 2.9.5 | “Broken line” – detonation distances alternate (farther / closer). | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.9.6 | C-UAS mode – the FCS shall automatically calculate the firing angle against a moving UAV and program the rounds in such a way as to create an optimal fragmentation volume. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.9.7 | The manufacturer may offer additional programmable ammunition (PA) operating modes ensuring optimal 40 × 53 mm fragmentation volume formation during burst firing. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.9.8 | The FCS may be used with the weapon mounted on a vehicle turret equipped with a frontal shield; therefore, the sight shall be mounted on or above the weapon (attached to the weapon mount/cradle). The capability to use standard sights shall also remain available. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.10 | The weight of the complete FCS set shall not exceed 6 kg. | YES/NO Weight: ____ kg. |  | Reference to the source of information |
| 2.11 | The FCS housing may be made of metal, plastic, or composite materials. Surfaces shall be non-reflective. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.12 | The FCS shall be mountable on a Picatinny Rail (MIL-STD-1913, STANAG 4694 NATO Accessory Rail) and shall withstand vibrations generated during firing.. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.13 | Power supply – commercial or special batteries. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.14 | It shall be possible to power the UVS from a vehicle 12–24 V DC power supply network. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.15 | Operating time with one battery set shall be not less than 5 hours. | YES/NO Operating time: ____ h. |  | Reference to the source of information |
| 2.16 | Operating temperature: from –40 °C to +40 °C. | YES/NO Operating temperature range: from __ °C to ___ °C; |  | Reference to the source of information |
| 2.17 | Storage temperature: from –40 °C to +70 °C. | YES/NO Storage temperature range: from __ °C to ___ °C; |  | Reference to the source of information |
| 2.18 | The FCS shall have two observation channels: a daytime optoelectronic channel and a thermal imaging channel. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.19 | The resolution of the daytime channel shall be not less than 1920 × 1200 px. | YES/NO Resolution: ____ x ____ px |  | Reference to the source of information |
| 2.20 | The thermal imaging channel shall use an uncooled sensor with a resolution of not less than 640 × 480 px. |  | YES/NO Resolution: ____ x ____ px | Reference to the source of information |
| 2.21 | The thermal imaging channel shall ensure identification of a 2.3 × 2.3 m NATO target at a distance of not less than 500 m. | YES/NO 2.3 x 2.3 NATO target identification range: _____ m.; |  | Reference to the source of information |
| 2.22 | Optical or digital image magnification shall be not less than 1×–6×. | YES/NO Optical / digital (delete as appropriate) zoom: ____ |  | Reference to the source of information |
| 2.23 | It shall be possible to manually enter or adjust the firing range. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 2.24 | The FCS shall have a convenient and rapidly accessible user interface enabling the operator to efficiently set parameters without loss of time. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3 | MANDATORY SIGHT SYSTEM CONFIGURATION |  |  |  |
| 3.1 | FCS with mounting elements – 1 set. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.2 | Vehicle adaptation kit – 1 set. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.3 | Tactical protective cover for the FCS – 1 unit. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.4 | FCS transportation / storage case – 1 unit. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.5 | Battery and charger set (if required) – 1 set. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.6 | Cleaning and maintenance kit – 1 unit. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 3.7 | 3.7. Operation and maintenance documentation in English language (Lithuanian preferred) – 1 set. | YES/NO Manufacturer’s technical documentation confirming compliance with the requirement |  | Reference to the source of information |
| 4. | SUPPORT AND SUSTAINMENT |  |  |  |
| 4.1 | The FCS shall be provided with a warranty period of 24 months from the date of signing the acceptance-transfer certificate. During this period, the Seller shall undertake to eliminate any arising malfunctions in Lithuania and no later than within 45 calendar days. | YES/NO Warranty period ___ months Defect rectification period ___ days |  | Reference to the source of information |
| 4.2 | A mandatory corrective maintenance action plan shall be prepared and ensured for the warranty period and, after its expiration, for a period of not less than 5 years. | YES/NO Manufacturer's Confirmation |  | Reference to the source of information |
| 4.3 | Logistic support shall be ensured, including the possibility of remote technical consultations (by telephone and electronic communication means). | YES/NO Manufacturer's Confirmation |  | Reference to the source of information |
| 4.4 | The availability of spare parts procurement shall be ensured for a period of 5 years. | YES/NO Manufacturer's Confirmation |  | Reference to the source of information |
| 4.5 | A list of spare parts required to sustain the system for 24 months shall be provided. | YES/NO Manufacturer's Confirmation |  | Reference to the source of information |
| 4.6 | User training (train-the-trainer) shall be organized for ten (10) soldiers. Upon completion of the training, the soldiers shall be capable of operating the FCS, performing preventive maintenance, and carrying out user-level repair works. | YES/NO Manufacturer's Confirmation |  |  |
| 4.7 | The FCS supplier shall ensure the possibility for users to incorporate the ballistic data of new ammunition, required for calculations, into the FCS database. | YES/NO Manufacturer's Confirmation |  |  |

*Documents that the Supplier must submit together with the offer in accordance with Clause 9.10.3 of the Procurement Conditions. Filling in this section is mandatory.

Notes:

1. The Supplier, when specifying a particular item or confirming product compliance, is required to clearly indicate, to the procuring organization, the precise location within the Supplier's CPP IS where the claim confirmation and proving data is contained. This indication should include the specific folder or file, the document name, and the page number.

2. Each technical requirement item must be substantiated by technical documents and/or manufacturer-approved declarations (if not otherwise specified). These supporting documents are to be included with the offer submission.

3. The phrase "or equivalent" is appended to all specific materials and trade names. In the event a Supplier proposes a product with equivalent properties, it is imperative that they take necessary measures during the offer submission to validate that the proposed product is indeed equivalent and fully compliant with the requirements outlined in the technical specifications.