Vandens gavybos vietoms skirtų vandens baseinų rizikos vertinimas ir rizikos valdymas

Leidinio duomenys

Pavadinimas:
Geriamojo vandens gavybos vietoms skirtų vandens baseinų rizikos vertinimas ir rizikos valdymas. Koncepcinio modelio pasiūlymas elektroniniam ataskaitų teikimui pagal naujos redakcijos Geriamojo vandens direktyvos 8 straipsnį. Versija 2025-12-18.

Autorė:
Fernanda Néry, Europos aplinkos agentūra

Bendraautoriai:
Anke Schneeweiss (Europos teminis biologinės įvairovės ir ekosistemų centras), Bert Leemans (Europos Komisija), Fanny Gard (Europos Komisija), Francesco Mundo (Europos aplinkos agentūra), Jonathan Maidens (Europos aplinkos agentūra), Jorgen Olsen (Europos aplinkos agentūra).

Parengimo data: 2025-11-28.

Kalba: anglų, vertimas į lietuvių kalbą neoficialus (neadaptuotas pirkimui)

Licencija: Creative Commons Attribution 4.0 International (CC BY 4.0).

Turinys

- Santrauka

- Aprašomasis modelio aprašymas

  - 2.1 Duomenų modelio klasės

  - 2.2 Duomenų modelio ryšiai

  - 2.3 Suderinimas su kitais duomenų srautais. Erdviniai duomenys

  - 2.4 GVD 8 straipsnio duomenų srautui būdingi aprašomieji duomenys

  - 2.5 Pastaba dėl vandens baseinų charakterizavimo

  - 2.6 Suderinimas su kitais duomenų srautais. Dokumentai ir metaduomenys

  - 2.7 Suderinimas su kitais duomenų srautais. Stebėsenos rezultatai

1 priedas. Vandens baseinų ataskaitų teikimas

2 priedas. Geriamojo vandens apsaugos teritorijų ataskaitų teikimas

3 priedas. Vandens gavybos vietų ir stebėsenos vietų ataskaitų teikimas

- Santrauka

Šiame dokumente pateikiamas siūlomas koncepcinis modelis elektroniniam ataskaitų teikimui pagal naujos redakcijos Geriamojo vandens direktyvos (GVD) 8 straipsnį dėl žmonėms vartoti skirto vandens gavybos vietoms skirtų vandens baseinų rizikos vertinimo ir rizikos valdymo. Dokumente nenagrinėjamos vandens baseinų charakterizavimo, pavojų vertinimo, rizikos vertinimo metodikos ar rizikos valdymo priemonių klasifikavimo metodai.

Modelyje CatchmentArea (vandens gavybos vietoms skirtas vandens baseinas) nustatomas kaip pagrindinis rizikos vertinimo vienetas. Kiekvienam vandens gavybos vietoms skirtam vandens baseinui privaloma pateikti poligoninę geometriją ir ryšį su Vandens pagrindų direktyvos (VPDD) vandens telkiniu.

Duomenų rinkinys RiskAssessmentRiskManagement veikia kaip aukštesnio lygmens klausimynas, apibūdinantis rizikos vertinimo būklę ir taikomas priemones. Valstybės narės gali susieti šiuos duomenis su papildoma dokumentacija naudodamos standartizuotą metaduomenų struktūrą.

Papildomai siūlomas WaterSupplyLink duomenų rinkinys, skirtas valdyti sudėtingus daugelio su daugeliu ryšius tarp vandens gavybos objektų ir vandens tiekimo zonų.

Vienas esminių pasiūlymo principų yra esamų duomenų srautų pakartotinis naudojimas ir suderinimas, siekiant išvengti dvigubo duomenų teikimo, ypač pagal Vandens pagrindų direktyvą bei WISE-6 vandens kokybės ataskaitų teikimą.

CatchmentArea yra vienintelis visiškai naujas erdvinis duomenų rinkinys. Vandens telkiniai, stebėsenos vietos ir geriamojo vandens apsaugos teritorijos remiasi esamais Europos registrais ir duomenimis, pateiktais vykdant trečiąjį Upių baseinų valdymo planų (RBMP) ciklą.

Geriamojo vandens apsaugos teritorijoms, įskaitant apsaugos zonas, bus naudojami iš anksto užpildyti ankstesnių VPD ataskaitų duomenys, todėl valstybėms narėms reikės pateikti tik pakeitimus ar papildymus.

Modelyje taip pat numatytos priemonės konfidencialiai informacijai apsaugoti. Valstybės narės galės riboti apsaugos zonų ir stebėsenos vietų erdvinių duomenų viešinimą naudodamos atributą confidentialityStatus, taip užtikrindamos jautrių duomenų apsaugą ir kartu jų prieinamumą atsakingoms institucijoms.

Siūloma sistema taip pat numato atskirą stebėsenos vietų ir saugomų teritorijų atnaujinimo ciklą, leidžiantį duomenis atnaujinti nepriklausomai nuo konkrečių sektorinių ataskaitų teikimo terminų.

- Modelio aprašymas

Elektroninio ataskaitų teikimo modelis pagal GVD 8 straipsnį grindžiamas šiomis prielaidomis:

- kiekviena geriamojo vandens gavybos vieta yra viename paviršinio arba požeminio vandens telkinyje;

- kiekviena geriamojo vandens gavybos vieta turi fizinį vandens gavybos vietoms skirtą vandens baseiną;

- kiekviena gavybos vieta tiekia vandenį vienai arba kelioms vandens tiekimo zonoms;

- pagrindinis rizikos vertinimo ir valdymo vienetas yra vandens gavybos vietoms skirtas vandens baseinas.

Toliau pateikiamas trumpas duomenų modelio klasių ir tarpusavio ryšių aprašymas (1 pav.)

1 pav.  Elektroninio ataskaitų teikimo pagal GVD 8 straipsnį koncepcinis modelis – apibendrinimas

  - Duomenų modelio klasės

C1 – CatchmentArea (Vandens gavybos vietoms skirtas vandens baseinas)

CatchmentArea gali apimti vieną arba daugiau vandens gavybos vietų tame pačiame vandens telkinyje, jei jų rizikos profilis yra vienodas. Kiekvienas objektas identifikuojamas unikaliu identifikatoriumi catchmentAreaIdentifier. Vieta pateikiama poligono geometrija.

C2 – WaterAbstraction (Vandens paėmimo objektas)

Elektroninio ataskaitų teikimo tikslais WaterAbstraction yra loginis objektas, apibūdinantis vandens paėmimą. Jis gali reprezentuoti vieną fizinę gavybos vietą arba kelių vietų grupę. Kiekvienas objektas turi identifikatorių waterAbstractionIdentifier ir yra susietas su CatchmentArea. Šiam objektui geometrijos teikti nereikia.

C3 – WaterSupplyZone (Vandens tiekimo zona)

Vandens tiekimo zona yra teritorija, kurioje tiekiamas ir vartojamas vanduo. Zona gali gauti vandenį iš vienos ar kelių vandens paėmimo vietų.

C4 – WaterBody (Vandens telkinys)

VPD valdymo vienetas, kuriame vyksta vandens paėmimas. Tai gali būti paviršinio arba požeminio vandens telkinys.

C5 – MonitoringSite (Stebėsenos vieta)

Vieta vandens telkinyje, naudojama vandens kokybei tikrinti ir pagrįsti rizikos vertinimą.

C6 – ProtectedArea (Saugoma teritorija)

Valstybės narės kiekviename CatchmentArea gali nustatyti vieną ar daugiau apsaugos zonų. Jos gali saugoti vandens paėmimo vietą, dalį baseino teritorijos arba visą baseiną. Kiekvienai teritorijai suteikiamas protectedAreaIdentifier identifikatorius. Galima taikyti konfidencialumo atributą confidentialityStatus.

C7 – RiskAssessmentRiskManagement

Kiekvienam CatchmentArea pateikiama rizikos vertinimo ir rizikos valdymo plano santrauka.

  - Duomenų modelio ryšiai

R1 – CatchmentArea [1] ↔ [1]  WaterAbstraction

Valstybė narė apibrėžia CatchmentArea rizikos vertinimo tikslais. Teritorijoje turi būti bent viena vandens gavybos vieta. Fizinių vietų koordinatės neteikiamos. Rizikos vertinimui naudojama loginė WaterAbstraction sąvoka.

Pastaba: Tai atitinka reikalavimą, kad vandens gavybos vietoms skirtas vandens baseinas (CatchmentArea) gali apimti kelias vandens gavybos vietas ir kad šių vandens gavybos buvimo vietų pateikti nereikalaujama.

R2 – WaterBody [1] ↔ [1..n] WaterAbstraction

Kiekviena vandens gavybos vieta (WaterAbstraction) turi būti susietas su vienu Vandens telkiniu (WaterBody). Viename vandens telkinyje gali būti keletas vandens paėmimo vietų, kurios gali būti sujungtos į vieną arba kelis WaterAbstraction objektus, priklausomai nuo rizikos profilio.

Pastaba: Jei visų vandens telkinio vandens gavybos vietų rizikos profilis yra panašus, jos gali būti agreguojamos į vieną vandens gavybos objektą (WaterAbstraction), kuriam atliekamas vienas vandens gavybos vietoms skirto vandens baseino (CatchmentArea) rizikos vertinimas. Kitais atvejais rizikos vertinimas gali būti atliekamas skirtingoms vandens telkinio vandens gavybos vietų grupėms (pavyzdžiui, gavybos vietoms, tiekiančioms vandenį tankiai apgyvendintai vandens tiekimo zonai, ir atskirai – mažiau svarbioms gavybos vietoms).

Pastaba: Vandens gavybos vieta (WaterAbstraction) neturi geometrijos. Jo vieta logiškai apibrėžiama pagal jam priskirtą vandens telkinį (WaterBody) ir vandens gavybos vietoms skirtą vandens baseiną (CatchmentArea), kuriam jis priklauso.

R3 – WaterAbstraction  [m] ↔ [n]  WaterSupplyZone

Tai daugelio su daugeliu ryšys. Vienas WaterAbstraction gali tiekti vandenį kelioms WaterSupplyZone, o viena WaterSupplyZone gali gauti vandenį iš kelių WaterAbstraction objektų.

R4 – CatchmentArea [0..1] ↔ [0..n]  ProtectedArea

Rizikoms valdyti gali būti nustatytos apsaugos zonos. Saugomų teritorijų gali nebūti (jei nenustatyta reikšmingų rizikų) arba jų gali būti daug. Saugomos teritorijos (ProtectedArea) geometrija pateikiama kaip vienos dalies arba daugiadalė geometrija.

Pastaba: Jei vienam vandens telkiniui (WaterBody) priskirtas daugiau nei vienas vandens gavybos vietoms skirtas vandens baseinas (CatchmentArea), saugoma teritorija (ProtectedArea) turi būti susieta su konkrečiu vandenvs baseinu. Toks susiejimas nurodomas reikšmėmis: relatedZoneIdentifier = CatchmentAreaIdentifier.thematicIdIdentifier ir relatedZoneIdentifierScheme = CatchmentAreaIdentifier.thematicIdIdentifierScheme.

Ypatingasis atvejis: jei apsaugos zona apima visą vandenvietės baseiną (CatchmentArea), tai nurodoma reikšme specialisedZoneType = 'catchmentArea'.

R5 – WaterBody ↔ ProtectedArea

Techninė pastaba: jei vandens telkiniui (WaterBody) priskirtas tik vienas vandens gavybos vietoms skirtas vandens baseinas (CatchmentArea), saugoma teritorija (ProtectedArea) turi būti tiesiogiai susieta su vandens telkiniu. Toks susiejimas nurodomas reikšmėmis: relatedZoneIdentifier = WaterBody.thematicIdIdentifier ir relatedZoneIdentifierScheme = WaterBody.thematicIdIdentifierScheme.

Ypatingasis atvejis: jei apsaugos zona (ProtectedArea) sutampa su visu vandens telkiniu (WaterBody), tai žymima reikšme specialisedZoneType = 'waterBody'.

R6 - WaterAbstraction [0..1] ↔ [0..n] ProtectedArea

Techninė pastaba: tiesioginis ryšys tarp saugomos teritorijos (ProtectedArea) ir vandens gavybos vietos (WaterAbstraction) nenustatomas. Vietoj to taikomas anksčiau aprašytas metodas: 1) jei vandens telkiniui (WaterBody) priskirtas tik vienas vandens baseinas (CatchmentArea), saugoma teritorija turi būti tiesiogiai susieta su vandens telkiniu; 2) jei vandens telkiniui priskirtas daugiau nei vienas vandens baseinas, saugoma teritorija turi būti susieta su konkrečiu vandens baseinu.

Ypatingasis atvejis: jei apsaugos zona skirta tiesiogiai apsaugoti vandens gavybos vietas (pvz., požeminio vandens gavybos vietų apsaugos juostos), tai žymima reikšme specialisedZoneType = 'waterAbstractionSafeguardZone'.

Ypatingasis atvejis: valstybės narės gali apriboti viešąją prieigą prie erdvinių duomenų rinkinių, jei tokia prieiga, be kita ko, galėtų neigiamai paveikti visuomenės saugumą arba aplinkos apsaugą. Apsaugos zonų buvimo vietos skelbimas gali būti ribojamas nurodant confidentialityStatus = 'N'. (Atkreiptinas dėmesys, kad toks pats metodas jau taikomas stebėsenos vietoms, kurios sutampa su vandens gavybos vietomis.)

R7 – CatchmentArea ↔ MonitoringSite

Nors stebėsenos vieta fiziškai yra vandens telkinyje, šiame ataskaitų teikimo modelyje ji naudojama CatchmentArea rizikos vertinimui pagrįsti.

Techninė pastaba: ryšys gali būti nustatomas pagal objektų ryšių seką CatchmentArea [1] ↔ [1] WaterAbstraction [1] → [1] WaterBody [1] ← [1..n] MonitoringSite ir taikant erdvinę sąlygą ST_Within(MonitoringSite.geometry, CatchmentArea.geometry) = TRUE, t. y. stebėsenos vieta (MonitoringSite) turi patekti į vandenvietės baseino (CatchmentArea) geometriją.

Valstybės narės gali pažymėti konkrečias stebėsenos vietas kaip susijusias su geriamuoju vandeniu, naudodamos atributą MonitoringSite.purpose.

  - Suderinimas su kitais duomenų srautais. Erdviniai duomenys

Šiame skyriuje aprašoma, kaip naujasis modelis integruojamas su jau egzistuojančiomis Europos duomenų teikimo sistemomis ir kaip pakartotinai naudojami anksčiau pateikti duomenys, siekiant sumažinti administracinę naštą bei išvengti dvigubo ataskaitų teikimo.

A1 – WaterSupplyZone (Vandens tiekimo zona)

Vandens tiekimo zonos jau yra teikiamos pagal ataskaitų teikimo prievolę „DWD exceedances, incidents and derogations – Article 18.1 (c, d, e)“. Prireikus šis duomenų rinkinys gali būti atnaujinamas vykdant kasmetinį ataskaitų teikimą.

Konsoliduotas vandens tiekimo zonų identifikatorių sąrašas bus naudojamas kokybės kontrolei vykdant ataskaitų teikimą pagal „GVD 8 straipsnį dėl vandens gavybos vietoms skirtų vandens baseinų rizikos vertinimo ir rizikos valdymo“.

Nenumatoma jokių vandens tiekimo zonų ataskaitų teikimo modelio pakeitimų.

A2 – MonitoringSite (Stebėsenos vieta)

WISE Europos stebėsenos vietų registras skelbiamas adresu:

https://dd.eionet.europa.eu/vocabulary/wise/MonitoringSite

Šis registras bus naudojamas GVD ataskaitų teikimo kokybės kontrolės tikslais tais atvejais, kai duomenų teikėjai pageidaus aiškiai nurodyti stebėsenos vietų, susijusių su kiekvienu vandens gavybos vietoms skirtu vandens baseinu, sąrašą.

Trečiojo Vandens pagrindų direktyvos (VPD) Upių baseinų valdymo planų (RBMP) ciklo metu stebėsenos vietų duomenys buvo teikiami pagal duomenų srautą:

„Water Framework Directive – River Basin Management Plans – 2022 Spatial Data“.

Šiuo metu diegiamas atskiras duomenų srautas, kuris leis prireikus kasmet atnaujinti MonitoringSite duomenų rinkinį.

Numatomi nedideli duomenų modelio supaprastinimai:

- bus pašalinti retai naudojami atributai;

- duomenų formatas keisis iš Shapefile į GeoPackage;

- duomenų teikimas bus perkeltas iš Reportnet2 platformos į Reportnet3 platformą.

A3 – WaterBody (Vandens telkinys)

WISE Europos vandens telkinių registras pateikiamas adresu:

https://dd.eionet.europa.eu/vocabulary/wise/WaterBody

Šis registras bus naudojamas GVD ataskaitų teikimo kokybės kontrolės tikslais, visų pirma siekiant patikrinti ryšį tarp vandens telkinių ir vandens gavybos vietoms skirtų vandens baseinų.

Trečiojo Upių baseinų valdymo planų ciklo metu vandens telkiniai buvo teikiami pagal:

„Water Framework Directive – River Basin Management Plans – 2022 Spatial Data“.

Šiuo metu vyksta pasirengimas ketvirtajam RBMP ciklui. Numatoma galimybė paviršinio ir požeminio vandens telkinius teikti nepriklausomai nuo kitų erdvinių duomenų rinkinių.

Taip pat planuojama:

- supaprastinti duomenų modelį;

- pereiti nuo Shapefile prie GeoPackage formato;

- pereiti nuo Reportnet2 prie Reportnet3 platformos.

Numatytasis GVD 8 straipsnio ataskaitų teikimo atskaitos taškas bus vandens telkiniai, kurie buvo pateikti trečiojo RBMP ciklo metu ir jau yra įtraukti į WISE registrą. Tačiau tikėtina, kad kai kurios valstybės narės norės juos atnaujinti rengdamosi tiek ketvirtajam RBMP ciklui, tiek GVD rizikos vertinimo ataskaitų teikimui.

A4 – ProtectedArea (Saugoma teritorija)

WISE Europos VPD saugomų teritorijų registras skelbiamas adresu:

https://dd.eionet.europa.eu/vocabulary/wise/WFDProtectedArea

Šis registras bus naudojamas GVD ataskaitų teikimo kokybės kontrolei, ypač tikrinant ryšius tarp apsaugos zonų ir vandens telkinių arba vandens gavybos vietoms skirtų vandens baseinų.

Trečiojo Upių baseinų valdymo planų ciklo metu buvo teikiami geriamojo vandens apsaugos teritorijų duomenys. Siekiant supaprastinti būsimą ataskaitų teikimą buvo atlikta išsami šių duomenų analizė.

Šiuo metu rengiamas konsoliduotas duomenų rinkinys, kuris apims informaciją, pateiktą pagal:

- „Water Framework Directive – River Basin Management Plans – 2022 Spatial Data“;

- ir (arba)

- „Water Framework Directive – River Basin Management Plans – 2022 RBD XML Data“.

Šis konsoliduotas rinkinys taps iš anksto užpildytu baziniu duomenų rinkiniu tiek:

- GVD 8 straipsnio rizikos vertinimo ataskaitoms;

- ketvirtajam RBMP ciklui.

Kaip minėta anksčiau, atnaujintas modelis:

- turės pasirenkamą atributą confidentialityStatus;

- reikalaus nurodyti saugomos teritorijos tipą;

- reikalaus saugomą teritoriją susieti su vandens telkiniu arba vandens gavybos vietoms skirtu vandens baseinu.

Taip pat:

- duomenų formatas bus keičiamas iš Shapefile į GeoPackage;

- duomenų teikimas bus perkeltas iš Reportnet2 į Reportnet3;

- šie duomenys bus teikiami tik GVD rizikos vertinimo duomenų sraute.

Apibendrinimas

Geriamojo vandens apsaugos teritorijos, įskaitant apsaugos zonas, galės būti atnaujinamos pagal GVD 8 straipsnio rizikos vertinimo ataskaitų teikimą. Papildomų ataskaitų ketvirtajam RBMP ciklui teikti nebereikės.

Santrauka

Didžioji dalis erdvinių duomenų, reikalingų GVD 8 straipsnio rizikos vertinimui, jau buvo pateikta pagal kitas ataskaitų teikimo prievoles. Jei reikia, šie duomenys gali būti atnaujinti tiek GVD, tiek ketvirtojo RBMP ciklo poreikiams.

Vienintelis visiškai naujas duomenų rinkinys yra:

CatchmentArea (vandens gavybos vietoms skirtas vandens baseinas).

1 lentelėje pateikiamas CatchmentArea ir WaterSupplyZone duomenų modelių palyginimas. Šių dviejų modelių struktūra yra beveik identiška, siekiant supaprastinti ataskaitų teikimą.

CatchmentArea modelyje pašalinti atributai:

- sizeValue

- sizeUom

kadangi jie gali būti apskaičiuojami pagal privalomą poligono geometriją.

Atributai:

- relatedZoneIdentifier

- relatedZoneIdentifierScheme

aiškiai nurodo ryšį tarp kiekvieno CatchmentArea ir VPD vandens telkinio.

Dalis atributų turi fiksuotas reikšmes:

- thematicIdIdentifierScheme;

- predecessorsIdentifierScheme;

- successorsIdentifierScheme;

- zoneType;

- specialisedZoneType.

Jos paliktos siekiant užtikrinti suderinamumą su kitomis ataskaitų teikimo prievolėmis.

Kiti atributai turi numatytąsias reikšmes:

- legalBasisName;

- legalBasisLink;

- legalBasisLevel,

tačiau valstybės narės gali naudoti ir nacionalines reikšmes.

1lentelė. CatchmentArea ir WaterSupplyZone duomenų modelių palyginimas

  - GVD 8 straipsnio duomenų srautui būdingi aprašomieji duomenys

Ankstesniame skyriuje buvo pristatyti svarbiausi erdviniai duomenų rinkiniai:

- CatchmentArea;

- ProtectedArea.

Šiame skyriuje aptariami vadinamieji aprašomieji duomenys, svarbūs GVD 8 straipsnio duomenų srautui:

- RiskAssessmentRiskManagement;

- WaterSupplyLink (2 pav., 2 lentelė).

2 pav. Naujos redakcijos Geriamojo vandens direktyvos (GVD) 8 straipsnyje nustatyto elektroninio duomenų teikimo klasių diagrama

Duomenų rinkinys RiskAssessmentRiskManagement pateikia kiekvieno vandens gavybos vietoms skirto vandens baseino GVD 8 straipsnio rizikos vertinimo būklės apžvalgą konkrečiai atskaitos datai.

Tai paprastas klausimynas, leidžiantis valstybėms narėms pateikti papildomus dokumentus arba nuorodas į nacionalines informacines sistemas.

Duomenų rinkinio WaterSupplyLink paskirtis yra spręsti daugelio su daugeliu ryšio problemą tarp vandens paėmimo objektų ir vandens tiekimo zonų (1 pav., 2.2 dalis).

Svarbu pažymėti, kad waterAbstractionIdentifier yra neprivalomas ir waterAbstractionName taip pat yra neprivalomas. Šie atributai įtraukti tik tam, kad valstybėms narėms būtų paprasčiau eksportuoti duomenis iš nacionalinių informacinių sistemų.

Vieninteliai privalomi atributai yra catchmentAreaIdentifier ir waterSupplyZoneIdentifier. Jų kombinacija turi būti unikali visame duomenų rinkinyje.

2 lentelė. RiskAssessmentRiskManagement ir WaterSupplyLink duomenų rinkinių detalizacija

2.5 Pastaba dėl vandens gavybos vietoms skirtų vandens baseinų charakterizavimo

„Žemės naudojimo, paviršinio nuotėkio ir požeminio vandens papildymo procesų aprašymas vandens gavybos vietoms skirtuose vandens baseinuose“ yra privaloma pirminio vandens gavybos vietoms skirtų vandens baseinų charakterizavimo dalis pagal Direktyvos (ES) 2020/2184 8 straipsnio 2 dalies a punkto iv papunktį.1

2 lentelėje daugiausia dėmesio skiriama supaprastintiems elektroninio ataskaitų teikimo reikalavimams, todėl šiuo metu joje nėra atributų, aprašančių kiekviename vandens gavybos vietoms skirtame vandens baseine esančias žemės naudojimo kategorijas. Tokia informacija Europos lygmeniu gali būti nuosekliai išvedama naudojant „Copernicus“ duomenų rinkinius.

Panašus principas gali būti taikomas ir kitoms charakteristikoms, pavyzdžiui:

- reljefo nuolydžiui;

- ilgalaikiam potencialiam požeminio vandens papildymui (mm/metus);

- upių debitams;

- paviršinio nuotėkio rodikliams.

Toliau pateikti du sąrašai rodo, kaip vandens gavybos vietoms skirtų vandens baseinų charakteristikos galėtų būti nustatomos GIS analizės būdu, naudojant valstybių narių pateiktas CatchmentArea ribas ir atitinkamus Copernicus duomenis.

RunOff – vyraujantis paviršinio nuotėkio pobūdis ir vidutinio nuolydžio klasė

| Kodas | Aprašymas |
| --- | --- |
| TypeSlowSteady | Vyraujantis lėtas ir tolygus paviršinis nuotėkis (slow/steady) |
| TypeRapidFlashy | Vyraujantis greitas ir staigus paviršinis nuotėkis (rapid/flashy) |
| TypeSeasonal | Vyraujantis sezoninis arba kintantis paviršinis nuotėkis (seasonal/variable) |
| TypeUnknown | Paviršinio nuotėkio pobūdis nežinomas arba netaikomas (unknown/Not applicable) |
| SlopeFlat | Vidutinis baseino nuolydis <2 % |
| SlopeModerate | Vidutinis baseino nuolydis nuo 2-15 % |
| SlopeSteep | Vidutinis baseino nuolydis >15 % |
| SlopeUnknown | Nuolydis nežinomas arba netaikomas (unknown/Not applicable) |

RechargeProcesses – papildymo procesų tipas ir intensyvumas

| Kodas | Aprašymas |
| --- | --- |
| TypeNatural | Natūralus papildymo procesas (krituliai, infiltracija) (precipitation/infiltration) |
| TypeArtificial | Dirbtinis papildymo procesas (valdomas papildymas arba injekcija) (managed recharge/injection) |
| TypeMixed | Mišrus natūralus ir dirbtinis papildymas |
| TypeUnknown | Papildymo procesas nežinomas arba netaikomas (unknown/Not applicable) |
| RateHigh | Vidutinis metinis požeminio vandens papildymas > 300 mm/metus; stipri infiltracija ir nuolatinis pasipildymas |
| RateAverage | Vidutinis metinis požeminio vandens papildymas 100-300 mm/metus; vidutinė infiltracija ir vandeningojo sluoksnio pasipildymas |
| RateLow | Vidutinis metinis požeminio vandens papildymas < 100 mm/metus; ribota infiltracija, papildymas silpnas arba sezoninis |
| RateUnknown | Papildymo intensyvumas nežinomas arba netaikomas (Unknown recharge rate/Not applicable) |

2.6 Suderinimas su kitais duomenų srautais – dokumentai ir metaduomenys

Kaip minėta ankstesniame skyriuje, valstybės narės gali pateikti papildomą informaciją pridėtiniuose dokumentuose arba nacionalinėse informacinėse sistemose. Siūlomas mechanizmas yra analogiškas tam, kuris jau naudojamas kituose WISE duomenų srautuose (3 lentelė).

Kiekvienas DocumentReference objektas iš esmės yra žyma arba nuoroda, nurodanti, kur galima rasti papildomą informaciją tam tikru klausimu konkrečiame Document objekte.

Document sąvoka yra bendro pobūdžio. Ji gali reikšti fizinį dokumentą (pvz., įkeltą PDF failą); nuolatinę internetinę nuorodą nacionalinėje sistemoje; API užklausą į informacinę sistemą, grąžinančią aktualią informaciją apie konkrečiam vandens gavybos vietoms skirtam vandens baseinui taikomas priemones.

dcMetadata duomenų rinkinyje naudojamas supaprastintas Dublin Core metaduomenų elementų rinkinys.

Dažniausiai lentelėje bus vienas įrašas kiekvienai valstybei narei, nors kai kuriuose duomenų srautuose (pvz., pagal VPD arba Potvynių direktyvą) gali būti atskiri įrašai skirtingiems valdymo vienetams.

Siūlomame modelyje Europos Komisijos prašymu įtrauktas papildomas atributas: numberOfCatchmentAreas

Bendras pateikiamų vandens gavybos vietoms skirtų vandens baseinų skaičius turi būti neneigiamas sveikasis skaičius; lygus faktiškai pateiktų vandens gavybos vietoms skirtų vandens baseinų skaičiui.

3 lentelė. DocumentReference, Document ir dcMetadata duomenų rinkinių specifikacija

2.7 Suderinimas su kitais duomenų srautais – stebėsenos rezultatai

GVD 8 straipsnio rizikos vertinimui ir vėlesnei stebėsenai nereikia kurti papildomų duomenų rinkinių ar naujų duomenų srautų.

Tam bus naudojamas gerai žinomas ir visose valstybėse narėse taikomas kasmetinis duomenų srautas WISE SoE Water Quality (WISE-6) (3 pav.).

WISE-6 naudojamas savanoriškam duomenų teikimui ir ataskaitoms pagal ES teisės aktus (pvz. Aplinkos kokybės standartų direktyvos stebėjimo sąrašui (Watch List)), kitoms vandens kokybės stebėsenos prievolėms.

Tas pats mechanizmas gali ir turi būti naudojamas GVD stebėsenos rezultatams teikti. Kaip ir Watch List atveju, duomenų teikėjai gali papildomai pažymėti atskirus įrašus kaip susijusius su GVD.

Informacija apie ataskaitų šablonus, metodines rekomendacijas, jau pateiktus nacionalinius duomenis pagal 2025 m. WISE-6 ir Watch List kvietimą skelbiama Reportnet3 platformoje https://reportnet.europa.eu/public/dataflow/1610.

3 pav. Klasių diagrama, skirta stebėsenos rezultatų teikimui pagal WISE SoE - Water quality (WISE-6) ataskaitų teikimo sistemą

1 priedas. Vandens gavybos vietoms skirtų vandens baseinų ataskaitų teikimas

Šiame priede pateikiama vandens gavybos vietoms skirto vandens baseino (CatchmentArea) duomenų rinkinio apžvalga, skirta duomenų teikėjams, nesusipažinusiems su WISE erdvinių duomenų teikimo reikalavimais

Apžvalga

- GVD 8 straipsnio ataskaitų teikimo vienetas yra vandens gavybos vietoms skirtas vandens baseinas (CatchmentArea).

- Šiame dokumente vandens gavybos vietoms skirtas vandens baseinas apibrėžiamas kaip teritorija „nustatyta pagal teisės aktuose įtvirtintą reikalavimą, susijusį su aplinkos politika arba politika ar veikla, galinčia turėti poveikį aplinkai bet kuriuo valdymo lygmeniu (tarptautiniu, Europos, nacionaliniu ar subnacionaliniu)“ (žr. INSPIRE temos Area Management/Restriction/Regulation Zones and Reporting Units (AM) aprašą) o konkrečiai – zonos, nustatytos pagal Geriamojo vandens direktyvos (GVD) 8 straipsnį. (http://inspire.ec.europa.eu/theme/am).

- Kiekvienas ataskaitų teikimo vienetas turi turėti unikalų nuolatinį Europos identifikatorių.

- Europos aplinkos agentūra prižiūrės tam skirtą identifikatorių schemą.

- Esama schema euDrinkingWaterReportingUnitCode bus išplėsta ir CatchmentArea objektams.

Specialieji reikalavimai

Vandens gavybos vietoms skirtas vandens baseinas (Catchment areas):

- privalo turėti poligoninę geometriją;

- privalo būti susietas su vienu ir tik vienu VPDD vandens telkiniu; Vandens telkinys (WaterBody) gali neturėti nė vieno arba turėti vieną ar daugiau susijusių vandens gavybos vietoms skirtų vandens baseinų (CatchmentArea).

Pastaba: teikiama tik informacija apie ryšį tarp vandens gavybos vietoms skirto vandens baseino (CatchmentArea) ir vandens telkinio (WaterBody). Atvirkštinis ryšys neteikiamas, duomenys apie pačius vandens telkinius neteikiami.

- turi būti susietas bent su viena vandens gavybos vieta.

Pastaba: vandens gavybos vietų duomenų teikti nereikalaujama.

- Informacija apie nacionalinius identifikatorius saugoma atributuose: inspireIdLocalId, inspireIdNamespace, inspireIdVersionId, beginLifespanVersion, endLifespanVersion. Ši informacija leidžia duomenų teikėjams užtikrinti pateiktų duomenų atsekamumą ir jų susiejimą su nacionalinėse sistemose ir (arba) nacionaliniuose ar regioniniuose INSPIRE duomenų rinkiniuose esančiais duomenimis.

- Nuolatinis Europos identifikatorius pateikiamas lauke thematicIdIdentifier ir turi būti unikalus schemoje euDrinkingWaterReportingUnitCode (visada 'euDrinkingWaterReportingUnitCode')

- Pokyčiams registruoti naudojami 5 atributai: predecessorsIdentifier, predecessorsIdentifierScheme, successorsIdentifier, successorsIdentifierScheme, wiseEvolutionType.

- Pirmojo duomenų teikimo ciklo metu šią informaciją pateikti paprasta: visiems vandens gavybos vietoms skirtiems vandens baseinams (CatchmentArea) nurodoma [wiseEvolutionType] = 'creation', o visų kitų atributų reikšmės yra NULL.

- Atributų nameTextInternational, nameText, nameLanguage apibrėžtys atitinka bendrąsias apibrėžtis, taikomas visuose WISE erdvinių duomenų modeliuose.

- Rizikos vertinimo vandens gavybos vietoms skirtame vandens baseine galiojimo laikotarpis raportuojamas naudojant atributus designationPeriodBegin, designationPeriodEnd. Pirmojo raportavimo ciklo metu designationPeriodEnd = NULL

- Šiems dviem atributams visiems vandens baseinams (CatchmentArea) nustatomos pastovios reikšmės:
• [zoneType] = 'drinkingWaterRiskAssessment'
• [specialisedZoneType] = 'catchmentArea'

- Numatytosios 3 teisinio pagrindo reikšmės

[legalBasisName] = 'Directive (EU) 2020/2184 of the European Parliament and of the Council of 16 December 2020 on the quality of water intended for human consumption (recast).'

[legalBasisLink] = 'https://eur-lex.europa.eu/eli/dir/2020/2184/oj'

[legalBasisLevel] = 'european'

- Šie du atributai susieja kiekvieną vandenvietės baseiną (CatchmentArea) su galiojančiu Vandens pagrindų direktyvos (VPD) paviršinio vandens telkiniu arba požeminio vandens telkiniu: [relatedZoneIdentifier] ir [relatedZoneIdentifierScheme].

- Galiausiai pasirenkamas atributas [link] leidžia pateikti veikiančią URL nuorodą į nacionalines sistemas, kuriose pateikiama išsamesnė informacija.

Išvada

Vandens gavybos vietoms skirtų vandens baseinų duomenų teikimas grindžiamas modeliu, panašiu į vandens tiekimo zonų duomenų teikimo modelį.

Pažymėtina, kad praktiniu požiūriu – ir pirmojo duomenų teikimo ciklo metu – būtina pateikti tik ribotą informacijos kiekį:

• [geometry] – geometrija;

• [thematicIdIdentifier] – unikalus identifikatorius;

• [nameTextInternational], [nameText], [nameLanguage] – pavadinimas (-ai);

• [designationPeriodBegin] – galiojimo laikotarpio pradžia;

• [relatedZoneIdentifier], [relatedZoneIdentifierScheme] – susijęs vandens telkinys.

Kalbant apie nacionalinių ir Europos identifikatorių skirtumą, duomenų teikėjai gali pasirinkti naudoti tą pačią reikšmę, jei nacionalinis identifikatorius atitinka WISE reikalavimus (apie 70 proc. vandens tiekimo zonų atitinka šį modelį, todėl tai dar labiau supaprastina duomenų teikimą duomenų teikėjų požiūriu).

Visi likę atributai turi fiksuotas reikšmes, numatytąsias reikšmes arba yra neprivalomi. Jie įtraukti į duomenų teikimo modelį siekiant užtikrinti aiškumą ir palengvinti pakartotinį duomenų panaudojimą.

2 priedas – geriamojo vandens apsaugos zonų raportavimas

Geriamojo vandens direktyvos (GVD) 8 straipsnio 2 dalies a punkto ii papunktyje nustatyta, kad žmonėms vartoti skirto vandens gavybos vietų vandenvietės baseinų rizikos vertinimas ir rizikos valdymas turi apimti „apsaugos zonų žemėlapių sudarymą, kai šios zonos yra nustatytos pagal Direktyvos 2000/60/EB 7 straipsnio 3 dalį“.

Šio priedo tikslas:

1) patikrinti, ar tokios zonos jau buvo pateiktos valstybių narių (VN) pagal Vandens pagrindų direktyvą (VPD) vykdyto 3-iojo upių baseinų valdymo planų (3-iojo UVPD) duomenų teikimo ciklo metu;

2) patikrinti, ar VPD ir GVD duomenų teikimas gali būti konsoliduotas ir supaprastintas, siekiant gauti geresnės kokybės duomenis ir išvengti dvigubo duomenų teikimo.

Skirsnyje „Pagrindinė informacija“ apibendrinamos teisinės nuorodos į geriamojo vandens apsaugos teritorijas ir apsaugos zonas tiek VPD, tiek naujos redakcijos GVD kontekste.

Skirsnyje „Duomenų teikimo gairės 2-uosiuose ir 3-iuosiuose UVPD“ apibendrinamos Komisijos gairės dėl šių teritorijų elektroninio duomenų teikimo pagal VPD.

Skirsnyje „Geriamojo vandens apsaugos teritorijos 3-iuosiuose UVPD“ pateikiami duomenys, jau pateikti pagal 3-iuosius UVPD, naudojant pavyzdžius, iliustruojančius klausimus, į kuriuos turėtų būti atsižvelgta siūlomame modelyje arba kuriuos gali reikėti papildomai aptarti su konkrečiomis valstybėmis narėmis.

Skirsnyje „Duomenų teikimo konsolidavimas“ pateikiama siūlomo modelio apžvalga, kad būtų galima vykdyti informacija pagrįstas diskusijas su valstybių narių atstovais, kurie gali būti nesusipažinę su modeliu, jau taikytu 3-iuosiuose UVPD.

Bendroji informacija

Geriamojo vandens direktyvos (GVD) 8 straipsnio 2 dalies a punkto ii papunktyje nustatyta, kad žmonėms vartoti skirto vandens gavybos vietų vandenvietės baseinų rizikos vertinimas ir rizikos valdymas turi apimti „apsaugos zonų žemėlapių sudarymą, kai šios zonos yra nustatytos pagal Direktyvos 2000/60/EB 7 straipsnio 3 dalį“.

Vandens pagrindų direktyvos (VPD) 7 straipsnyje dėl žmonėms vartoti skirto vandens gavybai naudojamų vandens telkinių nustatyta, kad:

1. Kiekviename upės baseino rajone valstybės narės nurodo:

— visus vandens telkinius, iš kurių kasdien vidutiniškai imama daugiau kaip 10 m3 arba daugiau kaip penkiasdešimčiai asmenų vartoti skirto vandens,

— tuos vandens telkinius, kurie bus skirti tokiam vartojimui ateityje. Valstybės narės, vadovaudamosi V priedu, stebi tuos vandens telkinius, iš kurių pagal V priedą kasdien imama daugiau kaip 100 m3 vandens.

Valstybės narės užtikrina būtiną nurodytų vandens telkinių apsaugą ir stengiasi išvengti jų kokybės suprastėjimo, kad tiekiant geriamąjį vandenį reikėtų jį mažiau valyti. Valstybės narės gali nustatyti aplink tuos vandens telkinius apsaugos zonas.

Vandens pagrindų direktyvos (VPD) 6 straipsnio 2 dalyje nustatyta, kad saugomų teritorijų registre/ registruose registruojami visi vandens telkiniai, nurodyti pagal 7 straipsnio 1 dalį, ir visos saugomos teritorijos, kurioms taikomas IV priedas.

IV priede detalizuojama: Į saugomų teritorijų registrą, kurio reikalaujama vadovaujantis 6 straipsniu, įrašomos šio tipo saugojamos teritorijos:

  - teritorijos, nustatytos žmonėms vartoti skirto vandens ėmimui pagal 7 straipsnį;

Apibendrinant:

VPD saugomų teritorijų registras apima visus pagal VPD 7 straipsnio 1 dalį nustatytus vandens telkinius ir apsaugos zonas, kurios šiems vandens telkiniams gali būti nustatytos pagal VPD 7 straipsnio 3 dalį.

Taikant VPD 7 straipsnio 2 dalį, dabar taikomi naujos redakcijos Geriamojo vandens direktyvos (GVD) reikalavimai (pagal naujos redakcijos GVD 26 straipsnio 1 dalį).

Duomenų teikimo gairės 2-iesiems ir 3-iesiems UVPD

2016 m. vasario 23 d. Komisija atliko 1-ųjų upių baseinų valdymo planų (UVPD) analizę ir pateikė gaires dėl geriamojo vandens apsaugos zonų erdvinių duomenų teikimo rengiant 2-uosius UVPD (nuoroda):

„Valstybių narių praktika nustatant geriamojo vandens apsaugos teritorijas skiriasi. Analizuojant pirmuosius UVPD nustatyti du pagrindiniai požiūriai, kuriems pateikiamos erdvinių duomenų teikimo gairės:

a) Kai kurios valstybės narės tiesiogiai vadovaujasi VPD 7 straipsniu ir saugomomis teritorijomis nustato vandens telkinius, naudojamus žmonėms vartoti skirto vandens gavybai (taip pat numatytus naudoti šiuo tikslu ateityje). Todėl šiuo atveju saugomų teritorijų erdvinės ribos nėra nustatomos atskirai – jos sutampa su VPD vandens telkiniais […].

b) Kai kurios valstybės narės nustato tik tą vandens telkinių dalį, kurią laiko saugoma teritorija. Todėl saugomų teritorijų erdvinė aprėptis gali skirtis nuo VPD vandens telkinių ir paprastai būna mažesnė. Taip dažnai yra didelių požeminio vandens telkinių atveju. […].“

Toks požiūris buvo išlaikytas ir 2022 m. balandžio 28 d. Komisijai atnaujinus gaires, skirtas 3-iesiems UVPD (nuoroda).

Apibendrinant:
Supaprastintai galima teigti, kad elektroninio duomenų teikimo pagal UVPD metu valstybės narės turėjo nurodyti visą vandens telkinį ir (arba) apsaugos zoną (-as), remiantis numanoma prielaida, kad pagal VPD 7 straipsnio 3 dalį nustatytos apsaugos zonos bus apibrėžtos pagal VPD 7 straipsnio 1 dalį nustatytuose vandens telkiniuose.

Tačiau elektroninio duomenų teikimo praktika atskleidė sudėtingesnę situaciją, kuri aprašoma kitame skyriuje.

Geriamojo vandens apsaugos teritorijos 3-iajame Upių baseinų valdymo planų (RBMP) cikle

Šiame skyriuje pateikiami pavyzdžiai iliustruoja skirtingus valstybių narių taikytus metodus bei duomenų modelio ir pateiktų duomenų stipriąsias bei silpnąsias puses.

Pavyzdys Nr. 1. „Viso vandens telkinio“ metodas

Vokietija trečiojo RBMP ciklo metu taikė vadinamąjį „viso vandens telkinio“ metodą.

Erdviniai duomenys buvo pateikiami tik vandens telkinių erdviniuose duomenų rinkiniuose. Informacija, kad konkretus vandens telkinys yra geriamojo vandens apsaugos teritorija, buvo teikiama VPD aprašomuosiuose duomenyse. Kitos apsaugos zonos nebuvo teikiamos.

Praktiškai tai reiškia, kad:

tam pačiam objektui buvo naudojamas tas pats identifikatorius tiek vandens telkiniui, tiek geriamojo vandens apsaugos teritorijai.

Pavyzdys Nr. 2. Apsaugos zonos

Liuksemburgas pateikė: vieną paviršinio vandens apsaugos zoną rezervuaro baseine (4.a pav.) ir kelias požeminio vandens apsaugos zonas aplink geriamojo vandens gavybos vietas (4.b pav.).

Kaip matyti iš pateiktų duomenų, originaliuose VPD erdviniuose duomenyse nebuvo tiesioginio ryšio su vandens telkiniu, nors Liuksemburgo atveju ši informacija galėjo būti gauta iš kitame duomenų sraute pateiktų aprašomųjų VPD duomenų.

Taip pat iš karto nebuvo įmanoma nustatyti, ar saugoma teritorija reiškia:

- visą vandens telkinį;

- apsaugos zoną;

- ar tik vandens gavybos vietos lokaciją.

Europos mastu pasitaikė visi trys atvejai, nors Liuksemburge buvo naudojamas tik apsaugos zonos metodas.

| a) Surface water safeguard zone | b) Groundwater safeguard zone |
| --- | --- |

4 pav. Liuksemburgo pateiktos geriamojo vandens apsaugos teritorijos [šaltinis:3 UVPD].

Pavyzdys Nr. 3. Mišrūs metodai

Portugalijos teisės aktai numato apsaugos perimetrų nustatymą aplink vandens gavybos vietas.

Požeminio vandens atveju apsaugos zonos nustatomos atsižvelgiant į:

- vandens gavybos apimtį;

- vandeningojo sluoksnio hidrogeologines savybes.

Ypatingi reikalavimai taikomi, pavyzdžiui, karstinėse teritorijose. Analogiški principai taikomi ir paviršiniams vandenims.

Išsami erdvinė informacija apie šias zonas skelbiama nacionalinėse Portugalijos sistemose, tačiau trečiojo RBMP ciklo duomenų turinys buvo kitoks (5.a pav.).

Pateikti duomenys rodo, kad juose vienu metu buvo:

- visi vandens telkiniai kaip apsaugos teritorijos;

- taškinės lokacijos;

- kai kuriais atvejais apsaugos zonų reprezentacijos.

Dauguma apsaugos teritorijų buvo susietos su vandens telkiniais, tačiau dviejuose upių baseinų rajonuose (PTRH9 ir PTRH10) šios informacijos trūko.

| a) Internal dataset | b) Public dataset |
| --- | --- |

5 pav. Geriamojo vandens apsaugos teritorijos pateiktos už 3-ius UVPD

Pavyzdys Nr. 4. Konfidencialumo problemos

Trečiojo RBMP ciklo duomenų analizė (5 pav.) parodė, kad dalis Portugalijos ir Ispanijos pateiktos informacijos nebuvo paskelbta viešose Europos duomenų bazėse.

Priežastis buvo ta (6 pav.), kad Portugalijos pateikime visi failai buvo pažymėti kaip riboto naudojimo, o Ispanijos pateikime riboto naudojimo buvo tik failas su taškiniais apsaugos teritorijų duomenimis.

Kadangi duomenų modelis neleido nurodyti konfidencialumo konkrečių objektų lygiu, buvo taikomas netiesioginis kriterijus - jei failas pažymėtas kaip riboto naudojimo, jo duomenys neviešinami.

Dėl to visi Portugalijos duomenys buvo pašalinti iš viešo Europos rinkinio, o Ispanijos atveju nebuvo skelbiami taškinės geometrijos objektai.

Dokumento autoriai pažymi, kad toks sprendimas nėra tinkamas. Valstybės narės turėtų galėti nustatyti konfidencialumo statusą kiekvienam apsaugos teritorijos įrašui atskirai.

| a) PT delivery – all files restricted | b) ES delivery – only one file restricted |
| --- | --- |

6 pav. Riboto naudojimo failai pateikti raportuojant 3-ius UVPD

Pavyzdys Nr. 5. Struktūrizuoti duomenys ir PDF dokumentai

Dar vienas pavyzdys, remiantis Liuksemburgo situacija, parodė struktūrizuotų elektroninių duomenų svarbą.

2023 m. rugsėjo mėn. Liuksemburgas dar nebuvo pateikęs pilnų elektroninių trečiojo RBMP ciklo duomenų. Todėl Europos Komisijos vertinimas buvo atliktas remiantis tik RBMP dokumentais.

Dokumentuose buvo nurodytos tik 45 apsaugos zonos iš 55, kurios vėliau buvo pateiktos elektroniniu būdu 2024 m. birželio mėn.

Šie neatitikimai parodė:

- struktūrizuotų duomenų ir PDF dokumentų skirtumus;

- būtinybę užtikrinti elektroninių duomenų išsamumą;

- poreikį naudoti vieną bendrą duomenų rinkinį tiek GVD, tiek VPD vertinimams.

Santrauka: padėtis trečiojo RBMP ciklo metu

Trečiojo RBMP ciklo metu:

- valstybės narės teikė erdvinius duomenis apie geriamojo vandens apsaugos teritorijas;

- jeigu apsaugos teritorija sutapo su visu vandens telkiniu ir naudojo tą patį identifikatorių, jos geometrijos pateikti nereikėjo;

- ryšio tarp apsaugos teritorijos ir vandens telkinio nurodymas buvo neprivalomas;

- modelis neskyrė skirtingų geriamojo vandens apsaugos teritorijų tipų;

- modelis nenumatė galimybės pažymėti konfidencialių objektų;

- dėl to viešinimo sprendimai buvo priimami failų lygmeniu, o ne konkrečių objektų lygmeniu.

Problemos, nustatytos Europos duomenų rinkiniuose

Pavyzdys Nr. 6. Trūkstama informacija apie susijusį vandens telkinį

Trečiojo RBMP ciklo erdviniuose duomenyse ryšio tarp geriamojo vandens apsaugos teritorijos ir vandens telkinio pateikimas buvo neprivalomas.

Todėl tokia informacija buvo pateikta tik apie 22 % visų erdvinių objektų.

4 lentelė. Erdvinių duomenų teikimas: geriamojo vandens apsaugos teritorijos, susietos su vandens telkiniais [šaltinis – 3-ieji UVPD].

Pavyzdys Nr. 7. Aprašomieji duomenys apie susijusį vandens telkinį

Aprašomuosiuose duomenyse šio ryšio pateikimas buvo privalomas.

Analizė parodė, kad:

- 96 % apsaugos teritorijų buvo susietos tik su vienu vandens telkiniu (5 lentelė).

Todėl būtina išsiaiškinti tik sudėtingesnius atvejus, kartu išlaikant kuo paprastesnį modelį visoms valstybėms narėms.

5 lentelė. Aprašomųjų duomenų teikimas: kiekvienai geriamojo vandens apsaugos teritorijai priskirtų skirtingų vandens telkinių skaičius [šaltinis – 3-ieji UVPD].

Pavyzdys Nr. 8. Erdvinių ir aprašomųjų duomenų suderinamumas

Erdviniai ir aprašomieji duomenys buvo teikiami skirtingais duomenų srautais.

Trečiojo RBMP ciklo kokybės kontrolėje šių duomenų tarpusavio suderinamumas nebuvo privalomai tikrinamas.

Iš daugiau kaip 83 000 pateiktų apsaugos teritorijų:

- 85 % atvejų ryšys su vandens telkiniu galėjo būti nustatytas;

- 12 % atvejų ryšys liko nežinomas;

- 3 % atvejų išliko neapibrėžtumas (6 lentelė).

6 lentelė. Skirtingų vandens telkinių, susietų su kiekviena geriamojo vandens apsaugos teritorija, skaičius [šaltinis – 3-ieji UVPD].

Išvados

- Siekiant išvengti dvigubo duomenų teikimo, turi būti sukurtas atskiras geriamojo vandens apsaugos teritorijų duomenų srautas, paremtas iš anksto užpildytais trečiojo RBMP ciklo duomenimis.

- Duomenys turi būti teikiami tik vieną kartą ir naudojami tiek GVD, tiek ketvirtojo RBMP ciklo reikmėms.

- Ryšys tarp apsaugos teritorijos ir vandens telkinio turi tapti privalomas.

- Atributui specialisedZoneType turi būti sukurtas oficialus klasifikatorius, leidžiantis atskirti „viso vandens telkinio“ ir „apsaugos zonos“ metodus.

- Modelyje turi būti įtrauktas papildomas atributas confidentialityStatus, leidžiantis neviešinti konkrečių apsaugos teritorijų lokalizacijos duomenų. Šis principas jau taikomas WISE MonitoringSite modelyje.

Ataskaitų teikimo konsolidavimas

Geriamojo vandens apsaugos teritorijų erdviniai duomenys turi būti teikiami naudojant WISE ProtectedArea modelį. Šis modelis jau buvo naudotas 2-ajame ir 3-ajame RBMP cikluose, todėl valstybėms narėms jis yra žinomas ir nereikalauja esminių pakeitimų.

Pagrindinis ataskaitų teikimo vienetas yra:

Geriamojo vandens apsaugos teritorija (Drinking Water Protection Area).

• Kiekvienas ataskaitinis vienetas turi turėti unikalų nuolatinį identifikatorių Europos lygmeniu.

• Siekdama užtikrinti identifikatorių unikalumą ir pastovumą, Europos aplinkos agentūra tvarko šiam tikslui skirtą identifikatorių schemą.

• Šiuo metu visoms pagal Vandens pagrindų direktyvą (VPD) nustatytoms saugomoms teritorijoms, įskaitant geriamojo vandens apsaugos teritorijas, naudojama identifikatorių schema euProtectedAreaCode.

• Europos VPD saugomų teritorijų registras skelbiamas adresu http://dd.eionet.europa.eu/vocabulary/wise/WFDProtectedArea/.

Unikalaus nuolatinio identifikatoriaus naudojimas leidžia tvarkyti Europos VPD saugomų teritorijų registrą.

Europos lygmens unikalus nuolatinis identifikatorius pateikiamas atribute [thematicIdIdentifier]. Jis turi atitikti WISE identifikatoriams nustatytus sintaksės reikalavimus ir būti unikalus atributo [thematicIdIdentifierScheme] reikšmės ribose, kuri visada yra 'euProtectedAreaCode'.

• Informacija apie nacionalinius saugomų teritorijų identifikatorius saugoma atributuose [inspireIdLocalId], [inspireIdNamespace], [inspireIdVersionId], [beginLifespanVersion] ir [endLifespanVersion]. Tai leidžia duomenų teikėjams užtikrinti pateiktų duomenų atsekamumą ir jų susiejimą su nacionalinėse sistemose ir (arba) nacionaliniuose ar regioniniuose INSPIRE duomenų rinkiniuose esančiais duomenimis.

• Duomenų teikėjams bus parengti iš anksto užpildyti „GeoPackage“ failai, kuriuose bus konsoliduota informacija, jau pateikta pagal 3-iuosius upių baseinų valdymo planus (UVPD).

• Jei duomenų atnaujinti nereikia, jų pakartotinai teikti nereikia.

• Jei sukurtos naujos zonos, esamos zonos panaikintos arba pakeistos, ši informacija turi būti pateikta, o visiems kitiems įrašams turi būti palikta atributo [wiseEvolutionType] reikšmė 'noChange'.

Geriamojo vandens apsaugos teritorijos geometrija ([geometry]) gali būti taškas, linija arba poligonas.

• Valstybės narės, taikančios „viso vandens telkinio“ metodą, gali pakartotinai naudoti vandens telkinio geometriją. Jos gali pasirinkti saugomai teritorijai ir vandens telkiniui naudoti tą patį identifikatorių (pagal skirtingas identifikatorių schemas) arba skirtingus identifikatorius.

• Kiekviena geriamojo vandens apsaugos teritorija privalo būti susieta su vienu ir tik vienu vandens telkiniu.

1 pastaba. Reikalaujama pateikti tik ryšį tarp geriamojo vandens apsaugos teritorijos ir vandens telkinio. Atvirkštinis ryšys neteikiamas.

• Su vienu vandens telkiniu gali būti susieta nuo nulio iki kelių geriamojo vandens apsaugos teritorijų.

2 pastaba. Duomenys apie vandens telkinius neteikiami. Vandens telkinių identifikatorių galiojimui tikrinti bus naudojamas Europos paviršinio ir požeminio vandens telkinių registras.

• Ryšys tarp saugomos teritorijos ir vandens telkinio turi būti pateikiamas atributuose [relatedZone] ir gali būti nustatomas naudojant vandens telkinio identifikatorių arba Geriamojo vandens direktyvos (GVD) vandenvietės baseino identifikatorių.

Duomenų teikimo modelyje numatyti penki atributai, leidžiantys duomenų teikėjams pateikti informaciją apie saugomos teritorijos pakeitimus, t. y. gyvavimo ciklo informaciją: [predecessorsIdentifier], [predecessorsIdentifierScheme], [successorsIdentifier], [successorsIdentifierScheme] ir [wiseEvolutionType].

• Atributų [predecessorsIdentifier] ir [predecessorsIdentifierScheme] reikšmės pateikiamos tik tuo atveju, jei esama saugoma teritorija buvo iš esmės pakeista ir dėl to jai turi būti suteiktas naujas identifikatorius.

• Paprastai atributų [successorsIdentifier] ir [successorsIdentifierScheme] reikšmių pateikti nereikia. Jos paliktos modelyje siekiant užtikrinti jo išsamumą ir todėl, kad naudojamos skelbiant Europos duomenų rinkinius (siekiant užtikrinti anksčiau pateiktų ir šiuo metu galiojančių duomenų atsekamumą).

• Atributų [nameTextInternational], [nameText] ir [nameLanguage] apibrėžtys atitinka bendrąsias apibrėžtis, taikomas visuose WISE erdvinių duomenų modeliuose.

• Geriamojo vandens apsaugos teritorijos galiojimo laikotarpis pateikiamas atributuose [designationPeriodBegin] ir [designationPeriodEnd].

• Šiems dviem atributams visoms geriamojo vandens apsaugos teritorijoms nustatomos fiksuotos reikšmės:

• [zoneType] = 'drinkingWaterProtectionArea'

• [specialisedZoneType] = <codelist to be defined>

• Nors atributas [specialisedZoneType] anksčiau nebuvo naudojamas geriamojo vandens apsaugos teritorijoms, jį reikėtų naudoti bent jau tam, kad būtų aiškiai atskirtas „viso vandens telkinio“ metodas ir „apsaugos zonos“ metodas.

• Šiems trims atributams visoms geriamojo vandens apsaugos teritorijoms nustatomos numatytosios reikšmės, tačiau prireikus gali būti pateikiamos ir konkrečios nacionalinės reikšmės:

• [legalBasisName] = „'Directive (EU) 2020/2184 of the European Parliament and of the Council of 16 December 2020 on the quality of water intended for human consumption (recast).“

• [legalBasisLink] = 'https://eur-lex.europa.eu/eli/dir/2020/2184/oj'

• [legalBasisLevel] = 'european'

• Šie du atributai susieja kiekvieną saugomą teritoriją su galiojančiu Vandens pagrindų direktyvos (VPD) paviršinio vandens telkiniu arba požeminio vandens telkiniu: [relatedZoneIdentifier] ir [relatedZoneIdentifierScheme].

• Atributas [confidentialityStatus] nebuvo VPD duomenų modelio dalis. Jį reikėtų įtraukti, kad duomenų teikėjai galėtų apriboti konkrečių saugomų teritorijų buvimo vietos skelbimą.

• Galiausiai pasirenkamas atributas [link] leidžia pateikti veikiančią URL nuorodą į nacionalines sistemas, kuriose pateikiama išsamesnė informacija.

Išvada

Geriamojo vandens apsaugos teritorijų ataskaitų teikimas bus grindžiamas tuo pačiu modeliu, kuris iki šiol buvo naudojamas Vandens pagrindų direktyvos (VPD) ataskaitų teikime.

Bus sukurtas atskiras duomenų srautas, skirtas tik geriamojo vandens apsaugos teritorijoms teikti. Tai leis atskirti šį procesą nuo VPD erdvinių duomenų teikimo ciklo ir suteiks duomenų teikėjams galimybę atnaujinti anksčiau pateiktus duomenis tada, kada to reikės.

Duomenų teikėjams bus parengti iš anksto užpildyti GeoPackage failai, kuriuose bus sujungta informacija iš VPD erdvinių ir aprašomųjų duomenų rinkinių.

Ataskaitų teikimo reikalavimai iš esmės bus tokie patys kaip pagal VPD, tačiau atsiras vienas svarbus naujas reikalavimas:

kiekviena apsaugos teritorija privalės būti susieta su vienu konkrečiu vandens telkiniu.

Kiekvienos apsaugos teritorijos konfidencialumo statusą galės nustatyti pats duomenų teikėjas.

3 priedas. Vandens gavybos vietų ir stebėsenos vietų ataskaitų teikimas

Direktyvos (ES) 2020/2184 8 straipsnio 2 dalies a punkto iii papunktis nustato, kad vandens gavybos vietoms skirtų vandens baseinų rizikos vertinimas ir rizikos valdymas turi apimti:

„visų vandens gavybos vietų vandens baseinuose geografines nuorodas; kadangi šie duomenys gali būti jautrūs, ypač visuomenės sveikatos ir visuomenės saugumo požiūriu, valstybės narės turi užtikrinti, kad tokie duomenys būtų apsaugoti ir perduodami tik kompetentingoms institucijoms bei vandens tiekėjams“.

Vandens gavybos vietų ir stebėsenos vietų teikti nereikia

Vandens gavybos vietų ir stebėsenos vietų atskiras ataskaitų teikimas nėra reikalaujamas.

Pagrindimas

- Stebėsenos vietų koordinatės jau buvo pateiktos pagal trečiojo Upių baseinų valdymo planų ciklo ataskaitas pagal Vandens pagrindų direktyvą.

- Kiekviena stebėsenos vieta gali būti pažymėta kaip konfidenciali. Tokių vietų koordinatės nėra viešinamos jokiuose Europos duomenų produktuose.

- Kiekviena stebėsenos vieta gali turėti vieną arba kelias paskirtis (purpose). Tai leidžia pažymėti, kad vieta naudojama konkrečioje stebėsenos programoje arba pagal tam tikrą teisinę prievolę. Žyma „GVD“ („DWD“) naudojama geriamojo vandens stebėsenai svarbioms vietoms identifikuoti.

- Siekiant išvengti dvigubo duomenų teikimo ir sudaryti galimybę atnaujinti jau pateiktus duomenis, bus sukurtas atskiras stebėsenos vietų duomenų srautas, paremtas iš anksto užpildytais VPD duomenimis.

- Tai leis duomenų teikėjams atnaujinti informaciją tada, kada to prireiks, nepriklausomai nuo konkrečių sektorinių ataskaitų teikimo ciklų (Geriamojo vandens direktyvos, Nitratų direktyvos, Vandens pagrindų direktyvos ir kt.).

- Kadangi modelis jau naudojamas ir valstybėms narėms pažįstamas, toliau pateikiama tik trumpa jo apžvalga, ypatingą dėmesį skiriant GVD poreikiams.

- Pagal VPD buvo pateikti duomenys apie 217 634 stebėsenos vietas. Tik 9 731 vieta buvo aiškiai pažymėta žyma „GVD“, o jų koordinatės neįtraukiamos į viešuosius Europos duomenų produktus. Dar apie 16 500 vietų buvo pažymėtos kaip konfidencialios, todėl jų vieta taip pat neviešinama.

Trumpas MonitoringSite modelio aprašymas

- Kiekviena stebėsenos vieta turi unikalų nuolatinį Europos identifikatorių.

- Europos aplinkos agentūra administruoja Europos stebėsenos vietų registrą.

- Registras skelbiamas adresu:

http://dd.eionet.europa.eu/vocabulary/wise/MonitoringSite/

Pagrindiniai reikalavimai

- Stebėsenos vieta privalo turėti taško geometriją.

- Kiekviena stebėsenos vieta turi būti susieta su vienu ir tik vienu vandens telkiniu. Vienas vandens telkinys gali turėti daug stebėsenos vietų.

- Teikiamas tik ryšys tarp stebėsenos vietos ir vandens telkinio. Atvirkštinis ryšys neteikiamas. Vandens telkiniai šiame duomenų sraute neteikiami.

Nacionaliniai identifikatoriai saugomi atributuose:

- inspireIdLocalId

- inspireIdNamespace

- inspireIdVersionId

- beginLifespanVersion

- endLifespanVersion1

Šie atributai užtikrina atsekamumą tarp Europos ir nacionalinių sistemų.

Nuolatinis Europos identifikatorius pateikiamas atribute:

thematicIdIdentifier

Jis turi būti unikalus schemoje:

euMonitoringSiteCode.

Modelyje naudojami Gyvavimo ciklo atributai:

- supersedesIdentifier

- supersedesIdentifierScheme

- supersededByIdentifier

- supersededByIdentifierScheme

- wiseEvolutionType

Jie leidžia registruoti stebėsenos vietų pakeitimus ir jų gyvavimo ciklą.

Duomenų teikėjams bus parengti iš anksto užpildyti GeoPackage failai.

- Jei pakeitimų nėra, duomenų teikti iš naujo nereikės.

- Jei vietos sukuriamos, panaikinamos ar pakeičiamos, šiuos pakeitimus bus galima pateikti, o likę įrašai išlaikys reikšmę:wiseEvolutionType = 'noChange'

• Atributų [nameTextInternational], [nameText] ir [nameLanguage] apibrėžtys atitinka bendrąsias apibrėžtis, taikomas visuose WISE erdvinių duomenų modeliuose.

• Bendras stebėsenos vietos veiklos laikotarpis pateikiamas atributuose [operationalActivityPeriodBegin] ir [operationalActivityPeriodEnd].

• Šie du atributai susieja kiekvieną stebėsenos vietą su galiojančiu Vandens pagrindų direktyvos (VPD) paviršinio vandens telkiniu arba požeminio vandens telkiniu: [featureOfInterestIdentifier] ir [featureOfInterestIdentifierScheme].

• Atributui [confidentialityStatus] gali būti priskirta reikšmė 'N', kad stebėsenos vietos buvimo vieta nebūtų skelbiama jokiuose būsimuose Europos duomenų leidiniuose.

• Atribute [purpose] gali būti nurodyta žyma 'DWD', kuri identifikuoja stebėsenos vietą kaip patenkančią į Geriamojo vandens direktyvos (GVD) stebėsenos programų taikymo sritį.

Svarbi pastaba: bet kuri stebėsenos vieta gali būti naudojama teikiant stebėsenos rezultatus pagal GVD 8 straipsnio 2 dalies c punktą.

Pasirenkamas atributas [link] leidžia pateikti aktyvią nuorodą į nacionalines sistemas, kuriose galima rasti papildomą informaciją.

Galutinė išvada

Vandens gavybos vietų ir stebėsenos vietų ataskaitų teikti nereikia.

Stebėsenos vietų ataskaitų teikimas ir toliau bus vykdomas pagal modelį, kuris jau naudojamas Vandens pagrindų direktyvos sistemoje.

Bus sukurtas atskiras stebėsenos vietų duomenų srautas, kuris bus nepriklausomas nuo VPD erdvinių duomenų teikimo ciklo ir leis atnaujinti duomenis tada, kai to reikės.

Duomenų teikėjams bus pateikti iš anksto užpildyti GeoPackage failai su konsoliduotais VPD erdviniais duomenimis.

Risk assessment and risk management 
of the catchment areas 
for abstraction points of 
water intended for human consumption

Imprint

Title: Risk assessment and risk management of the catchment areas for abstraction points of water intended for human consumption. Conceptual model proposal for the electronic reporting under the recast Drinking Water Directive Article 8. Version 2025-12-18.

Creator: Fernanda Néry, European Environment Agency
Contributors: Anke Schneeweiss, European Topic Centre Biodiversity and Ecosystems; Bert Leemans, European Commission; Fanny Gard, European Commission; Francesco Mundo, European Environment Agency; Jonathan Maidens, European Environment Agency; Jorgen Olsen, European Environment Agency.

Created: 2025-11-28

Language: eng

License: https://creativecommons.org/licenses/by/4.0/

Contents

1	Summary	1

2	Narrative description	2

2.1	Data model classes	2

2.2	Data model relationships	4

2.3	Alignment with other dataflows – spatial datasets	6

2.4	Descriptive data specific to the DWD Article 8 dataflow	10

2.5	Note on the characterisation of the catchment areas	11

2.6	Alignment with other dataflows – documents and metadata	13

2.7	Alignment with other dataflows – monitoring results	14

Annex 1 - Reporting of the catchment areas	15

Overview	15

Conclusion	17

Annex 2 - Reporting of drinking water protection areas	18

Background information	18

Reporting guidance in the 2nd and 3rd RBMPs	20

Drinking water protection areas in the 3rd RBMPs	21

Consolidating the reporting	27

Conclusion	30

Annex 3 - Reporting of abstraction points and monitoring sites	31

Rationale	31

Brief description of the MonitoringSite model	33

Conclusion	34

List of Figures

Figure 1. Conceptual model for the recast DWD Article 8 electronic reporting – overview.	2

Figure 2. Class diagram for the recast DWD Article 8 electronic reporting.	10

Figure 3. Class diagram for the reporting of monitoring results under WISE SoE - Water quality (WISE-6).	13

Figure 4. Drinking water protection areas reported by Luxembourg [source:3rd RBMPs].	20

Figure 5. Drinking water protection areas reported under the 3rd RBMPs [extract].	21

Figure 6. Restricted files in the 3rd RBMP deliveries.	22

Figure 7. Commission staff document: 3rd RBMP implementation assessment for Luxembourg [extract].	23

List of Tables

Table 1. Comparison between the CatchmentArea dataset and the WaterSupplyZone dataset.	9

Table 2. Details of the RiskAssessmentRiskManagement dataset and the WaterSupplyLink dataset.	11

Table 3. Details of the DocumentReference, Document and dcMetadata datasets.	12

Table 4. Spatial data reporting: drinking water protection areas associated with water bodies [source: 3rd RBMP].	24

Table 5. Descriptive data reporting: number of different water bodies related to each drinking water protection area [source: 3rd RBMP].	24

Table 6. Number of different water bodies related to each drinking water protection area [source: 3rd RBMPs]	25

# Summary

This document outlines a proposed conceptual model for the electronic reporting required under Article 8 of the recast Drinking Water Directive (DWD), on the risk assessment and management of catchment areas for abstraction points of water intended for human consumption. It does not cover the methodologies around catchment characterisation, hazard assessment, risk assessment, or definition and typology of risk management measures.

The model establishes the CatchmentArea as the primary unit for risk assessment, requiring polygon geometry and a mandatory link to a Water Framework Directive (WFD) water body.

The RiskAssessmentRiskManagement dataset functions as a high-level questionnaire regarding the status of risk assessments and measures, allowing Member States to link to supporting documentation via a standardized metadata structure.

Additionally, a WaterSupplyLink dataset is introduced to manage the complex many-to-many relationships that exist between water abstractions and water supply zones.

A central tenet of the proposal is the alignment with and reuse of existing dataflows to avoid double reporting, particularly regarding the WFD and the WISE-6 Water Quality reporting.

The CatchmentArea dataset is the only completely new spatial requirement; other elements such as water bodies, monitoring sites, and drinking water protection areas utilize existing European registers and data reported under the 3rd River Basin Management Plans (RBMPs).

For drinking water protection areas (including safeguard zones), the system will use pre-filled datasets from the previous WFD reporting as a baseline, so that Member States need only report updates or corrections.

Finally, the model incorporates specific mechanisms to handle data confidentiality and public security concerns. Member States may restrict the publication of spatial data for safeguard zones and monitoring sites using a confidentiality status attribute, ensuring that sensitive location data is protected while still being communicated to relevant authorities.

The proposal emphasizes a decoupled reporting cycle for monitoring sites and protected areas, allowing data providers to update information as needed independently of specific sectoral reporting deadlines.

# Narrative description

The conceptual model for the electronic reporting of the DWD Article 8 risk assessment and risk management of the catchment areas for abstraction points of water intended for human consumption is based on the following assumptions:

- Each drinking water abstraction point is located in one surface water body, or in one groundwater body.

- Each drinking water abstraction point has a physical catchment area or watershed.

- Each drinking water abstraction point provides water to one or more water supply zones.

- The primary unit for the risk assessment and risk management is the catchment area.

The following sections provide a brief narrative description of the classes and relationships of the class diagram (Figure 1).

Figure 1. Conceptual model for the recast DWD Article 8 electronic reporting – overview.

## Data model classes

C1 –	A CatchmentArea may include one or more abstraction points, in the same water body, if the risk profile is homogenous. A CatchmentArea is uniquely identified by a catchmentAreaIdentifier. The location of each CatchmentArea is reported using a polygon geometry.

C2 –	For the purposes on the electronic reporting, an WaterAbstraction is a logical node representing a withdrawal of water. It can represent a single physical abstraction point or a group of abstraction points within one WaterBody. Each WaterAbstraction is associated with a CatchmentArea. Each abstraction is uniquely identified by a waterAbstractionIdentifier. Note: the concept implies a location but does not require the reporting of a geometry.

C3 –	A WaterSupplyZone is the distribution area where the water is consumed. A water supply zone receives water from one or more water abstractions. Note: The water supply zones are reported under the DWD exceedances, incidents and derogations - Article 18.1 (c, d, e) dataflow.

C4 –	A WaterBody is the WFD management unit (groundwater or surface water) where the abstraction physically occurs. Note: the WFD water bodies are reported under the WISE Spatial electronic reporting.

C5 –	A MonitoringSite is a location in the WaterBody used to verify the quality of the water for the catchment area risk assessment. Note: the WFD monitoring sites are reported under the WISE Spatial electronic reporting.

C6 –	Within each CatchmentArea, Member States may define zero or more safeguard zones. Such drinking water protection zones may protect the water abstraction, one or more geographical areas in the catchment area (e.g. recharge zones), or even the entire catchment area. Each ProtectedArea is uniquely identified by a protectedAreaIdentifier. The location of each ProtectedArea is reported using single or multipart geometries. Member States may restrict the publication of the location of safeguard zones for water abstractions, using a confidentialityStatus value.

C7 –	The synthesis of RiskAssessmentRiskManagement plan is provided for each CatchmentArea.

## Data model relationships

The model is centred on the CatchmentArea as the main entity for the Risk AssessmentRiskManagement plan, and the concept of WaterAbstraction as the logical link between the resource (WaterBody) and the consumer (WaterSupplyZone).

R1 –	CatchmentArea [1] ↔ [1] WaterAbstraction

- Logic: A Member State defines a CatchmentArea for the purpose of risk assessment. The catchment area must contain at least one abstraction point but may potentially contain various abstraction points. The identity and location of the physical abstraction points is not needed and is not requested. The set of abstraction points for which the catchment area risk assessment was performed is encapsulated in the concept of WaterAbstraction.

- Note: This satisfies the requirement that a CatchmentArea can cover multiple abstractions points and that the location of the abstraction points is not requested.

R2 –	WaterBody [1] ↔ [1..n] WaterAbstraction

- Logic: Every WaterAbstraction must be assigned to exactly one WaterBody. A water body may have several physical abstraction points, aggregated into one or more WaterAbstraction.

- Note: If the risk profile is similar for all abstraction points in the water body, they may be aggregated into a single WaterAbstraction with a single CatchmentArea risk assessment. In other circumstances, the risk assessment may be done for different subsets of abstractions points in the waterbody (e.g. the subset of abstraction points supplying a densely populated water supply zone vs less critical abstraction points).

- Note: The WaterAbstraction has no geometry. Its location is logically defined by its parent WaterBody and the CatchmentArea it belongs to.

R3 –	WaterAbstraction [m] ↔ [n] WaterSupplyZone

- Logic: This is a many-to-many relationship. Water from one WaterAbstraction (logical unit) can be distributed to multiple WaterSupplyZone. One WaterSupplyZone can receive water from multiple WaterAbstraction.

R4 –	CatchmentArea [0..1] ↔ [0..n] ProtectedArea

- Logic: To manage risks found in the CatchmentArea, safeguard zones may be defined. There may be zero (if no significant risks) or many protected areas. The ProtectedArea geometry is reported using single or multipart geometries.

- Technical note: If more than one CatchmentArea exists per WaterBody, the ProtectedArea should be linked to a specific CatchmentArea. That will be denoted by relateZoneIdentifier = CatchmentAreaIdentifier.thematicIdIdentifier AND relatedZoneIdentifierScheme = CatchmentAreaIdentifier.thematicIdIdentifierScheme.

- Special case: If the safeguard zone encompasses the entire catchment area, that will be denoted by the value specialisedZoneType = 'catchmentArea'.

R5 –	WaterBody [0..1] ↔ [0..n] ProtectedArea

- Technical note: If only one CatchmentArea exists per WaterBody, the ProtectedArea should be linked to directly to the water body. That will be denoted by relateZoneIdentifier = WaterBody.thematicIdIdentifier AND relatedZoneIdentifierScheme = WaterBody.thematicIdIdentifierScheme.

- Special case: If the safeguard zone encompasses the entire water body, that will be denoted by the value specialisedZoneType = 'waterBody'.

R6 –	WaterAbstraction [0..1] ↔ [0..n] ProtectedArea

- Technical note: There will be no direct link between the ProtectedArea and the WaterAbstraction. Instead, the approach defined before should be followed: 1) if only one CatchmentArea exists per WaterBody, the ProtectedArea should be linked to directly to the water body; 2) if more than one CatchmentArea exists per WaterBody, the ProtectedArea should be linked to a specific CatchmentArea.

- Special case: If the safeguard zone is designed to directly protect the abstraction points (e.g. protection perimeters around groundwater abstraction points), that will be denoted by the value specialisedZoneType = 'waterAbstractionSafeguardZone'.

- Special case: Member States may limit public access to spatial data sets where such access would adversely affect inter alia the public security or the protection of the environment. The publication of the location of the safeguard zones may be prevented by specifying confidentialityStatus = 'N'. (Note that the same approach is already use for monitoring sites that are co-located with the abstraction points.)

R7 –	CatchmentArea [1] ↔ [1..n] MonitoringSite

- Logic: While the monitoring site physically sits in a WaterBody, for this specific reporting flow, it provides data validation for the CatchmentArea risk assessment.

- Technical note: The relationship can be obtained via CatchmentArea [1] ↔ [1] WaterAbstraction [1] → [1] WaterBody [1] ← [1..n] MonitoringSite and the spatial condition ST_Within(MonitoringSite.geometry, CatchmentArea.geometry) = TRUE.

- Note: Optionally, Member States may wish to detail the subset of monitoring sites relevant for drinking water monitoring. That information is provided in the MonitoringSite.purpose attribute.

## Alignment with other dataflows – spatial datasets

This section details the alignment and consolidation with data already provided under different reporting dataflows and explains the reuse of similar models to facilitate the processes both at national and at European level.

A1 –	WaterSupplyZone
The water supply zones are already reported under the "DWD exceedances, incidents and derogations - Article 18.1 (c, d, e)" reporting obligation. The dataset can be updated, if needed, under that annual dataflow.
The consolidated list of water supply zone identifiers will be used in the quality control of the "DWD Risk assessment and risk management of the catchment areas for abstraction points of water intended for human consumption - Article 8". 
No changes are foreseen to the water supply zone reporting.

A2 –	MonitoringSite
The WISE European register of monitoring sites is available at https://dd.eionet.europa.eu/vocabulary/wise/MonitoringSite. The register will be used in the quality control of the DWD reporting – if the data providers wish to explicitly report the list of monitoring sites associated with each catchment area. 
During the 3rd cycle of reporting of the Water Framework Directive River Basin Management Plans, the monitoring sites are reported under the "Water Framework Directive - River Basin Management Plans - 2022 Spatial data" reporting obligation.
Presently a separate dataflow is being implemented to allow the update of the MonitoringSite dataset, if needed, in an annual dataflow. 
Minor simplifications are foreseen to the data model (removal of seldom reported or seldom used attributes). The supported data format (for data providers not using the reporting platform API) will change from shapefile to geopackage. Data delivery will move from the Reportnet2 platform to the Reportnet3 platform.

A3 –	WaterBody
The WISE European register of water bodies is available at https://dd.eionet.europa.eu/vocabulary/wise/WaterBody. The register will be used in the quality control of the DWD reporting – namely to validate the association between water bodies and catchment areas.
During the 3rd cycle of reporting of the River Basin Management Plans (RBMPs) under  the Water Framework Directive (WFD), the water bodies were reported under the "Water Framework Directive - River Basin Management Plans - 2022 Spatial data" reporting obligation.
Presently, the preparation of the electronic reporting for 4th cycle of RBMPs is ongoing, including the possibility to report the surface water bodies and the groundwater bodies independently from other spatial datasets. Minor simplifications are foreseen to the data model (removal of seldom reported or seldom used attributes). The supported data format (for data providers not using the reporting platform API) will change from shapefile to geopackage. Data delivery will move from the Reportnet2 platform to the Reportnet3 platform. 
The default reference data for the DWD Article 8 reporting are the waterbodies reported in the 3rd RBMPs and already in the WISE register. However, it is likely that some Member States will prefer to update their datasets in preparation for both the 4th RBMPs and the DWD risk assessment reporting.

A4 –	ProtectedArea
The WISE European register of WFD Protected Area is available at https://dd.eionet.europa.eu/vocabulary/wise/WFDProtectedArea. The register will be used in the quality control of the DWD reporting – namely to validate the association between safeguard zones and water bodies or catchment areas.
During the 3rd cycle of reporting of the River Basin Management Plans (RBMPs) under the Water Framework Directive (WFD), the drinking water protection areas were reported. 
A detailed analysis of the data was made to support the simplification of the reporting.

A consolidated dataset is being prepared with the data reported under the "Water Framework Directive - River Basin Management Plans - 2022 Spatial data" reporting obligation and/or under the " Reporting obligation for: Water Framework Directive - River Basin Management Plans - 2022 RBD XML data" reporting obligation.

The consolidated dataset will be the pre-filled "baseline" for any updates needed - for both the DWD Article 8 risk assessment reporting and the 4th RBMPs.

As mentioned above, the revised data model will: 1) include an optional confidentialityStatus attribute, 2) require the specification of the type of zone and 3) require the association of the protected zone with a water body or with a catchment area.  The supported data format (for data providers not using the reporting platform API) will change from shapefile to geopackage. Data delivery will move from the Reportnet2 platform to the Reportnet3 platform – and be included in only in the DWD risk assessment reporting.

To synthesise: the drinking water protection areas (including safeguard zones) can be updated, if needed, in the DWD Article 8 risk assessment reporting. No further reporting is necessary in the 4th RBMPs.

To summarise:

Most of the spatial datasets related to DWD Article 8 risk assessment reporting has already been reported under other reporting obligations. If needed, the datasets may be updated in preparation for both DWD and the 4th RBMPs reporting.

Only the CatchmentArea dataset is new and specific to the DWD Article 8 risk assessment reporting.

Table 1 shows a comparison between the CatchmentArea data model and the WaterSupplyZone data model that is used in the DWD Article 18.1 (c, d, e) reporting.

The structure of the two models is nearly identical, to simplify the reporting. 
In the CatchmentArea dataset, the sizeValue and sizeUom attributes have been removed (and will be derived from the mandatory polygon geometry). The relatedZoneIdentifier and relatedZoneIdentifierScheme attributes will make explicit the link between each CatchmentArea and a WFD WaterBody.

Part of the attributes have fixed values (thematicIdIdentifierScheme, predecessorsIdentifierScheme, successorsIdentifierScheme, zoneType, specialisedZoneType) but have been kept to maintain the structural alignment with other reporting obligations and facilitate interoperability. Other attributes have default values (legalBasisName, legalBasisLink, legalBasisLevel) but allow the data providers to use the specific national values if desired.

Table 1. Comparison between the CatchmentArea dataset and the WaterSupplyZone dataset.

## Descriptive data specific to the DWD Article 8 dataflow

The previous section presented the spatial datasets most relevant for the DWD Article 8 dataflow, specifically the CatchmentArea dataset with the delineation of each risk assessment unit and the ProtectedArea dataset with the update to the safeguard zones.

The current section presents the so-call "descriptive data" relevant for the DWD Article 8 dataflow, specifically the RiskAssessmentRiskManagement dataset and the WaterSupplyLink dataset (Figure 2, Table 2).

Figure 2. Class diagram for the recast DWD Article 8 electronic reporting.

The RiskAssessmentRiskManagement dataset provides an overview of the status of the DWD Article 8 risk assessment for each catchment area, for a given reference date.

It is a simple questionnaire, allowing Member States to provide support documentation in ancillary documents or in national sites.

The function of the WaterSupplyLink dataset is merely to resolve the many-to-many relationship that may exist between water abstractions and water supply zones (see Figure 1 and section 2.2). It is important to emphasize that the waterAbstractionIdentifier is optional and the waterAbstractionName is also optional – these attributes are only included because they may facilitate the work of Member States when exporting the data from the national systems. The only mandatory attributes are the catchmentAreaIdentifier and the waterSupplyZoneIdentifier, the combination of which must be unique across the dataset.

Table 2. Details of the RiskAssessmentRiskManagement dataset and the WaterSupplyLink dataset.

## Note on the characterisation of the catchment areas

The "description of land-use, runoff, and recharge processes in the catchment areas for abstraction points" is a mandatory component of the initial characterisation of the catchment areas (Article 8(2)(a)(iv)) required for the risk assessment under the recast Drinking Water Directive (DWD).

Table 2 focuses exclusively on simplified electronic reporting requirements and, as such, does not presently include attributes detailing the land-use categories present in each catchment area – since that information can be consistently derived at European level using Copernicus datasets [].  Similar approaches are possible for other characteristics, e.g. slope [], long-term potential groundwater recharge (mm/year) [] or river discharge and surface run-off [].

The following two lists exemplify how characteristics of a catchment area could be captured, derived from a GIS analysis of relevant Copernicus datasets using the Member State reported Catchment Areas.

| RunOff- dominant surface run-off behaviour and average slope class of catchment |  |
| --- | --- |
| code | description |
| TypeSlow/steady | Dominant surface run-off behaviour slow/steady |
| TypeRapid/flashy | Dominant surface run-off behaviour rapid/flashy |
| TypeSeasonal | Dominant surface run-off behaviour seasonal/variable |
| TypeUnknown | Dominant surface run-off behaviour unknown/Not applicable |
| SlopeFlat | Average slope of catchment <2% |
| SlopeModerate | Average slope of catchment 2-15% |
| SlopeSteep | Average slope of catchment >15% |
| SlopeUnknown | Average slope of catchment unknown/Not applicable |

| RechargeProcesses - type and rate of recharge processes |  |
| --- | --- |
| code | description |
| TypeNatural | Natural recharge process (precipitation/infiltration) |
| TypeArtificial | Artifical recharge process (managed recharge/injection) |
| TypeMixed | Natural and artificial recharge process |
| TypeUnknown | Unknown recharge process/Not applicable |
| RateHigh | Mean annual groundwater recharge > 300 mm/year; strong infiltration and continuous replenishment |
| RateAverage | Mean annual groundwater recharge 100 - 300 mm/year; Moderate infiltration and aquifer replenishment |
| RateLow | Mean annual groundwater recharge < 100 mm/year; Limited infiltration, groundwater replenishment is weak or seasonal |
| RateUnknown | Unknown recharge rate/Not applicable |

## Alignment with other dataflows – documents and metadata

As mentioned in the previous section, Member States may provide support documentation in ancillary documents or in national sites. The proposal mechanism is identical to what is used in other WISE reporting dataflows (Table 3).

Each DocumentReference is basically a bookmark describing where further information about a given topic may be found in a Document. The concept of Document is generic: it can be a physical document (for example, an uploaded PDF) or a hyperlink to a persistent resolvable URL in a national site (for example, with an API call returning a page with up-to-date information with the measures implemented for a given catchment area).

Finally, the dcMetadata contains a simplified set of Dublin Core metadata elements.

Typically, the table will contain a single record per Member State – although in some dataflows (WFD, Floods) it may contain separate records for different units of management (RBDs, Floods Units of Management).

In the proposed model, an additional attribute was introduced as request by the European Commission: the total numberOfCatchmentAreas must be a non-negative integer that must match the total number of catchment areas being reported.

Table 3. Details of the DocumentReference, Document and dcMetadata datasets.

## Alignment with other dataflows – monitoring results

The reporting of monitoring results relevant for the DWD Article 8 risk assessment and subsequent monitoring does not require the definition of additional datasets or dataflows.

The WISE SoE Water quality (WISE-6) dataflow is a well-established and well-known annual dataflow used by all Member States to report monitoring results (Figure 3).

The WISE-6 dataflow is used both for voluntary reporting and for reporting under EU obligations (for example the Environmental Quality Standards Directive - Watch list annual reporting). It can, and should, also be used for the reporting of the DWD related monitoring results (like the Watch list reporting, data providers may choose to tag individual records as relevant for the DWD reporting).

Information about the reporting templates, guidance documents, and national data already reported in the 2025 WISE 6 and Watchlist data call is available in the Reportnet3 platform at https://reportnet.europa.eu/public/dataflow/1610.

Figure 3. Class diagram for the reporting of monitoring results under WISE SoE - Water quality (WISE-6).

# Annex 1 - Reporting of the catchment areas

This annex provides an overview of the CatchmentArea dataset, for data providers not familiar with the WISE spatial data reporting.

## Overview

- The DWD Article 8 reporting unit is the catchment area.

- For the purpose of the current document, the term catchment area is defined as 
a zone “established in accordance with a legislative requirement related to an environmental policy or a policy or activity that may have an impact on the environment at any level of administration (international, European, national and sub-national)” (see http://inspire.ec.europa.eu/theme/am), and specifically the zones established in accordance with Article 8 of the DWD.

- Each reporting unit shall have a unique persistent identifier at European level.

- To ensure the uniqueness and persistence of the identifiers, the European Environment Agency shall maintain an identifier scheme dedicated to this purpose.

- The 'euDrinkingWaterReportingUnitCode' identifier scheme is currently used for the DWD water supply zones. It will be extended to include the catchment areas.

- Catchment areas will be reported using the same generic model used for the water supply zones.

- The specific requirements applicable to the catchment areas are listed below:

- A catchment area MUST have polygon geometry.

- A catchment area MUST have one and only one associated water body. 
A water body MAY have zero or more associated catchment areas.
Note: only the relationship between catchment area and water body is requested. The inverse relationship is not reported. The water bodies are not reported.

- A catchment area MUST have one or more associated abstraction points. 
Note: the reporting of abstraction points is not requested.

- Information about the national identifiers of the catchment areas 
is kept in the [inspireIdLocalId], [inspireIdNamespace], [inspireIdVersionId], 
[beginLifespanVersion] and [endLifespanVersion] attributes. 
This information allows data providers to maintain traceability between the reported data and the data in their national systems and/or national or regional INSPIRE datasets.

- The European persistent unique identifier is reported in the [thematicIdIdentifier] attribute. It must follow the syntax requirements define for the WISE identifiers and it must be unique within the [thematicIdIdentifierScheme], which is always 'euDrinkingWaterReportingUnitCode'.

- The reporting model includes 5 attributes that allow the data providers to report changes to the catchment areas, i.e. to report life-cycle information: [predecessorsIdentifier], [predecessorsIdentifierScheme], [successorsIdentifier], [successorsIdentifierScheme] and [wiseEvolutionType].

- In the 1st reporting cycle, this information is simple to report: all catchment areas with have [wiseEvolutionType] = 'creation' and the remaining attributes have NULL values.

- The definition of the [nameTextInternational], [nameText] and [nameLanguage] attributes follows the generic definition used in all WISE spatial data models.

- The validity period for the risk assessment in a catchment area is reported in the attributes [designationPeriodBegin] and [designationPeriodEnd]. In the 1st reporting cycle, the attribute [designationPeriodEnd] will always be NULL.

- The following 2 attributes will have fixed values for all catchment areas:

- [zoneType] = 'drinkingWaterRiskAssessment'

- [specialisedZoneType] = 'catchmentArea'

- The following 3 attributes will have default values for all catchment areas:

- [legalBasisName] = 'Directive (EU) 2020/2184 of the European Parliament and of the Council of 16 December 2020 on the quality of water intended for human consumption (recast).'

- [legalBasisLink] = 'https://eur-lex.europa.eu/eli/dir/2020/2184/oj'

- [legalBasisLevel] = 'european'

- The following 2 attributes will relate each catchment area with a valid WFD surface water body or groundwater body: [relatedZoneIdentifier] and [relatedZoneIdentifierScheme].

- Finally, the optional [link] attribute allows the reporting of a resolvable URL to national systems where further information is made available.

## Conclusion

The reporting of catchment areas follows a model similar to the water supply zones.

Note that, in practice – and for the 1st cycle – only limited information is necessary:

- [geometry] – the geometry

- [thematicIdIdentifier] – the unique identifier

- [nameTextInternational], [nameText], [nameLanguage] – the name(s)

- [designationPeriodBegin] – the beginning of the validity period

- [relatedZoneIdentifier], [relatedZoneIdentifierScheme] – the related water body

With respect to the dichotomy between national and European identifiers, data providers may choose to the same value, if the national identifier complies with the WISE requirements (around 70% of the water supply zones follow this pattern, which further simplifies the reporting from the data providers perspective).

All the remaining attributes have fixed values or have default values or are optional.

They are kept in the reporting model to provide clarity and facilitate the reuse of the data.

# Annex 2 - Reporting of drinking water protection areas

Article 8.2(a)(ii) of the DWD defines that the risk assessment and risk management of the catchment areas for abstraction points of water intended for human consumption must include the “mapping of the safeguard zones, where those zones have been established in accordance with Article 7(3) of Directive 2000/60/EC”.

The purpose of this annex is:

- to verify whether such zones have already been reported by Member States (MS) under the 3rd cycle of reporting of the River Basin Management Plans (3rd RBMPs) under the Water Framework Directive (WFD), and

- to verify whether the WFD reporting and the DWD reporting can be consolidated and simplified to obtain data with better quality while avoiding double reporting.

The "Background information" section summarises the legal references to drinking water protected areas and safeguard zones in both the WFD and the recast DWD.

The "Reporting guidance in the 2nd and 3rd RBMPs" section summarises the Commission's guidance on the electronic reporting of these areas under WFD.

The "Drinking water protection areas in the 3rd RBMPs" presents the data already reported in the 3rd RBMPs using examples to illustrate the issues that should be addressed in the proposed model or that may require further discussion with specific MS.

The "Consolidating the reporting" section presents an overview of the proposed model to enable an informed discussion with MS representatives that might not be familiar with the model already used in the 3rd RBMPs.

## Background information

Article 8.2(a)(ii) of the DWD defines that the risk assessment and risk management of the catchment areas for abstraction points of water intended for human consumption must include the “mapping of the safeguard zones, where those zones have been established in accordance with Article 7(3) of Directive 2000/60/EC”.

Article 7 of the WFD, on the waters used for abstraction of drinking water, establishes that:

"1. Member States shall identify, within each river basin district:

- all bodies of water used for the abstraction of water intended for human consumption providing more than 10 m3 a day as an average or serving more than 50 persons, and

- those bodies of water intended for such future use.

Member States shall monitor, in accordance with Annex V, those bodies of water which according to Annex V, provide more than 100 m3 a day as an average.

2. For each body of water identified under paragraph 1, in addition to meeting the objectives of Article 4 in accordance with the requirements of this Directive, for surface water bodies including the quality standards established at Community level under Article 16, Member States shall ensure that under the water treatment regime applied, and in accordance with Community legislation, the resulting water will meet the requirements of Directive 80/778/EEC as amended by Directive 98/83/EC.

3. Member States shall ensure the necessary protection for the bodies of water identified with the aim of avoiding deterioration in their quality in order to reduce the level of purification treatment required in the production of drinking water. Member States may establish safeguard zones for those bodies of water."

Article 6(2) of the WFD specifies that the register of protected areas "shall include all bodies of water identified under Article 7(1) and all protected areas covered by Annex IV".

Annex IV further specifies that:

"The register of protected areas required under Article 6 shall include the following types of protected areas:

- areas designated for the abstraction of water intended for human consumption under Article 7."

To summarise:

The WFD register of protected areas includes all the water bodies identified under Article 7(1) of the WFD and the safeguard zones that may be established for those water bodies under Article 7(3) of the WFD.

For the purposes of Article 7(2) of the WFD, the requirements of recast DWD now apply (as per Article 26(1) of the recast DWD).

## Reporting guidance in the 2nd and 3rd RBMPs

In 23/02/2016, the Commission made an analysis of the 1st RBMPs and provided guidance on the spatial reporting of the drinking water safeguard zones (link) for the 2nd RBMPs:

"Member States practice varies in the designation of Drinking Water protected areas. The following two broad approaches could be identified in the first RBMP, for which guidance is provided as regards reporting of spatial data:

- Some Member States follow literally WFD Article 7 and designate as protected areas the water bodies used for the abstraction of water intended for human consumption (and those intended for such future use). Therefore, there is no different spatial delineation of the protected areas: these are coincident with the WFD water bodies […]

- Some Member States delineate the part of the water bodies which they consider are protected areas. The spatial extent of the protected areas can therefore be different, typically smaller than WFD water bodies. This is often the case in large groundwater bodies. […]"

This understanding was maintained in 28/04/2022, in the updated guidance provided by the Commission for the 3rd RBMPs (link).

To summarise:

In a simplified perspective, the electronic reporting for the RBMPs required MS to identify the entire water body and/or the safeguard zone(s) – the implicit assumption being that the safeguard zones under Article 7(3) would be defined within the water bodies identified under Article 7(1).

The reality of the electronic reporting revealed a more complex situation, which will be described in the next section.

## Drinking water protection areas in the 3rd RBMPs

The examples in this section will illustrate the different approaches followed by MS and characteristics and limitations of either the data model or of the reported data.

Example #1 – The "entire waterbody" approach

Germany used the “entire waterbody” approach in the 3rd RBMPs reporting. 
The spatial data was reported only in the water body spatial datasets. The classification as a drinking water protection area was reported under the WFD descriptive data. Other safeguard zones were not reported. (Implicitly, the same identifier is used for the water body and for the drinking water protection area.)

Example #2 – Safeguard zones

Luxembourg reported one drinking water safeguard zone in the catchment area of a reservoir (Figure 4.a) and several safeguard zone around groundwater abstractions (Figure 4.b).

As the images show, the original WFD spatial data does not contain a link to the water body – although in the case of Luxembourg that information could be obtained from the WFD descriptive data reported in another dataflow. It is also not possible to immediately determine if the protected area is the entire water body, a safeguard zone, or simply the location of the abstraction (all cases exist in the European dataset, although only the 2nd case exists for LU).

Figure 4. Drinking water protection areas reported by Luxembourg [source:3rd RBMPs].

| a) Surface water safeguard zone | b) Groundwater safeguard zone |
| --- | --- |

Example #3 – Mixed approaches

The Portuguese legislation foresees the establishment of protection perimeters around water abstractions (Article 37 of the transposition of the WFD, link). For groundwater, the safeguard zones depend on the abstraction volume and on the hydrogeological characteristics of the aquifer (see technical criteria Annex, link). Special cases apply for example to karstic areas. Similar principles apply to surface water (link).

Detailed spatial information about these safeguard zones is available in national sites – see the PT Open Data portal (link) or the PT INSPIRE portal (link) – although the water body identifiers do not appear to have been updated since the 2nd RBMPs.

The content of the data reported in the 3rd RBMPs is different (see Figure 5.a): it appears to contain both entire waterbodies (although different identifiers are used for the protected area and for the water body itself) and the point locations (likely for a centroid in the safeguard zone around surface water abstractions, or the water abstraction itself). In most cases, each protected area is linked to a water body – but that information is missing in two River Basin Districts (PTRH9 and PTRH10).

Figure 5. Drinking water protection areas reported under the 3rd RBMPs [extract].

| a) Internal dataset | b) Public dataset |
| --- | --- |

Example #4 – Confidentiality issues

Figure 5 shows an extract of the drinking water protection areas data reported in the 3rd RBMPs. The comparison between the internal dataset (Figure 5.a) and the public European dataset (Figure 5.b) shows that the information reported by Portugal and by the Jucar RBD (ES080) in Spain is not published. Figure 6 explains the reason: all the files in the PT delivery are restricted, while only the ProtectedAreaPoint_ES_20230925.gml file was restricted in the ES delivery.

Since the data model didn't allow Member States to explicitly define which spatial objects should not be published, the indirect criteria were applied: all the PT drinking water protection areas were omitted, all the ES drinking water protection areas with point geometry were omitted. In Spain, most drinking water protection areas were reported as polygons – except in the Jucar RBD (ES080) – hence the "blank" area in the map.

This approach is not adequate – MS should be able to set the confidentiality status at record level (i.e. for any specific protected area).

Figure 6. Restricted files in the 3rd RBMP deliveries.

| a) PT delivery – all files restricted | b) ES delivery – only one file restricted |
| --- | --- |

Example #5 – Structured data vs. PDF documents

A final example illustrates the need for structured electronic reporting, again with the example from Luxembourg.

By September 2023, Luxembourg had not yet submitted the full electronic reporting of the 3rd RBMPs. Therefore, the Commission's assessment (link) was based only on the RBMPs documents.

The document refers only 45 of the 55 drinking water protection zones reported in June 2024, but – unlike other MS assessments based exclusively on the electronic reporting – does provide an insight into the type of safeguard zones defined by Luxembourg ().

The discrepancies highlight the different roles of the structured data vs the background documents. They also highlight the need to guarantee the completeness and consistency of the electronic reporting – and to make sure that the same single set of structured data can be used for both the DWD and the WFD assessments.

Figure 7. Commission staff document: 3rd RBMP implementation assessment for Luxembourg [extract].

To summarise, in the 3rd RBMPs:

- MS reported spatial data on the location of drinking water protection areas. If the drinking water protection area matched the entire water body and the identifier was the same, then reporting the geometry of the protected area was not required.

- Reporting the association between a drinking water protection area and the related water body was optional in the 3rd RBMPs spatial data reporting.

- The data model did not foresee the distinction between different types of drinking water protected areas – although an existing attribute (specialisedZoneType) could have been used to distinguish between water bodies identified under Article 7(1) of the WFD and safeguard zones defined under Article 7(3) of the WFD.

- The data model also did not foresee that some of the reported information might be confidential – which led to the adoption of publication criteria based indirect information (namely, if the data provider restricted public access to the file, and the file contained drinking water protection zones, then the information is not added to the public European datasets and services). This approach was not ideal, since public information exists in national sites that is not currently published in the public European data sets. The data model must allow MS to specify which records can be published, and which cannot.

Issues and examples based on the European datasets

Example #6 – Spatial data with missing information about the associated water body

Reporting the association between drinking water protection areas and water bodies was optional in the 3rd RBMPs spatial data reporting. It was reported in only 22% of the reported spatial objects (Table 4).

Table 4. Spatial data reporting: drinking water protection areas associated with water bodies [source: 3rd RBMP].

Example #7 – Descriptive data with information about the associated water body

Reporting the association between water bodies and drinking water protection areas was mandatory in the 3rd RBMPs descriptive data reporting. The vast majority (96%) of the reported protection areas is associated with a single water body (Table 5).

It is important to clarify with MS where more complex situations arise and how they can be modelled – also at MS level – to keep the reporting as simple as possible for all MS.

Table 5. Descriptive data reporting: number of different water bodies related to each drinking water protection area [source: 3rd RBMP].

Example #8 – Consistency between spatial data and descriptive data

The spatial data and the descriptive data were reported in different dataflows.

Consistency between the spatial and descriptive reporting of drinking water protection areas was not enforced in the 3rd RBMPs quality control: therefore, not all drinking water protection areas are referenced in the descriptive data reporting.

Overall, more 83 thousand drinking water protection areas were reported in the 3rd RBMPs. In 85% of the cases, the association between drinking water protection areas and water bodies can be inferred from the non-spatial reporting. In 12% of the cases, the association is unknown, and in 3% of cases there is ambiguity (Table 6).

Table 6. Number of different water bodies related to each drinking water protection area [source: 3rd RBMPs]

Conclusions:

- To avoid double reporting and to allow the possibility to update the existing data, a separate dataflow must be created exclusively for the drinking water protection areas, based on the “prefilled” data obtained from the 3rd RBMPs.

- The data must be reported only once and be used both for the DWD reporting obligations and the WFD reporting for the 4th RBMPs.

- To allow the consolidation of the two reporting data flows, the association between protection areas and water bodies must be mandatory – but the remaining conceptual model remains unchanged.

- To clarify the distinction between the "entire water body approach" and the "safeguard zone approach", a codelist should be provided for the specialisedZoneType attribute. 
Member States should be consulted on the type of values that would be relevant (or otherwise a simple separation of the two cases is possible).

- To address concerns raised by some MS, an additional optional confidentialityStatus attribute can be added – allowing data providers to prevent the publication of the location of specific drinking water protection water (should that be a requirement of the national legislation). This attribute already exists in the WISE MonitoringSite dataset, with the same objective.

## Consolidating the reporting

The drinking water protection areas spatial data should be reported using the WISE ProtectedArea model.

The model was defined for the WFD reporting of different types of protected areas and was already used in the 2nd and 3rd RBMPs. The model is known to Member States, so this document provides only a summary.

The reporting unit is the drinking water protection area.

- Each reporting unit shall have a unique persistent identifier at European level.

- To ensure the uniqueness and persistence of the identifiers, the European Environment Agency shall maintain an identifier scheme dedicated to this purpose.

- The 'euProtectedAreaCode' identifier scheme is currently used for all the WFD Protected Areas, including drinking water protection areas.

- The European register of the WFD Protected Areas is available in http://dd.eionet.europa.eu/vocabulary/wise/WFDProtectedArea/

The existence of a unique persistent identifier allows the maintenance of the European register of the WFD Protected Areas.

- The European persistent unique identifier is reported in the [thematicIdIdentifier] attribute. It must follow the syntax requirements define for the WISE identifiers and it must be unique within the [thematicIdIdentifierScheme], which is always 'euProtectedAreaCode'.

- Information about the national identifiers of the protected areas 
is kept in the [inspireIdLocalId], [inspireIdNamespace], [inspireIdVersionId], 
[beginLifespanVersion] and [endLifespanVersion] attributes. 
This allows data providers to maintain traceability between the reported data and the data in their national systems and/or national or regional INSPIRE datasets.

- Pre-filled geopackage files will be created for the data providers, consolidating the existing information reported in the 3rd RBMPs.

- If no updates are required, there is no need to report again.

- If new zones were created, deleted or modified, that information must be provided (while keeping the remaining records with [wiseEvolutionType] = 'noChange').

- A drinking water protection area MAY have point, line or polygon [geometry].

- For MS that use the "entire water body" approach, the geometry of the water body can be reused. MS may choose to use the same identifier for the protected area and the water body (under different identifier schemes), or to use different identifiers.

- A drinking water protection area MUST have one and only one associated water body. Note #1: Only the relationship between drinking water protection and water body is requested. The inverse relationship is not reported.

- A water body MAY have zero or more associated drinking water protection area.

Note #2: The water bodies are not reported. The European register of surface water bodies and groundwater bodies will be used to check the validity of the water body identifiers.

- The association between protected area and water body must be reported in the [relatedZone] attributes – and may be established via the water body identifier OR via the DWD catchment area identifier.

- The reporting model includes 5 attributes that allow the data providers to report changes to the protected area, i.e. to report life-cycle information: [predecessorsIdentifier], [predecessorsIdentifierScheme], [successorsIdentifier], [successorsIdentifierScheme] and [wiseEvolutionType].

- The [predecessorsIdentifier] and [predecessorsIdentifierScheme] values are requested only an existing protected area has significantly changed and therefore requires a new identifier.

- Typically, the [successorsIdentifier] and [successorsIdentifierScheme] values are not necessary – they were maintained in the model for completeness's sake, and because they are used when the European datasets are published (to allow traceability between data reported in the past and data currently valid).

- The definition of the [nameTextInternational], [nameText] and [nameLanguage] attributes follows the generic definition used in all WISE spatial data models.

- The validity period for the drinking water protection area is reported in the attributes [designationPeriodBegin] and [designationPeriodEnd].

- The following 2 attributes will have fixed values for all drinking water protected areas:

- [zoneType] = 'drinkingWaterProtectionArea'

- [specialisedZoneType] = <codelist to be defined>

- Although the [specialisedZoneType] attribute was not previously used for drinking water protection areas, it should be used – at least to clarify whether the distinction between the "entire water body" approach and the "safeguard zone" approach.

- The following 3 attributes will have default values for all drinking water protected areas, but allow specific national values to be reported:

- [legalBasisName] = 'Directive (EU) 2020/2184 of the European Parliament and of the Council of 16 December 2020 on the quality of water intended for human consumption (recast).'

- [legalBasisLink] = 'https://eur-lex.europa.eu/eli/dir/2020/2184/oj'

- [legalBasisLevel] = 'european'

- The following 2 attributes will relate each protected area with a valid WFD surface water body or groundwater body: [relatedZoneIdentifier] and [relatedZoneIdentifierScheme].

- The [confidentialityStatus] attribute was not part of the WFD model: it should be added to allow data providers to prevent the publication of the location of specific protected areas.

- Finally, the optional [link] attribute allows the reporting of a resolvable URL to national systems where further information is made available.

## Conclusion

The reporting of drinking water protection areas follows the model previously used for the Water Framework Directive reporting.

A separate dataflow will be created exclusively for the reporting of the drinking water protection areas, decoupling it from the WFD spatial data reporting, and allowing data providers to update the previously reported data if and when needed.

Prefilled geopackages files will be created, containing the consolidated data obtained from the Water Framework Directive spatial and descriptive reporting.

The reporting requirements will be identical to those of the Water Framework Directive reporting, but the association of each protected area with one water body will now be mandatory.

The confidentiality status of each protected area can be set by the data providers.

# Annex 3 - Reporting of abstraction points and monitoring sites

Article 8.2(a)(iii) of the DWD defines that the risk assessment and risk management of the catchment areas for abstraction points of water intended for human consumption must include the “geo-references for all abstraction points in the catchment areas; given that those data are potentially sensitive, in particular in the context of public health and public security, the Member States shall ensure that such data are protected and communicated only to the relevant authorities and water suppliers”.

The reporting of abstraction points and monitoring sites is not requested.

## Rationale

- The location of the monitoring sites was reported by Member States under the 3rd cycle of reporting of the River Basin Management Plans under the Water Framework Directive.

- Each monitoring site MAY be marked as confidential. The location of confidential sites is not made publicly available in any European data product.

- Each monitoring site MAY have one or more purpose. This allows data providers to tag a site as being used in the scope of specific monitoring programmes or legal requirements. The 'DWD' tag is optional but is used by some data providers to identify sites relevant for the drinking water monitoring.

- To avoid double reporting and allow data providers to update the existing data, a separate dataflow will be created exclusively for the monitoring sites, based on the “prefilled” data obtained from the WFD reporting.

- This will allow data providers to update their data if and when needed, and independently of a specific sectorial reporting flow (drinking water directive, nitrates directive, water framework directive, etc.).

- The data model is already known, so the following pages will provide only an overview, with a focus on DWD.

- A total of 217 634 monitoring sites were reported under WFD. 
Only 9 731 sites were explicitly tagged as 'DWD' sites, and their location is not included in European data products. An additional set of 16 500 sites were marked as confidential by the data providers, and their location is also not included in European data products. The table below summarises the current situation.

## Brief description of the MonitoringSite model

- Each monitoring site has a unique persistent identifier at European level.

- To ensure the uniqueness and persistence of the identifiers, the European Environment Agency maintains a European register of the Monitoring Sites.

- The register is available in http://dd.eionet.europa.eu/vocabulary/wise/MonitoringSite/

- The specific requirements applicable to the monitoring sites are listed below:

- A monitoring site MUST have point geometry.

- A monitoring site MUST have one and only one associated water body. A water body MAY have zero or more associated monitoring sites.  Note: only the relationship between monitoring site and water body is requested. The inverse relationship is not reported. The water bodies are not reported.

- Information about the national identifiers of the monitoring site 
is kept in the [inspireIdLocalId], [inspireIdNamespace], [inspireIdVersionId], 
[beginLifespanVersion] and [endLifespanVersion] attributes. 
This information allows data providers to maintain traceability between the reported data and the data in their national systems and/or national or regional INSPIRE datasets.

- The European persistent unique identifier is reported in the [thematicIdIdentifier] attribute. It must follow the syntax requirements define for the WISE identifiers and it must be unique within the [thematicIdIdentifierScheme], which is always 'euMonitoringSiteCode'.

- The reporting model includes 5 attributes that allow the data providers to report changes to the monitoring site, i.e. to report life-cycle information: [supersedesIdentifier], [supersedesIdentifierScheme], [supersededByIdentifier], [supersededByIdentifierScheme] and [wiseEvolutionType].

- Pre-filled geopackage files will be created for the data providers.

- If no updates are required, there is no need to report again.

- If new zones were created, deleted or modified, that information can be provided (while keeping the remaining records with [wiseEvolutionType] = 'noChange').

- The definition of the [nameTextInternational], [nameText] and [nameLanguage] attributes follows the generic definition used in all WISE spatial data models.

- The overall operational period for the monitoring site is reported in the attributes [operationalActivityPeriodBegin] and [operationalActivityPeriodEnd].

- The following 2 attributes will relate each monitoring site with a valid WFD surface water body or groundwater body: [featureOfInterestIdentifier] and [featureOfInterestIdentifierScheme].

- The [confidentialityStatus] attribute can be set to 'N' to prevent the publication of the site location in any future European data products.

- The [purpose] attribute can include the tag 'DWD' to identify the site as relevant is the scope of the Drinking Water Directive monitoring programmes. Important note: any monitoring site can be used to report the results of monitoring under Article 8(2)(c) of the Drinking Water Directive.

- Finally, the optional [link] attribute allows the reporting of a resolvable URL to national systems where further information is made available.

## Conclusion

The reporting of abstraction points and monitoring sites is not requested.

The reporting of monitoring sites will continue to follow the model previously used for the Water Framework Directive reporting.

A separate dataflow will be created exclusively for the reporting of the monitoring sites, decoupling it from the WFD spatial data reporting cycle, and allowing data providers to update the previously reported data if and when needed.

Prefilled geopackages files will be created, containing the consolidated data obtained from the Water Framework Directive spatial reporting.

See for example https://www.eea.europa.eu/en/analysis/publications/land-cover-accounts-an-approach-to-geospatial-environmental-accounting: The methods are flexible and may be applied to any categorical geospatial data, while statistics can be broken down to any reference region, e.g. administrative NUTS units, river basins, floodplains (floodplains dashboard) or Natura 2000 areas (Natura2000 dashboard).

See for example: https://dataspace.copernicus.eu/explore-data/data-collections/copernicus-contributing-missions/collections-description/COP-DEM

See for example: https://doi.org/10.1016/j.scitotenv.2022.153464 and the data available in the European Geological Data Infrastructure (https://www.europe-geology.eu/) at https://maps.europe-geology.eu/?mapname=geoera#baslay=baseMapGEUS&layers=potentialgroundwaterrecharge

See for example: The model-based hydrology Environmental Climate Variables (ECVs) from the Copernicus Emergency Management Service (CEMS) Hydrological Data Collection Centre (HDCC) gathers near real-time and historical hydrological data to support its European Flood Awareness System (EFAS) and Global Flood Awareness System (GloFAS).