Annex 6 to Qualifications Assessment System

SMART THERMAL ENERGY METERING EQUIPMENT

DESCRIPTION OF THE NATURE OF THE PROCUREMENT OBJECT

- Procurement object: Procurement of smart thermal energy metering equipment and devices to enable AB Vilniaus šilumos tinklai and/or other buyers to ensure remote metering of the thermal energy it produces.

- Procurement object shall be divided into the following categories:

  - Category I – Hot water metering devices;

  - Category II – Domestic heat meters;

  - Category III – Data protocol transducer equipment for heat station controllers;

  - Category IV – Heat station controllers;

  - Category V – Input heat meters;

  - Category VI – Heat distributors;

  - Category VII – Concentrators;

  - Category VIII – Data transmission device.

- Codes in the Common Procurement Vocabulary (CPV):

  - Main CPV code - 38550000 Meters;

  - Additional CPV codes per category:

    - 38421100 Water meters;

    - 38551000 Energy meters;

    - 31711120 Transducers;

    - 31712100 Microelectronic machinery and apparatus;

    - 32441200 Telemetry and control equipment;

    - 32423000 Network hubs;

    - 32500000 Telecommunications equipment and supplies.

- Category I – Hot water metering devices (CPV code- 38550000 Meters)

  - Category value - EUR 14.801.000,00 excluding VAT.

  - Main equipment to be procured and types of equipment:

    - Hot water metering equipment with wireless data transmission technology (hereafter referred to as Meters). Wireless M-Bus, NB-IoT and/or LoRaWAN based mechanical and/or ultrasonic hot water meters.

  - Description of the technical requirements to be met by the procured goods:

    - Hot water meters are metering equipment which fall within the scope of the technical regulations on measuring equipment and which may be placed on the market and put into service without restriction, provided that they are marked with the markings and/or signs referred to in the technical regulations attesting to the conformity assessment of the metering equipment carried out in the Republic of Lithuania or in another Member State of the European Union or in a State of the European Economic Area, and that they comply with the other requirements laid down in the technical regulations.

    - Meters shall comply with the Rules for Legal Metrological Regulation of Measuring Instruments, approved by Order of the Minister of Economy of the Republic of Lithuania No. 4-761 of 24 October 2014 (current version), and the Technical Regulation of Measuring Instruments, approved by Order of the Minister of Economy of the Republic of Lithuania No. 4-699 of 30 October 2015 (current version) (hereinafter referred to as the Regulation).

    - The hot water meter must bear the CE mark (certified in the European Union) and the Initial Metrological Verification mark.

    - Detailed technical requirements shall be provided at the time of each procurement.

  - The characteristics, functional requirements and/or desired result of the procurement object:

    - Hot water meters operate either ultrasonically or mechanically.

    - Hot water meters shall be equipped with built-in wireless data reading capability using Wireless M-Bus, NB-IoT and/or LoRaWAN technology.

- Category II – Domestic heat meters (CPV code - 38421100 Water meters).

  - Category value - EUR 16.959.000,00 excluding VAT.

  - Main equipment to be procured and types of equipment:

    - Domestic heat meter - a non-accountable heat metering device installed in the flats or other premises of heat consumers that measures the amount of heat consumed. The readings of these metering devices will be used to allocate the amount of heat supplied to the building to heat consumers. These devices will be installed in the flats or other premises of heat consumers.

    - Domestic heat meters with data transmission technology through wireless (Wireless M-Bus, NB-IoT and/or LoRaWAN) technology (hereafter referred to as heat meters).

  - Description of the technical requirements to be met by procured goods:

    - Domestic heat meters shall comply with the provisions of the Rules on Legal Metrological Regulation of Measuring Instruments, approved by Order of the Minister of Economy of the Republic of Lithuania No. 4-761 of 24 October 2014 (current version).

    - The domestic heat meter shall be a single or composite measuring instrument consisting of sub-assemblies (a calculator, a flow sensor, and a pair of temperature sensors).

    - The domestic heat meter must bear the CE mark (certified in the European Union) and the Initial Metrological Verification mark.

    - Domestic heat meters must be supplied new with a first metrological check for the current year.

    - Detailed technical requirements shall be provided at the time of each procurement.

  - The characteristics, functional requirements and/or desired result of the procurement object:

    - Domestic heat meters operate either ultrasonically or mechanically.

    - Domestic heat meters shall be equipped with built-in wireless data reading capability (Wireless M-Bus, NB-IoT and/or LoRaWAN), which will be specified in the technical requirements for each procurement.

- Category III – Data protocol transducer equipment for heat station controllers. (CPV code- 31711120 Transducers)

  - Category value - EUR 3.801.000,00 excluding VAT.

  - Main equipment to be procured and types of equipment:

    - Data protocol transducers (electronic devices) for changing the data protocols of the communication interface of controllers or metering devices used in heat stations in order to allow remote data exchange to/from customer information systems (hereinafter referred to as data protocol transducers). The Data Protocol Transducer shall be installed in the heat station or other premises near the data source.

  - Description of the technical requirements to be met by procured goods:

    - Data protocol transducers - low-voltage devices which fall within the scope of the technical regulations for measuring instruments and which may be placed on the market and put into service without restriction, provided that they are marked with the markings and/or signs referred to in the technical regulations attesting to the assessment of conformity of the measuring instrument carried out in the Republic of Lithuania or in another Member State of the European Union or in a Member State of the European Economic Area, and that they comply with the other requirements laid down in the technical regulations.

    - Data protocol transducers must have EU conformity assessment documentation and must bear the CE (Certified in the European Union) marking.

  - The characteristics, functional requirements and/or desired result of the procurement object:

    - The purpose of data protocol transducers is the transfer of data from/to a data source (heat station controller, metering device, impulse signal) to/from an information system via a communication protocol.

    - Data source and integration protocols and other technical parameters shall be specified at the time of each procurement.

- Category IV – Heat station controllers (CPV code – 31712100 Microelectronic machinery and apparatus)

  - Category value - EUR 4.243.000,00 excluding VAT.

  - Main equipment to be procured and types of equipment:

    - Heat station controllers - electronic devices used in heat stations to control the processes that supply the energy needed to heat a building and provide hot water.

  - Description of the technical requirements to be met by procured goods:

    - Heat station controllers are microprocessor-based electronic devices that fall within the scope of the technical regulations and may be placed on the market and put into service without restriction, provided that they bear the markings and/or signs referred to in the technical regulations attesting the conformity assessment of the measuring instrument carried out in the Republic of Lithuania or in another Member State of the European Union or in a Member State of the European Economic Area, and that they comply with the other requirements laid down in the technical regulations.

    - Heat station controllers must have EU conformity assessment documents and bear the CE (Certified in the European Union) marking.

  - The characteristics, functional requirements and/or desired result of the procurement object:

    - The purpose of the heat station controller is to ensure and control the set parameters of the controlled process (building heating circuit and hot water temperature) and to transmit/receive data directly or via data protocol converters to/from the customer’s information systems.

    - The detailed technical parameters and functionality of the heat station controllers to be procured will be provided at the time of each procurement.

- Category V – Input heat meters (CPV code – 38551000 Energy meters)

  - Category value - EUR 3.383.000,00 excluding VAT.

  - Main equipment to be procured and types of equipment:

    - Input heat meters with the ability to transmit data via digital data transfer protocols to customer information systems. Input heat meter - a metering device for accounting of heat supplied to consumers, the readings of which are used to settle with the heat supplier for the heat consumed. These devices will be installed at the inlets of heat points or heat sources.

  - Description of the technical requirements to be met by procured goods:

    - Input heat meters are measuring instruments which fall within the scope of the technical regulations and may be placed on the market and put into service without restriction, provided that they bear the markings and/or signs specified in the technical regulations attesting to the conformity assessment of the measuring instrument carried out in the Republic of Lithuania or in another Member State of the European Union or in a Member State of the European Economic Area, and that they comply with the other requirements laid down in the technical regulations.

    - Input heat meters shall comply with the Rules for Legal Metrological Regulation of Measuring Instruments approved by Order No. 4-761 of the Minister of Economy of the Republic of Lithuania of 24 October 2014 (current version) and the Technical Regulation on Measuring Instruments approved by Order No. 4-699 of the Minister of Economy of the Republic of Lithuania of 30 October 2015 (current version).

    - Input heat meter shall be a single or composite instrument consisting of sub-assemblies (calculator, flow sensor and a pair of temperature sensors).

    - The heat input meter must bear the CE mark (certified in the European Union) and the Initial Metrological Verification mark.

    - Input heat meters are supplied new with the first metrological check of the current year.

  - The characteristics, functional requirements and/or desired result of the procurement object:

    - Input heat meters operate either ultrasonically or mechanically.

    - Input heat meters must have an integrated data reading capability, the requirements of which will be specified in the technical requirements for each procurement.

    - The detailed technical specifications of the heat meters to be procured will be provided at the time of each procurement.

- Category VI – Heat distributors (CPV code – 32441200 Telemetry and control equipment)

  - Category value - EUR 1.321.000,00 excluding VAT.

  - Main equipment to be procured and types of equipment:

    - Heat distributor indicators with wireless technology for data transmission (hereinafter referred to as heat distributors). Heat distributors are designed to calculate the amount of heat consumed in relative units by heat sources (radiators) in consumers’ apartments or other premises.

  - Description of the technical requirements to be met by procured goods:

    - Heat distributor means measuring instruments which fall within the scope of the technical regulations and may be placed on the market and put into service without restriction, provided that they bear the markings and/or signs referred to in the technical regulations attesting to the conformity assessment of the measuring instrument carried out in the Republic of Lithuania or in another Member State of the European Union or in a State of the European Economic Area, and that they comply with the other requirements laid down in the technical regulations.

    - Heat distributor shall comply with EN 834:1995 - Heat distributors for the determination of the heat consumed by space heating radiators. Electrically powered appliances or an applicable newer version.

  - The characteristics, functional requirements and/or desired result of the procurement object:

    - Heat distributors shall have a minimum of two temperature sensors: one for ambient temperature and one for radiator surface temperature.

    - Detailed technical specifications of the distributors to be procured will be provided at the time of each procurement.

- Category VII – Concentrators (CPV code – 32423000 Network hubs)

  - Category value - EUR 2.645.000,00 excluding VAT.

  - Main equipment to be procured and types of equipment:

    - Wireless M-Bus, NB-IoT and/or LoRa hubs with wireless data transmission technologies to the customer's information systems (hereinafter referred to as concentrators).

  - Description of the technical requirements to be met by procured goods:

    - Wireless M-Bus, NB-IoT and/or LoRaWAN concentrators are electronic devices which fall within the scope of the technical regulations and which may be placed on the market and put into service without restrictions, provided that they are marked with the markings and/or signs referred to in the technical regulations confirming the assessment of conformity of the electronic devices carried out in the Republic of Lithuania or in another Member State of the European Union or in the State of the European Economic Area, and that they comply with the other requirements laid down in the technical regulations.

    - Wireless M-Bus and NB-IoT and/or LoRaWAN concentrators must have EU conformity assessment documents and must bear the CE (Certified in the European Union) mark.

    - Wireless M-Bus, NB-IoT and/or LoRaWAN concentrators running Wireless M-Bus, NB-IoT and/or LoRaWAN open standard, wireless for meter reading.

  - The characteristics, functional requirements and/or desired result of the procurement object

    - Wireless M-Bus, NB-IoT and/or LoRaWAN concentrators compatible with the newest version of the Open Metering System (OMS), capable of operating in Lithuania and in a Member State of the European Union or in a country of the European Economic Cooperation.

    - Detailed technical specifications of the concentrators to be procured will be provided at the time of each procurement.

- Category VIII – Data transmission device (CPV code – 32500000 Telecommunications equipment and supplies)

  - Category value - EUR 648.000,00 excluding VAT.

  - Main equipment to be procured and types of equipment:

    - Data transmission devices include mobile modems, routers, data switches and fibre optic converters.

  - Description of the technical requirements to be met by procured goods:

    - Data transmission devices are electronic devices that fall within the scope of the technical regulations on the safety of electronic products and may be placed on the market and put into service without any restrictions, provided that they are marked with the markings and/or signs referred to in the technical regulations attesting to the conformity assessment of the device carried out in the Republic of Lithuania or in another Member State of the European Union or in the Member State of the European Economic Area, and that they comply with the other requirements laid down by the technical regulations.

    - Data transmission devices shall comply with the essential requirements of Directive 2011/65/EC or its newer version in force and shall comply with the requirements of IEEE 802.3 or its newer version in force.

    - Data transmissions devices must bear the CE mark (certified in the European Union).

  - The characteristics, functional requirements and/or desired result of the procurement object:

    - The purpose of data transmission devices is to transmit data to the customer’s information systems using mobile or wired technologies.

    - Detailed technical specifications of the data transmission devices to be procured will be provided at the time of each procurement.

- Qualification requirements for procurement objects:

  - The qualification requirements for all categories are set out in Annex 2 of the Qualifications Assessment System (QAS), “Qualification requirements for suppliers”. This stage of the procurement requires the necessary qualifications that will be required for all or part (by category) of the procurement. In cases where other regulatory provisions impose an obligation on the supplier to have a specific right to engage in the activity to be certified/licensed, the obligation to provide documents issued in accordance with the specific regulatory provisions or other documents conferring the right to act shall remain, even if the corresponding requirement is not included in the Procurement Documents with prior knowledge, and will have to be submitted upon request of the customer prior to the start of the supply of equipment.

  - If the supplier’s qualifications with regard to the right to pursue the activity in question have not been, or have not been fully, verified, the supplier undertakes that the contract will be performed only by persons qualified to do so.

- The specific requirements for the procurement object to be procured will be set out in the specific procurement documents.

- The procurement documents will require that the supplier is the manufacturer of the specified goods, or the manufacturer’s official distributor, or an authorised representative of the manufacturer’s official distributor, and has the right to sell the specified goods.

- The supplier may not offer goods (including their components, packaging) or services if the origin of the goods (including their components, packaging) or the provision of the services is from the countries or territories referred to in the list referred to in Article 92(15) of the Public Procurement Law.

- The supplier may not offer goods which, in accordance with Article 50(9) of the Public Procurement Law of the Republic of Lithuania in the field of water management, energy, transport or postal services, pose a threat to national security.

- The contracting entity shall, when carrying out procurement involving goods or services of the CPV codes listed in Article 92(13) of the Public Procurement Law, consider that the goods or services constitute a threat to national security when:

  - the manufacturer of the goods or the person controlling the manufacturer of the goods is registered (if the manufacturer or the person controlling the manufacturer is a natural person, he/she is a resident or a citizen) in the countries or territories referred to in the list referred to in Article 92(14) of the Public Procurement Law;

  - the supply of services is to be carried out from the countries or territories referred to in the list provided for in Article 92(14) of the Public Procurement Law.

- If, in describing the object of the technical specification or other procurement documents or their annexes, a specific model or source, a specific process or trademark, patent, types, specific origin or manufacture is referred to in the technical specification or other procurement documents or their annexes, such reference shall be understood by the supplier to mean “or equivalent”.

- If standards, technical certificates, or general technical specifications are referred to in the description of the object in the technical specification or in other procurement documents or their annexes, such reference shall be understood by the supplier to mean “or equivalent”.

- The Green Procurement Criteria will be set in specific procurements, in accordance with the procedure for the application of environmental criteria in green procurement, approved by Order No. D1-508 of the Minister of the Environment of the Republic of Lithuania on 28 June 2011.

_____________________